3GPP TSG-RAN WG4 Meeting #98-e	R4-2101069

# Electronic Meeting, 25 Jan. – 5 Feb., 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | DraftCR | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR V2X PSFCH demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-1-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The V2X PSFCH demodulation requirements are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new requirements for PSFCH demodulation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

# 11.X PSFCH demodulation requirements

## 11.X.1 NACK missed detection requirements

### 11.X.1.1 General

The NACK missed detection probability is the probability of not detecting an NACK when an NACK was sent. The test parameters in table 11.X.1.1-1 are configured.

Table 11.X.1.1-1: Test Parameters

| Parameter | unit | Test 1 |
| --- | --- | --- |
| Communication resource pool configuration |  | [As specified in Table A.X-X (Configuration #X-V2X)] |
| at antenna port | dBm/15kHz | -98 |
| Allocated resource blocks | RB | 1 |
| The number of PSFCH symbols | symbol | 2 |
| Number of information bits | bit | 1 |
| Synchronization source |  | GNSS |
| Timing offset | Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| sl-PSFCH-Period-r16 | slot | 1 |
| sl-NumMuxCS-Pair-r16 |  | 1 |
| sl-MinTimeGapPSFCH-r16 | slot | [2] |
| sl-PSFCH-HopID-r16 |  | [0/disable] |
| sl-PSFCH-CandidateResourceType-r16 |  | [‘startSubCH’] |

### 11.X.1.2 Minimum requirements

The NACK missed detection probability shall not exceed 1% at the SNR given in table 11.X.1.2.1.

Table 11.X.1.2-1: Minimum requirements

| Test num. | Bandwidth | Subcarrier spacing | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | NACK missed detection probability (%) | SNR (dB) |
| 1 | 20 MHz | 30kHz | TDLA30-180 | 1 | TBD |

## 11.X.2 DTX to NACK requirements

### 11.X.2.1 General

The DTX to NACK probability, i.e. the probability that NACK is detected when nothing was sent:

ProbPSFCH DTX→Ack bits= #(false NACK bits)#PSFCH DTX*#(ACK/NACK bits)

where:

-	#(false NACK bits) denotes the number of detected NACK bits.

-	#(ACK/NACK bits) denotes the number of encoded bits per slot

-	#(PSFCH DTX) denotes the number of DTX occasions

The test parameters in table 11.X.1.1-1 are configured.

### 11.X.2.2 Minimum requirements

The DTX to NACK probability shall not exceed 1% at the SNR given in table 11.X.2.2.3.

Table 11.X.2.2-3: Minimum requirements

| Test num. | Bandwidth | Subcarrier spacing | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | DTX to NACK probability (%) | SNR (dB) |
| 1 | 20 MHz | 30kHz | TDLA30-180 | 1 | TBD |

<< End of Change >>