3GPP TSG-RAN WG4 Meeting # 98-e	R4-2101495

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:	10.1.1

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.921: UE remaining parameters

Document for:	Approval

- Introduction

The latest WF on on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz was approved in [1].

- Between 6.425-7.125 GHz (31.0dB DL ACIR and 25.9dB UL ACIR)

| (dB) | ACLR | ACS |
| --- | --- | --- |
| BS | 38 | 42 |
| UE | 26 | 32 |

- Between 10-10.5 GHz (30.0dB DL ACIR and 23.9dB UL ACIR)

| (dB) | ACLR | ACS |
| --- | --- | --- |
| BS | 37 | 40 |
| UE | 24 | 31 |

This paper provides a TP for TR 38.921 to capture the above agreements and our proposal on remaining requirements.

References

[1] R4-2016905, WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz, Nokia, CATT, Ericsson, ZTE, Huawei, Qualcomm

TP to TR 38.921

< START OF CHANGE>

# 7	UE parameters

## 7.1	Transmitter characteristics

### 7.1.1	Power dynamic range

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. For existing FR1 bands, the minimum output power is -33 dBm for 100 MHz channel bandwidth. The minimum output power can be reused for both frequency ranges, 6.425-7.125GHz and 10.0-10.5GHz, i.e. power dynamic range is 56 dB for 100 MHz channel bandwidth.

### 7.1.2	Spectral mask

For existing FR1 band with frequency lower than 5GHz, the general NR spectrum are defined per channel bandwidth as below.

Table 6.5.2.2-1 of TS 38.101-1: General NR spectrum emission mask

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 |  | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 |  | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

It is proposed to relax the OBUE mask by 4 dB for 6.425-7.125GHz and 6 dB for 10.0-10.5 GHz given that ACLR is agreed with some reduction based on co-existence study.

### 7.1.3	ACLR

According to the simulation results in clause 4.3, it is agreed to specify 26 dB ACLR for 6.425-7.125GHz and 24 dB ACLR for 10.0-10.5GHz.

### 7.1.4	Spurious emissions

The general spurious emissions defined in TS 38.101-1 clause 6.5.3.1 can apply to both frequency ranges, 6.425-7.125GHz and 10.0-10.5GHz.

- 30MHz ≤ f ≤ 1 GHz: -36dBm/100kHz

- 1 GHz ≤ f ≤ 26 GHz: -30dBm/1MHz

### 7.1.5	Maximum output power

TR 38.820 indicates that 23dBm is feasible at 10-10.5GHz and this is an assumption for co-existence analysis. Hence the UE maximum output power for the considered frequency ranges could be 23dBm. Other UE power classes are not precluded for both frequency ranges, 6.425-7.125GHz and 10.0-10.5GHz.

### 7.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS to WP5D.

## 7.2	Receiver characteristics

### 7.2.1	Noise figure

The assumption from TR 38.820 is that the typical NF for NR UE at 10 GHz is 9 dB which is the same assumption for co-existence study as the UE operating below 6 GHz. The UE noise figure for the 2 studied frequency ranges should then have a 9dB value.

### 7.2.2.	Sensitivity

The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

### 7.2.3	Blocking response

The blocking characteristic specified in clause 7.6 for frequency larger than 3300 MHz of [4] is applicable for both frequency ranges, 6.425-7.125GHz and 10.0-10.5GHz.

### 7.2.4	ACS

According to the simulation results in clause 4.3, it is agreed to specify 32 dB ACS for 6.425-7.125GHz and 31 dB ACS for 10.0-10.5GHz.

< END OF CHANGE>