3GPP TSG-RAN WG4 Meeting #98-e	R4-2101674

Online, 25th Jan – 05th Feb, 2020

Title: 	Discussion on IMR configuration for L1-SINR measurement tests

Source: 	Huawei, HiSilicon

Agenda item:	7.9.3.1

Document for:	Discussion

Introduction

In RAN4#97e meeting, the L1-SINR measurement procedure tests were introduced. However, the parameters of IMR configuration are still TBD. In this contribution, we will provide the discussion on how to set the IMR configuration for L1-SINR measurement tests.

Discussion

For CMR+IMR scenario, the L1-SINR measurement requirements are applied under the condition that the IMR periodicity shall be same with the CMR periodicity. Then, the IMR configuration depends on the CMR configuration. The CMR configurations used for L1-SINR measurement procedure tests in CMR+IMR scenario can be summarized as Table 1.

| Case | Scenario | CMR Type | CMR configuration |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Reporting Type | Resource ID | Periodicity |
| EN-DC in FR1 and SA in FR2 | 2A | SSB | Periodic | - SSB#0 - SSB#1 | 20ms |
|  | 2D | CSI-RS | Aperiodic | - CSI-RS#0 - CSI-RS#1 | N/A |
| EN-DC in FR2 and SA in FR1 | 2C | SSB | Periodic | - SSB#0 - SSB#1 | 20ms |
|  | 2B | CSI-RS | Aperiodic | - CSI-RS#0 - CSI-RS#1 | N/A |

Then, the corresponding IMR configurations used for L1-SINR measurement procedure tests in CMR+IMR scenario can be summarized as Table 2.

| Case | Scenario | IMR Type | CMR configuration |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Reporting Type | Resource ID | Periodicity |
| EN-DC in FR1 and SA in FR2 | 2A | CSI-IM | Periodic | - CSI-IM#0 - CSI-IM#1 | 20ms |
|  | 2D | CSI-RS | Aperiodic | - CSI-RS#2 - CSI-RS#3 | N/A |
| EN-DC in FR2 and SA in FR1 | 2C | CSI-RS | Periodic | - CSI-IM#4 - CSI-IM#5 | 20ms |
|  | 2B | CSI-IM | Aperiodic | - CSI-IM#2 - CSI-IM#3 | N/A |

For Scenario 2A, one set of periodic CSI-IM resources with 20ms periodicity need to be configured as IMR. For Scenario 2B, one set of aperiodic CSI-IM resources need to be configured as IMR. For Scenario 2C, one set of periodic CSI-RS resources with 20ms periodicity need to be configured as IMR. For Scenario 2D, both CMR and IMR need to be configured as the aperiodic CSI-RS resources. In current TS38.133, only CSI-RS configurations are defined. Then, CSI-IM configurations need to be introduced. For CSI-RS configuration, only one set of periodic CSI-RS resources with 10ms periodicity and one set of aperiodic CSI-RS resources are defined. So, one set of periodic CSI-RS resources with 20ms periodicity and one additional set of aperiodic CSI-RS resources need to be introduced.

Proposal 1: It is suggested to introduce the following CSI-IM configurations and CSI-RS configurations for L1-SINR measurement procedure tests.

- One set of periodic CSI-IM resources with 20ms periodicity for Scenario 2A

- One set of aperiodic CSI-IM resources for Scenario 2B

- One set of periodic CSI-RS resources with 20ms periodicity for Scenario 2C

- One additional set of aperiodic CSI-RS resources for Scenario 2D

In last RAN4 meeting, the test cases of L1-SINR measurement requirements were agreed, however the test cases of L1-SINR accuracy requirements need to be further studied. For the tests of L1-RSRP accuracy requirements, the reporting type is configured as periodic for both SSB based and CSI-RS based L1-RSRP measurement. Then, we suggest to configure periodic reporting for all the CMR+IMR scenarios. The CMR and IMR configurations used for L1-SINR measurement accuracy tests in CMR+IMR scenario can be summarized as Table 3.

| Case | Scenario | CMR Type | IMR Type | Resource ID | Resource ID | Periodicity |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC in FR1 and SA in FR2 | 2A | SSB | CSI-IM | - SSB#0 - SSB#1 | - CSI-IM#0 - CSI-IM#1 | 20ms |
|  | 2D | CSI-RS | CSI-RS | - CSI-RS#0 - CSI-RS#1 | - CSI-RS#2 - CSI-RS#3 | 10ms |
| EN-DC in FR2 and SA in FR1 | 2C | SSB | CSI-RS | - SSB#0 - SSB#1 | - CSI-IM#4 - CSI-IM#5 | 20ms |
|  | 2B | CSI-RS | CSI-IM | - CSI-RS#0 - CSI-RS#1 | - CSI-IM#2 - CSI-IM#3 | 10ms |

It can be observed that one set of periodic CSI-IM resources with 20ms periodicity and one additional set of periodic CSI-RS resources with 10ms periodicity need to be introduced.

Proposal 2: It is suggested to introduce the following CSI-IM configurations and CSI-RS configurations for L1-SINR measurement accuracy tests.

- One set of periodic CSI-IM resources with 10ms periodicity for Scenario 2B

- One additional set of periodic CSI-RS resources with 10ms periodicity for Scenario 2D

Conclusions

This contribution provides the analysis on how to define the IMR configuration for L1-SINR measurement tests. The followings are provided:

Proposal 1: It is suggested to introduce the following CSI-IM configurations and CSI-RS configurations for L1-SINR measurement procedure tests.

- One set of periodic CSI-IM resources with 20ms periodicity for Scenario 2A

- One set of aperiodic CSI-IM resources for Scenario 2B

- One set of periodic CSI-RS resources with 20ms periodicity for Scenario 2C

- One additional set of aperiodic CSI-RS resources for Scenario 2D

Proposal 2: It is suggested to introduce the following CSI-IM configurations and CSI-RS configurations for L1-SINR measurement accuracy tests.

- One set of periodic CSI-IM resources with 10ms periodicity for Scenario 2B

- One additional set of periodic CSI-RS resources with 10ms periodicity for Scenario 2D

Based on the above proposals, the corresponding draft CR is submitted in [1].

Reference

- R4-2101675, “DraftCR on IMR configuration for L1-SINR measurement tests”, Huawei, HiSilicon