3GPP TSG RAN WG4 Meeting #98-e	R4-2101772

Electronic Meeting, January 25 – February 5, 2020

Agenda item:			7.14.2.1.3

Source:	vivo

Title:	Simulation results for CSI-SINR measurement accuracy

Document for:	Information

- Introduction

In the RAN4#97-e meeting, the updated simulation assumption for CSI-RS based measurement are provided in [1].

In this contribution, we provide simulation results for CSI-SINR measurement accuracy.

# Discussion

Simulation results for CSI-SINR measurement accuracy are provided in Tables 1, 2, 3, 4 for different configurations in both FR1 and FR2. CDF curves are further provided in Annex.

Table 1. simulation results of CSI-SINR accuracy under {4GHz, SINR = -6dB, 48PRB, D=3, 15KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.61 | 0.47 | 2.60 | -2.50 | 0.47 | 2.62 | -3.10 | 0.12 | 2.23 | -4.40 | -0.45 | 1.94 | -3.00 | 0.34 | 2.53 |
|  | 3 | -1.29 | 0.28 | 1.55 | -1.22 | 0.35 | 1.50 | -1.66 | -0.05 | 1.34 | -2.70 | -0.57 | 0.98 | -1.54 | 0.21 | 1.52 |
|  | 5 | -1.09 | 0.25 | 1.32 | -0.81 | 0.31 | 1.27 | -1.38 | -0.07 | 0.89 | -2.07 | -0.62 | 0.55 | -1.18 | 0.11 | 1.23 |
| EPA5 | 1 | -2.68 | 0.10 | 2.36 | -3.18 | 0.06 | 2.29 | -3.75 | -0.34 | 2.15 | -4.69 | -1.04 | 1.88 | -3.20 | -0.05 | 2.29 |
|  | 3 | -1.25 | 0.10 | 1.39 | -1.39 | 0.06 | 1.30 | -1.91 | -0.39 | 1.02 | -2.72 | -1.09 | 0.64 | -1.54 | -0.11 | 1.42 |
|  | 5 | -1.07 | 0.10 | 1.15 | -1.20 | 0.04 | 1.09 | -1.71 | -0.44 | 0.66 | -2.33 | -1.16 | 0.23 | -1.16 | -0.15 | 0.99 |
| ETU30 | 1 | -2.76 | 0.06 | 2.50 | -3.09 | -0.01 | 2.40 | -3.56 | -0.37 | 2.09 | -4.77 | -1.01 | 1.62 | -3.58 | -0.16 | 2.17 |
|  | 3 | -1.39 | 0.08 | 1.44 | -1.60 | 0.00 | 1.38 | -2.16 | -0.44 | 1.09 | -2.85 | -1.03 | 0.60 | -1.92 | -0.18 | 1.23 |
|  | 5 | -0.97 | 0.09 | 1.20 | -1.11 | -0.02 | 1.02 | -1.48 | -0.36 | 0.69 | -2.34 | -1.04 | 0.38 | -1.33 | -0.20 | 0.93 |
| ETU70 | 1 | -2.68 | 0.13 | 2.63 | -3.09 | -0.01 | 2.39 | -3.55 | -0.36 | 2.03 | -4.44 | -1.00 | 1.76 | -3.23 | -0.13 | 2.20 |
|  | 3 | -1.58 | 0.10 | 1.50 | -1.54 | 0.00 | 1.35 | -2.16 | -0.38 | 1.10 | -2.92 | -1.05 | 0.64 | -1.81 | -0.12 | 1.29 |
|  | 5 | -1.12 | 0.07 | 1.10 | -1.24 | -0.03 | 1.06 | -1.75 | -0.43 | 0.65 | -2.44 | -1.14 | 0.29 | -1.57 | -0.17 | 0.94 |

Table 2. simulation results of CSI-SINR accuracy under {4GHz, SINR = -6dB, 48PRB, D=3, 30KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.45 | 0.49 | 2.66 | -2.51 | 0.50 | 2.69 | -3.29 | 0.08 | 2.33 | -4.06 | -0.43 | 2.00 | -3.36 | 0.02 | 2.20 |
|  | 3 | -1.26 | 0.32 | 1.72 | -1.34 | 0.34 | 1.77 | -1.87 | 0.01 | 1.39 | -2.59 | -0.62 | 0.95 | -1.87 | -0.15 | 1.25 |
|  | 5 | -0.81 | 0.27 | 1.36 | -0.93 | 0.22 | 1.49 | -1.42 | -0.12 | 0.98 | -2.29 | -0.64 | 0.60 | -1.50 | -0.23 | 0.93 |
| EPA5 | 1 | -2.42 | 0.19 | 2.49 | -3.05 | 0.10 | 2.32 | -3.45 | -0.32 | 2.07 | -4.19 | -1.06 | 1.71 | -3.59 | -0.56 | 2.05 |
|  | 3 | -1.16 | 0.20 | 1.54 | -1.32 | 0.12 | 1.56 | -1.90 | -0.41 | 1.17 | -2.87 | -1.09 | 0.56 | -2.11 | -0.54 | 0.95 |
|  | 5 | -0.79 | 0.19 | 1.14 | -0.95 | 0.09 | 1.13 | -1.62 | -0.34 | 0.98 | -2.45 | -1.12 | 0.06 | -1.77 | -0.52 | 0.74 |
| ETU30 | 1 | -2.91 | 0.08 | 2.47 | -3.51 | -0.12 | 2.28 | -3.90 | -0.51 | 1.92 | -5.13 | -1.04 | 1.60 | -4.27 | -0.65 | 1.79 |
|  | 3 | -1.45 | 0.02 | 1.47 | -1.75 | -0.19 | 1.29 | -2.19 | -0.52 | 1.02 | -3.18 | -1.06 | 0.54 | -2.35 | -0.63 | 0.86 |
|  | 5 | -1.12 | 0.03 | 1.14 | -1.23 | -0.21 | 0.96 | -1.79 | -0.47 | 0.57 | -2.67 | -1.14 | 0.08 | -2.09 | -0.72 | 0.54 |
| ETU70 | 1 | -2.89 | 0.03 | 2.33 | -3.32 | -0.10 | 2.29 | -3.63 | -0.39 | 2.14 | -4.89 | -1.17 | 1.66 | -4.12 | -0.59 | 1.93 |
|  | 3 | -1.67 | -0.03 | 1.24 | -1.74 | -0.14 | 1.31 | -2.13 | -0.45 | 1.09 | -3.16 | -1.13 | 0.58 | -2.48 | -0.69 | 0.94 |
|  | 5 | -1.09 | -0.02 | 1.04 | -1.33 | -0.11 | 0.93 | -1.80 | -0.46 | 0.75 | -2.60 | -1.16 | 0.05 | -2.19 | -0.70 | 0.51 |

Table 3. simulation results of CSI-SINR accuracy under {30GHz, SINR = -6dB, 48PRB, D=3, 60KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| TDL-A 83Hz | 1 | -2.61 | 0.13 | 2.32 | -2.59 | 0.11 | 2.34 | -3.30 | -0.40 | 2.01 | -4.03 | -1.08 | 1.68 | -5.18 | -1.58 | 1.41 |
|  | 3 | -1.16 | 0.11 | 1.39 | -1.19 | 0.05 | 1.33 | -1.86 | -0.40 | 1.05 | -2.66 | -1.15 | 0.47 | -3.45 | -1.58 | 0.19 |
|  | 5 | -0.82 | 0.13 | 1.07 | -0.90 | 0.09 | 1.00 | -1.48 | -0.40 | 0.75 | -2.40 | -1.21 | -0.01 | -2.83 | -1.58 | -0.36 |
| TDL-C 830Hz | 1 | -2.55 | 0.05 | 2.25 | -2.59 | -0.04 | 1.98 | -3.14 | -0.43 | 1.75 | -4.12 | -1.14 | 1.55 | -4.92 | -1.65 | 1.23 |
|  | 3 | -1.32 | 0.11 | 1.23 | -1.31 | -0.04 | 1.31 | -1.74 | -0.35 | 0.95 | -2.66 | -1.11 | 0.22 | -3.37 | -1.67 | -0.12 |
|  | 5 | -0.88 | 0.08 | 0.98 | -0.96 | -0.01 | 1.10 | -1.37 | -0.35 | 0.60 | -2.43 | -1.14 | -0.06 | -2.83 | -1.73 | -0.44 |

Table 4. simulation results of CSI-SINR for {30GHz, SINR = -6dB, 48PRB, D=3, 120KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| TDL-A 83Hz | 1 | -2.84 | 0.09 | 2.43 | -2.52 | 0.04 | 2.22 | -3.32 | -0.43 | 2.07 | -4.48 | -1.16 | 1.66 | -9.46 | -3.99 | 0.21 |
|  | 3 | -1.37 | 0.08 | 1.29 | -1.36 | 0.00 | 1.29 | -1.86 | -0.46 | 0.93 | -2.88 | -1.16 | 0.44 | -6.81 | -4.13 | -1.35 |
|  | 5 | -1.06 | 0.04 | 1.05 | -1.11 | 0.03 | 0.96 | -1.52 | -0.46 | 0.59 | -2.55 | -1.17 | 0.20 | -6.29 | -4.19 | -2.19 |
| TDL-C 830Hz | 1 | -2.67 | 0.00 | 2.24 | -2.67 | -0.05 | 2.05 | -3.35 | -0.46 | 1.81 | -4.39 | -1.25 | 1.26 | -9.08 | -4.05 | -0.48 |
|  | 3 | -1.40 | 0.00 | 1.28 | -1.39 | -0.13 | 1.10 | -1.91 | -0.46 | 0.73 | -2.81 | -1.30 | 0.23 | -6.87 | -4.17 | -2.00 |
|  | 5 | -1.08 | 0.01 | 0.83 | -1.06 | -0.13 | 0.73 | -1.39 | -0.52 | 0.53 | -2.49 | -1.24 | -0.15 | -6.48 | -4.19 | -2.33 |

Following observations can be made based on simulation results above.

Observation 1. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-SINR measurement with 5 samples is within ±2 dB in both FR1 and FR2.

Observation 2. If timining delay between two cells is smaller than 2CP, the absolute accuary of CSI-SINR measurement with 5 samples is within ±3 dB in both FR1 and FR2.

Observation 3. If timining delay between two cells is smaller than 3us, the absolute accuary of CSI-SINR measurement with 5 samples is within ±2 dB in FR1 only.

Observation 4. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-SINR measurement with 3 samples is worse than ±2 dB in both FR1 and FR2.

# Summary

In this contribution we provided simulation results for CSI-SINR measurement accuracy.

Observation 1. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-SINR measurement with 5 samples is within ±2 dB in both FR1 and FR2.

Observation 2. If timining delay between two cells is smaller than 2CP, the absolute accuary of CSI-SINR measurement with 5 samples is within ±3 dB in both FR1 and FR2.

Observation 3. If timining delay between two cells is smaller than 3us, the absolute accuary of CSI-SINR measurement with 5 samples is within ±2 dB in FR1 only.

Observation 4. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-SINR measurement with 3 samples is worse than ±2 dB in both FR1 and FR2.

- References

- R4-2017230	Updated link-level simulation assumptions for CSI-RS based L3 measurements, CATT

# Annex

CDF curves for CSI-SINR measurement accuracy under 15kHz SCS and 60kHz SCS are provided in the following figures.

For 15kHz SCS,

- Delay = 0

- Delay = CP

- Delay = 2CP

- Delay = 3us

For 60kHz SCS,

- Delay = 0

- Delay = CP

- Delay = 2CP

- Delay = 3us