3GPP TSG-RAN WG4 Meeting # 98-e	R4-2101791

Electronic Meeting, 25 January – 5 February 2021

Agenda Item:	10.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals of UE Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#95-e [7] and RAN4#96-e [8], and the WF on BS and UE parameters was agreed in RAN4#97-e [9] where the UE ACLR and ACS (in the Small cell indoor/Indoor urban scenario) as well as the UE Spectral mask (in all scenarios) are FFS.

This contribution provides the proposals of the open UE parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided in [10-13].

# 2.	Discussion

2.1	UE ACLR and ACS

The downlink coexistence simulation results provided in [12] have shown that the downlink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with a downlink ACIR offset of -5dB at both 7GHz and 10GHz (i.e. 5dB less stringent ACIR). Therefore, it is proposed to apply the UE ACS agreed in the WF [9] to the Small cell indoor/Indoor urban scenario, i.e. 32dB and 31dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

The uplink coexistence simulation results provided in [13] have shown that the uplink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with an uplink ACLR offset of -5dB at both 7GHz and 10GHz (i.e. 5dB less stringent ACLR). Therefore, it is proposed to apply the UE ACLR agreed in the WF [9] to the Small cell indoor/Indoor urban scenario, i.e. 26dB and 24dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

2.2	UE Spectral mask

For the UE Spectrum emission mask for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, even though the UE ACLR agreed in the WF [9] is more relaxed compared to the NR ACLR in clause 6.5.2.4 of TS 38-101.1 [3], it is proposed to apply the General NR spectrum emission mask specified in clause 6.5.2.2 of TS 38.101-1 to conclude the study and reply to ITU-R as scheduled if no other alternative can be agreed in RAN4#98-e.

# 3.	Conclusion

This contribution provides the proposals of the open UE parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided in [10] and [11].

Proposals for approval:

1) To apply the UE ACS agreed in the WF to the Small cell indoor/Indoor urban scenario, i.e. 32dB and 31dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

2) To apply the UE ACLR agreed in the WF to the Small cell indoor/Indoor urban scenario, i.e. 26dB and 24dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

3) To apply the General NR spectrum emission mask specified in clause 6.5.2.2 of TS 38.101-1 for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz if no other alternative can be agreed in RAN4#98-e.

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.6.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[8]	R4-2011827, “WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Nokia.

[9]	R4-2016905, “WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz”, Nokia, CATT, Ericsson, ZTE, Huawei, Qualcomm.

[10]	R4-2014476, “Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[11]	R4-2014477, “Uplink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[12]	R4-2101794, “Downlink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[13]	R4-2101795, “Uplink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.