3GPP TSG-RAN WG4 #98-e	R4-2102103

Electronic meeting, 25th January – 5th February 2021

Source: 	Ericsson

Title:  	HST FR2 general aspects

Agenda Item:	11.7.1

Document for:	Approval

# Introduction

One of the first objectives of the FR2 HST WI is to establish what speed can be supported for FR2 in relevant layouts. Following this, requirements will be derived. This contribution presents an overview of which factors should be considered in order to establish the supported speed considering RF, RRM and demodulation performance. Companion papers provide more detail on layout considerations, RF and RRM. For demodulation, it is useful to check whether performance barriers could be expected when deriving requirements. In this paper, some parameters are proposed for making such a check.

# Discussion

To determine the maximum supportable speed, it is necessary to consider RF, RRM and demodulation performance in the context of a reference layout. Some progress was made on narrowing down layout options in the previous meeting. In order to further understand the layout, it is necessary to consider:

- BS and UE array sizes and orientation in relation to one another

- Uni- and Bi- directional deployment aspects. Note that these will differ compared to FR1 due to the beamforming and panel design. For example, a UE will not be able to receive from opposite directions in a bi-directional deployment using the same beam. We discuss these aspects in more detail in a companion contribution [1].

- Distance between BS. This needs to be related to the link budget, which is further discussed in [1].

Regarding RF, there are not any aspects that directly impact the supportable speed. Some aspects may indirectly impact the speed as they will affect the layout. This includes transmit power, array size, number of panels, number of active panels and spherical coverage expectations.

For RRM, the assumptions on the layout, antenna array and beam pattern feed into a consideration of the beam dwell times, beam management and handover. Realistic UE measurement and interruption times will need to be considered. There are also dependencies on considerations such as the type of deployment (uni-bi- directional), the number of BS per BBU etc. [1-2] cover aspects of deployment and RRM.

It is also important to consider whether there is any limiting factor in the maximum supportable Doppler from a demodulation perspective. Two aspects should be considered; the ability to resolve the Doppler shift and the performance itself.

In regard to the ability to resolve the Doppler shift, table 1 shows the speeds corresponding to the maximum resolvable Doppler at 28GHz for different DM-RS configurations for the BS and UE. In calculating the speeds in the table, the worst case is assumed in which the Doppler reverses at the UE passes a BS. Considering the deployment scenarios and beamforming in FR2, however it may not be necessary to detect a Doppler reversal, in which case the supportable Doppler and speed in the table can be increased by a factor of 2.

In general, resolving Doppler should not be an issue for supporting speeds up to 250 km/h in FR2. The number of needed symbols will depend on the deployment scenario (i.e. whether Doppler reversal is expected and SCS). To reduce the overhead needed for DM-RS, it may be preferable to find other means to avoid the need to detect Doppler reversal, in particular for supporting the 60kHz SCS. For example, it should be borne in mind that a particular Doppler direction is likely to be associated to a particular RX panel in FR2, so the Doppler direction could be implied from the panel.

Table 1: Supportable speeds considering Doppler resolution (@28GHz, assuming Doppler reversal and 0.1ppm frequency error)

|  | 1 symbol |  | 3 symbols |  | 4 symbols |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 60k SCS | 120k SCS | 60k SCS | 120k SCS | 60k SCS | 120k SCS |
| Maximum Doppler kHz | 25200 | 53200 | 6533 | 15867 | 4200 | 11200 |
| Maximum speed (UE) km/h | 972 | 2052 | 252 | 612 | 162 | 432 |
| Maximum speed (BS) km/h | 486 | 1026 | 126 | 306 | 81 | 216 |

To investigate the performance impact, it is useful to align on a baseline set of parameters. Table 2 proposes parameters for the BS and table 3 for the UE. In our opinion, these parameters could be used for checking the maximum supportable speed, but for the demodulation requirements the parameters should be re-discussed and re-approved when requirements are decided.

Proposal 1: Use the parameters in tables 2 and 3 for checking the maximum supportable speed from a demodulation perspective. Parameters to be re-discussed for setting demodulation requirements.

Table 2: PUSCH parameters for evaluating maximum supportable speed

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna layout |  | 1T2R |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1, pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 8 or 9 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
|  | Bandwidth | 100 MHz |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| MCS |  | MCS16 |
| Propagation channel |  | Single Tap |

Table 3: PUSCH parameters for evaluating maximum supportable speed

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| CBW and SCS |  |  | 120kHz, 100MHz (66PRB) |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | DDSU (FR2.120-2) S=11D+3G+0U |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | CSI-RS offset | Slots | 4 for CSI-RS resource 1 and 2 5 for CSI-RS resource 3 and 4 |
|  | CSI-RS periodicity | Slots | 80 (10ms) |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | Option 1: 1 Option 2: 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Propagation channel |  |  | HST Single tap (TS38.101-4 B.3) |
| Antenna configuration |  |  | 1x2 |
| Number of MIMO layers |  |  | 1 |
| MCS |  |  | Option 1: 16QAM 0.5 (MCS 13 with Table 1) Other options are not excluded |

Table 4: HST Single tap parameter for evaluation

| Parameter | Value |
| --- | --- |
| Ds | As in WF |
| Dmin | As in WF |
| v | 350km/h |
| fd | 9000Hz |

Figure 1 depicts some example Doppler trajectories for different deployment scenarios, assuming that Doppler reversal is experienced (which as described above is a worst-case assumption and may not be needed).

Figure 2 depicts some initial simulation results for PUSCH based on these trajectories. These initial results suggest that HST speeds can be supported from a demodulation perspective. Further work and checking should be carried out to verify that HST speeds will not later on present and problem when creating the DL and UL demodulation requirements.

# Conclusion

This contribution has reviewed the areas to be considered in order to evaluate the maximum supportable speed. For checking the demodulation performance, some example parameters are proposed.

# References

Ericsson, “HST Deployment Scenarios”, R4-2102104, RAN4#98-e, January 2021

Ericsson, “Overview of RRM requirements for NR high speed train scenario in FR2”, RAN4#98-e, January 2021

Ericsson, “Consideration on UE requirements on spherical coverage for FR2 HST”, RAN4#98-e, January 2021