3GPP TSG- RAN WG4 Meeting #98	R4-2102259

# Electronic Meeting,  January 25th – February 5th 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for test case 7 for Dormant SCell BWP switch delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case for TC#7: SA, spCell in FR2, SCell in FR2, DCI 0_1/1_0 within/after 3 OFDM symbols |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of agreed TC#7 testing UE requirements for dormancy SCell in NR SA with PCell and SCell in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test case for dormant SCell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New test case |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### A.x.x.x.1	Test Purpose and Environment

The purpose of this test is to verify that the Dormant SCell BWP switch delay requirements are within the requirements stated in section 8.6 for UE configured with a single downlink SCell, when the dormancy indication is received in any of the first 3 OFDM symbols or is received after the first 3 OFDM symbols.

The Supported test configurations are given in Table A.x.x.x.x-1. The test parameters are given in Tables A.x.x.x.x-2 and cell-specific parameters in A.x.x.x.x-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. There are two carriers both in FR2, with one cell on the PCC and 2 cells on SCC. Cell 1, Cell 2 and Cell 3 operate in either FDD or TDD duplex mode according to test configuration. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) with configured and activated SCell (SCell1) on radio channel 2 (SCC1). The UE is not aware of Cell 3 on radio channel 2 (SCC1). The UE is reporting CSI and shall not report CQI index 0 (out-of-range) in the available uplink resources to report CQI for the SCell. The UE shall be continuously scheduled in the PCell throughout the whole test.

The UE receives a DCI-based BWP switch command by which the SCell1 (Cell 2) is requested to switch the active BWP to the dormant BWP.

The point in time at which the DCI message is received at the UE antenna connector, in a subframe # denoted n, defines the start of time period T1. The UE shall accomplish the BWP switch to the dormant BWP latest in subframe (n + TBWPswitchDelay + X). The UE shall continue to shall report valid CQI if the UE has available uplink resources to report CQI for the dormant SCell. The UE shall continue to shall report L1-RSRP if the UE has available uplink resources to report L1-RSRP for the Dormant SCell. Any PCell interruption due to BWP switch on the SCell shall occur in the subframes n to (n+ TBWPswitchDelay + X).

Time T2 start at T1 + (TBWPswitchDelay + X). During T2 the UE shall continue to measure and report CQI and L1-RSRP in the available uplink resources to report CQI and L1-RSRP for the SCell.

Time T3 starts at T2 + 500ms. During T3 the UE shall continue to measure and report CQI and L1-RSRP in the available uplink resources to report CQI and L1-RSRP for the SCell.

Starting at T4 = T3 + 500ms, Cell 3 becomes detectable. During T3 the UE shall continue to measure and report CQI and L1-RSRP in the available uplink resources to report CQI and L1-RSRP for the SCell. The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T4. The UE is not required to read the neighbour cell SSB index in this test.

At time T5 starting at T4 + 1500ms a a DCI-based BWP switch command by which the SCell1 (Cell 2) is requested to switch the active BWP to the non-dormant BWP.

The point in time at which the DCI message is received at the UE antenna connector, in a subframe # denoted n, defines the start of time period T6. The UE shall accomplish the BWP switch to the non-dormant BWP latest in subframe (n + TBWPswitchDelay + X). The UE shall continue to shall report valid CQI if the UE has available uplink resources to report CQI for the non-dormant SCell. The UE shall continue to shall report L1-RSRP if the UE has available uplink resources to report L1-RSRP for the non-dormant SCell. Any PCell interruption due to BWP switch on the SCell shall occur in the subframes n to (n+ TBWPswitchDelay + X).

During T2, T3 and T4 the total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements and RRM measurements, clause 8.2.2.2.12.3, on dormant SCells, shall not exceed [0.5]%.

During T2, T3 and T4 the total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from L1-RSRP measurements and RRM measurements, clause 8.2.2.2.12.x, on dormant SCells, shall not exceed [0.5]%.

During T2, T3 and T4 the total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from RRM measurements and RRM measurements, clause 8.2.2.2.12.3, on dormant SCells, shall not exceed [0.5]%

During T1, T2, T3, T4, T5 and T6, the UE shall be continuously scheduled in the SCell1.

Table A.x.x.x.x.x-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.x.x.x.x.x-2: General test parameters for dormancy SCell in NR SA with PCell and SCell in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| PCell |  | 1 | Cell 1 |  |
| SCell |  | 1 | Cell 2 |  |
| Neighbour cell |  | 1 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1 | Cell 1 |  |
| RF Channel Number |  | 2 | Cell 2 and Cell 3 |  |
| Measurement gap type |  | 1 |  | No measurement gaps configured |
| SSB configuration |  | 1 | SSB.1 FR2 | for all cells |
| SMTC configuration |  | 1 | SMTC.1 | all cells |
| CSI-RS parameters |  | 1 | CSI-RS.3.2 FDD |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| T1 | s | 1 | [5] |  |
| T2 | s | 1 | [5] |  |

Table A.x.x.x.x.x-3: NR Cell specific test parameters for dormancy SCell in NR SA with PCell and SCell in FR2

| Parameter | Unit | Test configuration | Cell 1, Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.2 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |
| RLM-RS |  | 1 | CSI-RS |  | N/A |  |
| Note 2 | dBm/SCS | 1 | [-98] |  |  |  |
| Note 2 | dBm/15 kHz | 1 | [-98] |  |  |  |
|  | dB | 1 | [4] | [-1.46] | [-Infinity] | [-1.46] |
|  | dB | 1 | [4] | [4] | [-Infinity] | [4] |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | [-94] | [-94] | [-Infinity] | [-94] |
| Io | dBm/9.36 MHz | 1 | [-64.60] | [-62.25] | [--64.60] | [-62.25] |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### A.x.x.x.2	Test Requirements

During T1 the UE shall switch to the dormant BWP.

During T2, T3, T4 and T5 the UE shall not send ACK/NACK for the PDSCH data scheduled on the SCell.

During T2, T3, T4 and T5 the UE shall continue to send CSI reports for SCell1 with non-zero CQI index.

During T2, T3, T4 and T5 the UE shall continue to send L1-RSRP reports for SCell.

During T4 the UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T4.

During T2, T3, T4 and T5, the missing ACK/NACK sent in PCell shall be less than [0.5 + x + x]% of the total number of the expected ACK/NACK.

During T6, the UE shall send ACK/NACK for the PDSCH data scheduled after subframe (n+ TBWPswitchDelay + X) for the SCell1.

All of the above test requirements shall be fulfilled in order for the observed SCell1 BWP switch delays, Pcell interruption rate, correct CSI and L1-RSRP reporting and event triggeres reporting. The rate of correct observed SCell1 hibernation delay, activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

<End of Change>