3GPP TSG-RAN WG4 #98bis-e	R4-2105173

e-Meeting, Apr 12 – 20, 20210

Agenda item:	8.12.6

Source: 	Qualcomm Incorporated

Title: 	Simulation results for NR coexistence study: dense urban micro deployment at 60GHz

Document for:	Discussion

# Introduction

As the WI starts for extending current NR operation to 52.6-71GHz, in this contribution we present preliminary simulation results for NR adjacent channel coexistence study at 60GHz. This paper addresses downlink dense urban micro deployment. The simulation assumptions adopted in this study follows the ones considered in [1].

# Discussion

The simulation results presented in this paper refer to the dense urban micro deployment with a carrier frequency of 60GHz. The dense urban micro scenario consists of a single layer hexagonal grid, 3 sectors per site, 7 sites locations, BS height 10m, UE height 1.5m, and inter-side distance (ISD) equals 70m. Moreover, we consider in our contribution the case of coordinated and uncoordinated for the two considered operators as shown in Figure 1.

Figure 1 UMi deployment for the coordinated (left) and uncoordinated (right) scenarios

The list of simulation assumptions considered in our simulations are reported in Table 1.

Table 1. List of simulation assumptions for dense urban micro scenario [1].

| Deployment |  | Dense urban scenario outdoor A as specified in [1] |
| --- | --- | --- |
| System Parameters | Carrier Frequency | 60GHz |
|  | Channel BW | 400 and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | Umi model as specified in [2] |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table 7.3-1 of [2] |
|  | Max Tx Power | 20 dBm (≈EIRP = 40 dBm) allocated across all RBs in the CC bandwidth |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [2] |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

## Simulation results

In this section we present the preliminary DL results for the two UMi deployments under the 400 and 2000 MHz channel bandwidth configurations, where the measurements are conducted at the victim UEs and the aggressor nodes are BSs operating in adjacent channel. First the signal to noise (SNR) and signal to interference and noise ratio (SINR) cumulative distribution function (CDF) curves are shown in Figure 2, where “c” and “uc” represent the coordinated and the uncoordinated cases, respectively. We observe that as the component carrier bandwidth narrows, the SINR and SNR are improved due to the impact of higher thermal noise in the system (around 7 dB gap between the SNR curves for the 400 MHz and 2 GHz channel bandwidth). Additionally, we observe that the system is noise-limited, and that the uncoordinated deployment offers improved SNR in the region of 75-%tile and higher for the 400 MHz configuration.

Figure 2. SINR and SNR CDF UMi DL for 60 GHz.

In Figure 3, the mean throughput (TP) degradation because of the adjacent channel interference (ACI) is considered, whereas the 5%-tile throughput is observed to be 0, which is expected based on the low SINR values reported in Figure 2. As it can be observed, the un-coordinated deployment offers lower degradation since the interference is less aggressive compared to the coordinated case. We also observe that the 2GHz channel bandwidth offers slightly less degradation when compared to the 400MHz channel bandwidth. This is attributed to the relative nature of the throughput loss which is the ratio of the victim’s network throughput with ACI to the victim’s network throughput without ACI. This ratio is higher for the 400MHz case which leads in turn to slightly higher degradation as observed. The ACIR requirements for the different deployments that ensure at most a 5% mean throughput loss are listed in Table 2.

Figure 3.  Mean DL throughput degradation as a function of ACIR for 60GHz.

Table 2 DL ACIR requirement for 5% TP loss

| Case | ACIR requirement for 5% DL throughput loss |
| --- | --- |
| Coordinated (uncoordinated) with channel bandwidth = 400MHz | 17 dB (8 dB) |
| Coordinated (uncoordinated) with channel bandwidth = 2GHz | 15 dB (5 dB) |

# Conclusions

In this contribution we provided preliminary downlink simulation results for NR coexistence study in dense urban micro deployment for 60GHz. We investigated two different network deployments, namely, coordinated, and un-coordinated deployments. Based on the results obtained so far, we have the following observation:

Observation 1: For DL case, an ACIR higher than 15 dB and 17 dB would provide enough margin against adjacent channel interference for the 2GHz and 400MHz channel bandwidth, respectively, for the coordinated deployment case. On the other hand, for the un-coordinated case, ACIR higher than 5 dB and 8 dB for the two bandwidth configurations would suffice.

Proposal 1: We encourage other interested companies to provide simulation results for comparison.

In addition, RAN 4 should start discussing simulation parameters for 52.6-71GHz.

Proposal 1: We would like to invite other companies to provide their views on the different parameters for downlink and uplink cases as well as the deployment scenarios to be investigated.

# Reference

- 3GPP TR 38.808: "Study on supporting NR from 52.6 GHz to 71 GHz".

- 3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

- 3GPP TR 38.802: "Study on New Radio Access Technology Physical Layer Aspects".

- 3GPP TR 38.803: "Study on new radio access technology: Radio Frequency (RF) and co-existence aspects ".