3GPP TSG-RAN WG4 Meeting # 98-bis-e                	R4-2106587

E-meeting,12thApril  – 20th April, 2021

Agenda item:	8.12.2

Source: 	ZTE Corporation

Title: 	Discussion on band plan for 52.6-71GHz

Document for:	Approval

Introduction

In Rel-16, lots of regulatory information for 52.6-100GHz has been summarized and documented in TR 38.807. In this contribution, we want to share some further understanding how to define the relevant band for 52.6-71GHz.

Discussion

Firstly for frequency range 52.6-54.25GHz due to EESS (passive) protection e.g.weather prediction service, therefore in ITU-R Footnote 5.340, all emission is proposed to be prohibited. In other words, it could not be assigned or used for IMT transmission.

For frequency range 54.25-57.24GHz next to EESS (passive), it is not assigned for IMT currently in ITU-R and also not on the agenda of WRC-23, therefore in other words, these frequency range cannot be used for IMT in the near future. If possible, these frequency range could be used for IMT in future, the situation for coexistence environment might be quite similar as 23.6-24GHz and 24.25-27.5GHz.

In addition, based on the frequency bands summary in [3], it could be found that both mobile and fixed service on 52.6-54.25GHz and 54.25-55.78GHz is not allowed.

Proposal 1: 52.6-54.25GHz and 54.25-55.78GHz should be excluded from band definition for 52.6-71GHz.

Table 4.1-2: Frequency bands in the range 52.6 to 116 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 52.6-54.25 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 54.25-55.78 | No | No | EESS (passive) and SRS (passive) |
| 55.78-59 | Yes | Yes | EESS (passive) and SRS (passive) This band available for high-density applications in the fixed service, footnote 5.547 |
| 59-59.3 | Yes | Yes | EESS (passive) and SRS (passive) Radiolocation |
| 59.3-64 | Yes | Yes | Radiolocation |
| 64-65 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 65-66 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 66-71 | Yes | No | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in clause 2.2. |

In addition, for frequency range 66-71GHz, it has bee identified for IMT transmission based on permanent European Commission mandate to CEPT, therefore we think at least that licensed band for 66-71GHz should be defined. Whether licensed band could be extended down to 57GHz, this could be further discussed.

Proposal 2: for 66-71GHz, the licensed band should be defined at least.

Conclusions

In this contribution, we want to share some further considerations on band definition plan for 52.6-71GHz and proposals are made as following:

Proposal 1: 52.6-54.25GHz and 54.25-55.78GHz should be excluded in band definition for 60GHz;

Proposal 2: for 66-71GHz, the licensed band should be defined at least.

References

- RP-21086, revised WID for 52.6-71GHz, CMCC, approved.

- RP-210846,Regulatory update for the 52.6 to 71GHz frequency range,Apple Inc.

- TR 38.807