3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106807

# Electronic Meeting, Apr.12-20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0175 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-4: Introduction of PSCCH/PSSCH capability test for NR V2X. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-3-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN 4 has agreed to introduce PSCCH/PSSCH decoding capability test for NR V2X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requirments and test setup for NR V2X PSCCH/PSSCH decoding capability test Added the related resource pool configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for NR V2X will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.8, A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

11.1.8	PSSCH/PSCCH decoding capability test

11.1.8.1	2RX requirements

11.1.8.1.1	Test setup

The purpose of this test is to verify the maximum number of Sidelink processes per TTI supported by the V2X UE. The test procedure is specified as follows:

10 UEs transmit PSCCHs and corresponding PSSCHs to the tested UE per slot, each UE transmits one PSCCH and corresponding PSSCH on one subchannel. If the number of PSFCH(s) resources that the tested UE can transmits in a slot as specified in Clause 16.3 of TS 38.213 [11] is smaller than 10, x equals to the number of PSFCH(s) resources that the tested UE can transmit in a slot, otherwise x=10. The indication of priority level as specified in Clause 5.4.3.3 of TS 23.287 [12] and Clause 5.22.1.3.1 of TS 38.321 [8] is included in PSCCH. x UEs transmit PSCCHs and corresponding PSSCHs with high priority level on x subchannels that are randomly seleted from 10 subchannels per slot and 10-x UEs transmit PSCCHs and corresponding PSSCHs with low priority level on the remaining subchannels. The test time for UE supporting transmit less than 10 PSFCH(s) resources in a slot should be 10/x times that of UE supporting transmit more than 10 PSFCH(s) resources in a slot.

The probability of PSCCH miss detection is calculated as follows:

ProbPSCCH miss detection = #(false ACK/NACK)#(Tx high priority PSCCH/PSSCH)

Where:

- # (Tx high priority PSCCH/PSSCH) denotes the total number of transmitted PSCCHs/PSSCHs with high priority level.

- # (false ACK/NACK) denotes the total number of PSFCH resuorces with high priority level where no ACK/NACK is detected.

Table 11.1.8.1.1-1:  Test Parameters_

| Parameter |  |  | Unit | value |
| --- | --- | --- | --- | --- |
| Resourece pool |  |  |  | Configuration#3-V2X |
| Member ID |  |  |  | 0 |
| Sidelink UE i, 0 ≤ i ≤ 9 (Note 4) | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) |  | s | 0 |
|  | Frequency offset (Note 2) |  | Hz | 0 |
|  | Synchronization source |  |  | GNSS |
|  | Propagation Channel |  |  | Static propagation condition without external noise |
|  | Antenna configuration |  |  | 1x2 Low |
|  | PSSCH RMC |  |  | R.PSSCH.2-1.1 |
|  | PSCCH RMC (Note 3) |  |  | R.PSCCH.2-1.1 |
|  | Source ID |  |  | 0 |
|  | PSFCH periodicity |  | Slots | 1 |
|  | MinTimeGapPSFCH |  | Slots | 2 |
|  | PSFCH Resource(Note5) | RB index |  | 10i |
|  |  | CS pair index |  | 0 |
| Note 1: Member ID is an identifier uniquely identifying a member Note 2: Time offset of received signal by Sidelink UE with respect to GNSS reference timing. Note 3: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 4: OCC index for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission as per in Clause 8.4.1.3.2 of TS 38.211[9]. Note 5: Each UE occupies one sub-channel so that all sub-channels are filled. Note 6: The mapping procedure of PSSCH resource and PSFCH resource is specified in Clause 16.3 of TS 38.213 [11]. |  |  |  |  |

11.1.8.1.2	Minimum requirements

The minimum requirements are specified in Table 11.1.8.1.1-2 with the test parameters specified in Table 11.1.8.1.1-1

Table 11.1.8.1.1-2: Minimum performance

| Test Number | Bandwidth (MHz)/ Subcarrier spacing(kHz) | PSCCH Reference channel | Propagation Channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) |
| 1 | 40/30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |

<End of change>

<Start of change>

Table A.7-3 V2X sidelink communication resource pool for PSCCH/PSSCH decoding capability tests (Configuration#3-V2X)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| SL-ResourcePool-r16 | sl-PSCCH-Config-r16 | sl-TimeResourcePSCCH-r16 | n2 |
|  |  | sl-FreqResourcePSCCH-r16 | n10 |
|  | sl-SyncAllowed-r16 |  | gnss-Sync-r16 |
|  | sl-SubchannelSize-r16 |  | n10 |
|  | sl-TimeResource-r16 |  | 160 |
|  | sl-StartRB-Subchannel-r16 |  | 0 |
|  | sl-NumSubchannel-r16 |  | 10 |
|  | sl-Additional-MCS-Table-r16 |  | Not presented |
|  | sl-RB-Number-r16 |  | 51 |
|  | sl-X-Overhead-r16 |  | n0 |
|  | sl-PSFCH-Config-r16 | sl-PSFCH-Period-r16 | sl1 |
|  |  | sl-PSFCH-RB-Set-r16 | 100 |
|  |  | sl-NumMuxCS-Pair-r16 | n1 |
|  |  | sl-MinTimeGapPSFCH-r16 | sl2 |
|  |  | sl-PSFCH-HopID-r16 | 0 |
|  |  | sl-PSFCH-CandidateResourceType-r16 | startSubCH |

<End of change>