3GPP TSG-RAN WG4 Meeting #98bis-e 	R4-2107162

# Electronic Meeting, April 12  April 20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PRS-RSRP measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Incomplete requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.9.3	PRS-RSRP measurements

#### 9.9.3.1	Introduction

The requirements in clause 9.9.3.5 shall apply provided the UE has received a message from LMF via LPP [34] requesting the UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4].

#### 9.9.3.2	Requirements applicability

The requirements in clause 9.9.3 apply for periodic and triggered PRS-RSRP measurements, provided:

-	PRS-RSRP related side conditions given in clause 10.1.24 are met for a corresponding Band.

#### 9.9.3.3	Measurement Capability

UE PRS-RSRP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

#### 9.9.3.4	Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.24.3.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requriements specified in the clauses 10.1.24.

#### 9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured PRS frequency layers, within  ms.

If measurement gaps and processing time T have overlap between different frequency positioning frequency layers,

=

Editor’s note: FFS the PRS-RSRP measurement period when measurement gaps and processing time T do not have overlap between different positioning frequency layers.

where i is the index of PRS frequency layer,

is the measurement period for PRS-RSRP measurements in frequency layer i as defined further in this clause,

L is total number of positioning frequency layers, and  is the periodicity of PRS-RSRP measurement in frequency layer i as defined further in this clause.

=+

where

is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,	 is 	 is the carrier specific scaling factor for NR PRS-based measurements specified in clause 9.1.5.2 as CSSFwithin_gap,i,

is the scaling factor for Rx beam sweeping, and =1 if PRS layer i is in FR1 and =8 if PRS layer i is in FR2,

is the time duration specified in clause 5.1.6.5 of TS 38.214 [26, clause 5.1.6.5],

is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

is the number of PRS-RSRP measurement samples and = 4,

is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,  =  + 	 is periodicity of PRS-RSRP measurement in frequency layer i:

= *

where

corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

= LCM, the least common multiple between  and

is the maximum PRS resource periodicity among all PRS resources in frequency layer i,

is the measurement gap repetition period in frequency layer i.

The requirements in this section apply for PRS resources with PRS symbols available in at least some measurement gaps.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities, TPRS,i is the maximum PRS periodicity among the DL PRS resource sets on frequency i, for which there is at least one PRS resource with PRS symbols available in at least some measurement gaps.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TPRS-RSRP,Total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP measurement is configured together with RSTD, and TPRS-RSRP,Total<TRSTD,Total, the UE shall continue the PRS-RSRP measurement over the corresponding RSTD measurement period specified in claus 9.9.2.5.

When PRS-RSRP measurement is configured together with UE Rx-Tx, and TPRS-RSRP,Total<TUERxTx,Total, the UE shall continue the PRS-RSRP measurement over the corresponding UE Rx-Tx measurement period specified in claus 9.9.4.5.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing PRS-RSRP measurements session. The UE shall also meet the PRS-RSRP measurement requirements in this clause and measurement accuracy requirements in clause 10.1.24. However, in this case the PRS-RSRP measurement period TPRS-RSRP,total.HO shall be as follows:

TPRS-RSRP, total,HO=TPRS-RSRP, total+K*Teffect+THO   ms

where

K is the number of times handover occurs during TPRS-RSRP,total.HO;

Teffect is the largest Teffect,i among all PRS layers;

THO is the time during which the PRS-RSRP measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement, the UE behaviour at a serving cell (SpCell or SCell) change for the PRS-RSRP measurement is the same as the UE behaviour for the UE Rx-Tx time difference measurement specified in clause 9.9.4.5, and the PRS-RSRP measurement shall meet the accuracy requirements in clause 10.1.24.

When the PRS-RSRP measurement is configured together with RSTD measurement, the UE behaviour at a serving cell (SpCell or SCell) change for the PRS-RSRP measurement is the same as the UE behaviour for the RSTD measurement specified in clause 9.9.2.5, and the PRS-RSRP measurement shall meet the accuracy requirements in clause 10.1.24.