3GPP TSG-RAN WG4 Meeting#98bis             	R4-2107200

E-meeting, Apr. 12th – 20th, 2021

Title:	TP to TR 38.717-02-01: Addition of CA_48-53

Source:	Nokia, Globalstar

Agenda item:	10.1.3

Document for:	Approval

# 1	Introduction

This is a TP into TR 38.717-02-01 to add CA_48-53 from approved WID RP-202199.

************************************* Start of TP*****************************************

## 6.x	n48-n53

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n25 and n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| n53 | 2483.5 MHz | – | 2495 MHz | 2483.5 MHz | – | 2495 MHz | TDD |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n48 and n53

| NR CA configuration | UL | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48A-n53A | - | n48 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n53 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n53A |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n53 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3		UE co-existence studies

Table 6.x.1.3-1 lists up to 7th harmonics for n48A-n53A which shows that there are no harmonics issues.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 | 21300 | 22200 | 24850 | 25900 |
| n53 | 2483.5 | 2495 | 4967 | 4990 | 7450.5 | 7485 | 9934 | 9980 | 12417.5 | 12475 | 14901 | 14970 | 17384.5 | 17465 |

Table 6.x.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.x.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 |
| n53 | 2483.5 | 2495 | 2483.5 | 4967 | 4990 | 7450.5 | 7485 | 9934 | 9980 | 4967 |

#### 6.x.1.4		∆TIB and ∆RIB values

For CA_n48-n53, the TIB,c and RIB,c values are derived from CA_41-48 and are given in the tables below.

Table 6.x.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n48-n53 | n48 | 0.5 |
|  | n53 | 0.3 |

Table 6.x.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n48-n53 | n48 | 0.5 |
|  | n53 | 0 |

#### 6.x.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.x.1.3 there are no harmonics issues.

#### 6.x.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n48-n53 since it belongs to H-H configuration-

Table 6.x.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

************************************* End of TP*****************************************