3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108753

# Electronic Meeting, May. 19 – 27 2021

Agenda item:	9.1.3.3

Source:	CAICT, CMCC, Spirent

Title:	Spatial Correlation Reference Targets for FR1 CDL-C UMa

Document for:	Approval

- Introduction

In the last RAN#98bis-e meeting, it is agreed that CE vendors and other companies are encouraged to provide data for generating the reference for channel model validation, and further alignment is also required [1].

In this meeting, several contributions have provided the reference targets for the spatial channel model validation [2-4]. To make progress, further adjustments of the reference curves were made based on offline alignment efforts among companies, and the spatial correlation at 2450MHz was well aligned.

This contribution is providing the Spatial Correlation reference targets for FR1 CDL-C UMa at 2.45GHz, both reference curves and specific values are presented.

- Details

The Spatial Correlation reference target value for FR1 CDL-C UMa 2.45GHz beam1 and beam2 are as follows:

Note: The proposed harmonized value of Spatial Correlation is achieved by averaging the latest results of each azimuthal provided by Keysight, Spirent and CMCC. The initial value used to calculate the average adopts the SC values provided by different companies in their latest contributions and retains four decimal places (i.e., x.xxx). The target results are shown in four decimal places as well.

Table 1. Spatial correlation reference values for CDL-C UMa model at 2450MHz

| 2450 MHz |  |  |
| --- | --- | --- |
| Azim [] | ¦¦, B1 | ¦¦, B2 |
| 270 | 1 | 1 |
| 263 | 0.9980 | 0.9878 |
| 256 | 0.9930 | 0.9554 |
| 249 | 0.9863 | 0.9095 |
| 242 | 0.9781 | 0.8577 |
| 234.9 | 0.9666 | 0.8069 |
| 227.9 | 0.9497 | 0.7658 |
| 220.9 | 0.9248 | 0.7394 |
| 213.9 | 0.8889 | 0.7303 |
| 206.9 | 0.8437 | 0.7353 |
| 199.9 | 0.7855 | 0.7470 |
| 192.9 | 0.7156 | 0.7588 |
| 185.9 | 0.6353 | 0.7697 |
| 144.9 | 0.1185 | 0.8520 |
| 109.9 | 0.2094 | 0.6583 |
| 74.8 | 0.2764 | 0.2765 |
| 39.8 | 0.1579 | 0.1214 |
| 4.7 | 0.2290 | 0.0983 |
| 329.7 | 0.6775 | 0.3455 |
| 294.6 | 0.9594 | 0.8229 |

Figure 1. Spatial correlation reference curves for CDL-C Uma model at 2450MHz

For other FR1 test frequencies, offline discussion and alignment is on-going.

3	Conclusions

In this contribution spatial correlation validation targets for FR1 CDL-C UMa are proposed.

Proposal 1: Adopt the spatial correlation validation targets for FR1 CDL-C UMa as presented.

4	References

- R4-2106092, WF on NR MIMO OTA, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting # 98-bis-e, Apr. 12-20 2021

- R4-2108616, Reference Channel Emulation Curves for FR1, CMCC, Huawei, HiSilicon, CAICT

- R4-2108619, Reference Channel Emulation Curves and a New Beam Selection for CDL-C UMa, Keysight Rev of R4-2111273, Channel Model Targets, Spirent Communications