3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108784

Electronic Meeting, May. 19-27, 2021

Source:	ZTE Corporation

Title:	On Measurement Accuracy Requirements for RSTD

Agenda item:	6.5.2.2.2.1

Document for:	Discussion

# 1	Introduction

- The topic on accuracy requirements for RSTD measurements was discussed during the RAN4 98-bis-e meeting without a conclusion. The current status is summarized in the WF [1] as shown below.

- This paper discusses the open issue listed above in the WF and provide our view.

2	Discussions

For the group delay calibration margin for UE Rx - Tx and RSTD (they are discussed jointly), we cannot agree to Option 1 which is 32 Tc. 32 Tc = 16 ns can itself introduce a positioning error of 5 meters, not to mention other sources of errors. Considering other sources of errors, the positioning error can be larger than 10 meters if the margin is 32 Tc.

Observation 1: 32 Tc = 16 ns can itself introduce a positioning error of 5 meters, not to mention other sources of errors.

For UE Rx-Tx and RSTD measurements in FR1, add a group delay calibration margin of 4 ns * (100/BW), where BW is the PRS bandwidth in MHz, to the accuracy requirements.

3	Conclusion

Proposal 1: For UE Rx-Tx and RSTD measurements in FR1, add a group delay calibration margin of 4 ns * (100/BW), where BW is the PRS bandwidth in MHz, to the accuracy requirements.

References

- R4-2105750