3GPP TSG-RAN4 Meeting #99-e	R4-2108841

# Online,  19th May – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1826 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of DRX configuration in Event-triggered Test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Associated with the previously agreed CR (R4-2100601), there are still remaining TCs to apply similar corrections in FR2 (EN-DC, SA) and E-UTRA (SA) part. As shown in the example below, each test parameters clause for event trigger reporting test with DRx operation defines a policy to keep uplink time alignment during test by sending new Timing advance command. A.6.6.1.2.2 Test parameters UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. If time alignment timer is shorter than DRx cycle, there is no chance to send Timing Advance Command via PDSCH at OnDuration of DRx cycle before expiration of time alignment timer. That means that uplink time alignment is not able to be sustained and additional random access procedure to provide Timing advance comand with UE need to be conducted frequently during the test. In A.5.6.1.2 for instance, test2 refers to DRX.2 where TimeAlignmentTimer is 500ms and DRX cycle is 640ms. The DRX.2 should be replaced with DRX.7 where time alignment timer is set to Infinity. For E-UTRA standalone related TC (A.8), new DRX configuration is necessary to set the time alignment time to Infinity. Description of test purpose also needs to be corrected to specify the necessary period for sending new Timing Advance Command. There are typos at test number in a table. (Test 2 duplicated.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For FR2 related TC, change DRX configuration from DRX.2 to DRX.7 to correct time alignment timer to Infinity. - For E-UTRA related TC, change DRX configuration from DRX.10 to DRX.12 (new) to correct time alignment timer to Infinity. - Create a new DRX configuration (DRX.12). - Corrected descriptions at test requirement / parameter to specify the necessary period for sending new Timing Advance Command. - Corrected typos. (Test 2 -> Test 3) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary additional random access procedure not related to test purpose need to be conducted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.10, A.3.3.12 (new), A.4.6.1.2.2, A.4.6.1.4.2, A.4.6.1.7.2, A.4.6.2.2.1, A.4.6.2.6.1, A.5.6.1.2, A.5.6.1.4, A.5.6.2.2, A.5.6.2.4, A.5.6.2.6, A.5.6.2.8, A.6.6.1.2.2, A.6.6.1.4.2, A.6.6.1.7.2, A.6.6.2.2.1, A.6.6.2.6.1, A.6.6.3.2.1, A.7.6.1.2, A.7.6.1.4, A.7.6.2.2, A.7.6.2.4, A.7.6.2.6, A.7.6.2.8, A.7.6.2.11, A.8.4.2.2, A.8.4.2.4, A.8.4.2.6, A.8.4.2.8 Isolated impact analysis: No change to UE requirements, changes test parameters only. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Differences from Rel-15 are A.4.6.1.7.2, A.6.6.1.7.2 and A.7.6.2.11. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

<<Start of change>>

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms

Table A.3.3.10-1: DRX.10: DRX cycle = 640 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf6 |
| drx-InactivityTimer | psf1920 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf640, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | 500 ms |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity

Table A.3.3.11-1: DRX.11: DRX cycle = 20 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| drx-LongCycleStartOffset | 20 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for NR serving cell. The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [2] |  |

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity

Table A.3.3.12-1: DRX.12: DRX cycle = 640 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf6 |
| drx-InactivityTimer | psf1920 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf640, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX

A.4.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

A.4.6.1.2.2	Test parameters

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. The test parameters for PSCell are given in Table A.4.6.1.2.1-1, A.4.6.1.2.1-2, A.4.6.1.2.1-3 and A.4.6.1.2.1-4 below and the test parameters and applicability for the E-UTRAN cell are defined in A.3.7.2. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX

A.4.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

A.4.6.1.4.2	Test parameters

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. The test parameters for PSCell are given in Table A.4.6.1.4.2-1, A.4.6.1.4.2-2, A.4.6.1.4.2-3 A.4.6.1.4.2-4 and A.4.6.1.4.2-5 below and the test parameters and applicability for the E-UTRAN cell are defined in A.3.7.2. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3.

There are two BWPs configured in Cell 2, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 2. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16

A.4.6.1.7.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasFlag-r16. This test will partly verify the intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

A.4.6.1.7.2	Test parameters

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. The test parameters for PSCell are given in Table A.4.6.1.7.1-1, A.4.6.1.7.1-2, A.4.6.1.7.1-3 and A.4.6.1.7.1-4 below and the test parameters and applicability for the E-UTRAN cell are defined in A.3.7.2. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used

A.4.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.  The test parameters and configurations are given in Tables A.4.6.2.2.1-1, A.4.6.2.2.1-2, and A.4.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.4.6.2.2.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.4.6.2.2.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.4.6.2.2.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used

A.4.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.  The test parameters and configurations are given in Tables A.4.6.2.6.1-1, A.4.6.2.6.1-2, and A.4.6.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.4.6.2.6.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.4.6.2.6.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.4.6.2.6.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX

A.5.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.5.6.1.2.1-1.

Table A.5.6.1.2.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are three cells in the test, E-UTRAN PCell (Cell 1), FR2 PSCell (Cell 2) and a FR2 neighbour cell (Cell 3) on the same frequency as the PSCell. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.5.6.1.2.1-2 ~ Table A.5.6.1.2.1-6 below.

In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.5.6.1.2.1-2: General test parameters for intra-frequency event triggered reporting for EN-DC with TDD PSCell in FR2 without gap with DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1~4 | E-UTRAN PCell (Cell 1) PSCell (Cell 2) |  |  |
| Neighbour cell |  | 1~4 | Cell 3 |  | Cell to be identified. |
| RF Channel Number |  | 1~4 | 1: Cell 1 2: Cell 2 and Cell 3 |  | One TDD carrier frequency is used for the NR cells and one TDD or FDD carrier frequency is used for E-UTRAN cell. |
| SMTC configuration |  | 1~4 | SMTC.1 |  |  |
| A3-Offset | dB | 1~4 | -6 |  |  |
| CP length |  | 1~4 | Normal |  |  |
| Hysteresis | dB | 1~4 | 0 |  |  |
| Time To Trigger | s | 1~4 | 0 |  |  |
| Filter coefficient |  | 1~4 | 0 |  | L3 filtering is not used |
| DRX |  | 1~4 | DRX.1 | DRX.7 | DRX related parameters are defined in Table A.5.6.1.2.1-4 |
| Time offset between Cell 1 and Cell 2 |  | 1~4 | 3 s |  | Synchronous EN-DC |
| Time offset between Cell 2 and Cell 3 |  | 1~4 | 3 s |  | Synchronous cells |
| T1 | s | 1~4 | 5 |  |  |
| T2 | s | 1~4 | 10 | 52 |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX

A.5.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.5.6.1.4.1-1.

Table A.5.6.1.4.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are three cells in the test, E-UTRAN PCell (Cell 1), FR2 PSCell (Cell 2) and a FR2 neighbour cell (Cell 3) on the same frequency as the PSCell. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.5.6.1.4.1-2 ~ 6.

During the test, Cell 2 and Cell 3 are transmitted from the direction determined according to A3.8.

There are two BWPs configured in Cell 2, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 2. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.5.6.1.4.1-2: General test parameters for intra-frequency event triggered reporting for EN-DC with TDD PSCell in FR2 with per-UE gaps with DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1~4 | E-UTRAN PCell (Cell 1) PSCell (Cell 2) |  |  |
| Neighbour cell |  | 1~4 | Cell 3 |  | Cell to be identified. |
| RF Channel Number |  | 1~4 | 1: Cell 1 2: Cell 2 and Cell 3 |  | One TDD carrier frequency is used for the NR cells and one TDD or FDD carrier frequency is used for E-UTRAN cell. |
| Gap type |  | 1~4 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1~4 | 40 |  |  |
| Measurement gap length | ms | 1~4 | 6 |  |  |
| Measurement gap offset | ms | 1~4 | 39 |  |  |
| SMTC configuration |  | 1~4 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1~4 | CSI-RS.3.2 TDD |  |  |
| A3-Offset | dB | 1~4 | -6 |  |  |
| CP length |  | 1~4 | Normal |  |  |
| Hysteresis | dB | 1~4 | 0 |  |  |
| Time To Trigger | s | 1~4 | 0 |  |  |
| Filter coefficient |  | 1~4 | 0 |  | L3 filtering is not used |
| DRX |  | 1~4 | DRX.1 | DRX.7 | DRX related parameters are defined in Table A.5.6.1.4.1-5 |
| Time offset between Cell 1 and Cell 2 |  | 1~4 | 3 s |  | Synchronous EN-DC |
| Time offset between Cell 2 and Cell 3 |  | 1~4 | 3 s |  | Synchronous cells |
| T1 | s | 1~4 | 5 |  |  |
| T2 | s | 1~4 | 10 | 52 |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used

A.5.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR2 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.5.6.2.2.1-1, A.5.6.2.2.1-2, and A.5.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.5.6.2.2.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table A.5.6.2.2.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.2-1. Supported test configurations are shown in table A.5.6.2.2.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.5.6.2.2.1-1 EN-DC event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | LTE FDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.5.6.2.2.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  |  |  | One E-UTRAN TDD carrier frequenciy is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  |  |  | Two FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 |  | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 39 |  | 39 |  |  |
| SMTC-SSB parameters |  | Config 1,2 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1,2 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |  |  |
| CP length |  | Config 1,2 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |  |  |
| T2 | s | Config 1,2 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used

A.5.6.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR2 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.5.6.2.4.1-1, A.5.6.2.4.1-2, and A.5.6.2.4.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.5.6.2.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table A.5.6.2.4.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.2-1. Supported test configurations are shown in table A.5.6.2.4.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.5.6.2.4.1-1: EN-DC event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | LTE FDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.5.6.2.4.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  |  |  | One E-UTRAN TDD carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  |  |  | Two FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 |  | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 39 |  | 39 |  |  |
| SMTC-SSB parameters |  | Config 1,2 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1,2 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |  |  |
| CP length |  | Config 1,2 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |  |  |
| T2 | s | Config 1,2 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used

A.5.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.5.6.2.6.1-1, A.5.6.2.6.1-2, and A.5.6.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.5.6.2.6.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table A.5.6.2.6.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.5.6.2.6.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.5.6.2.6.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |  |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.5.6.2.6.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN TDD carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | One FR1 and one Fr2 NR carrier frequency is used. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 39 |  | 39 |  |  |
| SMTC-SSB |  | Config 1,4 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
| parameters on NR RF |  | Config 2,5 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
| Channel 1 |  | Config 3,6 | SSB.2 FR1 |  |  |  | As specified in clause A.3.10.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3,4,5,6 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| CSI-RS for tracking |  | Config 1,4 | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2,5 | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3,6 | TRS.1.2 TDD |  |  |  |  |
| offsetMO | dB | Config 1,2,3,4,5,6 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3,4,5,6 | -120 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 2,3,5,6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used

A.5.6.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.5.6.2.8.1-1, A.5.6.2.8.1-2, and A.5.6.2.8.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.5.6.2.8.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table A.5.6.2.8.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.5.6.2.8.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.5.6.2.8.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |  |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.5.6.2.8.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN TDD carrier frequenciy is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | One FR1 and one FR2 NR carrier frequency is used. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 39 |  | 39 |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1,4 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  |  |  | As specified in clause A.3.10.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3,4,5,6 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| CSI-RS for tracking |  | Config 1,4 | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2,5 | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3,6 | TRS.1.2 TDD |  |  |  |  |
| offsetMO | dB | Config 1,2,3,4,5,6 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3,4,5,6 | -120 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 2,3,5,6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.6.1.2	SA event triggered reporting tests without gap under DRX

A.6.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

A.6.6.1.2.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.2.2-1, A.6.6.1.2.2-2 and A.6.6.1.2.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX

A.6.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

A.6.6.1.4.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.4.2-1, A.6.6.1.4.2-2 and A.6.6.1.4.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16

A.6.6.1.7.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasFlag-r16. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

A.6.6.1.7.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.7.2-1, A.6.6.1.7.2-2 and A.6.6.1.7.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used

A.6.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.2.1-1, A.6.6.2.2.1-2 and A.6.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.2.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.2.2.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used

A.6.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.6.6.2.6.1-1, A.6.6.2.6.1-2 and A.6.6.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.6.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.2.6.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1

A.6.6.3.2.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3. There are two test cases. In test 1 the UE shall be configured with DRX cycle of 40 ms. In test 2 the UE shall be configured with DRX cycle of 640 ms.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indctated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.2.1-1. General test parameters are provided in Table A.6.6.3.2.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.2.1-3 and A.6.6.3.2.1-4, respectively.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.7.6.1.2	SA event triggered reporting test without gap under DRX

A.7.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.1.2.1-1.

Table A.7.6.1.2.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.1.2.1-2 ~ 6.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.1.2.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.7 | DRX related parameters are defined in Table A.7.6.1.2.1-5 |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX

A.7.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.1.4.1-1.

Table A.7.6.1.4.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.1.4.1-2, A.7.6.1.4.1-3 and A.7.6.1.4.1-4 below.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.1.4.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps with DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | 1, 2 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1, 2 | 40 |  |  |
| Measurement gap length | ms | 1, 2 | 6 |  |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.3.2 TDD |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.7 | DRX related parameters are defined in Table A.7.6.1.2.1-5 |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)

A.7.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.2.1-1, A.7.6.2.2.1-2, and A.7.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 13 as defined in Table A.7.6.2.2.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.2.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.2.1-1: SA event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)

A.7.6.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.4.1-1, A.7.6.2.4.1-2, and A.7.6.2.4.1-3.

In test 1&2 measurement gap pattern configuration # 13 as defined in Table A.7.6.2.4.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.4.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.4.1-1: SA event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)

A.7.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.6.1-1, A.7.6.2.6.1-2, and A.7.6.2.6.1-3.

In test 1&2 per-UE measurement gap pattern configuration # 0 as defined in Table A.7.6.2.6.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 no gap pattern is configured as defined in Table A.7.6.2.6.1-2. If a UE supports per-FR gap  it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Supported test configurations are shown in table A.7.6.2.6.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.6.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.7.6.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | One NR FR1 and one NR FR2 carrier frequency is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.10.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | Config 1 | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 | TRS.1.2 TDD |  |  |  |  |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -120 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)

A.7.6.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2. The test parameters and configurations are given in Tables A.7.6.2.8.1-1, A.7.6.2.8.1-2, and A.7.6.2.8.1-3.

In test 1&2 per-UE measurement gap pattern configuration # 0 as defined in Table A.7.6.2.8.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement no gap pattern is configured as defined in Table A.7.6.2.8.1-2.If a UE supports per-FR gap , it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Supported test configurations are shown in table A.7.6.2.8.1-1.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.8.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.7.6.2.8.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | One NR FR1 and one NR FR2 carrier frequency is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.10.1 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | Config 1 | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 | TRS.1.2 TDD |  |  |  |  |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | TBD |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.7.6.2.11	SA event triggered reporting test without gap under DRX

A.7.6.2.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD inter-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.2.11.1-1.

Table A.7.6.2.11.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) on NR RF channel 1 and a FR2 neighbour cell (Cell 2) on NR RF channel 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from cell 1 and cell 2 should be different in frequency location within the cell bandwidth. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.2.11.1-2 ~ 6.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided at least once with a period of shorter than time alignment timer with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.11.1-2: General test parameters for inter-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | 1, 2 | 2 TDD carrier frequency are used for the NR cells. |
| Active cell |  | 1, 2 | PCell (Cell 1) | Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. Cell 2 is on NR RF channel number 2. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -6 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 52 |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used

A.8.4.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21 of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.2.1-1, A.8.4.2.2.1-2, A.8.4.2.2.1-3 and A.8.4.2.2.1-4.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.2.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.2.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.2.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.2.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 |  | 11 | 2 |  | 11 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.2.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.2.1-4 |  |  |  |  |  |  |  |  |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used

A.8.4.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.4.1-1, A.8.4.2.4.1-2, A.8.4.2.4.1-3 and A.8.4.2.4.1-4.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.4.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.4.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 |  | 13 | 2 |  | 13 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.4.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.4.1-4 |  |  |  |  |  |  |  |  |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used

A.8.4.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 1.  The test parameters are given in Tables A.8.4.2.6.1-1, A.8.4.2.6.1-2 and A.8.4.2.6.1-3.

The cell specific test parameters for E-UTRA cell1 as PCell are defined in clause A.3.7.2.2.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.6.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.6.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have timing information of NR cell 2.

Table A.8.4.2.6.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.6.1-2: General test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.3.7.2.2. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [16]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 6 |  | 83 | 6 |  | 83 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.6.1-3 |  |  |  |  |  |  |  |  |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used

A.8.4.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21 of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.8.1-1, A.8.4.2.8.1-2 and A.8.4.2.8.1-3.

The cell specific test parameters for E-UTRA cell1 as PCell are defined in clause A.3.7.2.2.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.8.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.8.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.8.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.8.1-2: General test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.3.7.2.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [16]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |  |  |
| T2 | s | 1, 2 | 7 | 70 | 7 | 70 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.8.1-3 |  |  |  |  |  |  |  |

<<End of change>>