3GPP TSG-RAN WG4 Meeting #99-e                                         R4-2109249

Electronic Meeting, 19th-27th May, 2021

Source: 	Xiaomi

Title: 	Discussion on inter-frequency measurements requirement for R17 FR1 HST

Agenda Item:	9.7.2.1.3

Document for:	Discussion

# 1 Introduction

In the RAN4#98-bis-e meeting, the WF on RRM for FR1 HST was agreed [1]. In this paper, we provide our discussions on the RRM inter-frequency measurements requirement for R17 FR1 HST.

- Discussion

- Inter-frequency measurement - Define RRC Connected state inter-frequency measurement enhancements - Support of HST inter-frequency measurement enhancements is up to UE capability. Details are FFS - FFS whether enhancements for RRC IDLE inter-frequency measurements are needed

Connected mode

In last meeting, it was agreed to introduce the inter-frequency measurement for Rel-17 HST.

For inter-frequency measurement without MG, the R15 requirement for inter-frequency measurements is identical to the intra-frequency measurements. Thus we think the same enhancement requirement in R16 HST could be reused.

Proposal 1: For inter-frequency measurement without MG, the same enhancement for intra-frequency measurement without MG in R16 HST could be reused.

In R16 HST, the EUTRAN-NR inter-RAT measurement requirements were enhanced. Corresponding requirements for E-UTRAN TDD/FDD – NR inter-RAT measurement was specified in TS 36.133 in below. For NR inter-frequency measurement with MG, we think RAN4 could follow the same strategy.

Table 8.1.2.4.21.1.1-1A: Time period for PSS/SSS detection for UE configured with highSpeedInterRAT-r16 (Frequency range FR1) Condition NOTE1,2; TPSS/SSS_sync_irat. No DRX; Max(600ms, 8 Max(MGRP, SMTC period)) Nfreq. DRX cycle < 320ms; Max(600ms, ceil( 8 × M) × max(MGRP, SMTC period, DRX cycle)) ×Nfreq. DRX cycle ≥ 320ms; 8× DRX cycle ×Nfreq. NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 of TS 38.133 [50] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms.

Table 8.1.2.4.21.1.1-3A: Time period for time index detection for UE configured with highSpeedInterRAT-r16 (Frequency range FR1) Condition NOTE1,2; TSSB_time_index_irat. No DRX; Max(120ms, 3 Max(MGRP, SMTC period)) Nfreq. DRX cycle < 320ms; Max(120ms, Ceil(3 M) Max(MGRP, SMTC period, DRX cycle)) Nfreq. DRX cycle ≥ 320ms; 3 DRX cycle Nfreq. NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 of TS 38.133 [50] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms. Table 8.1.2.4.21.1.1-5A: Measurement period for inter-RAT measurements for UE configured with highSpeedInterRAT-r16 (Frequency range FR1) Condition NOTE1,2; TSSB_measurement_period_irat. No DRX; Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq. DRX cycle < 320ms; Max(200ms, ceil(8 × M) x max(MGRP, SMTC period, DRX cycle))×Nfreq. DRX cycle ≥ 320ms; 4× M DRX cycle ×Nfreq. NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 of TS 38.133 [50] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms.

Proposal 2: For inter-frequency measurement with MG, the enhanced requirement for E-UTRAN TDD/FDD – NR inter-RAT measurements defined in R16 HST could be used as baseline.

Idle mode

In high speed scenario, if the network is implemented by inter-frequency, the mobility performance in idle mode should be guaranteed, thus the inter-frequency measurement should be enhanced in HST scenario

Proposal 3: RAN4 to define the idle mode inter-frequency measurement enhancements for R17 FR1 HST.

If proposal 3 is approved, similar discussion as for inter-frequency measurement requirements in connected mode, we prefer to reuse the enhanced EUTRA-NR inter-RAT measurement requirements.

Table 4.2.2.5.6-2: Tdetect,NR_HST, TmeasureNR_HST, and Tevaluate,NR_HST for UE configured with highSpeedInterRAT-NR-r16 DRX cycle length [s]; Tdetect,NR_HST [s] (number of DRX cycles); Tmeasure,NR_HST [s] (number of DRX cycles); Tevaluate,NR_HST [s] (number of DRX cycles). 0.32; 4.16 x M2 (13 x M2)Note 2; 0.64 x M3 (2 x M3)Note 2; 0.96 x M4 (3 x M4) Note 2. 0.64; 7.68 (12); 1.28 (2); 1.92 (3). 1.28; 12.8(10); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: FR2 high speed requirements are not specified. Note 2: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=1..

Proposal 4: The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements.

- Conclusion

Proposal 1: For inter-frequency measurement without MG, the same enhancement for intra-frequency measurement without MG in R16 HST could be reused.

Proposal 2: For inter-frequency measurement with MG, the enhanced strategy for E-UTRAN TDD/FDD – NR measurements in R16 HST could be used as baseline.

Proposal 3: RAN4 to define the idle mode inter-frequency measurement enhancements for R17 FR1 HST.

Proposal 4: The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements.

- Reference

[1] R4-2105793 WF on FR1 HST RRM, CMCC