3GPP TSG-RAN WG4 Meeting #99-e	R4-2110292

# Online, 19th– 27th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2006 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on maintaining sync conditions for intra-band DAPS handover R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For FR1 DAPS hadover, the synchronous conditions are defined with adding 3 notes. In current specification, Notes 2/3 clairfies to leave enough time for UE performing DL-to-UL and UL-to-DL switching only from single cell perspective. However, the UE shall be allowed to switching time between both source cell and target cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To correct Notes 2/3 in Table 6.1.3.2-1. - To correct some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DAPS handover requirements are not properly defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.1.3.2	NR FR1 - NR FR1 DAPS Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR1 cell to NR FR1 cell. A DAPS handover is intra-frequency if the centre frequency of the SSB of the source cell and the centre frequency of the SSB of the target cell are the same, and 	the subcarrier spacing of the two SSBs are also the same.

Note:	For intra-frequency DAPS handover, no requirement applies if active DL and UL BWP of target cell is not confined within the active DL and UL BWP of the source cell respectively.

Note:	For inter-frequency DAPS handover, no requirement applies if the BWP of target cell is overlaped with the BWP of source cell in frequency domain.

An FR1 DAPS handover is synchronous if it meets the conditions in table 6.1.3.2-1, otherwise it is asynchronous

Table 6.1.3.2-1: Sync conditions for FR1 DAPS handover

| Type of handover | Maximum receive timing difference between source and target cell (µs) for sync DAPS handover | Maximum transmit timing difference between source and target cell (µs) for sync DAPS handover |
| --- | --- | --- |
| Intra-frequencyNote 1,2,3 | 6µs | 7.6 µs |
| Intra-band inter-frequency Note 1,2,3 | 6µs | 7.6 µs |
| Inter-band inter-frequency | 33 µs | 34.6 µs |
| Note 1: For synchonous DAPS handover, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. For asynchronous DAPS handover, if the receive time difference exceeds the cyclic prefix length of that SCS, interruptions may occur depending on UE implementation. The duration and frequency of occurrence of such interruptions is not specified. Note 2: For DAPS handover on a TDD band, a UE is not expected to transmit in the uplink to source or target cell earlier than NRX-TX after the end of the last received downlink symbol from source or target cell in the same TDD band where NRX-TX=25600Tc. Note 3: For DAPS handover on a TDD band, a UE is not expected to receive in the downlink from source or target cell earlier than NTX-RX after the end of the last transmitted uplink symbol toward source or target cell in the same TDD band where NTX-RX=25600Tc. |  |  |

6.1.3.2.1	DAPS handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

TRRC_procedure is the maximum RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

After successful RACH procedure of the target cell, when the UE receives an RRC message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure+ Tinterrupt2

Where:

TRRC_procedure is the RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

6.1.3.2.2	Interruption time

During Dhandover1, the UE is allowed an interruption of up to Tinterrupt1 on source cell.

For FR1-to-FR1 intra-frequency handover, Tinterrupt1 is specified in Table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Tinterrupt1 for FR1-to-FR1 intra-frequency DAPS HO

|  | NR Slot length (ms) | Interruption length Tinterrupt1 (slotsNote 1), synchronous DAPS HO | Interruption length Tinterrupt1 (slotsNote 1), asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 4 | 5 |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: It is assumed that the BWP of target cell is not larger than the BWP of source cell. It is assumed that the CBW of target cell is not larger than the CBW of source cell Note 3: Void |  |  |  |

For FR1-to-FR1 intra-band inter-frequency handover, Tinterrupt1 is specified in Table 6.1.3.2.2-2.

Table 6.1.3.2.2-2: Tinterrupt1 for FR1-to-FR1 intra-band inter-frequency DAPS HO

|  | NR Slot length (ms) | Tinterrupt1 (slotsNote 1), synchronous DAPS HO | Tinterrupt1 (slotsNote 1), asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ | 2+ TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ | 3+ TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ | 5+ TSMTC_duration * Nslotsubframe,μ |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: TSMTC_duration measured in subframes is the longest SMTC duration between source cell and target cell. Note 3: Void Note 4: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |

For FR1-to-FR1 inter-band handover, Tinterrupt1 is specified in Table 6.1.3.2.2-3.

Table 6.1.3.2.2-3: Tinterrupt1 for FR1-to-FR1 inter-band DAPS HO

|  | NR Slot length (ms) | Tinterrupt1 (slots) |  |
| --- | --- | --- | --- |
|  | of source cell | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 | 5 |

During Dhandover2, the UE is allowed an interruption of up to Tinterrupt2 on target cell.

For FR1-to-FR1 intra-frequency handover, Tinterrupt2 equals to 2ms when the BWP of target cell is smaller than the BWP of source cell, and Tinterrupt2 is specified in Table 6.1.3.2.2-4 when the same BWP is used for target cell and source cell.

Table 6.1.3.2.2-4: Tinterrupt2 for FR1-to-FR1 intra-frequency DAPS HO

|  | NR Slot length (ms) | Interruption length X (slotsNote 1) | Tinterrupt2 (slotsNote 1) for asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 4 | 5 |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: It is assumed that the BWP of target cell is the same as the BWP of source cell. Note 3: Void |  |  |  |

For FR1-to-FR1 intra-band inter-frequency handover, Tinterrupt2 is specified in Table 6.1.3.2.2-5.

Table 6.1.3.2.2-5: Tinterrupt2 for FR1-to-FR1 intra-band inter-frequency DAPS HO

|  | NR Slot length (ms) | Tinterrupt2 (slotsNote 1) for synchronous DAPS HO | Tinterrupt2 (slotsNote 1) for asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ | 2+ TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ | 3+ TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ | 5+ TSMTC_duration * Nslotsubframe,μ |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: TSMTC_duration measured in subframes is the longest SMTC duration between source cell and target cell. Note 3: Void. Note 4: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |

For FR1-to-FR1 inter-band handover, Tinterrupt2 is specified in Table 6.1.3.2.2-6.

Table 6.1.3.2.2-6: Tinterrupt2 for FR1-to-FR1 inter-band DAPS HO

|  | NR slot length (ms) | Tinterrupt2 (slots) |  |
| --- | --- | --- | --- |
|  | of target cell | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 | 5 |

<End of Change 1>