3GPP TSG-RAN WG4 Meeting #99 	R4-2110418

# Electronic Meeting, May. 19-27, 2021

Source:	Ericsson

Title:	GNSS requirements

Agenda item:	9.12.4.2

Document for:	Discussion

# 1	Introduction

Several timing requirements depends on UE and satellite positioning error. We analyze the required positioning accuracy, for UE initial transmission error, in this contribution.

# 2 	UE initial transmission error

Our main point is that the it is the total NTN UE Te error that decides UL performance, no matter the source of inaccuracy. If a UE is subject to a positioning error ΔUE-pos (in time), the error in satellite position is ΔSat-pos (in time) and the UE SSB timing estimation accuracy is ΔUE_timing_estimate then we get, where Te_NTN is the initial timing error:

ΔUE-pos + ΔSat-pos  + ΔUE_timing_estimate  < Te_NTN

Te is needed in existing specification to safeguard against Inter Symbol interference and maintain a desired UL throughput and capacity. We analyze the different terms

## 2.1	Initial error Te

In existing specification Te is:

Table x: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

In our contribution [1] we argue that existing Te_NTN  = 2*Te  for NTN UE. This means that Te_NTN range from 0.780 µs to 0.196 µs.

## 2.2	Satellite position error ΔSat-pos

The term ΔSat-pos  has been analysed in [2]. Where we get:

Table 1: Simulations of accuracy of propagation based on PV ephemeris [2]

| Prediction time ahead for pre-compensation | Delay error (us) | percentage of the Te error budget ± 0.39 us |
| --- | --- | --- |
| 10s | 0.012 us | <3% |
| 30s | 0.037 us | 9% |

Table 2: Simulations of accuracy of propagation based on orbital ephemeris [2]

| Prediction time ahead for pre-compensation | Delay error (us) | percentage of the Te error budget ± 0.39 us |
| --- | --- | --- |
| 10s | 0.012 us | < 3 % |
| 30s | 0.040 us | 10 % |

It is reasonable to assume that ΔSat-pos  < 0.012 µs.

## 2.3	ΔUE_timing_estimate

We assume that ΔUE_timing_estimate correspond to the exiting Te.

## 2.4	ΔUE-pos

Since ΔUE-pos + ΔSat-pos  + ΔUE_timing_estimate  < Te_NTN then we have

ΔUE-pos  < Te_NTN - ΔSat-pos  -   ΔUE_timing_estimate

Since we assume that ΔUE_timing_estimate = Te and ΔSat-pos < 0.012 µs we have:

ΔUE-pos  < Te - ΔSat-pos  = Te – 0.012 µs

In Table 3, the ΔUE-pos time is concerted into an lower bound for position error using ΔUE-pos * c.

Table 3: Simulations of accuracy of propagation based on orbital ephemeris [2]

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te | ΔUE-pos < Te – 0.012 µs | ΔUE-pos (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc | 0,379 | 114 |
|  |  | 30 | 10*64*Tc | 0,313 | 94 |
|  |  | 60 | 10*64*Tc | 0,313 | 94 |
|  | 30 | 15 | 8*64*Tc | 0,248 | 75 |
|  |  | 30 | 8*64*Tc | 0,248 | 75 |
|  |  | 60 | 7*64*Tc | 0,216 | 65 |
| 2 | 120 | 60 | 3.5*64*Tc | 0,102 | 31 |
|  |  | 120 | 3.5*64*Tc | 0,102 | 31 |
|  | 240 | 60 | 3*64*Tc | 0,086 | 26 |
|  |  | 120 | 3*64*Tc | 0,086 | 26 |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |

Observation 1: A UE specific margin of Te on top of existing UE initial access requirement will work if ΔUE-pos corresponds to table 3, that is from 114 m for SCS = 15 kHz in UL to 26 to 31 m for SCS = 120 kHz in UL.

# 3	Summary

Observation 1: A UE specific margin of Te on top of existing UE initial access requirement will work if ΔUE-pos corresponds to table 3, that is from 114 m for SCS = 15 kHz in UL to 26 to 31 m for SCS = 120 kHz in UL.

# References

- R4-2110416	Timing requirements	Ericsson

- R4-2106360, Discussion on timing requirements in NTN, MediaTek Inc