3GPP TSG-RAN WG4 Meeting # 99-e	R4-2110458

# Electronic Meeting, May. 19-27, 2021

Source:	ZTE Corporation

Title:	TP for TR38.717-03-01_CA_n41A-n79A-n258A

Agenda Item:	8.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/1UL CA_n41A-n79A-n258A in TR38.717-03-01.

- Reference

- TR38.717-03-01,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL,v0.2.0

- Text Proposal

----- Start of TP -----

## 6.X	CA_n41-n79-n258

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41A-n79A-n258A | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |
| CA_n41A-n79A-n258A | - | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |
|  |  | n258 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |

### 6.X.3	Co-existence studies

The coexistence studies of harmonic interference have been captured in the constituent fallback modes in TR 38.716-02-00.

    - 6.X.4	∆TIB and ∆RIB values

For CA_n41A-n79A-n258A,  the TIB,c and RIB values are given in the tables below, where same TIB,c and RIB values as CA_n41A-n79A for band n41 and band n79 .

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n79-n258 | n41 | 0.3 |
|  | n79 | 0.8 |
|  | n258 | 0 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Combination | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n41-n79-n258 | n41 | 0.5 |
|  | n79 | 0.5 |
|  | n258 | 0 |

### 6.X.5	MSD

MSD studies can be omitted because harmonic interference between FR1 bands have been already studied for CA_n41-n79, and harmonic interference between FR1 bands and FR2 band are negligible

----- End of TP -----