3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110509

# Electronic Meeting, May. 19-27, 2021

Title: 	Simulation results on NR-U CG-UCI performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.1.7.4.1

Document for:	Discussion

Background

At the last meeting, simulation assumptions of the CG-UCI were determined. The agreements and options derived from [1] are:

- BetaOffsetCG-UCI-index configuration - BetaOffsetCG-UCI-index = 8 - Number of CG-UCI information bits - 18 bits - Pattern of CG-UCI information bits - Use a bit pattern {c0,c1,…,c17} that consists of : - HARQ process number: [c0,c1,c2,c3] = [0 0 0 1] - RV sequence: [c4,c5] = [0 0] - NDI: [c6] - Option 1: [c6] = [1] - Option 2: [c6] = toggle for every new transmission, e.g. 0 for even transmissions and 1 for odd ones - COT sharing information field: [c7,c8,…c17] = [0,0,0,0,0,0,0,0,0,0,0]

In this paper, we will give our simulation results.

Simulation results

The simulation results are shown in Table 2-2 based on the simulation assumptions from Table 2-1:

Table 2-1: Simulation results of CG-UCI

| Parameters |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1T2R Low |
| SCS and BW |  | 15kHz: 20MHz; 30kHz: 20MHz |
| TDD pattern |  | 7D2S1U S=6D:4G:4U for 30kHz SCS 3D1S1U S=10D:2G:2U for 15kHz SCS |
| Interlace structure |  | First single interlace per slot |
| DM-RS type |  | type 1 with single-symbol |
| Number of DMRS symbols |  | dmrs-AdditionalPosition ‘pos1’ (1+1) |
| symbols length |  | 14 |
| start symbol index |  | 0 |
| Time domain resource allocation type |  | Type A and B |
| Frequency domain resource |  | Full applicable test bandwidth |
| MCS index |  | 20 |
| Carrier frequency (GHz) |  | 4 |
| Propagation condition |  | TDLA30-10 |
| Guard band |  | Not configured |
| PT-RS |  | Not configured |
| Timing offset |  | 0 |
| Frequency offset |  | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching |  | Disabled |
| Number of HARQ transmissions |  | 1 |
| RV sequence |  | 0 |
| CG-UCI | scaling | 1 |
|  | Payloads (bits) | 8 and 18 |
|  | bettaOffsetCG-UCI-index | 8 |
| Testing metric |  | SNR(dB)@1% BLER of CG-UCI |

Table 2-2: Simulation results of CG-UCI

| Information bits | SCS=15kHz |  | SCS=30kHz |  |
| --- | --- | --- | --- | --- |
|  | DMRS Type A | DMRS Type B | DMRS Type A | DMRS Type B |
| 18 bit | 2.23 | 2.08 | 2.18 | 2.31 |

Conclusion

In this paper, we give our simulation results on NR-U CG-UCI requirements.

Reference

[1]   R4-2106010 WF on NR-U BS demod requirements. Huawei, HiSilicon.