3GPP TSG-RAN WG4 Meeting#98bis             	R4-2110706

E-meeting, 19th – 27th May, 2021

Title:	TP to TR 38.717-03-01 Addition of CA_n12A-n66A-n77A

Source:	Nokia, AT&T

Agenda item:	8.10.2

Document for:	Approval

# 1	Introduction

This contribution is a TP to TR 38.717-03-01 to introduce CA_n12A-n66A-n77A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

### 6.x	CA_n12-n66-n77

#### 6.x.1		Operating bands for CA

Table 6.x.1-1: CA band combination of CA_n12 + n66 + n77

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n12A-n66A-n77A | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 745 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.x.2		Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported bandwidths per CA band combination of CA_n12 + n66 + n77

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n12A-n66A-n77A | - | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3  Co-existence studies

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. 3rd HAM from n66 UL might fall in n77 DL. MSD for this is already captured in 38.101-1

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 |
| 66 | 1710 | 1780 | 3420 | 3560 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Table 6.x.3-2 gives harmonic mixing issue for CA with Band n14, n66 and n77. No harmonic mixing issue is identified for this band combination.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 |
| 66 | 1710 | 1780 | 3420 | 3560 | 6840 | 7120 | 10260 | 10680 | 13680 | 14240 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

### 6.x.4	∆TIB,c and ∆RIB,c values

For three DLs of Band n12, n66 and n77, the same TIB,c and RIB,c values specified for DC_66_n77 and DC_12_n66, are used as below.

Table 6.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n12-n66-n77 | n12 | 0.8 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n12-n66-n77 | n12 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

#### 6.x.5	REFSENS requirements

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

*********************** End of the TP ***************************************************

# 3	References