3GPP TSG-RAN4 Meeting #99-e	R4-2110754

# Electronic Meeting, 19th May– 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2059 | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE DRX reference configuration_R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, In Reference DRX configuration DRX.10, TAT is set to 500ms, which is even shorter than DRX cycle (640ms). So it is impossible for SS to send MAC-CE in DRX on duration to keep UL synchonization.As a result UE has to perform RACH procedure to resume UL sync according to 38.321. Then conformant UE may fail the test due to the additional delay caused by unintended RACH procedure. 2. Furthermore, inactivity timer in DRX.9 and DRX.10 (100ms and 1920ms respectively) is longer than DRX cycle. In test SS shall provide UL grant at least every DRX cycle to UE, otherwise additional delay will be introduced due to lack of UL resource. According to 36.321 cl.5.7, it means that drx-InactivityTimer will be restarted every 40ms/640ms TS 321 cl.5.7: - if the PDCCH indicates a new transmission (DL, UL or SL): - except for an NB-IoT UE configured with a single DL and UL HARQ process and when PDCCH indicates the transmission is not for multiple TBs: - start or restart drx-InactivityTimer.. However, according to 36.133 cl.5, Note 1: For the performance requirements specified hereafter, the state when no DRX is used is defined as follows: - DRX and eDRX_CONN parameters are not configured; or - DRX or eDRX_CONN parameters are configured and ○ drx-InactivityTimer is running; or ○ drx-RetransmissionTimer is running; or ○ mac-ContentionResolutionTimer is running; or ○ a Scheduling Request sent on PUCCH/SPUCCH is pending; or ○ an uplink grant for a pending HARQ retransmission can occur and there is data in the corresponding HARQ buffer; or ○ a PDCCH/SPDCCH indicating a new transmission addressed to the C-RNTI of the UE has not been received after successful reception of a Random Access Response for the explicitly signaled preamble (only applicable to UEs in RRC_CONNECTED). Otherwise - It is the state when DRX is used.. Then it is impossible to verify inter-RAT measurement delay requirements with DRX since UE will always apply requirements corresponding to "no DRX". |
| Summary of change: |  | - Change TAT for DRX.10 to infinity. - Change inactivity timer for DRX.10 to psf2, i.e. 2ms, which is the configuration value used in DRX.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.9, A.3.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 CR 1192 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms

Table A.3.3.9-1: DRX.9: DRX cycle = 40 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf2 |
| drx-InactivityTimer | psf2 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf40, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | 500 ms |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = infinity

Table A.3.3.10-1: DRX.10: DRX cycle = 640 ms and time alignment timer (TAT) = 500 ms

| Field | Value |
| --- | --- |
| drx-onDurationTimer | psf6 |
| drx-InactivityTimer | psf2 |
| drx-RetransmissionTimer | psf16 |
| longDRX-CycleStartOffset | sf640, 0 |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: This DRX configuration is applicable for E-UTRA serving cell. For further information see clause 6.3.2 in TS 36.331 [16]. |  |

<End of modified section 1>