3GPP TSG-RAN4 Meeting #99-e	R4-2110897

# Electronic Meeting, May 19–27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to introduce gNB Rx-Tx time difference requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR is based on Big draftCR R4-2105751 endorsed at RAN4#98-bis-e. There is no exact gNB Rx-Tx accuracy requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces gNB Rx-Tx accuracy requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirements for gNB positioning measurement are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

13.2.2	Measurement Accuracy Requirements

13.2.2.1	Introduction

This clause defines accuracy requirements for gNB Rx-Tx time difference measurement in FR1 and FR2. The requirements are applicable for gNB supporting gNB Rx-Tx time difference measurement.

13.2.2.2	Requirements

The accuracy requirements in Table 13.2.2.2-1, Table 13.2.2.2-2, Table 13.2.2.2-3 and Table 13.2.2.2-4 are valid under the following conditions:

-	AWGN propagation conditions.

-	The measured signals are in the directions covered by RoAoA of OTA reference sensitivity requirements for gNB type 1-O and 2-O BS

Editor’s note: The requiremetns for Ês/Iot=-13dB, 24 ≤ BW ≤ 40 and comb+symbol size 2+1 is FFS.

Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C

| Accuracy Note 1 | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| TBD | ≥ -13 | 15 | 24 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| NOTE 1: The accuracy of gNB Rx-Tx time difference measurement is derived as the allowed value in this table plus an RF calibration margin Z, which is declared by the gNB manufacturer. |  |  |  |

Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-H

| Accuracy Note 1 | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| TBD | ≥ -13 | 15 | 24 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| NOTE 1: The accuracy of gNB Rx-Tx time difference measurement is derived as the allowed value in this table plus an RF calibration margin Z, which is declared by the gNB manufacturer. |  |  |  |

Table 13.2.2.2-3: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-O

| Accuracy Note 1 | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| TBD | ≥ -13 | 15 | 24 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| NOTE 1: The accuracy of gNB Rx-Tx time difference measurement is derived as the allowed value in this table plus an RF calibration margin Z, which is declared by the gNB manufacturer. |  |  |  |

Table 13.2.2.2-4: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O

| Accuracy Note 1 | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| TBD | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW ≤ 168 |
| TBD |  |  | 176 ≤ BW |
| TBD | ≥ -13 | 120 | 32 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW |
| TBD | ≥ +3 |  | 32 ≤ BW ≤ 40 |
| TBD |  |  | 44 ≤ BW ≤ 84 |
| TBD |  |  | 88 ≤ BW |
| NOTE 1: The accuracy of gNB Rx-Tx time difference measurement is derived as the allowed value in this table plus an RF calibration margin Z, which is declared by the gNB manufacturer. |  |  |  |

<End of Change 1>