3GPP TSG-RAN WG4 Meeting #99-e	R4-2111074

Electronic Meeting, 19 May – 27 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0389 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to add NR intra-band FR2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R17_Intra |  |  |  |  |  | Date: |  |  | 2021-05-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding approved NR Intra-band FR2 combinations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Approved NR Intra-band FR2 combinations at RAN4 98-bis-e: CA_n258O CA_n258P CA_n258Q Approved NR Intra-band FR2 combinations at RAN4 99-e: CA_n258(A-G) CA_n258(A-H) CA_n258(2G) CA_n258(G-H) Editorial: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Approved NR Intra-band FR2 combinations are not added |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 5.5A.1-1: NR CA configurations, bandwidth combination sets, and fallback group defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | Maximum aggregated BW (MHz) | BCS | Fallback group |
| CA_n257B | CA_n257B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n257C | CA_n257B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 | 1 |
| CA_n257D | CA_n257D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n257E | CA_n257D CA_n257E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n257F | CA_n257D CA_n257E CA_n257F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n257G | CA_n257G | 50, 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n257H | CA_n257G CA_n257H | 50, 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n257I | CA_n257G CA_n257H CA_n257I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n257J | CA_n257G CA_n257H CA_n257I CA_n257J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n257K | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n257L | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n257M | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n258B | CA_n258B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n258C | CA_n258B CA_n258C | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n258D | CA_n258D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n258E | CA_n258D CA_n258E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n258F | CA_n258D CA_n258E CA_n258F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n258G | CA_n258G | 50, 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n258H | CA_n258G CA_n258H | 50, 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n258I | CA_n258G CA_n258H CA_n258I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n258J | CA_n258G CA_n258H CA_n258I CA_n258J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n258K | CA_n258G CA_n258H CA_n258I CA_n258J CA_n258K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n258L | CA_n258G CA_n258H CA_n258I CA_n258J CA_n258K CA_n258L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n258M | CA_n258G CA_n258H CA_n258I CA_n258J CA_n258K CA_n258L CA_n258M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n258O | CA_n258O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n258P | CA_n258O CA_n258P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n258Q | CA_n258O CA_n258P CA_n258Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n259B | CA_n259B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n259C | CA_n259B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n259G | CA_n259G | 50, 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n259H | CA_n259G CA_n259H | 50, 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n259I | CA_n259G CA_n259H CA_n259I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n259J | CA_n259G CA_n259H CA_n259I CA_n259J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n259K | CA_n259G CA_n259H CA_n259I CA_n259J CA_n259K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n259L | CA_n259G CA_n259H CA_n259I CA_n259J CA_n259K CA_n259L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n259M | CA_n259G CA_n259H CA_n259I CA_n259J CA_n259K CA_n259L CA_n259M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n260B | CA_n260B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n260C | CA_n260B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n260D | CA_n260D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n260E | CA_n260D CA_n260E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n260F | CA_n260D CA_n260E CA_n260F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n260G | CA_n260G | 50, 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n260H | CA_n260G CA_n260H | 50, 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n260I | CA_n260G CA_n260H CA_n260I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n260J | CA_n260G CA_n260H CA_n260I CA_n260J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n260K | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n260L | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n260M | CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n260O | CA_n260O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n260P | CA_n260O CA_n260P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n260Q | CA_n260O CA_n260P CA_n260Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n261B | CA_n261B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n261C | CA_n261B | 50 | 400 | 400 |  |  |  |  |  | 850 | 0 |  |
| CA_n261D | CA_n261D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n261E | CA_n261D CA_n261E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n261F | CA_n261D CA_n261E CA_n261F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n261G | CA_n261G | 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n261H | CA_n261G CA_n261H | 100 | 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n261I | CA_n261G CA_n261H CA_n261I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n261J | CA_n261G CA_n261H CA_n261I CA_n261J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n261K | CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n261L | CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n261M | CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n261O | CA_n261O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n261P | CA_n261O CA_n261P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n261Q | CA_n261O CA_n261P CA_n261Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| NOTE 1: Void NOTE 2: For the NR CA configuration with more than two component carries, the bandwidths in a BCS which may introduce combinations more than requested unintentionally should be listed in a row separately. |  |  |  |  |  |  |  |  |  |  |  |  |

---Text Omitted---

Table 5.5A.2-2: NR CA configurations with multiple CA bandwidth classes defined for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | (BWChannel,block) (MHz) | BCS |
| CA_n258(A-G) | - | n258A | n258G |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n258(A-H) | - | n258A | n258H |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n258(2G) | - | n258G | n258G |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n258(G-H) | - | n258G | n258H |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
| CA_n260(A-D) | - | n260A | CA_n260D |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2A-D) | - | CA_n260(2A) |  | CA_n260D |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(A-2D) | - | n260A | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-2D) | - | CA_n260(2A) |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(A-D-O) | - | n260A | CA_n260D | CA_n260O |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(2A-D-O) | - | CA_n260(2A) |  | CA_n260D | CA_n260O |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(A-D-2O) | - | n260A | CA_n260D | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-D-2O) | - | CA_n260(2A) |  | CA_n260D | CA_n260(2O) |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(A-G) | CA_n260G | n260A | CA_n260G |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(2A-G) | CA_n260G | CA_n260(2A) |  | CA_n260G |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(A-2G) | CA_n260G | n260A | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2A-2G) | CA_n260G | CA_n260(2A) |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-2G-O) | - | CA_n260(2A) |  | CA_n260(2G) |  | CA_n260O |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(2A-2G-2O) | - | CA_n260(2A) |  | CA_n260(2G) |  | CA_n260(2O) |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(3A-2G) | - | CA_n260(3A) |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(4A-G) | - | CA_n260(4A) |  |  |  | CA_n260G |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(4A-2G) | - | CA_n260(4A) |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(A-2G-2O) | - | n260A | CA_n260(2G) |  | CA_n260(2O) |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-G-2O) | - | CA_n260(2A) |  | CA_n260G | CA_n260(2O) |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(3A-G) | CA_n260G | CA_n260(3A) |  |  | CA_n260G |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(A-2H) | - | n260A | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(2A-H) | - | CA_n260(2A) |  | CA_n260H |  |  |  |  |  |  |  |  |  | 1100 | 0 |
| CA_n260(2A-2H) | - | CA_n260(2A) |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(A-H) | CA_n260G CA_n260H | n260A | CA_n260H |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n260(A-O) | - | n260A | CA_n260O |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(A-O-P) | - | n260A | CA_n260O | CA_n260P |  |  |  |  |  |  |  |  |  | 900 | 0 |
| CA_n260(A-O-2P) | - | n260A | CA_n260O | CA_n260(2P) |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-O-P) | - | CA_n260(2A) |  | CA_n260O | CA_n260P |  |  |  |  |  |  |  |  | 1300 | 0 |
| CA_n260(2A-O-2P) | - | CA_n260(2A) |  | CA_n260O | CA_n260(2P) |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(2A-2O-P) | - | CA_n260(2A) |  | CA_n260(2O) |  | CA_n260P |  |  |  |  |  |  |  | 1500 | 0 |
| CA_n260(A-O-Q) | - | n260A | CA_n260O | CA_n260Q |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(A-O-2Q) | - | n260A | CA_n260O | CA_n260(2Q) |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(2A-O-Q) | - | CA_n260(2A) |  | CA_n260O | CA_n260Q |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(2A-O-2Q) | - | CA_n260(2A) |  | CA_n260O | CA_n260(2Q) |  |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(2A-2O-Q) | - | CA_n260(2A) |  | CA_n260(2O) |  | CA_n260Q |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(2A-O) | - | CA_n260(2A) |  | CA_n260O |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(A-2O) | - | n260A | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(A-2O-P) | - | n260A | CA_n260(2O) |  | CA_n260P |  |  |  |  |  |  |  |  | 1100 | 0 |
| CA_n260(A-2O-2P) | - | n260A | CA_n260(2O) |  | CA_n260(2P) |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(A-2O-Q) | - | n260A | CA_n260(2O) |  | CA_n260Q |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(A-2O-2Q) | - | n260A | CA_n260(2O) |  | CA_n260(2Q) |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(2A-2O) | - | CA_n260(2A) |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-2O-2P) | - | CA_n260(2A) |  | CA_n260(2O) |  | CA_n260(2P) |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(2A-2O-2Q) | - | CA_n260(2A) |  | CA_n260(2O) |  | CA_n260(2Q) |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(2A-3O) | - | CA_n260(2A) |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(3A-2O) | - | CA_n260(3A) |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(4A-O) | - | CA_n260(4A) |  |  |  | CA_n260O |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(4A-3O) | - | CA_n260(4A) |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  | 2200 | 0 |
| CA_n260(5A-O) | - | CA_n260(5A) |  |  |  |  | CA_n260O |  |  |  |  |  |  | 2200 | 0 |
| CA_n260(6A-O) | - | CA_n260(6A) |  |  |  |  |  | CA_n260O |  |  |  |  |  | 2600 | 0 |
| CA_n260(7A-O) | - | CA_n260(7A) |  |  |  |  |  |  | CA_n260O |  |  |  |  | 2950 | 0 |
| CA_n260(8A-O) | - | CA_n260(8A) |  |  |  |  |  |  |  | CA_n260O |  |  |  | 2950 | 0 |
| CA_n260(4A-2O) | - | CA_n260(4A) |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(4A-2Q) | - | CA_n260(4A) |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(3A-3O) | - | CA_n260(3A) |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(A-G-O) | - | n260A | CA_n260G | CA_n260O |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(A-G-2O) | - | n260A | CA_n260G | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(2A-G-O) | - | CA_n260(2A) |  | CA_n260G | CA_n260O |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(A-2G-O) | - | n260A | CA_n260(2G) |  | CA_n260O |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(A-3O) | - | n260A | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(3A-O) | - | CA_n260(3A) |  |  | CA_n260O |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(3A-O-P) | CA_n260O CA_n260P | CA_n260(3A) |  |  | CA_n260O | CA_n260P |  |  |  |  |  |  |  | 1700 | 0 |
| CA_n260(A-4O) | - | n260A | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-4O) | - | CA_n260(2A) |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(3A-4O) | - | CA_n260(3A) |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(4A-4O) | - | CA_n260(4A) |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(5A-4O) | - | CA_n260(5A) |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  | 2800 | 0 |
| CA_n260(A-P) | - | n260A | CA_n260P |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n260(A-3P) | - | n260A | CA_n260(3P) |  |  |  |  |  |  |  |  |  |  | 1300 | 0 |
| CA_n260(A-4P) | - | n260A | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(A-P-Q) | CA_n260P CA_n260Q | n260A | CA_n260P | CA_n260Q |  |  |  |  |  |  |  |  |  | 1100 | 0 |
| CA_n260(2A-P) | - | CA_n260(2A) |  | CA_n260P |  |  |  |  |  |  |  |  |  | 1100 | 0 |
| CA_n260(3A-P) | - | CA_n260(3A) |  |  | CA_n260P |  |  |  |  |  |  |  |  | 1500 | 0 |
| CA_n260(4A-P) | - | CA_n260(4A) |  |  |  | CA_n260P |  |  |  |  |  |  |  | 1900 | 0 |
| CA_n260(5A-P) | - | CA_n260(5A) |  |  |  |  | CA_n260P |  |  |  |  |  |  | 2300 | 0 |
| CA_n260(6A-P) | - | CA_n260(6A) |  |  |  |  |  | CA_n260P |  |  |  |  |  | 2700 | 0 |
| CA_n260(A-2P) | - | n260A | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(2A-2P) | - | CA_n260(2A) |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(2A-3P) | - | CA_n260(2A) |  | CA_n260(3P) |  |  |  |  |  |  |  |  |  | 1700 | 0 |
| CA_n260(2A-4P) | - | CA_n260(2A) |  | CA_n260(4P) |  |  |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(3A-2P) | - | CA_n260(3A) |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(4A-2P) | - | CA_n260(4A) |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  | 2200 | 0 |
| CA_n260(5A-2P) | - | CA_n260(5A) |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  | 2600 | 0 |
| CA_n260(5A-2O) | - | CA_n260(5A) |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(6A-2O) | - | CA_n260(6A) |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  | 2800 | 0 |
| CA_n260(5A-3O) | - | CA_n260(5A) |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  | 2600 | 0 |
| CA_n260(6A-3O) | - | CA_n260(6A) |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  | 2950 | 0 |
| CA_n260(7A-2O) | - | CA_n260(7A) |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  | 2950 | 0 |
| CA_n260(7A-3O) | - | CA_n260(7A) |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  | 2950 | 0 |
| CA_n260(6A-2P) | - | CA_n260(6A) |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  | 2950 | 0 |
| CA_n260(8A-2O) | - | CA_n260(8A) |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  | 2550 | 0 |
| CA_n260(A-Q) | - | n260A | CA_n260Q |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(A-2Q) | - | n260A | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-Q) | - | CA_n260(2A) |  | CA_n260Q |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-2Q) | - | CA_n260(2A) |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(3A-Q) | - | CA_n260(3A) |  |  | CA_n260Q |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(3A-2Q) | - | CA_n260(3A) |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(4A-Q) | - | CA_n260(4A) |  |  |  | CA_n260Q |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(D-2G) | - | CA_n260D | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2D-O) | - | CA_n260(2D) |  | CA_n260O |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(D-2O) | - | CA_n260D | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(A-I) | CA_n260I | n260A | CA_n260I |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(D-G) | CA_n260D CA_n260G | CA_n260D | CA_n260G |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(D-H) | CA_n260D CA_n260H | CA_n260D | CA_n260H |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n260(D-I) | CA_n260D CA_n260I | CA_n260D | CA_n260I |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(D-O) | CA_n260D CA_n260O | CA_n260D | CA_n260O |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(D-P) | CA_n260D CA_n260P | CA_n260D | CA_n260P |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n260(D-Q) | CA_n260D CA_n260Q | CA_n260D | CA_n260Q |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(E-O) | CA_n260E CA_n260O | CA_n260O | CA_n260E |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(E-P) | CA_n260E CA_n260P | CA_n260E | CA_n260P |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(E-Q) | CA_n260E CA_n260Q | CA_n260E | CA_n260Q |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(G-H) | CA_n260G CA_n260H | CA_n260G | CA_n260H |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
| CA_n260(G-I) | CA_n260G CA_n260I | CA_n260G | CA_n260I |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(G-O) | - | CA_n260G | CA_n260O |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n260(G-2O) | - | CA_n260G | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(2G-O) | - | CA_n260(2G) |  | CA_n260O |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(2G-2O) | - | CA_n260(2G) |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(G-3O) | - | CA_n260G | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(3G-O) | - | CA_n260(3G) |  |  | CA_n260O |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2G-3O) | - | CA_n260(2G) |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(G-4O) | - | CA_n260G | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(2G-4O) | - | CA_n260(2G) |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(4G-O) | - | CA_n260(4G) |  |  |  | CA_n260O |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(H-O) | - | CA_n260H | CA_n260O |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
| CA_n260(2H-O) | - | CA_n260(2H) |  | CA_n260O |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(O-2P) | - | CA_n260O | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(O-2Q) | - | CA_n260O | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(O-P) | - | CA_n260O | CA_n260P |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
| CA_n260(2O-P) | - | CA_n260(2O) |  | CA_n260P |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n260(2O-2P) | - | CA_n260(2P) |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(O-Q) | - | CA_n260O | CA_n260Q |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(2O-Q) | - | CA_n260(2O) |  | CA_n260Q |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2O-2Q) | - | CA_n260(2O) |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(P-Q) | - | CA_n260P | CA_n260Q |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-D) | - | n261A | CA_n261D |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2D) | - | n261A | CA_n261(2D) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-D-H) | - | n261A | CA_n261D | CA_n261H |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-D-O) | - | n261A | CA_n261D | CA_n261O |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-D-2O) | - | n261A | CA_n261D | CA_n261(2O) |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-G) | CA_n261G | n261A | CA_n261G |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(A-G-H) | CA_n261G CA_n261H | n261A | CA_n261G | CA_n261H |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-G-I) | CA_n261G CA_n261H CA_n261I | n261A | CA_n261G | CA_n261I |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-G-O) | - | n261A | CA_n261G | CA_n261O |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-G-2O) | - | n261A | CA_n261G | CA_n261(2O) |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2G-O) | - | n261A | CA_n261(2G) |  | CA_n261O |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2G-2O) | - | n261A | CA_n261(2G) |  | CA_n261(2O) |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-3G) | - | n261A | CA_n261(3G) |  |  | CA_n261O |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-3G-O) | - | n261A | CA_n261(3G) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2G) | CA_n261G | n261A | CA_n261(2G) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-4G) | - | n261A | CA_n261(4G) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-H) | CA_n261G CA_n261H | n261A | CA_n261H |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-2H) | - | n261A | CA_n261(2H) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-H-I) | - | n261A | CA_n261H | CA_n261I |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-I) | CA_n261G CA_n261H CA_n261I | n261A | CA_n261I |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2I) | - | n261A | CA_n261(2I) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-J) | CA_n261G CA_n261H CA_n261I | n261A | CA_n261J |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-K) | CA_n261G CA_n261H CA_n261I | n261A | CA_n261K |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-L) | CA_n261A CA_n261G CA_n261H CA_n261I | n261 | CA_n261L |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-O) | - | n261A | CA_n261O |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(A-2O) | - | n261A | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-3O) | - | n261A | CA_n261(3O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-4O) | - | n261A | CA_n261(4O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-5O) | - | n261A | CA_n261(5O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-6O) | - | n261A | CA_n261(6O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-7O) | - | n261A | CA_n261(7O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-P) | - | n261A | CA_n261P |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-2P) | - | n261A | CA_n261(2P) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-Q) | - | n261A | CA_n261Q |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2Q) | - | n261A | CA_n261(2Q) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(2A-G) | CA_n261G | CA_n261(2A) |  | CA_n261G |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(2A-H) | CA_n261G CA_n261H | CA_n261(2A) |  | CA_n261H |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(2A-I) | CA_n261G CA_n261H CA_n261I | CA_n261(2A) |  | CA_n261I |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(3A-G) | CA_n261G | CA_n261(3A) |  |  | CA_n261G |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(D-G) | CA_n261D CA_n261G | CA_n261D | CA_n261G |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(D-H) | CA_n261D CA_n261H | CA_n261D | CA_n261H |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(D-I) | CA_n261D CA_n261I | CA_n261D | CA_n261I |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(D-O) | CA_n261D CA_n261O | CA_n261D | CA_n261O |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(D-2O) | - | CA_n261D | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(D-P) | CA_n261D CA_n261P | CA_n261D | CA_n261P |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(D-Q) | CA_n261D CA_n261Q | CA_n261D | CA_n261Q |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(E-O) | CA_n261E CA_n261O | CA_n261E | CA_n261O |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(E-P) | CA_n261E CA_n261P | CA_n261E | CA_n261P |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(E-Q) | CA_n261E CA_n261Q | CA_n261E | CA_n261Q |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(G-I) | CA_n261G CA_n261H CA_n261I | CA_n261G | CA_n261I |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(G-H) | CA_n261G CA_n261H | CA_n261G | CA_n261H |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
| CA_n261(G-J) | CA_n261A CA_n261G CA_n261H CA_n261I | CA_n261G |  | CA_n261J |  |  |  |  |  |  |  |  |  |  | 0 |
| CA_n261(2G-2O) | - | CA_n261(2G) |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(G-O) | - | CA_n261G | CA_n261O |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n261(G-2O) | - | CA_n261G | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(2G-O) | - | CA_n261(2G) |  | CA_n261O |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(3G-O) | - | CA_n261(3G) |  |  | CA_n261O |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(H-I) | CA_n261G CA_n261H CA_n261I | CA_n261H | CA_n261I |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 4: Configurations for intra-band contiguous CA defined in Table 5.5A.1-1 NOTE 5: Configurations for intra-band non-contiguous CA defined in Table 5.5A.2-1 NOTE 6: Void NOTE 7: Unless otherwise stated, BCS0 is referred in each constituent CA configuration. NOTE 8: (BWChannel,block) denotes the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---