3GPP TSG-RAN4 Meeting #AH-1801	R4-1800034

# San Diego, California USA January 22-26, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.101-1 | CR | CRNum | rev | 0 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Band n71 updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Removes brackets from n71 REFSENS requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removes brackets from n71 REFSENS requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specificaiton retains tentative values for n71 REFSENS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3-1 and Table 7.3-2

Table 7.3-1: Reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | -95.8 | -94.0 | -92.7 |  | -89.6 | -88.6 |  |  |  | TDD |
|  | 30 |  | -96.1 | -94.1 | -92.9 |  | -89.7 | -88.7 | -87.9 | -86.6 | -85.6 |  |
|  | 60 | - | -96.5 | -94.4 | -93.1 |  | -89.9 | -88.8 | -88.0 | -86.7 | -85.7 |  |
| n77 (3.8 to 4.2 GHz) | 15 |  | -95.3 | -93.5 | -92.2 |  | -89.1 | -88.1 |  |  |  | TDD |
|  | 30 |  | -95.6 | -93.6 | -92.4 |  | -89.2 | -88.2 | -87.4 | -86.1 | -85.1 |  |
|  | 60 | - | -96.0 | -93.9 | -92.6 |  | -89.4 | -88.3 | -87.5 | -86.2 | -85.2 |  |
| n78 | 15 |  | -95.8 | -94.0 | -92.7 |  | -89.6 | -88.6 |  |  |  | TDD |
|  | 30 |  | -96.1 | -94.1 | -92.9 |  | -89.7 | -88.7 | -87.9 | -86.6 | -85.6 |  |
|  | 60 |  | -96.5 | -94.4 | -93.1 |  | -89.9 | -88.8 | -88.0 | -86.7 | -85.7 |  |
| n79 |  |  |  |  |  |  | -89.6 | -88.6 |  |  |  | TDD |
|  |  |  |  |  |  |  | -89.7 | -88.7 | -87.9 | -86.6 | -85.6 |  |
|  |  |  |  |  |  |  | -89.9 | -88.8 | -88.0 | -86.7 | -85.7 |  |
| [NOTE 1: Four Rx antenna ports shall be the baseline for this operating band.] NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3-2: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n71 | 15 | 25 | 251 | 201 | 201 | n/a | n/a | n/a | - |  |  | FDD |
|  | 30 |  |  |  |  | n/a | n/a | n/a | n/a | n/a | n/a |  |
|  | 60 |  |  |  |  | n/a | n/a | n/a | n/a | n/a | n/a |  |
|  | 15 |  |  |  |  |  |  |  | - |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  | - |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

< end of changes >