3GPP TSG-RAN WG4 Meeting #AH1807	R4-1808838

# Montreal, Canada, July 2 – 6 , 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction of requirement for BWP switching delay in TS38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | No BWP switching requirement is defined yet in spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce BWP switching delay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement for BWP switching delay is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

8.5	Active BWP switch delay

8.5.1	Introduction

The requirements in this section apply for a UE configured with more than one BWP on any of PCell and activated SCells in standalone NR, or any of PSCell and activated SCells in SCG in EN-DC. UE shall complete the switch of active DL and/or UL BWP within the delay defined in this section.

8.5.2	UE active BWP switch delay

For DCI-based active BWP switch, after UE receives BWP switching request at slot n on a serving cell, UE should be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the serving cell on which BWP switch occurs at slot n+Y.

For timer-based BWP switching, UE should be able to start BWP switch at slot n, which is the beginning of a subframe (FR1) or half-subframe (FR2) immediately after a BWP timer expires on a serving cell and be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the serving cell on which BWP switch occurs at slot n+Y.

Depending on UE capability, UE shall finish BWP switch within the time duration Y defined in Table 8.5.2-1, if the BWP switch involves changing the center frequency of the BWP or changing its BW.

Table 8.5.2-1: BWP switch delay with center frequency or BW changing

|  | NR Slot length (ms) | BWP switch delay Y (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | TBD | TBD |
| 1 | 0.5 | TBD | TBD |
| 2 | 0.25 | TBD | TBD |
| 3 | 0.125 | TBD | TBD |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing in SCS, the BWP switch delay is determined by the lager one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Depending on UE capability, UE shall finish BWP switch within the time duration Y defined in Table 8.5.2-2, if the BWP switch involves changing only the SCS.

Table 8.5.2-2: BWP switch delay without center frequency or BW changing

|  | NR Slot length (ms) | BWP switch delay Y (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | TBD | TBD |
| 1 | 0.5 | TBD | TBD |
| 2 | 0.25 | TBD | TBD |
| 3 | 0.125 | TBD | TBD |
| Note 1: Depends on UE capability. Note 2: The BWP switch delay is determined by the lager one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

----------------------END OF CHANGES----------------------------