3GPP TSG-RAN4 Meeting AH-1807 	R4-1809090

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 37.843 Changes on the document structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-06-25 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TRP grid types and measurement methods are not well organized in the TR structure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change of clause names and introduction of new clauses, together with editorial coorections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TRP methods are not well organized in the TR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## ----start of changes----

## 10.9	TRP

### 10.9.1	TRP methods

The TRP or the radiated power is simply the total power radiated by an antenna/EUT. Due to energy conservation, TRP is independent of choice of test distance, apart from measurement errors.

TRPReference is the total power radiated by the antenna of a transmitting device, when non-idealities such as mismatch and losses in the antenna are taken into account. In theory, TRPReference is defined as the integral of the antenna’s far field radiation pattern over a spherical surface, that is

TRPReference= θ=0πϕ=02πUθ,ϕ sinθdθdϕ

where U(θ,ϕ) is the radiation intensity at each angle in watts per steradian. In the far field, the radiation intensity can be defined as

Uθ,ϕ=EIRP(θ,ϕ)4π

Thus the definite integral for TRPReference becomes

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

In practice, discrete samples of EIRP are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral.  The obtained value, TRPEstimate , is an approximation of TRPReference and the difference between then is defined as the TRP systematic error.

TRPsyserr = | TRPEstimate – TRPreference |   (in dB)

When measuring radiated power, at each measurement point, two partial results for two orthogonal polarizations needs to be added. These can be the θ (theta) and φ (phi) polarizations or any pair of orthogonal polarizations.

Although the power measurements can be performed at distances less than 2d2/λ (the traditional far-field criteria) in following subclauses we describe several spherical sampling grids only assuming EIRP measurements. In practice, measurements at shorter distance will be performed only when far-field condition leads to unpracticable large measurement distance (e.g. in the higher part of the spurious emissions domain).

The distribution of sampling points on the spherical surface depends the type of sampling grids applied. In following subclauses, several spherical sampling grids are described.

### 10.9.1.1	Angular sampling on spherical surface

### 10.9.1.1.1	Spherical equal angle grids

#### 10.9.1.1.1.1.1 	General

With the spherical equal angle grid, the grid spacing is uniform in the θ and ϕ directions. The range of θ angles from 0 to π is divided into N equally spaced subintervals and the range of ϕ angles from 0 to 2π is divided into M equally spaced subintervals. The width of each subinterval in the θ- and ϕ-angle is given as

Δθ= πN  and ∆ϕ= 2πM

The total number of angular sampling points is equal to (N-1)×M.

Let n and m be the indices used to denote the nth θ and mth ϕ angles, respectively. In pactice, discrete samples of EIRP are measured at each sample point (θn, ϕm) by measuring its two orthogonally polarized components, EIRPp1(θn,ϕm) and EIRPp2(θn,ϕm). The EIRP sample are then used to approximate the definite integral for TRPReference as

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn.

The total number of angular sampling points is equal to (N-1)×M.

Note TRPEstimate = TRPReference as N and M approach ∞.

There is a trade-off between the accuracy of the TRPEstimate and the total number of sampling points. A large number of sampling points leads to long measurement time. Thus it is important to achieve short measurement time and fulfilling the minimum TRP systematic error. Subclauses 10.9.3.1 and 10.9.3.2 outline the criteria for determining the minimum number of sampling points to characterize. Other means for set the number of sampling points are not precluded.

For each emission frequency, the reference angular steps   Δϕ and Δθ in radians are calculated as:

Δϕref=λ/Dcyl

Δθref=λ/D

, where D and Dcyl are defined in subclause 10.9.2.1.

According to [x] and [y] using a grid with reference angular step is enough for accurate error-free TRP estimation. The reference angular steps are dependent only on the electrical size of the radiation source (i.e., physical size and the emission frequency) and is not affected by the correlation level. When sampling with the reference angular step, fine details of the radiation pattern are not observed, but the TRP value is not affected.

The equal angle grid points are not uniformly distributed on the sphere but are clustered towards the poles as shown in Figure 10.9.2-1.

Figure 10.9.2.1-1: Spherical equal angle sampling grid

#### 10.9.1.1.1.2	Spatial sampling criteria

Figure 10.9.2.2-1: the dimensions of a DUT are depth (d), width (w) and height (h)

The spherical and cylindrical diameters are calculated as

D=d2+w2+h2

Dcyl=d2+w2

Some basic definitions and relations are given here for readability.

#### 10.9.1.1.1.3	Rayleigh sampling criteria

For the ULA system, the array spatial pattern could be defined as in the following equation.

Where spatial frequency  is defined as following:

Similar to Nyquist sampling in the time domain signal, the Rayleigh resolution for spatial domain signal to avoid the aliasing can be derived as

If DUT is mounted along the yz plane as shown in Figure 10.9.x.x-1, based on the above considerations on the Rayleigh resolution for spatial domain signal, then subinterval in the  φ and θ angle is calculated as:

Where is diameter of radiated  parts of antenna along  y-axis,  is diameter of radiated parts of antenna along the z-axis and  is wavelength for the measured frequency.

Figure 10.9.2.3-1. Spherical coordinate for OTA conformance testing of DUT

In the NR coexistence study, it was assumed that antenna configuration for WA NR BS is 8x16 with two polarization. If DUT mounted along yz plane with antenna configuration 16x8 where 16 column are assumed along the y-axis and 8 rows are assumed along the z-axis. Antenna elements are uniformly distributed with separation distance λ/2, therefore aperture size Dy≈8λ and Dz≈4λ. The uniform sampling in the spherical coordinate for this approach is demonstrated in the Figure 10.9.2.3-2.

Figure 10.9.2.3-2. uniform sampling in the spherical coordinate, Red cross denotes the sampling point.

### 10.9.1.1.2	Spherical equal area grids

With the spherical equal area sampling grid, the spherical surface is partitioned into N equal area regions. Let n be the index for the nth region and there is one point (θn,ϕn) located in the center of each region. The definite integral for TRPReference can be approximated as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

The total number angular sampling points is N. Unlike the spherical equal angle grid, the TRPEstimate equation is not weighted by sin θ. As shown in Figure 10.9.3-1, the equal area grid points are distributed uniformly on the sphere but the pattern of θ and ϕ angles is irregular.

Figure 10.9.3-1: Spherical equal area sampling grid

The Fibonacci grid points are arranged along a generative spiral on the spherical surface. Similar to the equal area sampling grid, the Fibonacci grid generates points that are uniformly spaced in an isotropic way. Assume there are I points in the Fibonacci sampling grid, then the definite integral for TRPReference can be approximated as

TRPEstimate=1Ii=1IEIRP(θi,ϕi)

where i = 0 .. I-1

θi=cos-11+2i+1I  and

ϕi=2πiΨ,	where 	Ψ=1+52

The total number of angular sampling points is I. Like the spherical equal area grid, the TRPEstimate equation is not weighted by sin θ.

Figure 10.9.4-1: Spherical Fibonacci sampling grid

### 10.9.1.1.3	Orthogonal cuts grids

Compared to the TX spurious emissions the OBUE emissions are likely to experience the similar beamforming pattern as the main beam. Due to this reason, it is easily predictable where the maximum of the emissions is going to be, hence the possibility to enhance the measurement method to achieve better accuracy. Here we choose to apply the pattern multiplication method [ref].

In this method, at least two cuts (default) shall be used, an optional third cut can be added if needed. The alignment of the cuts must be along the symmetry planes of the antenna array.

The first mandatory cut is a horizontal cut passing including the peak direction of the main beam.

The second mandatory is a vertical cut passing including the peak direction of the main beam.

Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

The third optional cut is a vertical cut orthogonal to the first and the second cut.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

TRPcut-n=1Pj=1PEIRP(j)

where P is the number of smpling point. Here, overlining denotes angular average value in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NTRPcut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

| Figure 10.9.5-1: Example of orthogonal cuts geometry when the main lobe points along the x-axis. Two mandatory cuts grid (left) and the optional added third cut (right). The first two cuts are generated by rotating the EUT around its z-axis and y-axis, respectively, and the optional third cut is generated by rotating the EUT around its x-axis. |  |
| --- | --- |

Two cuts cut data gives a conservative TRP estimate (an overestimation of the real TRP). Through patter multiplication a less conservative estimate is obtained, based on the calculation of the antenna array factor as a product of two terms, corresponding to the two cuts.

The following conditions for being able to apply pattern multiplication method are mandatory:

    - The vertical cut is taken in the xz -plane

    - The frequency of the emission is within the BS RF Bandwidth or close to its edges (e.g. emissions will experience similar pattern as the wanted signal)

    - The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

    - The emission changes significantly when the Tx power is turned on or off.

    - The antenna arrays of the EUT

      - Have rectangular grids of antenna element positions

      - Have symmetry planes that are vertical and horizontal.

      - Have parallel antenna planes

### 10.9.1.1.4	Wave vector space sampling grid

Similar as Rayleigh sampling approach, DUT is placed on the yz plane in the spherical coordinate and normal vector of DUT is pointing along the x-axis as shown in Figure 10.9.x.x-1. The angle φ and θ represent azimuth and elevation respectively, Ky and Kz  represent the projection of normalized wave vector on y-axis and z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be  represented as following:

In the existing TR37.843, TRP is defined in the spherical coordinate as following:

As TRP is defined in the wave vector coordinate, therefore TRP definition should be revised accordingly in the corresponding coordinate. For the TRP definition in normalized wave vector space, according to the 2D Jacobian transformation, the above equation  could be adjusted as following, namely:

Based on the above two equations, then we could get

where relationship between (θ,φ) and (Ky,Kz )is demonstrated in the equation before. Similar as discrete sampling process in TR37.843,  the above equation is approximated in the far-field region as the sum of the total EIRP at a number of discrete directions as follows:

The above considerations could be applied for both polarization.

Uniform sampling on the yz plane in the wave vector coordinate as shown in Figure 10.9.6-1:

- Rayleigh resolution in y-axis:

- Rayleigh resolution in z-axis:

Where is diameter of radiated  parts of antenna along  y-axis,  is diameter of radiated parts of antenna along the z-axis.

Based on the uniform sampling grid on the yz plane, we could get the sampling point (Kym,Kzn ). in addition, according to the transformation between (Kym,Kzn ) and (φ,θ ), then azimuth and elevation (φm,n, θn) in the spherical coordinate could be derived correspondingly. Based on the (φm,n, θn) in the spherical coordinate, EIRP on the spherical coordinate could be measured. In the Figure 10.9.6-1 and 10.9.6-2, we demonstrate the sampling point in the wave vector space and spherical coordinate, it could be easily found that the sampling grid is not uniform in the spherical coordinate, that’s the reason why the second approach is called Non-uniform sampling.

Figure10.9.6-1. 2D unfolding diagram of  simulated 3D antenna pattern in normalized wave vector space and (b) sampling point in this space where Red cross denote the sampling point

Figure 10.9.6-2.  non-uniform sampling in the spherical coordinate where Red cross denotes the sampling point.

10.9.2 TRP for wanted signal

10.9.3 TRP for OBUE

10.9.4 TRP for ACLR

10.9.5 TRP for Spurious emissions

## ----endt of changes----