3GPP TSG-RAN5 Meeting #71	R5-163099

# Nanjing, China, 23rd -27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34.123-1 |  | CR |  |  | 3855 |  | rev | 1 |  | Current version: |  |  |  |  | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. Proposed change affects:; UICC apps; ME; Radio Access Network; Core Network. |
| Title: |  | Correction to UTRA SS-NITZ test cases 15.7.x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  | KeySight Technologies UK, Intel Corporation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | R5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | TEI8_Test |  |  |  |  |  |  |  |  |  |  | Date: |  | 2016-05-23 |  |  |
| Category: |  | F |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-12 |  |  |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |  |  |
| Reason for change: |  |  | In the SS-NITZ testcases the CC facility message (MT) and Status Enquiry MT messages are reaching almost instantaneously at UE side. This can cause a race condition at the UE, causing the UE to transmit the old status thus failing the test case. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | Added a delay of 500ms in between CC Facility MT message and Status Enquiry MT message. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | A conformant UE may fail the TC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 15.7.1, 15.7.2, 15.7.4, 15.7.5, 15.7.7, 15.7.8, 15.7.13, 15.7.14, 15.7.27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | x | Other core specifications |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  | x | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  | Associated TTCN CR R5s160356 R5-162384r1-> Reduced the delay to 100 ms based on R&S comments R5-162384r1->R5-163099 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Start of Modified Section >

15.7.1.4	Method of test

[Text skipped here here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  | Make the UE to join Call A-B and Call A-C |  |
| 2 | --> |  | FACILITY | BuildMPTY Invoke component |
| 3 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "MPTY request, call held" |
| 5 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "MPTY request " |
| 7 | <-- |  | FACILITY | Return Result component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY" |
| 10 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |
| 12 | SS |  |  | The DTCH is through connected in both directions. |

< End of Modified Section >

< Start of Modified Section >

15.7.2.4	Method of test

[Text skipped here here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  | Make the UE to join Call A-B and Call A-C |  |
| 2 | --> |  | FACILITY | BuildMPTY Invoke component |
| 3 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "MPTY request, call held" |
| 5 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "MPTY request" |
| 7 | <-- |  | FACILITY | Return Error component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call held" |
| 10 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, no auxiliary state |
| 12 | UE |  | Make the UE to join Call A-B and Call A-C |  |
| 13 | --> |  | FACILITY | BuildMPTY Invoke component |
| 14 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 15 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "MPTY request, call held" |
| 16 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 17 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "MPTY request" |
| 18 | <-- |  | FACILITY | Reject component |
| 18a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 19 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 20 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call held" |
| 21 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 22 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, no auxiliary state |

< End of Modified Section >

< Start of Modified Section >

15.7.4.4	Method of test

[Text skipped here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  | Make the UE to put the MultiParty call on hold |  |
| 2 | --> |  | FACILITY | HoldMPTY Invoke component |
| 3 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, hold request" |
| 5 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, hold request" |
| 7 | <-- |  | FACILITY | Return Result component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, call held" |
| 10 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, call held" |

< End of Modified Section >

< Start of Modified Section >

15.7.5.4	Method of test

[Text skipped here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  | Make the UE to put the MultiParty call on hold |  |
| 2 | --> |  | FACILITY | HoldMPTY Invoke component |
| 3 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, hold request" |
| 5 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, hold request" |
| 7 | <-- |  | FACILITY | Return Error component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY" |
| 10 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |
| 12 | UE |  | Make the UE to put the MultiParty call on hold |  |
| 13 | --> |  | FACILITY | HoldMPTY Invoke component |
| 14 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 15 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, hold request" |
| 16 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 17 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, hold request" |
| 18 | <-- |  | FACILITY | Reject component |
| 18a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 19 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 20 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY" |
| 21 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 22 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |

< End of Modified Section >

< Start of Modified Section >

15.7.7.4	Method of test

[Text skipped here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | Using MMI or AT commands, request a private communication with Call A-B |
| 2 | -> |  | FACILITY | TI for Call A-B, SplitMPTY Invoke component |
| 3 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | -> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Split request" |
| 5 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | -> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |
| 7 | <- |  | FACILITY | Return Result component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | -> |  | STATUS | Transaction identifier of Call A-B, state U10, no auxiliary state |
| 10 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | -> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call held" |
| 12 | SS |  |  | Check that the TCH is through-connected |

< End of Modified Section >

< Start of Modified Section >

15.7.8.4	Method of test

[Text skipped here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | Using MMI or AT commands, create a private communication for Call A-B |
| 2 | -> |  | FACILITY | TI for Call A-B, SplitMPTY Invoke component |
| 3 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | -> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Split request" |
| 5 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | -> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |
| 7 | <- |  | FACILITY | Return Error component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | -> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY" |
| 10 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | -> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |
| 12 | UE |  |  | Using MMI or AT commands, create a private communication for Call A-B |
| 13 | -> |  | FACILITY | TI for Call A-B, SplitMPTY Invoke component |
| 14 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 15 | -> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Split request" |
| 16 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 17 | -> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |
| 18 | <- |  | FACILITY | Reject component |
| 18a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 19 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 20 | -> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY" |
| 21 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 22 | -> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |

< End of Modified Section >

< Start of Modified Section >

15.7.13.4	Method of test

[Text skipped here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  | Make the UE to retrieve the held MultiParty call |  |
| 2 | --> |  | FACILITY | RetrieveMPTY Invoke component |
| 3 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, retrieve request" |
| 5 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, retrieve request" |
| 7 | <-- |  | FACILITY | Return Result component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY" |
| 10 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |

< End of Modified Section >

< Start of Modified Section >

15.7.14.4	Method of test

[Text skipped here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  | Make the UE to retrieve the held MultiParty call |  |
| 2 | --> |  | FACILITY | RetrieveMPTY Invoke component |
| 3 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 4 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, retrieve request" |
| 5 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 6 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, retrieve request" |
| 7 | <-- |  | FACILITY | Return Error component |
| 7a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 8 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 9 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, call held" |
| 10 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 11 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, call held" |
| 12 | UE |  | Make the UE to retrieve the held MultiParty call |  |
| 13 | --> |  | FACILITY | RetrieveMPTY Invoke component |
| 14 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 15 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, retrieve request" |
| 16 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 17 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, retrieve request" |
| 18 | <-- |  | FACILITY | Reject component |
| 18a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 19 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 20 | --> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY, call held" |
| 21 | <-- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 22 | --> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY, call held" |

< End of Modified Section >

< Start of Modified Section >

15.7.27.4	Method of test

[Text skipped here]

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | Using MMI or AT commands, put the MultiParty call on Hold to establish a new call |
| 2 | -> |  | FACILITY | TI for Call A-B or call A-C, HoldMPTY Invoke component |
| 3 | <- |  | FACILITY | Same TI as step 2, Return Result component |
| 3A | UE |  |  | Using MMI or AT commands, new active call is added to the MultiParty call |
| 3B | -> |  | CM SERVICE REQUEST |  |
| 3C |  |  | CM SERVICE ACCEPT |  |
| 4 | -> |  | SETUP | TI different from Call A-B or Call A-C (new Call A-D) |
| 5 | <- |  | ALERTING | Transaction identifier of Call A-D |
| 6 | <- |  | CONNECT | Transaction identifier of Call A-D |
| 7 | -> |  | CONNECT ACKNOWLEDGE | Transaction identifier of Call A-D |
| 7A | UE |  |  | Using MMI or AT commands, add the ‘Multiparty call on Hold’ to the conversation |
| 8 | -> |  | FACILITY | TI for Call A-B, Call A-C or call A-D, BuildMPTY Invoke component |
| 9 | <- |  | FACILITY | TI same as step 8, Return Result component |
| 9a |  |  |  | Wait for 100ms to allow UE to receive FACILITY message and act upon it |
| 10 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-B |
| 11 | -> |  | STATUS | Transaction identifier of Call A-B, state U10, auxiliary state "Call in MPTY" |
| 12 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-C |
| 13 | -> |  | STATUS | Transaction identifier of Call A-C, state U10, auxiliary state "Call in MPTY" |
| 14 | <- |  | STATUS ENQUIRY | Transaction identifier of Call A-D |
| 15 | -> |  | STATUS | Transaction identifier of Call A-D, state U10, auxiliary state "Call in MPTY" |
| 16 | SS |  |  | Check that the TCH is through-connected |

< End of Modified Section >