3GPP TSG-RAN5 Meeting #81	R5-187279

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0063 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of 7.8B.2.3 wideband intermodulation for inter-band EN-DC within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Test case 7.8B.2.3 wideband intermodulation for inter-band EN-DC within FR1 is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Chapter structures under 7.8 is added. Test case 7.8B.2.3 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 7.8B.2.3 remains missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

##### 7.5B.3.5	Test requirement

For the NR CC, the throughput measurement of the carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 under the conditions specified in clause 7.5.5 in TS 38.521-1 [8]

## 7.8	Intermodulation characteristics

## 7.8B	Intermodulation characteristics for EN-DC in FR1

### 7.8B.1	General

### 7.8B.2	Wide band Intermodulation

#### 7.8B.2.1	Intra-band contiguous EN-DC in FR1

TBD.

#### 7.8B.2.2	Intra-band non-contiguous EN-DC in FR1

TBD.

#### 7.8B.2.3	Inter-band EN-DC in FR1

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Modulated interferer details are TBD in 38.101-1

-	UL Power window is TBD

-	MU and TT missing in Annex F

##### 7.8B.2.3.1	Test Purpose

Same test purpose as in clause 7.8.2.1 in TS 38.521-1 [8] for the NR carrier.

##### 7.8B.2.3.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1.

##### 7.8B.2.3.3	Minimum Conformance Requirements

Same minimum conformance requirements as in clause 7.8.2 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.8B.2.3.

##### 7.8B.2.3.4	Test Description

Same test description as in clause 7.8.2.4  in TS 38.521-1 [8] for the NR carrier with the following exceptions:

For Initial conditions as in clause 7.8.2.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] subclause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-2.

3.1.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C0, C.1 and C.3., and uplink signals according to Annex H.1 and H.3.1.

4.1.	The UL Reference Measurement channels are set according to Table 4.6-2.

Step 6 of Initial conditions as in clause 7.8.2.4.2 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

###### 7.8B.2.3.5	Test Requirement

Same test requirement as in clause 7.8.2.5 in TS 38.521-1 [8].

<<< END OF CHANGES >>>