3GPP TSG-RAN5 Meeting #91-e	R5-212129

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1351 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NB-IoT Common contents of system information messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | No test scenario is using additionalTransmissionSIB1 in current planned NB_IOTenh2-UEConTest test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the default configuration of additionalTransmissionSIB1-r15 in MIB to FALSE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration and TTCN implementation will be uncessarily complex. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.4.3.2	NB-IoT Common contents of system information messages

-	MasterInformationBlock-NB

The MasterInformationBlock-NB includes the system information transmitted on BCH.

Table 8.1.4.3.2-1: MasterInformationBlock-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r13 | 2 | TBS = 208 bits, 16 repetitions |  |
| systemInfoValueTag-r13 | 0 |  |  |
| ab-Enabled-r13 | FALSE |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-SamePCI-r13 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r13 | 23 | As specified in TS 36.213 [29] Clause 16.8 | PRB30 |
|  | 24 | As specified in TS 36.213 [29] Clause 16.8 | PRB35 |
|  | 7 | As specified in TS 36.213 [29] Clause 16.8 | PRB17 |
|  | 8 | As specified in TS 36.213 [29] Clause 16.8 | PRB22 |
| } |  |  |  |
| inband-DifferentPCI-r13 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r13 | same | same number of ports as NRS |  |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ’00’B |  |  |
| } |  |  |  |
| guardband-r13 SEQUENCE { |  |  | Guardband |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ‘000’B |  |  |
| } |  |  |  |
| standalone-r13 SEQUENCE { |  |  | Standalone |
| spare | ‘0000 0’B |  |  |
| } |  |  |  |
| additionalTransmissionSIB1-r15 | FALSE | A Rel-13/Rel-14 UE will interpret FALSE as spare ‘0’ |  |
| additionalTransmissionSIB1-r15 | FALSE |  | Rel-15 |
| spare | ‘0000 0000 00’B |  | Rel-15 |
| } |  |  |  |
| } |  |  |  |

| Condition |  | Explanation |  |
| --- | --- | --- | --- |
| Inband-Same |  | In-band with same PCI test environment |  |
| Inband-Different |  | In-band with different PCI test environment |  |
| PRB30 |  | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 30 |  |
| PRB35 |  | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 35 |  |
| PRB17 |  | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 17 |  |
| PRB22 |  | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 22 |  |
| Rel-15 |  | For NB-IoT test cases applicable in TS 36.523-2 [19] from Rel-15 or later |  |
| Standalone |  | Standalone test environment |  |
| Guardband |  | Guard-band test environment |  |