3GPP TSG-RAN5 Meeting #92-e	R5-215186

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5343 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to reference sensitivity test case for LTE band 24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | new NS value of NS_56 and additional Tx-Rx spacing were introduced for LTE band 24 in TS 36.101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating conformance and test requirements for the reference sensitivity test case to include NS_56 and adding message content exceptions for LTE Band 24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance and test requirements will not reflect the recent updates for LTE Band 24 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3, 7.3.4.1, 7.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Rel-17 WIC to trigger spec. upgrade |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.3	Minimum conformance requirements

<<Unchanged text skipped>>

Unless given by Table 7.3.3-3, the minimum requirements specified in Tables 7.3.3-1, 7.3.3-1a and 7.3.3-2 shall be verified with the network signalling value NS_01 (Table 6.2.4.3-1) configured.

Table 7.3.3-3: Network signalling value for reference sensitivity

| E-UTRA Band | Network Signalling value |
| --- | --- |
| 2 | NS_03 |
| 4 | NS_03 |
| 10 | NS_03 |
| 12 | NS_06 |
| 13 | NS_06 |
| 14 | NS_06 |
| 17 | NS_06 |
| 19 | NS_08 |
| 21 | NS_09 |
| 23 | NS_03 |
| 24 | NS_56 |
| 25 | NS_03 |
| 30 | NS_21 |
| 48 | NS_27 |
| 53 | NS_45 |
| 66 | NS_03 |
| 70 | NS_03 |
| 71 | NS_35 |
| 85 | NS_06 |

The normative reference for this requirement is TS 36.101 [2] clause 7.3.1.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 7.3.4	Test description

#### 7.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 7.3.4.1-1 and 7.3.4.1-2. The details of the downlink and uplink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | QPSK | 6 | 6 | QPSK | 6 | 6 |
| 3MHz | QPSK | 15 | 15 | QPSK | 15 | 15 |
| 5MHz | QPSK | 25 | 25 | QPSK | 25 | 25 |
| 5MHz | QPSK | 25 | N/A | QPSK | 20 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 15 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 54 | N/A |
| 10MHz | QPSK | 50 | 50 | QPSK | 50 | 50 |
| 10MHz | QPSK | 50 | N/A | QPSK | 25 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 20 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 15 | N/A |
| 15MHz | QPSK | 75 | 75 | QPSK | 75 | 75 |
| 15MHz | QPSK | 75 | N/A | QPSK | 50 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 25 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 20 | N/A |
| 20MHz | QPSK | 100 | 100 | QPSK | 100 | 100 |
| 20MHz | QPSK | 100 | N/A | QPSK | 75 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 50 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 25 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 20 | N/A |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2. Depending on E-UTRA band, only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per Test Channel Bandwidth. The allocation shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). Note 3: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. Note 4: Applicable only to E-UTRA FDD Bands 31 and 72. The UL resource blocks shall be located at RBstart 10 (according to Table 7.3.3-2). Note 5: For E-UTRA FDD Band 24, above configurations be used to verify reference sensitivity for both Tx-Rx frequency separations specified in Table 5.3-1 |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 7.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions.

<<Unchanged text skipped>>

7.3.4.3.9	Message contents exceptions (network signalled value "NS_35")

1. Information element additionalSpectrumEmission is set to NS_35. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.4.3.9-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_35"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 32 | When IE AdditionalSpectrumEmission - v10l0 is used, 32 should be set to IE AdditionalSpectrumEmission. |  |
| AdditionalSpectrumEmission-v10l0 | 35 (NS_35) |  |  |

7.3.4.3.10	Message contents exceptions for Band 24

1. Information element additionalSpectrumEmission is set to NS_56. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.4.3.10-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 56 (NS_56) |  |  |

2. Information element ul-CarrierFreq-v9e0 in the SystemInformationblockType2 as part of the cell broadcast message shalll be used to provide the EARFCN of the uplink frequency according to the Tx-Rx spacing being used for the test. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.4.3.10-2: SystemInformationBlockType2: UL carrier frequency test requirement for two Tx-Rx Spacings

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ul-CarrierFreq-v9e0 | Per tables in 4.3.1.1.24 |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------