3GPP TSG-RAN5 Meeting #93-e	R5-216489

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2473 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 5GC test case 9.1.5.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | "Non-suitable cell" may cause calibration issue on FR2, which can be avoided by replacing with "Non-Suitable “Off” cell". |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Replacing "Non-suitable cell" with "Non-Suitable “Off” cell" 2. Editorial updating |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.5.1.8	Initial registration / Rejected / Serving network not authorized

9.1.5.1.8.1	Test Purpose (TP)

(1)

with { The UE has sent initial REGISTRAION REQUEST message }

ensure that {

when { the UE receives a REGISTRATION REJECT with cause #73 (Serving network not authorized) }

then { the UE stores the PLMN identity in the "forbidden PLMN list" and does not attempt to register on a cell belong to that PLMN }

}

9.1.5.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.2.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3	(Illegal UE); or

#6	(Illegal ME).

….

#73	(Serving network not authorized).

The UE shall set the 5GS update status to 5U2 NOT UPDATED, reset the registration attempt counter, store the PLMN identity in the "forbidden PLMN list" and enter state 5GMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [5].

9.1.5.1.8.3	Test description

9.1.5.1.8.3.1	Pre-test conditions

System Simulator:

-	3 NGC Cells NGC Cell E, NGC Cell I and NGC Cell A are configured as specified in TS 38.508-1 [4] table 6.3.2.2-1 and table 6.3.2.2-3.

UE:

-	The UE is in Automatic PLMN selection mode.

Preamble:

-	NGC Cell E is set to “Serving Cell”.

-	NGC Cell I is set to “Non-suitable “Off” cell”.

-	NGC Cell A is set to “Non-suitable “Off” cell”.

-	The UE is in state 0N-B with a successful registration on NGC Cell E according to 38.508-1 [4].

9.1.5.1.8.3.2	Test procedure sequence

Table 9.1.5.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on | - | - | - | - |
| 2-9 | Steps 2-9 of Table 4.5.2.2-2 in TS38.508-1 [4] are performed on NGC Cell E. | - | - | - | - |
| 10 | The SS transmits a REGISTRATION REJECT with cause #73 (Serving network not authorized). | <-- | REGISTRATION REJECT | - | - |
| 11 | The SS releases the RRC connection | - | - | - | - |
| 11A | The SS configures - NGC Cell I as “Suitable neighbour cell” | - | - | - | - |
| 12 | Check: Does the UE send in the next 30 sec a request for RRC connection establishment on Cell E or Cell I. | --> | NR RRC: RRCSetupRequest | 1 | F |
| 13 | The SS configures - NGC Cell A as “Serving Cell” | - | - | - | - |
| 14 | Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2? (Note 1) | - | - | 1 | P |
| 14A | Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. | - | - | - | - |
| 15 | The SS configures - NGC Cell A as “Non-suitable "Off" cell” - NGC Cell I as “Non-suitable "Off" cell” | - | - | - | - |
| 15A | The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on. | - | - | - | - |
| 16 | Set the UE in manual PLMN selection mode or request a PLMN search. | - | - | - | - |
| 17 | The user selects the PLMN of NGC Cell E. | - | - | - | - |
| 18 | The UE performs Registration procedure on NGC Cell E as specified in TS 38.508-1 [4] subclause 4.5.2. | - | - | - | - |
| 19 | Set the UE in Automatic PLMN selection mode. | - | - | - | - |
| 20 | Wait for 300s to allow the UE to switch to automatic PLMN selection mode. | - | - | - | - |
| Note 1: For Rel-16 or later Releases UE, the REGISTRATION REQUEST message shall be sent without integrity protection. |  |  |  |  |  |

9.1.5.1.8.3.3	Specific message contents

Table 9.1.5.1.8.3.3-1: REGISTRATION REJECT (step 10 Table 9.1.5.1.8.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM cause |  | ‘01001001’B | Cause #73 (Serving network not authorized) |  |