3GPP TSG-RAN5 Meeting #93-e	R5-217366

# Electronic Meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2140 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default message for 16Tx PMI reporting test cases in 5.4.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Several 16Tx PMI reporting test cases are specified by RAN4 and RAN5. Some commont messages could be specified in 38.508-1 to avoid redundant sections through test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding default configuration for 16Tx PMI reporting test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message need to be repeated added in each test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.4.2.5	Message contents for PMI reporting requirements

NZP-CSI-RS for Tracking

CSI-RS-ResourceMapping

Table 5.4.2.5-1: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 | TRS |
| cdm-Type | noCDM |  | TRS |
| density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

NZP CSI-RS for CSI Acquisition

CSI-RS-ResourceMapping

Table 5.4.2.5-2: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row3 | 001000 | K0 = 6, row3 | 2Tx |
| row4 | 001 | K0 = 0, row4 | 4Tx |
| other | 001100 | K0 = 4, K1 = 6, row8 | 8Tx |
|  | 011110 | K0=2, row 12 | 16Tx |
| } |  |  |  |
| nrofPorts | p2 |  | 2Tx |
|  | p4 |  | 4Tx |
|  | p8 |  | 8Tx |
|  | p16 |  | 16Tx |
| firstOFDMSymbolInTimeDomain | 13 | I0 = 13 | 2Tx, 4Tx |
|  | 5 | I0 = 5 | 8Tx, 16Tx |
| cdm-Type | fd-CDM2 |  | 2Tx, 4Tx |
|  | cdm4-FD2-TD2 |  | 8Tx, 16Tx |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2Tx | For test cases using 2 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |
| 4Tx | For test cases using 4 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |
| 8Tx | For test cases using 8 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |
| 16Tx | For test cases using 16 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |

<Unchanged Text Skipped>

CSI-IM Configuration

CSI-IM-Resource

Table 5.4.2.5-7: CSI-IM-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern0 SEQUENCE { |  |  | FR1 |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |
| pattern1 SEQUENCE { |  |  | FR2 |
| subcarrierLocation-p1 | s8 |  |  |
| symbolLocation-p1 | 13 |  |  |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| periodicityAndOffset | Not present |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

CSI-ReportConfig

Table 5.4.2.5-13: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId for CSI Acquisition |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId for CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE { | 2 entries |  |  |
| reportSlotOffsetList[1] | 4 | entry 1 | FR1_FDD_4Tx |
|  | 5 |  | FR1_FDD_8Tx, FR1_FDD_16Tx |
|  | 8 |  | FR1_TDD |
|  | 6 |  | FR2 |
| reportSlotOffsetList[2] | 4 | entry 2 | FR1_FDD_4Tx |
|  | 5 |  | FR1_FDD_8Tx, FR1_FDD_16Tx |
|  | 8 |  | FR1_TDD |
|  | 6 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  | FR1 |
| subbands7 | '1111111'B |  |  |
| } |  |  |  |
| csi-ReportingBand CHOICE { |  |  | FR2 |
| subbands9 | '111111111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table1 |  |  |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

<End of Changes >