3GPP TSG-RAN5 Meeting #94-e	R5-220715r3

# Electronic Meeting, 21th Feb– 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1618 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default configuration in Annex H |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In Table H.3.1-2, measId 1 is required to be associated to reportConfigId 1 and measObjectId 2 when condition Deactivated SCell or INTER-FREQ is used. However, No reportConfigNR is configured when condition Deactivated SCell or INTER-FREQ - PCell handover delay requirements defined in 38.133 concerns with Trs, which is specified in 38.133 as follows: 38.133 cl.6.1.1.2.2 Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. However, - CellGroupConfig given in 38.508-1 Table 4.6.3-19 is used in Table H.3.2-2. and no SMTC is configured in 38.508-1 Table 4.6.3-19.So no SMTC is provided in RRC messages for PSCell change/PCell HO/SCell addition. Then the spec yellow highlighted is not applicable. - In blind HO TCs 6.3.1.2/3, No MO is configured for target cell. Then the specs blue highlighted is not applicable. - SSB transmission periodicity is fixed to be 20ms according to 38.133 annex A.3.10. which is not 5ms. Then the specs purple highlighted is not applicable. As a result, there is actually no normative requirenments for blind HO TCs.To fix this issue. we suggest to add SMTC configuration in Table H.3.2-2. - To correct wrong reference in Table H.3.1-8 |
| Summary of change: |  | - ReportConfigNR is configured when condition Deactivated SCell or INTER-FREQ is used. - SMTC configuration is added in Table H.3.2-2. - Wrong reference in Table H.3.1-8 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Typos on table numbers are corrected. - "SMTC.1" is changed to "SMTC.n" for future compatibility. 2nd revision: Table H.3.1-11 derives from 38.508-1 Table 7.3.1-14. To improve readablity. All contents in Table H.3.1-11 is merged into 38.508-1 Table 7.3.1-14. H.3.1-11 can be voided. All reference to Table H.3.1-11 are redirected accordingly. 3nd revision: To correct typos |  |  |  |  |  |  |  |  |

<Start of modified section 1>

MeasConfig-DEFAULT

Configuration for NR measurement.

Table H.3.1-2: MeasConfig-DEFAULT: Configuration of NR measurement

| Derivation path: 38.508-1 [14] table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | n entries | n=1 without condition n=2 for INTER-FREQ OR INTER-RAT OR Deactivated SCell |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| measObjectId[2] | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTER-FREQ MO |  | INTER-FREQ |
| measObjectNR | MeasObjectNR-DEFAULT with Condition Deactivated SCell |  | Deactivated SCell |
| measObjectEUTRA | MeasObjectEUTRA-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | ReportConfigId |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT |  |  |
|  | ReportConfigNR-DEFAULT |  | INTER-FREQ OR Deactivated SCell |
| reportConfigInterRAT | ReportConfigInterRAT-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | MeasId |  |  |
| measObjectId[1] | 1 |  |  |
|  | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| reportConfigId[1] | ReportConfigId |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
|  | QuantityConfig | TS 38.508-1 Table 4.6.3-127 | L3 FILTERING NEEDED |
| measGapConfig | MeasGapConfig-DEFAULT |  | GAP NEEDED |
| } |  |  |  |

| Condition | Explanation |  |
| --- | --- | --- |
| GAP NEEDED | Measurement gap on the NR Cell is needed for measurement |  |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |  |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |  |
| Deactivated SCell |  | Configuration for measurement on deactivated SCell tests |
| L3 FILTERING NEEDED |  | L3 filtering is needed for measurement |

<End of modified section 1>

<Start of modified section 2>

RadioLinkMonitoringConfig -DEFAULT

Default configuration for RLM resources.

Table H.3.1-8: RadioLinkMonitoringConfig-DEFAULT: Default configuration for RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  | FR2 |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | both |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(0) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(1) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |

BeamFailureRecoveryConfig-DEFAULT

Default configuration for CBD and contention-free RACH in link recovery.

Table H.3.1-10: BeamFailureRecoveryConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig ::= SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | 0 |  |  |
| rach-ConfigBFR | RACH-ConfigGeneric specified in 38.508-1 [14] Table 7.3.1-14 |  | SSB |
|  | RACH-ConfigGeneric specified in 38.508-1 [14] Table 7.3.1-14 with condition PRACH.4 FR1 |  | CSI-RS and FR1 |
|  | RACH-ConfigGeneric specified in 38.508-1 [14] Table 7.3.1-14 with condition PRACH.4 FR2 |  | CSI-RS and FR1 |
| rsrp-ThresholdSSB | 58 | Actual value is 58 -156 = -98 dBm | SCS15 |
|  | 61 | Actual value is 61 -156 = -95 dBm | SCS30 |
|  | 47 | Actual value is 47 -156 = -109 dBm | SCS120 |
|  | 50 | Actual value is 50 -156 = -106 dBm | SCS240 |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF CHOICE { | 1 entry |  |  |
| ssb[1] SEQUENCE { |  |  | SSB |
| ssb | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| csi-RS[1] SEQUENCE { |  |  | CSI-RS |
| csi-RS | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  |  |
| ra-OccasionList | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| ra-ssb-OccasionMaskIndex | Not present |  |  |
| recoverySearchSpaceId | SearchSpaceId of the search space used for BFD RAR in DL active BWP |  |  |
| ra-Prioritization | Not present |  |  |
| beamFailureRecoveryTimer | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CBD |
| CSI-RS | Configuration for CSI-RS based CBD |

BWP-UplinkDedicated

Default BWP configuration for Beam Failure Recovery

Table H.3.1-10A: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| beamFailureRecoveryConfig | BeamFailureRecoveryConfig-DEFAULT |  |  |
| } |  |  |  |

Table H.3.1-11: Void

<End of modified section 2>

<Start of modified section 3>

H.3.2	RRC messages and information elements contents exceptions for handover

Table H.3.2-1: Void

Table H.3.2-2: RRCReconfiguration-HO

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB1 and SRB2 and DRBn and Re-establish_PDCP |  | RBConfig_KeyChange |
|  | RadioBearerConfig with conditions DRBn and Recover_PDCP |  | RBConfig_NoKeyChange |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table H.3.2-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBConfig_KeyChange | RadioBearerConfig to perform Intra-NR handover with security key change |
| RBConfig _NoKeyChange | RadioBearerConfig to perform Intra-NR handover without security key change |

Table H.3.2-3: CellGroupConfig

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| smtc | SMTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.n | SMTC.n is the SMTC reference configuration used in test case |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 3>