3GPPRAN5 - TTCN Workshop #56	R5w220005r2

# Electronic meeting, 3rd February 2022

Agenda Item:	10 (5G)

Source:	MCC TF160

Title:	Updates to NR RRC connection re-establishment procedure

Document for:	Endorsement

# 1	Introduction

The RRC Re-establishment procedure specified in TS 38.523-3 clause 7.3.5.4 needs a correction.

When preparing the prose CR for RAN5#93-e, the changes in step 8a/b were missed, these changes were agreed as per R5w210304r1. TTCN implementation in iwd-21wk49 is according to R5w210304r1.

When the RRC re-establishment procedure in the same cell is triggered due to a Handover failure, the following issue may happen:

According to TS 38.331 clause 5.3.5.5.2, “The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message”.

This implies that if UE doesn’t send the HARQ ACK SS will continue the retransmissions of RRCReconfiguration trigerring the handover.

In step 1, SS is reconfigured with the new C-RNTI and therefore RRCReconfiguration will be retransmitted with UE’s C-RNTI after step 2, that is not expected and causing a failure.

It is proposed to move the release and re-configuration of SRBs and DRBs from step 3 to step 0.

This discussion is related to R5s220201, the affected test cases are: 8.1.4.1.5, 8.1.4.1.6, 8.1.4.1.9.x, 8.1.5.6.1, 8.1.6.1.3.4.

# Proposal

The proposal shows the changes to be done on clause 7.3.5.4 in TS 38.523-3: the missing changes are shown in blue

#### 7.3.5.4	RRC connection re-establishment

In general, the re-establishment is done without activation time, i.e. the timing information for configuration of the SS and sending of the RRCReestablishment is ‘Now'.

The Source Cell and Target cell can be the same cell.

0. Source Cell:	If the Source Cell is the same as the Target Cell:

Release SRBs and DRBs.

(Re-)configure SRBs.

1.	Target Cell:	Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI.

NOTE 1:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

2.	Target Cell:	Receive RRCReestablismentRequest.

3.

Void

NOTE 2:	Void

4.	Target Cell:	Void

5.	Target Cell:	Activate Security.

6.	Target Cell:	Send RRCReestablishment.

7.	Target Cell:	Receive RRCReestablishmentComplete.

8.	Target Cell:	Reconfigure UL DCCH/DTCH DCI on USS.

8a. Target Cell	If the Source Cell is different from the Target Cell

Configure UL grant “OnSR” and stop periodic TA.

Else

Stop periodic TA.

8b	Target Cell	Reconfigure DRBs

9.	Target Cell:	Send RRCReconfiguration.

10.	Target Cell:	Receive RRCReconfigurationComplete.

11. Target Cell: 	Reconfigure DL DCCH/DTCH DCI on USS.

NOTE 3:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

12. Target Cell:	Re-configure RACH procedure as for initial access, enable TA transmissions.

NOTE 4:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

13.	Source Cell:	If the Source Cell is different from the Target Cell:

Release security.

Release  SRBs and DRBs.

(Re-)configure SRBs.

Stop periodic UL grant and periodic TA.

NOTE 5:	The FollowOnFlag is set to true in the ASP configuring the SRBs