3GPPRAN5 - TTCN Workshop #56	R5w220008r1

# Sophia Antipolis, France, 3rd February 2022

Agenda Item:	9

Source:	MCC TF160

Title:	MCX Dedicated Radio Bearers

Document for:	Endorsement

# 1	Introduction

For MCData test cases using the media plane a dedicated bearer is needed. Up to now there are no reference bearer and no packet filters in 36.508.

# 2	Issues

Issue 2-1:	It is not clear whether the dedicated bearer for MCData media plane traffic is AM or UM. Nevertheless the generic procedures in clause 5.4 of 36.579-1 imply that it is a UM bearer. 

NOTE 2-1a: 
In any case the generic procedures in clause 5.4 of 36.579-1 need to be reviewed for MCData and MCVideo as for MCData only for test cases using the media plane a dedicated bearer is needed and for MCVideo two dedicated bearers are needed.

NOTE 2-1b:
36.579-1/2/6/7 need to reviewed regarding use of 36.579-1 procedures 5.4.3/3A/3B and 5.4.4/4A/4B which are used for connection establishment even when there is SIP signalling only, i.e. when no dedicated bearer(s) are needed.

Assumption 2-1:	The dedicated bearer for MCData media plane traffic is a UM bearer.

Proposal 2-1:	36.579-1/2/6/7 to be reviewed regarding NOTE 2-1a/b.

Issue 2-2:	There are no reference bearer and packet filters in 36.508 for MCData. Generic procedures in clause 5.4 of 36.579-1 imply use of a (new) dedicated bearer #9 with QCI 70.

Assumption 2-2:	MCData uses reference default EPS bearer context #3 and if using the media plane a new reference dedicated bearer context #9 with QCI 70 which needs to be added to 36.508.

Proposal 2-2:	Reference dedicated bearer context #9 to be defined in 36.508 clause 6.6.2; new packet filter for TCP.
NOTE 2-2: 
Specification of reference dedicated bearer context #9 needs input regarding details like bit rates.

Issue 2-3:	Dedicated bearer for MCData needs to be specified for IPCAN model.

Proposal 2-3:	New PIXIT px_MCX_IPCAN_EpsBearerId_MCX_Data; packet filter to be added to 36.579-5 Table 5.2.1-2. 
ASP type definitions for IPCAN model need to be modified to be applicable for MCData too (see clause 3).

# 3	ASP Changes for IPCAN Model

After the discussions at TTCN Workshop #56 the CR R5-220459 has been raised at TSG-RAN WG5 Meeting #94-e proposing reference dedicated EPS bearer contexts for MCVideo and MCData:
In contrast to IMS the reference dedicated EPS bearer context for MCVideo needs only one dedicated bearer to which the streams for audio and video are mapped as well as the media plane control channel. This requires a slightly different ASP definition as shown below.

NOTE: The new definition allows a list of dedicated bearers, i.e. is still applicable even when more than one dedicated bearer would be required.

Further information can be found in R5-220488 updating Table 5.2.1-2 of 36.579-5.

type record of MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Type MCX_IPCAN_ActivateDedicatedEpsBearerMCX_Type; /* Establish dedicated EPS bearer(s) for the MCX PDN CellId : cell id of the cell Reference implementation: f_MCX_EUTRA_IPCAN_ActivateDedicatedEpsBearer */

type MCX_IPCAN_EpsBearerIdList_Type MCX_IPCAN_DeactivateDedicatedEpsBearerMCX_Type; /* Deactivate dedicated EPS bearer(s) for the MCX PDN CellId : cell id of the cell Reference implementation: f_EUTRA_DeactivateEPS_BearerContext */