3GPP TSG-RAN5 Meeting #98	R5-230650

# Athens, 27th  Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1392 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test applicability for EPS UPIP TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPIP_SEC_LTE-RAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EPS UPIP TC 7.3.4.4, 7.3.4.5, 7.3.4.6 does not have test applicability |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test applicability for EPS UPIP TC 7.3.4.4, 7.3.4.5, 7.3.4.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPS UPIP TC 7.3.4.4, 7.3.4.5, 7.3.4.6 does not have test applicability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | ZZZH shall be resolved according to R5-230651 in Table A.4.4-1 TS 36.523-2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release |  | Applicability |  | Additional Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Condition | Comment | Specific ICS |  | Specific IXIT | Number of TC Executions | Release other RAT |
| 6 | Idle mode operations |  |  |  |  |  |  |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.1a | PLMN selection / Automatic mode / between FDD and TDD | Rel-8 |  | C142 | UEs supporting E-UTRA FDD and E-UTRA TDD |  |  |  |  |  |
| 6.1.1.1b | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation | Rel-8 |  | R | UEs supporting E-UTRA. This test is 'cells on single frequency only' equivalent of TC 6.1.1.1 | pc_eFDD |  |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.2 | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  | Either TC 6.1.1.2 or TC 6.1.1.2a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.2a | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode / Single Frequency operation | Rel-8 |  | R | UEs supporting E-UTRA This test is 'cells on single frequency only' equivalent of 6.1.1.2 | pc_eFDD |  |  | Either TC 6.1.1.2 or TC 6.1.1.2a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.3 | Cell reselection of ePLMN in manual mode | Rel-8 |  | C388 | UEs supporting E-UTRA and (( NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  | Either TC 6.1.1.3 or TC 6.1.1.3b shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.3a | Cell reselection of ePLMN in manual mode / between FDD and TDD | Rel-9 (Note 3) |  | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |
| 6.1.1.3b | Cell reselection of ePLMN in manual mode / Single Frequency operation | Rel-8 |  | R | UEs supporting E-UTRA. This test is 'cells on single frequency only' equivalent of 6.1.1.3 | pc_eFDD |  |  | Either TC 6.1.1.3 or TC 6.1.1.3b shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.4 | PLMN selection in shared network environment / Automatic mode | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.4a | PLMN selection in shared network environment / Automatic mode / Between FDD and TDD | Rel-8 |  | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |
| 6.1.1.5 | Void |  |  |  |  |  |  |  |  |  |
| 6.1.1.6 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection | Rel-8 |  | C157a | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  | Either TC 6.1.1.6 or TC 6.1.1.6a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.6a | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Single Frequency operation | Rel-8 |  | C157 | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode. This test is 'cells on single frequency only' equivalent of 6.1.1.6 | pc_eFDD |  |  | Either TC 6.1.1.6 or TC 6.1.1.6a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.6b | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Two Frequencies operation | Rel-13 |  | C157b | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode. This test is 'cells on two frequencies only' and 'TDD cat.1bis UE only' equivalent of 6.1.1.6 | pc_eTDD |  |  | Either TC 6.1.1.6 or TC 6.1.1.6b shall be executed. (Note 21) |  |
| 6.1.1.7 | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer | Rel-10 |  | C179a | UEs supporting E-UTRA and MinimumPeriodicSearchTimer and not supporting "Fast First Higher Priority PLMN search" and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  | Either TC 6.1.1.7 or TC 6.1.1.7a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.7a | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer / Single Frequency operation | Rel-10 |  | C179 | UEs supporting E-UTRA and MinimumPeriodicSearchTimer and not supporting "Fast First Higher Priority PLMN search". This test is 'cells on single frequency only' equivalent of 6.1.1.7 | pc_eFDD |  |  | Either TC 6.1.1.7 or TC 6.1.1.7a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.8 | PLMN selection of RPLMN or (E)HPLMN; Automatic mode | Rel-8 |  | C212a | UEs supporting E-UTRA and EF_LRPLMSI_Exception and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.9 | PLMN selection of RPLMN or (E)HPLMN; Manual mode | Rel-8 |  | C213 | UEs supporting E-UTRA and ManualModeNetworkSelectionException | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.1.10 | eMTC / NTN / GSO | Rel-17 |  | C414 | UEs supporting E-UTRA and (Category M1 or Category M2) and NTN access and NTN features in GSO or NGSO scenario | pc_eFDD |  |  |  |  |
| 6.1.2.1 | Void |  |  |  |  |  |  |  |  |  |
| 6.1.2.2 | Cell selection / Qrxlevmin | Rel-8 |  | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.2a | Cell selection / Qqualmin | Rel-9 (Note 3) |  | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.2b | Cell selection / UE Cat 0 not allowed | Rel-12 |  | C224 | UEs supporting E-UTRA and UE Category 0 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.2c | Cell selection / Qrxlevmin / Enhanced Coverage | Rel-13 |  | C254 | UEs supporting E-UTRA and (CE mode A or CE mode B) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.2d | Cell selection / Qqualmin / Enhanced Coverage | Rel-13 |  | C254 | UEs supporting E-UTRA and (CE mode A or CE mode B) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.3 | Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (S<0 or barred) | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.3a | Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (Srxlev > 0 and Squal < 0) | Rel-9 (Note 3) |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.4 | Cell reselection | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.5 | Cell reselection for interband operation | Rel-8 |  | C184 a | UEs supporting E-UTRA and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.5a | Cell reselection for interband operation/ Power Class 2 UE operation/ Between FDD and TDD | Rel-14 (Note 17) |  | C281 | UEs supporting E-UTRA FDD and E-UTRA TDD and Bands38, 40, 41 or 42 Power class 2 operation and NOT Category M1 |  |  |  |  |  |
| 6.1.2.5b | Cell reselection for interband operation using Pcompensation / Between FDD and TDD | Rel-14 (Note 17) |  | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |
| 6.1.2.5c | Inter-band Cell reselection / Extended frequency list | Rel-12 |  | C184 a | UEs supporting E-UTRA and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.6 | Cell reselection using Qhyst, Qoffset and Treselection | Rel-8 |  | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.6a | Cell reselection using Treselection / Enhanced Coverage | Rel-13 |  | C254 | UEs supporting E-UTRA and (CE mode A or CE mode B) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.6b | Cell reselection from cell in enhanced coverage to inter-frequency cell in normal coverage | Rel-13 |  | C254b | UEs supporting E-UTRA and (CE mode A or CE mode B) and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.7 | Cell reselection / Equivalent PLMN | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  | Either TC 6.1.2.7 or TC 6.1.2.7a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.7a | Cell reselection / Equivalent PLMN / Single Frequency operation | Rel-8 |  | R | UEs supporting E-UTRA. This test is 'cells on single frequency only ' equivalent of 6.1.2.7 | pc_eFDD |  |  | Either TC 6.1.2.7 or TC 6.1.2.7a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.8 | Cell reselection using cell status and cell reservations / Access control class 0 to 9 | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  | Either TC 6.1.2.8 or TC 6.1.2.8a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.8a | Cell reselection using cell status and cell reservations / Access control class 0 to 9 / Single Frequency operation | Rel-8 |  | R | UEs supporting E-UTRA. This test is 'cells on single frequency only ' equivalent of 6.1.2.8 | pc_eFDD |  |  | Either TC 6.1.2.8 or TC 6.1.2.8a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.9 | Cell reselection using cell status and cell reservations / Access control class 11 to 15 | Rel-8 |  | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  | Either TC 6.1.2.9 or TC 6.1.2.9a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.9a | Cell reselection using cell status and cell reservations / Access control class 11 to 15 / Single Frequency operation | Rel-8 |  | R | UEs supporting E-UTRA. This test is 'cells on single frequency only ' equivalent of 6.1.2.9 | pc_eFDD |  |  | Either TC 6.1.2.9 or TC 6.1.2.9a shall be executed. (Note 4) |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.10 | Cell reselection in shared network environment | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.11 | Inter-frequency Cell reselection | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.11a | Inter-frequency Cell reselection / Extended frequency list | Rel-12 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.12 | Cell reselection / Cell-specific reselection parameters provided by the network in a neighbouring cell list | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.13 | Cell reselection, Sintrasearch, Snonintrasearch | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.14 | Speed-dependent Cell reselection | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.15 | Inter-frequency Cell reselection according to cell reselection priority provided by SIBs | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.15a | Inter-frequency Cell reselection according to cell reselection priority provided by SIBs / Between FDD and TDD | Rel-9 (Note 3) |  | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |
| 6.1.2.15b | Inter-band Cell reselection according to cell reselection priority provided by SIBs | Rel-8 |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.16 | Cell reselection / interband operation / Between FDD and TDD | Rel-9 (Note 3) |  | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |
| 6.1.2.17 | Cell reselection for Squal to check against SIntraSearchQ and SnonIntraSearchQ | Rel-9 (Note 3) |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.18 | Inter-frequency Cell reselection based on common priority information with parameters ThreshX, HighQ, ThreshX, LowQ and ThreshServing, LowQ | Rel-9 (Note 3) |  | C388 | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.1.2.19 | Intra-frequency Cell reselection / MFBI | Rel-9 (Note 3) |  | C189F | UEs supporting E-UTRA and MFBI feature indicated by Feature Group Indicator 31 | pc_eFDD |  |  |  |  |
|  |  |  |  | C189T |  | pc_eTDD |  |  |  |  |
| 6.1.2.20 | Inter-frequency Cell reselection / MFBI | Rel-9 (Note 3) |  | C189bF | UEs supporting E-UTRA and MFBI feature indicated by Feature Group Indicator 31 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  | C189bT |  | pc_eTDD |  |  |  |  |
| 6.1.2.21 | Inter-band Cell reselection / MFBI | Rel-9 (Note 3) |  | C189bF | UEs supporting E-UTRA and MFBI feature indicated by Feature Group Indicator 31 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  | C189bT |  | pc_eTDD |  |  |  |  |
| 6.1.2.22 | Cell reselection / MFBI / UE does not support multiBandInfoList | Rel-8 to Rel-9 only |  | C229 a | UEs supporting E-UTRA and not support MFBI feature indicated by Feature Group Indicator 31 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |  |
|  |  |  |  | C230 |  | pc_eTDD |  |  |  |  |
| 6.1.2.23 | Inter-band Cell reselection / MFBI frequency band priority adjustment/Inter-band CA | Rel-12 |  | C257 | UEs supporting E-UTRA and MFBI feature indicated by Feature Group Indicator 31 and freqBandIndicatorPriority-r12 and Inter-band Carrier Aggregation | pc_eFDD |  |  |  |  |
|  |  |  |  | C258 |  | pc_eTDD |  |  |  |  |
| 6.2.1.1 | Inter-RAT PLMN Selection / Selection of correct RAT for OPLMN / Automatic mode | Rel-8 |  | C150 | UEs supporting E-UTRA and UTRA, or E-UTRA and UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.1.2 | Inter-RAT PLMN Selection / Selection of correct RAT for UPLMN / Automatic mode | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.1.3 | Inter-RAT PLMN Selection / Selection of correct PLMN and RAT in shared network environment / Automatic mode | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.1.4 | Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.1.6 | Inter-RAT Background HPLMN Search / Search for correct RAT for HPLMN / Automatic Mode | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.2.1 | Inter-RAT Cell selection / From E-UTRA RRC_IDLE to UTRA_Idle / Serving cell becomes non-suitable | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.2.2 | Inter-RAT Cell selection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_idle / Serving cell becomes non-suitable | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.2.3 | Inter-RAT Cell selection / From E-UTRA RRC_IDLE to HRPD Idle / Serving cell becomes non-suitable | Rel-8 |  | C06 | UEs supporting E-UTRA and HRPD and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.2.4 | Inter-RAT Cell selection / From E-UTRAN RRC_IDLE to 1xRTT idle / Serving cell becomes non-suitable | Rel-8 |  | C07 | UEs supporting E-UTRA and 1xRTT and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.2.5 | Cell selection / No USIM | Rel-8 |  | C182 | UEs supporting E-UTRA and UTRA and not supporting of IMS emergency call and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.2.6 | Inter-RAT Cell selection / From GSM_Idle/GPRS Packet_idle to E-UTRA_RRC_IDLE / Serving cell becomes non-suitable | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.2.7 | Inter-RAT Cell selection / From GSM_Idle/GPRS Packet_idle to E-UTRA_RRC_IDLE, when the serving cell is barred | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.2.8 | Inter-RAT Cell selection / From UTRA_Idle to E-UTRA RRC_IDLE / Serving cell becomes non-suitable | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.1 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.1a | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle (Squal < ThreshServing, LowQ, Srxlev > ThreshX, LowP and Srxlev > ThreshX, HighP) | Rel-9 (Note 3) |  | C171 | UEs supporting E-UTRA and GERAN and Squal based cell reselection between E-UTRAN and GERAN and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 GERAN |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.2 | Void |  |  |  |  |  |  |  |  |  |
| 6.2.3.3 | Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.3a | Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE (QqualminEUTRA, SqualServingCell < Threshserving,low2, SqualnonServingCell,x > Threshx, low2 and SqualnonServingCell,x > Threshx, high2) | Rel-9 (Note 3) |  | C126 | UEs supporting E-UTRA and UTRA and supporting Squal based cell reselection to UTRAN from E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
| 6.2.3.4 | Inter-RAT cell reselection / From UTRA_CELL_PCH state to E-UTRA RRC_IDLE | Rel-8 |  | C77 | UEs supporting E-UTRA and UTRA and EUTRA Feature Group Indicator 1 and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.4a | Inter-RAT Cell reselection / From UTRA_CELL_PCH state to E-UTRA RRC_IDLE based on RSRQ+RSRP evaluation | Rel-9 (Note 3) |  | C77 | UEs supporting E-UTRA and UTRA and EUTRA Feature Group Indicator 1 and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.5 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.5a | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle (Squal > ThreshX, HighQ, Squal < ThreshServing, LowQ, Squal > ThreshX, LowQ and SnonIntraSearchQ) | Rel-9 (Note 3) |  | C127 | UEs supporting E-UTRA and UTRA and supporting Squal based cell reselection to E-UTRAN from UTRAN and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
| 6.2.3.6 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle according to RAT priority provided by dedicated signalling | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.7 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is higher reselection priority than E-UTRA | Rel-8 |  | C06 | UEs supporting E-UTRA and HRPD and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.7a | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is higher reselection priority than E-UTRA (Srxlev > ThreshHRPD, HighP) | Rel-9 |  | C06 | UEs supporting E-UTRA and HRPD and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.8 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD is lower reselection priority than E-UTRA | Rel-8 |  | C06 | UEs supporting E-UTRA and HRPD and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.8a | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is lower reselection priority than E-UTRA (Squal < ThreshServing, LowQ and Srxlev > ThreshHRPD, LowP | Rel-9 |  | C06 | UEs supporting E-UTRA and HRPD and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.9 | Inter-RAT Cell reselection: from E-UTRA RRC_IDLE to CDMA2000 1xRTT Dormant- When CDMA2000 1xRTT cell is higher reselection priority than E-UTRA | Rel-8 |  | C07 | UEs supporting E-UTRA and 1xRTT and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.9a | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is higher reselection priority than E-UTRA (Srxlev > Thresh1xRTT, HighP) | Rel-9 |  | C07 | UEs supporting E-UTRA and 1xRTT and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.10 | Inter-RAT Cell reselection: from E-UTRA RRC_IDLE to CDMA2000 1xRTT Idle - When CDMA2000 1xRTT is lower reselection priority than E-UTRA | Rel-8 |  | C07 | UEs supporting E-UTRA and 1xRTT and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.10a | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is lower reselection priority than E-UTRA (Squal < ThreshServing, LowQ and Srxlev > Thresh1xRTT, LowP) | Rel-9 (Note 3) |  | C07 | UEs supporting E-UTRA and 1xRTT and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.11 | Void |  |  |  |  |  |  |  |  |  |
| 6.2.3.12 | Void |  |  |  |  |  |  |  |  |  |
| 6.2.3.13 | Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE according to RAT priority provided by dedicated signalling | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.14 | Inter-RAT cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (priority of E-UTRA cells are higher than the serving cell) | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.15 | Inter-RAT Cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (priority of E-UTRA cells are lower than the serving cell) | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.16 | Inter-RAT Cell reselection / from GSM_Idle to E-UTRAN /based on H_PRIO criteria | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.17 | Inter-RAT Cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (priority E-UTRA cells) | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.18 | Inter-RAT Cell reselection / from GSM_Idle/GPRS Packet_Idle to E-UTRA (Not allowed E-UTRA cells) | Rel-8 |  | C05 | UEs supporting E-UTRA and GERAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.19 | Redirection to E-UTRA upon the release of the CS connection | Rel-8 |  | C115 | UEs supporting E-UTRA and GERAN and speech and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.20 | Void |  |  |  |  |  |  |  |  |  |
| 6.2.3.21 | Inter-RAT Cell reselection / From GPRS Packet_transfer (NC0 mode) to E-UTRA | Rel-8 |  | C66 | UEs supporting E-UTRA and GERAN and GERAN to E-UTRAN neighbour cell measurements and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.22 | Void |  |  |  |  |  |  |  |  |  |
| 6.2.3.23 | Inter-RAT Cell reselection from GPRS Packet transfer to E-UTRA in CCN Mode (PACKET CELL CHANGE CONTINUE) | Rel-8 |  | C114 | UEs supporting E-UTRA and GERAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  |  |
| 6.2.3.24 | Inter-RAT Cell reselection from GPRS Packet transfer to E-UTRA in CCN Mode (PACKET CELL CHANGE ORDER) | Rel-8 |  | C114 | UEs supporting E-UTRA and GERAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.26 | Inter-RAT Autonomous Cell reselection GPRS Packet_transfer to E-UTRA (NC1 mode) | Rel-8 |  | C114 | UEs supporting E-UTRA and GERAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.27 | Inter-RAT Cell selection from GPRS Packet_transfer to E-UTRA (NC2 Mode) | Rel-8 |  | C114 | UEs supporting E-UTRA and GERAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.28 | Inter-RAT Cell reselection from GPRS Packet_transfer to E-UTRA (Network Assisted Cell Change) | Rel-8 |  | C114 | UEs supporting E-UTRA and GERAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.29 | Inter-RAT Cell reselection from GPRS packet_transfer to E-UTRA in CCN mode (PACKET MEASUREMENT ORDER) | Rel-8 |  | C114 | UEs supporting E-UTRA and GERAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.30 | Inter-RAT Cell reselection failure from GPRS Packet transfer to E-UTRA (Network Assisted Cell Change) | Rel-8 |  | C114 | UEs supporting E-UTRA and GERAN and CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.31 | Inter-RAT Cell reselection / From UTRA_Idle (low priority) to E-UTRA RRC_IDLE (high priority) according to RAT priority provided by dedicated signalling | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.32 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle, Snonintrasearch | Rel-8 |  | C01 | UEs supporting E-UTRA and UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.3.33 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle / Squal based cell reselection parameters are broadcasted in E-UTRAN / UE does not support Squal based cell reselection in UTRAN | Rel-9 (Note 3) |  | C131 | UEs supporting E-UTRA and UTRA and not supporting Squal based cell reselection to E-UTRAN from UTRAN and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.3.34 | Inter-RAT Cell reselection from E-UTRA to UTRA / MFBI | Rel-9 |  | C189aF | UEs supporting E-UTRA and UTRA FDD and MFBI feature indicated by Feature Group Indicator 31 and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  | C189aT |  | pc_eTDD |  |  |  |  |
| 6.2.3.35 | Inter-RAT Cell reselection from UTRA to E-UTRA / MFBI | Rel-10 (Note 3) |  | C189cF | UEs supporting E-UTRA and UTRA and MFBI feature indicated by Feature Group Indicator 31 and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 UTRA FDD |
|  |  |  |  | C189cT |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.2.4.1 | Inter-RAT absolute priority based reselection in UTRA CELL_FACH to E-UTRA RRC_IDLE (Higher Priority Layers, Srxlev,x > Threshx,high and Srxlev,serv > Sprioritysearch1 and SqualServ > Sprioritysearch2) | Rel-11 (Note 3) |  | C01a | UEs supporting E-UTRA and UTRA FDD and support of High Priority layer measurements or support of all priority layer measurements and cell Reselection procedure in CELL_FACH and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.4.2 | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (Higher Priority Layers, no cell reselection to E-UTRA RRC_IDLE when Srxlev,serv < Sprioritysearch1) | Rel-11 (Note 3) |  | C01a | UEs supporting E-UTRA and UTRA FDD and support of High Priority layer measurements or support of all priority layer measurements and cell Reselection procedure in CELL_FACH and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.4.3 | Inter-RAT absolute priority based reselection in UTRA _CELL_FACH to E-UTRA RRC_IDLE (Higher Priority Layers, Squal,x > Threshx,high2 and Srxlev,serv > Sprioritysearch1 and SqualServ > Sprioritysearch2) | Rel-11 (Note 3) |  | C01a | UEs supporting E-UTRA and UTRA FDD and support of High Priority layer measurements or support of all priority layer measurements and cell Reselection procedure in CELL_FACH and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.4.4 | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (lower priority) to E-UTRA RRC_IDLE (higher priority) (All Layers, Srxlev,x > Threshx,high) | Rel-11 (Note 3) |  | C01b | UEs supporting E-UTRA and UTRA FDD and support of all priority layer measurements and cell reselection procedure in CELL_FACH and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.4.5 | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (lower priority) to E-UTRA RRC_IDLE (higher priority) (All Layers, Squal,x >ThreshX,high2) | Rel-11 (Note 3) |  | C01b | UEs supporting E-UTRA and UTRA FDD and support of all priority layer measurements and cell reselection procedure in CELL_FACH and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.4.6 | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (higher priority) to E-UTRA RRC_IDLE (lower priority) (All Layers, Srxlev,serv < Sprioritysearch1 , Srxlev,serv <Thresh serv,low and Srxlev,x > Threshx,low) | Rel-11 (Note 3) |  | C01b | UEs supporting E-UTRA and UTRA FDD and support of all priority layer measurements and cell reselection procedure in CELL_FACH and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.2.4.7 | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (higher priority) to E-UTRA RRC_IDLE (lower priority) (All Layers, Srxlev,serv < Sprioritysearch1 , Squal,serv <Thresh serv,low2 and Squal,x > ThreshX,low2) | Rel-11 (Note 3) |  | C01b | UEs supporting E-UTRA and UTRA FDD and support of all priority layer measurements and cell reselection procedure in CELL_FACH and NOT Category M1 | pc_eFDD |  |  |  | Rel-9 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.3.1 | Inter-frequency Cell reselection / From E-UTRA RRC_IDLE non-CSG cell to E-UTRA RRC_IDLE CSG cell | Rel-8 |  | C80 | UEs supporting E-UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.3.2 | Inter-RAT Cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA idle CSG cell | Rel-8 |  | C95 | UEs supporting E-UTRA and GERAN and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.3.3 | Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE CSG cell | Rel-8 |  | C76 | UEs supporting E-UTRA and UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.3.4 | Inter-RAT Cell reselection / From UTRA CELL_PCH state to E-UTRA RRC_IDLE CSG cell | Rel-8 |  | C82 | UEs supporting E-UTRA and UTRA and allowed CSG list and EUTRA Feature Group Indicator 1 and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.3.5 | Manual support for CSG ID selection | Rel-8 |  | C80 | UEs supporting E-UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.3.6 | Ignoring CSG cells in cell selection/reselection when allowed CSG list is empty or not supported | Rel-8 |  | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.3.7 | Inter-RAT Cell reselection from E-UTRA idle non-CSG cell to a UTRA CSG cell | Rel-8 |  | C76 | UEs supporting E-UTRA and UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.3.8 | Void |  |  |  |  |  |  |  |  |  |
| 6.3.9 | Manual CSG ID selection across PLMNs | Rel-9 |  | C80 | UEs supporting E-UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.3.10 | Void |  |  |  |  |  |  |  |  |  |
| 6.3.11 | Void |  |  |  |  |  |  |  |  |  |
| 6.3.12 | Void |  |  |  |  |  |  |  |  |  |
| 6.4.1 | Manual CSG ID selection / Hybrid cell whose CSG ID is not in the Allowed CSG list nor Operator’s list | Rel-9 (Note 3) |  | C80 | UEs supporting E-UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.4.2 | Inter-frequency Cell reselection / From E-UTRA RRC_IDLE non-CSG cell to E-UTRA RRC_IDLE member hybrid cell | Rel-9 (Note 3) |  | C80 | UEs supporting E-UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.4.3 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE non-CSG cell to UTRA_Idle member hybrid cell | Rel-9 (Note 3) |  | C76 | UEs supporting E-UTRA and UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.4.4 | Inter-RAT Cell reselection / From E-UTRA RRC_IDLE non-member hybrid cell to UTRA_Idle member hybrid cell | Rel-9 (Note 3) |  | C76 | UEs supporting E-UTRA and UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.4.5 | Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE member hybrid cell | Rel-9 (Note 3) |  | C76 | UEs supporting E-UTRA and UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.4.6 | Inter-RAT Cell reselection / From UTRA CELL_PCH to E-UTRA RRC_IDLE member hybrid cell | Rel-9 (Note 3) |  | C76 | UEs supporting E-UTRA and UTRA and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  | Rel-8 UTRA FDD |
|  |  |  |  |  |  | pc_eTDD |  |  |  | Rel-9 UTRA TDD |
| 6.4.7 | Inter-RAT Cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA RRC_IDLE member hybrid cell | Rel-9 (Note 3) |  | C95 | UEs supporting E-UTRA and GERAN and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.5.1 |  | WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qrxlevmeas, BeaconRSSI, WLAN identifier no match/match) | Rel-12 | C225 |  | UEs supporting E-UTRA and WLAN and allowed offload to and from WLAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.5.2 | WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qrxlevmeas, BackhaulRateDlWLAN) | Rel-12 |  | C225 | UEs supporting E-UTRA and WLAN and allowed offload to and from WLAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.5.3 |  | WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qqualmeas, BackhaulRateUlWLAN) | Rel-12 | C225 |  | UEs supporting E-UTRA and WLAN and allowed offload to and from WLAN and NOT Category M1 |  |  |  |  |  |
|  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.5.4 | WLAN Offload / Cell selection / EUTRA RRC_Idle to/from WLAN (Qqualmeas, ChannelUtilizationWLAN) | Rel-12 |  | C225 | UEs supporting E-UTRA and WLAN and allowed offload to and from WLAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.5.5 | WLAN offload / Cell selection / EUTRA RRC_Idle to/from WLAN (ANDSF and RAN rules co-existence) | Rel-12 |  | C225 | UEs supporting E-UTRA and WLAN and allowed offload to and from WLAN and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 6.5.6 | Void |  |  |  |  |  |  |  |  |  |
| 7 | Layer 2 |  |  |  |  |  |  |  |  |  |
| 7.1.1.1 | CCCH mapped to UL SCH/ DL-SCH / Reserved Logical Channel ID | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.1.1a | CCCH mapped to UL SCH/ DL-SCH / UE Cat 0 | Rel-12 |  | C224 | UEs supporting E-UTRA and UE Category 0 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.1.2 |  | DTCH or DCCH mapped to UL SCH/ DL-SCH / Reserved Logical Channel ID | Rel-8 | R |  | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.1 | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / Non-contention based random access procedure | Rel-8 |  | C12 | UEs supporting E-UTRA or (CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.1a | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / Non-contention based random access procedure for high speed scenario | Rel-14 |  | C313 | UEs supporting E-UTRA FDD or E-UTRA TDD and high speed enhancement for prach | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.2 | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE in PDCCH Order / Non-contention based random access procedure | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.3 |  | Correct selection of RACH parameters / Preamble selected by MAC itself / Contention based random access procedure | Rel-8 | C224c |  | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.3a | Correct selection of RACH parameters/ Preamble selected by MAC itself/ Contention based random access procedure/ Enhanced coverage | Rel-13 |  | C254a | UEs supporting E-UTRA and CE Mode A | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.3b | Correct selection of RACH parameters / Preamble selected by MAC itself / Contention based random access procedure for high speed scenario | Rel-14 |  | C313 | UEs supporting E-UTRA FDD or E-UTRA TDD and high speed enhancement for prach | pc_eFDD |  |  |  |  |
| 7.1.2.4 | Random access procedure / Successful | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.5 | Random access procedure / MAC PDU containing multiple RARs | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.6 |  | Maintenance of uplink time alignment | Rel-8 | R |  | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.7 | MAC contention resolution / Temporary C-RNTI | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.8 | MAC contention resolution / C-RNTI | Rel-8 |  | C12 | UEs supporting E-UTRA or (CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.9 | MAC back off indicator | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.10.1 | CA / Random access procedure / SCell / Intra-band Contiguous CA | Rel-11 |  | C190 | UEs supporting E-UTRA and Intra-band contiguous Uplink Carrier Aggregation and multiple timing advances | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.10.2 | CA / Random access procedure / SCell / Inter-band CA | Rel-11 |  | C191 | UEs supporting E-UTRA and Inter-band Uplink Carrier Aggregation and multiple timing advances and UL (Pcell) supported in each band of Inter-band CA combination under test | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.10.3 | CA / Random access procedure / SCell / Intra-band non-contiguous CA | Rel-11 |  | C192 | UEs supporting E-UTRA and Intra-band non-contiguous Uplink Carrier Aggregation and multiple timing advances | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.11.1 | CA / Maintenance of uplink time alignment / Multiple TA / Intra-band Contiguous CA | Rel-11 |  | C190 | UEs supporting E-UTRA and Intra-band contiguous Uplink Carrier Aggregation and multiple timing advances | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.11.2 | CA / Maintenance of uplink time alignment / Multiple TA / Inter-band CA | Rel-11 |  | C191 | UEs supporting E-UTRA and Inter-band Uplink Carrier Aggregation and multiple timing advances and UL (Pcell) supported in each band of Inter-band CA combination under test | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.11.3 | CA / Maintenance of uplink time alignment / Multiple TA / Intra-band non-contiguous CA | Rel-11 |  | C192 | UEs supporting E-UTRA and Intra-band non-contiguous Uplink Carrier Aggregation and multiple timing advances | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.2.11.4 | FDD-TDD CA / Maintenance of uplink time alignment / Multiple TA | Rel-12 |  | C233 | UEs supporting E-UTRA FDD and TDD and 3DL CA and 3UL CA with tdd-FDD-CA-PCellDuplex-r12 with the first and/or second bit set to "1 "and multiple timing advances |  |  |  |  |  |
| 7.1.2.12 | CA / Random access procedure / TDD SCell without PUSCH/PUCCH transmission | Rel-13 |  | C320 | UEs supporting E-UTRA FDD-TDD DL CA and SRS switching between a band pair. |  |  |  |  |  |
|  |  |  |  | C321 | UEs supporting E-UTRA TDD-TDD DL CA and SRS switching between a band pair. | pc_eTDD |  |  |  |  |
| 7.1.2.13 | CA / PUCCH SCell / Maintenance of uplink time alignment | Rel-13 |  | C301 | UEs supporting E-UTRA and DL CA and UL CA and PUCCH SCell | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.3.1 | Correct handling of DL assignment / Dynamic case | Rel-8 |  | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.3.2 | Correct handling of DL assignment / Semi-persistent case | Rel-8 |  | C100F | UEs supporting E-UTRA and semi-persistence scheduling and Feature Group Indicator 7 | pc_eFDD |  |  |  |  |
|  |  |  |  | C100T |  | pc_eTDD |  |  |  |  |
| 7.1.3.3 | MAC PDU header handling | Rel-8 |  | C224a | UEs supporting E-UTRA and NOT (UE Category 0 or UE Category M1) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.3.3a | MAC PDU header handling / UE with limited TB size | Rel-12 |  | C224b | UEs supporting E-UTRA and (UE Category 0 or UE Category M1) | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.3.4 | Correct HARQ process handling / DCCH and DTCH | Rel-8 |  | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |
| 7.1.3.4a | Correct HARQ process handling / DCCH and DTCH/ Enhanced Coverage / CE Mode A | Rel-13 |  | C254a | UEs supporting E-UTRA and CE mode A | pc_eFDD |  |  |  |  |
|  |  |  |  |  |  | pc_eTDD |  |  |  |  |

| 7.1.3.5 | Correct HARQ process handling / CCCH | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.5a | Correct HARQ process handling / CCCH/ Enhanced Coverage / CE Mode A | Rel-13 | C254a | UEs supporting E-UTRA and CE Mode A | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.6 | Correct HARQ process handling / BCCH | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.6a | Correct HARQ process handling / Enhanced Coverage / HARQ-ACK bundling | Rel-14 | C367 | UEs supporting E-UTRA FDD and CE Mode A and HARQ-ACK bundling | pc_eFDD |  |  |  |
| 7.1.3.7 | MAC padding | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.8 | Void |  |  |  |  |  |  |  |
| 7.1.3.9 | MAC reset / DL | Rel-8 | C12 | UEs supporting E-UTRA or (CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.11.1 | CA / Correct HARQ process handling / DCCH and DTCH / Pcell and Scell / Intra-band Contiguous CA | Rel-10 | C132 | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.11.2 | CA / Correct HARQ process handling / DCCH and DTCH / Pcell and Scell / Inter-band CA | Rel-10 | C151 | UEs supporting E-UTRA and Inter-band Carrier Aggregation | pc_eFDD |  | Note 11 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.11.3 | CA / Correct HARQ process handling / DCCH and DTCH / Pcell and Scell / Intra-band non-Contiguous CA | Rel-11 | C132a | UEs supporting E-UTRA and Downlink Intra-band non-contiguous CA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.11.4 | FDD-TDD CA / Correct HARQ process handling / DCCH and DTCH / FDD PCell and TDD SCell | Rel-12 | C235a | UE supporting E-UTRA FDD and TDD and 2DL CA and 1UL CA and Support of tdd-FDD-CA-PCellDuplex-r12 with the second bit setting to "1" |  |  |  |  |
| 7.1.3.11.5 | FDD-TDD CA / Correct HARQ process handling / DCCH and DTCH / TDD PCell and FDD SCell | Rel-12 | C234a | UE supporting E-UTRA FDD and TDD and 2DL CA and 1UL CA and Support of tdd-FDD-CA-PCellDuplex-r12 with the first bit setting to "1" |  |  |  |  |
| 7.1.3.12 | TDD additional special subframe configuration / Special subframe pattern 9 with Normal Cyclic Prefix / CRS based transmission scheme | Rel-11 (Note 7) | C175 | UEs supporting E-UTRA TDD and TDD special subframe config | pc_eTDD |  |  |  |
| 7.1.3.12a | TDD additional special subframe configuration / Special subframe pattern 7 with Extended Cyclic Prefix / CRS based transmission scheme | Rel-11 (Note 7) | C175 | UEs supporting E-UTRA TDD and TDD special subframe config | pc_eTDD |  |  |  |
| 7.1.3.13 | TDD additional special subframe configuration / Special subframe pattern 9 with Normal Cyclic Prefix / UE-specific reference signals based transmission scheme | Rel-11 (Note 7) | C175 | UEs supporting E-UTRA TDD and TDD special subframe config | pc_eTDD |  |  |  |
| 7.1.3.13a | TDD additional special subframe configuration / Special subframe pattern 7 with Extended Cyclic Prefix / UE-specific reference signals based transmission scheme | Rel-11 (Note 7) | C175 | UEs supporting E-UTRA TDD and TDD special subframe config | pc_eTDD |  |  |  |
| 7.1.3.14 | Correct handling of DL assignment / Dynamic case / EPDCCH | Rel-11 | C188 | UEs supporting E-UTRA and ePDCCH and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.15 | Correct handling of DL assignment / Semi-persistent case / EPDCCH | Rel-11 | C188 | UEs supporting E-UTRA and ePDCCH and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.16 | Correct handling of DL assignment / Dynamic case / eIMTA | Rel-12 | C256 | UEs supporting E-UTRA and eIMTA and NOT Category M1 | pc_eTDD |  |  |  |
| 7.1.3.16a | CA / Correct handling of DL assignment / Dynamic case / eIMTA / Inter-band CA | Rel-12 | C264 | UEs supporting E-UTRA and Inter-band Carrier Aggregation and eIMTA | pc_eTDD |  |  |  |
| 7.1.3.17 | CA / PUCCH SCell / Correct HARQ process handling | Rel-13 | C301 | UEs supporting E-UTRA and DL CA and UL CA and PUCCH SCell | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.18.1 | sTTI combination {slot, slot} / Correct handling of DL assignment / Collision handling | Rel-15 | C379 | UEs supporting E-UTRA and only {slot, slot} and not {subslot, subslot} combination in downlink and uplink CCs | pc_eFDD |  |  |  |
| 7.1.3.18.2 | sTTI combination {subslot, subslot} / Correct handling of DL assignment / Collision handling | Rel-15 | C380 | UEs supporting E-UTRA and {subslot, subslot} combination in downlink and uplink CCs | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.19 | Short TTI / Correct handling of DL assignment / HARQ sharing between PDSCH and slot/subslot-PDSCH | Rel-15 | C379a | UEs supporting E-UTRA and {slot, slot} combination in downlink and uplink CCs | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.20 | Short TTI / Correct handling of DL assignment / multiplexing of SPDCCH and slot/subslot-PDSCH | Rel-15 | C381 | UE supporting E-UTRA and {slot, slot} combination in downlink and uplink CCs and L1-based SPDCCH reuse | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.21 | Short TTI / Correct handling of DL assignment / DMRS sharing | Rel-15 | C380 | UEs supporting E-UTRA and {subslot, subslot} combination in downlink and uplink CCs and minimum processing timeline | pc_eFDD |  |  |  |
| 7.1.3.22 | Short Processing Time / Correct handling of DL assignment / HARQ process sharing | Rel-15 | C378 | UE supporting E-UTRA and short processing time | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.3.23 | Enhanced Coverage / DL Fexible starting PRB | Rel-15 | C406 | UEs supporting E-UTRA and CE Mode A and flexible starting PRB for PDSCH | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.1 | Correct handling of UL assignment / Dynamic case | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.1a | Correct handling of UL assignment / Dynamic case / Skip padding transmissions | Rel-14 | C325 | UE supporting skip of uplink transmissions if no data is available | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.2 | Correct handling of UL assignment / Semi-persistent case | Rel-8 | C100F | UEs supporting E-UTRA and semi-persistence scheduling and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C100T |  | pc_eTDD |  |  |  |
| 7.1.4.2a | Correct handling of UL assignment / Semi-persistent case / Skip padding transmissions / SPS activation and de-activation confirmation | Rel-14 | C326 | UE supporting skip of SPS uplink transmissions if no data is available | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.2b | Correct handling of UL assignment / Semi-persistent case / SPS interval shorter than 10 subframes | Rel-14 | C327 | UE supporting SPS interval shorter than 10 subframes | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.3 | Logical channel prioritization handling | Rel-8 | C19F | UEs supporting E-UTRA and Feature Group Indicator 6 and Feature Group Indicator 7 and NOT (UE Category 0 or UE Category 1 or UE Category M1) | pc_eFDD |  |  |  |
|  |  |  | C19T |  | pc_eTDD |  |  |  |
| 7.1.4.3a | Logical channel prioritization handling / UE with limited TB size | Rel-12 | C19aF | UEs supporting E-UTRA and Feature Group Indicator 6 and Feature Group Indicator 7 and (UE Category 0 or UE Category 1 or UE Category M1) | pc_eFDD |  |  |  |
|  |  |  | C19aT |  | pc_eTDD |  |  |  |
| 7.1.4.4 | Correct handling of MAC control information / Scheduling requests and PUCCH | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.5 | Correct handling of MAC control information / Scheduling requests and random access procedure | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.6 | Correct handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer and retransmission of BSR / Regular BSR | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.7 | Correct handling of MAC control information / Buffer status / UL resources are allocated / Padding BSR | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.7a | Correct handling of MAC control information / Buffer status / UL resources are allocated / Cancellation of Padding BSR | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.8 | Correct handling of MAC control information / Buffer status / Periodic BSR timer expires | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.9 | Void |  |  |  |  |  |  |  |
| 7.1.4.10 | MAC padding | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.11 | Correct HARQ process handling | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.11a | Correct HARQ process handling / Semi-persistent case / Non-adaptive retransmission / Fixed Redundancy Version | Rel-14 | C326 | UE supporting skip of SPS uplink transmissions if no data is available | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.12 | MAC reset / UL | Rel-8 | C16aF | UEs supporting E-UTRA and Feature Group Indicator 7 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  | C16aT |  | pc_eTDD |  |  |  |
| 7.1.4.12a | MAC Partial reset / UL for Voice and Video Enhancement | Rel-14 | C299 | UE supporting PUSCH enhancement for MMTEL voice and video enhancements mode | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.13 | MAC PDU header handling | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.14 | Correct HARQ process handling / TTI bundling | Rel-8 | C99F | UEs supporting E-UTRA and TTI bundling and Feature Group Indicator 7 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  | C99T |  | pc_eTDD |  |  |  |
| 7.1.4.14a | Correct HARQ process handling / feedback for UL data | Rel-15 | C393 | UEs supporting E-UTRA and TTI bundling and Feature Group Indicator 7 and (CE Mode A or CE Mode B) | pc_eFDD |  |  |  |
|  |  |  | C394 |  | pc_eTDD |  |  |  |
| 7.1.4.15 | UE power headroom reporting / Periodic reporting | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.16 | UE power headroom reporting / DL pathloss change reporting | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.18 | Correct handling of MAC control information / Buffer Status / UL data arrive in the UE Tx buffer / Extended buffer size | Rel-10 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.19.1 | CA / UE power headroom reporting / SCell activation and DL pathloss change reporting / Extended PHR / Intra-band Contiguous CA | Rel-10 | C133 | UEs supporting E-UTRA and Intra-band contiguous Uplink Carrier Aggregation and FGI 113 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.19.2 | CA / UE power headroom reporting / SCell activation and DL pathloss change reporting / Extended PHR / Inter-band CA | Rel-11 | C162 | UEs supporting E-UTRA and Inter-band Uplink Carrier Aggregation and UL (Pcell) supported in each band of Inter-band CA combination under test | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.19.3 | CA / UE power headroom reporting / SCell activation and DL pathloss change reporting / Extended PHR / Intra-band non-Contiguous CA | Rel-11 | C207 | UEs supporting E-UTRA and Uplink Intra-band non-Contiguous CA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.20.1 | CA / Correct handling of MAC control information / Buffer status / Intra-band Contiguous CA | Rel-10 | C133 | UEs supporting E-UTRA and Intra-band contiguous Uplink Carrier Aggregation and FGI 113 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.20.2 | CA / Correct handling of MAC control information / Buffer status / Inter-band CA | Rel-11 | C162 | UEs supporting E-UTRA and Inter-band Uplink Carrier Aggregation and UL (Pcell) supported in each band of Inter-band CA combination under test | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.20.3 | CA / Correct handling of MAC control information / Buffer status / Intra-band non-Contiguous CA | Rel-11 | C207 | UEs supporting E-UTRA and Uplink Intra-band non-Contiguous CA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.21 | UE power headroom reporting / Extended PHR | Rel-10 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.22 | Correct HARQ process handling / UL MIMO | Rel-10 | C158 | UE supporting E-UTRA and UL MIMO and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.23 | Correct HARQ process handling / TTI bundling with enhanced HARQ pattern | Rel-12 | C227 | UEs supporting E-UTRA FDD and TTI bundling and TTI bundling with enhanced HARQ pattern and Feature Group Indicator 7 and NOT Category M1 | pc_eFDD |  |  |  |
| 7.1.4.24 | Correct HARQ process handling / TTI bundling without resource allocation restriction | Rel-12 | C228 | UEs supporting E-UTRA and TTI bundling and NOT (UE Category 0 or Category M1) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.24a | Correct HARQ process handling / TTI bundling without resource allocation restriction / UE with limited TB size | Rel-12 | C228a | UEs supporting E-UTRA and TTI bundling and UE Category 0 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.24b | Correct HARQ process handling / Enhanced Coverage / CE Mode A | Rel-13 | C254a | UEs supporting E-UTRA and CE mode A | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.24c | Correct HARQ process handling / Enhanced Coverage / CE Mode B | Rel-13 | C255 | UEs supporting E-UTRA and CE mode B | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.24d | Correct HARQ process handling / Repetition with asynchronous PUSCH enhancement | Rel-14 | C334 | UEs supporting E-UTRA and PUSCH enhancement for MMTEL voice and video enhancements mode | pc_eFDD |  |  |  |
| 7.1.4.25.1 | FDD-TDD CA / Correct HARQ process handling / PUSCH / FDD PCell and TDD SCell | Rel-12 | C235 | UE supporting E-UTRA FDD and TDD and 2DL CA and 2UL CA with tdd-FDD-CA-PCellDuplex-r12 with the second bit set to "1 " |  |  |  |  |
| 7.1.4.25.2 | FDD-TDD CA / Correct HARQ process handling / PUSCH / TDD PCell and FDD SCell | Rel-12 | C234 | UE supporting E-UTRA FDD and TDD and 2DL CA and 2UL CA with tdd-FDD-CA-PCellDuplex-r12 with the first bit set to "1" |  |  |  |  |
| 7.1.4.26.1 | Correct handling of MAC control information / Buffer status / Split DRB | Rel-12 | C244 | UEs supporting E-UTRA and DC Split DRB | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.27.1 | DC power headroom reporting / PSCell activation and DL pathloss change reporting / SCG DRB | Rel-12 | C245 | UEs supporting E-UTRA and DC SCG DRB | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.27.2 | DC power headroom reporting/ PSCell addition and DL pathloss change reporting / Split DRB | Rel-12 | C244 | UEs supporting E-UTRA and DC Split DRB | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.28 | Correct handling of UL assignment / Dynamic case / eIMTA | Rel-12 | C256 | UEs supporting E-UTRA and eIMTA and NOT Category M1 | pc_eTDD |  |  |  |
| 7.1.4.28a | CA / Correct handling of UL assignment / Dynamic case / eIMTA / Inter-band CA | Rel-12 | C265 | UEs supporting E-UTRA and Inter-band Uplink Carrier Aggregation and eIMTA | pc_eTDD |  |  |  |
| 7.1.4.29.1 | CA / PUCCH SCell / Correct handling of MAC control information / Scheduling requests and PUCCH | Rel-13 | C301 | UEs supporting E-UTRA and DL CA and UL CA and PUCCH SCell | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.29.2 | CA / PUCCH SCell / UE power headroom reporting / Periodic reporting | Rel-13 | C301 | UEs supporting E-UTRA and DL CA and UL CA and PUCCH SCell | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.30 | Void |  |  |  |  |  |  |  |
| 7.1.4.31 | eLAA / Logical channel prioritization handling / laa-UL-Allowed | Rel-14 | C330 | UEs supporting E-UTRA and uplink LAA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.32.1 | eLAA / SCell PUSCH / Correct handling of UL assignment / DCI0A/0B / One step scheduling | Rel-14 | C330 | UEs supporting E-UTRA and uplink LAA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.32.2 | eLAA / SCell PUSCH / Correct handling of UL assignment / DCI4A/4B/One step scheduling | Rel-14 | C331 | UEs supporting E-UTRA and uplink LAA and UL MIMO | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.32.3 | eLAA / SCell PUSCH / Correct handling of UL assignment / DCI0A/0B / Two step scheduling | Rel-14 | C332 | UEs supporting E-UTRA and uplink LAA and two step scheduling | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.32.4 | eLAA / SCell PUSCH / Correct handling of UL assignment / DCI4A/4B / Two step scheduling | Rel-14 | C333 | UEs supporting E-UTRA and uplink LAA and two step scheduling and UL MIMO | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.33 | Void |  |  |  |  |  |  |  |
| 7.1.4.34 | Void |  |  |  |  |  |  |  |
| 7.1.4.35 | Void |  |  |  |  |  |  |  |
| 7.1.4.36 | Void |  |  |  |  |  |  |  |
| 7.1.4.37 | Short Processing Time / Correct handling of UL assignment | Rel-15 | C378 | UE supporting E-UTRA and short processing time | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.38.1 | sTTI combination {slot, slot} / Correct handling of UL assignment / Collision handling | Rel-15 | C379 | UEs supporting E-UTRA and {slot, slot} combination in downlink and uplink CCs | pc_eFDD |  |  |  |
| 7.1.4.38.2 | sTTI combination {subslot, subslot} / Correct handling of UL assignment / Collision handling | Rel-15 | C380 | UEs supporting E-UTRA and {subslot, subslot} combination in downlink and uplink CCs | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.39 | Short TTI / Correct handling of UL assignment / DMRS sharing | Rel-15 | C380 | UEs supporting E-UTRA and {subslot, subslot} combination in downlink and uplink CCs and minimum processing timeline | pc_eFDD |  |  |  |
| 7.1.4.40 | Short TTI / Correct handling of MAC control information / Scheduling requests and SPUCCH | Rel-15 | C379a | UEs supporting E-UTRA and {slot, slot} combination in downlink and uplink CCs | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.41 | Short TTI / Correct handling of UL assignment / HARQ sharing between PUSCH and slot/subslot-PUSCH | Rel-15 | C383 | UEs supporting E-UTRA and short processing time and {slot, slot} combination in downlink and uplink CCs | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.42 | Enhanced Coverage / UL Fexible starting PRB | Rel-15 | C407 | UEs supporting E-UTRA and CE Mode A and flexible starting PRB for PUSCH | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.4.43 | eMTC / NTN / UE specific TA report / UE specific Koffset | Rel-17 | C415 | UEs supporting E-UTRA and (Category M1 or Category M2) and NTN access and NTN features in GSO or NGSO scenario and Timing advance reporting in NTN cell and timing relationship enhancements using Differential Koffset | pc_eFDD |  |  |  |
| 7.1.4a.1 | Correct downlink reception and uplink transmission when specific valid subframes are signalled for BL UE | Rel-13 | C254 | UEs supporting E-UTRA and (CE Mode A or CE Mode B) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.5.1 | Inter-TTI PUSCH hopping by uplink grant | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.5.2 | Predefined intra-TTI PUSCH hopping (N_sb=1) | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.5.3 | Predefined intra-TTI PUSCH hopping (N_sb=2/3/4) | Rel-8 | C58F | UEs supporting E-UTRA and Feature Group Indicator 21 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  | C58T |  | pc_eTDD |  |  |  |
| 7.1.5.4 | Predefined inter-TTI PUSCH hopping (N_sb=1) | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.5.5 | Predefined inter-TTI PUSCH hopping (N_sb=2/3/4) | Rel-8 | C58F | UEs supporting E-UTRA and Feature Group Indicator 21 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  | C58T |  | pc_eTDD |  |  |  |
| 7.1.5.6 | PUSCH Hopping / multi-subframe repetitions | Rel-14 | C334 | UEs supporting E-UTRA and PUSCH enhancement for MMTEL voice and video enhancements mode | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.6.1 | DRX operation / Short cycle not configured / Parameters configured by RRC | Rel-8 | C08F | UEs supporting E-UTRA and Feature Group 5 and NOT Category M1 | pc_eFDD |  | If TC 7.1.6.5 is executed this test case is optional. (Note 13) |  |
|  |  |  | C08T |  | pc_eTDD |  |  |  |
| 7.1.6.1a | DRX operation / Short cycle not configured / Parameters configured by RRC / Enhanced Coverage / CE Mode A | Rel-13 | C08aF | UEs supporting E-UTRA and Feature Group 5 and CE Mode A | pc_eFDD |  |  |  |
|  |  |  | C08aT |  | pc_eTDD |  |  |  |
| 7.1.6.2 | DRX operation / Short cycle not configured / DRX command MAC control element reception | Rel-8 | C08bF | UEs supporting E-UTRA and Feature Group 5 | pc_eFDD |  |  |  |
|  |  |  | C08bT |  | pc_eTDD |  |  |  |
| 7.1.6.3 | DRX operation / Short cycle configured / Parameters configured by RRC | Rel-8 | C216F | UEs supporting E-UTRA and Feature Group 4 and Feature Group 5 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  | C216T |  | pc_eTDD |  |  |  |
| 7.1.6.4 | DRX operation / Short cycle configured / DRX command MAC control element reception | Rel-8 | C216F | UEs supporting E-UTRA and Feature Group 4 and Feature Group 5 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  | C216T |  | pc_eTDD |  |  |  |
| 7.1.6.5 | eDRX operation / Long cycle configured / Parameters configured by RRC | Rel-13 | C260 | UEs supporting E-UTRA and Extended Long DRX | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.6.6 | eMTC / NTN / DRX / (UL)HARQ RTT | Rel-17 | C414 | UEs supporting E-UTRA and (Category M1 or Category M2) and NTN access and NTN features in GSO or NGSO scenario | pc_eFDD |  |  |  |
| 7.1.7.1.1 | DL-SCH transport block size selection / DCI format 1 / RA type 0 | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.2 | DL-SCH transport block size selection / DCI format 1 / RA type 1 | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.3 | DL-SCH transport block size selection / DCI format 1A / RA type 2 / Localised VRB | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.4 | DL-SCH transport block size selection / DCI format 1A / RA type 2 / Distributed VRB | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.5 | DL-SCH transport block size selection / DCI format 2A / RA type 0 / Two transport blocks enabled / Transport block to codeword swap flag value set to ’0’ | Rel-8 | C56 | UEs supporting E-UTRA and (UE Category 2 to UE Category 5) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.6 | DL-SCH transport block size selection / DCI format 2A / RA type 1 / Two transport blocks enabled / Transport block to codeword swap flag value set to ’1’ | Rel-8 | C56 | UEs supporting E-UTRA and (UE Category 2 to UE Category 5) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.6a | DL-SCH transport block size selection / DCI format 2A / RA type 0 and RA type 1 / Two transport blocks enabled / 3 and 4 Layer Spatial Multiplexing | Rel-10 | C296 | UEs supporting E-UTRA and ((UE Category 5 to UE Category 7) or (UE Category 9 to UE Category 12) or UE DL Category 15 or UE DL Category 16 or UE DL Category 18 or UE DL Category 19 or UE DL Category 20 or UE DL Category 21) and 4-layer spatial multiplexing. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.7 | DL-SCH transport block size selection / DCI format 1 / RA type 0 / 256QAM | Rel-12 | C248 | UEs supporting E-UTRA and ((UE Category 11 to UE Category 12) or (UE DL Category 11 to UE DL Category 21)) and downlink 256QAM | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.8 | DL-SCH transport block size selection / DCI format 1 / RA type 1 / 256QAM | Rel-12 | C248 | UEs supporting E-UTRA and ((UE Category 11 to UE Category 12) or (UE DL Category 11 to UE DL Category 21)) and downlink 256QAM | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.9 | DL-SCH transport block size selection / DCI format 1B / RA type 2 / Localised VRB / 256QAM | Rel-12 | C248 | UEs supporting E-UTRA and ((UE Category 11 to UE Category 12) or (UE DL Category 11 to UE DL Category 21)) and downlink 256QAM | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.10 | DL-SCH transport block size selection / DCI format 1B / RA type 2 / Distributed VRB / 256QAM | Rel-12 | C248 | UEs supporting E-UTRA and ((UE Category 11 to UE Category 12) or (UE DL Category 11 to UE DL Category 21)) and downlink 256QAM | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.11 | DL-SCH transport block size selection / DCI format 2A / RA type 0 / Two transport blocks enabled / Transport block to codeword swap flag value set to ’0’ / 256QAM | Rel-12 | C248 | UEs supporting E-UTRA and ((UE Category 11 to UE Category 12) or (UE DL Category 11 to UE DL Category 21)) and downlink 256QAM | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.12 | DL-SCH transport block size selection / DCI format 2A / RA type 1 / Two transport blocks enabled / Transport block to codeword swap flag value set to ’1’ / 256QAM | Rel-12 | C248 | UEs supporting E-UTRA and ((UE Category 11 to UE Category 12) or (UE DL Category 11 to UE DL Category 21)) and downlink 256QAM | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.12a | DL-SCH transport block size selection / DCI format 2A / RA type 0 and RA type 1 / Two transport blocks enabled / 3 and 4 Layer Spatial Multiplexing / 256QAM | Rel-12 | C297 | UEs supporting E-UTRA and (UE Category 11 or UE Category 12 or UE DL Category 13 or UE DL Category 15 or UE DL Category 16 or UE DL Category 18 or UE DL Category 19) or UE DL Category 20 or UE DL Category 21 and 4-layer spatial multiplexing and downlink 256QAM. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.13 | DL-SCH transport block size selection / DCI format 6-1A / RA type 2 / Localised VRB | Rel-13 | C254d | UEs supporting E-UTRA and CE mode A and NOT Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.13a | DL-SCH transport block size selection / DCI format 6-1A / RA type 2 / Localised VRB / CAT M2 | Rel-14 | C254e | UEs supporting E-UTRA and Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.14 | DL-SCH transport block size selection / DCI format 6-1B | Rel-13 | C255 a | UEs supporting E-UTRA and CE mode B and NOT Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.1.14a | DL-SCH transport block size selection / DCI format 6-1B / CAT M2 | Rel-14 | C255b | UEs supporting E-UTRA and CE mode B and Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.2.1 | UL-SCH transport block size selection / DCI format 0 | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.2.2 | UL-SCH transport block size selection / DCI format 6-0A | Rel-13 | C254a | UEs supporting E-UTRA and CE mode A and NOT Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.2.2a | UL-SCH transport block size selection / DCI format 6-0A / CAT M2 | Rel-14 | C254e | UEs supporting E-UTRA and Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.2.3 | UL-SCH transport block size selection / DCI format 6-0B/ Uplink resource allocation type 2 | Rel-13 | C255 a | UEs supporting E-UTRA and CE mode B and NOT Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.2.3a | UL-SCH transport block size selection / DCI format 6-0B/ Uplink resource allocation type 2 / CAT M2 | Rel-14 | C255b | UEs supporting E-UTRA and CE mode B and Category M2 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.7.2.4 | UL-SCH transport block size selection / DCI format 0 / UL 256QAM | Rel-14 | C224d | UE supporting E-UTRA and UL 256QAM | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.8.1 | Periodic RI reporting using PUCCH / UE only supports 1 layer for spatial multiplexing in DL / Transmission mode 3/4 | Rel-8 | C103 | UEs supporting E-UTRA and (UE Category 0 or UE Category 1) and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.9.1.1 | CA / Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band Contiguous CA | Rel-10 | C132 | UEs supporting E-UTRA and Intra-band Contiguous Carrier Aggregation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.9.1.2 | CA / Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Inter-band CA | Rel-10 | C151 | UEs supporting E-UTRA and Inter-band Carrier Aggregation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.9.1.3 | CA / Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band non-Contiguous CA | Rel-11 | C132a | UEs supporting E-UTRA and Downlink Intra-band non-Contiguous CA Carrier Aggregation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.9.2 | CA / PUCCH SCell / Activation/Deactivation of SCells | Rel-13 | C301 | UEs supporting E-UTRA and DL CA and UL CA and PUCCH SCell | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.10.1 | Sending SR on PUCCH with DMRS generated by using virtual cell identity / nPUCCH-Identity | Rel-11 | C208 | UEs supporting E-UTRA and UL CoMP and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.10.2 | Transmitting data on PUSCH with DMRS generated by using virtual cell identity / nPUSCH-Identity | Rel-11 | C208 | UEs supporting E-UTRA and UL CoMP and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.11.1 | LAA transmits common control information in PDCCH scrambled with CC-RNTI | Rel-13 | C280 | UEs supporting E-UTRA and downlink LAA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.12.1 | DataInactivityTimer expiry | Rel-14 | C295 | UEs supporting E-UTRA and data inactivity monitoring | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.1.13.1.1 | Hibernation of SCells / Hibernation MAC control element reception / sCellHibernationTimer / dormantSCellDeactivationTimer / Intra-band Contiguous CA | Rel-15 | C373 | UEs supporting E-UTRA and Intra-band Carrier Aggregation and modification of SCell in dormant state | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.2.1 | UM RLC / Segmentation and reassembly / 5-bit SN / Framing info field | Rel-8 | C15F | UEs supporting E-UTRA and Feature Group Indicator 3 and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C15T |  | pc_eTDD |  |  |  |
| 7.2.2.2 | UM RLC / Segmentation and reassembly / 10-bit SN / Framing info field | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.3 | UM RLC / Reassembly / 5-bit SN / LI value > PDU size | Rel-8 | C15F | UEs supporting E-UTRA and Feature Group Indicator 3 and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C15T |  | pc_eTDD |  |  |  |
| 7.2.2.4 | UM RLC / Reassembly / 10-bit SN / LI value > PDU size | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.5.1 | UM RLC / 5-bit SN / Correct use of sequence numbering | Rel-8 | C15F | UEs supporting E-UTRA and Feature Group Indicator 3 and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C15T |  | pc_eTDD |  |  |  |
| 7.2.2.5.2 | UM RLC / 10-bit SN / Correct use of sequence numbering | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.6 | UM RLC / Concatenation, segmentation and reassembly | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.7 | UM RLC / In sequence delivery of upper layer PDUs without residual loss of RLC PDUs / Maximum re-ordering delay below t-Reordering | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.8 | UM RLC / In sequence delivery of upper layer PDUs without residual loss of RLC PDUs / Maximum re-ordering delay exceeds t-Reordering | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.9 | UM RLC / In sequence delivery of upper layer PDUs with residual loss of RLC PDUs / Maximum re-ordering delay exceeds t-Reordering | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.10 | UM RLC / Duplicate detection of RLC PDUs | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.2.2.11 | UM RLC / RLC re-establishment procedure | Rel-8 | C362 | UEs supporting E-UTRA and Feature Group Indicator 7 or (CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A” and Feature Group Indicator 7) | pc_eFDD |  |  |  |
|  |  |  | C363 |  | pc_eTDD |  |  |  |
| 7.2.2.12 | eMTC / NTN / AM RLC / Receiver status triggers / extended t-Reordering configured | Rel-17 | C414 | UEs supporting E-UTRA and (Category M1 or Category M2) and NTN access and NTN features in GSO or NGSO scenario | pc_eFDD |  |  |  |
| 7.2.3.1 | AM RLC / Concatenation and reassembly | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.2 | AM RLC / Segmentation and reassembly / No PDU segmentation | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.3 | AM RLC / Segmentation and reassembly / Framing info field | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.4 | AM RLC / Segmentation and reassembly / Different numbers of length indicators | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.5 | AM RLC / Reassembly / LI value > PDU size | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.6 | AM RLC / Correct use of sequence numbering | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.7 | AM RLC / Control of transmit window | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.8 | AM RLC / Control of receive window | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.9 | AM RLC / Polling for status | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.10 | AM RLC / Receiver status triggers | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.11 | Void |  |  |  |  |  |  |  |
| 7.2.3.12 | Void |  |  |  |  |  |  |  |
| 7.2.3.13 | AM RLC / Reconfiguration of RLC parameters by upper layers | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.14 | AM RLC / In sequence delivery of upper layers PDUs | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.15 | AM RLC / Re-ordering of RLC PDU segments | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.16 | AM RLC / Re-transmission of RLC PDU without re-segmentation | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.17 | AM RLC / Re-segmentation RLC PDU / SO, FI, LSF | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.18 | AM RLC / Reassembly / AMD PDU reassembly from AMD PDU segments, segmentation Offset and Last Segment Flag fields | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.19 | Void |  |  |  |  |  |  |  |
| 7.2.3.20 | AM RLC / Duplicate detection of RLC PDUs | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.2.3.21 | AM RLC / RLC re-establishment at RRC connection reconfiguration including mobilityControlInfo IE | Rel-8 | C12 | UEs supporting E-UTRA or (CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.1.1 | Maintenance of PDCP sequence numbers / User plane / RLC AM | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.1.2 | Maintenance of PDCP sequence numbers / User plane / RLC UM / Short PDCP SN (7 bits) | Rel-8 | C15F | UEs supporting E-UTRA and Feature Group Indicator 3 and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C15T |  | pc_eTDD |  |  |  |
| 7.3.1.3 | Maintenance of PDCP sequence numbers / User plane / RLC UM / Long PDCP SN (12 bits) | Rel-8 | C16F | UEs supporting E-UTRA and Feature Group Indicator 7 | pc_eFDD |  |  |  |
|  |  |  | C16T |  | pc_eTDD |  |  |  |
| 7.3.3.1 | Ciphering and deciphering / Correct functionality of EPS AS encryption algorithms / SNOW 3G | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.3.2 | Ciphering and deciphering / Correct functionality of EPS UP encryption algorithms / SNOW 3G | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.3.3 | Ciphering and deciphering / Correct functionality of EPS AS encryption algorithms / AES | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.3.4 | Ciphering and deciphering / Correct functionality of EPS UP encryption algorithms / AES | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.3.5 | Ciphering and deciphering / Correct functionality of EPS AS encryption algorithms / ZUC | Rel-11 (Note 3) | C215 | UEs supporting E-UTRA and ZUC algorithm | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.3.6 | Ciphering and deciphering / Correct functionality of EPS UP encryption algorithms / ZUC | Rel-11 (Note 3) | C215 | UEs supporting E-UTRA and ZUC algorithm | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.4.1 | Integrity protection / Correct functionality of EPS AS integrity algorithms / SNOW3G | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.4.2 | Integrity protection / Correct functionality of EPS AS integrity algorithms / AES | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.4.3 | Integrity protection / Correct functionality of EPS AS integrity algorithms / ZUC | Rel-11 (Note 3) | C215 | UEs supporting E-UTRA and ZUC algorithm | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.4.4 | User Plane Integrity Protection / Correct Functionality of User Plane Integrity Protection/ SNOW3G | Rel-17 | C4XXH | UEs supporting E-UTRA and user plane integrity protection with EPS | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.4.5 | User Plane Integrity Protection / Correct Functionality of User Plane Integrity Protection/ AES | Rel-17 | C4XXH | UEs supporting E-UTRA and user plane integrity protection with EPS | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.4.6 | User Plane Integrity Protection / Correct Functionality of User Plane Integrity Protection/ ZUC | Rel-17 | C4XYH | UEs supporting E-UTRA and user plane integrity protection with EPS and ZUC algorithm | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 7.3.5.1 | Void |  |  |  |  |  |  |  |
| 7.3.5.2 | PDCP handover / Lossless handover / PDCP sequence number maintenance | Rel-8 | C12 | UEs supporting E-UTRA or (CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |

<End of modified section 1>

<Start of modified section 2>

Table 4-1a: Applicability of tests Conditions

| C01 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| --- | --- |
| C01a IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND (A.4.5-2/3 OR A.4.5-2/4) AND NOT (A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C01b IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND A.4.5-2/4 AND NOT (A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C02 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 THEN R ELSE N/A |  |
| C02a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND NOT (A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C03 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/1 THEN R ELSE N/A |  |
| C04 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 THEN R ELSE N/A |  |
| C05 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C06 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C07 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C08F IF A.4.1-1/1 AND A.4.5-1a/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C08aF IF A.4.1-1/1 AND A.4.5-1a/5 AND A.4.4-1/122 THEN R ELSE N/A |  |
| C08bF IF A.4.1-1/1 AND A.4.5-1a/5 THEN R ELSE N/A |  |
| C08T IF A.4.1-1/2 AND A.4.5-1b/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C08aT IF A.4.1-1/2 AND A.4.5-1b/5 AND A.4.4-1/122 THEN R ELSE N/A |  |
| C08bT IF A.4.1-1/2 AND A.4.5-1b/5 THEN R ELSE N/A |  |
| C09F IF (A.4.1-1/1 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A |  |
| C09T IF (A.4.1-1/2 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A |  |
| C10F IF A.4.1-1/1 AND A.4.5-1a/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C10T IF A.4.1-1/2 AND A.4.5-1b/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C11F IF (A.4.1-1/1 AND A.4.5-1a/16 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A |  |
| C11T IF (A.4.1-1/2 AND A.4.5-1b/16 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A |  |
| C12 IF ( (A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.3.2-2A/1 OR ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15) THEN R ELSE N/A |  |
| C13F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/16 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C13T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/16 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C14F IF A.4.1-1/1 AND A.4.5-1a/5 AND A.4.5-1a/17 THEN R ELSE N/A |  |
| C14T IF A.4.1-1/2 AND A.4.5-1b/5 AND A.4.5-1b/17 THEN R ELSE N/A |  |
| C15F IF A.4.1-1/1 AND A.4.5-1a/3 AND A.4.5-1a/7 THEN R ELSE N/A |  |
| C15T IF A.4.1-1/2 AND A.4.5-1b/3 AND A.4.5-1b/7 THEN R ELSE N/A |  |
| C16F IF A.4.1-1/1 AND A.4.5-1a/7 THEN R ELSE N/A |  |
| C16aF IF A.4.1-1/1 AND A.4.5-1a/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C16T IF A.4.1-1/2 AND A.4.5-1b/7 THEN R ELSE N/A |  |
| C16aT IF A.4.1-1/2 AND A.4.5-1b/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C17F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1a/22 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C17T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1b/22 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C18 IF (A.4.1-1/1 OR A.4.1-1/2) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A |  |
| C19F IF A.4.1-1/1 AND A.4.5-1a/6 AND A.4.5-1a/7 AND NOT (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C19aF IF A.4.1-1/1 AND A.4.5-1a/6 AND A.4.5-1a/7 AND (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C19T IF A.4.1-1/2 AND A.4.5-1b/6 AND A.4.5-1b/7 AND NOT (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C19aT IF A.4.1-1/2 AND A.4.5-1b/6 AND A.4.5-1b/7 AND (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C20F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/16 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C20T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/16 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C21F IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C21T IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C21aF IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C21aT IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C22 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/3 AND A.4.4-2/2 AND NOT (A.4.4-2/32) THEN R ELSE N/A |  |
| C23 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/4 AND A.4.4-2/2 AND NOT (A.4.4-2/32) THEN R ELSE N/A |  |
| C24F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/16 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C24T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/16 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C25F IF A.4.1-1/1 AND A.4.1-1/4 AND A.4.5-1a/16 AND A.4.5-1a/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C25T IF A.4.1-1/2 AND A.4.1-1/4 AND A.4.5-1b/16 AND A.4.5-1b/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C26 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/1 AND NOT (A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C27 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-1/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C28F IF (A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/25) THEN R ELSE N/A |  |
| C28T IF (A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/25) THEN R ELSE N/A |  |
| C29F IF A.4.1-1/1 AND A.4.5-1a/7 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C29T IF A.4.1-1/2 AND A.4.5-1b/7 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C30F IF A.4.1-1/1 AND A.4.5-1a/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C30T IF A.4.1-1/2 AND A.4.5-1b/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C31F IF A.4.1-1/1 AND A.4.5-1a/7 AND A.4.5-1a/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C31T IF A.4.1-1/2 AND A.4.5-1b/7 AND A.4.5-1b/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C32F IF A.4.1-1/1 AND A.4.5-1a/7 AND A.4.5-1a/20 THEN R ELSE N/A |  |
| C32T IF A.4.1-1/2 AND A.4.5-1b/7 AND A.4.5-1b/20 THEN R ELSE N/A |  |
| C33F IF A.4.1-1/1 AND A.4.5-1a/20 THEN R ELSE N/A |  |
| C33T IF A.4.1-1/2 AND A.4.5-1b/20 THEN R ELSE N/A |  |
| C34 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/7 THEN R ELSE N/A |  |
| C35 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A |  |
| C36F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/8 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C36T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/8 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C37 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND A.4.5-2/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C38F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/10 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C38T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/10 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C39F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/5 AND A.4.5-1a/19 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C39T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/5 AND A.4.5-1b/19 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C40F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/5 AND A.4.5-1a/19 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C40T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/5 AND A.4.5-1b/19 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C41 Void |  |
| C42F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/12 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C42T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/12 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C44F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/5 AND A.4.5-1a/19 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C44T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/5 AND A.4.5-1b/19 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C45F IF (A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.1-2/1) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/25) THEN R ELSE N/A |  |
| C45T IF (A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.1-2/1) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/25) THEN R ELSE N/A |  |
| C46 IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.4-1/9 THEN R ELSE N/A |  |
| C47 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-2/1 THEN R ELSE N/A |  |
| C47a Void |  |
| C48 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C49 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/10 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C50 Void |  |
| C51 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/9 AND (A.4.4-1/12 OR A.4.4-1/13 OR A.4.4-1/14 OR A.4.4-1/15 OR A.4.4-1/93) THEN R ELSE N/A |  |
| C52 Void |  |
| C53 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.20/35 THEN R ELSE N/A |  |
| C54 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/18 THEN R ELSE N/A |  |
| C55 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/19 AND A.4.4-1/54 THEN R ELSE N/A |  |
| C56 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C57 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C58F IF A.4.1-1/1 AND A.4.5-1a/21 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C58T IF A.4.1-1/2 AND A.4.5-1b/21 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C59 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C60 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C61F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1a/16 AND A.4.5-1a/22 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C61T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1b/16 AND A.4.5-1b/22 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C62 Void |  |
| C63 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.5-1a/25 AND A.4.5-1a/30 AND A.4.5-1b/25 AND A.4.5-1b/30 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C64 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/20 THEN R ELSE N/A |  |
| C64a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/20 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C65 Void |  |
| C66 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/4 AND A.4.4-1/21 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C67 Void |  |
| C68 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C69 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C70 Void |  |
| C71 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 THEN R ELSE N/A |  |
| C71a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C71b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C72 Void |  |
| C73 Void |  |
| C74 Void |  |
| C75 Void |  |
| C76 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C77 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.5-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C78 Void |  |
| C79 Void |  |
| C80 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C80a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-1/49 AND A.4.4-1/103 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C81F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1a/8 AND [8]A.2/1 AND [8]A.3/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C81T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1b/8 AND [8]A.2/1 AND [8]A.3/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C82 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.5-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C83 Void |  |
| C84 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND [8]A.3/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C85 Void |  |
| C86 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/4 THEN R ELSE N/A |  |
| C86a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C87 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/5 THEN R ELSE N/A |  |
| C87a Void |  |
| C87b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C88 Void |  |
| C89 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/29 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C90F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C90T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C91F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C91T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C92F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C92T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C93F IF A.4.1-1/1 AND A.4.1-1/4 AND A.4.5-1a/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C93T IF A.4.1-1/2 AND A.4.1-1/4 AND A.4.5-1b/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C94 Void |  |
| C95 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C96F IF A.4.1-1/1 AND A.4.5-1a/10 AND A.4.4-2/2 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C96T IF A.4.1-1/2 AND A.4.5-1b/10 AND A.4.4-2/2 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C97 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/30 THEN R ELSE N/A |  |
| C97A IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/30 AND A.4.4-2/16 THEN R ELSE N/A |  |
| C98 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/18 AND A.4.4-1/30 THEN R ELSE N/A |  |
| C99F IF A.4.1-1/1 AND A.4.4-1/51 AND A.4.5-1a/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C99T IF A.4.1-1/2 AND A.4.4-1/51 AND A.4.5-1b/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C100F IF A.4.1-1/1 AND A.4.4-1/50 AND A.4.5-1a/7 THEN R ELSE N/A |  |
| C100T IF A.4.1-1/2 AND A.4.4-1/50 AND A.4.5-1b/7 THEN R ELSE N/A |  |
| C101 Void |  |
| C102 Void |  |
| C103 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/1 OR A.4.3.2-2/1) THEN R ELSE N/A |  |
| C104 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-1/31 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C105F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1a/8 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C105T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1b/8 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C106 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/34 AND A.4.4-2/2 THEN R ELSE N/A |  |
| C107F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.4-1/52 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C107T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.4-1/52 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C108 Void |  |
| C109 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND (A.4.4-1/35 OR A.4.4-1/36) THEN R ELSE N/A |  |
| C109a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND (A.4.4-1/35 OR A.4.4-1/36) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C110F IF A.4.1-1/1 AND A.4.4-1/52 AND A.4.4-2/2 AND A.4.5-1a/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C110T IF A.4.1-1/2 AND A.4.4-1/52 AND A.4.4-2/2 AND A.4.5-1b/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C111F IF A.4.1-1/1 AND A.4.4-1/38 AND A.4.4-2/2 AND A.4.4-1/52 AND A.4.5-1a/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C111T IF A.4.1-1/2 AND A.4.4-1/38 AND A.4.4-2/2 AND A.4.4-1/52 AND A.4.5-1b/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C112F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/7 AND A.4.5-1a/8 AND A.4.5-1a/22 AND A.4.5-1a/27 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C112T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/7 AND A.4.5-1b/8 AND A.4.5-1b/22 AND A.4.5-1b/27 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C113 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/5 THEN R ELSE N/A |  |
| C113a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113bF IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113bT IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113cF IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113cT IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113dF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113dT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113e IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A |  |
| C113gF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 AND A.4.3.3.3-2/2 THEN R ELSE N/A |  |
| C113gT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 AND A.4.3.3.3-2/2 THEN R ELSE N/A |  |
| C114 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/39 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C115 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C116 Void |  |
| C117F IF A.4.1-1/1 AND A.4.1-1/6 AND (([8]A.18a/14 AND [8]A.18a/18 AND [8]A.18a/22) OR ([8]A.18b/10 AND [8]A.18b/14)) AND A.4.5-1a/8 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C117T IF A.4.1-1/2 AND A.4.1-1/6 AND (([8]A.18a/14 AND [8]A.18a/18) OR ([8]A.18b/10 AND [8]A.18b/14)) AND A.4.5-1b/8 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C118F IF A.4.1-1/1 AND A.4.4-1/2 AND A.4.4-1/104 AND A.4.5-1a/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C118T IF A.4.1-1/2 AND A.4.4-1/2 AND A.4.4-1/104 AND A.4.5-1b/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C119F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.4-1/100 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C119T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.4-1/100 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C120F IF A.4.1-1/1 AND A.4.5-1a/7 AND A.4.4-1/40 AND A.4.4-1/41 THEN R ELSE N/A |  |
| C120T IF A.4.1-1/2 AND A.4.5-1b/7 AND A.4.4-1/40 AND A.4.4-1/41 THEN R ELSE N/A |  |
| C121 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C122 Void |  |
| C123 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-2/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C124 Void |  |
| C125 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND (A.4.4-2/5 OR (A.4.4-2/4 AND A.4.4-1/33)) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C126 IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.4-1/56 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C127 IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.4-1/57 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C128 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND (A.4.1-1/6 OR A.4.1-1/7) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C129 IF A.4.1-1/1 AND A.4.4-1/58 THEN R ELSE N/A |  |
| C129a IF A.4.1-1/1 AND A.4.4-1/58 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C130 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.5-1a/25 AND A.4.5-1b/25 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C131 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND NOT A.4.4-1/57 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C132 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C132a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.2-1/1 THEN R ELSE N/A |  |
| C133 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-2/1 OR A.4.3.3.1-2/2) THEN R ELSE N/A |  |
| C134F IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-3a/11 THEN R ELSE N/A |  |
| C134T IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-3b/11 THEN R ELSE N/A |  |
| C134aF IF A.4.1-1/1 AND A.4.3.3.2-1/1 AND A.4.5-3a/11 THEN R ELSE N/A |  |
| C134aT IF A.4.1-1/2 AND A.4.3.3.2-1/1 AND A.4.5-3b/11 THEN R ELSE N/A |  |
| C135 Void |  |
| C136 Void |  |
| C137 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/62 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C138 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND A.4.4-1/62 AND A.4.5-2/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C139 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/32 AND A.4.2.1.1-1/4 AND (A.4.5-1a/27 or A.4.5-1b/27) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C140 Void |  |
| C141 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND A.4.4-2/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C142 IF A.4.1-1/1 AND A.4.1-1/2 THEN R ELSE N/A |  |
| C142a IF A.4.1-1/1 AND A.4.1-1/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C143 IF A.4.4-1/2 AND A.4.4-1/49 AND A.4.4-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C144F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C144T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C145 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/65 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C146 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C147 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/63 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C148F IF A.4.1-1/1 AND A.4.5-1a/23 AND A.4.4-1/29 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C148T IF A.4.1-1/2 AND A.4.5-1b/23 AND A.4.4-1/29 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C149 Void |  |
| C150 IF (((A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6) OR ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.1-1/7)) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C151 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 THEN R ELSE N/A |  |
| C152F IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-3a/11 THEN R ELSE N/A |  |
| C152T IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-3b/11 THEN R ELSE N/A |  |
| C153 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-2/2 AND A.4.4-1/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C154F IF A.4.1-1/1 AND A.4.5-3a/15 THEN R ELSE N/A |  |
| C154T IF A.4.1-1/2 AND A.4.5-3b/15 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C155F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-3a/12 AND A.4.4-1/8 AND A.4.5-2/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C155T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-3b/12 AND A.4.4-1/53 AND A.4.5-2/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C155aF IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-3a/12 AND A.4.4-1/8 AND A.4.5-2/2 AND A.4.3.3.3-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C155aT IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-3b/12 AND A.4.4-1/53 AND A.4.5-2/2 AND A.4.3.3.3-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C155bF IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-3a/12 AND A.4.4-1/8 AND A.4.5-2/2 AND A.4.3.3.2-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C155bT IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-3b/12 AND A.4.4-1/53 AND A.4.5-2/2 AND A.4.3.3.2-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C156 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C157 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/69 THEN R ELSE N/A |  |
| C157a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/69 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16))THEN R ELSE N/A |  |
| C157b IF A.4.1-1/2 AND A.4.4-1/69 AND A.4.3.2-2A/2 AND A.4.3.2-3A/2 THEN R ELSE N/A |  |
| C158 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/70 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C159F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C159T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C160F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/7 AND A.4.5-1a/8 AND A.4.5-1a/22 AND A.4.5-1a/27 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C160T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/7 AND A.4.5-1b/8 AND A.4.5-1b/22 AND A.4.5-1b/27 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C161F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C161T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C162 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.3.3.3-2/1 AND A.4.3.3.3-2/2 THEN R ELSE N/A |  |
| C163 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/29 AND A.4.4-1/62 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C164 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/72 AND A.4.4-2/2 AND NOT (A.4.4-2/32) THEN R ELSE N/A |  |
| C165 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/3 AND A.4.4-1/62 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C166F IF A.4.1-1/1 AND A.4.5-1a/14 THEN R ELSE N/A |  |
| C166T IF A.4.1-1/2 AND A.4.5-1b/14 THEN R ELSE N/A |  |
| C167F IF A.4.1-1/1 AND A.4.5-1a/14 AND A.4.5-1a/25 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C167T IF A.4.1-1/2 AND A.4.5-1b/14 AND A.4.5-1b/25 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C168F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/15 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C168T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/15 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C169 Void |  |
| C170 IF A.4.1-1/1 AND A.4.4-1/76 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C171 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/79 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C172 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.4-1/37 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C173 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/80 AND A.4.4-2/1 THEN R ELSE N/A |  |
| C174 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/81 THEN R ELSE N/A |  |
| C175 IF A.4.1-1/2 AND A.4.4-1A/2 THEN R ELSE N/A |  |
| C176 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-1/1 THEN R ELSE N/A |  |
| C177 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND NOT A.4.3.2-1/1 THEN R ELSE N/A |  |
| C178 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 THEN R ELSE N/A |  |
| C179 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/84 AND NOT A.4.4-1/138 THEN R ELSE N/A |  |
| C179a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/84 AND NOT (A.4.4-1/138) AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16))THEN R ELSE N/A |  |
| C180 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/63 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C181 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/85 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C182 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND [8]A.2/2 AND NOT A.4.2.1.1-1/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C183 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.4-1/33 OR A.4.4-1/145) THEN R ELSE N/A |  |
| C184 IF ((A.4.1-1/1 AND A.4.1-2/1) OR (A.4.1-1/2 AND A.4.1-2/2)) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C184a IF ((A.4.1-1/1 AND A.4.1-2/1) OR (A.4.1-1/2 AND A.4.1-2/2)) AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C185F IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.1-2/1 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C185T IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.1-2/2 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C186F IF (A.4.1-1/1 AND A.4.5-1a/25 AND A.4.1-2/1) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/25) THEN R ELSE N/A |  |
| C186T IF (A.4.1-1/2 AND A.4.5-1b/25 AND A.4.1-2/2) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/25) THEN R ELSE N/A |  |
| C187 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/86 THEN R ELSE N/A |  |
| C188 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/87 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C189F IF A.4.1-1/1 AND A.4.5-1a/31 THEN R ELSE N/A |  |
| C189T IF A.4.1-1/2 AND A.4.5-1b/31 THEN R ELSE N/A |  |
| C189aF IF A.4.1-1/1 AND A.4.5-1a/31 AND [8]A.1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C189aT IF A.4.1-1/2 AND A.4.5-1b/31 AND [8]A.1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C189bF IF A.4.1-1/1 AND A.4.5-1a/31 AND((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C189bT IF A.4.1-1/2 AND A.4.5-1b/31 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C189cF IF A.4.1-1/1 AND A.4.5-1a/31 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C189cT IF A.4.1-1/2 AND A.4.5-1b/31 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C190 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-2/1 OR A.4.3.3.1-2/2) AND A.4.4-1A/3 THEN R ELSE N/A |  |
| C191 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.3.3.3-2/1 AND A.4.4-1A/3 AND A.4.3.3.3-2/2 THEN R ELSE N/A |  |
| C192 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.2-1/1 AND A.4.3.3.2-2/1 AND A.4.4-1A/3 THEN R ELSE N/A |  |
| C193F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C193T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C194 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 AND A.4.4-1A/4 THEN R ELSE N/A |  |
| C195 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 AND [8]A.10/37 AND A.4.4-2/1 THEN R ELSE N/A |  |
| C196 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/19 AND A.4.4-1/54 AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A |  |
| C197 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/4 AND [8]A.10/31 AND A.4.4-1/91 AND A.4.4-2/1 THEN R ELSE N/A |  |
| C198F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C198T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C199F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C199T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C200F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C200T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C201F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C201T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C202F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C202T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C203 Void |  |
| C203a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/62 AND A.4.4-1/63 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C204 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/30 AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A |  |
| C205 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND A.4.4-1/94 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C206F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/5 AND A.4.5-1d/2 AND A.4.5-1a/23 THEN R ELSE N/A |  |
| C206T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/5 AND A.4.5-1e/2 AND A.4.5-1b/23 THEN R ELSE N/A |  |
| C207 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.2-1/1 AND A.4.3.3.2-2/1 THEN R ELSE N/A |  |
| C208 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C209 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/33 AND (A.4.4-2/14 OR A.4.4-2/15) THEN R ELSE N/A |  |
| C210 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/33 AND (A.4.4-2/11 OR A.4.4-2/13) AND NOT (A.4.4-2/14) THEN R ELSE N/A |  |
| C211 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/33 AND A.4.4-2/14 THEN R ELSE N/A |  |
| C212 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/97 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C212a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/97 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C213 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/98 THEN R ELSE N/A |  |
| C214 Void |  |
| C215 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/99 THEN R ELSE N/A |  |
| C216F IF A.4.1-1/1 AND A.4.5-1a/4 AND A.4.5-1a/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C216T IF A.4.1-1/2 AND A.4.5-1b/4 AND A.4.5-1b/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C217 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/33 AND [45]A.12/40 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C218 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/33 AND [45]A.12/40 AND [45]A.12/41 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C219 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/33 AND [45]A.12/40 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C220 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/33 AND [45]A.12/40 AND [45]A.12/41 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C221 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2 OR A.4.3.3.2-1/1 OR A.4.3.3.3-1/1) AND A.4.4-1/101 AND NOT A.4.4-1/102 THEN R ELSE N/A |  |
| C222 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2 OR A.4.3.3.2-1/1 OR A.4.3.3.3-1/1) AND A.4.4-1/101 AND A.4.4-1/102 THEN R ELSE N/A |  |
| C223 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.3A/50 AND [45]A.4/2B AND [45]A.15/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C224 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.2-2/1 THEN R ELSE N/A |  |
| C224a IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT (A.4.3.2-2/1 OR A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C224b IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-2/1 OR A.4.3.2-2A/1) THEN R ELSE N/A |  |
| C224c IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C224d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4/183 THEN R ELSE N/A |  |
| C225 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/8 AND A.4.4-1/30 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C225a IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2 OR A.4.3.3.2-1/1 OR A.4.3.3.3-1/1) AND A.4.2.1.1-1/8 AND A.4.4-1/30 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C226 Void |  |
| C227 IF A.4.1-1/1 AND A.4.4-1/51 AND A.4.4-1/107 AND A.4.5-1a/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C228 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/51 AND NOT A.4.3.2-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C228a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/51 AND A.4.3.2-2/1 THEN R ELSE N/A |  |
| C229 IF A.4.1-1/1 AND NOT A.4.5-1a/31 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C229a IF A.4.1-1/1 AND NOT A.4.5-1a/31 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C230 IF A.4.1-1/2 AND NOT A.4.5-1b/31 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C230a IF A.4.1-1/2 AND NOT A.4.5-1b/31 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C231 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/32 AND A.4.2.1.1-1/4 AND (A.4.5-1a/9 or A.4.5-1b/9) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C232 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/33 AND [45]A.12/40 AND A.4.4-1/30 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C233 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/2 AND A.4.3.3-2/2 AND (A.4.4-1/108 OR A.4.4-1/109) AND A.4.4-1A/3 THEN R ELSE N/A |  |
| C234 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.3.3-2/1 AND A.4.4-1/108 THEN R ELSE N/A |  |
| C234a IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.4-1/108 THEN R ELSE N/A |  |
| C235 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.3.3-2/1 AND A.4.4-1/109 THEN R ELSE N/A |  |
| C235a IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.4-1/109 THEN R ELSE N/A |  |
| C236 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.3A/50 AND [45]A.4/2B AND [45]A.15/1 THEN R ELSE N/A |  |
| C237 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.3A/50 AND [45]A.4/2B AND [45]A.15/1 AND [45]A.15/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C238 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/110 THEN R ELSE N/A |  |
| C239 Void |  |
| C240 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/120 THEN R ELSE N/A |  |
| C241 Void |  |
| C242 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.3.3.3-2/2 THEN R ELSE N/A |  |
| C243 Void |  |
| C244 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/9 THEN R ELSE N/A |  |
| C245 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/10 THEN R ELSE N/A |  |
| C246 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/9 AND A.4.2.1.1-1/10 THEN R ELSE N/A |  |
| C247 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 AND A.4.4-1/115 THEN R ELSE N/A |  |
| C248 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/11 OR A.4.3.2-1/12 OR A.4.3.2-2/6 OR A.4.3.2-2/7 OR A.4.3.2-2/8 OR A.4.3.2-2/9 OR A.4.3.2-2/10 OR A.4.3.2-2/11 OR A.4.3.2-2/12 OR A.4.3.2-2/13 OR A.4.3.2-2/14 OR A.4.3.2-2/15 OR A.4.3.2-2/16) AND A.4.4-1/116 THEN R ELSE N/A |  |
| C249 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-1/33 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C250 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 AND A.4.4-2/1 THEN R ELSE N/A |  |
| C251 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/118 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C252 VOID |  |
| C253 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/121 AND A.4.4-1/115 THEN R ELSE N/A |  |
| C254 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A |  |
| C254a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 THEN R ELSE N/A |  |
| C254b IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.4-1/122 OR A.4.4-1/123) AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C254c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 THEN R ELSE N/A |  |
| C254e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND NOT A.4.3.2-2A/3 THEN R ELSE N/A |  |
| C254e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.2-2A/3 THEN R ELSE N/A |  |
| C255 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/123 THEN R ELSE N/A |  |
| C255a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/123 AND NOT A.4.3.2-2A/3 THEN R ELSE N/A |  |
| C255b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/123 AND A.4.3.2-2A/3 THEN R ELSE N/A |  |
| C256 IF A.4.1-1/2 AND A.4.4-1/124 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C257 IF A.4.1-1/1 AND A.4.5-1a/31 AND A.4.4-1/125 AND A.4.3.3.3-1/1 THEN R ELSE N/A |  |
| C258 IF A.4.1-1/2 AND A.4.5-1b/31 AND A.4.4-1/125 AND A.4.3.3.3-1/1 THEN R ELSE N/A |  |
| C259 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/11 THEN R ELSE N/A |  |
| C259cF IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A |  |
| C259cT IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A |  |
| C259dF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A |  |
| C259dT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A |  |
| C259e IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A |  |
| C259f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A |  |
| C259gF IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A |  |
| C259gT IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A |  |
| C259hF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A |  |
| C259hT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A |  |
| C260 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/128 THEN R ELSE N/A |  |
| C261 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/4 AND [8]A.10/31 AND A.4.4-2/1 THEN R ELSE N/A |  |
| C262 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/121 THEN R ELSE N/A |  |
| C263 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/121 AND A.4.2.1.1-1/4 THEN R ELSE N/A |  |
| C264 IF A.4.1-1/2 AND A.4.4-1/124 AND A.4.3.3.3-1/1 THEN R ELSE N/A |  |
| C265 IF A.4.1-1/2 AND A.4.4-1/124 AND A.4.3.3.3-2/1 THEN R ELSE N/A |  |
| C266 IF A.4.1-1/8 OR A.4.1-1/9 THEN R ELSE N/A |  |
| C266a IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/98 THEN R ELSE N/A |  |
| C267 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/12 THEN R ELSE N/A |  |
| C268 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/7 AND A.4.4-1A/8 THEN R ELSE N/A |  |
| C269 IF A.4.1-1/5 AND A.4.4-1/117 THEN R ELSE N/A |  |
| C270 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4 -1/131 THEN R ELSE NA |  |
| C271 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/199 THEN R ELSE N/A |  |
| C272 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/99 THEN R ELSE N/A |  |
| C273 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/121 THEN R ELSE N/A |  |
| C274 IF (A.4.1-1/1 OR A.4.1-1/2 ) AND A.4.2.1.1-1/13 THEN R ELSE N/A |  |
| C275 IF (A.4.1-1/8 OR A.4.1-1/9) AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A |  |
| C276 Void |  |
| C277 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/30 AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A |  |
| C278 Void |  |
| C279 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/129 AND A.4.4-1/130 THEN R ELSE N/A |  |
| C280 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/129 THEN R ELSE N/A |  |
| C281 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.4-1/139 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A |  |
| C282 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/140 THEN R ELSE N/A |  |
| C283 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.20/35 AND NOT A.4.4-1/25 THEN R ELSE N/A |  |
| C284 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/143 THEN R ELSE N/A |  |
| C285 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/132 THEN R ELSE N/A |  |
| C286 IF(A.4.1-1/1 OR A.4.1-1/2) AND NOT (A.4.3.2-2A/1) AND A.4.4-1/2 AND A.4.4-2/1 THEN R ELSE N/A |  |
| C287 IF(A.4.1-1/1 OR A.4.1-1/2) AND NOT (A.4.3.2-2A/1) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/5 THEN R ELSE N/A |  |
| C288 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1A/10 THEN R ELSE N/A |  |
| C289 Void |  |
| C290 IF (A.4.1-1/8 OR A.4.1-1/9) AND (A.4.4-1/132 OR A.4.4-1/144) THEN R ELSE N/A |  |
| C291 IF (A.4.1-1/8 OR A.4.1-1/9) AND (A.4.4-1/132 OR A.4.4-1/144) AND A.4.4-1/99 THEN R ELSE N/A |  |
| C292 Void |  |
| C293 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-2/24 AND A.4.4-1/19 THEN R ELSE N/A |  |
| C294 Void |  |
| C295 IF(A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/14 THEN R ELSE N/A |  |
| C296 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/5 OR A.4.3.2-1/6 OR A.4.3.2-1/7 OR A.4.3.2-1/9 OR A.4.3.2-1/10 OR A.4.3.2-1/11 OR A.4.3.2-1/12 OR A.4.3.2-2/10 OR A.4.3.2-2/11 OR A.4.3.2-2/13 OR A.4.3.2-2/14 OR A.4.3.2-2/15 OR A.4.3.2-2/16) AND A.4.4-1/159 THEN R ELSE N/A |  |
| C297 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/11 OR A.4.3.2-1/12 OR A.4.3.2-2/8 OR A.4.3.2-2/10 OR A.4.3.2-2/11 OR A.4.3.2-2/13 OR A.4.3.2-2/14 OR A.4.3.2-2/15 OR A.4.3.2-2/16) AND A.4.4-1/159 AND A.4.4-1/116 THEN R ELSE N/A |  |
| C298 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/160 THEN R ELSE N/A |  |
| C299 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/161 THEN R ELSE N/A |  |
| C300 Void |  |
| C301 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3-1/1 OR A.4.3.3-1/2 OR A.4.3.3-1/3 OR A.4.3.3-1/4) AND (A.4.3.3-2/1 OR A.4.3.3-2/2) AND A.4.4-1/163 THEN R ELSE N/A |  |
| C302 IF A.4.4-1/148 THEN R ELSE N/A |  |
| C303 IF A.4.4-1/148 AND A.4.4-1/153 THEN R ELSE N/A |  |
| C304 IF A.4.4-1/148 AND A.4.4-1/155 THEN R ELSE N/A |  |
| C305 IF A.4.4-1/148 AND A.4.4-1/156 THEN R ELSE N/A |  |
| C306 IF A.4.4-1/148 AND A.4.4-1/157 THEN R ELSE N/A |  |
| C307 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 THEN R ELSE N/A |  |
| C308 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/152 THEN R ELSE N/A |  |
| C309 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/153 THEN R ELSE N/A |  |
| C310 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/155 THEN R ELSE N/A |  |
| C311 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/156 THEN R ELSE N/A |  |
| C312 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/157 THEN R ELSE N/A |  |
| C313 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/164 |  |
| C314 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A |  |
| C314a IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/17 THEN R ELSE N/A |  |
| C315 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 AND [8]A.10/19 THEN R ELSE N/A |  |
| C316 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/54 AND [8]A.10/17 AND A.4.2.1.1-1/4 THEN R ELSE N/A |  |
| C317 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/55 AND [8]A.10/17 THEN R ELSE N/A |  |
| C318 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A |  |
| C319 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A |  |
| C320 IF A.4.1-1/1 AND A.4.3.3-1/1 AND A.4.4-1/109 AND A.4.4-1/166 THEN R ELSE N/A |  |
| C321 IF A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.4-1/166 THEN R ELSE N/A |  |
| C322 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/165 THEN R ELSE N/A |  |
| C323 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/8 OR A.4.1-1/9) THEN R ELSE N/A |  |
| C324 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/120 AND A.4.4-1/169 THEN R ELSE N/A |  |
| C325 IF A.4.4-1/173 THEN R ELSE N/A |  |
| C326 IF A.4.4-1/172 THEN R ELSE N/A |  |
| C327 IF (A.4.4-1/170 OR A.4.4-1/171) THEN R ELSE N/A |  |
| C328 IF A.4.4-1/148 AND A.4.4-1/153 AND A.4.4-1/156 THEN R ELSE N/A |  |
| C329 Void |  |
| C330 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 THEN R ELSE N/A |  |
| C331 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 AND A.4.4-1/70 THEN R ELSE N/A |  |
| C332 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 AND A.4.4-1/176 THEN R ELSE N/A |  |
| C333 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 AND A.4.4-1/70 AND A.4.4-1/176 THEN R ELSE N/A |  |
| C334 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/161 THEN R ELSE N/A |  |
| C335 Void |  |
| C336 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/149 AND A.4.4-1/177 THEN R ELSE N/A |  |
| C337 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/5 AND A.4.4-1/149 THEN R ELSE N/A |  |
| C338 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/11 AND A.4.4-1/149 THEN R ELSE N/A |  |
| C339 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/167 AND A.4.3.2-1A/2 THEN R ELSE N/A |  |
| C340 Void |  |
| C341 Void |  |
| C342 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-2/27 AND A.4.4-2/31 THEN R ELSE N/A |  |
| C343 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/154 AND A.4.4-1/178 THEN R ELSE N/A |  |
| C344 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND NOT(A.4.4-1/154) AND A.4.4-1/178 THEN R ELSE N/A |  |
| C345 IF A.4.4-1/148 AND A.4.4-1/178 THEN R ELSE N/A |  |
| C346 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/178 THEN R ELSE N/A |  |
| C347 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.3.2-1A/2 THEN R ELSE N/A |  |
| C348 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1A/11 THEN R ELSE N/A |  |
| C349 IF A.4.1-1/8 AND A.4.4-1A/12 THEN R ELSE N/A |  |
| C350 IF A.4.1-1/8 AND A.4.2.1.1-1/15 THEN R ELSE N/A |  |
| C351 IF A.4.1-1/8 AND A.4.2.1.1-1/11 AND (A.4.5-1a/3 or A.4.5-1b/3) AND (A.4.5-1a/7 or A.4.5-1b/7) THEN R ELSE N/A |  |
| C352 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/179 THEN R ELSE N/A |  |
| C353 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/179 AND A.4.4-1/180 THEN R ELSE N/A |  |
| C354 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/11 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A |  |
| C355 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/181 THEN R ELSE N/A |  |
| C356 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/182 THEN R ELSE N/A |  |
| C357 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/33 THEN R ELSE N/A |  |
| C358 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/184 THEN R ELSE N/A |  |
| C359 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/185 THEN R ELSE N/A |  |
| C360 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/186 THEN R ELSE N/A |  |
| C361 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/187 THEN R ELSE N/A |  |
| C362 IF A.4.1-1/1 AND A.4.5-1a/7 OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/7) THEN R ELSE N/A |  |
| C363 IF A.4.1-1/2 AND A.4.5-1b/7 OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/7) THEN R ELSE N/A |  |
| C364 IF (A.4.1-1/1 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.5-1a/25) THEN R ELSE N/A |  |
| C365 IF (A.4.1-1/2 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.5-1b/25) THEN R ELSE N/A |  |
| C366 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND [8] A. 20/90 THEN R ELSE N/A |  |
| C367 IF A.4.1-1/1 AND A.4.4-1/122 AND A.4.4-1/188 THEN R ELSE N/A |  |
| C368 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/189 AND A.4.4-1/190 THEN R ELSE N/A |  |
| C369 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/191 THEN R ELSE N/A |  |
| C370 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/192 THEN R ELSE N/A |  |
| C371 Void |  |
| C372 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/195 THEN R ELSE N/A |  |
| C373 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND (A.4.4-1/196 OR A.4.4-1/197) THEN R ELSE N/A |  |
| C374 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.4-1/197 THEN R ELSE N/A |  |
| C375 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.4-1/198 THEN R ELSE N/A |  |
| C376 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/143 AND A.4.4-1/200 THEN R ELSE N/A |  |
| C377 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/202 AND (A.4.4-1/132 OR A.4.4-1/144) THEN R ELSE N/A |  |
| C378 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/204 THEN R ELSE N/A |  |
| C379 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/204 AND NOT A.4.4-1/206 THEN R ELSE N/A |  |
| C379a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/205 THEN R ELSE N/A |  |
| C380 IF A.4.1-1/1 AND A.4.4-1/203 AND A.4.4-1/206 THEN R ELSE N/A |  |
| C381 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/205 AND A.4.4-1/207 THEN R ELSE N/A |  |
| C382 IF A.4.1-1/2 AND A.4.4-1/203 AND A.4.4-1/205 AND A.4.4-1/208 THEN R ELSE N/A |  |
| C383 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/204 AND A.4.4-1/205 AND A.4.4-1/209 THEN R ELSE N/A |  |
| C384 IF (A.4.1-1/1) AND (A.4.4-1/122 OR A.4.4-1/123) AND (A.4.4-1/210) THEN R ELSE N/A |  |
| C385 IF (A.4.1-1/1) AND (A.4.4-1/122 OR A.4.4-1/123) AND A.4.4-1/121 AND (A.4.4-1/210) THEN R ELSE N/A |  |
| C386 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/4 AND A.4.4-2/1 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A |  |
| C387 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/143 AND A.4.4-1/201 THEN R ELSE N/A |  |
| C388 IF (A.4.1-1/1 OR A.4.1-1/2) AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C389 IF A.4.1-1/1 AND A.4.1-1/2 AND ((NOT A.4.3.2-2A/1) OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A |  |
| C390 IF A.4.1-1/8 AND A.4.4-1/210 THEN R ELSE N/A |  |
| C391 IF A.4.1-1/8 AND A.4.4-1/121 AND A.4.4-1/210 THEN R ELSE N/A |  |
| C392 IF A.4.1-1/8 AND A.4.4-1/212 THEN R ELSE N/A |  |
| C393 IF A.4.1-1/1 AND A.4.4-1/51 AND A.4.5-1a/7 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A |  |
| C394 IF A.4.1-1/2 AND A.4.4-1/51 AND A.4.5-1b/7 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A |  |
| C395 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1a/7 AND A.4.4-1/213 THEN R ELSE N/A |  |
| C396 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/199 AND (A.4.4-1/41 OR A.4.4-1/42 OR A.4.4-1/43 OR A.4.4-1/44 OR A.4.4-1/46 OR A.4.4-1/47 OR A.4.4-1/48) THEN R ELSE N/A |  |
| C397 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/1 THEN R ELSE N/A |  |
| C398 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/214 THEN R ELSE N/A |  |
| C399 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/217 THEN R ELSE N/A |  |
| C400 IF A.4.1-1/8 AND A.4.4-1/218 THEN R ELSE N/A |  |
| C401 Void |  |
| C402 IF A.4.1-1/8 AND A.4.4-1/219THEN R ELSE N/A |  |
| C403 IF A.4.1-1/9 AND A.4.4-1A/17 THEN R ELSE N/A |  |
| C404 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/220 THEN R ELSE N/A |  |
| C405 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/216 |  |
| C406 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/222 THEN R ELSE N/A |  |
| C407 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/223 THEN R ELSE N/A |  |
| C408 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.4-1/215 THEN R ELSE N/A |  |
| C409 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/3 THEN R ELSE N/A |  |
| C410 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/4 THEN R ELSE N/A |  |
| C411 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 AND A.4.4-1/224 THEN R ELSE N/A |  |
| C412 IF A.4.1-1/8 AND A.4.4-1/230 AND A.4.4-1/234 THEN R ELSE N/A |  |
| C413 IF A.4.1-1/8 AND A.4.4-1/230 AND A.4.4-1/234 AND A.4.4-1/231 AND A.4.4-1/233 THEN R ELSE N/A |  |
| C414 IF A.4.1-1/1 AND (A.4.3.2-2A/1 OR A.4.3.2-2A/3) AND A.4.4-1/230 AND A.4.4-1/234 THEN R ELSE N/A |  |
| C415 IF A.4.1-1/1 AND (A.4.3.2-2A/1 OR A.4.3.2-2A/3) AND A.4.4-1/230 AND A.4.4-1/234 AND A.4.4-1/231 AND A.4.4-1/233 THEN R ELSE N/A |  |
| C416 IF A.4.4-1/25 AND A.4.4-1/235 AND A.4.4-1/117 THEN R ELSE N/A |  |
| C4XXH IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/ ZZZH THEN R ELSE N/A |  |
| C4XYH IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/ ZZZH AND A.4.4-1/99 THEN R ELSE N/A |  |

<End of modified section 2>