3GPP TSG-RAN5 Meeting #98	R5-231001

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2730 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE CellGroupConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align the IE CellGroupConfig with non used extension groups in default information elements contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The non used extension groups have been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IE CellGroupConfig remains inconsistent and includes non used ASN.1 fields. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	CellGroupConfig

Table 4.6.3-19: CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId |  |  |
|  | CellGroupId condition NR-DC_SCG |  | NR-DC_SCG |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | EN-DC |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 | entry 1 |  |
|  | RLC-BearerConfig with conditions AM and DRB2 and Re-establish_RLC |  | PSCell_change |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | SRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB1 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB2 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB2 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | DRBn, NR-DC_SCG |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRBn | entry 1 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | PCell_change |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRB1 and Re-establish_RLC | entry 3 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1+n entries | n is the number of DRBs established before RRC resume | RESUME |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 and RESUME | entry 1 |  |
| RLC-BearerConfig[k+1, k=1..n] | RLC-BearerConfig with condition DRBk and RESUME | entry [k+1, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n entries | n is the number of DRBs established before RRC re-establishement | REEST |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with condition DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | MRBm AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM |  | UM_PTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm_DRBn AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM |  | UM_PTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRBn | entry 2 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM | entry 2 | AM_PTP_UM_PTM, UMPTP_UMPTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | MRBm_DRBn AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM | entry 2 | AM_PTP_UM_PTM, UMPTP_UMPTM |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig |  |  |
|  | MAC-CellGroupConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add |
| physicalCellGroupConfig | PhysicalCellGroupConfig |  |  |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add, MRBm, MRBm_DRBn |
| spCellConfig | Not present |  | SRB2_DRB1, SRB2_DRB2, SCell_add, AM_PTP, UM_bi_PTP, UM_PTP |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC, EN-DC AND MEAS |
|  | ServCellIndex with condition NR-DC_SCG |  | NR-DC_SCG |
| reconfigurationWithSync | Not present |  |  |
| reconfigurationWithSync SEQUENCE { |  |  | EN-DC, PCell_change, PSCell_change, NR-DC_SCG |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| newUE-Identity | RNTI-Value |  |  |
|  | RNTI-Value with condition NR-DC_SCG |  | NR-DC_SCG |
| t304 | ms1000 |  |  |
| rach-ConfigDedicated | Not present |  |  |
| rach-ConfigDedicated CHOICE { |  |  | CFRA |
| uplink | RACH-ConfigDedicated |  |  |
| supplementaryUplink | RACH-ConfigDedicated |  | SUL AND SIG |
| } |  |  |  |
| } |  |  |  |
| rlf-TimersAndConstants CHOICE { |  |  |  |
| setup | RLF-TimersAndConstants |  |  |
| } |  |  |  |
| rlf-TimersAndConstants | Not present |  | MEAS, RESUME |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig |  | EN-DC, SRB1, PCell_change, PSCell_change, NR-DC_SCG, REEST |
|  | Not present |  |  |
|  | ServingCellConfig with condition MEAS |  | MEAS |
|  | ServingCellConfig with condition RESUME |  | RESUME |
|  | ServingCellConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
| } |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  | SCell_add |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL |  |  |
|  | ServingCellConfigCommon |  | RF AND UL_CA |
| sCellConfigDedicated | ServingCellConfig with condition No_UL and Scell_Add |  |  |
|  | ServingCellConfig with condition Scell_Add |  | RF AND UL_CA |
| } |  |  |  |
| } |  |  |  |
| sCellToReleaseList | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| CFRA | This condition applies when CFRA is configured |
| SUL | Supplementary Uplink |
| SRB1 | Establishment of SRB1 |
| SRB2_DRB1 | Establishment of SRB2 and DRB1 |
| SRB2_DRB2 | Establishment of SRB2 and DRB2 |
| DRBn | Establishment of DRBn |
| PCell_change | Intra-NR PCell change NR) |
| PSCell_change | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| MEAS | A NR or EN-DC measurement is configured |
| NR-DC_SCG | Add SCG (NR-DC) |
| RESUME | Used in RRCResume Message |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure. |
| UL_CA | Uplink CA |
| MRBm | Establishment of MRBm |
| MRBm_DRBn | Establishment of MRBm and DRBn |
| AM_PTP | Multicast MRB with RLC-AM entity configuration for receiving PTP transmission |
| UM_bi_PTP | Multicast MRB with bidirectional RLC-UM configuration for receiving PTP transmission |
| UM_PTP | Multicast MRB with DL only RLC-UM configuration for receiving PTP transmission |
| UM_PTM | Multicast MRB with DL only RLC-UM entity for receiving PTM transmission |
| AMPTP_UMPTM | Multicast MRB with two RLC entities, one RLC-AM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |
| UMPTP_UMPTM | Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |

<End of modified section>