3GPP TSG-RAN5 Meeting #98	R5-231027

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1413 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of default configuration for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Common configurations of SIBs/SIB-NB need to be updated for NTN. - Common contents of SI/SI-NB messages need to be updated for NTN. - Common contents of SIB/SIB-NB messages need to be updated for NTN. - UE positioning needs to be pre-configured for NTN testing. - NB-IoT NTN duplex mode needs to be specified. - NB-IoT NTN operating bands needs to be added. - SIB31/SIB-31-NB needs to be cover the serving area of UE location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Common configurations of SIBs/SIB-NB were updated for NTN. - Common contents of SI/SI-NB messages were updated for NTN. - Common contents of SIB/SIB-NB messages were updated for NTN. - UE pre-configured location was defined for NTN testing. - NB-IoT NTN duplex mode was specified. - NB-IoT NTN operating bands was specified. - SIB31/SIB31-NB was updated to cover the serving area of UE location. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Default test configuration for NTN will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.1, 4.4.3.2, 4.4.3.3, 4.xx(new), 8.1.2, 8.1.3.1, 8.1.4.3.1, 8.1.4.3.2 and 8.1.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.4.3	Common parameters for simulated E-UTRA cells

The parameters specified in this sub clause apply to all simulated E-UTRA cells unless otherwise specified.

4.4.3.1	Common configurations of system information blocks

For BL/CE testing the same content of system information blocks SIB2 to SIB20 are used for broadcasting on BCCH and/or BCCH-BR.

4.4.3.1.1	Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario

- E-UTRA TDD single cell scenario

- E-UTRA FDD intra-frequency multi cell scenario

- E-UTRA TDD intra-frequency multi cell scenario

- E-UTRA FDD and E-UTRA TDD dual mode multi cell roaming scenario

- 3GPP inter-RAT E-UTRA FDD + NR FDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA FDD + NR TDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA TDD + NR FDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA TDD + NR TDD multi cell scenario for RRM

Combination 2 applies to the following test case scenarios:

- E-UTRA FDD intra-frequency multi cell scenario with neighbouring cell related information

- E-UTRA TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination 3 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency multi cell scenario

- E-UTRA TDD inter-frequency multi cell scenario

- E-UTRA FDD inter-band multi cell scenario

- E-UTRA TDD inter-band multi cell scenario

- E-UTRA FDD and E-UTRA TDD dual mode multi cell non-roaming scenario

- E-UTRA FDD intra-band carrier aggregation component carriers cell scenario

- E-UTRA FDD inter-band carrier aggregation component carriers cell scenario

- E-UTRA TDD intra-band carrier aggregation component carriers cell scenario

- E-UTRA FDD and E-UTRA TDD inter-band carrier aggregation component carriers cell scenario

- E-UTRA FDD dual connectivity cell scenario

- E-UTRA TDD dual connectivity cell scenario

Combination 4 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA FDD multi cell scenario

Combination 5 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + GERAN multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + GERAN multi cell scenario

Combination 6 applies to the following test case scenarios:

- 3GPP2 inter-RAT E-UTRA FDD + HRPD multi cell scenario

- 3GPP2 inter-RAT E-UTRA TDD + HRPD multi cell scenario

- 3GPP2 inter-RAT E-UTRA FDD + 1xRTT multi cell scenario

- 3GPP2 inter-RAT E-UTRA TDD + 1xRTT multi cell scenario

Combination 7 applies to the following test case scenarios:

- E-UTRA FDD + home eNB multi cell scenario

- E-UTRA TDD + home eNB multi cell scenario

Combination 8 applies to the following test case scenarios:

- E-UTRA FDD ETWS single cell scenario

- E-UTRA TDD ETWS single cell scenario

Combination 9 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + 3GPP inter-RAT UTRA multi-cell scenario

- E-UTRA TDD inter-frequency + 3GPP inter-RAT UTRA multi-cell scenario

Combination 10 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD + GERAN multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + GERAN multi cell scenario.

Combination 10a applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + 3GPP inter-RAT E-UTRA FDD + UTRA FDD + GERAN multi cell scenario

- E-UTRA TDD inter-frequency + 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + GERAN multi cell scenario

Combination 11 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD + home eNB multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + home eNB multi cell scenario

Combination 12 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + GERAN cell scenario

- E-UTRA TDD inter-frequency + GERAN cell scenario

Combination 13 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + home eNB

- E-UTRA TDD inter-frequency + home eNB

Combination 14 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + GERAN + home eNB multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + GERAN + home eNB multi cell scenario

Combination 15 applies to the following test case scenarios:

- 3GPP E-UTRA FDD single cell scenario + MBMS

- 3GPP E-UTRA TDD single cell scenario + MBMS

- 3GPP E-UTRA FDD intra-frequency multi cell scenario + MBMS

- 3GPP E-UTRA TDD intra-frequency multi cell scenario + MBMS

Combination 16 applies to the following test case scenarios:

- 3GPP E-UTRA FDD inter-frequency multi cell scenario + MBMS

- 3GPP E-UTRA TDD inter-frequency multi cell scenario + MBMS

Combination 17 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + CMAS single cell scenario

- 3GPP E-UTRA TDD + CMAS single cell scenario

Combination 18 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS inter-frequency multi cell scenario (non-MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-frequency multi cell scenario (non-MBMS cell broadcasting SIB15)

- 3GPP E-UTRA FDD + SC-PTM inter-frequency multi cell scenario (non- SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-frequency multi cell scenario (non- SC-PTM cell broadcasting SIB15)

Combination 19 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS intra-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS intra-frequency multi cell scenario (MBMS cell broadcasting SIB15)

Combination 20 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS inter-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA FDD + MBMS inter-band multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-band multi cell scenario (MBMS cell broadcasting SIB15)

Combination 21 applies to the following test case scenarios:

- E-UTRA FDD SIMTC single cell scenario

- E-UTRA TDD SIMTC single cell scenario

Combination 22 applies to the following test case scenarios:

- RAN assisted WLAN offload: E-UTRA FDD + WLAN AP scenarios

- RAN assisted WLAN offload: E-UTRA TDD + WLAN AP scenarios

Combination 23 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + sidelink direct communication

- E-UTRA TDD single cell scenario + sidelink direct communication

- E-UTRA FDD intra-frequency multi cell scenario + sidelink direct communication

- E-UTRA TDD intra-frequency multi cell scenario + sidelink direct communication

Combination 24 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + sidelink direct discovery

- E-UTRA TDD single cell scenario + sidelink direct discovery

- E-UTRA FDD intra-frequency multi cell scenario + sidelink direct discovery

- E-UTRA TDD intra-frequency multi cell scenario + sidelink direct discovery

Combination 25 applies to the following test case scenarios:

- 3GPP E-UTRA FDD single cell scenario + SC-PTM

- 3GPP E-UTRA TDD single cell scenario + SC-PTM

- 3GPP E-UTRA FDD intra-frequency multi cell scenario + SC-PTM

- 3GPP E-UTRA TDD intra-frequency multi cell scenario + SC-PTM

Combination 26 applies to the following test case scenarios:

- 3GPP E-UTRA FDD inter-frequency multi cell scenario + SC-PTM

- 3GPP E-UTRA TDD inter-frequency multi cell scenario + SC-PTM

Combination 27 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + SC-PTM intra-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM intra-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

Combination 28 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + SC-PTM inter-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA FDD + SC-PTM inter-band multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-band multi cell scenario (SC-PTM cell broadcasting SIB15)

Combination 29 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + V2X sidelink communication.

- E-UTRA TDD single cell scenario + V2X sidelink communication.

- E-UTRA FDD intra-frequency multi cell scenario + V2X sidelink communication.

- E-UTRA TDD intra-frequency multi cell scenario + V2X sidelink communication.

- E-UTRA FDD inter-frequency + V2X sidelink communication with inter-frequency scheduling

- E-UTRA TDD inter-frequency + V2X sidelink communication with inter-frequency scheduling

- E-UTRA FDD inter-band + V2X sidelink communication with inter-frequency scheduling

- E-UTRA TDD inter-band + V2X sidelink communication with inter-frequency scheduling

Combination 30 applies to the following test case scenarios:

- E-UTRA FDD intra-band carrier aggregation component carriers cell scenario + E-UTRA FDD intra-frequency neighbour.

- E-UTRA FDD inter-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

- E-UTRA TDD intra-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

- E-UTRA FDD and E-UTRA TDD inter-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

Combination 31 applies to the following test case scenarios:

-	3GPP inter-RAT E-UTRA FDD + NR FDD multi cell scenario

-	3GPP inter-RAT E-UTRA FDD + NR TDD multi cell scenario

-	3GPP inter-RAT E-UTRA TDD + NR FDD multi cell scenario

-	3GPP inter-RAT E-UTRA TDD + NR TDD multi cell scenario

Combination 32 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + GSO NTN

Combination 33 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD intra-frequency multi cell scenario with neighbouring cell related information + NGSO NTN

Combination 34 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + NGSO NTN

Table 4.4.3.1.1-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB14 | SIB15 | SIB17 | SIB18 | SIB19 | SIB20 | SIB21 | SIB24 | SIB31 |  | SIB32 |  |
| 1 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | X | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | X | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | X | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | X | X | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | X | X |  |  |  |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | X | X |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10a | X | X |  | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | X | X | X |  | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | X | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | X | X | X | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | X | X | X |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | X | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | X | X |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| 19 | X | X |  |  |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| 20 | X | X |  | X |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| 21 | X | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 23 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 24 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| 25 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| 26 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| 27 | X | X |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |
| 28 | X | X |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |
| 29 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 30 | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 32 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| 33 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  | X |
| 34 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  | X |

4.4.3.1.2	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables.

Table 4.4.3.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |

Table 4.4.3.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |

Table 4.4.3.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-4: Scheduling for combination 4

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB6 |

Table 4.4.3.1.2-5: Scheduling for combination 5

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-6: Scheduling for combination 6

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB8 |

Table 4.4.3.1.2-7: Scheduling for combination 7

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB11 |
| 4 | See sub clause 4.4.3.4 | SIB10 |

Table 4.4.3.1.2-9: Scheduling for combination 9

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5, SIB6 |

Table 4.4.3.1.2-10: Scheduling for combination 10

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB6 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-10a: Scheduling for combination 10a

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5, SIB6 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-11: Scheduling for combination 11

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB6 |
| 5 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-12: Scheduling for combination 12

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-13: Scheduling for combination 13

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-14: Scheduling for combination 14

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB7 |
| 5 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-15: Scheduling for combination 15

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB13 |

Table 4.4.3.1.2-16: Scheduling for combination 16

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB13 |

Table 4.4.3.1.2-17: Scheduling for combination 17

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB12 |

Table 4.4.3.1.2-18: Scheduling for combination 18

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-19: Scheduling for combination 19

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB13 |
| 4 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-20: Scheduling for combination 20

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB13 |
| 5 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-21: Scheduling for combination 21

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB14 |

Table 4.4.3.1.2-22: Scheduling for combination 22

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB17 |

Table 4.4.3.1.2-23: Scheduling for combination 23

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB18 |

Table 4.4.3.1.2-24: Scheduling for combination 24

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB19 |

Table 4.4.3.1.2-25: Scheduling for combination 25

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-26: Scheduling for combination 26

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-27: Scheduling for combination 27

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB15 |
| 4 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-28: Scheduling for combination 28

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB15 |
| 5 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-29: Scheduling for combination 29

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB21 |

Table 4.4.3.1.2-30: Scheduling for combination 30

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-31: Scheduling for combination 31

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB24 |

Table 4.4.3.1.2-32: Scheduling for combination 32

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB31 |

Table 4.4.3.1.2-33: Scheduling for combination 33

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB31 |
| 5 | See sub clause 4.4.3.4 | SIB32 |

Table 4.4.3.1.2-33: Scheduling for combination 34

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB31 |
| 4 | See sub clause 4.4.3.4 | SIB32 |

4.4.3.2	Common contents of system information messages

-	MasterInformationBlock

The MasterInformationBlock includes the system information transmitted on BCH.

Table 4.4.3.2-1: MasterInformationBlock

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock ::= SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| phich-Config | PHICH-Config-DEFAULT | See subclause 4.6.3 |  |
| systemFrameNumber | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-BR-r13 | 0 | Value 0 means that SystemInformationBlockType1-BR is not scheduled. |  |
|  | 10 | 4 PDSCH repetitions and504 bits TBS carrying for PDSCH carrying SystemInformationBlockType1-BR (TS 36.213 [23] Table 7.1.6.1 and Table 7.1.7.2.7-1) | CEmodeA, CEmodeB |
| systemInfoUnchanged-BR-r15 | FALSE |  |  |
| partEARFCN-17 CHOICE { |  |  |  |
| spare | '00'B |  |  |
| earfcn-LSB | 2 least significant bits of the EARFCN for NTN bands where 100 kHz raster is used |  | NTN |
| } |  |  |  |
| spare | '0'B |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |
| NTN | Non-Terrestrial Networks test environment |

[Skip unchanged text]

4.4.3.3	Common contents of system information blocks

-	SystemInformationBlockType2

[Skip unchanged text]

-	SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell.

Table 4.4.3.3-21: SystemInformationBlockType31

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | NTN |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | GSO |
| positionX-r17 | -26044672 |  |  |
| positionY-r17 | 19284189 |  |  |
| positionZ-r17 | -1364331 |  |  |
| velocityVX-r17 | 82 |  |  |
| velocityVY-r17 | -37 |  |  |
| velocityVZ-r17 | -2465 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 7850100 |  |  |
| nta-CommonDrift-r17 | 579 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  |  |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

-	SystemInformationBlockType32

The IE SystemInformationBlockType32 contains satellite assistance information for prediction of discontinuous coverage.

Table 4.4.3.3-22: SystemInformationBlockType32

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType32-r17 ::= SEQUENCE { |  |  |  |
| satelliteInfoList-r17 | FFS |  | NGSO |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NGSO | Non-geosynchronous Orbit scenario |

[Skip unchanged text]

4.12	GNSS Requirements for aerial testing

4.12.1	General

This clause defines the GNSS scenarios and requirements which apply for all aerial test cases that require simulated GNSS signals, unless otherwise specified.

The term SV ID used in this clause is defined as the satellite PRN for GPS and Modernized GPS, as Code Number for Galileo, as the satellite Slot Number for GLONASS and as the Ranging Code Number for BDS.

4.12.2	GNSS Scenarios

The PIXIT px_GnssScenario2012 is specified in TS 37.571-5 [68] clause 6.1.2.

If px_GnssScenario2012 = FALSE, the following GNSS scenarios shall be used.

-	Rinex navigationdata: the required navigation data file(s) available in the GNSS orbital data sig zip file specified in TS 37.571-5 [68] Annex B are given in Table 4.12.2-0.

Table 4.12.2-0: Rinex navigation data files for Aerial testing

| GNSS supported by UE | Rinex file(s) (1) |
| --- | --- |
| GPS | Sig GNSS GPS 2020_9_17 Rinex.txt |
| GLONASS | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| Galileo | Sig GNSS Galileo 2020_9_17 Rinex.txt |
| BDS | Sig GNSS BDS 2020_9_17 Rinex.txt |
| Note 1: Where the UE supports more than one GNSS then all the relevant Rinex navigation data files are used |  |

-	UE location(s) and motion:

Latitude: the simulated latitude is given in Table 4.12.2-2

Longitude: the simulated longitude is given in Table 4.12.2-2

Height: the simulated heights above sea level are given in Table 4.12.2-2

Motion: the simulated motion(s) are given in Table 4.12.2-1

Table 4.12.2-1: UE locations and motions for aerial testing

| Scenario number and description | Step # | Action (Location details given in Table 4.12.2-2) | Notes |
| --- | --- | --- | --- |
| Scenario #1: move from location #1 to location #2 to location #3 | 1 | Static at location #1 |  |
|  | 2 | Trigger from test case to move in a straight line at 5m/s from current location to next location |  |
|  | 3 | Static at location #2 |  |
|  | 4 | Trigger from test case to move in a straight line at 5m/s from current location to next location | Simulation will take 40 seconds to move between locations given the nominal values specified in the test case. An additional 10 seconds is added to allow for UE position update. Total time 50 seconds. |
|  | 5 | Static at location #3 |  |

Table 4.12.2-2: Location descriptions for aerial testing

| Location number | Latitude (degrees) | Longitude (degrees) | Height above sea level (m) | Notes |
| --- | --- | --- | --- | --- |
| #1 | 43.308318 | -5.697122 | See test case |  |
| #2 | 43.308318 | -5.697122 | See test case |  |
| #3 | 43.308318 | -5.697122 | See test case |  |

-	Nominal start time:

Current date at 07:31:00 (GPS time).

-	Visible satellites to be simulated are given in Table 4.12.2-3 and are above 15 degrees elevation with respect to the UE. These satellites have been selected to give a reasonable HDOP for the duration of the test.

Table 4.12.2-3: Satellites to be simulated for Aerial testing

| GNSS supported by UE | SV IDs of Satellites to be simulated (1) |
| --- | --- |
| GPS | 3, 4, 6, 17, 19, 28 |
| GLONASS | 3, 4, 5, 10, 18, 19 |
| Galileo | 3, 5, 13, 15, 21, 27 |
| BDS | 38, 40, 42, 43, 59, 60 |
| Note 1: Where the UE supports more than one GNSS then all the relevant satellites are simulated Note 2: For BDS, the satellite types are as follows: GEO: 59, 60, IGSO: 38, 40, MEO: 42, 43 |  |

-	The levels of the simulated satellites are given in Table 4.11.2-6 and shall be generated with an accuracy of +/- 3 dB.

-	Ionospheric model: simulated values are given in Tables 4.11.2-7 and 4.11.2-8.

-	Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [70].

If px_GnssScenario2012 = TRUE, the following GNSS scenarios shall be used.

-	Yuma / Rinex Almanac data: the required file(s) available in the GNSS data sig zip file specified in TS 37.571-5 [68] Annex B are given in Table 4.12.2-4.

Table 4.12.2-4: Yuma / Rinex Almanac data files for Aerial testing

| GNSS supported by UE | Yuma / Rinex file(s) (1) |
| --- | --- |
| GPS | globalAlmanac_gpsYuma.txt |
| GLONASS | globalAlmanac_glonass.agl |
| Galileo | globalAlmanac_galileo.xml |
| BDS | Navmodel_beidou.rnx |
| Note 1: Where the UE supports more than one GNSS then all the relevant Yuma / Rinex data files are used |  |

-	UE location(s) and motion:

Latitude: the simulated latitude is given in Table 4.12.2-2

Longitude: the simulated longitude is given in Table 4.12.2-2

Height: the simulated heights above sea level are given in Table 4.12.2-2

Motion: the simulated motion(s) are given in Table 4.12.2-1

-	Nominal start time:

Current date at 07:31:00 (GPS time).

-	Visible satellites to be simulated are given in Table 4.12.2-5 and are above 15 degrees elevation with respect to the UE. These satellites have been selected to give a reasonable HDOP for the duration of the test.

Table 4.12.2-5: Satellites to be simulated for Aerial testing

| GNSS supported by UE | SV IDs of Satellites to be simulated (1) |
| --- | --- |
| GPS | 2, 12, 14, 24, 25, 29 |
| GLONASS | 4, 5, 13, 14, 15, 23 |
| Galileo | 2, 7, 8, 11, 19, 30 |
| BDS | 13, 15, 19, 27, 28, 30 |
| Note 1: Where the UE supports more than one GNSS then all the relevant satellites are simulated |  |

-	The levels of the simulated satellites are given in Table 4.11.2-6 and shall be generated with an accuracy of +/- 3 dB.

-	Ionospheric model: simulated values are given in Tables 4.11.2-7 and 4.11.2-8.

-	Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [70].

4.xx	UE Positioning Requirements for NTN testing

Currently for NTN UEs that set the PICS complying with TS 36.523-2 clause A.4.4, Rel-13 and forward NB-IoT test cases and Rel-17 NTN specific test cases are applicable.

This clause defines the test environment which applies to all test cases executed for eMTC and NB-IoT NTN UEs, unless otherwise specified.

-	UE's positioning engine (e.g., standalone GNSS receiver) should be enabled to allow it to acquire the position. This could be done by use of AT command, such as AT+CPOS or other commands. UE position during the test is stationary. Below position could be used to pre-configure UT position:

Latitude: 25.08439333

Longitude: 121.56076999

Accuracy: 100 m

[Skip unchanged text]

8.1	NB-IoT Common test environment

8.1.1	NB-IoT Environmental conditions

Same environmental conditions as the ones described in section 4.1 apply to NB-IoT UEs.

8.1.2	NB-IoT Common requirements of test equipment

Same common requirements of test equipment as the ones described in section 4.2 apply to NB-IoT with the following exceptions:

-	Only FDD Mode is required for Rel-13 and Rel-14 NB-IoT

-	Both FDD and TDD Mode are required for Rel-15 and forward NB-IoT

-	Only FDD Mode is required for Rel-17 and forward NB-IoT NTN

-	Supported physical channels for NB-IoT are:

| Physical channel | Minimum number | Comments |
| --- | --- | --- |
| NPBCH | 1 | Narrowband Physical Broadcast Channel |
| NPDCCH | 1 | The Narrowband Physical Downlink Control channel carries control information. |
| NPDSCH | 1 | Narrowband Physical Downlink Shared Channel |
| NPUSCH | 1 | Narrowband Physical Uplink Shared Channel |
| NPRACH | 1 | Narrowband Physical Random Access Channel |

-	Supported physical signals for NB-IoT are:

| Physical signal |  | Minimum number |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Narrowband Reference Signal |  | NA |  | DL |  |
| Narrowband Primary Synchronization Signal |  | NA |  | DL |  |
| Narrowband Secondary Synchronization Signal |  | NA |  | DL |  |
| Demodulation Reference Signal |  | NA |  | UL |  |
| Wake-Up Signal |  | NA |  | DL |  |

8.1.3	NB-IoT Reference test conditions

This clause contains the reference test conditions, which apply to all NB-IoT test cases unless otherwise specified.

8.1.3.1	NB-IoT Test frequencies

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41, 42, 43, 65, 66, 70, 71, 72, 73, 74, 85, 87, 88 and 103 which are defined in Table 5.5-1 in [21]. For satellite access (NTN), NB-IoT is designed to operate in the E-UTRA operating bands 255 and 256 which are defined in TS 36.102 Table 5.2-1. NB-IoT system operates in HD-FDD and TDD duplex mode or in TDD mode.

The test frequencies are based on the E-UTRA frequency bands defined in the core specifications.

The raster spacing is 100 kHz.

E-UTRA/FDD is designed to operate in paired bands of 3GPP TS 36.101 [27] and TS 36.102. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.1 and 8.1.3.1.2.

E-UTRA/TDD is designed to operate in unpaired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.2.

USA & Canada emission requirements for specific operating bands are indicated by network signaling value NS_04 on indicated bands in clause 8.1.3.1.1. The normative reference for this requirement is TS 36.101 [27] clause 5.5F.

8.1.3.1.1	NB-IoT FDD Mode Test frequencies

[Skip unchanged text]

8.1.4.3	NB-IoT Common parameters for simulated cells

The parameters specified in this sub clause apply to all simulated cells unless otherwise specified.

8.1.4.3.1	NB-IoT Common configurations of system information blocks

8.1.4.3.1.1	NB-IoT Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

-	single cell scenario

Combination 2 applies to the following test case scenarios:

-	intra-frequency multi cell scenario

-	intra-frequency multi cell scenario with neighbouring cell related information

Combination 3 applies to the following test case scenarios:

-	inter-frequency multi cell scenario

Combination 4 applies to the following test case scenarios:

- access barring single cell scenario

Combination 5 applies to the following test case scenarios:

-	SC-PTM single cell scenario (SC-PTM cell broadcasting SIB20-NB)

Combination 6 applies to the following test case scenarios:

-	single cell on multi carriers scenario

Combination 7 applies to the following test case scenarios:

-	single cell NPRACH resources using preamble format 2 on non-anchor carriers scenario

Combination 8 is the default combination which applies to the following test case scenarios:

-	single cell scenario + GSO NTN

Combination 9 is the default combination which applies to the following test case scenarios:

-	intra-frequency multi cell scenario + NGSO NTN

-	intra-frequency multi cell scenario with neighbouring cell related information + NGSO NTN

Combination 10 is the default combination which applies to the following test case scenarios:

-	single cell scenario + NGSO NTN

Table 8.1.4.3.1.1-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. |  | SIB2-NB |  |  | SIB3-NB |  | SIB4-NB |  | SIB5-NB |  | SIB14-NB |  | SIB16-NB |  | SIB20-NB |  | SIB22-NB |  | SIB23-NB |  | SIB31-NB |  | SIB32-NB |  |
| 1 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | X |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | X |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | X |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 6 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| 7 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |
| 8 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| 9 |  | X |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  | X |  |
| 10 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  | X |  |

8.1.4.3.1.2	NB-IoT Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables.

Table 8.1.4.3.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |

Table 8.1.4.3.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB4-NB |

Table 8.1.4.3.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB5-NB |

Table 8.1.4.3.1.2-4: Scheduling for combination 4

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB14-NB |

Table 8.1.4.3.1.2-5: Scheduling for combination 5

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB20-NB |

Table 8.1.4.3.1.2-6: Scheduling for combination 6

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-7: Scheduling for combination 7

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |
| 3 | 64 | every16thRF | 256 | SIB23-NB |

Table 8.1.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |

Table 8.1.4.3.1.2-9: Scheduling for combination 9

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB4-NB |
| 4 | 64 | every16thRF | 256 | SIB31-NB |
| 5 | 64 | every16thRF | 256 | SIB32-NB |

Table 8.1.4.3.1.2-10: Scheduling for combination 10

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB32-NB |

8.1.4.3.2	NB-IoT Common contents of system information messages

-	MasterInformationBlock-NB

The MasterInformationBlock-NB includes the system information transmitted on BCH.

Table 8.1.4.3.2-1: MasterInformationBlock-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r13 | 2 | TBS = 208 bits, 16 repetitions |  |
| systemInfoValueTag-r13 | 0 |  |  |
| ab-Enabled-r13 | FALSE |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-SamePCI-r13 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r13 | 23 | As specified in TS 36.213 [29] Clause 16.8 | PRB30 |
|  | 24 | As specified in TS 36.213 [29] Clause 16.8 | PRB35 |
|  | 7 | As specified in TS 36.213 [29] Clause 16.8 | PRB17 |
|  | 8 | As specified in TS 36.213 [29] Clause 16.8 | PRB22 |
| } |  |  |  |
| inband-DifferentPCI-r13 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r13 | same | same number of ports as NRS |  |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ’00’B |  |  |
| } |  |  |  |
| guardband-r13 SEQUENCE { |  |  | Guardband |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ‘000’B |  |  |
| } |  |  |  |
| standalone-r13 SEQUENCE { |  |  | Standalone |
| spare | ‘0000 0’B |  |  |
| } |  |  |  |
| } |  |  |  |
| additionalTransmissionSIB1-r15 | FALSE |  |  |
| ab-Enabled-5GC-r16 | FALSE |  |  |
| partEARFCN-17 CHOICE { |  |  |  |
| spare | '00'B |  |  |
| earfcn-LSB | 2 least significant bits of the EARFCN for NTN bands where 100 kHz raster is used |  | NTN |
| } |  |  |  |
| spare | ‘0000 00’B |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |

[Skip unchanged text]

8.1.4.3.3	NB-IoT Common contents of system information blocks

-	SystemInformationBlockType2-NB

[Skip unchanged text]

-	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information for the serving cell.

Table 8.1.4.3.3-10: SystemInformationBlockType31-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | NTN |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | GSO |
| positionX-r17 | -26044672 |  |  |
| positionY-r17 | 19284189 |  |  |
| positionZ-r17 | -1364331 |  |  |
| velocityVX-r17 | 82 |  |  |
| velocityVY-r17 | -37 |  |  |
| velocityVZ-r17 | -2465 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 7850100 |  |  |
| nta-CommonDrift-r17 | 579 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  |  |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

-	SystemInformationBlockType32-NB

The IE SystemInformationBlockType32-NB contains satellite assistance information for prediction of discontinuous coverage.

Table 8.1.4.3.3-11: SystemInformationBlockType32-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType32-NB-r17 ::= SEQUENCE { |  |  |  |
| satelliteInfoList-r17 | FFS |  | NGSO |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NGSO | Non-geosynchronous Orbit scenario |

[End of change]