3GPP TSG-RAN5 Meeting #100	R5-234685

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2569 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Random Access applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm India Pvt Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CSI-RS based CFRA test applicability in 38.533 should require csi-RS-CFRA-ForHO instead of (csi-RSRP-AndRSRQ-MeasWithSSB or csi-RSRP-AndRSRQ-MeasWithoutSSB). A UE could support any of the latter two optional capabilities and yet not support the first one. In such case, the test would apply to a UE not supporting CSI-RS based CFRA. In addition, 2-step RACH applicability needs to be updated to consider twoStepRACH-r16 optional capability, in addition to UE release 16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the test applicability for CFRA tests according to the ‘Reason for change’ statement - Updated the test applicability for 2-step RACH tests according to the ‘Reason for change’ statement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case applicability will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2.2, 4.3.2.2.3, 4.3.2.2.4, 5.3.2.2.2, 5.3.2.2.3, 5.3.2.2.4, 6.3.2.2.2, 6.3.2.2.3, 6.3.2.2.4, 7.3.2.2.2, 7.3.2.2.3, 7.3.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

##### 4.3.2.2.2	EN-DC FR1 non-contention based random access

4.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

4.3.2.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble which requires UE to support csi-RS-CFRA-ForHO.

4.3.2.2.2.3	Minimum conformance requirement

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-1 [2] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-1 [2] Table 6.3.4.3-1.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs is configured, with the UE selected CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs, UE shall have the capability to select the Random Access Preamble corresponding to the selected CSI-RS, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS, and PRACH occasion shall be randomly selected with equal probability amongst the selected CSI-RS associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s), if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE's C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2 and A.4.3.2.2.2. Non-contention based random access procedure is not initialized for Other SI requested from UE or for beam failure recovery, so the requirements related to those features are omitted.

4.3.2.2.2.4	Test description

4.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 4.3.2.2.2.4.1-1.

Table 4.3.2.2.2.4.1-1: Non-contention based random access test
in FR1 for PSCell in EN-DC supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 4.3.2.2.2-1 | 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.3.2.2.2-2 | 2 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.3.2.2.2-3 | 3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.3.2.2.2-4 | 4 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 4.3.2.2.2.4.1-2.

Table 4.3.2.2.2.4.1-2: Initial conditions for Non-contention based random access test in FR1 for EN-DC

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.1-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.3.2.2.2.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The E-UTRAN PCell power levels and settings are specified in Table A.6.1.1-1. Cell 2 is the NR FR1 PSCell. The connection setup is done according to the settings in clause C.1.3, with downlink signal levels as per clause C.1.2. General Test parameters are defined in Table 4.3.2.2.2.5-1.

3.	Downlink signals for NR cell are initially set up according to clause C.2.1.

4.3.2.2.2.4.2	Test procedure

For this test two cells are used, an E-UTRA serving cell (PCell) and an NR FR1 PSCell. For the NR PSCell, the System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink. There are two subtests, to test both SSB-based non-contention based random access (subtest 1) and CSI-RS-based non-contention based random access (subtest 2).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC , DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

2.	Set the parameters according to Table 4.3.2.2.2.5-1.

3.	SS sends a RRCReconfiguration to trigger a contention-free random access procedure according to the Tables for Non-Contention Random Access in clause 4.3.2.2.2.4.3.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble:

4.1.	The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble:

5.1.	Set the parameters according to Table 4.3.2.2.2.5-1 Subtest 2.

5.2.	Repeat steps 1-3.

5.3.	The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response:

6.1.	Repeat steps 1-3.

6.2.	The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3.	As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4.	The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5.	As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 4.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response:

7.1.	Repeat steps 1-3.

7.2.	The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

7.3.	As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4.	The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

7.5.	As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 4.3.2.2.2.5.

4.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 with the following exceptions.

Table 4.3.2.2.2.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

Table 4.3.2.2.2.4.3-1: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | 0 |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 15 |  | 15kHz |
|  | kHz 30 |  | 30kHz |
| } |  |  |  |

Table 4.3.2.2.2.4.3-2: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 1 entry |  |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_51 |  | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.3.2.2.2.4.3-3: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  |  |
| msg1-FDM | one |  |  |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 4.3.2.2.2.4.3-4: ServingCellConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | -5 |  |  |
| } |  |  |  |

Table 4.3.2.2.2.4.3-5: CellGroupConfig for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-19 with Condition CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| newUE-Identity | 1 |  |  |
| } |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config | SRS-Config |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

4.3.2.2.2.5	Test requirement

Table 4.3.2.2.2.5-1 defines the primary level settings for non-contention based random access test in FR1 for PSCell in EN-DC. Tables 4.3.2.2.2.5-2, 4.3.2.2.2.5-3 and 4.3.2.2.2.5-4 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 4.3.2.2.2.5-1: General test parameters for non-contention
based random access test in FR1 for PSCell in EN-DC

| Parameter |  |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2 |  | SSB.3 FR1 | SSB.3 FR1 | As defined in clause A.3.1 |
|  |  | Config 3,4 |  | SSB.4 FR1 | SSB.4 FR1 |  |
| CSI-RS Configuration |  | Config 1,2 |  | N/A | CSI-RS.1.1 FDD | As defined in clause A.1.4 |
|  |  | Config 3,4 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 2 |  | Config 1,2 |  | FDD | FDD |  |
|  |  | Config 3,4 |  | TDD | TDD |  |
| TDD Configuration |  | Config 3,4 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OCNG pattern 1 | OCNG pattern 1 | As defined in clause A.2.1 |
| PDSCH parameters Note 4 |  | Config 1,2 |  | SR1.1 FDD | SR1.1 FDD | As defined in clause A.1.1 |
|  |  | Config 3,4 |  | SR2.1 TDD | SR2.1 TDD |  |
| RMSI CORESET reference channel |  | Config 1,2 |  | CR.1.1 FDD | CR.1.1 FDD |  |
|  |  | Config 3,4 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET reference channel |  | Config 1,2 |  | CCR.1.1 FDD | CCR.1.1 FDD |  |
|  |  | Config 3,4 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |  |
| SSB with index 0 |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3,4 |  | -101 | -101 |  |
|  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 | Config 1,2 | dBm/15kHz | -95 | -95 |  |
|  |  | Config 3,4 |  | -98 | -98 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 3,4 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 | Config 1,2 | dBm/15kHz | -115 | -115 |  |
|  |  | Config 3,4 |  | -118 | -118 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1,2 | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3,4 |  | -62.2/38.16MHz | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [13] |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2] |
| PRACH Configuration |  |  |  | PRACH.2 FR1 | PRACH.3 FR1 | As defined in clause A.7.1 |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. NOTE 2: Es/Iot, SS-RSRP and Io levels have been derived from other parameters for information purpose. They are not settable parameters. NOTE 3: Void. NOTE 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble:

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble:

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.2.5-4.

Test 4: Correct behaviour when not receiving Random Access Response:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.2.5-4.

Table 4.3.2.2.2.5-2: Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 4.3.2.2.2.5-3: Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 |

Table 4.3.2.2.2.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [7]. |  |  |  |

##### 4.3.2.2.3	EN-DC FR1 2-step contention based random access

4.3.2.2.3.1	Test purpose

The purpose of this test is to verify that the behaviour of the 2-step random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

4.3.2.2.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and twoStepRACH-r16.

4.3.2.2.3.3	Minimum conformance requirement

The UE shall select the type of random access at initiation of the random access procedure based on network configuration, as specified in clause 5.1.1 in TS 38.321 [12].

The UE shall have capability to calculate MsgA PRACH transmission power according to the PRACH power formula defined in clause 7.4 of TS 38.213 [3] and the MsgA PUSCH power formula of clause 7.1.1 of TS 38.213 [8] and apply this power level at the first MsgA or additional MsgA repetitions. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1 [2] for frequency range 1 and in Table 6.3.4.2-1 of TS 38.101-2 [3] for frequency range 2. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1 [2] for frequency range 1 and clause 6.3.4.3 of TS38.101-2 [3] for frequency range 2.

The UE shall switch to 4-step RA type procedure if the MsgA transmission counter has exceeded msgA-TransMax, if configured, as specified in clause 5.1.4a of TS 38.321 [12]. The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in clause 5.1.4a in TS 38.321 [12].

The requirements in this clause apply for UE in SA operation mode or any MR-DC operation mode.

4.3.2.2.3.3.1	Correct behaviour when transmitting MsgA

With the UE selected SSB with SS-RSRP above msgA-RSRP-ThresholdSSB, the UE shall have the capability to select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB if the association between Random Access Preambles and SS blocks is configured, as specified in clause 5.1.2a in TS 38.321 [7].

With the UE selected SSB with SS-RSRP above msgA-RSRP-ThresholdSSB, UE shall have the capability to transmit MsgA PRACH on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured, if the association between PRACH occasions and SSBs is configured.

The PRACH preamble and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2a in TS 38.321 [7].

In association with the MsgA PRACH, the UE should have the capability to transmit MsgA PUSCH on the corresponding PUSCH occasion associated with a DMRS resource, which is mapped from the MsgA PRACH occasion, and preamble index as defined in clause 8.1A in TS 38.213 [3].

4.3.2.2.3.3.2	Correct behaviour when receiving MsgB

The UE shall  stop monitoring for MsgB, when the UE has successfully received the PDCCH addressed to UE as specified in clause 8.2A in TS 38.213 [8] containing a successRAR MAC subPDU or a fallbackRAR MAC subPDU as described in clause 5.1.4a in TS 38.321 [12].

The UE shall send ACK if Success RAR is received in MsgB and the Contention Resolution is successful, as defined in clause 5.1.4a in TS 38.321 [12].

If MsgB contains a fallbackRAR MAC subPDU the UE shall fallback to the 4-step RA type by transmitting the msg3 containing the payload of MsgA PUSCH and monitor contention resolution as described in clause 8.2A in TS 38.213 [8].

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires unless the Random Access Response reception is considered as successful, as defined in clause 5.1.4a in TS 38.321 [12].

4.3.2.2.3.3.3	Correct behaviour when not receiving MsgB

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires unless the Random Access Response reception is considered as successful, as defined in clause 5.1.4a in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2.3 and A.4.3.2.2.3.

4.3.2.2.3.4	Test description

4.3.2.2.3.4.1	Initial conditions

This test can be run in the configurations defined in Table 4.3.2.2.3.4.1-1.

Table 4.3.2.2.3.4.1-1: Contention based random access test
in FR1 for PSCell in EN-DC supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 4.3.2.2.3-1 | 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.3.2.2.3-2 | 2 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.3.2.2.3-3 | 3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.3.2.2.3-4 | 4 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 4.3.2.2.3.4.1-2.

Table 4.3.2.2.3.4.1-2: Initial conditions for Contention based random access test in FR1 for EN-DC

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.1-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.3.2.2.3.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The E-UTRAN PCell power levels and settings are specified in Table A.6.1.1-1. Cell 2 is the NR FR1 PSCell. The connection setup is done according to the settings in clause C.1.3, with downlink signal levels as per clause C.1.2. General Test parameters are defined in Table 4.3.2.2.3.5-1.

3.	Downlink signals for NR cell are initially set up according to clause C.2.1.

4.3.2.2.3.4.2	Test procedure

For this test two cells are used, an E-UTRA serving cell (PCell) and an NR FR1 PSCell. For the NR PSCell, the System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink:

1.	Ensure the UE is in state E-UTRA RRC_CONNECTED with generic procedure parameters Connectivity E-UTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 4.3.2.2.3.5-1.

3.	The test system shall send a RRCReconfiguration message to the UE to add NR PSCell, then the UE shall trigger a random access procedure.

4.	Test 1: Correct behaviour when transmitting MsgA:

4.1.	The UE shall send a MsgA to the System Simulator. The System Simulator shall check that the MsgA preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SSB_RP above the configured rsrp-ThresholdSSB.

4.2	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 4.3.2.2.3.5.

5.	Test 2: Correct behaviour when receiving MsgB:

5.1.	Repeat steps 1-3.

5.2.	The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB containing identifiers that do not match the transmitted Random Access Preamble.

5.3.	As the received MsgB contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

5.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

5.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

5.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 4.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 4.3.2.2.3.5.

6.	Test 3: Correct behaviour when not receiving MsgB:

6.1.	Repeat steps 1-3.

6.2.	The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

6.3.	As no MsgB was received within the RA Response window, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

6.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

6.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 4.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 4.3.2.2.3.5.

4.3.2.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.3.2.2.3.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

4.3.2.2.3.5	Test requirement

Table 4.3.2.2.3.5-1 defines the primary level settings for contention based random access test in FR1 for PSCell in EN-DC.

Table 4.3.2.2.3.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2 |  | SSB.3 FR1 | As defined in clause A.3.1 |
|  |  | Config 3,4 |  | SSB.4 FR1 |  |
| Duplex Mode for Cell 2 |  | Config 1,2 |  | FDD |  |
|  |  | Config 3,4 |  | TDD |  |
| TDD Configuration |  | Config 3,4 |  | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OCNG pattern 1 | As defined in clause A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,2 |  | SR1.1 FDD | As defined in clause A.1.1. |
|  |  | Config 3,4 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 |  |
|  |  | Config 3,4 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP | Config 1,2 | dBm / 15kHz | -95 |  |
|  |  | Config 3,4 |  | -98 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 |  |
|  |  | Config 3,4 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP | Config 1,2 | dBm / 15kHz | -115 |  |
|  |  | Config 3,4 |  | -118 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1,2 | dBm | -65.3/9.36 MHz | For symbols without SSB index 1 |
|  |  | Config 3,4 |  | -62.2/38.16 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 1 |  |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  | - | AWGN |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. NOTE 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. NOTE 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Test 1: Correct behaviour when transmitting MsgA:

-	The MsgA shall be one of the Random Access Preambles associated with SSB index 0.

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.3.5-2.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 4.3.2.2.3.5-2A.

Test 2: Correct behaviour when receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.3.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.3.5-3.

-	The transmit timing of all MsgA PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.3.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 4.3.2.2.3.5-2A.

Test 3: Correct behaviour when not receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.3.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.3.5-3.

-	The transmit timing of all MsgA PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.3.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 4.3.2.2.3.5-2A.

Table 4.3.2.2.3.5-2: MsgA PRACH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 4.3.2.2.3.5-2A: MsgA PUSCH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 4.3.2.2.3.5-3: Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | MsgA PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 dB |

Table 4.3.2.2.3.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 4.3.2.2.4	EN-DC FR1 2-step non-contention based random access

4.3.2.2.4.1	Test purpose

The purpose of this test is to verify that the behaviour of the 2-step non-contention based random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

4.3.2.2.4.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC and twoStepRACH-r16.

4.3.2.2.4.3	Minimum conformance requirement

The UE shall select the type of random access at initiation of the random access procedure based on network configuration, as specified in clause 5.1.1 in TS 38.321 [12].

The UE shall have capability to calculate MsgA PRACH transmission power according to the PRACH power formula defined in clause 7.4 of TS 38.213 [3] and the MsgA PUSCH power formula of clause 7.1.1 of TS 38.213 [8] and apply this power level at the first MsgA or additional MsgA repetitions. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1 [2] for frequency range 1 and in Table 6.3.4.2-1 of TS 38.101-2 [3] for frequency range 2. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1 [2] for frequency range 1 and clause 6.3.4.3 of TS38.101-2 [3] for frequency range 2.

The UE shall switch to 4-step RA type procedure if the MsgA transmission counter has exceeded msgA-TransMax, if configured, as specified in clause 5.1.4a of TS 38.321 [12]. The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in clause 5.1.4a in TS 38.321 [12].

The requirements in this clause apply for UE in SA operation mode or any MR-DC operation mode.

4.3.2.2.4.3.1	Correct behaviour when transmitting MsgA

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above msgA-RSRP-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2a in TS 38.321 [12].

In association with the MsgA PRACH, the UE should have the capability to transmit MsgA PUSCH on the corresponding PUSCH occasion associated with a DMRS resource, which is mapped from the MsgA PRACH occasion, and preamble index as defined in clause 8.1A in TS 38.213 [8].

4.3.2.2.4.3.2	Correct behaviour when receiving MsgB

The UE may stop monitoring for MsgB, when the UE has successfully received the PDCCH addressed to UE as specified in clause 8.2A in TS 38.213 [8] containing a successRAR MAC subPDU or a fallbackRAR MAC subPDU as described in clause 5.1.4a in TS 38.321 [12].

If MsgB contains a fallbackRAR MAC subPDU the UE shall fallback to the 4-step RA type by transmitting the msg3 containing the payload of MsgA PUSCH as described in clause 8.2A in TS 38.213 [8].

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated MsgA PRACH and MsgA PUSCH transmission power if all received MsgBs contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

4.3.2.2.4.3.3	Correct behaviour when not receiving MsgB

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2a in TS 38.321 [12] for the next available PRACH occasion, and transmit MsgA with the calculated MsgA PRACH and MsgA PUSCH transmission power, if no MsgB is received within the MsgB Response window configured in RACH-ConfigGenericTwoStepRA and the Random Access Response Reception has not been considered as successful as defined in clause 5.1.4a in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2.3 and A.4.3.2.2.4.

4.3.2.2.4.4	Test description

4.3.2.2.4.4.1	Initial conditions

This test can be run in the configurations defined in Table 4.3.2.2.4.4.1-1.

Table 4.3.2.2.4.4.1-1: Non-contention based random access test
in FR1 for PSCell in EN-DC supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 4.3.2.2.4-1 | 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.3.2.2.4-2 | 2 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.3.2.2.4-3 | 3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.3.2.2.4-4 | 4 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 4.3.2.2.4.4.1-2.

Table 4.3.2.2.4.4.1-2: Initial conditions for non-contention based random access test in FR1 for EN-DC

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.1-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.3.2.2.4.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The E-UTRAN PCell power levels and settings are specified in Table A.6.1.1-1. Cell 2 is the NR FR1 PSCell. The connection setup is done according to the settings in clause C.1.3, with downlink signal levels as per clause C.1.2. General Test parameters are defined in Table 4.3.2.2.4.5-1.

3. Downlink signals for NR cell are initially set up according to clause C.2.1.

4.3.2.2.4.4.2	Test procedure

For this test two cells are used, an E-UTRA serving cell (PCell) and an NR FR1 PSCell. For the NR PSCell, the System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state E-UTRA RRC_CONNECTED with generic procedure parameters Connectivity E-UTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 4.3.2.2.4.5-1.

3.	The test system shall send a RRCReconfiguration message to the UE to add NR PSCell, then the UE shall trigger a random access procedure.

4.	Test 1: Correct behaviour when transmitting MsgA:

4.1.	The UE shall send a MsgA to the System Simulator. The System Simulator shall check that the MsgA preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex.

4.2	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.4.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 4.3.2.2.4.5.

5.	Test 2: Correct behaviour when receiving MsgB:

5.1.	Repeat steps 1-3.

5.2.	The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB containing identifiers that do not match the transmitted Random Access Preamble.

5.3.	As the received MsgB contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

5.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

5.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

5.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.4.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 4.3.2.2.4.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 4.3.2.2.4.5.

6.	Test 3: Correct behaviour when not receiving MsgB:

6.1.	Repeat steps 1-3.

6.2.	The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

6.3.	As no MsgB was received within the MsgB Response window configured in RACH-ConfigGenericTwoStepRA, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

6.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

6.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 4.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 4.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 4.3.2.2.4.5.

4.3.2.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.3.2.2.4.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

4.3.2.2.4.5	Test requirement

Table 4.3.2.2.4.5-1 defines the primary level settings for non-contention based random access test in FR1 for PSCell in EN-DC.

Table 4.3.2.2.4.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2 |  | SSB.3 FR1 | As defined in clause A.3.1 |
|  |  | Config 3,4 |  | SSB.4 FR1 |  |
| Duplex Mode for Cell 2 |  | Config 1,2 |  | FDD |  |
|  |  | Config 3,4 |  | TDD |  |
| TDD Configuration |  | Config 3,4 |  | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OCNG pattern 1 | As defined in clause A.2.1. |
| PDSCH parameters Note 4 |  | Config 1,2 |  | SR1.1 FDD | As defined in clause A.1.1. |
|  |  | Config 3,4 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 |  |
|  |  | Config 3,4 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP | Config 1,2 | dBm / 15kHz | -95 |  |
|  |  | Config 3,4 |  | -98 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1,2 | dBm/15kHz | -98 |  |
|  |  | Config 3,4 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP | Config 1,2 | dBm / 15kHz | -115 |  |
|  |  | Config 3,4 |  | -118 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1,2 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3,4 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 2 |  |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  | - | AWGN |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. NOTE 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. NOTE 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Test 1: Correct behaviour when transmitting MsgA:

-	The MsgA shall be one of the Random Access Preambles associated with SSB index 0.

-	The MsgA shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex.

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.4.5-2.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 4.3.2.2.4.5-2A.

Test 2: Correct behaviour when receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.4.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.4.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.4.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 4.3.2.2.4.5-2A.

Test 3: Correct behaviour when not receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 4.3.2.2.4.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 4.3.2.2.4.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 4.3.2.2.4.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 4.3.2.2.4.5-2A.

Table 4.3.2.2.4.5-2: MsgA PRACH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 4.3.2.2.4.5-2A: MsgA PUSCH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 4.3.2.2.4.5-3: Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | MsgA PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 dB |

Table 4.3.2.2.4.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

<< Skipped sections >>

##### 5.3.2.2.2	EN-DC FR2 non-contention based random access

Editor's note: This test case is incomplete for Test 3 and 4. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

5.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

5.3.2.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC with FR2. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble which requires UE to support csi-RS-CFRA-ForHO.

5.3.2.2.2.3	Minimum conformance requirement

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-2 [3] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-2 [3] Tables 6.3.4.3-1 and 6.3.4.3-2.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs is configured, with the UE selected CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs, UE shall have the capability to select the Random Access Preamble corresponding to the selected CSI-RS, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS, and PRACH occasion shall be randomly selected with equal probability amongst the selected CSI-RS associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s), if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE's C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2.

Non-contention based random access procedure is not initialized for Other SI requested from UE or for beam failure recovery, so the requirements related to those features are omitted.

5.3.2.2.2.4	Test description

5.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 5.3.2.2.2.4.1-1.

Table 5.3.2.2.2.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 5.3.2.2.2-1 | 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD |
| 5.3.2.2.2-2 | 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 5.3.2.2.2.4.1-2.

Table 5.3.2.2.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.1 and Table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 5.3.2.2.2.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The E-UTRAN PCell power levels and settings are specified in Table A.6.1.1-1. Cell 2 is the NR FR2 PSCell. The connection setup is done according to the settings in clause C.1.3, with downlink signal levels as per clause C.1.2. General Test parameters are defined in Table 5.3.2.2.2.5-1.

3.	Downlink signals for NR cell are initially set up according to clause C.2.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

5.3.2.2.2.4.2	Test procedure

For this test two cells are used, an E-UTRA serving cell (PCell) and an NR FR2 PSCell. For the NR PSCell, the System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state E-UTRA RRC_CONNECTED with generic procedure parameters Connectivity E-UTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 5.3.2.2.2.5-1.

3.	SS sends a RRCReconfiguration to trigger a contention-free random access procedure according to Table 5.3.2.2.2.4.3-2.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

5.1. Set the parameters according to Table 5.3.2.2.2.5-1 Subtest 2.

5.2. Repeat steps 1-3

5.3. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response

6.1. Repeat steps 1-3

6.2. The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 5.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 5.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response

7.1. Repeat steps 1-3.

7.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

7.3. As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

7.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 5.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 5.3.2.2.2.5.

5.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3.1 with the following exceptions:

Table 5.3.2.2.2.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

Table 5.3.2.2.2.4.3-1: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| rsrp-ThresholdSSB | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| I139 | 0 |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3-2: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 1 entry |  |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3-3: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -100 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3-4: ServingCellConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000'B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3-5: CellGroupConfig for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| newUE-Identity | 1 |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config | SRS-Config |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.2.2.2.5	Test requirement

Table 5.3.2.2.2.5-2 defines the primary level settings for non-contention based random access test in FR2 for NR Standalone. Tables 5.3.2.2.2.5-3, 5.3.2.2.2.5-4 and 5.3.2.2.2.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 5.3.2.2.2.5-1: General test parameters

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration | Config 1,2 |  | SSB.1 FR2 | SSB.1 FR2 | As defined in A.3.2 |
| CSI-RS Configuration | Config 1,2 |  | N/A | CSI-RS.3.1 TDD | As defined in A.1.4 |
| Duplex Mode for Cell 2 | Config 1,2 |  | TDD | TDD |  |
| TDD Configuration | Config 1,2 |  | TDDConf.3.1 | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1,2 | MHz | 100: NRB,c = 24 | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | OP.3 | As defined in A.2.1. |
| PDSCH Reference Channel Note 2 | Config 1,2 |  | SR3.1 TDD | SR3.1 TDD | As defined in A.1.1. |
| RMSI CORESET Reference Channel | Config 1,2 |  | CR.3.1 TDD | CR.3.1 TDD | As defined in A.1.2 |
| RMC CORESET Reference Channel | Config 1,2 |  | CCR.3.1 TDD | CCR.3.1 TDD | As defined in A.1.3 |
| NR RF Channel Number |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [3] |
| PRACH Configuration |  |  | PRACH.2 FR2 | PRACH.3 FR2 | As defined in A.7.2, with exceptions as defined below. |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | -100 | As defined in TS 38.331 [13] |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. NOTE 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. NOTE 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. NOTE 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |  |

Table 5.3.2.2.2.5-2: OTA-related test parameters

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  | Rough | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 | -80.6 |  |
|  | Es/IotBB | dB | 21.09 | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 | -95.0 |  |
|  | Es/IotBB | dB | 6.69 | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN | AWGN |  |
| NOTE 1: No artificial noise is applied in this test. NOTE 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	The selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 5.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 5.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 5.3.2.2.2.5-5.

Test 4: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 5.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 5.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 5.3.2.2.2.5-5.

Table 5.3.2.2.2.5-3 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 5.3.2.2.2.5-4 Relative power tolerance Test requirements

| Measured power | Power step P (dB) | PRACH (dB) |
| --- | --- | --- |
| Measured power of both PRACHs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either PRACHs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| NOTE 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 5.3.2.2.2.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 5.3.2.2.3	EN-DC FR2 2-step contention based random access

Editor's note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis and test requirements are missing.

- Test procedure is FFS

5.3.2.2.3.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

5.3.2.2.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with FR2 and twoStepRACH-r16.

5.3.2.2.3.3	Minimum conformance requirement

Same as in clause 4.3.2.2.3.3

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2 and A.5.3.2.2.3.

5.3.2.2.3.4	Test description

5.3.2.2.3.4.1	Initial conditions

This test can be run in the configurations defined in Table 5.3.2.2.3.4.1-1.

Table 5.3.2.2.3.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 5.3.2.2.3-1 | 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD |
| 5.3.2.2.3-2 | 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 5.3.2.2.3.4.1-2.

Table 5.3.2.2.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.1 and Table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 5.3.2.2.3.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The E-UTRAN PCell power levels and settings are specified in Table A.6.1.1-1. Cell 2 is the NR FR2 PSCell. The connection setup is done according to the settings in clause C.1.3, with downlink signal levels as per clause C.1.2. General Test parameters are defined in Table 5.3.2.2.3.5-1.

3.	Downlink signals for NR cell are initially set up according to clause C.2.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

5.3.2.2.3.4.2	Test procedure

For this test two cells are used, an E-UTRA serving cell (PCell) and an NR FR2 PSCell. For the NR PSCell, the System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state E-UTRA RRC_CONNECTED with generic procedure parameters Connectivity E-UTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 5.3.2.2.3.5-1.

3.	The test system shall send a RRCReconfiguration message to the UE to add NR PSCell, then the UE shall trigger a random access procedure.

FFS

5.3.2.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3.1 with the following exceptions:

Table 5.3.2.2.3.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |
| Common exceptions to the contents of TS 38.508-1 [14] | FFS |

5.3.2.2.3.5	Test requirement

Table 5.3.2.2.3.5-2 defines the primary level settings for contention based random access test in FR2 for NR Standalone. The test requirements are FFS.

Table 5.3.2.2.3.5-1: General test parameters

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1,2 |  | SSB.1 FR2 | As defined in A.3.2 |
| Duplex Mode for Cell 2 | Config 1,2 |  | TDD |  |
| TDD Configuration | Config 1,2 |  | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1,2 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | As defined in A.2.1 |
| PDSCH Reference Channel Note 2 | Config 1,2 |  | SR.3.1 TDD | As defined in A.1.1 |
| RMSI CORESET Reference Channel | Config 1,2 |  | CR.3.1 TDD | As defined in A.1.2 |
| RMC CORESET Reference Channel | Config 1,2 |  | CCR.3.1 TDD | As defined in A.1.3 |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 of TS 38.101-2 [3] |
| MsgA Configuration |  |  | FR2 MsgA configuration 1 | As defined in FFS, with exceptions as defined below |
| msgA-RSRP-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| msgA-PreambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [13] |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. NOTE 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. NOTE 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. NOTE 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table 5.3.2.2.3.5-2: OTA-related test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  |  | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  |  | Rough |  |
| SSB with index 0 | Es Note1 |  | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP |  | dBm/SCS | -80.6 |  |
|  | Es/IotBB |  | dB | 21.09 |  |
|  | Io |  | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 |  | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | SSB_RP | dBm/SCS | -95.0 |  |
|  |  | Es/IotBB | dB | 6.69 |  |
|  |  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  |  | - | AWGN |  |
| NOTE 1: No artificial noise is applied in this test. NOTE 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### 5.3.2.2.4	EN-DC FR2 2-step non-contention based random access

Editor's note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis and test requirements are missing.

- Test procedure is FFS

5.3.2.2.4.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

5.3.2.2.4.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with FR2 and twoStepRACH-r16.

5.3.2.2.4.3	Minimum conformance requirement

Same as in clause 4.3.2.2.4.3

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2 and A.5.3.2.2.4.

5.3.2.2.4.4	Test description

5.3.2.2.4.4.1	Initial conditions

This test can be run in the configurations defined in Table 5.3.2.2.4.4.1-1.

Table 5.3.2.2.4.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 5.3.2.2.4-1 | 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD |
| 5.3.2.2.4-2 | 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 5.3.2.2.4.4.1-2.

Table 5.3.2.2.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.1 and Table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 5.3.2.2.4.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The E-UTRAN Pcell power levels and settings are specified in Table A.6.1.1-1. Cell 2 is the NR FR2 PSCell. The connection setup is done according to the settings in clause C.1.3, with downlink signal levels as per clause C.1.2. General Test parameters are defined in Table 5.3.2.2.4.5-1.

3.	Downlink signals for NR cell are initially set up according to clause C.2.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

5.3.2.2.4.4.2	Test procedure

For this test two cells are used, an E-UTRA serving cell (Pcell) and an NR FR2 PSCell. For the NR PSCell, the System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state E-UTRA RRC_CONNECTED with generic procedure parameters Connectivity E-UTRA/EPC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 5.3.2.2.4.5-1.

3.	The test system shall send a RRCReconfiguration message to the UE to add NR PSCell, then the UE shall trigger a random access procedure.

FFS

5.3.2.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3.1 with the following exceptions:

Table 5.3.2.2.4.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |
| Common exceptions to the contents of TS 38.508-1 [14] | FFS |

5.3.2.2.4.5	Test requirement

Table 5.3.2.2.4.5-2 defines the primary level settings for non-contention based random access test in FR2 for NR Standalone. The test requirements are FFS.

Table 5.3.2.2.4.5-1: General test parameters

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1,2 |  | SSB.1 FR2 | As defined in A.3.2 |
| Duplex Mode for Cell 2 | Config 1,2 |  | TDD |  |
| TDD Configuration | Config 1,2 |  | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1,2 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | As defined in A.2.1 |
| PDSCH Reference Channel Note 2 | Config 1,2 |  | SR.3.1 TDD | As defined in A.1.1 |
| RMSI CORESET Reference Channel | Config 1,2 |  | CR.3.1 TDD | As defined in A.1.2 |
| RMC CORESET Reference Channel | Config 1,2 |  | CCR.3.1 TDD | As defined in A.1.3 |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 of TS 38.101-2 [3] |
| MsgA Configuration |  |  | FR2 MsgA configuration 2 | As defined in FFS, with exceptions as defined below |
| msgA-RSRP-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| msgA-PreambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [13] |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. NOTE 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. NOTE 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. NOTE 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table 5.3.2.2.4.5-2: OTA-related test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  |  | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  |  | Rough |  |
| SSB with index 0 | Es Note1 |  | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP |  | dBm/SCS | -80.6 |  |
|  | Es/IotBB |  | dB | 21.09 |  |
|  | Io |  | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 |  | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | SSB_RP | dBm/SCS | -95.0 |  |
|  |  | Es/IotBB | dB | 6.69 |  |
|  |  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  |  | - | AWGN |  |
| NOTE 1: No artificial noise is applied in this test. NOTE 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

<< Skipped sections >>

##### 6.3.2.2.2	NR SA FR1 non-contention based random access

6.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

6.3.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble which requires UE to support csi-RS-CFRA-ForHO.

6.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.3.2.2.2.

6.3.2.2.2.4	Test description

6.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.2.2.4.1-1.

Table 6.3.2.2.2.4.1-1: Non-Contention based random access test in FR1 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 6.3.2.2.2-1 | 1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.3.2.2.2-2 | 2 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.2.2.4.1-2.

Table 6.3.2.2.2.4.1-2: Initial conditions for Non-Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.3.2.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (Pcell). The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.3.2.2.2.4.2	Test procedure

The test consists of a single cell, configured as Pcell in FR1. The System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink. There are two subtests, to test both SSB-based non-contention based random access (subtest 1) and CSI-RS-based non-contention based random access (subtest 2).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to Table 6.3.2.2.2.5-1 Subtest 1.

3.	The SS shall signal a Random Access Preamble ID via a RRCReconfiguration message to the UE and initiate a Non-contention based Random Access procedure according to Table 6.3.2.2.2.4.3-3 and Table 6.3.2.2.2.4.3-4.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

5.1. Set the parameters according to Table 6.3.2.2.2.5-1 Subtest 2.

5.2. Repeat steps 1-3.

5.3. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response

6.1. Repeat steps 1-3.

6.2. The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 6.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response

7.1. Repeat steps 1-3.

7.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

7.3. As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

7.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 6.3.2.2.2.5.

6.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.2.2.2.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

Table 6.3.2.2.2.4.3-1: FrequencyInfoUL-SIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-2: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 15 |  |  |
|  | kHz 30 |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-3: CellGroupConfig for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| newUE-Identity | 1 |  |  |
| t304 | ms1000 |  |  |
| rach-ConfigDedicted |  |  |  |
| uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config | SRS-Config |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-4: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 1 entry |  |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_51 |  | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-5: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  |  |
| msg1-FDM | one |  |  |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-6: ServingCellConfigCommonSIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | -5 |  |  |
| } |  |  |  |

6.3.2.2.2.5	Test requirement

Table 6.3.2.2.2.5-1 defines the primary level settings for non-contention based random access test in FR1 for NR Standalone. Tables 6.3.2.2.2.5-2, 6.3.2.2.2.5-3 and 6.3.2.2.2.5-4 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 6.3.2.2.2.5-1: General test parameters for non-contention based random access test in FR1 for NR Standalone

| Parameter |  |  | Unit | Subtest 1 | Subtest 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB.2 FR1 | SSB.2 FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.10 |
| CSI-RS Configuration |  | Config 1 |  | N/A | CSI-RS.1.1 FDD | As defined in A.3.1.4 |
|  |  | Config 2 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD | FDD |  |
|  |  | Config 2 |  | TDD | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OCNG pattern 1 | OCNG pattern 1 | As defined in A. 2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD | As defined in A.1.1. |
|  |  | Config 2 |  | SR.2.1 TDD | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | Config 2 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | Config 2 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |  |
| SSB with index 0 |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  | -101 | -101 |  |
|  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 | Config 1 | dBm/15kHz | -95 | -95 |  |
|  |  | Config 2 |  | -98 | -98 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 | Config 1 | dBm/15kHz | -115 | -115 |  |
|  |  | Config 2 |  | -118 | -118 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | FR1 PRACH configuration 2 | FR1 PRACH configuration 3 | As defined in A. 7.1. |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Es/Iot, SS-RSRP and Io level have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void. Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	The selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.2.5-4.

Test 4: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.2.5-4.

Table 6.3.2.2.2.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 6.3.2.2.2.5-3 Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 |

Table 6.3.2.2.2.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 6.3.2.2.3	NR SA FR1 2-step contention based random access

6.3.2.2.3.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

6.3.2.2.3.2	Test applicability

This test applies to all types of NR FR1 UE from Release 16 onwards and twoStepRACH-r16.

6.3.2.2.3.3	Minimum conformance requirement

Same as in clause 4.3.2.2.3.3.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2 and A.6.3.2.2.3.

6.3.2.2.3.4	Test description

6.3.2.2.3.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.2.3.4.1-1.

Table 6.3.2.2.3.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 6.3.2.2.3-1 | 1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.3.2.2.3-2 | 2 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.2.3.4.1-2.

Table 6.3.2.2.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.1 and Table E.3-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.3.2.2.3.4.3.

2.	Cell 1 is the NR FR1 serving cell (PCell). The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.3.2.2.3.4.2	Test procedure

The test consists of a single cell, configured as Pcell in FR1. The System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 6.3.2.2.1.5-1.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when transmitting MsgA:

4.1.	The UE shall send a MsgA to the System Simulator. The System Simulator shall check that the MsgA preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SSB_RP above the configured rsrp-ThresholdSSB.

4.2	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 6.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 6.3.2.2.3.5.

5.	Test 2: Correct behaviour when receiving MsgB:

5.1.	Repeat steps 1-3.

5.2.	The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB containing identifiers that do not match the transmitted Random Access Preamble.

5.3.	As the received MsgB contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

5.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

5.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

5.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 6.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 6.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 6.3.2.2.3.5.

6.	Test 3: Correct behaviour when not receiving MsgB:

6.1.	Repeat steps 1-3.

6.2.	The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

6.3.	As no MsgB was received within the RA Response window, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

6.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

6.6	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 6.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 6.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 6.3.2.2.3.5.

6.3.2.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3.1 with the following exceptions:

Table 6.3.2.2.3.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

6.3.2.2.3.5	Test requirement

Table 6.3.2.2.3.5-2 defines the primary level settings for contention based random access test in FR1 for NR Standalone.

Table 6.3.2.2.1.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 | As defined in A.3.1, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB.2 FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.1 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.1 |
| Duplex Mode for Cell 2 |  | Config 1 |  | FDD |  |
|  |  | Config 2 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  | Config 2 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP | Config 1 | dBm/15kHz | -95 |  |
|  |  | Config 2 |  | -98 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP | Config 1 | dBm/15kHz | -115 |  |
|  |  | Config 2 |  | -118 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 1 | As defined in FFS. |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Test 1: Correct behaviour when transmitting MsgA:

-	The MsgA shall be one of the Random Access Preambles associated with SSB index 0.

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.3.5-2.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 6.3.2.2.3.5-2A.

Test 2: Correct behaviour when receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.3.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.3.5-3.

-	The transmit timing of all MsgA PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.3.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 6.3.2.2.3.5-2A.

Test 3: Correct behaviour when not receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.3.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.3.5-3.

-	The transmit timing of all MsgA PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.3.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 6.3.2.2.3.5-2A.

Table 6.3.2.2.3.5-2: MsgA PRACH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 6.3.2.2.3.5-2A: MsgA PUSCH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 6.3.2.2.3.5-3: Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | MsgA PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 dB |

Table 6.3.2.2.3.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 6.3.2.2.4	NR SA FR1 2-step non-contention based random access

6.3.2.2.4.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

6.3.2.2.4.2	Test applicability

This test applies to all types of NR FR1 UE from Release 16 onwards and twoStepRACH-r16.

6.3.2.2.4.3	Minimum conformance requirement

Same as in clause 4.3.2.2.4.3.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2 and A.6.3.2.2.4.

6.3.2.2.4.4	Test description

6.3.2.2.4.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.2.4.4.1-1.

Table 6.3.2.2.4.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 6.3.2.2.4-1 | 1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.3.2.2.4-2 | 2 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.2.4.4.1-2.

Table 6.3.2.2.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.1 and Table E.3-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.3.2.2.4.4.3.

2.	Cell 1 is the NR FR1 serving cell (PCell). The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.3.2.2.4.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR1. The System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 6.3.2.2.1.5-1.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when transmitting MsgA:

4.1.	The UE shall send a MsgA to the System Simulator. The System Simulator shall check that the MsgA preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex.

4.2	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 6.3.2.2.4.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 6.3.2.2.4.5.

5.	Test 2: Correct behaviour when receiving MsgB:

5.1.	Repeat steps 1-3.

5.2.	The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB containing identifiers that do not match the transmitted Random Access Preamble.

5.3.	As the received MsgB contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

5.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

5.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

5.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 6.3.2.2.4.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 6.3.2.2.4.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 6.3.2.2.4.5.

6.	Test 3: Correct behaviour when not receiving MsgB:

6.1.	Repeat steps 1-3.

6.2.	The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

6.3.	As no MsgB was received within the MsgB Response window configured in RACH-ConfigGenericTwoStepRA, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

6.4.	The System Simulator shall transmit a MsgB containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit an ACK and stop monitoring for MsgB.

6.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 6.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in clause 6.3.2.2.3.5. Measure the power of the first MsgA PUSCH transmission and it shall not exceed the values specified in clause 6.3.2.2.4.5.

6.3.2.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3.1 with the following exceptions:

Table 6.3.2.2.4.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

6.3.2.2.4.5	Test requirement

Table 6.3.2.2.4.5-2 defines the primary level settings for non-contention based random access test in FR1 for NR Standalone.

Table 6.3.2.2.1.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 | As defined in A.3.1, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB.2 FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.1 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.1 |
| Duplex Mode for Cell 2 |  | Config 1 |  | FDD |  |
|  |  | Config 2 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  | Config 2 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP | Config 1 | dBm/15kHz | -95 |  |
|  |  | Config 2 |  | -98 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP | Config 1 | dBm/15kHz | -115 |  |
|  |  | Config 2 |  | -118 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 2 | As defined in FFS. |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Test 1: Correct behaviour when transmitting MsgA:

-	The MsgA shall be one of the Random Access Preambles associated with SSB index 0.

-	The MsgA shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex.

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.4.5-2.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 6.3.2.2.4.5-2A.

Test 2: Correct behaviour when receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.4.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.4.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.4.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 6.3.2.2.4.5-2A.

Test 3: Correct behaviour when not receiving MsgB:

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.4.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.4.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.4.5-4.

-	The power of the first MsgA PUSCH transmission shall be 6.6 dBm for test configuration 1 and 9.6 dBm for test configuration 2 within the accuracy specified in Table 6.3.2.2.4.5-2A.

Table 6.3.2.2.4.5-2: MsgA PRACH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 6.3.2.2.4.5-2A: MsgA PUSCH Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 6.3.2.2.4.5-3: Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | MsgA PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 dB |

Table 6.3.2.2.4.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

<< Skipped sections >>

##### 7.3.2.2.2	NR SA FR2 non-contention based random access

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

7.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

7.3.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Additionally Test 2 is applicable to UE that supports csi-RS-CFRA-ForHO.

7.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.2.2.

7.3.2.2.2.4	Test description

7.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 7.3.2.2.2.4.1-1.

Table 7.3.2.2.2.4.1-1: Non-Contention based random access test in FR2 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.3.2.2.2-1 | 1 | NR: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 7.3.2.2.2.4.1-2.

Table 7.3.2.2.2.4.1-2: Test Environment for Non-Contention based random access test in FR2 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram | FFS |  |  |

1. Message contents are defined in clause 7.3.2.2.2.4.3.

2. Cell 1 is the NR FR2 serving cell (PCell). The connection setup is done according to the settings in FFS.

7.3.2.2.2.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR2. The System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink. There are two subtests, to test both SSB-based non-contention based random access (subtest 1) and CSI-RS-based non-contention based random access (subtest 2).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.3.2.2.2.5-1 Subtest 1.

3.	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

5.1. Set the parameters according to Table 7.3.2.2.2.5-1 Subtest 2.

5.2. Repeat steps 1-3

5.3. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response

6.1. Repeat steps 1-3

6.2. The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response

7.1. Repeat steps 1-3.

7.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

7.3. As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

7.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.2.5.

7.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.2.2.2.4.3-1: FrequencyInfoUL-SIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-2: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | Not present |  |  |
| rsrp-ThresholdSSB | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-3: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 2 entries |  |  |
| ssb[1] | 0 |  |  |
| ssb[2] | 1 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-4: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -100 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-5: ServingCellConfigCommonSIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

7.3.2.2.2.5	Test requirement

Table 7.3.2.2.2.5-2 defines the primary level settings for non-contention based random access test in FR2 for NR Standalone. Tables 7.3.2.2.2.5-3, 7.3.2.2.2.5-4 and 7.3.2.2.2.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 7.3.2.2.2.5-1: General test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | SSB.1 FR2 | As defined in A.3.2 |
| CSI-RS Configuration | Config 1 |  | N/A | CSI-RS.3.1 TDD | As defined in A.1.4 |
| Duplex Mode for Cell 2 | Config 1 |  | TDD | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | OP.3 | As defined in A.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR3.1 TDD | SR3.1 TDD | As defined in A.1.1. |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [3] |
| PRACH Configuration |  |  | PRACH.2 FR2 | PRACH.3 FR2 | As defined in A.7.2, with exceptions as defined below. |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |  |

Table 7.3.2.2.2.5-2: OTA-related test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  | Rough | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 | -80.6 |  |
|  | Es/IotBB | dB | 21.09 | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 | -95.0 |  |
|  | Es/IotBB | dB | 6.69 | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	The selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.2.5-5.

Test 4: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.2.5-5.

Table 7.3.2.2.2.5-3: Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 7.3.2.2.2.5-4: Relative power tolerance Test requirements

| Measured power | Power step P (dB) | PRACH (dB) |
| --- | --- | --- |
| Measured power of both PRACHs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either PRACHs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| Note 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 7.3.2.2.2.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 7.3.2.2.3	NR SA FR2 2-step contention based random access

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for Te Timing error are in [ ]

-	TT analysis is missing

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

7.3.2.2.3.1	Test purpose

The purpose of this test is to verify that the behaviour of the 2-step RA type random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

7.3.2.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting twoStepRACH-r16.

7.3.2.2.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.2.3.

7.3.2.2.3.4	Test description

7.3.2.2.3.4.1	Initial conditions

This test can be run in the configurations defined in Table 7.3.2.2.3.4.1-1.

Table 7.3.2.2.3.4.1-1: 2-step contention based random access test in FR2 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.3.2.2.3-1 | 1 | NR: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 7.3.2.2.3.4.1-2.

Table 7.3.2.2.3.4.1-2: Test Environment for 2-step RACH contention based random access test in FR2 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.2.3.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram | FFS |  |  |

1. Message contents are defined in clause 7.3.2.2.3.4.3.

2. Cell 1 is the NR FR2 serving cell (PCell). The connection setup is done according to the settings in FFS.

7.3.2.2.3.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR2. The System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 7.3.2.2.3.5-1 and 7.3.2.2.3.5-2.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when transmitting MsgA

4.1. The UE shall send the MsgA PRACH and MsgA PUSCH to the System Simulator with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB. The UE shall send the MsgA PUSCH on the MsgA PUSCH occasion that is associated with the chosen preamble.

5.	Test 2: Correct behaviour when receiving MsgB

5.1. Repeat steps 1-3.

5.2. The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

5.3. The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [20], and transmit MsgA with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB’s contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble of MsgA.

5.4 The System Simulator shall transmit a MsgB containing fallbackRAR message with a Random Access Preamble identifier matching the transmitted MsgA PRACH after 3 MsgA transmissions have been received by the System Simulator.

5.5 As the received Random Access Response MsgB with a fallbackRAR contains Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, the UE may stop monitoring for MsgB(s) and shall transmit the msg3 on the indicated PUSCH resources.

5.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.3.5.

6.	Test 3: Correct behaviour when not receiving MsgB

6.1. Repeat steps 1-3.

6.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

6.3. The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [20], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB is received within the MsgB Response window.

6.4. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.3.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.3.5.

7.3.2.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.2.2.3.4.3-1: FrequencyInfoUL-SIB for 2-step contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 7.3.2.2.3.4.3-2: RACH-ConfigCommon for 2-step contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| msgA-TotalNumberOfRA-Preambles-r16 | 48 |  |  |
| msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16 CHOICE { | oneFourth, n48 |  |  |
| one | n4 |  | FR2 |
| } |  |  |  |
| msgA-CB-PreamblesPerSSB-PerSharedRO-r16 | oneFourth, n48 |  |  |
| msgA-SSB-SharedRO-MaskIndex-r16 | Not present |  |  |
| groupB-ConfiguredTwoStepRA-r16 | Not present |  |  |
| msgA-PRACH-RootSequenceIndex-r16 CHOICE {} | Not present |  |  |
| msgA-TransMax-r16 | N8 |  |  |
| msgA-RSRP-Threshold-r16 | RSRP_69 +ΔDL |  |  |
| msgA-RSRP-ThresholdSSB-r16 | RSRP_69 +ΔDL |  |  |
| msgA-SubcarrierSpacing-r16 | 120 kHz |  |  |
| msgA-RestrictedSetConfig-r16 | Not present |  |  |
| ra-PrioritizationForAccessIdentityTwoStep-r16 SEQUENCE { |  |  |  |
| ra-Prioritization-r16 | RA-Prioritization |  |  |
| ra-PrioritizationForAI-r16 | '10'B |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer-r16 | sf32 |  |  |
| } |  |  |  |

Table 7.3.2.2.3.4.3-3: RACH-ConfigGeneric for 2-step contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGenericTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| msgA-PRACH-ConfigurationIndex-r16 | 190 |  |  |
| msgA-RO-FDM-r16 | four |  |  |
| msgA-RO-FrequencyStart-r16 | 0 |  |  |
| msgA-ZeroCorrelationZoneConfig-r16 | 11 |  |  |
| msgA-PreamblePowerRampingStep-r16 | dB2 |  |  |
| msgA-PreambleReceivedTargetPower-r16 | -100 |  |  |
| msgB-ResponseWindow-r16 | sl10 |  |  |
| preambleTransMax-r16 | n10 |  |  |
| } |  |  |  |

Table 7.3.2.2.3.4.3-4: ServingCellConfigCommonSIB for 2-step contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

Table 7.3.2.2.3.4.3-5: MsgA-PUSCH-Config for 2-step contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Resource-r16 ::= SEQUENCE { |  |  |  |
| msgA-MCS | 1 |  |  |
| nrofSlotsMsgA-PUSCH | 1 |  |  |
| nrofMsgA-PO-PerSlot | 1 |  |  |
| msgA-PUSCH-TimeDomainOffset | 1 |  |  |
| msgA-PUSCH-TimeDomainAllocation-r16 | 3 | Equivalent to PUSCH start symbol = 0 and allocation length = 10 |  |
| mappingTypeMsgA-PUSCH | typeA |  |  |
| nrofPRBs-PerMsgA-PO | 2 |  |  |
| nrofMsgA-PO-FDM | One |  |  |
| msgA-DMRS-AdditionalPosition | pos1 |  |  |
| msgA-PUSCH-NrofPorts | 1 |  |  |
| msgA-DeltaPreamble | 3 |  |  |
| msgA-Alpha | alpha1 |  |  |
| deltaMCS | Disabled |  |  |
| } |  |  |  |

7.3.2.2.3.5	Test requirement

Table 7.3.2.2.3.5-2 defines the primary level settings for 2-step contention based random access test in FR2 for NR Standalone. Tables 7.3.2.2.3.5-3, 7.3.2.2.3.5-4 and 7.3.2.2.3.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 7.3.2.2.3.5-1: General test parameters for 2-step contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | As defined in A.3.2 |
| Duplex Mode for Cell 1 | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OCNG pattern 1 | As defined in A.2.1 |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR.3.1 TDD | As defined in A.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [3] |
| MsgA Configuration |  |  | FR2 MsgA configuration 1 | As defined in A.7.2, with exceptions as defined below |
| msgA-RSRP-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PMsgA0 -1), where PMsgA0 is the measured first MsgA PRACH power with -80.6dBm/SCS applied, msgA-PreambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send MsgA. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table 7.3.2.2.3.5-2: OTA-related test parameters for 2-step contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 2b | As defined in A.9.2.2 |
| Assumption for UE beamsNote 2 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

Test 1: Correct behaviour when transmitting MsgA

-	The MsgA with preamble shall be one of the Random Access Preambles associated with SSB index 0.

Test 2: Correct behaviour when receiving MsgB

-	The power of the first MsgA PRACH preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.3.5-3.

- 	The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [8], where μ indicates the MsgA PUSCH numerology.

-	The relative power for MsgA ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.3.5-4.

-	The transmit timing of all MsgA transmissions shall be within the accuracy specified in Table 7.3.2.2.3.5-5.

Test 3: Correct behaviour when not receiving MsgB

-	The power of the first MsgA PRACH preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.3.5-3.

- 	The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [8], where μ indicates the MsgA PUSCH numerology.

-	The relative power for MsgA ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.3.5-4.

-	The transmit timing of all MsgA transmissions shall be within the accuracy specified in Table 7.3.2.2.3.5-5.

Table 7.3.2.2.3.5-3: Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 7.3.2.2.3.5-4: Relative power tolerance Test requirements

| Measured power | Power step P (dB) | MsgA (dB) |
| --- | --- | --- |
| Measured power of both MsgAs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either MsgAs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| Note 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 7.3.2.2.3.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 7.3.2.2.4	NR SA FR2 2-step non-contention based random access

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for Te Timing error are in [ ]

-	TT analysis is missing

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

7.3.2.2.4.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH and MsgA PUSCH power settings and timing are within specified limits.

7.3.2.2.4.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting twoStepRACH-r16.

7.3.2.2.4.3	Minimum conformance requirement

Same as in clause 4.3.2.2.4.3.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2 and A.7.3.2.2.4.

7.3.2.2.4.4	Test description

7.3.2.2.4.4.1	Initial conditions

This test can be run in the configurations defined in Table 7.3.2.2.4.4.1-1.

Table 7.3.2.2.4.4.1-1: 2-step non-contention based random access test in FR2 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.3.2.2.4-1 | 1 | NR: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 7.3.2.2.4.4.1-2.

Table 7.3.2.2.4.4.1-2: Test Environment for 2-step RACH non-dsscontention based random access test in FR2 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.2.4.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram | FFS |  |  |

1.	Message contents are defined in clause 7.3.2.2.4.4.3.

2.	Cell 1 is the NR FR2 serving cell (PCell). The connection setup is done according to the settings in FFS.

7.3.2.2.4.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR2. The System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.3.2.2.4.5-1 and  7.3.2.2.4.5-2.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when transmitting MsgA:

4.1.	The UE shall send the MsgA PRACH and MsgA PUSCH to the System Simulator with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB. The UE shall send the MsgA PUSCH on the MsgA PUSCH occasion that is associated with the chosen preamble.

5.	Test 2: Correct behaviour when receiving MsgB:

5.1.	Repeat steps 1-3.

5.2.	The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a MsgB containing identifiers that do not match the transmitted Random Access Preamble.

5.3.	As the received MsgB contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [12], and transmit with the calculated MsgA PRACH transmission power when the backoff time expires.

5.4.	The System Simulator shall transmit a MsgB containing containing fallbackRAR with a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 MsgA transmissions have been received by the System Simulator.

5.5.	As the received MsgB contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for MsgB(s) and shall transmit the msg3 on the indicated PUSCH resources.

5.6.	Measure the power and timing of the first preamble and it shall not exceed the values specified in clause 7.3.2.2.4.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in clause 7.3.2.2.4.5.

6.	Test 3: Correct behaviour when not receiving MsgB:

6.1.	Repeat steps 1-3.

6.2.	The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

6.3.	The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [20], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB is received within the MsgB Response window in RACH-ConfigGenericTwoStepRA.

6.4.	Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.4.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.4.5.

7.3.2.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.2.2.4.4.3-1: FrequencyInfoUL-SIB for 2-step non-contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 7.3.2.2.4.4.3-2: RACH-ConfigCommon for 2-step non-contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| msgA-TotalNumberOfRA-Preambles-r16 | 48 |  |  |
| msgA-SSB-SharedRO-MaskIndex-r16 | Not present |  |  |
| groupB-ConfiguredTwoStepRA-r16 | Not present |  |  |
| msgA-PRACH-RootSequenceIndex-r16 CHOICE {} | Not present |  |  |
| msgA-TransMax-r16 | N8 |  |  |
| msgA-RSRP-Threshold-r16 | RSRP_69 +ΔDL |  |  |
| msgA-RSRP-ThresholdSSB-r16 | RSRP_69 +ΔDL |  |  |
| msgA-SubcarrierSpacing-r16 | 120 kHz |  |  |
| msgA-RestrictedSetConfig-r16 | Not present |  |  |
| ra-PrioritizationForAccessIdentityTwoStep-r16 SEQUENCE { |  |  |  |
| ra-Prioritization-r16 | RA-Prioritization |  |  |
| ra-PrioritizationForAI-r16 | '10'B |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer-r16 | Not present |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-3: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 2 entries |  |  |
| ssb[1] | 0 |  |  |
| ssb[2] | 1 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.4.4.3-3: RACH-ConfigGeneric for 2-step non-contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGenericTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| msgA-PRACH-ConfigurationIndex-r16 | 190 |  |  |
| msgA-RO-FDM-r16 | four |  |  |
| msgA-RO-FrequencyStart-r16 | 0 |  |  |
| msgA-ZeroCorrelationZoneConfig-r16 | 11 |  |  |
| msgA-PreamblePowerRampingStep-r16 | dB2 |  |  |
| msgA-PreambleReceivedTargetPower-r16 | -100 |  |  |
| msgB-ResponseWindow-r16 | sl10 |  |  |
| preambleTransMax-r16 | n10 |  |  |
| } |  |  |  |

Table 7.3.2.2.4.4.3-4: ServingCellConfigCommonSIB for 2-step non-contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

Table 7.3.2.2.4.4.3-5: MsgA-PUSCH-Config for 2-step non-contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Resource-r16 ::= SEQUENCE { |  |  |  |
| msgA-MCS | 1 |  |  |
| nrofSlotsMsgA-PUSCH | 1 |  |  |
| nrofMsgA-PO-PerSlot | 1 |  |  |
| msgA-PUSCH-TimeDomainOffset | 1 |  |  |
| msgA-PUSCH-TimeDomainAllocation-r16 | 3 | Equivalent to PUSCH start symbol = 0 and allocation length = 10 |  |
| mappingTypeMsgA-PUSCH | typeA |  |  |
| nrofPRBs-PerMsgA-PO | 2 |  |  |
| nrofMsgA-PO-FDM | One |  |  |
| msgA-DMRS-AdditionalPosition | pos1 |  |  |
| msgA-PUSCH-NrofPorts | 1 |  |  |
| msgA-DeltaPreamble | 3 |  |  |
| msgA-Alpha | alpha1 |  |  |
| deltaMCS | Disabled |  |  |
| } |  |  |  |

7.3.2.2.4.5	Test requirement

Table 7.3.2.2.4.5-2 defines the primary level settings for 2-step non-contention based random access test in FR2 for NR Standalone. Tables 7.3.2.2.4.5-3, 7.3.2.2.4.5-4 and 7.3.2.2.4.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 7.3.2.2.4.5-1: General test parameters for non-contention based random access test for 2-step RA type in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | As defined in A.3.10 |
| Duplex Mode for Cell 1 | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR3.1 TDD | As defined in A.3.1.1. |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power (PCMAX,f,c) |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| MsgA Configuration |  |  | FR2 MsgA configuration 2 | As defined in A.3.20.3, with exceptions as defined below. |
| msgA-RSRP-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| msgA-PreambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PMsgA0 -1), where PMsgA0 is the measured first MsgA PRACH power with -80.6dBm/SCS applied, msgA-PreambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send MsgA. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table 7.3.2.2.4.5-2 OTA-related test parameters for non-contention based random access test for 2-step RA type in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | As defined in A.9.1 |
| Assumption for UE beams Note 2 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

Test 1: Correct behaviour when transmitting MsgA

-	The MsgA with preamble shall be one of the Random Access Preambles associated with SSB index 0.

Test 2: Correct behaviour when receiving MsgB

-	The power of the first MsgA PRACH preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.4.5-3.

- 	The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [8], where μ indicates the MsgA PUSCH numerology.

-	The relative power for MsgA ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.4.5-4.

-	The transmit timing of all MsgA transmissions shall be within the accuracy specified in Table 7.3.2.2.4.5-5.

Test 3: Correct behaviour when not receiving MsgB

-	The power of the first MsgA PRACH preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.4.5-3.

- 	The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [8], where μ indicates the MsgA PUSCH numerology.

-	The relative power for MsgA ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.4.5-4.

-	The transmit timing of all MsgA transmissions shall be within the accuracy specified in Table 7.3.2.2.4.5-5.

Table 7.3.2.2.4.5-3: Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 7.3.2.2.4.5-4: Relative power tolerance Test requirements

| Measured power | Power step P (dB) | MsgA (dB) |
| --- | --- | --- |
| Measured power of both MsgAs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either MsgAs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| Note 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 7.3.2.2.4.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

<< End of Changes >>