3GPP TSG WG RAN5 Meeting #103																R5-242105

Fukuoka, Japan

20th – 24th May 2024

3GPP TSG WG RAN5 Meeting #102																		R5-240019

Athens, Greece

26th February– 1st March 2024

# Report from the RAN WG5#102 Meeting

# Athens, Greece

26th February – 1st March 2024

# v 1.0

# Chair:					Jacob John, Motorola Mobility

Meeting Secretary:		Ingbert Sigovich, ETSI/MCC Project manager

Vice Chair (SIG sub WG):	Xiaozhong Chen, CATT

RF session Chair (RF sub WG):	Pradeep Gowda, Qualcomm Inc

RF session Secretary:		Amy Tao, Bureau Veritas

Contents:

1	Opening of the meeting	20

1.1	Welcoming brief by the host	20

2	Reports	20

2.1	Live Reports	20

2.2	General Reports for information	23

3	Incoming Liaison Statements	23

4	RAN5 General Issues	24

4.1	New Work Item proposals - for intro only	24

4.2	General Discussion Papers	39

4.2.1	5GS	39

4.2.2	NR Applicability specifications (Software Implementation)	39

4.2.3	All other topics	39

4.3	RAN5 PRDs/Templates	40

4.4	Meeting schedule for 2024-25	40

4.5	Tdocs for mid-week joint session	40

4.5.1	Agenda for mid-week session	40

4.5.2	RF group docs for WG review/verdict - original A.I. retained	40

4.5.3	Sig group docs for WG review/verdict - original A.I. retained	41

4.5.4	Other open issues from joint sessions - original A.I. retained	42

4.5.5	Other	43

5	RF Functional Area	43

5.1	Review action points (fm A.I. 2.1)	43

5.2	Review incoming LS (fm A.I. 3) & new subject discussion papers	43

5.3	Open Work Items	43

5.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083) NR_CADC_NR_LTE_DC_R16-UEConTest	43

5.3.1.1	TS 38.508-1	43

5.3.1.1.1	Test frequencies (Clause 4.3.1)	43

5.3.1.1.2	Test environment for RF (Clauses 5)	44

5.3.1.1.3	Test environment for RRM (Clause 7)	44

5.3.1.1.4	Other clauses / Annexes	44

5.3.1.2	TS 38.508-2	44

5.3.1.3	TS 38.521-1	44

5.3.1.3.1	Tx Requirements (Clause 6)	44

5.3.1.3.2	Rx Requirements (Clause 7)	47

5.3.1.3.3	Clauses 1-5 / Annexes	49

5.3.1.4	TS 38.521-2	49

5.3.1.4.1	Tx Requirements (Clause 6)	49

5.3.1.4.2	Rx Requirements (Clause 7)	49

5.3.1.4.3	Clauses 1-5 / Annexes	49

5.3.1.5	TS 38.521-3	49

5.3.1.5.1	Tx Requirements (Clause 6)	49

5.3.1.5.2	Rx Requirements (Clause 7)	51

5.3.1.5.3	Clauses 1-5 / Annexes	51

5.3.1.6	TS 38.521-4	51

5.3.1.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	51

5.3.1.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	51

5.3.1.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	51

5.3.1.6.4	Clauses 1-4 / Annexes	51

5.3.1.7	TS 38.522	51

5.3.1.8	TS 38.533	52

5.3.1.9	TR 38.903 (NR MU & TT analyses)	52

5.3.1.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	52

5.3.1.11	Discussion Papers / Work Plan / TC lists	53

5.3.2	5G V2X with NR sidelink (UID-880069) 5G_V2X_NRSL_eV2XARC-UEConTest	53

5.3.2.1	TS 38.508-1	53

5.3.2.1.1	Test frequencies (Clause 4.3.1)	53

5.3.2.1.2	Test environment for RF (Clauses 5)	53

5.3.2.1.3	Test environment for RRM (Clause 7)	53

5.3.2.1.4	Other clauses / Annexes	53

5.3.2.2	TS 38.508-2	53

5.3.2.3	TS 38.509	53

5.3.2.4	TS 38.521-1	53

5.3.2.4.1	Tx Requirements (Clause 6)	53

5.3.2.4.2	Rx Requirements (Clause 7)	55

5.3.2.4.3	Clauses 1-5 / Annexes	55

5.3.2.5	TS 38.521-3	55

5.3.2.5.1	Tx Requirements (Clause 6)	55

5.3.2.5.2	Rx Requirements (Clause 7)	55

5.3.2.5.3	Clauses 1-5 / Annexes	55

5.3.2.6	TS 38.521-4	55

5.3.2.6.1	V2X Requirements (Clause 11)	55

5.3.2.6.2	Clauses 1-4 / Annexes	55

5.3.2.7	TS 38.522	55

5.3.2.8	TS 38.533	55

5.3.2.9	TS 36.509	55

5.3.2.10	TR 38.903 (NR MU & TT analyses)	55

5.3.2.11	TR 38.905 (NR Test Points Radio Transmission and Reception)	55

5.3.2.12	Discussion Papers / Work Plan / TC lists	56

5.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest	56

5.3.3.1	TS 38.508-1	56

5.3.3.1.1	Test frequencies (Clause 4.3.1)	56

5.3.3.1.2	Test environment for RF (Clauses 5)	57

5.3.3.1.3	Test environment for RRM (Clause 7)	57

5.3.3.1.4	Other clauses / Annexes	57

5.3.3.2	TS 38.508-2	57

5.3.3.3	TS 38.521-1	59

5.3.3.3.1	Tx Requirements (Clause 6)	59

5.3.3.3.2	Rx Requirements (Clause 7)	60

5.3.3.3.3	Clauses 1-5 / Annexes	63

5.3.3.4	TS 38.521-2	64

5.3.3.4.1	Tx Requirements (Clause 6)	64

5.3.3.4.2	Rx Requirements (Clause 7)	64

5.3.3.4.3	Clauses 1-5 / Annexes	64

5.3.3.5	TS 38.521-3	64

5.3.3.5.1	Tx Requirements (Clause 6)	64

5.3.3.5.2	Rx Requirements (Clause 7)	64

5.3.3.5.3	Clauses 1-5 / Annexes	64

5.3.3.6	TS 38.521-4	65

5.3.3.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	65

5.3.3.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	65

5.3.3.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	65

5.3.3.6.4	Clauses 1-4 / Annexes	65

5.3.3.7	TS 38.522	65

5.3.3.8	TS 38.533	66

5.3.3.9	TR 38.903 (NR MU & TT analyses)	66

5.3.3.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	66

5.3.3.11	Discussion Papers / Work Plan / TC lists	68

5.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest	68

5.3.4.1	TS 38.508-1	68

5.3.4.1.1	Test frequencies (Clause 4.3.1)	68

5.3.4.1.2	Test environment for RF (Clauses 5)	68

5.3.4.1.3	Test environment for RRM (Clause 7)	68

5.3.4.1.4	Other clauses / Annexes	68

5.3.4.2	TS 38.508-2	68

5.3.4.3	TS 38.509	69

5.3.4.4	TS 38.521-1	69

5.3.4.4.1	Tx Requirements (Clause 6)	69

5.3.4.4.2	Rx Requirements (Clause 7)	71

5.3.4.4.3	Clauses 1-5 / Annexes	71

5.3.4.5	TS 38.521-3	72

5.3.4.5.1	Tx Requirements (Clause 6)	72

5.3.4.5.2	Rx Requirements (Clause 7)	72

5.3.4.5.3	Clauses 1-5 / Annexes	72

5.3.4.6	TS 38.521-4	72

5.3.4.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	72

5.3.4.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	72

5.3.4.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	72

5.3.4.6.4	Clauses 1-4 / Annexes	72

5.3.4.7	TS 38.522	73

5.3.4.8	TS 38.533	74

5.3.4.9	TR 38.903 (NR MU & TT analyses)	80

5.3.4.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	82

5.3.4.11	Discussion Papers / Work Plan / TC lists	83

5.3.5	Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2 / 3 / y=1 or x=1 / 2 / y=2) (UID-930051) ENDC_PC2_R17_xLTE_yNR-UEConTest	83

5.3.5.1	TS 38.508-2	83

5.3.5.2	TS 38.521-3	84

5.3.5.2.2	Rx Requirements (Clause 7)	84

5.3.5.2.3	Clauses 1-5 / Annexes	85

5.3.5.3	TS 38.522	85

5.3.5.4	Discussion Papers / Work Plan / TC lists	85

5.3.6.	Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 (UID-950062) NR_FR2_FWA_Bn257_Bn258-UEConTest	85

5.3.6.1	TS 38.508-1	85

5.3.6.2	TS 38.508-2	85

5.3.6.3	TS 38.521-2	85

5.3.6.3.1	Tx Requirements (Clause 6)	85

5.3.6.3.2	Rx Requirements (Clause 7)	85

5.3.6.3.3	Clauses 1-5 / Annexes	85

5.3.6.4	TS 38.521-4	85

5.3.6.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	85

5.3.6.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	85

5.3.6.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	85

5.3.6.4.4	Clauses 1-4 / Annexes	85

5.3.6.5	TS 38.522	85

5.3.6.6	TS 38.533	85

5.3.6.7	TR 38.903 (NR MU & TT analyses)	85

5.3.6.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	85

5.3.6.9	Discussion Papers / Work Plan / TC lists	85

5.3.7	NR coverage enhancements (UID-950063) NR_cov_enh-UEConTest	85

5.3.7.1	TS 38.508-1	85

5.3.7.2	TS 38.508-2	85

5.3.7.3	TS 38.521-1	85

5.3.7.3.1	Tx Requirements (Clause 6)	85

5.3.7.3.2	Rx Requirements (Clause 7)	86

5.3.7.3.3	Clauses 1-5 / Annexes	86

5.3.7.4	TS 38.521-2	86

5.3.7.4.1	Tx Requirements (Clause 6)	86

5.3.7.4.2	Rx Requirements (Clause 7)	87

5.3.7.4.3	Clauses 1-5 / Annexes	87

5.3.7.5	TS 38.522	87

5.3.7.6	TR 38.903 (NR MU & TT analyses)	88

5.3.7.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	88

5.3.7.8	Discussion Papers / Work Plan / TC lists	89

5.3.8	Support of reduced capability NR devices (UID-950066) NR_redcap_plus_ARCH-UEConTest	89

5.3.8.1	TS 38.508-1	89

5.3.8.2	TS 38.508-2	89

5.3.8.3	TS 38.521-1	89

5.3.8.3.1	Tx Requirements (Clause 6)	89

5.3.8.3.2	Rx Requirements (Clause 7)	89

5.3.8.3.3	Clauses 1-5 / Annexes	90

5.3.8.4	TS 38.521-2	90

5.3.8.4.1	Tx Requirements (Clause 6)	90

5.3.8.4.2	Rx Requirements (Clause 7)	90

5.3.8.4.3	Clauses 1-5 / Annexes	90

5.3.8.5	TS 38.521-4	90

5.3.8.5.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	90

5.3.8.5.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	92

5.3.8.5.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	93

5.3.8.5.4	Clauses 1-4 / Annexes	93

5.3.8.6	TS 38.522	93

5.3.8.7	TS 38.533	95

5.3.8.8	TR 38.903 (NR MU & TT analyses)	118

5.3.8.9	TR 38.905 (NR Test Points Radio Transmission and Reception)	128

5.3.8.10	Discussion Papers / Work Plan / TC lists	128

5.3.9	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest	128

5.3.9.1	TS 38.508-1	128

5.3.9.2	TS 38.508-2	128

5.3.9.3	TS 38.522	128

5.3.9.4	TS 38.533	128

5.3.9.5	TR 38.903 (NR MU & TT analyses)	129

5.3.9.6	Discussion Papers / Work Plan / TC lists	129

5.3.10	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest	129

5.3.10.1	TS 38.508-1	129

5.3.10.2	TS 38.508-2	131

5.3.10.3	TS 38.509	131

5.3.10.4	TS 38.521-5	131

5.3.10.5	TS 38.522	141

5.3.10.6	TS 38.533	142

5.3.10.7	TR 38.903 (NR MU & TT analyses)	144

5.3.10.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	144

5.3.10.9	Discussion Papers / Work Plan / TC lists	144

5.3.11	Further enhancement on NR demodulation performance (UID-960075) NR_demod_enh2-UEConTest	145

5.3.11.1	TS 38.508-1	145

5.3.11.2	TS 38.508-2	145

5.3.11.3	TS 38.521-4	145

5.3.11.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	147

5.3.11.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	148

5.3.11.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	148

5.3.11.3.4	Clauses 1-4 / Annexes	148

5.3.11.4	TS 38.522	149

5.3.11.5	TR 38.903 (NR MU & TT analyses)	149

5.3.11.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	150

5.3.11.7	Discussion Papers / Work Plan / TC lists	150

5.3.12	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest	150

5.3.12.1	TS 38.508-1	150

5.3.12.2	TS 38.508-2	150

5.3.12.3	TS 38.521-4	151

5.3.12.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	151

5.3.12.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	151

5.3.12.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	151

5.3.12.3.4	Clauses 1-4 / Annexes	151

5.3.12.4	TS 38.522	151

5.3.12.5	TS 38.533	153

5.3.12.6	TR 38.903 (NR MU & TT analyses)	153

5.3.12.7	Discussion Papers / Work Plan / TC lists	153

5.3.13	NR support for high speed train scenario in frequency range 2 (FR2) (UID-960080) NR_HST_FR2-UEConTest	153

5.3.13.1	TS 38.508-1	153

5.3.13.2	TS 38.508-2	153

5.3.13.3	TS 38.521-2	153

5.3.13.3.1	Tx Requirements (Clause 6)	153

5.3.13.3.2	Rx Requirements (Clause 7)	153

5.3.13.3.3	Clauses 1-5 / Annexes	153

5.3.13.4	TS 38.521-4	153

5.3.13.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	153

5.3.13.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	153

5.3.13.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	153

5.3.13.4.4	Clauses 1-4 / Annexes	153

5.3.13.5	TS 38.522	153

5.3.13.6	TS 38.533	154

5.3.13.7	TR 38.903 (NR MU & TT analyses)	154

5.3.13.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	154

5.3.13.9	Discussion Papers / Work Plan / TC lists	154

5.3.14	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest	154

5.3.14.1	TS 38.508-1	154

5.3.14.2	TS 38.508-2	155

5.3.14.3	TS 38.521-1	156

5.3.14.3.1	Tx Requirements (Clause 6)	156

5.3.14.3.2	Rx Requirements (Clause 7)	156

5.3.14.3.3	Clauses 1-5 / Annexes	156

5.3.14.4	TS 38.522	156

5.3.14.5	TS 38.533	156

5.3.14.6	TR 38.903 (NR MU & TT analyses)	156

5.3.14.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	156

5.3.14.8	Discussion Papers / Work Plan / TC lists	156

5.3.15	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest	156

5.3.15.1	TS 38.508-1	156

5.3.15.2	TS 38.508-2	156

5.3.15.3	TS 38.521-1	156

5.3.15.3.1	Tx Requirements (Clause 6)	156

5.3.15.3.2	Rx Requirements (Clause 7)	156

5.3.15.3.3	Clauses 1-5 / Annexes	156

5.3.15.4	TS 38.522	156

5.3.15.5	TS 38.533	156

5.3.15.6	TR 38.903 (NR MU & TT analyses)	156

5.3.15.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	156

5.3.15.8	Discussion Papers / Work Plan / TC lists	156

5.3.16	UE power saving enhancements for NR (UID-960086) NR_UE_pow_sav_enh_plus_CT-UEConTest	156

5.3.16.1	TS 38.508-1	156

5.3.16.2	TS 38.508-2	156

5.3.16.3	TS 38.522	156

5.3.16.4	TS 38.533	157

5.3.16.5	TR 38.903 (NR MU & TT analyses)	158

5.3.16.6	Discussion Papers / Work Plan / TC lists	158

5.3.17	NR RRM enhancement (UID-960089) NR_RRM_enh-UEConTest	158

5.3.17.1	TS 38.508-1	158

5.3.17.2	TS 38.508-2	158

5.3.17.3	TS 38.522	158

5.3.17.4	TS 38.533	158

5.3.17.5	TR 38.903 (NR MU & TT analyses)	164

5.3.17.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	165

5.3.17.7	Discussion Papers / Work Plan / TC lists	165

5.3.18	RF requirements enhancement for NR frequency range 1 (FR1) (UID-960090) NR_RF_FR1_enh-UEConTest	165

5.3.18.1	TS 38.508-1	165

5.3.18.2	TS 38.508-2	165

5.3.18.3	TS 38.521-1	165

5.3.18.3.1	Tx Requirements (Clause 6)	165

5.3.18.3.2	Rx Requirements (Clause 7)	173

5.3.18.3.3	Clauses 1-5 / Annexes	173

5.3.18.4	TS 38.522	173

5.3.18.5	TS 38.533	174

5.3.18.6	TR 38.903 (NR MU & TT analyses)	175

5.3.18.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	175

5.3.18.8	Discussion Papers / Work Plan / TC lists	176

5.3.19	Further enhancements of NR RF requirements for frequency range 2 (FR2) (UID-970070) NR_RF_FR2_req_enh2-UEConTest	176

5.3.19.1	TS 38.508-1	176

5.3.19.2	TS 38.508-2	176

5.3.19.3	TS 38.521-2	176

5.3.19.3.1	Tx Requirements (Clause 6)	176

5.3.19.3.2	Rx Requirements (Clause 7)	177

5.3.19.3.3	Clauses 1-5 / Annexes	178

5.3.19.4	TS 38.521-3	178

5.3.19.4.1	Tx Requirements (Clause 6)	178

5.3.19.4.2	Rx Requirements (Clause 7)	179

5.3.19.4.3	Clauses 1-5 / Annexes	179

5.3.19.5	TS 38.522	179

5.3.19.6	TS 38.533	179

5.3.19.7	TR 38.903 (NR MU & TT analyses)	180

5.3.19.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	180

5.3.19.9	Discussion Papers / Work Plan / TC lists	180

5.3.20	NR Positioning Enhancements  (UID-970075) NR_pos_enh-UEConTest	181

5.3.20.1	TS 38.508-1	181

5.3.20.1.1	Test frequencies (Clause 4.3.1)	181

5.3.20.1.2	Test environment for RF (Clauses 5)	181

5.3.20.1.3	Test environment for RRM (Clause 7)	181

5.3.20.1.4	Other clauses / Annexes	181

5.3.20.2	TS 37.571-1	181

5.3.20.3	TS 37.571-3	187

5.3.20.4	TS 37.571-5	187

5.3.20.5	TR 38.903 (NR MU & TT analyses)	188

5.3.20.6	Discussion Papers / Work Plan / TC lists	189

5.3.21	Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink (UID-981033) DL_intrpt_combos_TxSW_R17-UEConTest	189

5.3.21.1	TS 38.508-2	189

5.3.21.2	TS 38.521-1	189

5.3.21.2.1	Tx Requirements (Clause 6)	189

5.3.21.2.2	Rx Requirements (Clause 7)	189

5.3.21.2.3	Clauses 1-5 / Annexes	189

5.3.21.3	TS 38.522	190

5.3.21.4	TS 38.533	190

5.3.21.5	Discussion Papers / Work Plan / TC lists	190

5.3.22	NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) (UID-981034) LTE_NBIOT_eMTC_NTN_req-UEConTest	190

5.3.22.1	TS 36.508	190

5.3.22.2	TS 36.509	192

5.3.22.3	TS 36.521-2	192

5.3.22.4	TS 36.521-3	193

5.3.22.5	TS 36.521-4	200

5.3.22.6	TR 36.903 (E-UTRAN RRM TT analyses)	205

5.3.22.7	TR 36.904 (E-UTRAN Radio Reception TT analyses)	206

5.3.22.8	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )	206

5.3.22.9	Discussion Papers / Work Plan / TC lists	206

5.3.23	NR and MR-DC measurement gap enhancements (UID-981035) NR_MG_enh-UEConTest	208

5.3.23.1	TS 38.508-1	208

5.3.23.2	TS 38.508-2	208

5.3.23.3	TS 38.522	208

5.3.23.4	TS 38.533	208

5.3.23.5	TR 38.903 (NR MU & TT analyses)	210

5.3.23.6	Discussion Papers / Work Plan / TC lists	212

5.3.24	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest	212

5.3.24.1	TS 38.508-1	212

5.3.24.2	TS 38.508-2	212

5.3.24.3	TS 38.522	212

5.3.24.4	TS 38.533	213

5.3.24.5	TS 36.508	213

5.3.24.6	TR 38.903 (NR MU & TT analyses)	214

5.3.24.7	Discussion Papers / Work Plan / TC lists	214

5.3.25	Additional NR bands for UL-MIMO in Rel-18 (UID-1000050) NR_bands_UL_MIMO_R18-UEConTest	214

5.3.25.1	TS 38.508-1	214

5.3.25.2	TS 38.508-2	214

5.3.25.3	TS 38.521-1	215

5.3.25.3.1	Tx Requirements (Clause 6)	215

5.3.25.3.2	Rx Requirements (Clause 7)	215

5.3.25.4	TS 38.522	216

5.3.25.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	216

5.3.25.6	Discussion Papers / Work Plan / TC lists	216

5.3.26	High power UE (power class 2) for NR FR1 FDD single band (UID-1000051) HPUE_NR_FR1_FDD_R18-UEConTest	216

5.3.26.1	TS 38.508-1	216

5.3.26.2	TS 38.508-2	216

5.3.26.3	TS 38.521-1	216

5.3.26.3.1	Tx Requirements (Clause 6)	216

5.3.26.3.2	Rx Requirements (Clause 7)	216

5.3.26.3.3	Clauses 1-5 / Annexes	217

5.3.26.4	TS 38.522	217

5.3.26.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	217

5.3.26.6	Discussion Papers / Work Plan / TC lists	217

5.3.27	High power UE (power class 1.5) for NR FR1 TDD single band (UID-1000053) HPUE_NR_FR1_TDD_R18-UEConTest	217

5.3.27.1	TS 38.508-1	217

5.3.27.2	TS 38.508-2	217

5.3.27.3	TS 38.521-1	217

5.3.27.3.1	Tx Requirements (Clause 6)	217

5.3.27.3.2	Rx Requirements (Clause 7)	218

5.3.27.3.3	Clauses 1-5 / Annexes	218

5.3.27.4	TS 38.522	219

5.3.27.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	219

5.3.27.6	Discussion Papers / Work Plan / TC lists	219

5.3.28	Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations (UID-1000054) HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest	219

5.3.28.1	TS 38.508-1	219

5.3.28.2	TS 38.508-2	219

5.3.28.3	TS 38.521-1	220

5.3.28.3.1	Tx Requirements (Clause 6)	220

5.3.28.3.2	Rx Requirements (Clause 7)	220

5.3.28.3.3	Clauses 1-5 / Annexes	220

5.3.28.4	TS 38.521-3	221

5.3.28.4.1	Tx Requirements (Clause 6)	221

5.3.28.4.2	Rx Requirements (Clause 7)	221

5.3.28.4.3	Clauses 1-5 / Annexes	222

5.3.28.5	TS 38.522	223

5.3.28.6	TR 38.905 (NR Test Points Radio Transmission and Reception )	223

5.3.28.7	Discussion Papers / Work Plan / TC lists	225

5.3.29	Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1 / 2) bands UL (UID-1000056) NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-UEConTest	225

5.3.29.1	TS 38.508-1	225

5.3.29.2	TS 38.508-2	225

5.3.29.3	TS 38.521-1	225

5.3.29.3.1	Tx Requirements (Clause 6)	225

5.3.29.3.2	Rx Requirements (Clause 7)	225

5.3.29.3.3	Clauses 1-5 / Annexes	225

5.3.29.4	TS 38.522	225

5.3.29.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	225

5.3.29.6	Discussion Papers / Work Plan / TC lists	225

5.3.30	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest	225

5.3.30.1	TS 38.508-1	225

5.3.30.1.1	Test frequencies (Clause 4.3.1)	225

5.3.30.1.2	Test environment for RF (Clauses 5)	225

5.3.30.1.3	Test environment for RRM (Clause 7)	225

5.3.30.1.4	Other clauses / Annexes	225

5.3.30.2	TS 38.508-2	225

5.3.30.3	TS 38.521-1	225

5.3.30.3.1	Tx Requirements (Clause 6)	225

5.3.30.3.2	Rx Requirements (Clause 7)	225

5.3.30.3.3	Clauses 1-5 / Annexes	225

5.3.30.4	TS 38.521-2	225

5.3.30.4.1	Tx Requirements (Clause 6)	225

5.3.30.4.2	Rx Requirements (Clause 7)	225

5.3.30.4.3	Clauses 1-5 / Annexes	226

5.3.30.5	TS 38.521-3	226

5.3.30.5.1	Tx Requirements (Clause 6)	226

5.3.30.5.2	Rx Requirements (Clause 7)	226

5.3.30.5.3	Clauses 1-5 / Annexes	226

5.3.30.6	TS 38.521-4	226

5.3.30.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	226

5.3.30.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	226

5.3.30.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	226

5.3.30.6.4	Clauses 1-4 / Annexes	226

5.3.30.7	TS 38.522	226

5.3.30.8	TS 38.533	227

5.3.30.9	TR 38.903 (NR MU & TT analyses)	227

5.3.30.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	227

5.3.30.11	Discussion Papers / Work Plan / TC lists	227

5.3.31	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest	227

5.3.31.1	TS 38.508-1	227

5.3.31.1.1	Test frequencies (Clause 4.3.1)	227

5.3.31.1.2	Test environment for RF (Clauses 5)	227

5.3.31.1.3	Test environment for RRM (Clause 7)	227

5.3.31.1.4	Other clauses / Annexes	227

5.3.31.2	TS 38.508-2	227

5.3.31.3	TS 38.521-1	227

5.3.31.3.1	Tx Requirements (Clause 6)	227

5.3.31.3.2	Rx Requirements (Clause 7)	227

5.3.31.3.3	Clauses 1-5 / Annexes	227

5.3.31.4	TS 38.521-2	227

5.3.31.4.1	Tx Requirements (Clause 6)	227

5.3.31.4.2	Rx Requirements (Clause 7)	227

5.3.31.4.3	Clauses 1-5 / Annexes	227

5.3.31.5	TS 38.521-3	227

5.3.31.5.1	Tx Requirements (Clause 6)	227

5.3.31.5.2	Rx Requirements (Clause 7)	227

5.3.31.5.3	Clauses 1-5 / Annexes	227

5.3.31.6	TS 38.521-4	227

5.3.31.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	227

5.3.31.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	227

5.3.31.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	228

5.3.31.6.4	Clauses 1-4 / Annexes	228

5.3.31.7	TS 38.521-5	228

5.3.31.8	TS 38.533	228

5.3.31.9	TS 36.521-4	228

5.3.31.10	TR 38.903 (NR MU & TT analyses)	228

5.3.31.11	TR 38.905 (NR Test Points Radio Transmission and Reception)	228

5.3.31.12	Discussion Papers / Work Plan / TC lists	228

5.3.32	High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 (UID-1000059) LTE_NR_HPUE_FWVM_R18-UEConTest	228

5.3.32.1	TS 38.508-2	228

5.3.32.2	TS 38.521-1	228

5.3.32.2.1	Tx Requirements (Clause 6)	228

5.3.32.2.2	Rx Requirements (Clause 7)	229

5.3.32.2.3	Clauses 1-5 / Annexes	229

5.3.32.3	TS 38.522	229

5.3.32.4	TS 36.521-1	229

5.3.32.5	TS 36.521-2	229

5.3.32.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	229

5.3.32.7	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )	229

5.3.32.8	Discussion Papers / Work Plan / TC lists	229

5.3.33	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest	229

5.3.33.1	TS 36.508	229

5.3.33.2	TS 36.521-1	229

5.3.33.3	TS 36.521-2	230

5.3.33.4	TS 36.521-3	231

5.3.33.5	TS 37.571-1	231

5.3.33.6	TS 37.571-3	231

5.3.33.7	TS 37.571-5	231

5.3.33.8	TR 36.903 (E-UTRAN RRM TT analyses)	231

5.3.33.9	TR 36.904 (E-UTRAN Radio Reception TT analyses)	231

5.3.33.10	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )	231

5.3.33.11	Discussion Papers / Work Plan / TC lists	231

5.3.34	Air-to-ground network for NR (UID-1020087) NR_ATG-UEConTest	231

5.3.34.1	TS 38.508-1	231

5.3.34.1.1	Test frequencies (Clause 4.3.1)	231

5.3.34.1.2	Test environment for RF (Clauses 5)	231

5.3.34.1.3	Test environment for RRM (Clause 7)	232

5.3.34.1.4	Other clauses / Annexes	232

5.3.34.2	TS 38.508-2	232

5.3.34.3	TS 38.521-1	233

5.3.34.3.1	Tx Requirements (Clause 6)	233

5.3.34.3.2	Rx Requirements (Clause 7)	236

5.3.34.3.3	Clauses 1-5 / Annexes	240

5.3.34.4	TS 38.521-4	241

5.3.34.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	241

5.3.34.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	241

5.3.34.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	241

5.3.34.4.4	Clauses 1-4 / Annexes	241

5.3.34.5	TS 38.522	241

5.3.34.6	TS 38.533	241

5.3.34.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	241

5.3.34.8	Discussion Papers / Work Plan / TC lists	242

5.3.35	4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC (UID-1020088) 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest	242

5.3.35.1	TS 38.508-1	242

5.3.35.1.1	Test frequencies (Clause 4.3.1)	242

5.3.35.1.2	Test environment for RF (Clauses 5)	242

5.3.35.1.3	Test environment for RRM (Clause 7)	242

5.3.35.1.4	Other clauses / Annexes	242

5.3.35.2	TS 38.508-2	242

5.3.35.3	TS 38.521-1	242

5.3.35.3.1	Tx Requirements (Clause 6)	242

5.3.35.3.2	Rx Requirements (Clause 7)	243

5.3.35.3.3	Clauses 1-5 / Annexes	243

5.3.35.4	TS 38.521-3	243

5.3.35.4.1	Tx Requirements (Clause 6)	243

5.3.35.4.2	Rx Requirements (Clause 7)	243

5.3.35.5	TS 38.522	243

5.3.35.5.3	Clauses 1-5 / Annexes	243

5.3.35.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	243

5.3.35.7	Discussion Papers / Work Plan / TC lists	243

5.3.36	Enhanced Test Methods for FR2 NR UEs FS_FR2_enhTestMethods (RAN4 Study Item)	244

5.3.36.1	Discussion Papers / Work Plan to track adoption of the TR 38.884 outcomes into RAN5 test specifications	244

5.4.1	TS 38.508-1	244

5.4.1.1	Test frequencies (Clause 4.3.1)	244

5.4.1.2	Test environment for RF (Clauses 5)	247

5.4.1.3	Test environment for RRM (Clause 7)	248

5.4.1.4	Other clauses / Annexes	249

5.4.2	TS 38.508-2	250

5.4.3	TS 38.509	251

5.4.4	TS 38.521-1	251

5.4.4.1	Tx Requirements (Clause 6)	251

5.4.4.2	Rx Requirements (Clause 7)	257

5.4.4.3	Clauses 1-5 / Annexes	261

5.4.5	TS 38.521-2	262

5.4.5.1	Tx Requirements (Clause 6)	262

5.4.5.2	Rx Requirements (Clause 7)	264

5.4.5.3	Clauses 1-5 / Annexes	265

5.4.6	TS 38.521-3	266

5.4.6.1	Tx Requirements (Clause 6)	266

5.4.6.2	Rx Requirements (Clause 7)	269

5.4.6.3	Clauses 1-5 / Annexes	271

5.4.7	TS 38.521-4	272

5.4.7.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	272

5.4.7.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	273

5.4.7.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	274

5.4.7.4	Clauses 1-4 / Annexes	274

5.4.8	TS 38.521-5	276

5.4.9	TS 38.522	276

5.4.10	TS 38.533	278

5.4.10.1	EN-DC with all NR cells in FR1 (Clause 4)	278

5.4.10.2	NE-DC with all NR cells in FR1 (Clause 4A)	281

5.4.10.3	EN-DC with at least 1 NR Cell in FR2 (Clause5)	281

5.4.10.4	NR Standalone in FR1 (Clause 6)	284

5.4.10.5	NR standalone with at least one NR cell in FR2 (Clause7)	286

5.4.10.6	E-UTRA – NR Inter-RAT with E-UTRA serving cell (Clause 8)	286

5.4.10.7	Clauses 1-3 / Annexes	286

5.4.11	TS 38.551	289

5.4.12	TS 38.561	292

5.4.13	TS 36.508	294

5.4.14	TS 36.521-3	294

5.4.15	TS 37.571-1	294

5.4.16	TS 37.571-3	294

5.4.17	TS 37.571-5	294

5.4.18	TR 38.903 (NR MU & TT analyses)	294

5.4.19	TR 38.905 (NR Test Points Radio Transmission and Reception )	296

5.4.20	Discussion Papers / Work Plan / TC lists	298

5.5	Routine Maintenance for LTE only TEIx_Test	307

5.5.1	LTE RF	307

5.5.1.1	TS 36.508	307

5.5.1.2	TS 36.509	307

5.5.1.3	TS 36.521-1	307

5.5.1.3.1	Tx Requirements (Clause 6)	307

5.5.1.3.2	Rx Requirements (Clause 7)	307

5.5.1.3.3	Clauses 1-5 / 8-10 / Annexes	310

5.5.1.4	TS 36.521-2	310

5.5.1.5	TS 36.521-3	311

5.5.1.6	RRM Test & Radio Reception Test Tolerances	311

5.5.1.6.1	TR 36.903 (E-UTRAN RRM TT analyses)	311

5.5.1.6.2	TR 36.904 (E-UTRAN Radio Reception TT analyses)	311

5.5.1.6.3	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )	311

5.5.1.7	TS 34.121-1	311

5.5.1.8	TS 34.121-2	311

5.5.1.9	TS 34.122	311

5.5.1.10	TS 34.108	311

5.5.1.11	TR 34.902 (UTRAN RRM Test Tolerance analyses)	311

5.5.1.12	Discussion Papers / Work Plan / TC lists	311

5.6	Other Routine Maintenance TEIx_Test	311

5.6.1	TS 34.108	311

5.6.2	TS 34.121-1 All sections other than annexes	311

5.6.3	TS 34.121-1 Annexes only	311

5.6.4	TS 34.121-2	311

5.6.5	TS 34.122	311

5.6.6	TS 34.171	311

5.6.7	TS 34.172	311

5.6.8	TS 34.114	311

5.6.9	TS 37.571-1	311

5.6.10	TS 37.571-3	311

5.6.11	TS 37.571-5	311

5.6.12	TS 51.010-1 (RF/Performance)	311

5.6.13	TS 51.010-2 (RF/Performance)	312

5.6.14	TS 51.010-7 (RF/Performance)	312

5.6.15	TS 37.544	312

5.6.16	TR 37.901	312

5.6.17	TR 37.901-5	312

5.6.18	TR 38.918	313

5.6.19	Discussion Papers / Work Plan / TC lists	313

5.7	Outgoing liaison statements for provisional approval	313

5.8	AOB	315

6	Signalling Protocol Functional Area	315

6.1	Review action points (fm A.I. 2.1)	315

6.2	Review incoming LS (fm A.I. 3) & new subject discussion papers	315

6.3	Open Work Items	316

6.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083)  NR_CADC_NR_LTE_DC_R16-UEConTest	316

6.3.1.1	TS 38.508-1	316

6.3.1.2	TS 38.508-2	317

6.3.1.3	TS 38.523-1	317

6.3.1.4	TS 38.523-2	317

6.3.1.5	TS 38.523-3	317

6.3.1.6	Discussion Papers / Work Plan / TC lists	317

6.3.2	5G V2X with NR sidelink (UID-880069)  5G_V2X_NRSL_eV2XARC-UEConTest	317

6.3.2.1	TS 38.508-1	317

6.3.2.2	TS 38.508-2	318

6.3.2.3	TS 38.509	318

6.3.2.4	TS 38.523-1	318

6.3.2.5	TS 38.523-2	318

6.3.2.6	TS 38.523-3	319

6.3.2.7	TS 36.509	320

6.3.2.8	TS 37.571-4	320

6.3.2.9	Discussion Papers / Work Plan / TC lists	320

6.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest	320

6.3.3.1	TS 38.508-1	320

6.3.3.2	TS 38.508-2	320

6.3.3.3	TS 38.523-1	320

6.3.3.4	TS 38.523-2	320

6.3.3.5	TS 38.523-3	320

6.3.3.6	Discussion Papers / Work Plan / TC lists	320

6.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest	320

6.3.4.1	TS 38.508-1	320

6.3.4.2	TS 38.508-2	320

6.3.4.3	TS 38.509	320

6.3.4.4	TS 38.523-1	320

6.3.4.5	TS 38.523-2	321

6.3.4.6	TS 38.523-3	321

6.3.4.7	Discussion Papers / Work Plan / TC lists	322

6.3.5.	NR coverage enhancements (UID-950063) NR_cov_enh-UEConTest	322

6.3.5.1	TS 38.508-1	322

6.3.5.2	TS 38.508-2	322

6.3.5.3	TS 38.523-1	322

6.3.5.4	TS 38.523-2	322

6.3.5.5	TS 38.523-3	322

6.3.5.6	Discussion Papers / Work Plan / TC lists	322

6.3.6	Support of reduced capability NR devices (UID-950066) NR_redcap_plus_ARCH-UEConTest	322

6.3.6.1	TS 38.508-1	322

6.3.6.2	TS 38.508-2	322

6.3.6.3	TS 38.523-1	322

6.3.6.4	TS 38.523-2	324

6.3.6.6	TS 34.229-2	324

6.3.6.7	Discussion Papers / Work Plan / TC lists	324

6.3.7	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest	327

6.3.7.1	TS 38.508-1	327

6.3.7.2	TS 38.508-2	327

6.3.7.3	TS 38.523-1	327

6.3.7.4	TS 38.523-2	329

6.3.7.5	TS 38.523-3	329

6.3.7.6	Discussion Papers / Work Plan / TC lists	329

6.3.8	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest	329

6.3.8.1	TS 38.508-1	329

6.3.8.2	TS 38.508-2	330

6.3.8.3	TS 38.509	331

6.3.8.4	TS 38.523-1	331

6.3.8.5	TS 38.523-2	334

6.3.8.6	TS 38.523-3	336

6.3.8.7	Discussion Papers / Work Plan / TC lists	336

6.3.9	Enhancement of Private Network Support for NG-RAN including CT aspects (UID-960076) NG_RAN_PRN_enh_plus_CT-UEConTest	336

6.3.9.1	TS 38.508-1	336

6.3.9.2	TS 38.508-2	336

6.3.9.3	TS 38.523-1	337

6.3.9.4	TS 38.523-2	340

6.3.9.5	TS 38.523-3	342

6.3.9.6	Discussion Papers / Work Plan / TC lists	342

6.3.10	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest	342

6.3.10.1	TS 38.508-1	342

6.3.10.2	TS 38.508-2	342

6.3.10.3	TS 38.523-1	342

6.3.10.4	TS 38.523-2	342

6.3.10.5	TS 38.523-3	342

6.3.10.6	Discussion Papers / Work Plan / TC lists	342

6.3.11	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest	342

6.3.11.1	TS 38.508-1	342

6.3.11.2	TS 38.508-2	344

6.3.11.3	TS 38.523-1	345

6.3.11.4	TS 38.523-2	345

6.3.11.5	TS 38.523-3	345

6.3.11.6	Discussion Papers / Work Plan / TC lists	345

6.3.12	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest	345

6.3.12.1	TS 38.508-1	345

6.3.12.2	TS 38.508-2	345

6.3.12.3	TS 38.523-1	345

6.3.12.4	TS 38.523-2	345

6.3.12.5	TS 38.523-3	345

6.3.12.6	Discussion Papers / Work Plan / TC lists	345

6.3.13	UE power saving enhancements for NR (UID-960086) NR_UE_pow_sav_enh_plus_CT-UEConTest	345

6.3.13.1	TS 38.508-1	345

6.3.13.2	TS 38.508-2	345

6.3.13.3	TS 38.523-1	346

6.3.13.4	TS 38.523-2	347

6.3.13.5	TS 38.523-3	347

6.3.13.6	Discussion Papers / Work Plan / TC lists	347

6.3.14	NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects (UID-960087) LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest	347

6.3.14.1	TS 36.508	347

6.3.14.2	TS 36.523-1	349

6.3.14.3	TS 36.523-2	351

6.3.14.4	TS 36.523-3	352

6.3.14.5	Discussion Papers / Work Plan / TC lists	352

6.3.15	NR QoE management and optimizations for diverse services (UID-970072) NR_QoE-UEConTest	354

6.3.15.1	TS 38.508-1	354

6.3.15.2	TS 38.508-2	354

6.3.15.3	TS 38.523-1	354

6.3.15.4	TS 38.523-2	354

6.3.15.5	TS 38.523-3	354

6.3.15.6	Discussion Papers / Work Plan / TC lists	354

6.3.16	User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects) (UID-970074) UPIP_SEC_LTE-RAN-UEConTest	354

6.3.16.1	TS 38.508-1	354

6.3.16.2	TS 38.508-2	354

6.3.16.3	TS 38.523-1	354

6.3.16.4	TS 38.523-2	356

6.3.16.5	TS 38.523-3	357

6.3.16.6	Discussion Papers / Work Plan / TC lists	357

6.3.17	NR Positioning Enhancements (UID-970075) NR_pos_enh-UEConTest	357

6.3.17.1	TS 38.508-1	357

6.3.17.2	TS 37.571-2	357

6.3.17.3	TS 37.571-3	357

6.3.17.4	TS 37.571-4	357

6.3.17.5	TS 37.571-5	357

6.3.17.6	Discussion Papers / Work Plan / TC lists	357

6.3.18	Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT / MCVideo / MCData) (UID – 970077) MCProtoc16_enh2MCPTT_eMCData2-ConTest	357

6.3.18.1	TS 36.579-1	357

6.3.18.2	TS 36.579-2	358

6.3.18.3	TS 36.579-3	358

6.3.18.4	TS 36.579-4	358

6.3.18.5	TS 36.579-5	359

6.3.18.6	TS 36.579-6	359

6.3.18.7	TS 36.579-7	361

6.3.18.8	TS 36.579-8 (pCRs only)	362

6.3.18.9	TS 36.579-9 (pCRs only)	362

6.3.18.10	Discussion Papers / Work Plan / TC lists	363

6.3.19	NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) (UID-981034) LTE_NBIOT_eMTC_NTN_req-UEConTest	363

6.3.19.1	TS 36.508	363

6.3.19.2	TS 36.509	363

6.3.19.3	TS 36.523-2	363

6.3.19.4	TS 36.523-3	363

6.3.19.5	Discussion Papers / Work Plan / TC lists	363

6.3.20	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest	363

6.3.20.1	TS 38.508-1	363

6.3.20.2	TS 38.508-2	363

6.3.20.3	TS 38.523-1	363

6.3.20.4	TS 38.523-2	364

6.3.20.5	TS 38.523-3	365

6.3.20.6	TS 36.508	365

6.3.20.7	Discussion Papers / Work Plan / TC lists	365

6.3.21	Support of Uncrewed Aerial Systems Connectivity / Identification / and Tracking (UID-991034) ID_UAS-UEConTest	365

6.3.21.1	TS 38.508-1	365

6.3.21.2	TS 38.508-2	365

6.3.21.3	TS 38.523-1	365

6.3.21.4	TS 38.523-2	366

6.3.21.5	TS 38.523-3	366

6.3.21.6	TS 36.508	366

6.3.21.7	TS 36.523-1	366

6.3.21.8	TS 36.523-2	368

6.3.21.9	TS 36.523-3	368

6.3.21.10	Discussion Papers / Work Plan / TC lists	368

6.3.22	IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS (UID-1000052) ING_5GS-UEConTest	368

6.3.22.1	TS 38.508-1	368

6.3.22.2	TS 38.508-2	368

6.3.22.3	TS 38.523-1	368

6.3.22.4	TS 38.523-2	369

6.3.22.5	TS 38.523-3	371

6.3.22.6	Discussion Papers / Work Plan / TC lists	371

6.3.23	IMS Data Channel (UID-1000055) IMSProtoc17_dataCH-UEContest	371

6.3.23.1	TS 36.508	371

6.3.23.2	TS 36.509	371

6.3.23.3	TS 38.508-1	371

6.3.23.4	TS 38.509	371

6.3.23.5	TS 34.229-1	371

6.3.23.6	TS 34.229-2	371

6.3.23.7	TS 34.229-3	371

6.3.23.8	TS 34.229-5	371

6.3.23.9	Discussion Papers / Work Plan / TC lists	371

6.3.24	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest	371

6.3.24.1	TS 38.508-1	371

6.3.24.2	TS 38.508-2	371

6.3.24.3	TS 38.523-1	371

6.3.24.4	TS 38.523-2	371

6.3.24.5	TS 38.523-3	371

6.3.24.6	Discussion Papers / Work Plan / TC lists	371

6.3.25	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest	371

6.3.25.1	TS 38.523-3	371

6.3.25.2	Discussion Papers / Work Plan / TC lists	371

6.3.26	Access Traffic Steering / Switch and Splitting support in the 5G system architecture; Phase 2 (UID-1010050) ATSSS_Ph2-UEConTest	371

6.3.26.1	TS 38.508-1	371

6.3.26.2	TS 38.508-2	372

6.3.26.3	TS 38.523-1	372

6.3.26.4	TS 38.523-2	374

6.3.26.5	TS 38.523-3	375

6.3.26.6	Discussion Papers / Work Plan / TC lists	375

6.3.27	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest	375

6.3.27.1	TS 36.508	375

6.3.27.2	TS 36.523-1	375

6.3.27.3	TS 36.523-2	375

6.3.27.4	TS 36.523-3	376

6.3.27.5	Discussion Papers / Work Plan / TC lists	376

6.3.28	Signal level Enhanced Network Selection (UID-1010052) SENSE-UEContest	376

6.3.28.1	TS 36.508	376

6.3.28.2	TS 36.523-1	376

6.3.28.3	TS 36.523-2	378

6.3.28.4	TS 36.523-3	379

6.3.28.5	Discussion Papers / Work Plan / TC lists	379

6.4	Routine Maintenance for TS 38 Series TEIx_Test	379

6.4.1	TS 38.508-1	379

6.4.1.1	Generic Procedures and Test Procedures (Clauses 4.5 / 4.5A & 4.9)	379

6.4.1.2	Default NG-RAN RRC messages and IEs (Clause 4.6)	380

6.4.1.3	Default 5GC NAS messages and IEs (Clause 4.7)	387

6.4.1.4	Test environment for SIG (Clause 6)	387

6.4.1.5	Other clauses / Annexes	388

6.4.2	TS 38.508-2	389

6.4.3	TS 38.509	390

6.4.4	TS 38.523-1	391

6.4.4.1	Clauses 1 - 5	391

6.4.4.2	Idle Mode (Clause 6)	391

6.4.4.3	Layer 2	395

6.4.4.3.1	NR Layer 2	395

6.4.4.3.1.1	Common Test Case Specific Values for Layer 2 (Clause 7.1.0)	395

6.4.4.3.1.2	MAC	395

6.4.4.3.1.3	RLC	399

6.4.4.3.1.4	PDCP	399

6.4.4.3.1.5	SDAP	399

6.4.4.4	RRC	399

6.4.4.4.1	NR RRC	399

6.4.4.4.1.1	RRC Connection Management Procedures (clause 8.1.1)	399

6.4.4.4.1.2	RRC Reconfiguration (clause 8.1.2)	400

6.4.4.4.1.3	RRC Measurement Configuration Control and Reporting (clause 8.1.3)	401

6.4.4.4.1.4	RRC Handover (clause 8.1.4)	401

6.4.4.4.1.5	RRC Others (clause 8.1.5)	402

6.4.4.4.1.6	RRC SON and MDT support for NR (clause 8.1.6)	404

6.4.4.4.1.7	RRC Non-public networks (clause 8.1.7)	406

6.4.4.4.2	MR-DC RRC	406

6.4.4.4.2.1	RRC UE Capability / Others (clause 8.2.1)	406

6.4.4.4.2.2	RRC Radio Bearer (clause 8.2.2)	407

6.4.4.4.2.3	RRC Measurement / Handovers (clause 8.2.3)	407

6.4.4.4.2.4	RRC Carrier Aggregation (clause 8.2.4)	407

6.4.4.4.2.5	RRC Reconfiguration / Radio Link Failure (clause 8.2.5)	408

6.4.4.4.2.6	RRC Others (clause 8.2.6)	408

6.4.4.4.2.7	RRC Resume (clause 8.2.7)	408

6.4.4.5	5GS Mobility Management	408

6.4.4.5.1	MM Primary authentication and key agreement (clause 9.1.1)	408

6.4.4.5.2	MM Security mode control / Identification & Generic UE configuration update (clauses 9.1.2 / 9.1.3 & 9.1.4)	408

6.4.4.5.3	MM Registration & De-registration (clauses 9.1.5 & 9.1.6)	409

6.4.4.5.4	MM Service Request (clause 9.1.7)	409

6.4.4.5.5	MM SMS Over NAS (clause 9.1.8)	409

6.4.4.5.6	RACS (clause 9.1.9)	409

6.4.4.5.7	MM Network slice-specific authentication and authorization (clause 9.1.10)	410

6.4.4.5.8	MM SNPN(clause 9.1.11)	410

6.4.4.5.9	MM NSAC/NSSRG(clauses 9.1.12 & 9.1.13)	410

6.4.4.6	5GS Non-3GPP Access Mobility Management (clause 9.2)	410

6.4.4.7	5GS Inter-system Mobility (clause 9.3)	410

6.4.4.8	5GS Session Management	410

6.4.4.8.1	SM PDU session authentication and authorization (clause 10.1.1)	410

6.4.4.8.2	SM Network-requested PDU session modification & release (clauses 10.1.2 & 10.1.3)	410

6.4.4.8.3	SM UE-requested PDU session establishment / modification & release (clauses 10.1.4 / 10.1.5 & 10.1.6)	410

6.4.4.8.4	SM NSAC (clauses 10.1.8)	410

6.4.4.9	EN-DC Session Management (clause 10.2)	411

6.4.4.10	5GS Non-3GPP Access & ATSS Session Management (clauses 10.3 & 10.4)	411

6.4.4.11	5GS Multilayer and Services	411

6.4.4.11.1	EPS Fallback (clause 11.1)	411

6.4.4.11.2	5G-SRVCC (clause 11.2)	412

6.4.4.11.3	Unified Access Control (UAC) (clause 11.3)	412

6.4.4.11.4	Emergency Services (clause 11.4)	413

6.4.4.11.5	eCall over IMS (clause 11.5)	415

6.4.4.11.6	3GPP PS Data Off (clause 11.6)	415

6.4.4.11.7	Inter-system mobility between untrusted Non-3GPP and 3GPP system (clause 11.8)	415

6.4.4.11.8	MBS (clause 14)	416

6.4.5	TS 38.523-2	419

6.4.6	TS 38.523-3	420

6.4.7	Discussion Papers / Work Plan / TC lists	420

6.5	Routine Maintenance for TS 36 Series TEIx_Test	423

6.5.1	Routine Maintenance for TS 36.508	423

6.5.2	Routine Maintenance for TS 36.509	423

6.5.3	Routine Maintenance for TS 36.523-1	423

6.5.3.1	Idle Mode	423

6.5.3.2	Layer 2	423

6.5.3.2.1	MAC	423

6.5.3.2.2	RLC	423

6.5.3.2.3	PDCP	423

6.5.3.3	RRC	423

6.5.3.3.1	RRC Part 1 (clauses 8.1 and 8.5)	423

6.5.3.3.2	RRC Part 2 (clause 8.2) /	425

6.5.3.3.3	RRC Part 3 (clause 8.3)	425

6.5.3.3.4	Inter-RAT (clauses 8.4 & 8.4A)	425

6.5.3.3.5	RRC LTE MDT (clause 8.6)	425

6.5.3.3.6	RRC ANR for UTRAN (clause 8.7)	425

6.5.3.4	EPS Mobility Management	425

6.5.3.5	EPS Session Management	426

6.5.3.6	General Tests	426

6.5.3.7	Interoperability Radio Bearers	427

6.5.3.8	Multilayer Procedures	427

6.5.3.9	PWS - ETWS / CMAS	427

6.5.3.10	Non-3GPP	427

6.5.3.11	Others (TS 36.523-1 clauses not covered by other AIs under AI 6.5.3 / e.g. eMBMS / Home (e)NB / MBMS in LTE / D2D / SC-PTM / NB-IoT / CIoT...)	427

6.5.4	Routine Maintenance for TS 36.523-2	428

6.5.5	Routine Maintenance for TS 36.523-3	429

6.5.6	Discussion Papers / Work Plan / TC lists	429

6.6	Other Maintenance TEIx_Test	431

6.6.1	Routine Maintenance for TS 34.108	431

6.6.2	Routine Maintenance for TS 34.109	431

6.6.3	Routine Maintenance for TS 34.123	431

6.6.3.1	TS 34.123-1	431

6.6.3.2	TS 34.123-2	431

6.6.3.3	TS 34.123-3	431

6.6.4	Discussion Papers / Work Plan / TC lists	432

6.6.6	Routine Maintenance for TS 34.229	432

6.6.6.1	TS 34.229-1	432

6.6.6.2	TS 34.229-2	432

6.6.6.3	TS 34.229-3	432

6.6.6.4	TS 34.229-4	432

6.6.6.5	TS 34.229-5	432

6.6.6.6	Discussion Papers / Work Plan / TC lists	435

6.6.7	Routine Maintenance for TS 37.571	436

6.6.7.1	TS 37.571-2	436

6.6.7.2	TS 37.571-3	437

6.6.7.3	TS 37.571-4	437

6.6.7.4	TS 37.571-5	437

6.6.7.5	Discussion Papers / Work Plan / TC lists	437

6.6.8	Routine Maintenance for TS 51.010	437

6.6.8.1	TS 51.010-1 (Signalling)	437

6.6.8.2	TS 51.010-2 (Signalling)	438

6.6.8.3	TS 51.010-5 (Signalling)	438

6.6.8.4	TS 51.010-7 (Signalling)	439

6.6.8.5	Discussion Papers / Work Plan / TC list & CR summary	439

6.6.9	Routine Maintenance for TS 36.579	439

6.6.9.1	TS 36.579-1	439

6.6.9.2	TS 36.579-2	441

6.6.9.3	TS 36.579-3	444

6.6.9.4	TS 36.579-4	444

6.6.9.5	TS 36.579-5	444

6.6.9.6	TS 36.579-6	444

6.6.9.7	TS 36.579-7	445

6.6.9.8	Other Specs	445

6.6.9.9	Discussion Papers / Work Plan / TC lists	445

6.7	Outgoing liaison statements for provisional approval	445

6.8	AOB	445

7	Closing Joint Session	445

7.1	Agenda for closing session	445

7.2	Pointer CRs	446

7.3	Open Issues	446

7.3.1	RF group docs still requiring WG verdict/confirmation - original A.I. retained	446

7.3.2	Sig group docs still requiring WG verdict/confirmation - original A.I. retained	446

7.3.3	Other open issues from joint sessions - original A.I. retained	446

7.3.4	Other	446

7.4	iWD/PRD Updates	446

7.4.1	iWD-003: Record of RAN5 owned test cases not ready for RAN5 agreement or verifiable on one UE only	446

7.4.2	PRD17: Guidance to using Work Item Codes with RAN5 test cases	446

7.4.3	PRD20: Status updates E-UTRA CA	446

7.4.4	PRD21: Status Updates and Completion Declaration Statements (CDS) for NR bands / NR band CBW extensions / 5G NR CADC configurations for PC3 / PC1.5 and PC2	447

7.4.5	Other PRD updates	450

7.5	Work Items/ Study Items	450

7.5.1	Final version of Work Item Proposals	450

7.5.2	Active Work Items/ Study Item: work plans (wp) / status reports (sr) / Work Item Descriptions (wid)	454

7.5.3	Work Plan updates of recently closed work items	468

7.6	Docs still needing agreement/endorsement/approval (e.g. Outgoing LS / Reports / New Specs / Info for certification bodies etc.)	469

7.7	Confirmation of Future RAN5 Matters	471

7.8	AOB	472

Annex A: Contribution documents and status	473

A1: List of TDocs	473

A2: Tdoc decision timing	529

Annex B: List of change requests	530

Annex C: Lists of liaisons	587

C1: Incoming liaison statements	587

C2: Outgoing liaison statements	587

Annex D: List of agreed/approved new and revised Work Items	588

Annex E: List of draft Technical Specifications and Reports	589

Annex F: List of action items	590

Annex G: List of decisions	591

Annex H: List of participants	592

Annex I: List of future meetings	593

## 1	Opening of the meeting

### 1.1	Welcoming brief by the host

R5-240001	Agenda - opening session

Type: agenda		For: Information
					Source: WG Chairman

Discussion:

The RAN5 Chair Mr. Jacob John welcomed all delegates to the RAN5#102 meeting in Athens, Greece.

Then the RAN5 Chair reminded:

Reminder for IPR declaration

I draw your attention to your obligations under the 3GPP Partner Organizations’ IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

Delegates are asked to take note that they are thereby invited:

•	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

•	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

Antitrust Guidance

“I also draw your attention to the fact that 3GPP activities are subject to all applicable antitrust and competition laws and that compliance with said laws is therefore required of any participant of this TSG/WG meeting including the Chairman and Vice Chairman. In case of question I recommend that you contact your legal counsel.

The leadership shall conduct the present meeting with impartiality and in the interests of 3GPP.

Furthermore, I would like to remind you that timely submission of work items in advance of TSG/WG meetings is important to allow for full and fair consideration of such matters.”

http://www.3gpp.org/about-3gpp/legal-matters/21-3gpp-calendar/1616-statement-of-antitrust-compliance

Then Vodafone explained on behalf of the host the practicalities.

Then the new delgates from FirstNet, China Telecom, .. introduced themselves or were introduced.

Decision: 		The document was approved.

R5-240004	RAN5#102 Session Programme

Type: agenda		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

## 2	Reports

### 2.1	Live Reports

R5-240005	RAN5 Leadership Team

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-240006	RAN5#101 WG Minutes

Type: report		For: Approval
					Source: ETSI Secretariat

Decision: 		The document was approved.

R5-240007	RAN5#101 WG Action Points

Type: report		For: Information
					Source: ETSI Secretariat

Decision: 		The document was noted.

R5-240008	Latest RAN Plenary notes

Type: report		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-240009	Latest RAN Plenary draft Report

Type: report		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-240010	Post Plenary Active Work Item update

Type: other		For: Information
					Source: ETSI Secretariat

Decision: 		The document was noted.

R5-240526	MCC TF160 Status Report

Type: report		For: Approval
					Source: MCC TF160

Abstract:

Test models & system interface design
Progress at TTCN Workshop #64 (8th Feb’24):

Test Models – 5G:

Rel-15:

Endorsed updating the sequence of NR-DC inter-cell PSCell change.

Rel-16:

eMIMO: Endorsed ASP updates to handle up to two CoresetPoolIndex.

NR-U: Endorsed initial Test Models & ASP updates to support shared spectrum access.

Rel-17:

NR coverage enhancements: Endorsed ASP updates to add support of RA Msg3 repetitions & retransmissions.

MUSIM: endorsed new MMI command for MUSIM UE Assistance Information.

Prose CRs to TS 38.523-3 submitted at RAN5#102 for the above.

TTCN development 
Progress for period: Dec’23 to Feb’24:

Completed:

5G Rel-16:

MR-DC & NR CA enhancements

5G Rel-17:

NR IIoT & URLLC enhancements

NR RAN slicing enhancements

NR UE power saving enhancements

Progressed 5G:

5G Rel-16:

Redirection with MPS indication

5G Rel-17:

RedCap

NR Multi-SIM (MUSIM)

Non-public networks (NPN) enhancements

SON/MDT enhancements

EN-DC user-plane integrity protection (UPIP)

Started:

5G Rel-17:

IMS voice & NW usability no E-UTRA in 5GS

Progressed 4G:

NB-IoT:

Rel-17 non-terrestrial networks (NTN) GSO

Mission Critical:

Rel-16 MCPTT / MCVideo / MCData over LTE

TTCN funding 2024

Status:

2024 workload is estimated at 101 person-months (pm), see previous slides.

PCG#51/OP#50 approved the 3GPP funding of 58 pm for 2024 TTCN tasks.

CTIA/PTCRB and GCF have agreed to continue TF160 financial support in 2024.

3GPP companies / 3GPP MRPs committed to provide 19 pm as voluntary contributions for 2024 TTCN development.

Total resources of 97 pm  estimated funding gap of 4 pm.

TTCN deliveries and baseline
2024 schedule:

One TTCN-2 full delivery (FDD) and five TTCN-3 full deliveries.

Type definitions baseline upgrade to 3GPP Release 18 required in 2024:

Rel-18 Stage3 freeze planned by 3GPP in March 2024.

Rel-18 ASN.1 freeze planned by 3GPP in June 2024.

Decision: 		The document was revised to R5-241458.

### 2.2	General Reports for information

R5-240011	RAN5 SR to RP#102

Type: report		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-240012	TF160 SR to RP#102

Type: report		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-241182	GCF 3GPP TCL after GCF CAG#77

Type: report		For: Information
					Source: Ericsson

Decision: 		The document was noted.

## 3	Incoming Liaison Statements

R5-240017	Reply LS on frequencyInfo for NR SL RSRP measurements

Type: LS in		For: Information
					Original outgoing LS: R2-2313618, to TSG RAN WG5, cc TSG RAN WG1, TSG RAN WG4
					Source: TSG WG RAN2

Abstract:

RAN2 thanks RAN5 for the LS on frequencyInfo for NR SL RSRP measurements. RAN2 understands that, without field description, the exact meaning of frequencyInfoSL-r16 in IE SL-MeasObject-r16 can be unclear, and this ambiguity can lead to unpredictable UE behaviour during testing.

By further checking with RAN1, the field frequencyInfoSL-r16 should be the indication of the carrier on which SL-RSRP measurement is performed.

RAN2 agrees to add, in TS 38.331, the field description for frequencyInfoSL in IE SL-MeasObjectList as " It indicates the lowest usable subcarrier on the carrier where SL RSRP is measured, determined according to sl-AbsoluteFrequencyPointA in IE SL-FreqConfig/SL-FreqConfigCommon and offsetToCarrier in IE SCS-SpecificCarrier configured for sl-SCS-SpecificCarrierList in IE SL-FreqConfig/SL-FreqConfigCommon. See TS 38.211 [16], clause 8.2.5."

Actions: To RAN5: RAN2 asks RAN5 to take the above information into account in related work.

Discussion:

associated CR R5-240764

Decision: 		The document was noted.

R5-240019	draft RAN5 meeeting report

Type: report		For: Information
					Source: ETSI Secretariat

Decision: 		The document was noted.

## 4	RAN5 General Issues

### 4.1	New Work Item proposals - for intro only

R5-240094	New WID on UE Conformance - IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects

Type: WID new		For: Endorsement
					Source: CMCC, MTK, CAICT

Abstract:

Release 17 RAN1/2/3/4 work item LTE_NBIOT_eMTC_NTN was carried out to enable NB-IoT and eMTC to support Non-Terrestrial Networks (NTN) under the following assumptions:

•	Transparent payload based GEO and NGSO network scenarios addressing at least 3GPP power class 3 UE with GNSS capability in both Earth fixed &/or moving cell configurations.

Release 18 RAN1/2/3/4 work item IoT_NTN_enh defines further enhancements for NB-IoT NTN and eMTC NTN in order to:

•	Improve mobility aspects:

o	Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel 17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

o	Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2]

o	Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

o	Define UE RRM core requirements for the above mobility enhancement features [RAN4].

•	Improve performance in terms of throughput:

o	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

o	Specify UE RRM performance requirements to support the agreed mobility enhancements for NB-IoT/eMTC [RAN4]

o	Specify UE and Base Station demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC [RAN4]

•	Optimize the GNSS operation with sparse use of GNSS and power efficiency for long-term connection (compared to Rel-17):

o	Specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

•	Further enhance support for discontinuous coverage:

o	Specify mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

NOTE:	It is expected a UE can move while under discontinuous coverage.

The Release18 work item carried out by RAN1/2/3/4 aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks) according to the following assumptions:

-	GEO and NGSO (LEO and MEO).

-	Earth fixed Tracking area. Earth fixed & Earth moving cells for NGSO

-	FDD mode

-	UEs with GNSS capabilities

Release 18 CT1 work item 5GSAT_Ph2 defines the following contents:

1.	Procedures for determining and negotiating out-of-coverage period due to discontinuous coverage and for power saving during coverage gaps:

-	Negotiation of “discontinuous coverage support” capability between the UE and the AMF

-	Negotiation of “out-of-coverage period” between the UE and the AMF

-	Indication of losing coverage by the UE

-	AMF requesting the UE to perform Registration update upon return to coverage

2.	Handling signalling overload due to loss of coverage and return to coverage of many UEs at the same time.

For Rel-18 RAN1/2/3/4 work item IoT_NTN_enh, the overall completion level for core part has reached 100% after RP#102 (RAN1: 100%, RAN2: 100%, RAN3: 100%, RAN4: 100%). The overall completion level for performance part has reached 10% after RP#102, and the target completion date for performance part is RP#104 (2024-06).

For Rel-18 CT1 work item 5GSAT_Ph2, the overall completion level has reached 90% after CT#102, and is expected to be completed by CT#103 (2024-03).

Therefore it is justified to introduce the conformance testing for IoT NTN enhancement UE requirements into RAN5 specifications.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this RAN5 work item is to define the corresponding IoT NTN enhancement UE conformance requirements, the test cases for IoT NTN enhancement UE, applicability, test environment, test points and update the relevant conformance specifications for the R18 IoT NTN enhancement UE requirements introduced by the RAN1, RAN2 and RAN4 work item IoT_NTN_enh.

Due to lack of real commercial deployment interest of eMTC NTN, conformance test development of eMTC enhancement is considered to be outside of the scope of this WI.

Decision: 		The document was revised to R5-241459.

R5-240142	New WID on UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

Type: WID new		For: Agreement
					Source: China Telecom

Discussion:

replaced by R5-240278.

Decision: 		The document was withdrawn.

R5-240146	New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation

Type: WID new		For: Endorsement
					Source: MediaTek Inc., China Telecom

Abstract:

To address various IoT demands and enable IoT operation in an NTN, the ongoing Rel-18 WI on IoT NTN finally adds the missing RAN4 piece by specifying RAN4 core and performance requirements in addition to the related RAN1/2/3 work carried out in Rel-17, and these Rel-18 requirements will be applicable to the Rel-17 specifications for NB-IoT and eMTC operation over NTN in a release dependence manner. The WI specifies two new FDD bands NB-IoT/eMTC NTN operation by leveraging the studies and requirements of NR NTN bands n255 and n256:

o	S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

o	L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

Recently some demands for new IoT NTN bands are emerging, for example, a new band demand with 1610 – 1626.5 MHz in UL at L-band, and 2483.5 – 2500 MHz in DL at S-band. This FDD band 254 will be Release-independent from Rel-17 for the UE.

The Rel-18 WI IoT_NTN_FDD_LS_band-Core has been 100% completed at RAN#102 meeting (Dec. 2023) and the IoT_NTN_FDD_LS_band-Perf will be completed by Jun. 2024. According to the requirements of IoT NTN bands, it is justified now to start the work on the corresponding UE conformance test specifications for L+S band for IoT NTN operation in 3GPP RAN WG5.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to ‘IoT_NTN_FDD_LS_band’, by analysing the test case impact, applicability and test environment, and updating the relevant conformance specifications.  This work item will cover Signalling, RF, Demod and RRM conformance test specifications for FDD IoT NTN band 254.

Discussion:

r1

Decision: 		The document was revised to R5-241460.

R5-240278	New WID on UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

Type: WID new		For: Endorsement
					Source: China Telecom

Abstract:

MUSIM UE's hardware capabilities are shared by the SIMs, and to use the hardware efficiently and economically, the related capabilities need to be dynamically split between the two SIMs. This can lead to a temporary hardware conflict for the UE, which may require UE to release some resources (e.g. SCell/SCG) from one SIM. If the NW A is not aware of the UE’s reduced capability change in RF chain, there may be data loss due to demodulation failure and wasting radio resources in NW A. To avoid this, assistance from UE to network A on these temporary UE (capability) restrictions can be beneficial.

In NR Rel-17 specification, gap patterns particularly for MUSIM purpose were introduced. However, corresponding RRM requirements are not specified due to lack of RAN4 TUs for Rel-17 MUSIM WI, and the related work will be done in Rel-18.

Release 18 work item NR_DualTxRx_MUSIM defines the following new features for MUSIM UEs:

1. Specify procedures to indicate preference on temporary UE capability restriction and removal of restriction with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purposee, including UE assistance information of UE capability restriction and early indication[RAN2]

2.Specify MN-SN coordination of R17 MUSIM gaps when network A is NR-DC in Rel-18 [RAN2]

3. Specify priority of periodic gap(s) in addition to scheduling gaps preference which was defined in Rel-17 and RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2]

For Rel-18 RAN2/4 work item NR_DualTxRx_MUSIM, the overall completion level for core part has reached 100% after RP#102, and the performance part target is RP#104 (2024-06).

Therefore it is justified to introduce the conformance testing for Dual Transmission/Reception (Tx/Rx) Multi-SIM UEs requirements into RAN5 specifications.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to the WI Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR to improve the performance of MUSIM UEs, including protocol and RRM test cases.

Discussion:

r1

Decision: 		The document was revised to R5-241461.

R5-240283	New WID on UE Conformance - Further NR coverage enhancements

Type: WID new		For: Endorsement
					Source: China Telecom

Abstract:

Coverage is one of the key factors that an operator considers when commercializing cellular communication networks due to its direct impact on service quality as well as CAPEX and OPEX. UL performance could be the bottleneck in most of scenarios in real deployment, while there are emerging vertical use cases that have UL heavy traffic, e.g., video uploading. In Rel-17, NR coverage has been extended for some of the bottleneck channels, in particular for PUSCH, PUCCH and Msg3. However, not all needs for coverage enhancement have been addressed by the Rel-17 WID, due to its limited scope, i.e., PRACH coverage enhancement, UE transmission power and DFT-S-OFDM waveform.

The Rel-18 work item “Further NR coverage enhancements” has specified further uplink coverage enhancements including multiple PRACH transmission, increasing of UE Tx power and dynamic waveform switching.

The overall completion level for the core part of NR_cov_enh2 WI is already 100% at the RAN #102 in December 2023. The corresponding UE conformance specifications are now required to be implemented in RAN5.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to the WID on Further NR coverage enhancements. This work item will cover RF and Protocol conformance test specifications.

Discussion:

Apple: pls. add the performance part.

China Telecom: there is no UE impact.

Apple: RF specs are listed.

China Telecom:it's only for BS conformance testing.

RAN5 Vice Chair RF: pls remove TR 38.903.

Decision: 		The document was revised to R5-241462.

R5-240303	New WID on UE Conformance - Further RF requirements enhancement for NR and EN-DC in frequency range 1

Type: WID new		For: Endorsement
					Source: Huawei, Hisilicon, China Telecom

Abstract:

3GPP TSG RAN WG4 has created Rel-18 work item for Further RF requirements enhancement for NR frequency range 1 (FR1). Requirements for following aspects are introduced under this WI:

1.	4Tx RF requirements for CPE/FWA/vehicle/industrial devices for non-CA case

a)	Tx requirements, including MPR, configured output power, etc. Both UL-MIMO and TxD are considered for the 4Tx architecture.

b)	Rx requirements, including REFSENS, ACS, etc. Both UL-MIMO and TxD are considered for the 4Tx architecture.

c)	New capability signalling for 4Tx-TxD per-band per-band combination.

2.	8Rx requirements for CPE/FWA/vehicle/industrial device, including core requirements and performance requirements

3.	MSD improvement for inter-band CA/EN-DC/DC combinations

a)	Specific lower-MSD requirements for inter-band CA and EN-DC configurations

b)	New capability signalling for lower MSD

The core part has been 100% completed and the Performance part has been 85% completed at RAN#102 (December 2023). It’s time for 3GPP TSG RAN WG5 to start working on the corresponding conformance testing part to meet the industry interest.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the conformance testing aspects for the Rel-18 Further RF requirements enhancement for NR and EN-DC in frequency range 1 WI, including RF, RRM, Demod and Protocol areas.

For enhancements for 4Tx, example bands considered in core WI are n41, n77/n78, n1.

For enhancements for 8Rx, example bands considered in core WI are n41, n77/n78, n79, n7.

In conformance test WI, example bands and example configurations would be selected from the above list based on industry interest to complete the structure of new RF test cases.

For lower-MSD improvement, RAN5 needs to decide whether the test framework is per-band-combination basis or general for all band combinations. If the test framework is decided as per-band-combination basis, example band combination(s) would be selected based on industry interest to complete the relevant RF work.

Discussion:

r1

RAN2 impact.

Decision: 		The document was revised to R5-241463.

R5-240344	New WID on UE Conformance - Network energy savings for NR

Type: WID new		For: Endorsement
					Source: Huawei, HiSilicon

Abstract:

Network energy saving is of great importance for environmental sustainability, to reduce environmental impact (greenhouse gas emissions), and for operational cost savings. In Rel-18, 3GPP created the WID for Network energy savings for NR to study the solutions to improve network energy savings. Following features are supported under this WI:

1.	SSB-less SCell operation for inter-band CA for FR1 and co-located cells

2.	Enhancement on cell DTX/DRX mechanism for RRC CONNECTED state

3.	CSI enhancements in spatial domain and power domain

4.	CHO enhancement for NES to enable UE fast handover when source enters NES mode

5.	Cell selection mechanism to prevent legacy UE from camping on cells adopting Rel-18 NES techniques

6.	Inter-node beam (de)activation and enhancements on restricting paging in a limited area

The core part of this work item has completed 100% in RAN#102 (2023-12). Therefore, it’s time for 3GPP TSG RAN WG5 to start working on the corresponding conformance testing part to meet the industry interest.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of the present work item is to provide conformance testing for Rel-18 Network energy savings for NR to reflect the relevant core and performance requirements.

Discussion:

+NTT DOCOMO, Lenovo, Motorola Mobility

Decision: 		The document was revised to R5-241464.

R5-240345	New WID on UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18

Type: WID new		For: Endorsement
					Source: Huawei, HiSilicon, CMCC

Abstract:

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. The capability is conditional mandatory and the condition is described in the field, i.e. indicated in the RAN4 specification which combinations should mandatorily support simultaneous Rx/Tx. In Rel-17, RAN4 agreed the general principle for judging the mandatory capability for a band combination.

To facilitate the application of the general principle in Rel-17, Rel-18 introduced a dedicated WI Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 to analyse the simultaneous Rx/Tx capability for the requested band combinations case by case.

The core part of this work item has been completed 85% in RAN#102 (2023-12). Therefore, it’s time for 3GPP TSG RAN WG5 to start working on the corresponding conformance testing part to meet the industry interest.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of the present work item is to provide conformance testing for Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18. The example band combinations considered in core WI are as below table:

Discussion:

need 1 more supporting company: CATT added.

Decision: 		The document was revised to R5-241465.

R5-240367	New WID on UE Conformance – Multi-carrier enhancements for NR

Type: WID new		For: Endorsement
					Source: China Telecom, Huawei, HiSilicon

Abstract:

The Rel-18 ‘Multi-carrier enhancements for NR’ is RAN1 leading WI, the scope of core part WID is captured in RP-222251 and is copied below for information.

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

•	Identify the maximum number of cells that can be scheduled simultaneously

•	Consider both intra-band and inter-band CA operation

•	Consider both FR1 and FR2

•	The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

•	UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4)

o	UE capability and RRC configuration related signalling (RAN2)

o	Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands

o	Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier

o	Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL

o	Note: The number of TAGs is limited to up to 2.

o	Note: Extension of TX switching for 2 bands to multiple TAG configurations is included in the scope. The work is limited to RAN4.

•	Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4)

o	Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

After RAN#102, according to the latest SR RP-233545, the completion of the core part has achieved 100%, and the performance part is targeted at June 2024. As a result, it is proper time to start the work on the corresponding UE conformance specifications for Rel-18 NR_MC_enh requirements in 3GPP RAN WG5.

4	Objective

The objective of this work item is to enable UE conformance testing for the Rel-18 NR Multi-carrier enhancements WI which includes analysing the test case impact, applicability, test environment, and updating the relevant conformance specifications.

The conformance testing aspects for this WI would consist of RF and RRM areas, and protocol conformance specifications.

Decision: 		The document was revised to R5-241466.

R5-240390	New WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects

Type: WID new		For: Endorsement
					Source: China Unicom, Ericsson, Huawei, Hisilicon, Qualcomm

Abstract:

With rapid development of 5G industry, emerging new class of 5G uses cases, such as smart city verticals, e-Health devices and low-tier surveillance cameras and sensors, have drawn the industry’s attention to develop a further low cost, low consumption feature compared to RedCap NR devices. To further expand the market for RedCap use cases, a R18 study item“Study on further NR RedCap (reduced capability) UE complexity reduction”was approved in Sep 2021, enabling the enhanced support of RedCap UE devices with relatively low cost, low energy consumption, and low data rate requirements features. At the RP#97 meeting, the follow-up WI Enhanced support of reduced capability NR devices has been introduced into 3GPP, and the overall completion of the core part has already achieved 100% at the RP#102 meeting in Dec 2023, and the performance part is expected to be 100% complete by June 2024. Furthermore, the CT1 WI CT1 aspects of NR_REDCAP_Ph2 is already 100% complete.

Therefore, it is proposed to introduce an associated RAN5 work item to enable UE conformance tests for enhanced support of reduced capability NR devices in R18.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this WI is to enable UE conformance testing for the corresponding R18 enhanced support of NR redcap device WIs listed under clause 2.2, including the following areas:

	Protocol test cases for enhanced support of NR Redcap device.

	RF test cases for enhanced support of NR Redcap device.

	RRM test cases for enhanced support of NR Redcap device.

	Demod test cases for enhanced support of NR Redcap device.

Decision: 		The document was revised to R5-241467.

R5-240458	New WID on UE Conformance - Introduction of 900 MHz LTE band in the US

Type: WID new		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Abstract:

3GPP TSG RAN WG4 has created Rel-18 work item of a new band to be developed for LTE and NB-IoT that makes use of the paired FDD spectrum allocated in 896-901 MHz (UL) and 935-940 MHz (DL). In May 2020 the FCC enabled broadband deployment in the 900 MHz band in the US.

To support early deployments, the use of existing Band 8 devices and infrastructure was utilized as it provided similar spectral specifications except for the duplex spacing.  3GPP Band 8 allocations utilize a 45 MHz duplex spacing and the FCC 900 MHz allocation employs a 39 MHz duplexing scheme.

The creation of a new 900 MHz band to support LTE with 5 MHz, 3 MHz, 1.4 MHz channels and NB-IoT with the necessary filter and duplex spacing will allow for continued growth of private broadband networks in the US.

Requirements for following aspects are introduced under RAN4 WI:

•	Specify a new E-UTRA/LTE FDD operating band operating with

o	UL: 896 – 901 MHz and DL 935 – 940 MHz.

o	To support 3 MHz, 1.4 MHz and NB-IoT channel bandwidths with 15 kHz SCS and 3.75kHz SCS.

•	Study and specify the co-existence requirements

o	Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

o	Assume use of 3GPP general SEM or FCC SEM and study whether additional coexistence protection is needed for Band 5.

o	Assume reusing B8 duplexer

•	Specify UE RF core requirements to operate in North America with primary operations in the US.

o	Assume power class 3 (23 dBm)

•	Specify necessary BS RF core requirements

•	Specify the corresponding RRM requirements, i.e., add the new band to group of bands.

o	Specific lower-MSD requirements for inter-band CA and EN-DC configurations

a)	New capability signalling for lower MSD

The core and performance parts have been 100% completed at RAN#100 (June 2023). It’s time for 3GPP TSG RAN WG5 to start working on the corresponding conformance testing part to meet the industry interest.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to update relevant UE conformance test cases for the RF, RRM and protocol aspects of the Rel-18 core and performance parts for LTE FDD Band 106.

Discussion:

r1

Decision: 		The document was revised to R5-241468.

R5-240517	New WID on UE Conformance - NR support for dedicated spectrum less than 5MHz for FR1

Type: WID new		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Certain licensees operate regionally and nationally in the USA, internationally across Europe and in parts of SE Asia and Africa. These networks support the family of 3GPP technologies including GSM and LTE for mission-critical applications. These operational networks are now looking to invest in the future of 3GPP by deploying 5G services. This includes the use of NR to support multiple low latency use cases (see 3GPP TR 38.825) resulting from various vertical industry domains, e.g., electrical power distribution grid & rail communication. These use cases often require less than the latency offered by LTE technology and thus need NR to fully support them.

•	Band 26 and Band 8 in the USA (Band 106 is now allocated in RAN4 for Band 8 in the USA) using 3 MHz wide channels have set a precedent of existing networks requiring a growth path to NR to meet the needs of utilities, the critical infrastructure industry (CII), and enterprise customers – what 3GPP refers to as Smart Energy and Infrastructure (SEI).

•	The CII in the U.S. is currently planning to deploy private LTE systems in this spectrum allocation to leverage the transformative power of broadband in support of infrastructure modernization for energy, transportation, logistics and other industries.

•	Rail Communication in Europe is used for operational purposes to ensure the safety of millions of rail passengers. The Future Railway Mobile Communication System (FRMCS) forms the basis for digitizing rail operations to increase train path utilization and improve punctuality. Possibilities in 5G NR to operate in bandwidths <5 MHz (e.g., from around 3 MHz upwards) would enable parallel operation of FRMCS and GSM-R and massive infrastructure reuse. The responsible authorities and organizations in Europe have set the start of the migration from GSM-R to FRMCS for 2025.

•	The provision of simultaneous use of the 2x5.6MHz FDD in the 900MHz frequency band and the associated provision of bandwidths less than 5MHz for 5G NR thus has a key function to be able to start the migration from GSM-R to FRMCS in Europe.

•	For Public Protection and Disaster Relief (PPDR), 2 x 3MHz FDD in band 28 has been identified in Europe.

•	The lower 700 MHz spectrum in the USA was auctioned in 6 MHz blocks, based on the previous use for NTSC television channels. There is interest in deploying 3 MHz B12 LTE along with 3 MHz of NR in n85 in a 6 MHz block.

NR specifications starting in Rel-15 defined a minimum bandwidth of 5 MHz channels. Although NR can support multiple channel bandwidths due to the flexible numerology implementation, channel bandwidths smaller than 5 MHz were not supported in NR.

Parent WIs have specified necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz. RAN4 has specified below requirements:

•	Necessary RF requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz, including in bands n26, n28, n85, and n100.

•	RRM and demodulation requirements to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

•	Requirements for UE speeds up to 500km/h for Band n100.

Parent WIs consider single-carrier operation, excluding RedCap.

Until RAN#102 the RAN4 Core WI reached 100% completion. Performance WI reached 40% completion and is expected to be completed in June 2024. It is therefore proposed to introduce an associated RAN5 work item to enable UE conformance test for dedicated spectrum less than 5MHz for FR1.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this WI is to enable UE conformance testing for the corresponding R18 support for dedicated spectrum less than 5MHz for FR1 WIs listed under clause 2.2, including the following areas:

	RF test cases for frequency bands with interested deployers (n26, n28, n85, n100)

	RRM test cases

	Demodulation test cases

Discussion:

TF160 manager comments

r2

Decision: 		The document was revised to R5-241469.

R5-240594	New WID on UE Conformance - XR (eXtended Reality) enhancements for NR

Type: WID new		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell, CMCC, Huawei, Hisilicon, Qualcomm

Abstract:

RAN2 has specified the following enhancements under the Rel-18 work item Core part: XR (eXtended Reality) enhancements for NR (NR_XR_enh-Core):

The enhancements related to power saving:

-	DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

The enhancements related to capacity:

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

The enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

Also CT1 has decided to specify following enhancements:

-	Impacts on the NAS to support PDU set UL handling.

Work in RAN2 has been concluded and CT1 work is expected to finish later on, therefore, it is justified to start work in RAN5.

4	Objective

RAN2 has specified enhancements for following topics under the Rel-18 NR_XR_enh-Core work item:

-	The enhancements related to power saving

-	The enhancements related to capacity

-	The enhancements for XR Awareness

The objective for RAN5 is to specify UE conformance specifications for the same.

Objectives regarding the CT1 will be clarified later on, when CT1 work is about to be finished.

Discussion:

r1

+CT1 aspects in title for the pklenary!

Decision: 		The document was revised to R5-241470.

R5-240610	New WID on UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2 (FR2)

Type: WID new		For: Endorsement
					Source: Samsung

Abstract:

In Rel-17 work item on NR support for high speed train scenario in frequency range 2 (FR2), RAN4 has focused on train roof-mounted high-power devices for NR SA single carrier scenario in FR2, by studying the FR2 HST deployment scenario and specifying the channel modelling, RF, RRM and demodulation requirements for FR2 HST.

During the Rel-17 work item, most of focus was devoted into open space deployment scenarios (Scenario-A and B, differentiated by Dmin, which is the perpendicular distance between RRH site and railway track), while limited study was provided on the tunnel scenario due to the limited Rel-17 effort and relative prioritization, which is identified by operators as important high speed train scenario. Furthermore, the intra-band carrier aggregation (CA) operation is also of interests to operators, which is not yet covered in Rel-17 work item scope.

In the Rel-17 WI, similar as FR2 handheld and other UE types, the single active panel operation was focused, i.e. only one active antenna panel at a time as baseline antenna assumption. Based on the Rel-17 conclusion, two panels shall be physically installed to flexibly support either forward or backward incoming signal direction; however, the restriction of only one active antenna panel limits utilizing two neighbouring RRHs to serve one HST UE in the bi-directional RRH deployment scenario. Accordingly, it is of importance to introduce the support of simultaneous reception with maximum 2 active panels at the train roof-mounted FR2 high power devices for the bi-directional RRH deployment scenario.

Another issue identified in Rel-17 work item is the large propagation delay difference from neighbouring RRHs to UE. For example in the uni-directional RRH deployment, it is identified that the signal propagation delay difference can be as much as 2.5us, which is much larger than CP length with 120kHz subcarrier spacing. In Rel-17 work item, the larger autonomous timing adjust step Tq is specified for FR2 HST UE, and the RAN4-based solutions for the uplink timing issue are focused, while other solutions, including involving other RAN working groups, are not fully studied.

The Rel-18 WI NR_HST_FR2_enh-Core has been 100% completed and NR_HST_FR2_enh-Perf has been 80% completed at RAN#102 meeting. According to the requirements of NR network deployment and performance optimization, it is justified now to start the work on the corresponding UE conformance test specifications for NR HST FR2 enhancement features in 3GPP RAN WG5 to meet the market requirements in time.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to ‘NR_HST_FR2_enh’, by analysing the test case impact, applicability and test environment, and updating the relevant conformance specifications.  This work item will cover Signalling, RF, Demod and RRM conformance test specifications for NR_HST_FR2_enh.

Discussion:

+Nokia

Decision: 		The document was revised to R5-241471.

R5-240612	New WID on UE Conformance – NR MIMO Evolution for Downlink and Uplink

Type: WID new		For: Endorsement
					Source: Samsung, Huawei, Hisilicon

Abstract:

MIMO is one of the key technologies in NR systems and is successful in commercial deployment. In Rel-15/16/17, MIMO features were investigated and specified for both FDD and TDD systems, of which major parts were for downlink MIMO operation. In Rel-18, it is important to identify and specify necessary enhancements for uplink MIMO, while necessary enhancements on downlink MIMO that facilitate the use of large antenna array, not only for FR1 but also for FR2, would still need to be introduced to fulfil the request for evolution of NR deployments. This comprises the following areas of enhancement.

First, significant loss of performance for a UE at high/medium speed has been observed in commercial deployments especially in multi-user MIMO (MU-MIMO) scenarios. As the performance loss is partly caused by outdated CSI, enhancements on CSI acquisition to alleviate such loss can be beneficial. Second, the unified TCI framework was introduced in Rel-17 which facilitates streamlined multi-beam operation targeting FR2. As Rel-17 focuses on single-TRP use cases, extension of unified TCI framework that focuses on multi-TRP use cases is beneficial. Third, due to the increasing need for multiplexing capacity of downlink and uplink demodulation reference signal (DMRS) from various use cases, there is a need for increasing the number of orthogonal ports for DMRS. Fourth, features for facilitating multi-TRP deployments have been introduced in Rel-16/17 focusing on non-coherent joint transmission (NC-JT). As coherent joint transmission (CJT) improves coverage and average throughput in commercial deployments with high-performance backhaul and synchronization, enhancement on CSI acquisition for FDD and TDD, targeting FR1, can be beneficial in expanding the utility of multi-TRP deployments. Fifth, as advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) become more relevant, introducing necessary enhancements to support for 8 antenna ports as well as 4 and more layers for UL transmission can offer the needed improvement for UL coverage and average throughput. Sixth, with the introduction of features for UL panel selection in Rel-17, advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) can benefit from higher UL coverage and average throughput with simultaneous UL multi-panel transmission. Finally, some further enhancement to facilitate UL multi-TRP deployments via two timing advances (TAs) and enhanced UL power control can offer additional UL performance improvement.

The Rel-18 WI NR_MIMO_evo_DL_UL-Core has been 100% completed and NR_MIMO_evo_DL_UL-Perf has been 25% completed at RAN#102 meeting. According to the requirements of NR network deployment and performance optimization, it is justified now to start the work on the corresponding UE conformance test specifications for NR MIMO evolution for DL and UL features in 3GPP RAN WG5 to meet the market requirements in time.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to ‘WI NR_MIMO_evo_DL_UL’, by analysing the test case impact, applicability and test environment, and updating the relevant conformance specifications.  This work item will cover Signaling, RF, Demod and RRM conformance test specifications for NR_MIMO_evo_DL_UL.

Decision: 		The document was revised to R5-241472.

R5-241052	New WID on UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirement for NR Ues

Type: WID new		For: Approval
					Source: Apple (UK) Limited

Abstract:

Wireless cellular systems based on the fifth generation (5G) are expected to enable superior end-user experience on environments with high traffic, allowing higher downlink and uplink speeds. The forefront of the NR deployment is the channel coding and multi-antenna reception featuring Multiple-Input-Multiple-Output (MIMO), therefore, is paramount that 3GPP standardization definitions enable robust MIMO radiated performance that correlates with realistic user case scenarios.

The core part of RAN4 MIMO OTA WI has reached 100% completion of FR1 MIMO OTA core requirements at RAN#102 with progress being made in the performance part as well. According to the requirements of NR network deployment and performance optimization, it’s justified now to start the work on the corresponding UE conformance test specifications for NR MIMO features in 3GPP RAN WG5 to meet the market requirements in time.

The progress in RAN4 will be closely monitored, status and expectations will be updated accordingly.

During the course of this work item, ongoing communication with 3GPP RAN WG4, CTIA OTA Working Group (MOSG, 5G mm-wave OTA Sub-Working group and MUSG), and CCSA TC9 WG1 shall be maintained to ensure industry coordination on this topic.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to enable the UE conformance testing corresponding to NR MIMO OTA WI listed under clause 2.2, by analysing the test cases impact, applicability, and test environment.  The work item’s outcome shall be captured in TS 38.551.

This work item will investigate and specify the following aspects:

FR1 MIMO:

-	Definition of conformance tests for NR FR1 MIMO OTA requirements, including both SA and NSA:

o	Based on the conclusion of core requirement definitions, define the applicable test cases

o	Band n1, n5, n8, n28 and n79 are the first priority.

o	Enable 4x4 MIMO OTA requirements for bands where 4x4 MIMO is applicable and enable 2x2 MIMO OTA requirements for all other bands.

-	NR FR1 MIMO OTA MU/TT definition.

RAN5 has the primary responsibility for MU comprehensive assessment for FR1 MIMO OTA. This work item scope includes further optimizing MU values, and the determination of TT for the defined test requirements.

Discussion:

title in database was changed.

FR1 is initiated, FR2 is in progress.

Decision: 		The document was revised to R5-241473.

R5-241419	New WID on UE Conformance - NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

Type: WID new		For: Endorsement
					Source: Apple Benelux B.V., Nokia

Abstract:

RAN5 has completed definition of several FR2 enhancement-based test requirements in Release 16 and 17. It is now critical to continue the work on further FR2 RF enhancements in RAN5 as part of Rel-18 scope. The conformance tests would incorporate verification of core requirements for the below features:

-	UL 256 QAM – is expected to provide improved throughput and capacity increase and the same could be extremely useful for some industry use cases, e.g., the machine transmits the photograph with super high resolution to the cloud, which needs Gbps data rate. In scenarios with lower path loss, the possibility to use 256QAM would be higher.

-	Beam correspondence during initial access: UE beam correspondence functionality for RRC_CONNECTED, RRC_INACTIVE and initial access in IDLE is specified in the RAN1 and RAN2 specifications already in Release 15 and RAN4 core requirements are being defined for FR2 UE beam correspondence requirements in RRC_INACTIVE and initial access in IDLE.  The objective is to ensure good UE RACH MSG1 performance and uplink coverage in FR2 deployments.

Until RAN#102, the Core part of the RAN4 Rel-18 WI “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” is 100% complete and Performance part is 85% completed. And have therefore reached a significant level of completion.  This necessitates the introduction of the RAN5 Conformance work item to enable the industry to pursue certification testing of the FR2 features included in Release 18.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The purpose of this work item is to specify conformance test cases for the following FR2 UE features and associated requirements:

UL 256QAM

•	Investigate and enable UL 256QAM based conformance test requirements for FR2-1

•	Specify UE RF conformance test requirements aligned with core requirements and based on the following:

•	First priority: Targeted power classes are PC1, PC2 and PC5

•	Second priority: Targeted power class is PC3

Beam correspondence requirements for RRC_INACTIVE and initial access

•	Create conformance test cases containing UE beam correspondence test requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF]

•	For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT

•	Requirements for other transmission within RRC_INACTIVE state are not precluded.

•	For initial access, specify test requirements and conformance test procedures and test points associated with verification of beam correspondence core requirements.

•	Study the potential impact on testability aspects on RAN5 test procedures including, but not limited to, test time

•	Analyze and finalize creation of any new conformance test functions or modifications to existing conformance test functions to support the new FR2 RF tests added by this work item.

Decision: 		The document was revised to R5-241474.

R5-241420	New WID on UE Conformance - Introduction of the satellite L-/S- band for NR

Type: WID new		For: Endorsement
					Source: Apple Benelux B.V., Globalstar

Abstract:

The RAN5 Release 17 conformance test specification for Non-Terrestrial Networks (NTN) conformance titled TS 38.521-5 was approved at RAN#102 in December 2023. The objective of the corresponding Core Work Item approved at RAN#86 meeting was to introduce the support of non-terrestrial networks into the NR protocol and NG-RAN architecture, as well as the definition of requirements for devices and satellite access nodes, focussing on the two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively. At the same time there exist other satellite deployments which, as was presented during the RAN4#99 meeting, use mixed L-/S-band pairing, Thus, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band labelled as n254, where the UL is on the L-band and the DL is on the S-band.

Since the Core and Performance parts of the RAN4 Work item on introduction of the L-/S- band has achieved 100% completion as of RAN#102, it is necessary to introduce the conformance test requirements related to this band within the RAN5 test specifications.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this Testing Part Work Item is to:

-	Introduce and specify a new release-independent NTN FDD band with a UE transmitting at 1610-1626.5MHz and SS transmitting at 2483.5-2500MHz within the conformance test specifications.

-	Define channel bandwidths and SCS to be utilized in the conformance tests as defined in TS 38.101-5.

-	Support UE Power class 3 (+23dBm).

-	Introduce the test requirement updates for the following test cases

-	Maximum Output Power

-	Maximum Power Reduction

-	Additional Maximum Power Reduction

-	Additional Spurious Emissions

-	Spurious Co-existence

-	Reference Sensitivity

-	Blocking Characteristics

-	Additional spectrum emission

-	Update the band grouping in RRM test specifications with n254

Decision: 		The document was withdrawn.

### 4.2	General Discussion Papers

#### 4.2.1	5GS

#### 4.2.2	NR Applicability specifications (Software Implementation)

#### 4.2.3	All other topics

R5-241452	Handling of Rel-18 ASN.1 / NAS tabular

Type: discussion		For: discussion
					Source: TF160 manager

Abstract:

Recap – decisions at RAN5#101:

New Rel-18 Test Work Items should only be proposed to RAN5 once Rel-18 Stage3 freeze has been declared by 3GPP

RAN: declared in Dec’23 ; SA/CT: targeted for Mar’24.

Rel-18 ASN.1 freeze date (targeted Jun’24) should be carefully considered for the new Rel-18 Test Work Item proposal date & 1st CRs submission at RAN5.

Status at the start of RAN5#102:

17 new Rel-18 Test Work Items proposed for endorsement.

Dozens of CRs introducing Rel-18 ASN.1 & NAS details submitted for agreement.

- There is a need to refine RAN5 timeline for introduction of Rel-18 ASN.1/NAS type definitions & IEs into RAN5 specifications .

Criteria for consideration

for Rel-18 ASN.1/NAS introduction:

Specification & implementation of existing test cases shall not be compromised:

Only stable & implementable ASN.1/NAS versions shall be used in RAN5 test specifications.

Note: Not yet frozen ASN.1/NAS is considered unstable by 3GPP (i.e. not ready for implementation).

Discrepancies between prose & implementation (e.g. TTCN) shall be avoided:

We shall keep a one-to-one match between ASN.1/NAS messages specification in prose & corresponding ASN.1/NAS messages implementation in the test system (e.g. TTCN).

RAN5 shall be able to progress the work on the approved Rel-18 Test Work Items to achieve the deadlines targeted by the industry.

Proposal1:  No update to existing ASN.1/NAS default message contents with Rel-18 content shall be allowed until RAN5#104 (Aug’24).

At RAN5#104, Rel-18 Jun’24 core specifications with frozen ASN.1/NAS will be available.

Proposal2: New Rel-18-dedicated ASN.1/NAS default message contents (e.g. new Tables in TS 38.508-1) may be added:

At RAN5#102/RAN5#103: with restriction that all IEs/fields values are set as FFS.

This is to aid the structuring of TS 38.508-1 & facilitate later introduction of real contents by various Rel-18 Work Items.

From RAN5#104: no restriction.

Discussion:

Conclusion: the proposals are accepted.

Decision: 		The document was noted.

### 4.3	RAN5 PRDs/Templates

R5-240013	RAN5#102 LS Template

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

### 4.4	Meeting schedule for 2024-25

R5-240014	Meeting schedule for 2024-25

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was revised to R5-241658.

### 4.5	Tdocs for mid-week joint session

#### 4.5.1	Agenda for mid-week session

R5-240002	Agenda - midweek session

Type: agenda		For: Information
					Source: WG Chairman

Decision: 		The document was approved.

#### 4.5.2	RF group docs for WG review/verdict - original A.I. retained

R5-240398	Addition of test frequencies for new EN-DC comb within FR2

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3029  Cat: F (Rel-18)

					Source: KDDI Corporation

Abstract:

AI 5.3.1.1.1

Discussion:

r2

Decision: 		The document was revised to R5-241927.

R5-241927	Addition of test frequencies for new EN-DC comb within FR2

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3029  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-240398)

Decision: 		The document was agreed.

R5-240861	Update for additional NR-CA and NR-DC band configurations

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3075  Cat: F (Rel-18)

					Source: Verizon

Abstract:

AI 5.3.3.1.1

Discussion:

cover: +R16!

r1

Decision: 		The document was revised to R5-241928.

R5-241928	Update for additional NR-CA and NR-DC band configurations

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3075  rev 1 Cat: F (Rel-18)

					Source: Verizon

(Replaces R5-240861)

Decision: 		The document was agreed.

#### 4.5.3	Sig group docs for WG review/verdict - original A.I. retained

R5-241560	Updates to the NTN test environment

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1468  Cat: F (Rel-18)

					Source: MCC TF160, MediaTek

Abstract:

reissued from R5-241559 because of title change

AI 6.3.14.1

Decision: 		The document was agreed.

R5-241539	Correction to default configuration of frequencyInfoSL

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3067  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240764)

Abstract:

AI 6.3.2.1

Discussion:

agreed in SIG.

Seen again in the joint on Wed.

Decision: 		The document was agreed.

#### 4.5.4	Other open issues from joint sessions - original A.I. retained

R5-240664	Way Forward on Machine Readable PICS

Type: discussion		For: Endorsement
					Source: Qualcomm CDMA Technologies

Abstract:

1. RAN5 to endorse creation of “machine readable PICS” during every spec release of

TS 38.508-2, draft version to be released in TS 38.508-1 v18.2.0

2. W/F on output format- -

A. Basic Excel Translation of TS 38.508-2

B. Allow DUT Declaration i.e allow user to enter PICS value

C. Data Processing of above(ex - in a separate tab of excel)

Providing all above as an output will be most valuable to ecosystem as Step-1, this

will allow to

1.Fill DUT PICS compliance in standard format

2.Help further automation to derive test case applicability

3. RAN5 (Contributing companies, other resources) to produce SW and support

maintenance for creation of “machine readable PICS”

A. SW is not released outside of ETSI

B. Procedure, Documentation, ToR (?)

C. Resolve Challenges in implementing above proposals

Discussion:

r1

The way forward was accepted.

Noted.

Decision: 		The document was revised to R5-241574.

R5-241574	Way Forward on Machine Readable PICS

Type: discussion		For: Endorsement
					Source: Qualcomm CDMA Technologies

(Replaces R5-240664)

Decision: 		The document was noted.

#### 4.5.5	Other

## 5	RF Functional Area

### 5.1	Review action points (fm A.I. 2.1)

### 5.2	Review incoming LS (fm A.I. 3) & new subject discussion papers

### 5.3	Open Work Items

#### 5.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083) NR_CADC_NR_LTE_DC_R16-UEConTest

##### 5.3.1.1	TS 38.508-1

###### 5.3.1.1.1	Test frequencies (Clause 4.3.1)

R5-240233	Update inter-band NR CA configuration of three bands CA_n1A-n8A-n78A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3016  Cat: F (Rel-18)

					Source: CU Digital Technology

Decision: 		The document was agreed.

R5-240274	Addition of test frequencies for new R16 NR CA combos within FR1

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3022  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240882	Addition of test frequencies for DC_2A_n2A and DC_66A_n66A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3077  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Discussion:

r3

Decision: 		The document was revised to R5-241705.

R5-241705	Addition of test frequencies for DC_2A_n2A and DC_66A_n66A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3077  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

(Replaces R5-240882)

Decision: 		The document was agreed.

R5-241091	Correction to test frequencies for CA_n48 - 2A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3083  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

()

r1

Decision: 		The document was revised to R5-241706.

R5-241706	Correction to test frequencies for CA_n48 - 2A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3083  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241091)

Decision: 		The document was agreed.

###### 5.3.1.1.2	Test environment for RF (Clauses 5)

###### 5.3.1.1.3	Test environment for RRM (Clause 7)

###### 5.3.1.1.4	Other clauses / Annexes

##### 5.3.1.2	TS 38.508-2

R5-240276	Addition of UE capability for new R16 NR CA combos within FR1

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0570  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240410	Addition of UE capability for new EN-DC comb within FR2

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0581  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

##### 5.3.1.3	TS 38.521-1

###### 5.3.1.3.1	Tx Requirements (Clause 6)

R5-240138	Update test configuration table for NS_13

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2608  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241727.

R5-241727	Update test configuration table for NS_13

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2608  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-240138)

Decision: 		The document was agreed.

R5-240315	Editorial correction to test requirement of MPR for CA test case

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2628  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Editorial

Decision: 		The document was agreed.

R5-240326	Addition of delta TIBc and UE maximum output power for new R16 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2632  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r2

Decision: 		The document was revised to R5-241728.

R5-241728	Addition of delta TIBc and UE maximum output power for new R16 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2632  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-240326)

Decision: 		The document was agreed.

R5-240443	Corrections to spurious emissions for CA_n3A-n28A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2641  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240779	Add UE maximum power requirements for CA_n1A-n28A and CA_n3A-n28A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2660  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

r1

Decision: 		The document was revised to R5-241729.

R5-241729	Add UE maximum power requirements for CA_n1A-n28A and CA_n3A-n28A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2660  rev 1 Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240779)

Decision: 		The document was agreed.

R5-240780	Add spurious emissions for UE co-existence requirements for CA_n1A_n28A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2661  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

TP analysis in R5-240778 (CR 0863)

Decision: 		The document was revised to R5-241730.

R5-241730	Add spurious emissions for UE co-existence requirements for CA_n1A_n28A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2661  rev 1 Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240780)

Decision: 		The document was agreed.

R5-241367	General updates of Spurious emissions for UE co-existence for Inter-band CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2731  Cat: F (Rel-18)

					Source: China Unicom

Discussion:

r2

Decision: 		The document was revised to R5-241880.

R5-241880	General updates of Spurious emissions for UE co-existence for Inter-band CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2731  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-241367)

Decision: 		The document was agreed.

###### 5.3.1.3.2	Rx Requirements (Clause 7)

R5-240232	Update 3DL CA reference sensitivity exceptions TC for CA_n1A-n8A-n78A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2612  Cat: F (Rel-18)

					Source: CU Digital Technology

Decision: 		The document was agreed.

R5-240328	Addition of delta RIBc and reference sensitivity for new R16 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2634  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r1

Decision: 		The document was revised to R5-241731.

R5-241731	Addition of delta RIBc and reference sensitivity for new R16 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2634  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-240328)

Decision: 		The document was agreed.

R5-240457	Correction to Reference sensitivity power level for inter-band 4DL CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2644  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

content to be merged into R5-241349.

Decision: 		The document was withdrawn.

R5-240619	FR1 CA - Test requirement correction for n2-n48 combo in test 7.3A.1_1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2650  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240620	Corrections for combo n48A-n66A in test case 7.3A.1_1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2651  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Anritsu

Decision: 		The document was agreed.

R5-240772	Addition of new test case 7.4A.4 Maximum input level for CA (5DL CA)

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2657  Cat: F (Rel-18)

					Source: KTL

Decision: 		The document was agreed.

R5-240773	Addition of new test case 7.5A.4 Adjacent channel selectivity for (5DL CA)

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2658  Cat: F (Rel-18)

					Source: KTL

Decision: 		The document was agreed.

R5-241096	Correction to Reference sensitivity for Rel-16 CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2703  Cat: F (Rel-18)

					Source: Anritsu, Huawei, HiSilicon

Discussion:

Conflicts with R5-230314

r1

Decision: 		The document was revised to R5-241732.

R5-241732	Correction to Reference sensitivity for Rel-16 CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2703  rev 1 Cat: F (Rel-18)

					Source: Anritsu, Huawei, HiSilicon

(Replaces R5-241096)

Decision: 		The document was agreed.

###### 5.3.1.3.3	Clauses 1-5 / Annexes

R5-240228	General updates of TS 38.521-1 clause 5 for R16 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2609  Cat: F (Rel-18)

					Source: CU Digital Technology

Decision: 		The document was agreed.

##### 5.3.1.4	TS 38.521-2

###### 5.3.1.4.1	Tx Requirements (Clause 6)

R5-241005	Update to MU and TT for AMPR for CA test case

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1027  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, China Unicom

Decision: 		The document was agreed.

###### 5.3.1.4.2	Rx Requirements (Clause 7)

###### 5.3.1.4.3	Clauses 1-5 / Annexes

##### 5.3.1.5	TS 38.521-3

###### 5.3.1.5.1	Tx Requirements (Clause 6)

R5-241079	Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1743  Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

Discussion:

()

r1

Decision: 		The document was revised to R5-241785.

R5-241785	Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1743  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

(Replaces R5-241079)

Decision: 		The document was agreed.

R5-241080	Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1744  Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

Discussion:

()

r1

Decision: 		The document was revised to R5-241786.

R5-241786	Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1744  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

(Replaces R5-241080)

Decision: 		The document was agreed.

R5-241081	Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1745  Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

Discussion:

()

r1

Decision: 		The document was revised to R5-241787.

R5-241787	Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1745  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

(Replaces R5-241081)

Decision: 		The document was agreed.

R5-241082	Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1746  Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

Discussion:

()

r1

Decision: 		The document was revised to R5-241788.

R5-241788	Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1746  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

(Replaces R5-241082)

Decision: 		The document was agreed.

###### 5.3.1.5.2	Rx Requirements (Clause 7)

###### 5.3.1.5.3	Clauses 1-5 / Annexes

R5-240934	Update to R16 Configuration for DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1729  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, KDDI, WE Certification, AT&T

Abstract:

TS38.521-3 clause 5 jumbo CR for WIC "NR_CADC_NR_LTE_DC_R16-UEConTest"

Decision: 		The document was agreed.

##### 5.3.1.6	TS 38.521-4

###### 5.3.1.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.1.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.1.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.1.6.4	Clauses 1-4 / Annexes

##### 5.3.1.7	TS 38.522

R5-240089	Update to R16 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0367  Cat: F (Rel-18)

					Source: CMCC, Sporton

Discussion:

r1

Decision: 		The document was revised to R5-241843.

R5-241843	Update to R16 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0367  rev 1 Cat: F (Rel-18)

					Source: CMCC, Sporton

(Replaces R5-240089)

Decision: 		The document was agreed.

##### 5.3.1.8	TS 38.533

##### 5.3.1.9	TR 38.903 (NR MU & TT analyses)

##### 5.3.1.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-240234	Update reference sensitivity test cases for CA_n1A-n8A-n78A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0850  Cat: F (Rel-18)

					Source: CU Digital Technology

Decision: 		The document was agreed.

R5-240324	Addition of reference sensitivity and spurious emissions TP analysis for new R16 NR CA combos within FR1

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0854  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240778	Introduction of spurious emission TP analysis for CA_n1A_n28A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0863  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Requirement CR in R5-240780 (CR 2661)

Decision: 		The document was agreed.

R5-241368	Update of TP analysis for Spurious emissions test cases for FR1 UL CA

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0878  Cat: F (Rel-18)

					Source: China Unicom

Discussion:

r1

Decision: 		The document was revised to R5-241903.

R5-241903	Update of TP analysis for Spurious emissions test cases for FR1 UL CA

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0878  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-241368)

Decision: 		The document was agreed.

##### 5.3.1.11	Discussion Papers / Work Plan / TC lists

#### 5.3.2	5G V2X with NR sidelink (UID-880069) 5G_V2X_NRSL_eV2XARC-UEConTest

##### 5.3.2.1	TS 38.508-1

###### 5.3.2.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.2.1.2	Test environment for RF (Clauses 5)

###### 5.3.2.1.3	Test environment for RRM (Clause 7)

###### 5.3.2.1.4	Other clauses / Annexes

##### 5.3.2.2	TS 38.508-2

R5-240999	Adding PICS for V2X testing

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0598  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

xxxx

r1

Decision: 		The document was revised to R5-241707.

R5-241707	Adding PICS for V2X testing

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0598  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240999)

Decision: 		The document was agreed.

##### 5.3.2.3	TS 38.509

##### 5.3.2.4	TS 38.521-1

###### 5.3.2.4.1	Tx Requirements (Clause 6)

R5-241001	Adding AMPR test cases for V2X

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2678  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R4-241000

Discussion:

overlap with R5-240049

r1

Decision: 		The document was revised to R5-241733.

R5-241733	Adding AMPR test cases for V2X

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2678  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241001)

Decision: 		The document was agreed.

R5-241002	Update to ASEM test cases for V2X

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2679  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241000

Decision: 		The document was agreed.

R5-241003	Update to ASE test case for V2X

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2680  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241000

Decision: 		The document was agreed.

R5-241208	Correction to test case 6.4E.2.4.1D In-band emissions for V2X / non-concurrent operation / SL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2722  Cat: F (Rel-18)

					Source: MTCC, KTL

Decision: 		The document was agreed.

R5-241210	Correction to test case 6.4E.2.4.2 In-band emissions for V2X / con-current operation

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2723  Cat: F (Rel-18)

					Source: MTCC, KTL

Decision: 		The document was agreed.

###### 5.3.2.4.2	Rx Requirements (Clause 7)

###### 5.3.2.4.3	Clauses 1-5 / Annexes

##### 5.3.2.5	TS 38.521-3

###### 5.3.2.5.1	Tx Requirements (Clause 6)

###### 5.3.2.5.2	Rx Requirements (Clause 7)

###### 5.3.2.5.3	Clauses 1-5 / Annexes

##### 5.3.2.6	TS 38.521-4

###### 5.3.2.6.1	V2X Requirements (Clause 11)

###### 5.3.2.6.2	Clauses 1-4 / Annexes

##### 5.3.2.7	TS 38.522

R5-241004	Adding test applicability for V2X test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0386  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.2.8	TS 38.533

##### 5.3.2.9	TS 36.509

##### 5.3.2.10	TR 38.903 (NR MU & TT analyses)

##### 5.3.2.11	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-241000	TP analysis for AMPR, ASEM and ASE for V2X

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0870  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-241001, R5-241002 and R5-241003

Decision: 		The document was agreed.

##### 5.3.2.12	Discussion Papers / Work Plan / TC lists

#### 5.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest

##### 5.3.3.1	TS 38.508-1

###### 5.3.3.1.1	Test frequencies (Clause 4.3.1)

R5-240275	Addition of test frequencies for new R17 NR CA combos within FR1

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3023  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240460	Introduction of test frequencies for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A)

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3036  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-240775	Introduction of test frequencies for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3068  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-240878	Update NR inter-band CA configurations FR1 four bands table

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3076  Cat: F (Rel-18)

					Source: Verizon Spain

Decision: 		The document was agreed.

R5-240886	Update additional inter-band NR CA configurations

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3078  Cat: F (Rel-18)

					Source: Verizon

Discussion:

+!

r1

Decision: 		The document was revised to R5-241708.

R5-241708	Update additional inter-band NR CA configurations

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3078  rev 1 Cat: F (Rel-18)

					Source: Verizon

(Replaces R5-240886)

Decision: 		The document was agreed.

R5-241363	Addition of test frequencies for CA n77(2A), BCS1, UL CA..

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3094  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r2

Decision: 		The document was revised to R5-241944.

R5-241944	Addition of test frequencies for CA n77(2A), BCS1, UL CA..

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3094  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-241363)

Decision: 		The document was agreed.

###### 5.3.3.1.2	Test environment for RF (Clauses 5)

###### 5.3.3.1.3	Test environment for RRM (Clause 7)

###### 5.3.3.1.4	Other clauses / Annexes

##### 5.3.3.2	TS 38.508-2

R5-240277	Addition of UE capability for new R17 NR CA combos within FR1

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0571  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240461	Introduction of CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) for physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0583  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-240863	Update for additional NR-DC band configurations

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0592  Cat: F (Rel-18)

					Source: Verizon Spain

Discussion:

cover: +R16!

r1

Decision: 		The document was revised to R5-241709.

R5-241709	Update for additional NR-DC band configurations

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0592  rev 1 Cat: F (Rel-18)

					Source: Verizon Spain

(Replaces R5-240863)

Decision: 		The document was agreed.

R5-240887	Update additional FR1 NR CA inter-band band configurations

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0593  Cat: F (Rel-18)

					Source: Verizon

Decision: 		The document was agreed.

R5-240919	Editorial correction to supported EN-DC configuration table

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0596  Cat: F (Rel-18)

					Source: WE Certification

Discussion:

D->F

r1

Decision: 		The document was revised to R5-241710.

R5-241710	Editorial correction to supported EN-DC configuration table

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0596  rev 1 Cat: F (Rel-18)

					Source: WE Certification

(Replaces R5-240919)

Decision: 		The document was agreed.

##### 5.3.3.3	TS 38.521-1

###### 5.3.3.3.1	Tx Requirements (Clause 6)

R5-240327	Addition of spurious emissions, delta TIBc and UE maximum output power for new R17 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2633  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r1

Decision: 		The document was revised to R5-241734.

R5-241734	Addition of spurious emissions, delta TIBc and UE maximum output power for new R17 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2633  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-240327)

Decision: 		The document was agreed.

R5-240465	Add UE maximum power requirements for CA_n71A-n78A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2646  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-240466	Add general spurious emissions for UE co-existence requirements for CA_n71A-n77A and CA_n71A-n78A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2647  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

TP analysis in R5-240464 (CR 0860)

Decision: 		The document was agreed.

R5-240835	Update of 6.3C.1 for minimum output power for SUL bands

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2668  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240836	Update of 6.3C.2 for transmit OFF power for SUL bands

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2669  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

###### 5.3.3.3.2	Rx Requirements (Clause 7)

R5-240329	Addition of delta RIBc and reference sensitivity for new R17 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2635  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r1

Decision: 		The document was revised to R5-241735.

R5-241735	Addition of delta RIBc and reference sensitivity for new R17 NR CA combos within FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2635  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-240329)

Decision: 		The document was agreed.

R5-240463	Adding Reference sensitivity test requirements for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A)

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2645  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

TP analysis in R5-240462 (CR 0859)

Discussion:

r1

Decision: 		The document was revised to R5-241736.

R5-241736	Adding Reference sensitivity test requirements for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A)

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2645  rev 1 Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240463)

Decision: 		The document was agreed.

R5-240777	Adding Reference sensitivity test requirements for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2659  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

TP analysis in R5-240776 (CR 0862)

Decision: 		The document was agreed.

R5-240880	Update reference sensitivity requirement for 4DL CA table

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2670  Cat: F (Rel-18)

					Source: Verizon

Discussion:

content has been merged into R5-241349r1.

Decision: 		The document was withdrawn.

R5-240888	Update reference sensitivity requirement table for additional band configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2671  Cat: F (Rel-18)

					Source: Verizon

Decision: 		The document was agreed.

R5-241097	Correction to Reference sensitivity for CA_n28A-n41A-n79A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2704  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Editorial

Table 7.3A.0.5-2 has further errors for other CA combinations, but it was not corrected in this CR due to the huge amount of corrections. It should be corrected in the future meeting.

Discussion:

r2

Decision: 		The document was revised to R5-241737.

R5-241737	Correction to Reference sensitivity for CA_n28A-n41A-n79A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2704  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241097)

Decision: 		The document was agreed.

R5-241098	Clarification of asymmetric BW in Rx test cases for CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2705  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-241738.

R5-241738	Clarification of asymmetric BW in Rx test cases for CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2705  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241098)

Decision: 		The document was agreed.

R5-241122	Correction to Reference sensitivity for CA_n26A-n70A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2715  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Editorial

Depending on RAN4 CR R4-2400593

Discussion:

r1

Decision: 		The document was revised to R5-242015.

R5-242015	Correction to Reference sensitivity for CA_n26A-n70A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2715  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241122)

Decision: 		The document was agreed.

R5-241349	Update of 4DL CA test cases

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2728  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Verizon

Abstract:

Applicability update in CR R5-241350, test point selection in CR R5-241351

Discussion:

r3

Decision: 		The document was revised to R5-241907.

R5-241907	Update of 4DL CA test cases

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2728  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Verizon

(Replaces R5-241349)

Decision: 		The document was agreed.

###### 5.3.3.3.3	Clauses 1-5 / Annexes

R5-240229	General updates of TS 38.521-1 clause 5 for R17 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2610  Cat: F (Rel-18)

					Source: CU Digital Technology, Verizon, Nokia, KDDI

Decision: 		The document was revised to R5-241868.

R5-241868	General updates of TS 38.521-1 clause 5 for R17 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2610  rev 1 Cat: F (Rel-18)

					Source: CU Digital Technology, Verizon, Nokia, KDDI

(Replaces R5-240229)

Decision: 		The document was agreed.

R5-241121	Correction to note for inter-band CA including n48

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2714  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400597

Discussion:

conflicts with R5-240228.

r2

Decision: 		The document was revised to R5-241945.

R5-241945	Correction to note for inter-band CA including n48

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2714  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241121)

Decision: 		The document was agreed.

##### 5.3.3.4	TS 38.521-2

###### 5.3.3.4.1	Tx Requirements (Clause 6)

###### 5.3.3.4.2	Rx Requirements (Clause 7)

###### 5.3.3.4.3	Clauses 1-5 / Annexes

R5-240876	Update CA bandwidth classes table

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1023  Cat: F (Rel-18)

					Source: Verizon

Discussion:

general clause updated which don't have test clause impact hence withdrawn

Decision: 		The document was withdrawn.

##### 5.3.3.5	TS 38.521-3

###### 5.3.3.5.1	Tx Requirements (Clause 6)

###### 5.3.3.5.2	Rx Requirements (Clause 7)

R5-241108	Correction to Reference sensitivity for DC_21A_n28A

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1748  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

###### 5.3.3.5.3	Clauses 1-5 / Annexes

R5-240935	Update to R17 Configuration for DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1730  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Abstract:

TS38.521-3 clause 5 jumbo CR for WIC "NR_CADC_NR_LTE_DC_R17-UEConTest"

Discussion:

late doc

Decision: 		The document was withdrawn.

##### 5.3.3.6	TS 38.521-4

###### 5.3.3.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.3.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.3.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.3.6.4	Clauses 1-4 / Annexes

##### 5.3.3.7	TS 38.522

R5-240090	Update to R17 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0368  Cat: F (Rel-18)

					Source: CMCC

Discussion:

no test case changes at RAN5#102

Decision: 		The document was withdrawn.

R5-241350	Update of applicability for FR1 4DL CA test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0399  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Test case update in CR R5-241349

Discussion:

remove the overlap with Jumbo CR R5-240932

r1

Decision: 		The document was revised to R5-241946.

R5-241946	Update of applicability for FR1 4DL CA test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0399  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241350)

Decision: 		The document was agreed.

##### 5.3.3.8	TS 38.533

##### 5.3.3.9	TR 38.903 (NR MU & TT analyses)

##### 5.3.3.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-240325	Addition of reference sensitivity and spurious emissions TP analysis for new R17 NR CA combos within FR1

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0855  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240462	Introduction of reference sensitivity test point analysis for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A)

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0859  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Requirement CR in R5-240463 (CR 2645)

Decision: 		The document was agreed.

R5-240464	Introduction of spurious emission TP analysis for CA_n71A-n78A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0860  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Requirement CR in R5-240466 (CR 2647)

Decision: 		The document was agreed.

R5-240776	Introduction of reference sensitivity test point analysis for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0862  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Requirement CR in R5-240777 (CR 2659)

Decision: 		The document was agreed.

R5-240883	Update reference sensitivity test cases for four bands configurations

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0867  Cat: F (Rel-18)

					Source: Verizon

Abstract:

Associate with TS 38.521-1 CR 2670

Discussion:

3 WICS!

r1

content is covered by R5-241351

Decision: 		The document was revised to R5-241701.

R5-241701	Update reference sensitivity test cases for four bands configurations

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0867  rev 1 Cat: F (Rel-18)

					Source: Verizon

(Replaces R5-240883)

Decision: 		The document was withdrawn.

R5-240891	Update reference sensitivity test cases for additional band configurations

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0868  Cat: F (Rel-18)

					Source: Verizon

Abstract:

Associated with  TS 38.521-1 CR 2671

Decision: 		The document was agreed.

R5-241351	Update of test point selection for 4DL CA configurations

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0877  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Verizon

Abstract:

Test case update in CR R5-241349

Discussion:

r1

Decision: 		The document was revised to R5-241908.

R5-241908	Update of test point selection for 4DL CA configurations

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0877  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Verizon

(Replaces R5-241351)

Decision: 		The document was agreed.

##### 5.3.3.11	Discussion Papers / Work Plan / TC lists

R5-241364	Discussion 2 regarding limiting number of test frequencies for CA combinations

Type: discussion		For: Agreement
					Source: Ericsson

Discussion:

"Implemented CR R5-241363

prop1b endorsed. How to maintain the table AP to be raised and concluded in future meeting"

Decision: 		The document was noted.

#### 5.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest

##### 5.3.4.1	TS 38.508-1

###### 5.3.4.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.4.1.2	Test environment for RF (Clauses 5)

###### 5.3.4.1.3	Test environment for RRM (Clause 7)

###### 5.3.4.1.4	Other clauses / Annexes

##### 5.3.4.2	TS 38.508-2

R5-241165	Removal of duplicated RSSI measurements and channel occupancy reporting parameter

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0600  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241286	Addition of PICS for UL LBT Failure Detection and Recovery

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0604  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

r1

Decision: 		The document was revised to R5-241711.

R5-241711	Addition of PICS for UL LBT Failure Detection and Recovery

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0604  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241286)

Decision: 		The document was agreed.

##### 5.3.4.3	TS 38.509

##### 5.3.4.4	TS 38.521-1

###### 5.3.4.4.1	Tx Requirements (Clause 6)

R5-240810	Updates to NR-U Tx test cases

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2662  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R5-241739.

R5-241739	Updates to NR-U Tx test cases

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2662  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240810)

Decision: 		The document was agreed.

R5-240817	Addition of test case 6.3F.4.3 relative power tolerance for shared spectrum channel access

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2663  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

TP analysis in R5-240858

Decision: 		The document was agreed.

R5-240821	Updates of test case 6.2F.4 Configured transmitted power for shared spectrum access

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2665  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

TP analysis in R5-240859

Decision: 		The document was agreed.

R5-241177	Addition of test case 6.5F.3.3 	Additional spurious emissions for shared spectrum channel access

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2719  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Addition of new tc 6.5F.3.3

Discussion:

' '

r1

Decision: 		The document was revised to R5-241740.

R5-241740	Addition of test case 6.5F.3.3 	Additional spurious emissions for shared spectrum channel access

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2719  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-241177)

Decision: 		The document was agreed.

R5-241195	Updates of test case 6.2F.3 UE additional maximum output power reduction for shared spectrum access

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2720  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Updates of test requirements 6.2F.3.5

Discussion:

cl. aff.

r1

Decision: 		The document was revised to R5-241741.

R5-241741	Updates of test case 6.2F.3 UE additional maximum output power reduction for shared spectrum access

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2720  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-241195)

Decision: 		The document was agreed.

R5-241258	Updates of test case 6.5F.2.4.2, Shared spectrum channel access ACLR with additional requirement for NS_29

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2724  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Updates and also editorial correction due to 6.5F.2.4.2 placed in wrong section

Discussion:

cl. aff.

r1

Decision: 		The document was revised to R5-241742.

R5-241742	Updates of test case 6.5F.2.4.2, Shared spectrum channel access ACLR with additional requirement for NS_29

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2724  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-241258)

Decision: 		The document was agreed.

###### 5.3.4.4.2	Rx Requirements (Clause 7)

###### 5.3.4.4.3	Clauses 1-5 / Annexes

R5-240819	Addition of MU and TT for NR-U test cases

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2664  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240828	Addition of RMC tables for shared spectrum access in Annex A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2667  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

##### 5.3.4.5	TS 38.521-3

###### 5.3.4.5.1	Tx Requirements (Clause 6)

###### 5.3.4.5.2	Rx Requirements (Clause 7)

###### 5.3.4.5.3	Clauses 1-5 / Annexes

##### 5.3.4.6	TS 38.521-4

###### 5.3.4.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-240829	Addition of test case 6.2A.4.1.1, 4Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA)

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0801  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

RAN4 dependent

Decision: 		The document was withdrawn.

R5-241139	Update to NR-U demod test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0808  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241821.

R5-241821	Update to NR-U demod test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0808  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241139)

Decision: 		The document was agreed.

###### 5.3.4.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.4.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.4.6.4	Clauses 1-4 / Annexes

R5-240830	Core spec alignment, addition of RMC and UL-DL configuration for NR-U

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0802  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Core spec alignment

Discussion:

overlapping with R5-241139

r1

Decision: 		The document was revised to R5-241822.

R5-241822	Core spec alignment, addition of RMC and UL-DL configuration for NR-U

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0802  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240830)

Decision: 		The document was agreed.

##### 5.3.4.7	TS 38.522

R5-240771	Correction to applicability for NR-U test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0378  Cat: F (Rel-18)

					Source: TTA

Decision: 		The document was agreed.

R5-240792	Addition of applicability of the NR-U RRM test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0381  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

xxxx

no WIC, CR#

r1

w/d

content merged to R5-241287r1.

Decision: 		The document was revised to R5-241702.

R5-241702	Addition of applicability of the NR-U RRM test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0381  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240792)

Decision: 		The document was withdrawn.

R5-240857	Addition of applicability for NR-U test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0383  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241131	 Adding applicability for newly introduced NR-U test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0388  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Adding new TC 6.5F.3.3

Decision: 		The document was agreed.

R5-241287	Update to NR-U test applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0394  Cat: F (Rel-18)

					Source: Qualcomm Germany, Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-241851.

R5-241851	Update to NR-U test applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0394  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany, Ericsson

(Replaces R5-241287)

Decision: 		The document was agreed.

##### 5.3.4.8	TS 38.533

R5-240518	Update to NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2894  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-240519	Update to NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2895  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-240803	Addition of NR_U test case 11.5.1.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2972  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240804	Addition of NR_U test case 11.5.1.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2973  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240805	Addition of NR_U test case 11.5.1.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2974  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240806	Addition of NR_U test case 11.5.1.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2975  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240807	Addition of NR_U test case 11.5.1.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2976  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240808	Addition of NR_U test case 11.5.2.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2977  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-241827.

R5-241827	Addition of NR_U test case 11.5.2.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2977  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240808)

Decision: 		The document was agreed.

R5-240809	Addition of NR_U test case 11.5.2.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2978  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240811	Addition of NR_U test case 11.5.2.5

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2979  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240812	Addition of NR_U test case 11.5.2.6

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2980  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240813	Addition of NR_U test case 11.5.2.7

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2981  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240814	Addition of NR_U test case 11.5.2.8

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2982  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240815	Addition of NR_U test case 11.5.2.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2983  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240816	Addition of NR_U test case 11.5.2.10

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2984  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241241	Addition of SA FR1 NR-U intra-frequency SS-SINR measurement accuracy test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3032  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241242	Addition of SA FR1 NR-U inter-frequency SS-SINR measurement accuracy test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3033  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241243	Addition of SA FR1 NR-U intra-frequency RSSI measurement accuracy test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3034  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241271	Core alignment for NSA NR-U tests

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3041  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

TEI15 ?!

r1

Decision: 		The document was revised to R5-241828.

R5-241828	Core alignment for NSA NR-U tests

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3041  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241271)

Decision: 		The document was agreed.

R5-241272	Addition of NR-U SA BWP switch tests including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3042  Cat: F (Rel-18)

					Source: Qualcomm Germany

Abstract:

RAN4 dependency R4-2400725

Decision: 		The document was agreed.

R5-241273	Addition of UL active BWP switch test 10.3.5.1 including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3043  Cat: F (Rel-18)

					Source: Qualcomm Germany

Abstract:

RAN4 dependency R4-2400725

Decision: 		The document was agreed.

R5-241274	Update of NR-U Annex E and F including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3044  Cat: F (Rel-18)

					Source: Qualcomm Germany

Abstract:

RAN4 dependency

Decision: 		The document was agreed.

R5-241275	Update to NR-U general sections

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3045  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241276	Addition of NR-U UE Timing Tests 10.2.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3046  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241277	Addition of NR-U L1-RSRP accuracy test 10.5.4.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3047  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241278	Addition of NR-U RSSI measurement accuracy tests 10.5.5.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3048  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241279	Addition of NR-U Channel occupancy measurement accuracy tests 10.5.6.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3049  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241280	Addition of NR-U L1-RSRP tests 10.4.3.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3050  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241281	Addition of NR-U IRAT tests 10.4.4.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3051  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241282	Update to NSA NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3052  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241283	Update to SA NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3053  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241284	Update to OCNG noise power in UL CCA model

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3054  Cat: F (Rel-18)

					Source: Qualcomm Germany

Abstract:

RAN4 dependency R4-2400725

Decision: 		The document was agreed.

R5-241285	Core alignment for NR-U common sections

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3055  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

##### 5.3.4.9	TR 38.903 (NR MU & TT analyses)

R5-241288	Addition of TTs for NR-U Test Cases 10.3.1.2 and 11.4.1.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0692  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

+

r1

Decision: 		The document was revised to R5-241881.

R5-241881	Addition of TTs for NR-U Test Cases 10.3.1.2 and 11.4.1.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0692  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241288)

Decision: 		The document was agreed.

R5-241289	Addition of TTs for NR-U Test Cases 10.3.1.3 and 11.4.1.3

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0693  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

+

r1

Decision: 		The document was revised to R5-241882.

R5-241882	Addition of TTs for NR-U Test Cases 10.3.1.3 and 11.4.1.3

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0693  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241289)

Decision: 		The document was agreed.

R5-241290	Addition of TTs for NR-U Test Cases 10.3.4.1, 10.3.4.2, 11.4.4.1 and 11.4.4.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0694  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

+

r1

Decision: 		The document was revised to R5-241883.

R5-241883	Addition of TTs for NR-U Test Cases 10.3.4.1, 10.3.4.2, 11.4.4.1 and 11.4.4.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0694  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241290)

Decision: 		The document was agreed.

R5-241291	Addition of TTs for NR-U Test Cases 10.3.5.1 and 11.4.5.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0695  Cat: F (Rel-18)

					Source: Qualcomm Germany

Abstract:

RAN4 dependency R4-2400725

Discussion:

+

r1

Decision: 		The document was revised to R5-242000.

R5-242000	Addition of TTs for NR-U Test Cases 10.3.5.1 and 11.4.5.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0695  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241291)

Decision: 		The document was agreed.

R5-241292	Addition of TTs for NR-U Test Cases 10.3.5.2.1, 10.3.5.2.2, 11.4.5.2.1 and 11.4.5.2.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0696  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

+

r1

Decision: 		The document was revised to R5-241884.

R5-241884	Addition of TTs for NR-U Test Cases 10.3.5.2.1, 10.3.5.2.2, 11.4.5.2.1 and 11.4.5.2.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0696  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241292)

Decision: 		The document was agreed.

R5-241293	Addition of TTs for NR-U Test Cases 10.3.5.3.1 and 11.4.5.3.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0697  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

+

r1

Decision: 		The document was revised to R5-241885.

R5-241885	Addition of TTs for NR-U Test Cases 10.3.5.3.1 and 11.4.5.3.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0697  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241293)

Decision: 		The document was agreed.

##### 5.3.4.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-240858	Addition of TP analysis for NR-U test case 6.3F.4.3

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0864  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Implemented in R5-240817

Decision: 		The document was agreed.

R5-240859	Addition of TP analysis for NR-U test case 6.2F.4, Configured transmitted power for shared spectrum

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0865  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Implemented in R5-240821

Decision: 		The document was agreed.

##### 5.3.4.11	Discussion Papers / Work Plan / TC lists

R5-241294	Discussion on MU impact to energy detection threshold in shared spectrum

Type: discussion		For: Endorsement
					Source: Qualcomm Germany

Discussion:

r1

Decision: 		The document was revised to R5-241823.

R5-241823	Discussion on MU impact to energy detection threshold in shared spectrum

Type: discussion		For: Endorsement
					Source: Qualcomm Germany

(Replaces R5-241294)

Discussion:

"Revised from: R5-241294r1.

proposal is endorsed"

Decision: 		The document was noted.

#### 5.3.5	Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2 / 3 / y=1 or x=1 / 2 / y=2) (UID-930051) ENDC_PC2_R17_xLTE_yNR-UEConTest

##### 5.3.5.1	TS 38.508-2

R5-240914	Update for additional ENDC band configurations with PC2 UL

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0595  Cat: F (Rel-18)

					Source: Verizon

Decision: 		The document was revised to R5-241720.

R5-241720	Update for additional ENDC band configurations with PC2 UL

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0595  rev 1 Cat: F (Rel-18)

					Source: Verizon

(Replaces R5-240914)

Decision: 		The document was agreed.

##### 5.3.5.2	TS 38.521-3

5.3.5.2.1	Tx Requirements (Clause 6)

###### 5.3.5.2.2	Rx Requirements (Clause 7)

R5-240916	Update reference sensitity minimum requirement for additional ENDC band configurations with PC2 UL

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1726  Cat: F (Rel-18)

					Source: Verizon Spain

Discussion:

CR cover value : 18.1.0.

Content has been merged into R5-240168r1.

Decision: 		The document was withdrawn.

R5-240917	Update reference sensitity test configuration and test requirement tables for ENDC band configurations with PC2 UL

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1727  Cat: F (Rel-18)

					Source: Verizon Spain

Discussion:

CR cover value : 18.1.0.

Content has been merged into R5-240168r1.

Decision: 		The document was withdrawn.

###### 5.3.5.2.3	Clauses 1-5 / Annexes

##### 5.3.5.3	TS 38.522

##### 5.3.5.4	Discussion Papers / Work Plan / TC lists

##### 5.3.6.	Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 (UID-950062) NR_FR2_FWA_Bn257_Bn258-UEConTest

##### 5.3.6.1	TS 38.508-1

##### 5.3.6.2	TS 38.508-2

##### 5.3.6.3	TS 38.521-2

###### 5.3.6.3.1	Tx Requirements (Clause 6)

###### 5.3.6.3.2	Rx Requirements (Clause 7)

###### 5.3.6.3.3	Clauses 1-5 / Annexes

##### 5.3.6.4	TS 38.521-4

###### 5.3.6.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.6.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.6.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.6.4.4	Clauses 1-4 / Annexes

##### 5.3.6.5	TS 38.522

##### 5.3.6.6	TS 38.533

##### 5.3.6.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.6.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.6.9	Discussion Papers / Work Plan / TC lists

#### 5.3.7	NR coverage enhancements (UID-950063) NR_cov_enh-UEConTest

##### 5.3.7.1	TS 38.508-1

##### 5.3.7.2	TS 38.508-2

##### 5.3.7.3	TS 38.521-1

###### 5.3.7.3.1	Tx Requirements (Clause 6)

R5-241432	Updates to FR1 RF phase continuity test

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2733  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

feedback from Huawei

r2

Decision: 		The document was revised to R5-241947.

R5-241947	Updates to FR1 RF phase continuity test

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2733  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241432)

Decision: 		The document was agreed.

###### 5.3.7.3.2	Rx Requirements (Clause 7)

###### 5.3.7.3.3	Clauses 1-5 / Annexes

R5-241434	Updates and corrections to Annex in FR1 RF spec

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2734  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

Huawei reviewed.

r2

Decision: 		The document was revised to R5-241948.

R5-241948	Updates and corrections to Annex in FR1 RF spec

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2734  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241434)

Decision: 		The document was agreed.

##### 5.3.7.4	TS 38.521-2

###### 5.3.7.4.1	Tx Requirements (Clause 6)

R5-241433	Updates to FR2 RF phase continuity test

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1038  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

r2

Decision: 		The document was revised to R5-241949.

R5-241949	Updates to FR2 RF phase continuity test

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1038  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241433)

Decision: 		The document was agreed.

###### 5.3.7.4.2	Rx Requirements (Clause 7)

###### 5.3.7.4.3	Clauses 1-5 / Annexes

R5-241435	Updates to Annex E content and structure

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1039  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

r1

Decision: 		The document was revised to R5-241950.

R5-241950	Updates to Annex E content and structure

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1039  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241435)

Decision: 		The document was agreed.

##### 5.3.7.5	TS 38.522

R5-241438	Applicability updates for Phase continuity tests

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0404  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

r1

Decision: 		The document was revised to R5-241844.

R5-241844	Applicability updates for Phase continuity tests

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0404  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241438)

Decision: 		The document was agreed.

##### 5.3.7.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.7.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-241436	Updates to TP analysis for FR2 RF phase continuity test

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0880  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

r1

Decision: 		The document was revised to R5-241743.

R5-241743	Updates to TP analysis for FR2 RF phase continuity test

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0880  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241436)

Decision: 		The document was agreed.

R5-241437	Updates to TP analysis for FR1 RF phase continuity test

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0881  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

r1

Decision: 		The document was revised to R5-241744.

R5-241744	Updates to TP analysis for FR1 RF phase continuity test

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0881  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241437)

Decision: 		The document was agreed.

##### 5.3.7.8	Discussion Papers / Work Plan / TC lists

#### 5.3.8	Support of reduced capability NR devices (UID-950066) NR_redcap_plus_ARCH-UEConTest

##### 5.3.8.1	TS 38.508-1

R5-240676	Correction to default configuration of SMTC for NCD-SSB

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3065  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

Abstract:

RAN4 dependency

R4-2401304 -> R4-2403401

Discussion:

r1

Decision: 		The document was revised to R5-242038.

R5-242038	Correction to default configuration of SMTC for NCD-SSB

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3065  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

(Replaces R5-240676)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

##### 5.3.8.2	TS 38.508-2

##### 5.3.8.3	TS 38.521-1

###### 5.3.8.3.1	Tx Requirements (Clause 6)

###### 5.3.8.3.2	Rx Requirements (Clause 7)

R5-241369	Update of Refsens TC for RedCap UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2732  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was agreed.

###### 5.3.8.3.3	Clauses 1-5 / Annexes

R5-240826	Editorial, correcting Test case title for 7.3I.2 in Annex A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2666  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

##### 5.3.8.4	TS 38.521-2

###### 5.3.8.4.1	Tx Requirements (Clause 6)

R5-240837	Corrections on 6.2.3 for FR2 Redcap UE MPR test case for NS_202 and NS_203

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1021  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

r1

Decision: 		The document was revised to R5-241781.

R5-241781	Corrections on 6.2.3 for FR2 Redcap UE MPR test case for NS_202 and NS_203

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1021  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-240837)

Decision: 		The document was agreed.

R5-240838	Corrections on 6.3.1 for FR2 Redcap UE minimum output power

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1022  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

###### 5.3.8.4.2	Rx Requirements (Clause 7)

###### 5.3.8.4.3	Clauses 1-5 / Annexes

##### 5.3.8.5	TS 38.521-4

###### 5.3.8.5.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-240677	Addition of RedCap demod test case 6.3.1.1.1

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0797  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r2

Decision: 		The document was revised to R5-241960.

R5-241960	Addition of RedCap demod test case 6.3.1.1.1

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0797  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240677)

Decision: 		The document was agreed.

R5-240678	Addition of RedCap demod test case 6.3.1.2.1

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0798  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241817.

R5-241817	Addition of RedCap demod test case 6.3.1.2.1

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0798  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240678)

Decision: 		The document was agreed.

R5-240679	Addition of RedCap demod test case 6.3.2.2.7

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0799  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241961.

R5-241961	Addition of RedCap demod test case 6.3.2.2.7

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0799  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240679)

Decision: 		The document was agreed.

R5-240827	Correction of TT for RedCap demod test case 5.3.2.1.4 and 5.3.2.2.4

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0800  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241113	Correction to 1Rx FR1 PDSCH and PDCCH performance for RedCap

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0805  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241142	Addition of 6.2.1.1.2.1 test for RedCap

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0810  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

Decision: 		The document was agreed.

R5-241143	Update incorrect reference to SDR test in Table 5.1.1.11-1 for RedCap

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0811  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

Decision: 		The document was agreed.

R5-241175	Correction RedCap CSI Test Case message exceptions

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0823  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

###### 5.3.8.5.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

R5-241141	Addition of FR2 SDR test for RedCap

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0809  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

Decision: 		The document was agreed.

###### 5.3.8.5.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.8.5.4	Clauses 1-4 / Annexes

R5-240864	Addition of Measurement uncertainty for Redcap PDSCH demodulation test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0803  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

##### 5.3.8.6	TS 38.522

R5-240680	Addition of applicability for RedCap Demod and RRM test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3066  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Decision: 		The document was withdrawn.

R5-240791	Addition of event triggered reporting test cases applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0380  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R5-241849.

R5-241849	Addition of event triggered reporting test cases applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0380  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240791)

Decision: 		The document was agreed.

R5-240831	Addition of applicability for RedCap test case 6.2.2.1.2.4

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0382  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241140	Update to applicability spec for Redcap Demod test case

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0389  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

Decision: 		The document was agreed.

R5-241240	Addition of missing applicability to new SS-RSRQ RedCap  test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0393  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

xxxx

r1

Decision: 		The document was revised to R5-241962.

R5-241962	Addition of missing applicability to new SS-RSRQ RedCap  test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0393  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241240)

Decision: 		The document was agreed.

R5-241302	Applcability update for several RedCap tests

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0395  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241379	Add subtest selection criteria to RedCap Performance test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0401  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Decision: 		The document was agreed.

R5-241446	Addition of applicability for RedCap Demod and RRM test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0406  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was revised to R5-241848.

R5-241848	Addition of applicability for RedCap Demod and RRM test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0406  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241446)

Decision: 		The document was agreed.

##### 5.3.8.7	TS 38.533

R5-240204	Correction of Redcap inter-RAT E-UTRAN CGI test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2879  Cat: F (Rel-18)

					Source: MediaTek Inc.

Abstract:

RAN4 dependency

Discussion:

r2

Decision: 		The document was revised to R5-242037.

R5-242037	Correction of Redcap inter-RAT E-UTRAN CGI test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2879  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-240204)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240449	Resubmission of Annex B Corrections to BWP RMCs and configurations for RedCap RRM TCs

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2893  Cat: F (Rel-18)

					Source: ETSI MCC (  Huawei, HiSilicon, Starpoint)

Abstract:

This is a resubmission of the missed changes in A.8A.1, A.8A.2 of R5-236736, CR# 2763

Decision: 		The document was agreed.

R5-240681	Addition of RedCap RRM test case 17.5.3.1.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2901  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240682	Addition of RedCap RRM test case 17.5.3.2.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2902  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240683	Addition of RedCap RRM test case 17.5.5.1.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2903  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Decision: 		The document was agreed.

R5-240684	Addition of RedCap RRM test case 17.5.5.2.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2904  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Decision: 		The document was agreed.

R5-240685	Addition of RedCap RRM test case 17.5.6.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2905  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency

R4-2401306->R4-2403402 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240686	Addition of RedCap RRM test case 17.7.1.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2906  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240687	Addition of RedCap RRM test case 17.7.1.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2907  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240688	Addition of RedCap RRM test case 17.7.2.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2908  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240689	Addition of RedCap RRM test case 17.7.2.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2909  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240690	Addition of RedCap RRM test case 17.7.3.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2910  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240691	Addition of RedCap RRM test case 17.7.3.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2911  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240692	Addition of RedCap RRM test case 17.7.4.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2912  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240693	Correction to NCD-SSB and SMTC RMCs

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2913  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

Abstract:

RAN4 dependency

R4-2401304 -> R4-2403401

Discussion:

r1

Decision: 		The document was revised to R5-242039.

R5-242039	Correction to NCD-SSB and SMTC RMCs

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2913  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

(Replaces R5-240693)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240694	Correction to RedCap inter-frequency measurement test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2914  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint, Anritsu, Keysight

Abstract:

RAN4 dependency

R4-2401304 -> R4-2403401

Discussion:

r1

Decision: 		The document was revised to R5-242040.

R5-242040	Correction to RedCap inter-frequency measurement test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2914  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint, Anritsu, Keysight

(Replaces R5-240694)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240695	Correction to RedCap RRM test case 16.5.2.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2915  Cat: F (Rel-18)

					Source: Huawei,HiSilicon,Starpoint

Abstract:

RAN4 dependency

R4-2401306 -> R4-2403402

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240696	Correction to RedCap RRM test case 16.5.4.1 and 16.5.4.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2916  Cat: F (Rel-18)

					Source: Huawei,HiSilicon,Starpoint

Abstract:

RAN4 dependency

R4-2401306 -> R4-2403402

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240697	Correction to RedCap RRM test case 17.4.1.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2917  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency

R4-2401306 -> R4-2403402

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240698	Correction to RedCap RRM test case 17.6.4.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2918  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RAN4 dependency

R4-2401304 -> R4-2403401

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240699	Correction to SSB time offset in NCD-SSB test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2919  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

Abstract:

RAN4 dependency

R4-2401304 -> R4-2403401

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240705	Correction to RedCap RRM test case 16.2.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2925  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401306 -> R4-2403402

Discussion:

for email agreement

Power level changes for RedCap SDT in R4-2401306 are not agreed in RAN4.

r1 is needed to reverse these changes

Decision: 		The document was revised to R5-241670.

R5-241670	Correction to RedCap RRM test case 16.2.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2925  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240705)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240706	Correction to RedCap RRM test case 16.2.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2926  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401306 -> R4-2403402

Discussion:

for email agreement

Power level changes for RedCap SDT in R4-2401306 are not agreed in RAN4.

r1 is needed to reverse these changes.

Decision: 		The document was revised to R5-241671.

R5-241671	Correction to RedCap RRM test case 16.2.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2926  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240706)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240707	Correction to RedCap RRM test cases 16.3.2.2.X with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2927  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240708	Correction to RedCap RRM test case 16.6.5.3 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2928  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241886.

R5-241886	Correction to RedCap RRM test case 16.6.5.3 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2928  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240708)

Decision: 		The document was agreed.

R5-240709	Correction to RedCap RRM test case 16.6.5.4 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2929  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240710	Correction to RedCap RRM test case 16.7.2.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2930  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240711	Correction to RedCap RRM test case 16.7.2.3.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2931  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

r2

Decision: 		The document was revised to R5-242041.

R5-242041	Correction to RedCap RRM test case 16.7.2.3.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2931  rev 1 Cat: F (Rel-18)

					Source: Huawei,HiSilicon

(Replaces R5-240711)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240712	Correction to RedCap RRM test case 16.7.2.4.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2932  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

r1

Decision: 		The document was revised to R5-242042.

R5-242042	Correction to RedCap RRM test case 16.7.2.4.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2932  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240712)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240713	Correction to RedCap RRM test case 16.7.2.4.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2933  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

r1

Decision: 		The document was revised to R5-242043.

R5-242043	Correction to RedCap RRM test case 16.7.2.4.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2933  rev 1 Cat: F (Rel-18)

					Source: Huawei,HiSilicon

(Replaces R5-240713)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240714	Correction to RedCap RRM test case 16.7.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2934  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240715	Correction to RedCap RRM test case 16.7.3.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2935  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240716	Correction to RedCap RRM test case 16.7.3.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2936  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240717	Correction to RedCap RRM test case 16.7.3.3.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2937  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240718	Correction to RedCap RRM test case 16.7.3.4.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2938  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240719	Correction to RedCap RRM test case 16.7.3.4.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2939  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241963.

R5-241963	Correction to RedCap RRM test case 16.7.3.4.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2939  rev 1 Cat: F (Rel-18)

					Source: Huawei,HiSilicon

(Replaces R5-240719)

Decision: 		The document was agreed.

R5-240720	Correction to RedCap RRM test case 16.7.4.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2940  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240721	Correction to RedCap RRM test case 16.7.4.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2941  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240722	Correction to RedCap RRM test case 16.7.4.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2942  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240723	Correction to RedCap RRM test case 16.7.4.2.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2943  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240724	Correction to RedCap RRM test case 16.7.4.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2944  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240725	Correction to RedCap RRM test case 16.7.4.3.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2945  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240726	Correction to RedCap RRM test case 16.7.4.4.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2946  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240727	Correction to RedCap RRM test case 16.7.4.4.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2947  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240728	Correction to RedCap RRM test case 16.7.5.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2948  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240729	Correction to RedCap RRM test case 16.7.5.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2949  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240730	Correction to RedCap RRM test case 16.7.6.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2950  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240731	Correction to RedCap RRM test case 16.7.6.2 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2951  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240732	Correction to Annex F for RedCap RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2952  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241878.

R5-241878	Correction to Annex F for RedCap RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2952  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240732)

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241672.

R5-241672	Correction to Annex F for RedCap RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2952  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241878)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240790	Addition of Test Tolerance into Annex F for RedCap test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2961  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240794	Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE test case 16.6.3.1 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2963  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

r1

Decision: 		The document was revised to R5-242033.

R5-242033	Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE test case 16.6.3.1 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2963  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240794)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240795	Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE test case 16.6.3.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2964  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

r1

Decision: 		The document was revised to R5-242034.

R5-242034	Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE test case 16.6.3.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2964  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240795)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240796	Addition of NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE test case 16.6.3.3 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2965  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

r1

Decision: 		The document was revised to R5-242035.

R5-242035	Addition of NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE test case 16.6.3.3 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2965  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240796)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240797	Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2Rx UE test case 16.6.3.4 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2966  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

r1

Decision: 		The document was revised to R5-242036.

R5-242036	Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2Rx UE test case 16.6.3.4 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2966  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240797)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240798	Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)  test case 17.6.2.1 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2967  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240799	Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2968  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240800	Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.3 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2969  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240801	Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.4 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2970  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240802	Addition of cell configuration mapping for event triggered measurement test cases for RedCap

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2971  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240915	Typo correction to SSB and SMTC configuration for RedCap Event Trigger TC 16.6.2.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2986  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

merged into R5-240694r1

Decision: 		The document was withdrawn.

R5-241114	Correction to OCNG references in 16.1.1 and 16.1.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2990  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241115	Correction to title of RedCap test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2991  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241116	Correction to PRACH config in 16.5.2.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2992  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241117	Correction to 16.7.2.4.x and 16.7.3.4.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2993  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was withdrawn.

R5-241118	Addition of message contents and cell mapping for 16.7.1.2.1 and 16.7.1.2.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2994  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was withdrawn.

R5-241124	Correction to test configuration in 16.4.1.2 and 16.5.2.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2998  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400301

Discussion:

r1

Decision: 		The document was revised to R5-241964.

R5-241964	Correction to test configuration in 16.4.1.2 and 16.5.2.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2998  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241124)

Decision: 		The document was agreed.

R5-241125	Correction to PDCCH level in CSI-RS based RLM test cases for RedCap

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2999  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400291

Discussion:

r1

Decision: 		The document was revised to R5-241965.

R5-241965	Correction to PDCCH level in CSI-RS based RLM test cases for RedCap

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2999  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241125)

Decision: 		The document was agreed.

R5-241126	Correction to gap offset and SMTC configuration in 16.6.2.9 and 16.6.2.10

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3000  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400303

Discussion:

r1

w/d

Decision: 		The document was revised to R5-241824.

R5-241824	Correction to gap offset and SMTC configuration in 16.6.2.9 and 16.6.2.10

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3000  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241126)

Decision: 		The document was withdrawn.

R5-241127	Correction to SSB configuration in 16.6.2.11 and 16.6.2.12

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3001  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400305

Discussion:

r1

w/d

Decision: 		The document was revised to R5-241825.

R5-241825	Correction to SSB configuration in 16.6.2.11 and 16.6.2.12

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3001  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241127)

Decision: 		The document was withdrawn.

R5-241209	Corrections to NR SA FR1 RedCap Event Trigger TCs 16.6.2.9, 16.6.2.10

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3011  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

Depending on RAN4 CR R4-2400673

Discussion:

merged into R5-240694r1

Decision: 		The document was withdrawn.

R5-241211	Annex E.14 correction for new RedCap test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3012  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241212	Annex F correction for RedCap test cases including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3013  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

Changes will be included in R5-240732r1.

Decision: 		The document was withdrawn.

R5-241213	Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 1 Rx UE test case 16.7.1.1.1 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3014  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241214	Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE test case 16.7.1.1.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3015  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241215	Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.1 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3016  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241216	Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE test case 16.7.1.2.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3017  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241217	Addition of NR SA FR1 SS-RSRQ measurement accuracy for 1 Rx UE test case 16.7.2.1 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3018  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400659

Decision: 		The document was agreed.

R5-241218	Addition of NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE test case 16.7.2.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3019  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400659

Decision: 		The document was agreed.

R5-241219	Addition of RRM RedCap test cases 16.7.1.3.1, 16.7.1.3.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3020  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241220	Addition of RRM RedCap test cases 16.7.1.4.1, 16.7.1.4.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3021  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241227	Editorial Correction to RedCap PRACH TC 16.3.2.2.8

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3022  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

editorial

Decision: 		The document was agreed.

R5-241228	Editorial corrections for RedCap tets cases 18.3.1.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3023  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

editorial

Decision: 		The document was agreed.

R5-241229	Core Spec alignment for RedCap tets cases 18.3.1.x

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0691  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was withdrawn.

R5-241264	Core Spec alignment for RedCap tets cases 18.3.1.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3035  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241296	Addition of RedCap SA FR2 SSB RLM OOS in DRX test case 17.5.1.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3057  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241297	Addition of RedCap SA FR2 SSB BFR test cases 17.5.2.1 and 17.5.2.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3058  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241298	Update to RRM test 16.3.1.8 including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3059  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241299	Correction to stationary relaxed measurement criterion tests

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3060  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241300	Update to Re-establishment test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3061  Cat: F (Rel-18)

					Source: Qualcomm Germany

Abstract:

RAN4 dependency R4-2400726 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-241301	Applcability update for several RedCap tests

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3062  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

##### 5.3.8.8	TR 38.903 (NR MU & TT analyses)

R5-240733	TT analysis for RedCap RRM test case 16.2.1.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0656  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401306

Discussion:

for email agreement

Power level changes in R4-2401306 are not agreed in RAN4.

Email withdrawn.

Decision: 		The document was withdrawn.

R5-240734	TT analysis for RedCap RRM test case 16.2.1.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0657  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401306

Discussion:

for email agreement

Power level changes for RedCap SDT in R4-2401306 are not agreed in RAN4.

r1 is needed to updated TT analysis based on the power level for Rel-17 SDT agreed in R4-2403525 and remove the analysis for RedCap SDT.

But after discussing with RRM TT reviewers all agreed that TT analysis for Rel-17 SDT tests needs to wait for RAN4’s further conclusions.

Email withdrawn.

Decision: 		The document was withdrawn.

R5-240735	TT analysis for RedCap RRM test case 16.3.2.2.2 and 16.3.2.2.4

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0658  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240736	TT analysis for RedCap RRM test case 16.3.2.2.6 and 16.3.2.2.8

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0659  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240737	TT analysis for RedCap RRM test case 16.6.5.3

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0660  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240738	TT analysis for RedCap RRM test case 16.6.5.4

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0661  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240739	TT analysis for RedCap RRM test case 16.7.2.3.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0662  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240740	TT analysis for RedCap RRM test case 16.7.2.3.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0663  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

r1

Decision: 		The document was revised to R5-242044.

R5-242044	TT analysis for RedCap RRM test case 16.7.2.3.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0663  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240740)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240741	TT analysis for RedCap RRM test case 16.7.2.4.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0664  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240742	TT analysis for RedCap RRM test case 16.7.2.4.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0665  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2401304 -> R4-2403401

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240743	TT analysis for RedCap RRM test case 16.7.3.1 and 16.7.3.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0666  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240744	TT analysis for RedCap RRM test case 16.7.3.3.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0667  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241887.

R5-241887	TT analysis for RedCap RRM test case 16.7.3.3.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0667  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240744)

Decision: 		The document was agreed.

R5-240745	TT analysis for RedCap RRM test case 16.7.3.3.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0668  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241888.

R5-241888	TT analysis for RedCap RRM test case 16.7.3.3.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0668  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240745)

Decision: 		The document was agreed.

R5-240746	TT analysis for RedCap RRM test case 16.7.3.4.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0669  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241889.

R5-241889	TT analysis for RedCap RRM test case 16.7.3.4.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0669  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240746)

Decision: 		The document was agreed.

R5-240747	TT analysis for RedCap RRM test case 16.7.3.4.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0670  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240748	TT analysis for RedCap RRM test case 16.7.4.1.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0671  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240749	TT analysis for RedCap RRM test case 16.7.4.1.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0672  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240750	TT analysis for RedCap RRM test case 16.7.4.2.1 16.7.4.3.1 and 16.7.4.4.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0673  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241890.

R5-241890	TT analysis for RedCap RRM test case 16.7.4.2.1 16.7.4.3.1 and 16.7.4.4.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0673  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240750)

Decision: 		The document was agreed.

R5-240751	TT analysis for RedCap RRM test case 16.7.4.2.2 16.7.4.3.2 and 16.7.4.4.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0674  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-241891.

R5-241891	TT analysis for RedCap RRM test case 16.7.4.2.2 16.7.4.3.2 and 16.7.4.4.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0674  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240751)

Decision: 		The document was agreed.

R5-240752	TT analysis for RedCap RRM test case 16.7.5.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0675  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240753	TT analysis for RedCap RRM test case 16.7.5.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0676  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240754	TT analysis for RedCap RRM test case 16.7.6.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0677  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240755	TT analysis for RedCap RRM test case 16.7.6.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0678  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240787	Addition of Test Tolerance for NR SA Event triggered reporting RedCap test cases

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0679  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240788	Test Tolerances for NR SA FR1 - E-UTRAN event-triggered reporting tests for 1 Rx RedCap UE

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0680  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240789	Test Tolerances for NR SA FR1 - E-UTRAN event-triggered reporting tests for 2 Rx RedCap UE

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0681  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

R4-2401942 revised to R4-2403337 agreed

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-241221	Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 1 Rx UE test case 16.7.1.1.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0685  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241222	Test Tolerance for NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE test case 16.7.1.1.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0686  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241223	Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0687  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241224	Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0688  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241225	Test Tolerance for NR SA FR1 SS-RSRQ measurement accuracy UE test case 16.7.2.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0689  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400659

Decision: 		The document was agreed.

R5-241226	Test Tolerance NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE test case 16.7.2.2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0690  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400659

Decision: 		The document was agreed.

R5-241303	Update of TT analysis for 6.3.1.4 and 16.3.1.8

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0698  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

+

r1

Decision: 		The document was revised to R5-241892.

R5-241892	Update of TT analysis for 6.3.1.4 and 16.3.1.8

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0698  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241303)

Decision: 		The document was agreed.

##### 5.3.8.9	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.8.10	Discussion Papers / Work Plan / TC lists

#### 5.3.9	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest

##### 5.3.9.1	TS 38.508-1

##### 5.3.9.2	TS 38.508-2

##### 5.3.9.3	TS 38.522

##### 5.3.9.4	TS 38.533

R5-240756	Correction to SDT RRM test case 6.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2953  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

R4-2401300 -> R4-2403525

Discussion:

for email agreement

r1 is needed in accordance of R4-2403525 and R5-240734r1, overlapping with QC’s CR R5-241295.

Decision: 		The document was revised to R5-241673.

R5-241673	Correction to SDT RRM test case 6.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2953  rev 1 Cat: F (Rel-18)

					Source: Huawei,HiSilicon

(Replaces R5-240756)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240757	Correction to Annex F for SDT RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2954  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

R4-2401300 -> R4-2403525

Discussion:

for email agreement

R5-240757r1 is needed to capture the updated TT analysis in R5-240756r1

But after discussing with RRM TT reviewers all agreed that TT analysis for Rel-17 SDT tests needs to wait for RAN4’s further conclusions.

Email withdrawn.

Decision: 		The document was withdrawn.

R5-241295	Cleanup on FR1 CG-SDT test 6.2.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3056  Cat: F (Rel-18)

					Source: Qualcomm Germany

Discussion:

overlap with Huawei’s CR R5-240756r1

for email agreement

is not dependent on R4-2402005.

Anritsu agreed to R5-241295

Email agreed

Decision: 		The document was agreed.

##### 5.3.9.5	TR 38.903 (NR MU & TT analyses)

##### 5.3.9.6	Discussion Papers / Work Plan / TC lists

#### 5.3.10	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest

##### 5.3.10.1	TS 38.508-1

R5-240839	Corrections on 4.6.2 and 4.6.3 for the conditions in SIB and RRC IE

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3073  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

r1

Decision: 		The document was revised to R5-241809.

R5-241809	Corrections on 4.6.2 and 4.6.3 for the conditions in SIB and RRC IE

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3073  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-240839)

Decision: 		The document was withdrawn.

R5-241355	Correction of test environment for NR NTN RF testing

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3093  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

r1

Decision: 		The document was revised to R5-242007.

R5-242007	Correction of test environment for NR NTN RF testing

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3093  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241355)

Decision: 		The document was agreed.

R5-241393	Updates to Test environment for NR NTN RRM testing

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3101  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

Abstract:

This CR depends on discussion paper R5-241392.

Discussion:

r1

Decision: 		The document was revised to R5-242016.

R5-242016	Updates to Test environment for NR NTN RRM testing

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3101  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

(Replaces R5-241393)

Decision: 		The document was agreed.

R5-241400	Common Test environment updates for NR NTN RF, demod and RRM testing

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3102  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

r1

Decision: 		The document was revised to R5-242017.

R5-242017	Common Test environment updates for NR NTN RF, demod and RRM testing

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3102  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241400)

Decision: 		The document was agreed.

##### 5.3.10.2	TS 38.508-2

R5-240399	Addition of NR NTN capabilities

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0580  Cat: F (Rel-18)

					Source: CAICT

Discussion:

r2

Decision: 		The document was revised to R5-241909.

R5-241909	Addition of NR NTN capabilities

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0580  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240399)

Decision: 		The document was agreed.

R5-241270	Addition of PICS to support NTN RRM

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0603  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

##### 5.3.10.3	TS 38.509

##### 5.3.10.4	TS 38.521-5

R5-240201	Addition and correction to the NTN related abbreviations in 38.521-5

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0002  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

RAN4#110 t-doc

R4-2401169

Discussion:

r1

Decision: 		The document was revised to R5-242031.

R5-242031	Addition and correction to the NTN related abbreviations in 38.521-5

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0002  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240201)

Discussion:

RAN4 did not progress

Decision: 		The document was withdrawn.

R5-240840	Corrections on 5.3.3 for minimum guardband and transmission bandwidth configuration

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0003  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240841	Corrections on 6.2.2 for maximum output power reduction

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0004  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240842	Corrections on 6.2.3 for additional maximum output power reduction

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0005  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

r2

Decision: 		The document was revised to R5-241805.

R5-241805	Corrections on 6.2.3 for additional maximum output power reduction

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0005  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-240842)

Decision: 		The document was agreed.

R5-240843	Corrections on 6.3.2 for Transmit OFF power

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0006  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240844	Corrections on 6.3.3 for Transmit ON OFF time mask

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0007  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

There is overlapping in clause 2 for the addition of references among R5-240844, R5-241148 and R5-241149.

r2

Decision: 		The document was revised to R5-241806.

R5-241806	Corrections on 6.3.3 for Transmit ON OFF time mask

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0007  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-240844)

Decision: 		The document was agreed.

R5-240845	Corrections on 6.4.1 for Frequency error

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0008  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Editorial Correction]

Discussion:

overlapping with R5-241395 on the spec number correction.

"[Editorial Correction]

ZTE: There is overlapping with R5-241395(KS) on the spec number correction. Need futher discussion.

Withdrawn content merged into 1361"

Decision: 		The document was withdrawn.

R5-240846	Corrections on 6.5.2 for out of band emission

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0009  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240847	Corrections on 6.5.3 for spurious emission

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0010  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

There is overlapping in clause 2 for the addition of references among R5-240844, R5-241148 and R5-241149.

r1

Decision: 		The document was revised to R5-241807.

R5-241807	Corrections on 6.5.3 for spurious emission

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0010  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-240847)

Decision: 		The document was agreed.

R5-240848	Corrections on 6.5.4 for transmit intermodulation

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0011  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

overlapping with R5-240959 on the initial conditions for the test.

ZTE: There is overlapping with R5-240959(QC) on the initial conditions for the test. Need further discussion to handle.

Decision: 		The document was withdrawn.

R5-240849	Corrections on 7.3.2 for Reference sensitivity power level

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0012  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

overlapping with R5-240960 on the TBD references.

r2

Decision: 		The document was revised to R5-241877.

R5-241877	Corrections on 7.3.2 for Reference sensitivity power level

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0012  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-240849)

Decision: 		The document was agreed.

R5-240850	Corrections on 7.4 for maximum input level

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0013  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240851	Corrections on 7.5 for adjacent channel selectivity

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0014  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Editorial Correction]

Decision: 		The document was agreed.

R5-240852	Corrections on 7.6 for blocking characteristics

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0015  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240853	Corrections on A.3.2.1.1 for the reference channel for NTN PDSCH requirement

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0016  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Core spec alignment]

Decision: 		The document was agreed.

R5-240854	Update of 6.2.1 for maximum output power

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0017  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240855	Update of chapter 4 for RF general description

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0018  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

There is overlapping with R5-240201 on the NTN abbreviation.

r1

Decision: 		The document was revised to R5-241810.

R5-241810	Update of chapter 4 for RF general description

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0018  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-240855)

Decision: 		The document was agreed.

R5-240955	Update to NTN Frequency Error TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0019  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Discussion:

"dependent on R5-240961

contents merged in R5-241361"

Decision: 		The document was withdrawn.

R5-240956	Update to NTN Add Spurious Emission TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0020  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Discussion:

r1

Decision: 		The document was revised to R5-242008.

R5-242008	Update to NTN Add Spurious Emission TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0020  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

(Replaces R5-240956)

Decision: 		The document was agreed.

R5-240957	Update to NTN General Spurious emission TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0021  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Discussion:

r1

Decision: 		The document was revised to R5-242009.

R5-242009	Update to NTN General Spurious emission TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0021  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

(Replaces R5-240957)

Decision: 		The document was agreed.

R5-240958	Updates to NTN Spur emission UE Coex

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0022  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Discussion:

r1

Decision: 		The document was revised to R5-242010.

R5-242010	Updates to NTN Spur emission UE Coex

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0022  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

(Replaces R5-240958)

Decision: 		The document was agreed.

R5-240959	Update to NTN Tx Intermod TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0023  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland, ZTE

Discussion:

r1

Decision: 		The document was revised to R5-242011.

R5-242011	Update to NTN Tx Intermod TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0023  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland, ZTE

(Replaces R5-240959)

Decision: 		The document was agreed.

R5-240960	Update to NTN RefSens TC

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0024  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Discussion:

"deferred to address overlap

merged into R5-240849"

Decision: 		The document was withdrawn.

R5-240961	Update to NTN Annex F MU TT

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0025  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Discussion:

r1

Decision: 		The document was revised to R5-242012.

R5-242012	Update to NTN Annex F MU TT

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0025  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

(Replaces R5-240961)

Decision: 		The document was agreed.

R5-241148	Update to PDSCH Mapping Type A test case for Satellite Access

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0026  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was agreed.

R5-241149	General updates to RF NTN clauses

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0027  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was agreed.

R5-241361	Splitting the NR NTN frequency error test case

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0028  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion paper in R5-241360, applicability update in CR R5-241362

Discussion:

cl. aff.

Empty!

-> late doc !

r5

Decision: 		The document was revised to R5-242013.

R5-242013	Splitting the NR NTN frequency error test case

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0028  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241361)

Decision: 		The document was agreed.

R5-241394	UL RMCs updates for NR NTN

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0029  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

This CR depends on RAN4 CR R4-24yyyyy.

Discussion:

r1

Decision: 		The document was revised to R5-242018.

R5-242018	UL RMCs updates for NR NTN

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0029  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241394)

Decision: 		The document was agreed.

R5-241395	Updates to NR NTN Frequency error test

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0030  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241808.

R5-241808	Updates to NR NTN Frequency error test

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0030  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241395)

Decision: 		The document was withdrawn.

R5-241396	Updates to NR NTN Minimum output power test

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0031  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

This CR depends on R5-24xxxx and on R5-24xxx2.

Decision: 		The document was agreed.

R5-241397	Updates to NR NTN Transmit OFF power test

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0032  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

This CR depends on R5-24xxxx.

Decision: 		The document was agreed.

R5-241398	Updates to NR NTN Maximum Input Level test

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0033  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

This CR depends on R5-24xxxx and on R5-24xxx2.

Decision: 		The document was agreed.

R5-241440	Updates to NTN TC 6.3.3 on Tx on-off time mask

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0034  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Decision: 		The document was agreed.

R5-241441	Updates to NTN TC 6.5.2.2 on Spectrum emission mask

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0035  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Decision: 		The document was agreed.

R5-241442	Updates to NTN TC 6.5.2.4  on ACLR

Type: CR		For: Agreement
					38.521-5 v18.0.0	  CR-0036  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Decision: 		The document was agreed.

##### 5.3.10.5	TS 38.522

R5-240400	Update of Applicability and Additional Information of RF conformance test cases for Satellite Access

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0374  Cat: F (Rel-18)

					Source: CAICT

Discussion:

Conflicts with R5-241444 and R5-241399.

r2

Decision: 		The document was revised to R5-241910.

R5-241910	Update of Applicability and Additional Information of RF conformance test cases for Satellite Access

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0374  rev 1 Cat: F (Rel-18)

					Source: CAICT, ROHDE & SCHWARZ, Keysight Technologies UK Ltd, Apple Benelux B.V., QUALCOMM Europe Inc. - Italy

(Replaces R5-240400)

Decision: 		The document was agreed.

R5-241362	Aligning the applicability for NR NTN frequency error

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0400  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion paper in R5-241360, test case update in CR R5-241361

Discussion:

cl. aff.

Empty!

-> late doc !

r1

Decision: 		The document was revised to R5-241906.

R5-241906	Aligning the applicability for NR NTN frequency error

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0400  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241362)

Discussion:

content merged into 0400r2(CAICT)"

Decision: 		The document was withdrawn.

R5-241380	Applicability spec update for NR NTN test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0402  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

r1?

content merged into 0400r2(CAICT)"

Decision: 		The document was withdrawn.

R5-241399	Applicability updates for NR NTN Minimum output power and maximum Input level tests

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0403  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

"KS: Further discussion required to solve the overlapping between R5-241399 and R5240400(CAICT)

content merged into 0400r2(CAICT)"

Decision: 		The document was withdrawn.

R5-241444	Applicability updates for NTN RF tests

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0405  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

"Apple: Condition in R5-241444 has to be updated depending on conclusion of R5-240400(CAICT)

content merged into 0400r2(CAICT)"

Decision: 		The document was withdrawn.

##### 5.3.10.6	TS 38.533

R5-240202	Addition and correction to the NTN related abbreviations in 38.533

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2878  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

RAN4#110 t-doc

R4-2401169

Discussion:

r1

Decision: 		The document was revised to R5-242032.

R5-242032	Addition and correction to the NTN related abbreviations in 38.533

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2878  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240202)

Discussion:

RAN4 did not progress

Decision: 		The document was withdrawn.

R5-241265	General update to NTN tests

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3036  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241266	Addition of NTN timing accuracy test 14.3.1.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3037  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241267	Addition of NTN time-based CHO tests 14.2.1.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3038  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241268	Addition of NTN intra-frequency cell reselection tests 14.1.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3039  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241269	Addition of NTN SS-SINR measurement accuracy tests 14.6.3.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3040  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

##### 5.3.10.7	TR 38.903 (NR MU & TT analyses)

R5-241443	MU TT updates for NTN RF tests

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0704  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Decision: 		The document was withdrawn.

##### 5.3.10.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-241401	Description of ephemeris calculation process

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0879  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

Discussion:

r1

Decision: 		The document was revised to R5-242014.

R5-242014	Description of ephemeris calculation process

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0879  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

(Replaces R5-241401)

Decision: 		The document was agreed.

##### 5.3.10.9	Discussion Papers / Work Plan / TC lists

R5-241360	On the splitting of the NR NTN Frequency Error test cases

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Abstract:

Test case update in CR R5-241361, applicability update in CR R5-241362

Discussion:

noted and proposal endorsed

Decision: 		The document was noted.

R5-241392	Ephemeris definition for NR NTN RRM neighbor cells

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, Thales

Discussion:

r2

Decision: 		The document was revised to R5-242001.

R5-242001	Ephemeris definition for NR NTN RRM neighbor cells

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, Thales

(Replaces R5-241392)

Discussion:

"Revised from: R5-241392r2.

CR implemented R5-241393

noted proposals 1-4 endorsed and implemented in cr's"

Decision: 		The document was noted.

R5-241413	Neighbor GSO satellite ephemeris file for NTN NR UE testing

Type: discussion		For: Information
					Source: THALES

Abstract:

This contribution provides neighbor GSO satellite ephemeris data and channel (e.g. Doppler, Delay) information. The serving GSO satellite ephemeris data has been already provided in R5-237213.

Discussion:

r2

Decision: 		The document was revised to R5-241792.

R5-241792	Neighbor GSO satellite ephemeris file for NTN NR UE testing

Type: discussion		For: Information
					Source: THALES

(Replaces R5-241413)

Discussion:

update ephemeris content

Decision: 		The document was noted.

#### 5.3.11	Further enhancement on NR demodulation performance (UID-960075) NR_demod_enh2-UEConTest

##### 5.3.11.1	TS 38.508-1

##### 5.3.11.2	TS 38.508-2

##### 5.3.11.3	TS 38.521-4

R5-240251	Addition of MMSE-IRC CQI reporting test case with FDD 4Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0792  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

date

r1

Decision: 		The document was revised to R5-241811.

R5-241811	Addition of MMSE-IRC CQI reporting test case with FDD 4Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0792  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240251)

Decision: 		The document was agreed.

R5-240252	Addition of MMSE-IRC CQI reporting test case with TDD 2Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0793  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

date

r1

Decision: 		The document was revised to R5-241812.

R5-241812	Addition of MMSE-IRC CQI reporting test case with TDD 2Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0793  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240252)

Decision: 		The document was agreed.

R5-240253	Addition of MMSE-IRC CQI reporting test case with TDD 4Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0794  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

date

r1

Decision: 		The document was revised to R5-241813.

R5-241813	Addition of MMSE-IRC CQI reporting test case with TDD 4Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0794  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240253)

Decision: 		The document was agreed.

R5-240254	Completion of MMSE-IRC CQI reporting test case with FDD 2Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0795  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

date

r1

Decision: 		The document was revised to R5-241814.

R5-241814	Completion of MMSE-IRC CQI reporting test case with FDD 2Rx

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0795  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240254)

Decision: 		The document was agreed.

###### 5.3.11.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-241151	Update to FR1 PDSCH inter-cell interference test case

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0816  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241815.

R5-241815	Update to FR1 PDSCH inter-cell interference test case

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0816  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241151)

Decision: 		The document was agreed.

R5-241152	Update to FR1 PDSCH intra-cell interference test case

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0817  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241816.

R5-241816	Update to FR1 PDSCH intra-cell interference test case

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0817  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241152)

Decision: 		The document was agreed.

###### 5.3.11.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.11.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.11.3.4	Clauses 1-4 / Annexes

R5-241146	Correction to RMC name in Table A.3.2.2.2-29

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0814  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

Decision: 		The document was agreed.

R5-241147	Correction to RMC name in Table A.3.2.1.1-18

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0815  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Abstract:

R4-2400636

Discussion:

late doc

correct WIC should be NR_demod_enh2-UEConTest.

r1

Decision: 		The document was revised to R5-242019.

R5-242019	Correction to RMC name in Table A.3.2.1.1-18

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0815  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241147)

Decision: 		The document was agreed.

##### 5.3.11.4	TS 38.522

R5-240255	Addition of MMSE-IRC CQI reporting test applicability rule

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0371  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

date

r1

Decision: 		The document was revised to R5-241845.

R5-241845	Addition of MMSE-IRC CQI reporting test applicability rule

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0371  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240255)

Decision: 		The document was agreed.

R5-241153	Applicability update for PDSCH interference test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0390  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241846.

R5-241846	Applicability update for PDSCH interference test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0390  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241153)

Decision: 		The document was agreed.

##### 5.3.11.5	TR 38.903 (NR MU & TT analyses)

R5-241150	TT analysis for FR1 PDSCH with inter-cell interference test cases

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0683  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Abstract:

resubmission of R5-237698

Decision: 		The document was agreed.

##### 5.3.11.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.11.7	Discussion Papers / Work Plan / TC lists

#### 5.3.12	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest

##### 5.3.12.1	TS 38.508-1

##### 5.3.12.2	TS 38.508-2

R5-240506	Addition of feMIMO physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0585  Cat: F (Rel-18)

					Source: Samsung

Discussion:

R5!

Author???

r3

Decision: 		The document was revised to R5-241958.

R5-241958	Addition of feMIMO physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0585  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-240506)

Discussion:

for email agreement

r3

Decision: 		The document was revised to R5-241674.

R5-241674	Addition of feMIMO physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0585  rev 2 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-241958)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240522	Update the existing PICS of inter-band CA between FR1 and FR2

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0586  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

##### 5.3.12.3	TS 38.521-4

###### 5.3.12.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-240507	Adding measurement uncertainty and test tolerance for Demod PDCCH with intra-slot repetition test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0796  Cat: F (Rel-18)

					Source: Samsung

Discussion:

Author!

AI!

r2

Decision: 		The document was revised to R5-241959.

R5-241959	Adding measurement uncertainty and test tolerance for Demod PDCCH with intra-slot repetition test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0796  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-240507)

Decision: 		The document was agreed.

###### 5.3.12.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.12.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.12.3.4	Clauses 1-4 / Annexes

##### 5.3.12.4	TS 38.522

R5-240505	Addition of applicability for FeMIMO test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0375  Cat: F (Rel-18)

					Source: Samsung

Discussion:

r3

Decision: 		The document was revised to R5-241847.

R5-241847	Addition of applicability for FeMIMO test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0375  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-240505)

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241675.

R5-241675	Addition of applicability for FeMIMO test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0375  rev 2 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-241847)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240520	Removal of NOTE 1 from applicability of FR2 feMIMO test case

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0377  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

##### 5.3.12.5	TS 38.533

##### 5.3.12.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.12.7	Discussion Papers / Work Plan / TC lists

#### 5.3.13	NR support for high speed train scenario in frequency range 2 (FR2) (UID-960080) NR_HST_FR2-UEConTest

##### 5.3.13.1	TS 38.508-1

##### 5.3.13.2	TS 38.508-2

##### 5.3.13.3	TS 38.521-2

###### 5.3.13.3.1	Tx Requirements (Clause 6)

###### 5.3.13.3.2	Rx Requirements (Clause 7)

###### 5.3.13.3.3	Clauses 1-5 / Annexes

##### 5.3.13.4	TS 38.521-4

###### 5.3.13.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.13.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.13.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.13.4.4	Clauses 1-4 / Annexes

##### 5.3.13.5	TS 38.522

R5-240508	Addition of applicability for HST FR2 test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0376  Cat: F (Rel-18)

					Source: Samsung

Decision: 		The document was agreed.

##### 5.3.13.6	TS 38.533

##### 5.3.13.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.13.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.13.9	Discussion Papers / Work Plan / TC lists

#### 5.3.14	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest

##### 5.3.14.1	TS 38.508-1

R5-240346	Update of RRC message RRCReconfigurationComplete, RRCSetupComplete and SecurityModeComplete for SL relay

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3025  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

realy

r1

Decision: 		The document was revised to R5-241722.

R5-241722	Update of RRC message RRCReconfigurationComplete, RRCSetupComplete and SecurityModeComplete for SL relay

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3025  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240346)

Decision: 		The document was agreed.

R5-240347	Update of Test procedure for establishing unicast mode ProSe Direct communication

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3026  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

r1

title '.'

r3

Decision: 		The document was revised to R5-241723.

R5-241723	Update of Test procedure for establishing unicast mode ProSe Direct communication

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3026  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240347)

Decision: 		The document was agreed.

##### 5.3.14.2	TS 38.508-2

R5-240348	Addition of Sidelink Capabilities to support direct to indirect path switch for NR sidelink U2N Relay

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0577  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-241724.

R5-241724	Addition of Sidelink Capabilities to support direct to indirect path switch for NR sidelink U2N Relay

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0577  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240348)

Decision: 		The document was agreed.

##### 5.3.14.3	TS 38.521-1

###### 5.3.14.3.1	Tx Requirements (Clause 6)

###### 5.3.14.3.2	Rx Requirements (Clause 7)

###### 5.3.14.3.3	Clauses 1-5 / Annexes

##### 5.3.14.4	TS 38.522

##### 5.3.14.5	TS 38.533

##### 5.3.14.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.14.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.14.8	Discussion Papers / Work Plan / TC lists

#### 5.3.15	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest

##### 5.3.15.1	TS 38.508-1

##### 5.3.15.2	TS 38.508-2

##### 5.3.15.3	TS 38.521-1

###### 5.3.15.3.1	Tx Requirements (Clause 6)

###### 5.3.15.3.2	Rx Requirements (Clause 7)

###### 5.3.15.3.3	Clauses 1-5 / Annexes

##### 5.3.15.4	TS 38.522

##### 5.3.15.5	TS 38.533

##### 5.3.15.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.15.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.15.8	Discussion Papers / Work Plan / TC lists

#### 5.3.16	UE power saving enhancements for NR (UID-960086) NR_UE_pow_sav_enh_plus_CT-UEConTest

##### 5.3.16.1	TS 38.508-1

##### 5.3.16.2	TS 38.508-2

##### 5.3.16.3	TS 38.522

R5-240889	Update to RRM Power saving enhancement 5.5.5.9 test case applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0384  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-241850.

R5-241850	Update to RRM Power saving enhancement 5.5.5.9 test case applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0384  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-240889)

Decision: 		The document was agreed.

##### 5.3.16.4	TS 38.533

R5-240890	Removal of Editor Note affecting to TC 5.5.5.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2985  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was revised to R5-241826.

R5-241826	Removal of Editor Note affecting to TC 5.5.5.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2985  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-240890)

Decision: 		The document was agreed.

R5-241123	Correction to PDCCH level in 6.5.1.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2997  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400291

Discussion:

r1

Decision: 		The document was revised to R5-241967.

R5-241967	Correction to PDCCH level in 6.5.1.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2997  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241123)

Decision: 		The document was agreed.

##### 5.3.16.5	TR 38.903 (NR MU & TT analyses)

##### 5.3.16.6	Discussion Papers / Work Plan / TC lists

#### 5.3.17	NR RRM enhancement (UID-960089) NR_RRM_enh-UEConTest

##### 5.3.17.1	TS 38.508-1

##### 5.3.17.2	TS 38.508-2

##### 5.3.17.3	TS 38.522

##### 5.3.17.4	TS 38.533

R5-240700	Correction to RRM enh test case 6.5.8.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2920  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240701	Correction to RRM enh test case 6.6.2.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2921  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Discussion:

r2

Decision: 		The document was revised to R5-241829.

R5-241829	Correction to RRM enh test case 6.6.2.9

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2921  rev 1 Cat: F (Rel-18)

					Source: Huawei,HiSilicon

(Replaces R5-240701)

Decision: 		The document was agreed.

R5-241197	Addition of new test case 4.7.8.1 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3007  Cat: F (Rel-18)

					Source: Sporton

Discussion:

r1

Decision: 		The document was revised to R5-241830.

R5-241830	Addition of new test case 4.7.8.1 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3007  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-241197)

Decision: 		The document was agreed.

R5-241198	Addition of new test case 4.7.8.2 EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3008  Cat: F (Rel-18)

					Source: Sporton

Decision: 		The document was agreed.

R5-241311	Addition of Clause 4.7.9.0 minimum conformance requirements for CSI-RSRQ

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3064  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241312	Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3065  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241313	Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3066  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241314	Addition of Clause 4.7.10.0 minimum conformance requirements for CSI-SINR

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3067  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241315	Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.10.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3068  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241316	Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell l test case 4.7.10.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3069  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241317	Addition of Clause 5.7.8.0 minimum conformance requirements for CSI-RSRQ

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3070  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241318	Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 5.7.8.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3071  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241319	Addition of EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.8.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3072  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241320	Addition of Clause 5.7.9.0 minimum conformance requirements for CSI-SINR

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3073  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241321	Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3074  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241322	Addition of EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3075  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241323	Addition of Clause 6.7.11.0 minimum conformance requirements for CSI-RSRQ

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3076  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241324	Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3077  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241325	Addition of SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3078  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241326	Addition of Clause 6.7.12.0 minimum conformance requirements for CSI-SINR

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3079  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241327	Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3080  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241328	Addition of SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3081  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241329	Addition of Clause 7.7.8.0 minimum conformance requirements for CSI-RSRQ

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3082  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241330	Addition of SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.8.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3083  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241331	Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.8.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3084  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241332	Addition of Clause 7.7.9.0 minimum conformance requirements for CSI-SINR

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3085  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241333	Addition of SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.9.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3086  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241334	Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.9.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3087  Cat: F (Rel-18)

					Source: Apple

Discussion:

typo

r1

Decision: 		The document was revised to R5-241831.

R5-241831	Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.9.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3087  rev 1 Cat: F (Rel-18)

					Source: Apple

(Replaces R5-241334)

Decision: 		The document was agreed.

R5-241335	Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex E

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3088  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241336	Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3089  Cat: F (Rel-18)

					Source: Apple

Discussion:

r1

Decision: 		The document was revised to R5-241893.

R5-241893	Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3089  rev 1 Cat: F (Rel-18)

					Source: Apple

(Replaces R5-241336)

Decision: 		The document was agreed.

##### 5.3.17.5	TR 38.903 (NR MU & TT analyses)

R5-241337	Addition of TT analysis grouping for test cases 4.5.9.1 to Table 8-1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0699  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-241338	Addition of TT analysis grouping for test cases 5.5.6.3.1 to Table 8-2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0700  Cat: F (Rel-18)

					Source: Apple

Discussion:

r1

Decision: 		The document was revised to R5-241894.

R5-241894	Addition of TT analysis grouping for test cases 5.5.6.3.1 to Table 8-2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0700  rev 1 Cat: F (Rel-18)

					Source: Apple

(Replaces R5-241338)

Decision: 		The document was agreed.

R5-241339	Addition of TT analysis grouping for test cases 5.5.6.4.1 to Table 8-2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0701  Cat: F (Rel-18)

					Source: Apple

Discussion:

r1

Decision: 		The document was revised to R5-241895.

R5-241895	Addition of TT analysis grouping for test cases 5.5.6.4.1 to Table 8-2

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0701  rev 1 Cat: F (Rel-18)

					Source: Apple

(Replaces R5-241339)

Decision: 		The document was agreed.

##### 5.3.17.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.17.7	Discussion Papers / Work Plan / TC lists

#### 5.3.18	RF requirements enhancement for NR frequency range 1 (FR1) (UID-960090) NR_RF_FR1_enh-UEConTest

##### 5.3.18.1	TS 38.508-1

##### 5.3.18.2	TS 38.508-2

R5-241029	Update to PICS for R17 FR1 enhancement

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0599  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.18.3	TS 38.521-1

###### 5.3.18.3.1	Tx Requirements (Clause 6)

R5-241009	Addition of 6.2H.1.3 AMPR for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2682  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241970.

R5-241970	Addition of 6.2H.1.3 AMPR for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2682  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241009)

Decision: 		The document was agreed.

R5-241010	Addition of 6.3H.1.4 power control for CA with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2683  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Test cases of power control for UL-MIMO are leveraged. No test details are specified.

Decision: 		The document was agreed.

R5-241011	Addition of 6.4H.1.1 frequency error for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2684  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241971.

R5-241971	Addition of 6.4H.1.1 frequency error for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2684  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241011)

Decision: 		The document was agreed.

R5-241012	Addition of 6.4H.1.2.1 EVM for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2685  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r3

Decision: 		The document was revised to R5-241972.

R5-241972	Addition of 6.4H.1.2.1 EVM for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2685  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241012)

Decision: 		The document was agreed.

R5-241013	Addition of 6.4H.1.2.2 Carrier leakage for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2686  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241973.

R5-241973	Addition of 6.4H.1.2.2 Carrier leakage for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2686  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241013)

Decision: 		The document was agreed.

R5-241014	Addition of 6.4H.1.2.3 In-band emission for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2687  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241974.

R5-241974	Addition of 6.4H.1.2.3 In-band emission for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2687  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241014)

Decision: 		The document was agreed.

R5-241015	Addition of 6.4H.1.3 time alignment error for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2688  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241008

Discussion:

r1

Decision: 		The document was revised to R5-241975.

R5-241975	Addition of 6.4H.1.3 time alignment error for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2688  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241015)

Decision: 		The document was agreed.

R5-241016	Addition of 6.4H.1.4 Coherent requirement for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2689  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241976.

R5-241976	Addition of 6.4H.1.4 Coherent requirement for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2689  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241016)

Decision: 		The document was agreed.

R5-241017	Addition of 6.5H.1.1 Occupied bandwidth for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2690  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r2

Decision: 		The document was revised to R5-241977.

R5-241977	Addition of 6.5H.1.1 Occupied bandwidth for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2690  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241017)

Decision: 		The document was agreed.

R5-241018	Addition of 6.5H.1.2.2 ASEM for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2691  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241978.

R5-241978	Addition of 6.5H.1.2.2 ASEM for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2691  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241018)

Decision: 		The document was agreed.

R5-241019	Addition of 6.5H.1.3.1 General spurious emission for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2692  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241979.

R5-241979	Addition of 6.5H.1.3.1 General spurious emission for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2692  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241019)

Decision: 		The document was agreed.

R5-241020	Addition of 6.5H.1.3.2 Spurious emission UE co-existence for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2693  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r2

Decision: 		The document was revised to R5-241980.

R5-241980	Addition of 6.5H.1.3.2 Spurious emission UE co-existence for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2693  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241020)

Decision: 		The document was agreed.

R5-241021	Addition of 6.5H.1.3.3 Additional spurious emission for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2694  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241981.

R5-241981	Addition of 6.5H.1.3.3 Additional spurious emission for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2694  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241021)

Decision: 		The document was agreed.

R5-241022	Addition of 6.5H.1.4 Transmit intermodulation for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2695  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

General principle of TP selection for CA+UL-MIMO applies.

Discussion:

r1

Decision: 		The document was revised to R5-241982.

R5-241982	Addition of 6.5H.1.4 Transmit intermodulation for CA with UL-MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2695  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241022)

Decision: 		The document was agreed.

R5-241023	Update to minimum requirement of 6.4D.4 for UL switching

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2696  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 alignment

Decision: 		The document was agreed.

R5-241024	Addition of 6.3A.3.4 1Tx-2Tx UL switching between two bands

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2697  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241008

Discussion:

r1

Decision: 		The document was revised to R5-241983.

R5-241983	Addition of 6.3A.3.4 1Tx-2Tx UL switching between two bands

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2697  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241024)

Decision: 		The document was agreed.

R5-241025	Addition of 6.3A.3.5 2Tx-2Tx UL switching between two bands

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2698  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241008

Discussion:

r1

Decision: 		The document was revised to R5-241984.

R5-241984	Addition of 6.3A.3.5 2Tx-2Tx UL switching between two bands

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2698  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241025)

Decision: 		The document was agreed.

R5-241026	Addition of 6.3C.3.3 2Tx-2Tx UL switching between two uplink carriers in SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2699  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241008

Discussion:

r2

Decision: 		The document was revised to R5-241985.

R5-241985	Addition of 6.3C.3.3 2Tx-2Tx UL switching between two uplink carriers in SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2699  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241026)

Decision: 		The document was agreed.

R5-241027	Addition of 6.3C.3.4 1Tx-2Tx UL switching between two bands in SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2700  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241008

Discussion:

r2

Decision: 		The document was revised to R5-241986.

R5-241986	Addition of 6.3C.3.4 1Tx-2Tx UL switching between two bands in SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2700  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241027)

Decision: 		The document was agreed.

R5-241028	Addition of 6.3C.3.5 2Tx-2Tx UL switching between two bands in SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2701  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-241008

Discussion:

r2

Decision: 		The document was revised to R5-241987.

R5-241987	Addition of 6.3C.3.5 2Tx-2Tx UL switching between two bands in SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2701  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241028)

Decision: 		The document was agreed.

###### 5.3.18.3.2	Rx Requirements (Clause 7)

###### 5.3.18.3.3	Clauses 1-5 / Annexes

##### 5.3.18.4	TS 38.522

R5-240368	Applicability statement for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0373  Cat: F (Rel-18)

					Source: China Telecom

Decision: 		The document was agreed.

R5-241030	Update to test applicability for R17 FR1 enhancement

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0387  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

Tdoc#

r1

Decision: 		The document was revised to R5-241988.

R5-241988	Update to test applicability for R17 FR1 enhancement

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0387  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241030)

Decision: 		The document was agreed.

##### 5.3.18.5	TS 38.533

R5-240369	Addition of NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2884  Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240370	Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2885  Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241896.

R5-241896	Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2885  rev 1 Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

(Replaces R5-240370)

Decision: 		The document was agreed.

R5-240371	Cell configuration mapping for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2886  Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241832.

R5-241832	Cell configuration mapping for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2886  rev 1 Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

(Replaces R5-240371)

Decision: 		The document was agreed.

R5-240372	Test tolerances and measurement uncertainty for DL interruptions test cases 6.5.7A.1, 6.5.7B.1 and 6.5.7C.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2887  Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.18.6	TR 38.903 (NR MU & TT analyses)

R5-240373	Test Tolerances for NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0648  Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240374	Test Tolerances for NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0649  Cat: F (Rel-18)

					Source: China Telecom, Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.18.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-241008	Addition of test point analysis for R17 FR1 enhancement test cases

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0872  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-241009~R5-241028

Decision: 		The document was agreed.

##### 5.3.18.8	Discussion Papers / Work Plan / TC lists

#### 5.3.19	Further enhancements of NR RF requirements for frequency range 2 (FR2) (UID-970070) NR_RF_FR2_req_enh2-UEConTest

##### 5.3.19.1	TS 38.508-1

##### 5.3.19.2	TS 38.508-2

##### 5.3.19.3	TS 38.521-2

###### 5.3.19.3.1	Tx Requirements (Clause 6)

R5-241406	Updates to UE Maximum Output Power - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1034  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

Abstract:

This CR is dependent on RAN5 discussion paper R5-241403 and CRs R5-241404 and R5-241405.

Discussion:

-

r2

Decision: 		The document was revised to R5-242025.

R5-242025	Updates to UE Maximum Output Power - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1034  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

(Replaces R5-241406)

Decision: 		The document was agreed.

R5-241429	Update to FR2 Tx Power test with UL-Gaps

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1035  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Abstract:

Core Spec Alignnment TS 38.101-2  V18.4.0

Discussion:

inputs from R&S and Huawei

r1

Decision: 		The document was revised to R5-241966.

R5-241966	Update to FR2 Tx Power test with UL-Gaps

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1035  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241429)

Decision: 		The document was agreed.

R5-241430	Update to FR2 Tx OFF Power test specific to UL-Gaps

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1036  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Decision: 		The document was agreed.

R5-241448	Addition of CA test for EIRP test with ULGaps

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1040  Cat: F (Rel-18)

					Source: Apple Trading

Abstract:

R5-241448

Decision: 		The document was revised to R5-241782.

R5-241782	Addition of CA test for EIRP test with ULGaps

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1040  rev 1 Cat: F (Rel-18)

					Source: Apple Trading

(Replaces R5-241448)

Decision: 		The document was agreed.

###### 5.3.19.3.2	Rx Requirements (Clause 7)

R5-241431	Updates to FR2 ACS test

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1037  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Abstract:

Core Spec Alignment 38.101-2 V18.4.0

Discussion:

late doc

Decision: 		The document was revised to R5-241780.

R5-241780	Updates to FR2 ACS test

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1037  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-241431)

Decision: 		The document was agreed.

###### 5.3.19.3.3	Clauses 1-5 / Annexes

R5-241405	Updates to Annex F for UE Maximum Output Power - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1033  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

Abstract:

This CR is dependent on RAN5 discussion paper R5-241403 and CR R5-241404.

Discussion:

-

r2

Decision: 		The document was revised to R5-242026.

R5-242026	Updates to Annex F for UE Maximum Output Power - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1033  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

(Replaces R5-241405)

Decision: 		The document was agreed.

##### 5.3.19.4	TS 38.521-3

###### 5.3.19.4.1	Tx Requirements (Clause 6)

R5-241408	Updates to 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1755  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

Abstract:

This CR is dependent on RAN5 CR  R5-241407.

Discussion:

had wrong ver in 3GU

r2

Decision: 		The document was revised to R5-242027.

R5-242027	Updates to 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1755  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

(Replaces R5-241408)

Decision: 		The document was agreed.

###### 5.3.19.4.2	Rx Requirements (Clause 7)

###### 5.3.19.4.3	Clauses 1-5 / Annexes

R5-241407	Updates to Annex F for 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1754  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

Abstract:

This CR is dependent on RAN5 discussion paper R5-241403 and CR R5-241404.

Discussion:

had wrong ver in 3GU

r1

Decision: 		The document was revised to R5-242028.

R5-242028	Updates to Annex F for 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1754  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

(Replaces R5-241407)

Decision: 		The document was agreed.

##### 5.3.19.5	TS 38.522

##### 5.3.19.6	TS 38.533

R5-241428	Addition of gap pattern configs

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3090  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Abstract:

Core Spec Alignnment TS 38.133 V18.4.0

Decision: 		The document was withdrawn.

##### 5.3.19.7	TR 38.903 (NR MU & TT analyses)

R5-241404	Measurement uncertainty definition for UE Maximum Output Power - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0703  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

Abstract:

This CR is dependent on RAN5 discussion paper R5-241403.

Discussion:

-

r2

Decision: 		The document was revised to R5-242029.

R5-242029	Measurement uncertainty definition for UE Maximum Output Power - EIRP with UL Gaps test case

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0703  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple Inc

(Replaces R5-241404)

Decision: 		The document was agreed.

##### 5.3.19.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.19.9	Discussion Papers / Work Plan / TC lists

R5-240944	Time synchronization required for UE beamlock function during initial access

Type: discussion		For: Endorsement
					Source: Apple AB Denmark

Abstract:

To support beam correspondence testing during initial access in RAN5, UE beam lock function is required.

Decision: 		The document was noted.

R5-241402	Initial discussions for Rel-18 beam correspondence for initial access

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd

Decision: 		The document was noted.

R5-241403	FR2 UL Gaps measurement uncertainties and test tolerances

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, Apple Inc

Discussion:

r3

Decision: 		The document was revised to R5-242024.

R5-242024	FR2 UL Gaps measurement uncertainties and test tolerances

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, Apple Inc

(Replaces R5-241403)

Discussion:

"Revised from: R5-241403r3.

TC implemented CRs R5-241404 to 1408

noted proposals1a/b/2a/b/3 are endorsed. Update(add pc3) prop2a  in the final "

Decision: 		The document was noted.

#### 5.3.20	NR Positioning Enhancements  (UID-970075) NR_pos_enh-UEConTest

##### 5.3.20.1	TS 38.508-1

###### 5.3.20.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.20.1.2	Test environment for RF (Clauses 5)

###### 5.3.20.1.3	Test environment for RRM (Clause 7)

###### 5.3.20.1.4	Other clauses / Annexes

##### 5.3.20.2	TS 37.571-1

R5-240101	Update TC 15.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0448  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-241897.

R5-241897	Update TC 15.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0448  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-240101)

Decision: 		The document was agreed.

R5-240102	Update TC 15.2.9 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0449  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-241898.

R5-241898	Update TC 15.2.9 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0449  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-240102)

Decision: 		The document was agreed.

R5-240103	Update TC 15.2.10 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0450  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240104	Update TC 16.2.7 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0451  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240105	Update TC 16.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0452  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240106	Update TC 16.3.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0453  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-241899.

R5-241899	Update TC 16.3.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0453  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-240106)

Decision: 		The document was agreed.

R5-240107	Addition of NR PRS-based measurement requirements in annex C to include TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0454  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240108	Addition of new RRC_INACTIVE NR RSTD reporting delay test case 14.4.3

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0455  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240109	Addition of new NR RSTD reporting delay test case 14.4.4 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0456  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240110	Addition of new NR RSTD accuracy test case 14.5.3 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0457  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240111	Addition of new NR RSTD accuracy test case 14.5.4 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0458  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-241911.

R5-241911	Addition of new NR RSTD accuracy test case 14.5.4 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0458  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-240111)

Decision: 		The document was agreed.

R5-240112	Addition of new NR PRS-RSRP reporting delay test case 16.4.3 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0459  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240113	Addition of new NR PRS-RSRP reporting delay test case 16.4.4 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0460  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240114	Addition of new NR PRS-RSRP accuracy test case 16.5.3 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0461  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240115	Addition of new NR PRS-RSRP accuracy test case 16.5.4 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0462  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240116	Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.1

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0463  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240117	Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.2

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0464  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240118	Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.3

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0465  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240119	Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.4

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0466  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240120	Addition of measurement period requirements in RRC_INACTIVE state for NR positioning methods

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0467  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-240121	Correction to PRS-RSRPP positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0468  Cat: F (Rel-17)

					Source: CATT, Spirent

Discussion:

title typo

r1

Decision: 		The document was revised to R5-241912.

R5-241912	Correction to PRS-RSRPP positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0468  rev 1 Cat: F (Rel-17)

					Source: CATT, Spirent

(Replaces R5-240121)

Decision: 		The document was agreed.

R5-240122	Correction to PRS-RSRP positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0469  Cat: F (Rel-17)

					Source: CATT, Spirent

Discussion:

title typo

r1

Decision: 		The document was revised to R5-241913.

R5-241913	Correction to PRS-RSRP positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0469  rev 1 Cat: F (Rel-17)

					Source: CATT, Spirent

(Replaces R5-240122)

Decision: 		The document was agreed.

R5-240123	Correction to NR RSTD positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0470  Cat: F (Rel-17)

					Source: CATT, Spirent

Discussion:

title typo

r1

Decision: 		The document was revised to R5-241914.

R5-241914	Correction to NR RSTD positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0470  rev 1 Cat: F (Rel-17)

					Source: CATT, Spirent

(Replaces R5-240123)

Decision: 		The document was agreed.

R5-240124	Correction to NR UE Rx-Tx time difference positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0471  Cat: F (Rel-17)

					Source: CATT, Spirent

Discussion:

title typo

r1

Decision: 		The document was revised to R5-241915.

R5-241915	Correction to NR UE Rx-Tx time difference positioning test cases

Type: CR		For: Agreement
					37.571-1 v17.3.1	  CR-0471  rev 1 Cat: F (Rel-17)

					Source: CATT, Spirent

(Replaces R5-240124)

Decision: 		The document was agreed.

##### 5.3.20.3	TS 37.571-3

R5-240125	Addition of test applicabilities for Release-17 PRS-RSRP, PRS-RSRPP and RSTD test cases

Type: CR		For: Agreement
					37.571-3 v17.3.0	  CR-0167  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

##### 5.3.20.4	TS 37.571-5

R5-240127	Introduction of BDS B2a and B3I performance default test conditions in TS 37.571-5

Type: CR		For: Agreement
					37.571-5 v17.3.0	  CR-0228  Cat: F (Rel-17)

					Source: CATT, CAICT

Discussion:

date

r1

Decision: 		The document was revised to R5-241916.

R5-241916	Introduction of BDS B2a and B3I performance default test conditions in TS 37.571-5

Type: CR		For: Agreement
					37.571-5 v17.3.0	  CR-0228  rev 1 Cat: F (Rel-17)

					Source: CATT, CAICT

(Replaces R5-240127)

Decision: 		The document was agreed.

R5-240128	Corrected CellIdentity for DL-TDOA measurement period test

Type: CR		For: Agreement
					37.571-5 v17.3.0	  CR-0229  Cat: F (Rel-17)

					Source: CATT, Spirent

Discussion:

date

r1

Decision: 		The document was revised to R5-241917.

R5-241917	Corrected CellIdentity for DL-TDOA measurement period test

Type: CR		For: Agreement
					37.571-5 v17.3.0	  CR-0229  rev 1 Cat: F (Rel-17)

					Source: CATT, Spirent

(Replaces R5-240128)

Decision: 		The document was agreed.

##### 5.3.20.5	TR 38.903 (NR MU & TT analyses)

R5-240129	Addition of TT analysis for positioning test case 15.2.8

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0641  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-240130	Addition of TT analysis for positioning test case 15.2.9 and 15.2.10

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0642  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

filename: +

Decision: 		The document was revised to R5-241900.

R5-241900	Addition of TT analysis for positioning test case 15.2.9 and 15.2.10

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0642  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-240130)

Decision: 		The document was agreed.

R5-240131	Addition of TT analysis for positioning test case 16.2.7

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0643  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-240132	Addition of TT analysis for positioning test case 16.2.8

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0644  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-240133	Addition of TT analysis for positioning test case 16.3.4

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0645  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

##### 5.3.20.6	Discussion Papers / Work Plan / TC lists

#### 5.3.21	Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink (UID-981033) DL_intrpt_combos_TxSW_R17-UEConTest

##### 5.3.21.1	TS 38.508-2

R5-240288	Addition of r16 and r17 UL tx switching and DL interruption supporting PICS for inter band CA with 3 bands

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0572  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

date

Decision: 		The document was withdrawn.

##### 5.3.21.2	TS 38.521-1

###### 5.3.21.2.1	Tx Requirements (Clause 6)

###### 5.3.21.2.2	Rx Requirements (Clause 7)

###### 5.3.21.2.3	Clauses 1-5 / Annexes

R5-240250	Addition of DL interruption allowed indication for CA_n1-n3-n78

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2613  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

date

r1

Decision: 		The document was revised to R5-241941.

R5-241941	Addition of DL interruption allowed indication for CA_n1-n3-n78

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2613  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240250)

Decision: 		The document was agreed.

##### 5.3.21.3	TS 38.522

##### 5.3.21.4	TS 38.533

##### 5.3.21.5	Discussion Papers / Work Plan / TC lists

#### 5.3.22	NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) (UID-981034) LTE_NBIOT_eMTC_NTN_req-UEConTest

##### 5.3.22.1	TS 36.508

R5-240075	Update to common requirements of test equipment for IoT-NTN

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1454  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

R5-240215	Addition of ephemeris information for GEO condition

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1457  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Discussion:

conflict with R5-241383.

we all agree the ephemeris should be different for each cell.

Decision: 		The document was withdrawn.

R5-241347	Editorial correction of test environment for IoT NTN RF testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1464  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Editorial

Discussion:

TF160 comments

r1

Decision: 		The document was revised to R5-241951.

R5-241951	Editorial correction of test environment for IoT NTN RF testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1464  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241347)

Decision: 		The document was agreed.

R5-241382	Updates to Test environment for eMTC NTN RRM testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1465  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

Abstract:

This CR depends on discussion paper R5-241381.

Discussion:

r3

Decision: 		The document was revised to R5-241952.

R5-241952	Updates to Test environment for eMTC NTN RRM testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1465  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

(Replaces R5-241382)

Decision: 		The document was agreed.

R5-241383	Updates to Test environment for NB-IoT NTN RRM testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1466  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

Abstract:

This CR depends on discussion paper R5-24yyyy.

Discussion:

TF160 GER and Anritsu comments

r2

Decision: 		The document was revised to R5-242002.

R5-242002	Updates to Test environment for NB-IoT NTN RRM testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1466  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

(Replaces R5-241383)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-241390	Common Test environment updates for IoT NTN RF, demod and RRM testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1467  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

TF160 GER comments.

R&S feedback.

r2

Decision: 		The document was revised to R5-241793.

R5-241793	Common Test environment updates for IoT NTN RF, demod and RRM testing

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1467  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241390)

Decision: 		The document was agreed.

##### 5.3.22.2	TS 36.509

##### 5.3.22.3	TS 36.521-2

R5-240073	Update to R18 IoT NTN test cases applicability

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1021  Cat: F (Rel-18)

					Source: CMCC, MediaTek, CAICT, Sporton, Keysight

Discussion:

r3

Decision: 		The document was revised to R5-241918.

R5-241918	Update to R18 IoT NTN test cases applicability

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1021  rev 1 Cat: F (Rel-18)

					Source: CMCC, MediaTek, CAICT, Sporton, Keysight

(Replaces R5-240073)

Decision: 		The document was agreed.

R5-240206	Update PICS of NTN test cases

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1022  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was withdrawn.

R5-241359	Aligning the applicability for IoT NTN frequency error

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1027  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion paper in R5-241357, test case update in CR R5-241358

Discussion:

cl. aff.

Empty!

->late doc !

r2

Decision: 		The document was revised to R5-241953.

R5-241953	Aligning the applicability for IoT NTN frequency error

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1027  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241359)

Decision: 		The document was agreed.

##### 5.3.22.4	TS 36.521-3

R5-240023	Editorial corrections for NB-IoT NTN TA cases and RLM cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-2997  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Discussion:

overlap with R5-241176, R5-240922

r1

Decision: 		The document was revised to R5-241794.

R5-241794	Editorial corrections for NB-IoT NTN TA cases and RLM cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-2997  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-240023)

Decision: 		The document was agreed.

R5-240024	Update NB-IoT NTN UL timing accuracy cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-2998  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Decision: 		The document was agreed.

R5-240029	Update NB-IoT NTN UL timing accuracy cases TT in Annex

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-2999  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

IoT NTN TT

Decision: 		The document was agreed.

R5-240030	Update GNSS margin value for NB-IoT NTN timing accuracy test cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3000  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

CoreSpecAlign, RAN4 dependency

Discussion:

overlap with R5-241250

the changes have already been 100% covered by R&S CR R5-241250.

Decision: 		The document was withdrawn.

R5-240035	Add a chapter title for ReEST (CH13.3.1)

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3001  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

cl. aff.

merge correction into R5-241247r1.

Decision: 		The document was withdrawn.

R5-240199	Correction of NB-IoT NTN cell reselection test cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3002  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

conflicts with R5-241244-8-9

Decision: 		The document was withdrawn.

R5-240200	Addition and correction to the NTN related abbreviations in 36.521-3

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3003  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

RAN4#110 t-doc

R4-2401021

Discussion:

r1

Decision: 		The document was revised to R5-242004.

R5-242004	Addition and correction to the NTN related abbreviations in 36.521-3

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3003  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240200)

Decision: 		The document was agreed.

R5-240210	Update ephemeris information of several RRM cases in 36.521-3

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3004  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was withdrawn.

R5-240211	Update TT value to message content and test requirement of RA cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3005  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was agreed.

R5-240214	Editorial removal to editors notes of DCQR cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3006  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc., Rohde & Schwarz

Abstract:

Editorial

Discussion:

D->F

r1

Decision: 		The document was revised to R5-241795.

R5-241795	Editorial removal to editors notes of DCQR cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3006  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc., Rohde & Schwarz

(Replaces R5-240214)

Decision: 		The document was agreed.

R5-240216	Update ephemeris to GEO condition test cases in 36.521-3

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3007  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was withdrawn.

R5-240219	Update cell specific parameters for NB-IoT NTN RA test cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3008  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

CoreSpecAlign

Decision: 		The document was agreed.

R5-240615	NTN NB-IoT - Message exception correction in RRM test case 13.1.1.2

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3009  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240922	Editorial update for UE timing advance for satellite access TC 13.4.2.2

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3010  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-241176	Correction to UE timing advance for satellite access TC 13.4.2.1

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3011  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

Depending on RAN4 CR R4-2400646

Discussion:

r2

Decision: 		The document was revised to R5-242003.

R5-242003	Correction to UE timing advance for satellite access TC 13.4.2.1

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3011  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241176)

Discussion:

for email agreement

RAN4 dependency was agreed in R4-2403453.

Email agreed

Decision: 		The document was agreed.

R5-241244	Core Spec alignment for s-IntraSearchP IE for Re-selection NB-IOT NTN RRM TCs

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3012  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400665

Decision: 		The document was agreed.

R5-241245	Re-Addition of omitted change for RRM test case 13.1.1.2

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3013  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241246	Editorial corrections to NB-IOT NTN RRM Reselection test cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3014  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

editorial

Decision: 		The document was agreed.

R5-241247	Editorial corrections to NB-IOT NTN RRM TCs

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3015  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

editorial

Discussion:

r1

Decision: 		The document was revised to R5-241796.

R5-241796	Editorial corrections to NB-IOT NTN RRM TCs

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3015  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241247)

Decision: 		The document was agreed.

R5-241248	Core Spec alignment for RRM NB-IOT NTN test cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3016  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400664

Decision: 		The document was agreed.

R5-241249	Core Spec alignment for test config for RRM NB-IOT NTN test cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3017  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400663

Discussion:

Conflicts resolved by withdrawn R5-240199(MTK)

RAN4 did not progress"

Decision: 		The document was withdrawn.

R5-241250	Core Spec alignment for RRM NB-IOT NTN UL Timing test cases

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3018  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on R4-2400666

Decision: 		The document was agreed.

R5-241385	Channel quality reporting updates

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3019  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

Core spec alignment.

Decision: 		The document was agreed.

R5-241386	Radio Link Monitoring updates

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3020  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-241389	Annex A.3 update

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3021  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

This CR depends on RAN4 CR R4-2402910 -> R4-243452 agreed

Discussion:

late doc

for email agreement

r1

Decision: 		The document was revised to R5-241676.

R5-241676	Annex A.3 update

Type: CR		For: Agreement
					36.521-3 v18.3.0	  CR-3021  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241389)

Discussion:

Email agreed

Decision: 		The document was agreed.

##### 5.3.22.5	TS 36.521-4

R5-240031	Editorial correction to the wrong table number in 36.521-4 annex C

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0001  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

editorial

Discussion:

D->F

r1

Decision: 		The document was revised to R5-241797.

R5-241797	Editorial correction to the wrong table number in 36.521-4 annex C

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0001  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240031)

Decision: 		The document was agreed.

R5-240072	Update of Annex F Measurement Uncertainties in TS 36.521-4

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0002  Cat: F (Rel-18)

					Source: CMCC, Keysight Technologies UK Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241798.

R5-241798	Update of Annex F Measurement Uncertainties in TS 36.521-4

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0002  rev 1 Cat: F (Rel-18)

					Source: CMCC, Keysight Technologies UK Ltd

(Replaces R5-240072)

Decision: 		The document was agreed.

R5-240074	Clear-up CR for Editor notes of applicability

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0003  Cat: F (Rel-18)

					Source: CMCC, MediaTek, CAICT, Sporton, Keysight

Decision: 		The document was revised to R5-241804.

R5-241804	Clear-up CR for Editor notes of applicability

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0003  rev 1 Cat: F (Rel-18)

					Source: CMCC, MediaTek, CAICT, Sporton, Keysight

(Replaces R5-240074)

Decision: 		The document was agreed.

R5-240097	Editorial correction to the wrong citation number

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0004  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

Editorial

Discussion:

D->F

r1

Decision: 		The document was revised to R5-241799.

R5-241799	Editorial correction to the wrong citation number

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0004  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240097)

Decision: 		The document was agreed.

R5-240098	Editorial alignment for the test applicability

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0005  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

Editorial

Discussion:

D->F

r1

Decision: 		The document was revised to R5-241800.

R5-241800	Editorial alignment for the test applicability

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0005  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240098)

Decision: 		The document was agreed.

R5-240099	Update of reference measurement channels in Annex A.3.12

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0006  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Discussion:

r1

added Keysight's comments

Decision: 		The document was revised to R5-241801.

R5-241801	Update of reference measurement channels in Annex A.3.12

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0006  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240099)

Decision: 		The document was agreed.

R5-240136	Editorial correction to 6.4B.1 Frequency Error for UE category NB1 and NB2 TC

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0007  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Decision: 		The document was withdrawn.

R5-240137	Editorial correction to Additional spurious emissions TCS

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0008  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial , RAN4

Discussion:

r1

Decision: 		The document was revised to R5-242020.

R5-242020	Editorial correction to Additional spurious emissions TCS

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0008  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-240137)

Decision: 		The document was agreed.

R5-240144	Update content of Statistical testing of Performance Requirements with probability of misdetection in Annex G.4

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0009  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was agreed.

R5-240145	Update TT value to NTN demod cases

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0010  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was agreed.

R5-240198	Addition and correction to the NTN related abbreviations in 36.521-4

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0011  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

RAN4#110 t-doc

R4-2400554

Discussion:

r1

Decision: 		The document was revised to R5-242005.

R5-242005	Addition and correction to the NTN related abbreviations in 36.521-4

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0011  rev 1 Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

(Replaces R5-240198)

Decision: 		The document was agreed.

R5-240862	Clarification on NPDSCH repetitions for Demod NB-IoT NTN test cases

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0012  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-241090	Correction to UL RMC descriptions for NB-IoT NTN

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0013  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was withdrawn.

R5-241358	Splitting the IoT NTN frequency error test case

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0014  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Keysight Technologies UK Ltd

Abstract:

Discussion paper in R5-241357, applicability update in CR R5-241359

Discussion:

cl. aff.

Empty!

-> late doc !

r3

Decision: 		The document was revised to R5-241954.

R5-241954	Splitting the IoT NTN frequency error test case

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0014  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Keysight Technologies UK Ltd

(Replaces R5-241358)

Decision: 		The document was agreed.

R5-241384	UL RMCs updates for IoT NTN

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0015  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

This CR depends on RAN4 CR R4-24yyyyy.

Discussion:

r1

Decision: 		The document was revised to R5-242006.

R5-242006	UL RMCs updates for IoT NTN

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0015  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241384)

Decision: 		The document was agreed.

R5-241387	Updates to PDSCH RMC

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0016  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-241388	Updates to IoT NTN Frequency error test

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0017  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241802.

R5-241802	Updates to IoT NTN Frequency error test

Type: CR		For: Agreement
					36.521-4 v18.0.0	  CR-0017  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241388)

Decision: 		The document was withdrawn.

##### 5.3.22.6	TR 36.903 (E-UTRAN RRM TT analyses)

R5-240025	Update NB-IoT NTN UL timing accuracy cases TT

Type: CR		For: Agreement
					36.903 v18.1.0	  CR-0460  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

IoT NTN TT

Decision: 		The document was agreed.

R5-240217	Addition of IoT NTN test cases grouping

Type: CR		For: Agreement
					36.903 v18.1.0	  CR-0461  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was agreed.

##### 5.3.22.7	TR 36.904 (E-UTRAN Radio Reception TT analyses)

R5-240076	Update of grouping of test cases defined in TS 36.521-4

Type: CR		For: Agreement
					36.904 v18.0.0	  CR-0066  Cat: F (Rel-18)

					Source: CMCC, MediaTek

Discussion:

date

r1

Decision: 		The document was revised to R5-241803.

R5-241803	Update of grouping of test cases defined in TS 36.521-4

Type: CR		For: Agreement
					36.904 v18.0.0	  CR-0066  rev 1 Cat: F (Rel-18)

					Source: CMCC, MediaTek

(Replaces R5-240076)

Decision: 		The document was agreed.

##### 5.3.22.8	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )

R5-241391	Description of ephemeris calculation process

Type: CR		For: Agreement
					36.905 v18.3.0	  CR-0272  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

Discussion:

r1

Decision: 		The document was revised to R5-241955.

R5-241955	Description of ephemeris calculation process

Type: CR		For: Agreement
					36.905 v18.3.0	  CR-0272  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Thales

(Replaces R5-241391)

Decision: 		The document was agreed.

##### 5.3.22.9	Discussion Papers / Work Plan / TC lists

R5-240070	Discussion on the closure of R18 IoT NTN UEConTest WI

Type: discussion		For: Endorsement
					Source: CMCC

Discussion:

r3

Decision: 		The document was revised to R5-241942.

R5-241942	Discussion on the closure of R18 IoT NTN UEConTest WI

Type: discussion		For: Endorsement
					Source: CMCC

(Replaces R5-240070)

Discussion:

"Revised from: R5-240070r3.

noted , prop1 endorsed (for skipping eMTc in the WP progress), ob5 needs further check before closing NBIoT WP"

Decision: 		The document was noted.

R5-241357	On the splitting of the IoT NTN Frequency Error test cases

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Abstract:

Test case update in CR R5-241358, applicability update in CR R5-241359

Discussion:

r1

Decision: 		The document was revised to R5-241791.

R5-241791	On the splitting of the IoT NTN Frequency Error test cases

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

(Replaces R5-241357)

Discussion:

prop endorsed

Decision: 		The document was noted.

R5-241381	Ephemeris definition for IoT NTN RRM neighbour cells

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, Thales

Discussion:

r2

Decision: 		The document was revised to R5-241943.

R5-241943	Ephemeris definition for IoT NTN RRM neighbour cells

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, Thales

(Replaces R5-241381)

Discussion:

"Revised from: R5-241381r2.

noted proposals endorsed and implemneted in CRs"

Decision: 		The document was noted.

#### 5.3.23	NR and MR-DC measurement gap enhancements (UID-981035) NR_MG_enh-UEConTest

##### 5.3.23.1	TS 38.508-1

##### 5.3.23.2	TS 38.508-2

##### 5.3.23.3	TS 38.522

##### 5.3.23.4	TS 38.533

R5-240147	Update of MG enhancements TC 6.6.18.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2876  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240148	Update of MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2877  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240634	Correction to FR2 SA event triggered reporting tests with Pre-MG including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2898  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Associated CR for TR 38.903: R5-240635

Corresponding RAN4 Tdoc is R4-2401584r1->R4-2403425 agreed

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241677.

R5-241677	Correction to FR2 SA event triggered reporting tests with Pre-MG including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2898  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-240634)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240636	Correction to FR2 SA event triggered reporting tests with concurrent gaps including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2899  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Associated CR for TR 38.903: R5-240637

Corresponding RAN4 Tdoc is R4-2401584r1->R4-2403425 agreed

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241678.

R5-241678	Correction to FR2 SA event triggered reporting tests with concurrent gaps including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2899  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-240636)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240638	Correction to FR2 SA event triggered reporting tests with NCSG including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2900  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Associated CR for TR 38.903: R5-240639

Corresponding RAN4 Tdoc is R4-2401584r1->R4-2403425 agreed

Discussion:

for email agreement

r2

Email agreed

Decision: 		The document was revised to R5-241679.

R5-241679	Correction to FR2 SA event triggered reporting tests with NCSG including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2900  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-240638)

Discussion:

Email agreed

Decision: 		The document was agreed.

##### 5.3.23.5	TR 38.903 (NR MU & TT analyses)

R5-240149	Addition of TT for MG enhancements TC 6.6.18.3

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0646  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240150	Addition of TT for MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0647  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240635	Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with Pre-MG

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0653  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Associated CR for TS 38.533: R5-240634

Corresponding RAN4 Tdoc is R4-2401584r1->R4-2403425 agreed

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241680.

R5-241680	Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with Pre-MG

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0653  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-240635)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240637	Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with concurrent gaps

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0654  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Associated CR for TS 38.533:  R5-240636.

Corresponding RAN4 Tdoc is R4-2401584r1->R4-2403425 agreed

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241681.

R5-241681	Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with concurrent gaps

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0654  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-240637)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240639	Introduction of Test Tolerance analysis for SA FR2 event triggered reporting tests with NCSG

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0655  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Associated CR for TS 38.533: R5-240638

Corresponding RAN4 Tdoc is R4-2401584r1->R4-2403425 agreed

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241682.

R5-241682	Introduction of Test Tolerance analysis for SA FR2 event triggered reporting tests with NCSG

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0655  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-240639)

Discussion:

Email agreed

Decision: 		The document was agreed.

##### 5.3.23.6	Discussion Papers / Work Plan / TC lists

#### 5.3.24	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest

##### 5.3.24.1	TS 38.508-1

R5-240378	Update of conditional PSCell addition configuration for NR-DC

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3028  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.24.2	TS 38.508-2

##### 5.3.24.3	TS 38.522

R5-240521	Removal of NOTE 1 from applicability of FR1 and FR2 MRDC test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2896  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

R5-240783	Removal of NOTE 1 from applicability of FR1 and FR2 MRDC test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0379  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

##### 5.3.24.4	TS 38.533

R5-240381	Update of test procedure in 4.5.10.1 PSCell activation for EN-DC

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2888  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240382	Correction to 4.5.11.1 EN-DC FR1 Conditional PSCell Addition

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2889  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240383	Update of measurement configuration for 5.5.13.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2890  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240384	Update of measurement configuration for 7.5.12.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2891  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240385	Update of message contents for 4.5.3.7 and 6.5.3.11

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2892  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.24.5	TS 36.508

R5-240973	Addition of conditional PSCell addition and change configuration for EN-DC

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1462  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241712.

R5-241712	Addition of conditional PSCell addition and change configuration for EN-DC

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1462  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240973)

Decision: 		The document was agreed.

R5-240974	Addition of SCG activation and deactivation configuration for EN-DC

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1463  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

comments from TF160

r1

Decision: 		The document was revised to R5-241713.

R5-241713	Addition of SCG activation and deactivation configuration for EN-DC

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1463  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240974)

Decision: 		The document was agreed.

##### 5.3.24.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.24.7	Discussion Papers / Work Plan / TC lists

#### 5.3.25	Additional NR bands for UL-MIMO in Rel-18 (UID-1000050) NR_bands_UL_MIMO_R18-UEConTest

##### 5.3.25.1	TS 38.508-1

##### 5.3.25.2	TS 38.508-2

R5-241262	Addition of n5 with UL MIMO capabilities

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0602  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was agreed.

##### 5.3.25.3	TS 38.521-1

###### 5.3.25.3.1	Tx Requirements (Clause 6)

R5-241261	Addition of n5 into TC 6.2D.1 MOP for UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2726  Cat: F (Rel-18)

					Source: China Unicom

Discussion:

cl. aff.

r1

Decision: 		The document was revised to R5-241745.

R5-241745	Addition of n5 into TC 6.2D.1 MOP for UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2726  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-241261)

Decision: 		The document was agreed.

R5-241263	Addition of n5 into TC 6.2D.2 MPR for UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2727  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was agreed.

###### 5.3.25.3.2	Rx Requirements (Clause 7)

5.3.25.3.3	Clauses 1-5 / Annexes

R5-241260	Update of Operating bands for UL MIMO band n5

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2725  Cat: F (Rel-18)

					Source: China Unicom

Abstract:

Core spec alignment

Decision: 		The document was agreed.

##### 5.3.25.4	TS 38.522

##### 5.3.25.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

##### 5.3.25.6	Discussion Papers / Work Plan / TC lists

#### 5.3.26	High power UE (power class 2) for NR FR1 FDD single band (UID-1000051) HPUE_NR_FR1_FDD_R18-UEConTest

##### 5.3.26.1	TS 38.508-1

##### 5.3.26.2	TS 38.508-2

R5-241259	Addition of RF baseline implementation capability of PC2 config n8

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0601  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was agreed.

##### 5.3.26.3	TS 38.521-1

###### 5.3.26.3.1	Tx Requirements (Clause 6)

R5-240279	Addition of PC2 for n8 into TC 6.2.1 MOP

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2621  Cat: F (Rel-18)

					Source: CU Digital Technology

Decision: 		The document was agreed.

R5-240280	Addition of PC2 for n8 into TC 6.2.3 A-MPR

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2622  Cat: F (Rel-18)

					Source: CU Digital Technology

Decision: 		The document was withdrawn.

###### 5.3.26.3.2	Rx Requirements (Clause 7)

R5-240281	Addition of PC2 for n8 into TC 7.3.2 Reference Sensitivity

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2623  Cat: F (Rel-18)

					Source: CU Digital Technology

Decision: 		The document was agreed.

###### 5.3.26.3.3	Clauses 1-5 / Annexes

##### 5.3.26.4	TS 38.522

##### 5.3.26.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

##### 5.3.26.6	Discussion Papers / Work Plan / TC lists

#### 5.3.27	High power UE (power class 1.5) for NR FR1 TDD single band (UID-1000053) HPUE_NR_FR1_TDD_R18-UEConTest

##### 5.3.27.1	TS 38.508-1

##### 5.3.27.2	TS 38.508-2

R5-240079	Introduction of common ICS for PC1.5 n39

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0564  Cat: F (Rel-18)

					Source: CMCC

Abstract:

R4 dependency

Discussion:

CR coversheet:  38.521-1

r1

Decision: 		The document was revised to R5-242023.

R5-242023	Introduction of common ICS for PC1.5 n39

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0564  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240079)

Decision: 		The document was agreed.

##### 5.3.27.3	TS 38.521-1

###### 5.3.27.3.1	Tx Requirements (Clause 6)

R5-240080	Addition of PC1.5 n39 MOP

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2602  Cat: F (Rel-18)

					Source: CMCC

Abstract:

This CR is R4 dependency. The related R4 CR is R4-2400229.

Decision: 		The document was agreed.

R5-240082	Addition of PC1.5 n39 MPR

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2604  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

R5-240083	Addition of  PC1.5 n39 Configured tx power requirements

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2605  Cat: F (Rel-18)

					Source: CMCC

Abstract:

R4 dependency

Decision: 		The document was agreed.

R5-240085	Addition of PC1.5 n39 AMPR

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2607  Cat: F (Rel-18)

					Source: CMCC

Discussion:

late doc

Decision: 		The document was withdrawn.

###### 5.3.27.3.2	Rx Requirements (Clause 7)

###### 5.3.27.3.3	Clauses 1-5 / Annexes

R5-240084	Addition of PC1.5 n39 Modified MPR behavior

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2606  Cat: F (Rel-18)

					Source: CMCC

Abstract:

This CR is R4 dependency. The related R4 CR is R4-2400229.

Decision: 		The document was agreed.

##### 5.3.27.4	TS 38.522

##### 5.3.27.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

##### 5.3.27.6	Discussion Papers / Work Plan / TC lists

#### 5.3.28	Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations (UID-1000054) HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest

##### 5.3.28.1	TS 38.508-1

##### 5.3.28.2	TS 38.508-2

R5-240173	Correction to HPUE PICS Mnemonic

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0567  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240330	Addition of RF baseline implementation capabilities for new PC2 EN-DC combos within FR1

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0575  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240892	Update for additional band configurations with PC2 UL

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0594  Cat: F (Rel-18)

					Source: Verizon Spain

Discussion:

2 WICS!

r1

Decision: 		The document was revised to R5-241714.

R5-241714	Update for additional band configurations with PC2 UL

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0594  rev 1 Cat: F (Rel-18)

					Source: Verizon Spain

(Replaces R5-240892)

Decision: 		The document was agreed.

##### 5.3.28.3	TS 38.521-1

###### 5.3.28.3.1	Tx Requirements (Clause 6)

R5-240894	Addition of PC2 max power requirements for bands CA_n77(2A) and CA_n14A-n77A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2673  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was agreed.

###### 5.3.28.3.2	Rx Requirements (Clause 7)

R5-240896	Addition of band CA_n14A-n77A PC2 reference sensitivity test

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2674  Cat: F (Rel-18)

					Source: WE Certification, AT&T

Discussion:

r1

Decision: 		The document was revised to R5-241746.

R5-241746	Addition of band CA_n14A-n77A PC2 reference sensitivity test

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2674  rev 1 Cat: F (Rel-18)

					Source: WE Certification, AT&T

(Replaces R5-240896)

Decision: 		The document was agreed.

###### 5.3.28.3.3	Clauses 1-5 / Annexes

R5-240893	Addition of CA_n14A-n77A PC2 and CA_n77(2A) PC2 to Ch 5

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2672  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Discussion:

. CR cover value : NR_PC2_CA_R17_2BDL_2BUL-UEConTest.

r1

Decision: 		The document was revised to R5-241747.

R5-241747	Addition of CA_n14A-n77A PC2 and CA_n77(2A) PC2 to Ch 5

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2672  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

(Replaces R5-240893)

Decision: 		The document was agreed.

##### 5.3.28.4	TS 38.521-3

###### 5.3.28.4.1	Tx Requirements (Clause 6)

R5-240331	Addition of UE maximum output power for new PC2 EN-DC combos within FR1

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1716  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240885	Addition of test requirements for EN-DC PC2 combos DC_12A_n77A, DC_14A_n77A and DC_30A_n77A max power test

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1725  Cat: F (Rel-18)

					Source: WE Certification, AT&T

Decision: 		The document was agreed.

###### 5.3.28.4.2	Rx Requirements (Clause 7)

R5-240332	Addition of reference sensitivity for new PC2 EN-DC combos within FR1

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1717  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

R&S commented offline.

r1

Decision: 		The document was revised to R5-241904.

R5-241904	Addition of reference sensitivity for new PC2 EN-DC combos within FR1

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1717  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-240332)

Decision: 		The document was agreed.

R5-241059	Addition of REFSENS for 2CC EN-DC for 19A-n78A in PC2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1735  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was withdrawn.

R5-241061	Addition of REFSENS for 3CC EN-DC in PC2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1737  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was withdrawn.

R5-241062	Addition and correction of REFSENS for 2CC EN-DC in PC2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1738  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was agreed.

R5-241068	Addition of REFSENS for 3CC EN-DC in PC2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1741  Cat: F (Rel-18)

					Source: NTT DOCOMO INC., Verizon

Abstract:

RAN4 dependent

Depending on RAN4 CR R4-2400588, R4-2400584

Discussion:

r3

Decision: 		The document was revised to R5-241989.

R5-241989	Addition of REFSENS for 3CC EN-DC in PC2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1741  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC., Verizon

(Replaces R5-241068)

Decision: 		The document was agreed.

###### 5.3.28.4.3	Clauses 1-5 / Annexes

R5-240884	Addition of many EN-DC PC2 combos to Ch 5

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1724  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was agreed.

R5-241069	Update to FR1 EN-DC Configurations for n78 and n79

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2702  Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

Abstract:

RAN4 dependent

Decision: 		The document was withdrawn.

R5-241072	Update to FR1 EN-DC Configurations for n78 and n79

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1742  Cat: F (Rel-18)

					Source: NTT DOCOMO INC..

Abstract:

RAN4 dependent

Decision: 		The document was agreed.

##### 5.3.28.5	TS 38.522

##### 5.3.28.6	TR 38.905 (NR Test Points Radio Transmission and Reception )

R5-240674	Addition of reference sensitivity TP analysis for new PC2 EN-DC combos within FR1

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0861  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-240895	Update reference sensitivity test cases for additional band configurations with PC2 UL

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0869  Cat: F (Rel-18)

					Source: Verizon Spain

Discussion:

2 WICs!

r1

Decision: 		The document was revised to R5-241748.

R5-241748	Update reference sensitivity test cases for additional band configurations with PC2 UL

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0869  rev 1 Cat: F (Rel-18)

					Source: Verizon Spain

(Replaces R5-240895)

Decision: 		The document was agreed.

R5-241054	Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC2

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0874  Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

Discussion:

r1

Decision: 		The document was revised to R5-241749.

R5-241749	Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC2

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0874  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

(Replaces R5-241054)

Decision: 		The document was agreed.

R5-241056	Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC2

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0876  Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

Discussion:

r1

Decision: 		The document was revised to R5-241750.

R5-241750	Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC2

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0876  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

(Replaces R5-241056)

Decision: 		The document was agreed.

##### 5.3.28.7	Discussion Papers / Work Plan / TC lists

#### 5.3.29	Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1 / 2) bands UL (UID-1000056) NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-UEConTest

##### 5.3.29.1	TS 38.508-1

##### 5.3.29.2	TS 38.508-2

##### 5.3.29.3	TS 38.521-1

###### 5.3.29.3.1	Tx Requirements (Clause 6)

###### 5.3.29.3.2	Rx Requirements (Clause 7)

###### 5.3.29.3.3	Clauses 1-5 / Annexes

##### 5.3.29.4	TS 38.522

##### 5.3.29.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

##### 5.3.29.6	Discussion Papers / Work Plan / TC lists

#### 5.3.30	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest

##### 5.3.30.1	TS 38.508-1

###### 5.3.30.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.30.1.2	Test environment for RF (Clauses 5)

###### 5.3.30.1.3	Test environment for RRM (Clause 7)

###### 5.3.30.1.4	Other clauses / Annexes

##### 5.3.30.2	TS 38.508-2

##### 5.3.30.3	TS 38.521-1

###### 5.3.30.3.1	Tx Requirements (Clause 6)

###### 5.3.30.3.2	Rx Requirements (Clause 7)

###### 5.3.30.3.3	Clauses 1-5 / Annexes

##### 5.3.30.4	TS 38.521-2

###### 5.3.30.4.1	Tx Requirements (Clause 6)

###### 5.3.30.4.2	Rx Requirements (Clause 7)

###### 5.3.30.4.3	Clauses 1-5 / Annexes

##### 5.3.30.5	TS 38.521-3

###### 5.3.30.5.1	Tx Requirements (Clause 6)

###### 5.3.30.5.2	Rx Requirements (Clause 7)

###### 5.3.30.5.3	Clauses 1-5 / Annexes

R5-240936	Update to R18 Configuration for DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1731  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Abstract:

TS38.521-3 clause 5 jumbo CR for WIC "NR_CADC_NR_LTE_DC_R18-UEConTest"

Discussion:

late doc

Decision: 		The document was withdrawn.

##### 5.3.30.6	TS 38.521-4

###### 5.3.30.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.30.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.30.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.30.6.4	Clauses 1-4 / Annexes

##### 5.3.30.7	TS 38.522

R5-240091	Update to R18 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0369  Cat: F (Rel-18)

					Source: CMCC

Discussion:

no test case changes at RAN5#102

Decision: 		The document was withdrawn.

##### 5.3.30.8	TS 38.533

##### 5.3.30.9	TR 38.903 (NR MU & TT analyses)

##### 5.3.30.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.30.11	Discussion Papers / Work Plan / TC lists

#### 5.3.31	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest

##### 5.3.31.1	TS 38.508-1

###### 5.3.31.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.31.1.2	Test environment for RF (Clauses 5)

###### 5.3.31.1.3	Test environment for RRM (Clause 7)

###### 5.3.31.1.4	Other clauses / Annexes

##### 5.3.31.2	TS 38.508-2

##### 5.3.31.3	TS 38.521-1

###### 5.3.31.3.1	Tx Requirements (Clause 6)

###### 5.3.31.3.2	Rx Requirements (Clause 7)

###### 5.3.31.3.3	Clauses 1-5 / Annexes

##### 5.3.31.4	TS 38.521-2

###### 5.3.31.4.1	Tx Requirements (Clause 6)

###### 5.3.31.4.2	Rx Requirements (Clause 7)

###### 5.3.31.4.3	Clauses 1-5 / Annexes

##### 5.3.31.5	TS 38.521-3

###### 5.3.31.5.1	Tx Requirements (Clause 6)

###### 5.3.31.5.2	Rx Requirements (Clause 7)

###### 5.3.31.5.3	Clauses 1-5 / Annexes

##### 5.3.31.6	TS 38.521-4

###### 5.3.31.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.31.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.31.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.31.6.4	Clauses 1-4 / Annexes

##### 5.3.31.7	TS 38.521-5

##### 5.3.31.8	TS 38.533

##### 5.3.31.9	TS 36.521-4

##### 5.3.31.10	TR 38.903 (NR MU & TT analyses)

##### 5.3.31.11	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.31.12	Discussion Papers / Work Plan / TC lists

#### 5.3.32	High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 (UID-1000059) LTE_NR_HPUE_FWVM_R18-UEConTest

##### 5.3.32.1	TS 38.508-2

##### 5.3.32.2	TS 38.521-1

###### 5.3.32.2.1	Tx Requirements (Clause 6)

R5-240454	Editorial correction to UE Power Class for n100 and n101

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2642  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Editorial

Decision: 		The document was agreed.

###### 5.3.32.2.2	Rx Requirements (Clause 7)

###### 5.3.32.2.3	Clauses 1-5 / Annexes

##### 5.3.32.3	TS 38.522

##### 5.3.32.4	TS 36.521-1

##### 5.3.32.5	TS 36.521-2

##### 5.3.32.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.32.7	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )

##### 5.3.32.8	Discussion Papers / Work Plan / TC lists

#### 5.3.33	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest

##### 5.3.33.1	TS 36.508

##### 5.3.33.2	TS 36.521-1

R5-240940	Update to CBW for multiples CA combos

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5490  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International

Abstract:

Clause 5 added CA combos: CA_30A-48A, CA_2A-2A-29A-66A, CA_2A-29A-66A-66A, CA_2A-2A-29A-66A-66A, CA_2A-2A-29A-30A-66A

Decision: 		The document was revised to R5-241853.

R5-241853	Update to CBW for multiples CA combos

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5490  rev 1 Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International

(Replaces R5-240940)

Decision: 		The document was agreed.

R5-240941	Update to Transmit test cases for CA_30A-48A

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5491  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International

Abstract:

Update Delta TIB and 6.2.2A.2 for CA_30A-48A

Decision: 		The document was agreed.

R5-240942	Update to Receiver test cases for multiple CA combos

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5492  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International

Abstract:

Update Delta RIB and 7.3A.3, 7.3A.6, 7.3A.9 and 7.3A.10 for CA_30A-48A, CA_2A-2A-29A-66A,  CA_2A-29A-66A-66A, CA_2A-2A-29A-30A-66A and CA_2A-2A-29A-66A-66A.

TP analysis in R5-241202 (36.905 CR 0271)

Discussion:

r1

Decision: 		The document was revised to R5-241874.

R5-241874	Update to Receiver test cases for multiple CA combos

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5492  rev 1 Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International

(Replaces R5-240942)

Decision: 		The document was agreed.

##### 5.3.33.3	TS 36.521-2

R5-240833	Update PRD20 E-UTRA CA list v160

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1024  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

late doc

Decision: 		The document was agreed.

R5-240937	Additional supported capabilities for multiple CA combos

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1025  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International

Abstract:

Added CA_30A-48A, CA_2A-2A-29A-66A,  CA_2A-29A-66A-66A, CA_2A-2A-29A-30A-66A and CA_2A-2A-29A-66A-66A declaration

Decision: 		The document was agreed.

##### 5.3.33.4	TS 36.521-3

##### 5.3.33.5	TS 37.571-1

##### 5.3.33.6	TS 37.571-3

##### 5.3.33.7	TS 37.571-5

##### 5.3.33.8	TR 36.903 (E-UTRAN RRM TT analyses)

##### 5.3.33.9	TR 36.904 (E-UTRAN Radio Reception TT analyses)

##### 5.3.33.10	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )

R5-241202	Adding Test Point Analysis for 4DL and 5DL RefSens

Type: CR		For: Agreement
					36.905 v18.3.0	  CR-0271  Cat: F (Rel-18)

					Source: Sporton, Bureau Veritas

Abstract:

affected R5-240942 TS 36.521-1 CR 5492

Decision: 		The document was agreed.

##### 5.3.33.11	Discussion Papers / Work Plan / TC lists

#### 5.3.34	Air-to-ground network for NR (UID-1020087) NR_ATG-UEConTest

##### 5.3.34.1	TS 38.508-1

###### 5.3.34.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.34.1.2	Test environment for RF (Clauses 5)

R5-240046	Introduction of RF Test Environment for ATG

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3001  Cat: F (Rel-18)

					Source: CMCC

Discussion:

TF160 manager: not complying to Proposal1 in R5-241452

r1

Decision: 		The document was revised to R5-241919.

R5-241919	Introduction of RF Test Environment for ATG

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3001  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240046)

Decision: 		The document was agreed.

###### 5.3.34.1.3	Test environment for RRM (Clause 7)

R5-240047	Introduction of RRM Test Environment for ATG

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3002  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241920.

R5-241920	Introduction of RRM Test Environment for ATG

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3002  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240047)

Decision: 		The document was agreed.

###### 5.3.34.1.4	Other clauses / Annexes

R5-240045	Introduction of Common test environments for ATG

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3000  Cat: F (Rel-18)

					Source: CMCC

Discussion:

Conflict with R5-240489

TF160 manager: not complying to Proposal1 in R5-241452

Decision: 		The document was withdrawn.

R5-241926	Update of abbreviations for ATG

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3106  Cat: F (Rel-18)

					Source: CMCC

Discussion:

late doc

Decision: 		The document was agreed.

##### 5.3.34.2	TS 38.508-2

R5-240048	Introduction of common ICS for ATG

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0563  Cat: F (Rel-18)

					Source: CMCC

Discussion:

TF160 manager: not complying to Proposal1 in R5-241452

r1

Decision: 		The document was revised to R5-241715.

R5-241715	Introduction of common ICS for ATG

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0563  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240048)

Decision: 		The document was agreed.

##### 5.3.34.3	TS 38.521-1

###### 5.3.34.3.1	Tx Requirements (Clause 6)

R5-240050	Introduction of General description for ATG UE Tx TCs

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2586  Cat: F (Rel-18)

					Source: CMCC

Abstract:

R4-2400230 revised to R4-2403864

Discussion:

r1

Decision: 		The document was revised to R5-242030.

R5-242030	Introduction of General description for ATG UE Tx TCs

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2586  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240050)

Discussion:

for email agreement

The pending RAN4 CR R4-2400230 is agreed.

r1

Decision: 		The document was revised to R5-241683.

R5-241683	Introduction of General description for ATG UE Tx TCs

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2586  rev 2 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-242030)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-240051	Introduction of MOP TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2587  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241751.

R5-241751	Introduction of MOP TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2587  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240051)

Decision: 		The document was agreed.

R5-240052	Introduction of Configured transmitted power TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2588  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241752.

R5-241752	Introduction of Configured transmitted power TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2588  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240052)

Decision: 		The document was agreed.

R5-240053	Introduction of General description of Occupied bandwidth for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2589  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

R5-240054	Introduction of Occupied bandwidth TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2590  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241753.

R5-241753	Introduction of Occupied bandwidth TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2590  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240054)

Decision: 		The document was agreed.

R5-240055	Introduction of General description of Out of band emission for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2591  Cat: F (Rel-18)

					Source: CMCC

Abstract:

Core spec alignment

Decision: 		The document was agreed.

R5-240056	Introduction of General description of Spurious emissions for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2592  Cat: F (Rel-18)

					Source: CMCC

Abstract:

core spec alignment

Decision: 		The document was agreed.

R5-240057	Introduction of General Spurious emissions TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2593  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241754.

R5-241754	Introduction of General Spurious emissions TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2593  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240057)

Decision: 		The document was agreed.

R5-240264	Addition of new test case 6.3J.2 Transmit OFF power for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2614  Cat: F (Rel-18)

					Source: CAICT

Decision: 		The document was revised to R5-241755.

R5-241755	Addition of new test case 6.3J.2 Transmit OFF power for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2614  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240264)

Decision: 		The document was agreed.

R5-240265	Addition of new test case 6.3J.1 Minimum output power for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2615  Cat: F (Rel-18)

					Source: CAICT

Abstract:

TP analysis in R5-230270.

Decision: 		The document was revised to R5-241756.

R5-241756	Addition of new test case 6.3J.1 Minimum output power for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2615  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240265)

Decision: 		The document was agreed.

###### 5.3.34.3.2	Rx Requirements (Clause 7)

R5-240058	Introduction of General description of Rx TCs for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2594  Cat: F (Rel-18)

					Source: CMCC

Abstract:

core spec alignment

Decision: 		The document was agreed.

R5-240059	Introduction of Diversity characteristics description for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2595  Cat: F (Rel-18)

					Source: CMCC

Abstract:

core spec alignment

Decision: 		The document was agreed.

R5-240060	Introduction of General description of Reference sensitivity for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2596  Cat: F (Rel-18)

					Source: CMCC

Abstract:

core spec alignment

Decision: 		The document was agreed.

R5-240061	Introduction of General description of Blocking characteristics for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2597  Cat: F (Rel-18)

					Source: CMCC

Abstract:

core spec alignment

Decision: 		The document was agreed.

R5-240062	Introduction of Spurious response TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2598  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241757.

R5-241757	Introduction of Spurious response TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2598  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240062)

Decision: 		The document was agreed.

R5-240063	Introduction of General description of Intermodulation characteristics for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2599  Cat: F (Rel-18)

					Source: CMCC

Abstract:

core spec alignment

Decision: 		The document was agreed.

R5-240064	Introduction of Wide band intermodulation TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2600  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241758.

R5-241758	Introduction of Wide band intermodulation TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2600  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240064)

Decision: 		The document was agreed.

R5-240065	Introduction of Spurious emissions TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2601  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241759.

R5-241759	Introduction of Spurious emissions TC for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2601  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240065)

Decision: 		The document was agreed.

R5-240266	Addition of new test case 7.4J Maximum input level for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2616  Cat: F (Rel-18)

					Source: CAICT

Abstract:

TP analysis in R5-230270.

Decision: 		The document was revised to R5-241760.

R5-241760	Addition of new test case 7.4J Maximum input level for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2616  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240266)

Decision: 		The document was agreed.

R5-240267	Addition of new test case 7.5J Adjacent channel selectivity for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2617  Cat: F (Rel-18)

					Source: CAICT

Abstract:

TP analysis in R5-230270.

Decision: 		The document was revised to R5-241761.

R5-241761	Addition of new test case 7.5J Adjacent channel selectivity for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2617  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240267)

Decision: 		The document was agreed.

R5-240268	Addition of new test case 7.6J.2 In-band blocking for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2618  Cat: F (Rel-18)

					Source: CAICT

Abstract:

TP analysis in R5-230270.

Decision: 		The document was revised to R5-241762.

R5-241762	Addition of new test case 7.6J.2 In-band blocking for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2618  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240268)

Decision: 		The document was agreed.

R5-240269	Addition of new test case 7.6J.3 Out-of-band blocking for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2619  Cat: F (Rel-18)

					Source: CAICT

Abstract:

TP analysis in R5-230270.

Decision: 		The document was revised to R5-241763.

R5-241763	Addition of new test case 7.6J.3 Out-of-band blocking for ATG

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2619  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240269)

Decision: 		The document was agreed.

###### 5.3.34.3.3	Clauses 1-5 / Annexes

R5-240049	Introduction of common parts for ATG UE RF test cases

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2585  Cat: F (Rel-18)

					Source: CMCC

Discussion:

Conflict with R5-241001

Decision: 		The document was agreed.

R5-240334	Addition of Measurement Uncertainties and Test Tolerances for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2637  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-241764.

R5-241764	Addition of Measurement Uncertainties and Test Tolerances for ATG UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2637  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-240334)

Decision: 		The document was agreed.

##### 5.3.34.4	TS 38.521-4

###### 5.3.34.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.34.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.34.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.34.4.4	Clauses 1-4 / Annexes

##### 5.3.34.5	TS 38.522

R5-240066	Addition of TC applicability statements for ATG UE

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0366  Cat: F (Rel-18)

					Source: CMCC, CAICT

Discussion:

r2

Decision: 		The document was revised to R5-241842.

R5-241842	Addition of TC applicability statements for ATG UE

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0366  rev 1 Cat: F (Rel-18)

					Source: CMCC, CAICT

(Replaces R5-240066)

Decision: 		The document was agreed.

##### 5.3.34.6	TS 38.533

##### 5.3.34.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-240270	Addition of test points analysis for ATG test cases in 38.521-1

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0851  Cat: F (Rel-18)

					Source: CAICT, CMCC

Abstract:

Test cases CRs in R5-230265 to R5-240269.

Discussion:

r1

Decision: 		The document was revised to R5-241765.

R5-241765	Addition of test points analysis for ATG test cases in 38.521-1

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0851  rev 1 Cat: F (Rel-18)

					Source: CAICT, CMCC

(Replaces R5-240270)

Decision: 		The document was agreed.

R5-240388	TP analysis for NR ATG test cases

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0856  Cat: F (Rel-18)

					Source: China Mobile Com. Corporation

Decision: 		The document was withdrawn.

R5-240389	TP analysis for ATG Configured Tx power

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0857  Cat: F (Rel-18)

					Source: China Mobile Com. Corporation

Discussion:

late doc

Decision: 		The document was withdrawn.

##### 5.3.34.8	Discussion Papers / Work Plan / TC lists

#### 5.3.35	4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC (UID-1020088) 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest

##### 5.3.35.1	TS 38.508-1

###### 5.3.35.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.35.1.2	Test environment for RF (Clauses 5)

###### 5.3.35.1.3	Test environment for RRM (Clause 7)

###### 5.3.35.1.4	Other clauses / Annexes

##### 5.3.35.2	TS 38.508-2

##### 5.3.35.3	TS 38.521-1

###### 5.3.35.3.1	Tx Requirements (Clause 6)

R5-240375	Addition of new test case 6.2H.3.2 MPR for inter-band CA with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2640  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Associated discussion paper R5-240377

Decision: 		The document was agreed.

###### 5.3.35.3.2	Rx Requirements (Clause 7)

###### 5.3.35.3.3	Clauses 1-5 / Annexes

##### 5.3.35.4	TS 38.521-3

###### 5.3.35.4.1	Tx Requirements (Clause 6)

R5-240376	Addition of new test case 6.2H.2.3 MPR for inter-band EN-DC with UL MIMO

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1720  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Associated discussion paper R5-240377

Decision: 		The document was revised to R5-241725.

R5-241725	Addition of new test case 6.2H.2.3 MPR for inter-band EN-DC with UL MIMO

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1720  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240376)

Decision: 		The document was agreed.

###### 5.3.35.4.2	Rx Requirements (Clause 7)

##### 5.3.35.5	TS 38.522

###### 5.3.35.5.3	Clauses 1-5 / Annexes

##### 5.3.35.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.35.7	Discussion Papers / Work Plan / TC lists

R5-240377	Discussion on work plan of 3Tx inter-band UL CA or EN-DC

Type: discussion		For: Endorsement
					Source: Huawei, HiSilicon

Abstract:

Associated CRs R5-240375 and R5-240376

Discussion:

"Associated CRs R5-240375 (38.521-1) and R5-240376 (38.521-3)

noted, proposals endorsed"

Decision: 		The document was noted.

#### 5.3.36	Enhanced Test Methods for FR2 NR UEs FS_FR2_enhTestMethods (RAN4 Study Item)

##### 5.3.36.1	Discussion Papers / Work Plan to track adoption of the TR 38.884 outcomes into RAN5 test specifications

R5-241439	Work Plan for Rel17 FR2 RF Enhanced Test Methods

Type: Work Plan		For: Information
					Source: Apple Benelux B.V.

Abstract:

Internal work plan for RAN5 to incorporate FR2 enhanced test methods topics.

Post RAN5#102 update

Discussion:

"Internal work plan for RAN5 to incorporate FR2 enhanced test methods topics.

Post RAN5#102 update"

Decision: 		The document was noted.

5.4	Routine Maintenance for 5G NR only TEIx_Test

#### 5.4.1	TS 38.508-1

##### 5.4.1.1	Test frequencies (Clause 4.3.1)

R5-240258	Correction of Notes in tables of test channel bandwidths

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3019  Cat: F (Rel-18)

					Source: CAICT

Abstract:

Discussion in R5-240257.

Discussion:

r1

revisions related to the proposal in R5-240257 cancelled.

Decision: 		The document was revised to R5-241921.

R5-241921	Correction of Notes in tables of test channel bandwidths

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3019  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240258)

Decision: 		The document was agreed.

R5-240272	Style correction for clause title of 4.3.1.1.1.100 and 4.3.1.1.1.101

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3021  Cat: F (Rel-18)

					Source: CAICT

Abstract:

Editorial

Decision: 		The document was agreed.

R5-240309	Updating test frequency for SUL band n83 20MHz CBW

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3024  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241716.

R5-241716	Updating test frequency for SUL band n83 20MHz CBW

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3024  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240309)

Discussion:

rev!

Decision: 		The document was revised to R5-241869.

R5-241869	Updating test frequency for SUL band n83 20MHz CBW

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3024  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241716)

Decision: 		The document was agreed.

R5-240468	Test frequencies and channel bandwidth updates for band n71 and n25 in Rel-17

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3037  Cat: F (Rel-18)

					Source: Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc.

Abstract:

Accompanying discussion document in R5-240467.

38.521-1 CR 2648 (R5-240469)

Discussion:

+TEI17!

Decision: 		The document was agreed.

R5-240617	Added 30kHz SCS for SSB in n53 to 30kHz SCS test frequencies

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3060  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple

Decision: 		The document was agreed.

R5-240972	Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3080  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA

Abstract:

This CR depends on discussion paper R5-240970 and CR R5-240971.

Discussion:

3GU bug

Anritsu check.

r1

Decision: 		The document was revised to R5-241703.

R5-241703	Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3080  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA

(Replaces R5-240972)

Decision: 		The document was agreed.

R5-241092	Correction to CBW selection to avoid unintentional asymmetric CBW

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3084  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241093	Correction to point A value for n83

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3085  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-241717.

R5-241717	Correction to point A value for n83

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3085  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241093)

Decision: 		The document was agreed.

R5-241094	Correction to parameters for Rel-17 bands

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3086  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-241718.

R5-241718	Correction to parameters for Rel-17 bands

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3086  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241094)

Decision: 		The document was agreed.

##### 5.4.1.2	Test environment for RF (Clauses 5)

R5-240616	RF TRx testing - P-Max configuration extension to RX tests to enable TxD

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3059  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

Huawei: is TEI17?

r1

Decision: 		The document was revised to R5-241704.

R5-241704	RF TRx testing - P-Max configuration extension to RX tests to enable TxD

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3059  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-240616)

Decision: 		The document was agreed.

R5-241095	Correction to message exceptions for Performance test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3087  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

Condition was clarified in Table 5.4.2.0-7 instead of removing searchSpaceType to solve the concern from R&S.

r2

Decision: 		The document was revised to R5-242022.

R5-242022	Correction to message exceptions for Performance test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3087  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241095)

Decision: 		The document was agreed.

R5-241346	Update of PDSCH Config for RF test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3092  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

r1

Decision: 		The document was revised to R5-241968.

R5-241968	Update of PDSCH Config for RF test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3092  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241346)

Decision: 		The document was agreed.

##### 5.4.1.3	Test environment for RRM (Clause 7)

R5-241230	Addition of missing section for RRM CA Configuration

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3091  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

r2

Decision: 		The document was revised to R5-241875.

R5-241875	Addition of missing section for RRM CA Configuration

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3091  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241230)

Decision: 		The document was agreed.

##### 5.4.1.4	Other clauses / Annexes

R5-240271	Correction of errors in Annex C

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3020  Cat: F (Rel-18)

					Source: CAICT

Decision: 		The document was agreed.

R5-240856	Corrections on C.1 for the parameters for calculation of test frequencies

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3074  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-240971	Annex C update to asymmetric channel bandwidths test frequencies calculation

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3079  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA

Abstract:

This CR depends on discussion paper R5-240970.

R4-2400357

Discussion:

3GU bug

Decision: 		The document was agreed.

R5-240996	Correction to SUL configuration messages

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3082  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

CR cover value : TEI15_test

r1

Decision: 		The document was revised to R5-241719.

R5-241719	Correction to SUL configuration messages

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3082  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240996)

Decision: 		The document was agreed.

#### 5.4.2	TS 38.508-2

R5-240093	Update NR band and CADC configs status in ICS Annex B

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0565  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

R5-240313	Introducing indicator for Power Class of CA configuration with single uplink carrier

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0573  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

associated discussion paper: R5-240311

Decision: 		The document was agreed.

R5-240316	Introducing SUL configuration SUL_n78A-n81A

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0574  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240335	Editorial correction to note numbering for inter-band EN-DC capabilities table

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0576  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Editorial

Decision: 		The document was agreed.

#### 5.4.3	TS 38.509

#### 5.4.4	TS 38.521-1

##### 5.4.4.1	Tx Requirements (Clause 6)

R5-240081	Addition of PC2 n40 MOP

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2603  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

R5-240307	Updating test case AMPR for inter-band CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2624  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-240308

associated with discussion paper R5-240306

Decision: 		The document was agreed.

R5-240310	Updating test frequency range for SUL band n83

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2625  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240319	Updating AMPR testing for SUL band n81

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2630  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240320	Updating UTRA ACLR testing for SUL band n81

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2631  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240333	Updating PC2 test requirements in MPR test case for band n1 and n3

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2636  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240336	Addition of new test case 6.3G.4.1 Absolute power tolerance for Tx Diversity

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2638  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

This CR is a resubmission of R5-228059 agreed at RAN5#97

Decision: 		The document was agreed.

R5-240337	Removal of square brackets for Tx Diversity capability

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2639  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241879.

R5-241879	Removal of square brackets for Tx Diversity capability

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2639  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240337)

Decision: 		The document was agreed.

R5-240470	Addition of 35 MHz CBW for transmitter requirements

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2649  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

+TEI7!

Decision: 		The document was agreed.

R5-240622	Message exceptions clarifications for 6.3G.3.3

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2653  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

Huawei: is TEI17?

r2

reissued as R5-241726 because of title change.

Decision: 		The document was revised to R5-241726.

R5-241726	Message exceptions clarifications for 6.3G.3.3

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2653  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-240622)

Abstract:

reissued from R5-241726 because of title change.

Discussion:

reissued as R5-241xxxx because of title change.

CR: 2735??

Decision: 		The document was withdrawn.

R5-241779	Corrections for 6.3G.3.3

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2735  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

reissued from R5-241726 because of title change.

CR: 2735

Decision: 		The document was agreed.

R5-240623	MBW table reference corrected for inter-band case in test case 6.5A.4.1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2654  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-240625	p-Max and p-NR-FR1 adjustment when higherPowerLimit-r17 applies

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2656  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Mediatek

Discussion:

R5-

r2

Decision: 		The document was revised to R5-241766.

R5-241766	p-Max and p-NR-FR1 adjustment when higherPowerLimit-r17 applies

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2656  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Mediatek

(Replaces R5-240625)

Decision: 		The document was agreed.

R5-240992	Clarification of trace mode in emission testing_FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2675  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was revised to R5-241767.

R5-241767	Clarification of trace mode in emission testing_FR1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2675  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240992)

Decision: 		The document was agreed.

R5-240997	Update to message exception of SUL time mask test cases

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2676  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240998	Update to AMPR, ASEM and ASE for intra-band CA for CA_NS_04

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2677  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241768.

R5-241768	Update to AMPR, ASEM and ASE for intra-band CA for CA_NS_04

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2677  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240998)

Decision: 		The document was agreed.

R5-241007	Adding the support of NS_47 for PC 1.5

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2681  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, SoftBank Corp.

Abstract:

TP in R5-241006

Decision: 		The document was agreed.

R5-241099	Correction to UL configuration for intra-band contiguous CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2706  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241100	Correction to Rel-15 A-MPR

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2707  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-241922.

R5-241922	Correction to Rel-15 A-MPR

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2707  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241100)

Decision: 		The document was agreed.

R5-241101	Correction to Rel-16 A-MPR

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2708  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241102	Correction to test configuration in 6.2D.2

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2709  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

r1

TEI17_Test, NR_bands_UL_MIMO_PC3_R17-UEConTest to

TEI16_Test, NR_eMIMO-UEConTest

Decision: 		The document was revised to R5-241969.

R5-241969	Correction to test configuration in 6.2D.2

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2709  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241102)

Decision: 		The document was agreed.

R5-241103	Correction to applicability of powerBoosting for PC3 UE

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2710  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241104	Addition of CBW 35 MHz and 45 MHz to OBW for inter-band CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2711  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241154	Editorial correction in FR1 test case 6.4G.2.1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2716  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

Editorial

Decision: 		The document was agreed.

R5-241155	Editorial correction in FR1 test case 6.5A.2.4.1.1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2717  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

Editorial

Discussion:

R5-

r1

Decision: 		The document was revised to R5-241769.

R5-241769	Editorial correction in FR1 test case 6.5A.2.4.1.1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2717  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-241155)

Decision: 		The document was agreed.

R5-241156	Editorial correction in FR1 test case 6.2A.2.1

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2718  Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

Editorial

Discussion:

e doc

Decision: 		The document was agreed.

R5-241354	Correction in A-MPR test case

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2730  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

##### 5.4.4.2	Rx Requirements (Clause 7)

R5-240312	Updating FR1 PC2 REFSENS exceptions testing

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2626  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

associated discussion paper: R5-240311

Discussion:

r1

Decision: 		The document was revised to R5-241721.

R5-241721	Updating FR1 PC2 REFSENS exceptions testing

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2626  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240312)

Discussion:

rev!

Decision: 		The document was revised to R5-241870.

R5-241870	Updating FR1 PC2 REFSENS exceptions testing

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2626  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241721)

Decision: 		The document was agreed.

R5-240314	Correcting errors in REFSENS for CA test case for PC3 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2627  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241770.

R5-241770	Correcting errors in REFSENS for CA test case for PC3 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2627  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240314)

Discussion:

rev!

Decision: 		The document was revised to R5-241871.

R5-241871	Correcting errors in REFSENS for CA test case for PC3 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2627  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241770)

Decision: 		The document was agreed.

R5-240317	Updating REFSENS testing for SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2629  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-240318

Decision: 		The document was revised to R5-241771.

R5-241771	Updating REFSENS testing for SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2629  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240317)

Discussion:

rev!

Decision: 		The document was revised to R5-241872.

R5-241872	Updating REFSENS testing for SUL configuration

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2629  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-241771)

Decision: 		The document was agreed.

R5-240455	Correction to Reference sensitivity power level test configuration for CA_n28A-n78A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2643  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

TPA in R5-240456 (CR 0858)

Discussion:

+TEI15!

r2

Decision: 		The document was revised to R5-241772.

R5-241772	Correction to Reference sensitivity power level test configuration for CA_n28A-n78A

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2643  rev 1 Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240455)

Decision: 		The document was agreed.

R5-240469	Addition of reference sensitivity channel bandwidths for n25 and n71

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2648  Cat: F (Rel-18)

					Source: Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc.

Abstract:

Depending on R4-2400358 (CR 2003).

Accompanying discussion documents in R5-240467 and R5-240523.

38.508-1 CR 3037 (R5-240468)

Discussion:

+TEI17!

Decision: 		The document was agreed.

R5-241105	Addition of CBW 35 MHz, 45 MHz, 70 MHz to ACS for CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2712  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241106	Addition of CBW 35 MHz, 45 MHz, 70 MHz to Narrow band blocking for inter-band CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2713  Cat: F (Rel-18)

					Source: Anritsu, Rohde&Schwarz

Discussion:

conflicts with R5-241352

r1

Decision: 		The document was revised to R5-241773.

R5-241773	Addition of CBW 35 MHz, 45 MHz, 70 MHz to Narrow band blocking for inter-band CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2713  rev 1 Cat: F (Rel-18)

					Source: Anritsu, Rohde&Schwarz

(Replaces R5-241106)

Decision: 		The document was agreed.

R5-241196	Correction of 7.6A.2 for inband blocking for CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2721  Cat: F (Rel-18)

					Source: ZTE Corporation, Rohde&Schwarz

Abstract:

[Editorial Correction]

Discussion:

There is overlapping with R5-241349 in Table 7.6A.2.3.4.1-1.

r2

Decision: 		The document was revised to R5-241774.

R5-241774	Correction of 7.6A.2 for inband blocking for CA

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2721  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation, Rohde&Schwarz

(Replaces R5-241196)

Decision: 		The document was agreed.

R5-241352	Update of Narrow Band Blocking for CA test case

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2729  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

fully covered by Anritsu's CR R5-241106r1

Decision: 		The document was withdrawn.

##### 5.4.4.3	Clauses 1-5 / Annexes

R5-240231	General updates of clause 5 for R17 new CBW configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2611  Cat: F (Rel-18)

					Source: CU Digital Technology, Nokia

Discussion:

3GU bug

conflicts with R5-240273.

r1

Decision: 		The document was revised to R5-241775.

R5-241775	General updates of clause 5 for R17 new CBW configurations

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2611  rev 1 Cat: F (Rel-18)

					Source: CU Digital Technology, Nokia

(Replaces R5-240231)

Decision: 		The document was agreed.

R5-240273	Addition of asymmetric UL and DL channel bandwidth combinations of band n8 in 5.3.6

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2620  Cat: F (Rel-18)

					Source: CAICT

Decision: 		The document was agreed.

R5-240621	TT Formula vs MU to be added for FR1 EVM test as in FR2

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2652  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-240624	Added 30kHz SCS for SSB in n53 to be aligned with core specs

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2655  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple

Abstract:

Core specs alignment

Discussion:

R5-

r1

Decision: 		The document was revised to R5-241776.

R5-241776	Added 30kHz SCS for SSB in n53 to be aligned with core specs

Type: CR		For: Agreement
					38.521-1 v18.1.0	  CR-2655  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Apple

(Replaces R5-240624)

Decision: 		The document was agreed.

#### 5.4.5	TS 38.521-2

##### 5.4.5.1	Tx Requirements (Clause 6)

R5-240403	FR2c MU - Tx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1015  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

Overlap with R5-241084

Decision: 		The document was withdrawn.

R5-240407	FR2 MU - PC1 UL MIMO - Minimum output power test - 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1017  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240993	Clarification of trace mode in emission testing_FR2

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1025  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241783.

R5-241783	Clarification of trace mode in emission testing_FR2

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1025  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240993)

Decision: 		The document was agreed.

R5-240995	Adding FR2 test case of SRS time mask

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1026  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241990.

R5-241990	Adding FR2 test case of SRS time mask

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1026  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240995)

Decision: 		The document was agreed.

R5-241084	Update for FR2c MU

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1028  Cat: F (Rel-18)

					Source: Anritsu, Keysight

Abstract:

AP#99.21; Depending on the outcome of MU discussion R5-241083

Discussion:

Conflicts with R5-240403 and R5-240404 MU.

r2

Decision: 		The document was revised to R5-241859.

R5-241859	Update for FR2c MU

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1028  rev 1 Cat: F (Rel-18)

					Source: Anritsu, Keysight

(Replaces R5-241084)

Decision: 		The document was agreed.

R5-241174	Correction to CA A-MPR requirements

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1030  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depending on RAN4 CR R4-2400512.

Decision: 		The document was agreed.

R5-241343	Correction of MPR CA test cases

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1031  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-241353	Editorial correction of TT for Minimum Output Power for UL MIMO

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1032  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Editorial

Decision: 		The document was agreed.

##### 5.4.5.2	Rx Requirements (Clause 7)

R5-240404	FR2c MU - Rx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1016  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

Tx!

r1

Overlap with R5-241084

w/d

Decision: 		The document was revised to R5-241860.

R5-241860	FR2c MU - Rx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1016  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-240404)

Decision: 		The document was withdrawn.

R5-240962	Update to FR2 ACS TC

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1024  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Abstract:

Core spec alignment.

Decision: 		The document was agreed.

R5-241107	Clarification of test procedure of EIS spherical coverage for inter-band CA

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1029  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

##### 5.4.5.3	Clauses 1-5 / Annexes

R5-240409	Blocking measurement procedure updates in section K.1.8

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1018  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-240604	CR on Coarse&Fine Beam Peak Search Grids

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1019  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, CAICT

Abstract:

Discussion paper in TDoc # matching this TDoc# -1

Decision: 		The document was agreed.

R5-240626	FR2 DL RMCs - Missing notes update

Type: CR		For: Agreement
					38.521-2 v18.1.0	  CR-1020  Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

Core specs alignment

Decision: 		The document was agreed.

#### 5.4.6	TS 38.521-3

##### 5.4.6.1	Tx Requirements (Clause 6)

R5-240260	Removal of LTE anchor agnostic approach testing in 6.5B.3.3.2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1712  Cat: F (Rel-18)

					Source: CAICT

Abstract:

Discussion in R5-240259.

Discussion:

r3

new Tdoc needed.

Decision: 		The document was revised to R5-241991.

R5-241991	Removal of LTE anchor agnostic approach testing in 6.5B.3.3.2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1712  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240260)

Decision: 		The document was agreed.

R5-240263	Correction of applicability for test with LTE anchor agnostic approach in 6.5B.3.3.1

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1714  Cat: F (Rel-18)

					Source: CAICT

Discussion:

r1

Decision: 		The document was revised to R5-241923.

R5-241923	Correction of applicability for test with LTE anchor agnostic approach in 6.5B.3.3.1

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1714  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240263)

Decision: 		The document was agreed.

R5-240338	Update of MOP test configuration for DC_20A_n28A

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1718  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240339	Correction to H3 title for clauses including NE-DC test cases

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1719  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Core spec alignment

Decision: 		The document was agreed.

R5-240405	FR2c MU - Updates in 38.521-3

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1721  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

Overlap with R5-241084

Decision: 		The document was withdrawn.

R5-240879	Addition of general spurious emissions test for EN-DC combos DC_2A_n66A and DC_30A_n66A

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1723  Cat: F (Rel-18)

					Source: WE Certification, AT&T

Decision: 		The document was agreed.

R5-240967	Correct clause 6.2B.4.1.2 initial conditions

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1732  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

had wrong ver in 3GU

+5GS!

r2

Decision: 		The document was revised to R5-241789.

R5-241789	Correct clause 6.2B.4.1.2 initial conditions

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1732  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-240967)

Decision: 		The document was agreed.

R5-240994	Clarification of trace mode in emission testing_Iw

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1733  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241784.

R5-241784	Clarification of trace mode in emission testing_Iw

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1733  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240994)

Decision: 		The document was agreed.

R5-241085	Update for FR2c MU

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1747  Cat: F (Rel-18)

					Source: Anritsu, Keysight

Abstract:

AP#99.21; Depending on the outcome of MU discussion R5-241083

Discussion:

conflicts with R5-240405.

r1

Decision: 		The document was revised to R5-241861.

R5-241861	Update for FR2c MU

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1747  rev 1 Cat: F (Rel-18)

					Source: Anritsu, Keysight

(Replaces R5-241085)

Decision: 		The document was agreed.

R5-241172	Editorial correction to FR1 inter-band co-existence

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1753  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

editorial

Discussion:

had wrong ver in 3GU

r1

Decision: 		The document was revised to R5-241790.

R5-241790	Editorial correction to FR1 inter-band co-existence

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1753  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241172)

Decision: 		The document was agreed.

##### 5.4.6.2	Rx Requirements (Clause 7)

R5-240305	Correction to test configuration for non-exception REFSENS testing for 3CC EN-DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1715  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-240627	Minimum requirements update for DC_5A_n78A

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1722  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-241058	Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1734  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was withdrawn.

R5-241060	Update of REFSENS for 3CC EN-DC in PC3

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1736  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was withdrawn.

R5-241064	Update of REFSENS for 3CC EN-DC in PC3

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1739  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

RAN4 dependent

Depending on RAN4 CR R4-2400584

Discussion:

Rel-8!

Decision: 		The document was withdrawn.

R5-241066	Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1740  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

RAN4 dependent

Depending on RAN4 CR R4-2400584

Discussion:

r1

Decision: 		The document was revised to R5-241992.

R5-241992	Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1740  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

(Replaces R5-241066)

Decision: 		The document was agreed.

R5-241109	Correction to note application in Table 7.3B.2.0.3.4-2

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1749  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241110	Correction to p-Max value in Rx test cases for FR1 intra-band contiguous EN-DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1750  Cat: F (Rel-18)

					Source: Anritsu, Eurofins KCTL, Huawei, HiSilicon

Decision: 		The document was agreed.

R5-241111	Correction to UL power control for FR1 intra-band non-contiguous EN-DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1751  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241112	Correction to p-Max value in out of band test cases for FR1 inter-band EN-DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1752  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

r3

Decision: 		The document was revised to R5-241924.

R5-241924	Correction to p-Max value in out of band test cases for FR1 inter-band EN-DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1752  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241112)

Decision: 		The document was agreed.

##### 5.4.6.3	Clauses 1-5 / Annexes

R5-240262	Correction of applicability and test coverage rules for SA and NSA capable devices

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1713  Cat: F (Rel-18)

					Source: CAICT

Discussion:

r1

Decision: 		The document was revised to R5-241905.

R5-241905	Correction of applicability and test coverage rules for SA and NSA capable devices

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1713  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240262)

Decision: 		The document was agreed.

R5-240933	Update to R15 Configuration for DC

Type: CR		For: Agreement
					38.521-3 v18.1.1	  CR-1728  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Abstract:

TS38.521-3 clause 5 jumbo CR for WIC "TEI15_Test, 5GS_NR_LTE-UEConTest"

Decision: 		The document was withdrawn.

#### 5.4.7	TS 38.521-4

##### 5.4.7.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-240218	Corrections to test parameters for CSI test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0791  Cat: F (Rel-18)

					Source: MediaTek (Hefei) Inc.

Abstract:

Core spec alignment

Decision: 		The document was agreed.

R5-241144	Correction to numCDM group for PDCCH test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0812  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-241145	Update to power imbalance test cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0813  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was agreed.

R5-241157	HST-DPS channel profile clarifications

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0818  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was agreed.

R5-241169	Corrections to URLLC Test Cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0819  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

Tdoc# ..8

r1

Decision: 		The document was revised to R5-241818.

R5-241818	Corrections to URLLC Test Cases

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0819  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241169)

Decision: 		The document was agreed.

R5-241370	Editorial correction to 5.2.2.1.1_1 Modulation format

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0824  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Abstract:

Editorial CR

Decision: 		The document was agreed.

##### 5.4.7.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

R5-240877	Update on test case 7.5.1 to add FR2 testing direction

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0804  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-241170	Correction to FR2 SDR requirements

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0820  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depending on RAN4 CR R4-2400308.

Decision: 		The document was agreed.

R5-241171	Correction to FR2 256QAM CQI reporting

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0821  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depending on RAN4 CR R4-2400315.

Decision: 		The document was agreed.

##### 5.4.7.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

##### 5.4.7.4	Clauses 1-4 / Annexes

R5-241137	Addition of SDR RMC for more FDD channel BW

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0806  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241819.

R5-241819	Addition of SDR RMC for more FDD channel BW

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0806  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241137)

Decision: 		The document was agreed.

R5-241138	Update Es for 1024QAM scenarios

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0807  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Abstract:

R4-2400635 revised to R4-2403072 and agreed

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241684.

R5-241684	Update Es for 1024QAM scenarios

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0807  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241138)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-241173	Correction to CSI reference measurement channels for 256QAM

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0822  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depending on RAN4 CR R4-2400312.

Discussion:

the corresponding RAN4 CR will not be agreed in this meeting.

Decision: 		The document was withdrawn.

R5-241418	Correction to max throughput values for PDSCH RMC

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0825  Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241820.

R5-241820	Correction to max throughput values for PDSCH RMC

Type: CR		For: Agreement
					38.521-4 v18.1.0	  CR-0825  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241418)

Decision: 		The document was agreed.

#### 5.4.8	TS 38.521-5

#### 5.4.9	TS 38.522

R5-240212	Editorial Correction to HST TCs on release information

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0370  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-240261	Update of Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0372  Cat: F (Rel-18)

					Source: CAICT

Abstract:

Discussion in R5-240259.

Discussion:

r2

new Tdoc needed.

Decision: 		The document was revised to R5-241993.

R5-241993	Update of Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0372  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-240261)

Decision: 		The document was agreed.

R5-240932	Correction to applicability of 5G test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0385  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Rohde&Schwarz

Abstract:

TS38.522 jumbo CR for closed 5G WIC

Decision: 		The document was agreed.

R5-241204	Add information to non-TXD test cases with UE supports TXD

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0391  Cat: F (Rel-18)

					Source: Sporton

Discussion:

r2

Decision: 		The document was revised to R5-241873.

R5-241873	Add information to non-TXD test cases with UE supports TXD

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0391  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-241204)

Decision: 		The document was agreed.

R5-241239	Correction to applicability notes for FR2 RRM RLM test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0392  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241305	Update to Applicability General Section

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0396  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

R5-241306	Update to test selection criteria for RRM tests

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0397  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was revised to R5-241841.

R5-241841	Update to test selection criteria for RRM tests

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0397  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Germany

(Replaces R5-241306)

Decision: 		The document was agreed.

R5-241348	Alignment of status of FR2 UL MIMO test cases

Type: CR		For: Agreement
					38.522 v18.1.0	  CR-0398  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

#### 5.4.10	TS 38.533

##### 5.4.10.1	EN-DC with all NR cells in FR1 (Clause 4)

R5-240213	Editorial correction to HST TCs on test applicability description

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2880  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Discussion:

resolve the overlapping with Anritsu CR R5-241119

Decision: 		The document was withdrawn.

R5-240702	Correction to EN-DC and NR SA inter-frequency measurement test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2922  Cat: F (Rel-18)

					Source: Huawei,HiSilicon,Starpoint

Abstract:

RAN4 dependency

Decision: 		The document was withdrawn.

R5-240703	Correction to EN-DC and NR SA SCell activation test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2923  Cat: F (Rel-18)

					Source: Huawei,HiSilicon,Starpoint

Discussion:

cl. aff.

CR coversheet: NR_redcap_plus_ARCH-UEConTest

r3

Decision: 		The document was revised to R5-241994.

R5-241994	Correction to EN-DC and NR SA SCell activation test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2923  rev 1 Cat: F (Rel-18)

					Source: Huawei,HiSilicon,Starpoint

(Replaces R5-240703)

Decision: 		The document was agreed.

R5-240704	Correction to PCI updating formulas in RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2924  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

Discussion:

r2

Decision: 		The document was revised to R5-241995.

R5-241995	Correction to PCI updating formulas in RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2924  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

(Replaces R5-240704)

Decision: 		The document was agreed.

R5-240758	Correction to FR1 4-step RACH test cases with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2955  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240759	Correction to FR1 2-step RACH test cases with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2956  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240760	Correction to FR1 inter frequency SS SINR relative accuracy test cases with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2957  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240761	Correction to FR1 L1 RSRP absolute accuracy test cases with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2958  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240762	Correction to FR1 LTE RSRP accuracy test case 6.7.5.1 with TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2959  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-240763	Correction to Annex F for R15 RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2960  Cat: F (Rel-18)

					Source: Huawei,HiSilicon

Abstract:

RRM TT

Decision: 		The document was withdrawn.

R5-240968	Correct Table 4.3.2.2.1.4.1-2 & Table 4.3.2.2.2.4.1-2 & Table 4.3.2.2.3.4.1-2 & Table 4.3.2.2.4.4.1-2 of test frequency

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2989  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

TEI16->15

r1

Decision: 		The document was revised to R5-241833.

R5-241833	Correct Table 4.3.2.2.1.4.1-2 & Table 4.3.2.2.2.4.1-2 & Table 4.3.2.2.3.4.1-2 & Table 4.3.2.2.4.4.1-2 of test frequency

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2989  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-240968)

Decision: 		The document was agreed.

R5-241119	Correction to test applicability of 4.6.1.7

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2995  Cat: F (Rel-18)

					Source: Anritsu, MediaTek Beijing Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241834.

R5-241834	Correction to test applicability of 4.6.1.7

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2995  rev 1 Cat: F (Rel-18)

					Source: Anritsu, MediaTek Beijing Inc.

(Replaces R5-241119)

Decision: 		The document was agreed.

R5-241160	Correction to EN-DC FR1 Beam Failure TCs 4.5.5.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3005  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-241200	Correct of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3009  Cat: F (Rel-18)

					Source: Sporton

Abstract:

Test tolerance analysis in R5-241199 TR38.903 CR 0684

Decision: 		The document was agreed.

##### 5.4.10.2	NE-DC with all NR cells in FR1 (Clause 4A)

##### 5.4.10.3	EN-DC with at least 1 NR Cell in FR2 (Clause5)

R5-241128	Correction to test parameters in 5.6.1.3 and 5.6.1.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3002  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400297

Discussion:

r2

Decision: 		The document was revised to R5-242021.

R5-242021	Correction to test parameters in 5.6.1.3 and 5.6.1.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3002  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241128)

Decision: 		The document was agreed.

R5-241162	Correction to EN-DC FR2 Beam Failure TCs 5.5.5.x

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3006  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-241232	Correction for test case 5.5.6.2.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3025  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-241835.

R5-241835	Correction for test case 5.5.6.2.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3025  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241232)

Decision: 		The document was agreed.

R5-241233	Editorial correction in Table 5.6.1.3.5-2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3026  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

editorial

Decision: 		The document was agreed.

R5-241234	Corrections to BFD-LR test case 5.5.5.5

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3027  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-241836.

R5-241836	Corrections to BFD-LR test case 5.5.5.5

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3027  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241234)

Decision: 		The document was agreed.

R5-241235	Correction to BFD-LR test case 5.5.5.5 message content

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3028  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-241236	Removal of Editors note for RRM FR2 RLM in-sync test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3029  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

'

r2

Decision: 		The document was revised to R5-241996.

R5-241996	Removal of Editors note for RRM FR2 RLM in-sync test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3029  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241236)

Decision: 		The document was agreed.

R5-241237	Revision of Editors note for RRM FR2 RLM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3030  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

Qualcomm & Keysight commented

r3

Decision: 		The document was revised to R5-241997.

R5-241997	Revision of Editors note for RRM FR2 RLM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3030  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241237)

Decision: 		The document was agreed.

R5-241238	Editor correction to Test Procedure 5.5.1.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3031  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

editorial

Decision: 		The document was agreed.

##### 5.4.10.4	NR Standalone in FR1 (Clause 6)

R5-240026	Editorial corrections for 6.7.3.1 and 6.7.3.2.2

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2875  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-241129	Correction to TRS Configuration in 6.3.2.1.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3003  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400287

Discussion:

r1

Decision: 		The document was revised to R5-241998.

R5-241998	Correction to TRS Configuration in 6.3.2.1.3

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3003  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241129)

Decision: 		The document was agreed.

R5-241130	Correction to PDCCH level in 6.5.1.8

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3004  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2400291

Discussion:

r1

Decision: 		The document was revised to R5-241999.

R5-241999	Correction to PDCCH level in 6.5.1.8

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3004  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241130)

Decision: 		The document was agreed.

R5-241231	Corrections to test procedure for 2-step RACH TC 6.3.2.2.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3024  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-241837.

R5-241837	Corrections to test procedure for 2-step RACH TC 6.3.2.2.4

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3024  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-241231)

Decision: 		The document was agreed.

R5-241304	Update to RRM test 6.3.1.4 including TT

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3063  Cat: F (Rel-18)

					Source: Qualcomm Germany

Decision: 		The document was agreed.

##### 5.4.10.5	NR standalone with at least one NR cell in FR2 (Clause7)

##### 5.4.10.6	E-UTRA – NR Inter-RAT with E-UTRA serving cell (Clause 8)

##### 5.4.10.7	Clauses 1-3 / Annexes

R5-240222	Correction to Table H.3.1-8A

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2881  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial, CoreSpecAlig

Discussion:

+ NR_feMIMO-UEConTest

r1

Decision: 		The document was revised to R5-241838.

R5-241838	Correction to Table H.3.1-8A

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2881  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-240222)

Decision: 		The document was agreed.

R5-240223	Correction to Table H.3.1-12

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2882  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial, CoreSpecAlig

Discussion:

TEI17->16, + NR_feMIMO-UEConTest

r1

Decision: 		The document was revised to R5-241839.

R5-241839	Correction to Table H.3.1-12

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2882  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-240223)

Decision: 		The document was agreed.

R5-240224	Correction to Table H.3.1-12A

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2883  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial, CoreSpecAlig

Discussion:

+ NR_feMIMO-UEConTest

r1

Decision: 		The document was revised to R5-241840.

R5-241840	Correction to Table H.3.1-12A

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2883  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-240224)

Decision: 		The document was agreed.

R5-240602	Introducing the Re-Positioning Concept for FR2 RRM TCs

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2897  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Rohde&Schwarz

Abstract:

Discussion paper in TDoc # matching this TDoc# -1

Discussion:

r1

Decision: 		The document was revised to R5-241862.

R5-241862	Introducing the Re-Positioning Concept for FR2 RRM TCs

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2897  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, Rohde&Schwarz

(Replaces R5-240602)

Decision: 		The document was agreed.

R5-240793	Correction of Test tolerance for FR2 Inter frequency test case

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2962  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240920	Cell Mapping correction for TC 6.5.6.1.1 in Annex E

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2987  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240921	Cell Mapping correction for TC 6.6.1.8 in Annex E

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2988  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-241120	Correction to QuantityConfigu-DEFAULT in H.3.1

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-2996  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-241201	Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3010  Cat: F (Rel-18)

					Source: Sporton

Abstract:

Test tolerance analysis in R5-241199 TR38.903 CR 0684

Discussion:

CR cover value : -.

r1

Decision: 		The document was revised to R5-241901.

R5-241901	Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3010  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-241201)

Decision: 		The document was agreed.

R5-241447	Correction to Annex F for R15 RRM test cases

Type: CR		For: Agreement
					38.533 v18.1.0	  CR-3091  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

#### 5.4.11	TS 38.551

R5-240597	Introduce Annex for maximum uncertainty of test system and test tolerance

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0001  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241934.

R5-241934	Introduce Annex for maximum uncertainty of test system and test tolerance

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0001  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

(Replaces R5-240597)

Decision: 		The document was agreed.

R5-240598	Correction and alignment of Annex B title

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0002  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240599	Further clarifications in Annex A such as MPAC description and coordinate system

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0003  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240600	Align Test Case Structure to typical RAN5 spec

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0004  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Discussion:

r2

Decision: 		The document was revised to R5-241935.

R5-241935	Align Test Case Structure to typical RAN5 spec

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0004  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

(Replaces R5-240600)

Decision: 		The document was agreed.

R5-241047	Editorial update on Annex C

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0005  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial changes

Discussion:

CR cover value : TEIx_Test.

r1

Decision: 		The document was revised to R5-241936.

R5-241936	Editorial update on Annex C

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0005  rev 1 Cat: F (Rel-17)

					Source: Apple (UK) Limited

(Replaces R5-241047)

Decision: 		The document was agreed.

R5-241048	Editorial update on Annex E

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0006  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial changes on 38.551 Annex E

Discussion:

CR cover value : TEIx_Test.

r1

Decision: 		The document was revised to R5-241937.

R5-241937	Editorial update on Annex E

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0006  rev 1 Cat: F (Rel-17)

					Source: Apple (UK) Limited

(Replaces R5-241048)

Decision: 		The document was agreed.

R5-241049	Editorial update on clause 3

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0007  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial update on clause 3

Discussion:

CR cover value : TEIx_Test.

r1

Decision: 		The document was revised to R5-241938.

R5-241938	Editorial update on clause 3

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0007  rev 1 Cat: F (Rel-17)

					Source: Apple (UK) Limited

(Replaces R5-241049)

Decision: 		The document was agreed.

R5-241050	Editorial update on clause 4

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0008  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial update on 38.551 clause 4

Discussion:

CR cover value : TEIx_Test.

r1

Decision: 		The document was revised to R5-241939.

R5-241939	Editorial update on clause 4

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0008  rev 1 Cat: F (Rel-17)

					Source: Apple (UK) Limited

(Replaces R5-241050)

Decision: 		The document was agreed.

R5-241416	Add missing abbreviations

Type: CR		For: Agreement
					38.551 v17.0.0	  CR-0009  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

#### 5.4.12	TS 38.561

R5-240596	CR on Alternate TRS Procedure with Linearization

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0001  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Abstract:

Discussion paper in TDoc # matching this TDoc# -1

Discussion:

"Discussion paper in R5-240595

KS: conflicts R5-241206(R&S), Will be discussed offline and online"

Decision: 		The document was withdrawn.

R5-241206	CR on EIS search interpolation

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0002  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Discussion:

cl. aff.

offline discussions with Keysight

r2

Decision: 		The document was revised to R5-241929.

R5-241929	CR on EIS search interpolation

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0002  rev 1 Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

(Replaces R5-241206)

Decision: 		The document was agreed.

R5-241421	Updates across TS 38.561 V17.0.0

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0003  Cat: F (Rel-17)

					Source: Apple Benelux B.V.

Abstract:

Editorial and format, reference corrections. To incorporate feedback from editHelp!

Discussion:

r1

Decision: 		The document was revised to R5-241932.

R5-241932	Updates across TS 38.561 V17.0.0

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0003  rev 1 Cat: F (Rel-17)

					Source: Apple Benelux B.V.

(Replaces R5-241421)

Decision: 		The document was agreed.

R5-241422	Update of TT within TRP and TRS tests

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0004  Cat: F (Rel-17)

					Source: Apple Benelux B.V.

Abstract:

Associated with Discussion Paper R5-241423 on FR1 OTA TT

Discussion:

r1

Decision: 		The document was revised to R5-241931.

R5-241931	Update of TT within TRP and TRS tests

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0004  rev 1 Cat: F (Rel-17)

					Source: Apple Benelux B.V.

(Replaces R5-241422)

Decision: 		The document was agreed.

R5-241425	Updates to Annex A.4.2.12

Type: CR		For: Agreement
					38.561 v17.0.0	  CR-0005  Cat: F (Rel-17)

					Source: Apple Benelux B.V.

Abstract:

Associated with Discussion Paper on FR1 OTA TT

Discussion:

late doc

Decision: 		The document was withdrawn.

#### 5.4.13	TS 36.508

#### 5.4.14	TS 36.521-3

#### 5.4.15	TS 37.571-1

#### 5.4.16	TS 37.571-3

#### 5.4.17	TS 37.571-5

#### 5.4.18	TR 38.903 (NR MU & TT analyses)

R5-240402	FR2c MU definition in 38.903

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0650  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241863.

R5-241863	FR2c MU definition in 38.903

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0650  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-240402)

Decision: 		The document was agreed.

R5-240406	FR2 MU - PC1 UL MIMO - Minimum output power test - 38.903

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0651  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240606	CR to define FR2d QoQZ MUs and misc QoQZ MU corrections

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0652  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

Discussion paper in TDoc # matching this TDoc# -1

Discussion:

r1

Decision: 		The document was revised to R5-241864.

R5-241864	CR to define FR2d QoQZ MUs and misc QoQZ MU corrections

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0652  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-240606)

Decision: 		The document was agreed.

R5-241086	Update for FR2c MU

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0682  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

AP#99.21; Depending on the outcome of MU discussion R5-241083

Discussion:

r1

Decision: 		The document was revised to R5-241865.

R5-241865	Update for FR2c MU

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0682  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-241086)

Decision: 		The document was agreed.

R5-241199	Correct of test tolerance analysis of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0684  Cat: F (Rel-18)

					Source: Sporton

Abstract:

affected TS 38.533 CR 3009/3010

Discussion:

r1

Decision: 		The document was revised to R5-241902.

R5-241902	Correct of test tolerance analysis of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0684  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-241199)

Decision: 		The document was agreed.

R5-241345	Documentation of MU for n259

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0702  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Associated discussion paper in R5-241344

Discussion:

r1

Decision: 		The document was revised to R5-241866.

R5-241866	Documentation of MU for n259

Type: CR		For: Agreement
					38.903 v18.1.0	  CR-0702  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-241345)

Decision: 		The document was agreed.

#### 5.4.19	TR 38.905 (NR Test Points Radio Transmission and Reception )

R5-240308	Updating test principle for AMPR for inter-band UL CA

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0852  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-240307

associated with discussion paper R5-240306

Decision: 		The document was agreed.

R5-240318	Updating test points for FR1 REFSENS for SUL test case

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0853  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-240317

Decision: 		The document was agreed.

R5-240456	Correction to reference sensitivity test point analysis for CA_n28A-n78A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0858  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Requirement CR in R5-240455 (CR 2643)

Discussion:

+TEI15!

r1

Decision: 		The document was revised to R5-241777.

R5-241777	Correction to reference sensitivity test point analysis for CA_n28A-n78A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0858  rev 1 Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240456)

Decision: 		The document was agreed.

R5-240881	Test point analysis of EN-DC spurious emissions for DC_2A_n66A and DC_30A_n66A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0866  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was revised to R5-241778.

R5-241778	Test point analysis of EN-DC spurious emissions for DC_2A_n66A and DC_30A_n66A

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0866  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

(Replaces R5-240881)

Decision: 		The document was agreed.

R5-241006	Test points analysis for FR1 AMPR test case with NS_47 and PC 1.5

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0871  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, SoftBank Corp.

Abstract:

TC in R5-241007

Decision: 		The document was agreed.

R5-241053	Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC3

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0873  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Discussion:

'-'

+?

Decision: 		The document was withdrawn.

R5-241055	Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC3

Type: CR		For: Agreement
					38.905 v18.1.0	  CR-0875  Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

Decision: 		The document was withdrawn.

#### 5.4.20	Discussion Papers / Work Plan / TC lists

R5-240257	Discussion on UE channel bandwidths capabilities

Type: discussion		For: Endorsement
					38.508-1 v..
					Source: CAICT

Abstract:

Corresponding CR: R5-240258.

Discussion:

Corresponding CR: R5-240258

Decision: 		The document was noted.

R5-240259	Discussion on removal of LTE anchor agnostic approach testing in 6.5B.3.3.2 of 38.521-3

Type: discussion		For: Endorsement
					38.521-3 v..
					Source: CAICT

Abstract:

Corresponding CRs: R5-240260 and R5-240261.

Discussion:

"Corresponding CRs: R5-240260(38.521-3) and R5-240261(38.522)

concl6 is accepted by proponents to result in addiitonal testing outside of ran4 requirements and is currently OK to have in ran5 spec . Any further changes to this addiitonal test point reduction needs to be justified with technical CR's"

Decision: 		The document was noted.

R5-240306	Discussion on updating FR1 Inter-band 2UL CA AMPR test case

Type: discussion		For: Endorsement
					Source: Huawei, Hisilicon

Abstract:

associated CR: R5-240307 and R5-240308

Discussion:

noted, proposals endorsed

Decision: 		The document was noted.

R5-240311	Discussion on FR1 REFSENS exceptions test requirements for CA

Type: discussion		For: Endorsement
					Source: Huawei, Hisilicon

Abstract:

associated CR: R5-240312 and R5-240313

Discussion:

noted, proposals endorsed

Decision: 		The document was noted.

R5-240401	Discussion on MU and TT analysis for FR2c

Type: discussion		For: Endorsement
					38.521-2 v..
					Source: Keysight Technologies UK Ltd

Abstract:

CRs: R5-240402, R5-240403,

R5-240404, R5-240405

Discussion:

"Associated CRs R5-240402 to R5-240405

2/21: Offline discussions are occurring.

2/28:

Moderator (AT&T): This Tdoc can be noted and Proposals 2, 3, 4, 5, 9, and 10 can be endorsed with the assumption that the numbers in Proposal 9 are considered to be in [ ].

Noted and prop 2,3,4,5,9 &10 endorsed

"

Decision: 		The document was noted.

R5-240408	Discussion on ACS and IBB testing

Type: discussion		For: Endorsement
					38.521-2 v..
					Source: Keysight Technologies UK Ltd

Abstract:

CR R5-240409

Discussion:

"Associated CR R5-240409

2/28:

Moderator (AT&T): No comments received on this Tdoc. This Tdoc can be noted and Proposal 1 can be endorsed.

Noted and prop1 endorsed

LS response to Ran4 indicating the proposal to be drafted by keys"

Decision: 		The document was noted.

R5-240467	Bands n25 and n71 asymmetric UL/DL REFSENS

Type: discussion		For: Approval
					Source: Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc.

Abstract:

Accompanying CRs in R5-240468 (CR 3037),  R5-240469 (CR 2648)

Discussion:

noted, proposals endorsed

Decision: 		The document was noted.

R5-240523	Test frequencies for asymmetric UL/DL CBW

Type: discussion		For: Approval
					Source: Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc.

Abstract:

Accompanying discussion doc in R5-240467, CR in R5-240468 (CR 3037)

Discussion:

Proposal is endorsed

Decision: 		The document was noted.

R5-240595	On Alternate EIS Search Procedure with Interpolation

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd

Abstract:

CR in TDoc # matching this TDoc# + 1

Discussion:

CR in R5-240596

Decision: 		The document was noted.

R5-240601	On Re-Positioning Concept for FR2 RRM TCs

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd

Abstract:

CR in TDoc # matching this TDoc# + 1

Discussion:

"CR in R5-240602

2/23: Offline discussions are occurring.

2/28:

Moderator (AT&T): Changes in the associated CR are editorial in nature so there is no need to endorse the proposals. This Tdoc can be noted.

Noted no proposals endorsed"

Decision: 		The document was noted.

R5-240603	On Coarse&Fine Beam Peak Search Grids

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, CAICT

Abstract:

CR in TDoc # matching this TDoc# + 1

Discussion:

"CR in R5-240604

2/23: Offline discussions are occurring.

2/28:

Moderator (AT&T): R&S has confirmed that they are OK with the proposals. This Tdoc can be noted and Proposal 1 can be endorsed.

Noted and prop1 endorsed

"

Decision: 		The document was noted.

R5-240605	On 30cm QoQZ MU Topics

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd

Abstract:

CR in TDoc # matching this TDoc# + 1

Discussion:

"CR in R5-240604

2/23: Offline discussions are occurring.

2/28:

Moderator (AT&T): This Tdoc can be noted and Proposals 1, 2, 3, 4, 5, 6, and 7 can be endorsed.

Noted and proposals endorsed

"

Decision: 		The document was noted.

R5-240618	Further discussion on FR1 EVM testing in shorter transient period

Type: discussion		For: Endorsement
					38.521-1 v..
					Source: Keysight Technologies UK Ltd

Discussion:

"No associated CR

noted prop1 and 2 endorsed"

Decision: 		The document was noted.

R5-240786	FR2 RRM test cases: Known Issue List

Type: discussion		For: Information
					Source: Ericsson

Abstract:

Document for tracking FR2 RRM known issues

Discussion:

r1

Decision: 		The document was revised to R5-241867.

R5-241867	FR2 RRM test cases: Known Issue List

Type: discussion		For: Information
					Source: Ericsson

(Replaces R5-240786)

Decision: 		The document was noted.

R5-240970	Test frequencies calculation review for asymmetric channel bandwidths

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA

Abstract:

associated CRs R5-240971, R5-240972

Discussion:

Noted, proposal 1 and 3 endorsed.

Prop2 need more discussion

Decision: 		The document was noted.

R5-241051	On TRP test result variations adopting reduced grids

Type: discussion		For: Information
					38.870 v..
					Source: Apple (UK) Limited

Abstract:

Discussion TD on TRP test result variations adopting reduced grids

Decision: 		The document was noted.

R5-241078	On the QoQZ MU for n262 and TRP

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Discussion:

"2/21: Offline discussions are occurring.

2/28:

Moderator (AT&T): Some of the proposals in this paper can be addressed in the KS paper in R5-240605. This Tdoc can be noted and Proposals 4 and 5 can be endorsed.

Noted and prop4/5 endorsed"

Decision: 		The document was noted.

R5-241083	MU discussion on FR2c

Type: discussion		For: Discussion
					Source: Anritsu

Abstract:

AP#99.21; Related CR: R5-241084, R5-241085, R5-241086

Discussion:

r1

Decision: 		The document was revised to R5-241858.

R5-241858	MU discussion on FR2c

Type: discussion		For: Discussion
					Source: Anritsu

(Replaces R5-241083)

Discussion:

"Revised from: R5-241083r1.

Noted and prop1,2,3,7,9,10,11,12,13,14 and 15 endorsed"

Decision: 		The document was noted.

R5-241087	QoQZ up to FR2d

Type: discussion		For: (not specified)
					Source: Anritsu

Abstract:

AP#100.21

Discussion:

"AP#100.21

2/21: Offline discussions are occurring.

2/28:

Moderator (AT&T): This proposals in this paper can be addressed in the KS paper in R5-240605. This Tdoc can be noted."

Decision: 		The document was noted.

R5-241088	MU discussion on FR1 EVM including symbols with transient period

Type: discussion		For: (not specified)
					Source: Anritsu

Abstract:

AP#97.27

Discussion:

"AP#97.27

noted prop1-3 endorsed"

Decision: 		The document was noted.

R5-241205	EIS search interpolation methods

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Abstract:

Associated CR in R5-241206. Associated RF action point AP#101.21.

Discussion:

Associated CR in R5-241206. Associated RF action point AP#101.21.

Decision: 		The document was noted.

R5-241307	Status on Test Selection Criteria for 38.522

Type: discussion		For: Endorsement
					Source: Qualcomm Germany

Discussion:

associated CRs R5-241306(QC), R5-240932(BV), R5-241379(BV)

Decision: 		The document was noted.

R5-241344	On the MU for n259

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Abstract:

Associated update of TR 38.903 in CR R5-241345

Discussion:

"Associated update of TR 38.903 in CR R5-241345

2/21: Offline discussions are occurring.

2/28:

Moderator (AT&T): This Tdoc can be noted and Proposals 1, 2, 3, 4, and 6 can be endorsed with the assumption that the numbers in Proposal 6 are considered to be in [ ].

noted and prop 1-4 and 6 endorsed"

Decision: 		The document was noted.

R5-241356	On the MU for FR1 EVM including symbols with transient period

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Discussion:

noted prop1 and 3 endorsed

Decision: 		The document was noted.

R5-241415	On Coarse Measurement Grids

Type: discussion		For: Information
					38.561 v..
					Source: Keysight Technologies UK Ltd

Decision: 		The document was noted.

R5-241423	Views on FR1 TRP TRS TT

Type: discussion		For: Endorsement
					38.561 v..
					Source: Apple Benelux B.V.

Abstract:

Associated CR in R5-241422

Discussion:

r1

Decision: 		The document was revised to R5-241930.

R5-241930	Views on FR1 TRP TRS TT

Type: discussion		For: Endorsement
					38.561 v..
					Source: Apple Benelux B.V.

(Replaces R5-241423)

Discussion:

"Revised from: R5-241423r1.

Associated CR in R5-241422

correct TR to TS in the final

prop1 endorsed"

Decision: 		The document was noted.

R5-241424	Empirical data inputs on coarse grids

Type: discussion		For: Endorsement
					38.561 v..
					Source: Apple Benelux B.V.

Abstract:

Associated CR in R5-24XXXX

Decision: 		The document was withdrawn.

R5-241426	On Rel17 TRP TRS status updates

Type: discussion		For: Agreement
					38.561 v..
					Source: Apple Benelux B.V.

Decision: 		The document was noted.

R5-241427	Maintainence Work Plan for Rel17 TRP TRS

Type: Work Plan		For: Information
					Source: Apple Benelux B.V.

Decision: 		The document was revised to R5-241933.

R5-241933	Maintainence Work Plan for Rel17 TRP TRS

Type: Work Plan		For: Information
					Source: Apple Benelux B.V.

(Replaces R5-241427)

Decision: 		The document was noted.

R5-241445	Views on FR1 EVM test with transients

Type: discussion		For: Agreement
					38.521-1 v..
					Source: Apple Trading

Decision: 		The document was noted.

### 5.5	Routine Maintenance for LTE only TEIx_Test

#### 5.5.1	LTE RF

##### 5.5.1.1	TS 36.508

##### 5.5.1.2	TS 36.509

##### 5.5.1.3	TS 36.521-1

###### 5.5.1.3.1	Tx Requirements (Clause 6)

R5-240969	Update to freq error to refer to cat 1bis refsens values table

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5498  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Decision: 		The document was withdrawn.

R5-241057	Update to freq error cat 1bis test procedure

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5499  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Decision: 		The document was agreed.

R5-241168	Correction to Cat1bis frequency error

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5501  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

R5

overlap with R5-241057. Changes already contained there.

Decision: 		The document was withdrawn.

###### 5.5.1.3.2	Rx Requirements (Clause 7)

R5-240671	Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking

Type: CR		For: Agreement
					36.521-1 v16.11.0	  CR-5489  Cat: F (Rel-16)

					Source: Qualcomm Technologies Ireland

Abstract:

Core spec alignment. RAN 4 spec TS 36.101 -> Table 7.6.3.1-1 -> Notes 3-7 (R4-1908437) are not integrated in RAN 5 TS 36.521-1.

Decision: 		The document was withdrawn.

R5-240952	Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5493  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Abstract:

RAN 4 spec TS 36.101 -> Table 7.6.3.1-1 -> Notes 3-7 (R4-1908437) are not integrated in RAN 5 TS 36.521-1.

Discussion:

r1

Decision: 		The document was revised to R5-241925.

R5-241925	Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5493  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

(Replaces R5-240952)

Decision: 		The document was agreed.

R5-240963	Correct Table 7.3A.11.4.1-1 of CA Configurations

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5494  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

TEI16->15

r1

Decision: 		The document was revised to R5-241854.

R5-241854	Correct Table 7.3A.11.4.1-1 of CA Configurations

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5494  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-240963)

Decision: 		The document was agreed.

R5-240964	Correct Table 7.4A.8.4.1-2 & Table 7.4A.8_H.4.1-2 & Table 7.4A.9.4.1-1 of CA Configurations

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5495  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

TEI16->15

r1

Decision: 		The document was revised to R5-241855.

R5-241855	Correct Table 7.4A.8.4.1-2 & Table 7.4A.8_H.4.1-2 & Table 7.4A.9.4.1-1 of CA Configurations

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5495  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-240964)

Decision: 		The document was agreed.

R5-240965	Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5496  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

TEI16->15

r1

Decision: 		The document was revised to R5-241856.

R5-241856	Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5496  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-240965)

Decision: 		The document was agreed.

R5-240966	Correct typos in Table 7.3A.5.4.1-1

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5497  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

+LTE_CA..

r1

Decision: 		The document was revised to R5-241857.

R5-241857	Correct typos in Table 7.3A.5.4.1-1

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5497  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-240966)

Decision: 		The document was agreed.

R5-241089	Editorial correction to CA_1A-3A-7A-28A in 7.3A.9

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5500  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Editorial

Decision: 		The document was agreed.

###### 5.5.1.3.3	Clauses 1-5 / 8-10 / Annexes

R5-240071	Update of Annex F Measurement Uncertainties in TS 36.521-1

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5487  Cat: F (Rel-18)

					Source: CMCC, Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-240614	E-UTRA V2X spurious emissions MU definition up to 26GHz

Type: CR		For: Agreement
					36.521-1 v18.3.0	  CR-5488  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

##### 5.5.1.4	TS 36.521-2

R5-240770	Correction to applicability for handover test cases

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1023  Cat: F (Rel-18)

					Source: TTA

Decision: 		The document was agreed.

R5-241203	Add information in Power Control test cases specific to Power Class 3

Type: CR		For: Agreement
					36.521-2 v18.3.0	  CR-1026  Cat: F (Rel-18)

					Source: Sporton

Decision: 		The document was agreed.

##### 5.5.1.5	TS 36.521-3

##### 5.5.1.6	RRM Test & Radio Reception Test Tolerances

###### 5.5.1.6.1	TR 36.903 (E-UTRAN RRM TT analyses)

###### 5.5.1.6.2	TR 36.904 (E-UTRAN Radio Reception TT analyses)

###### 5.5.1.6.3	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )

##### 5.5.1.7	TS 34.121-1

##### 5.5.1.8	TS 34.121-2

##### 5.5.1.9	TS 34.122

##### 5.5.1.10	TS 34.108

##### 5.5.1.11	TR 34.902 (UTRAN RRM Test Tolerance analyses)

##### 5.5.1.12	Discussion Papers / Work Plan / TC lists

### 5.6	Other Routine Maintenance TEIx_Test

#### 5.6.1	TS 34.108

#### 5.6.2	TS 34.121-1 All sections other than annexes

#### 5.6.3	TS 34.121-1 Annexes only

#### 5.6.4	TS 34.121-2

#### 5.6.5	TS 34.122

#### 5.6.6	TS 34.171

#### 5.6.7	TS 34.172

#### 5.6.8	TS 34.114

#### 5.6.9	TS 37.571-1

#### 5.6.10	TS 37.571-3

#### 5.6.11	TS 37.571-5

#### 5.6.12	TS 51.010-1 (RF/Performance)

R5-241342	Correction of coding scheme in USF testing

Type: CR		For: Agreement
					51.010-1 v13.13.0	  CR-5170  Cat: F (Rel-13)

					Source: ROHDE & SCHWARZ, Apple

Decision: 		The document was agreed.

#### 5.6.13	TS 51.010-2 (RF/Performance)

#### 5.6.14	TS 51.010-7 (RF/Performance)

#### 5.6.15	TS 37.544

R5-241207	CR on Measurement Distance

Type: CR		For: Agreement
					37.544 v16.2.0	  CR-0032  Cat: F (Rel-16)

					Source: ROHDE & SCHWARZ

Discussion:

cl. aff.

r1

Decision: 		The document was revised to R5-241940.

R5-241940	CR on Measurement Distance

Type: CR		For: Agreement
					37.544 v16.2.0	  CR-0032  rev 1 Cat: F (Rel-16)

					Source: ROHDE & SCHWARZ

(Replaces R5-241207)

Decision: 		The document was agreed.

#### 5.6.16	TR 37.901

#### 5.6.17	TR 37.901-5

R5-240860	Update SNR conditions in UDP and TCP FR2 FRC scenarios

Type: CR		For: Agreement
					37.901-5 v16.11.0	  CR-0038  Cat: F (Rel-16)

					Source: Keysight Technologies

Discussion:

3GU bug

Decision: 		The document was agreed.

R5-241410	General cleanup of annex for app layer tput test cases

Type: CR		For: Agreement
					37.901-5 v16.11.0	  CR-0039  Cat: F (Rel-16)

					Source: QUALCOMM Europe Inc. - Italy

Discussion:

3GU bug

r1

Decision: 		The document was revised to R5-241852.

R5-241852	General cleanup of annex for app layer tput test cases

Type: CR		For: Agreement
					37.901-5 v16.11.0	  CR-0039  rev 1 Cat: F (Rel-16)

					Source: QUALCOMM Europe Inc. - Italy

(Replaces R5-241410)

Decision: 		The document was agreed.

#### 5.6.18	TR 38.918

#### 5.6.19	Discussion Papers / Work Plan / TC lists

### 5.7	Outgoing liaison statements for provisional approval

R5-241876	LS reply on FR2 ACS/IBB testing

Type: LS out		For: Approval
					to TSG WG RAN4
					Source: TSG WG RAN5

Abstract:

RAN5 thanks RAN4 for the LS. RAN5 has discussed it over the attached discussion paper [2] and has made the following observations:

•	ACS/IBB test cases do not require to test at EIS level but a specific total DL power for the wanted signal and the modulated interferer respectively.

•	When a specific total DL power is required in a test, it is expected that power is equally split between both polarizations.

•	When blocking measurement procedure in section K.1.8 in [4] was defined, it was decided not to test both polarizations at the same time and to add a clarification indicating that each polarization needs to transmit 3dB below the level stated in the requirement.

•	Testing both polarizations simultaneously or sequentially should not imply changes in the power levels to test.

RAN5 concludes that current 3dB power backoff per polarization applied in the blocking measurement procedure defined in section K.1.8 in [3] is correct. However, for wanted signal in particular, section K.1.8 in [3], step 5, talks about EIS level which is incorrect and misleading, and that has probably driven into the LS request in [1].

To resolve the ambiguity, RAN5 has agreed to remove “EIS” term in attached CR [3] as follows:

5)	Apply a signal with the specified reference measurement channel on the θ-polarization, setting the power level of the signal 3dB below the EIS level stated in the requirement.

6)	Apply the blocking signal with the same polarization and coming from the same direction as the downlink signal. Set the power level of the blocking signal 3dB below the level stated in the requirement.

2. Actions: To RAN4 group: RAN5 asks RAN4 to consider the observations and attachments shared in this LS explaining RAN5 view on why no changes are required in the blocking measurement procedure other than the clarification above.

Discussion:

Adan (sent)

Decision: 		The document was approved.

R5-241956	LS to RAN4 on TT work for Rel-18 NR FR1 TRP TRS

Type: LS out		For: Approval
					to TSG WG RAN4
					Source: TSG WG RAN5

Abstract:

Following the finalization of the RAN4 Rel-18 NR FR1 TRP TRS WI core and performance parts, RAN5 will subsequently introduce a new WID to deliver conformance test cases for Rel-18 NR FR1 TRP and TRS requirements. It is general process that RAN5 will have primary responsibility for Measurement Uncertainty (MU) finalization for Rel-18 FR1 TRP/TRS for which a preliminary/placeholder MU table has been added in Annex B of TR 38.870 and TS 38.161. Further optimizing and finalizing MU values and then determine Test Tolerance (TT) for the defined test requirements will occur in RAN5 and incorporated in TS 38.561.

With regards to the derivation of TT for OTA testing, there has been precedence of RAN4 kindly providing recommendations on TT along with core requirements, methodology and MU.

1)	[2] shows the earliest such precedence wherein the “TRP and TRS OTA performance requirements, measurement uncertainty and test tolerance values are agreed as package as summarized” in [3] as inputs to TS 34.114 [5], while allowing for further refinements to MU in RAN5.

2)	The above precedence was continued in [4] wherein RAN4 provided test tolerance recommendations for LTE MIMO OTA.

3)	For 5G NR FR2 OTA, due to ecosystem needs RAN4 shifted the MU work to RAN5 as per [6] in response to the RAN5 LS in [7] and RAN5 picked up all subsequent TT definitions.

4)	As recently as Release 17 for 5G NR FR1 TRP/TRS, the approach from 1) and 2) was continued and as per RAN5 request in [8] RAN4 provided the test tolerance recommendations in [9] to be utilized as a package together with Rel-17 core requirements.

Based on the process being followed in RAN4 to derive NR FR1 TRP TRS requirements built on performance campaign test results, RAN5 understands that there is a deep relation between radiated performance requirement definition in RAN4, derived from test campaign results, and TT as indicated by the precedence in 1), 2) and 4) above.

Therefore, RAN5 requests RAN4 to provide recommendations on test tolerance values for Release 18 NR FR1 TRP and TRS enhancements along with a thorough description of the core requirements definition process so implications from MU and TT can be clarified and considered during the definition of Rel-18 FR1 TRP TRS TT in the RAN5 conformance test specification.

2	Actions

To RAN4: RAN5 asks RAN4 to take the above information into account and provide RAN5 with recommendations on test tolerance values for Release 18 NR FR1 TRP and TRS along with a thorough description of the core requirements definition process so that implications from MU and TT can be clarified and considered during the definition of Release 18 FR1 TRP TRS TT in RAN5 conformance test specification.

Discussion:

Ashwin 2

Decision: 		The document was approved.

R5-241957	LS on 3GPP 5G NR FR1 OTA Conformance Test Specification

Type: LS out		For: Approval
					to ETSI MSG TFES, GCF CAG, GCF PAG, CTIA OTA WG, CTIA Certification, GSMA TSG-AP, NGMN Alliance, PTCRB plenary, CCSA TC9 WG1, cc GCF SG
					Source: TSG WG RAN5

Abstract:

TSG RAN WG5 (RAN5) would like to inform the industry on the publishing of the UE conformance test specification for 5G NR FR1 TRP TRS in TS 38.561 [1].

RAN5 has successfully concluded the Release 17 UE Conformance Test Work item on NR FR1 TRP TRS UE conformance test with the key outcome that RAN5 has specified the UE conformance test procedures, associated permitted test methods, test requirements (with test tolerances) for NR FR1 TRP TRS in the published Technical Specification TS 38.561 [1].

In the RAN5 Release 17 UE Conformance Test Work item, the conformance test requirements have been defined for Browsing mode (testing with Hand Phantom only) for bands n41 and n78. In addition, the defined conformance test requirements cover Power Class 2 handheld devices wider than 72mm and narrower than or equal to 92 mm.

3GPP TSG RAN WG5 (RAN5) asks all the standardization and certification bodies to take into consideration the completed Release 17 5G NR FR1 OTA UE conformance test specifications defined in TS 38.561 [1].

2	Actions

To all the OTA standardization and certification bodies: 3GPP TSG RAN WG5 (RAN5) asks all the standardization and certification bodies to take into consideration the completed Release 17 5G NR FR1 OTA UE conformance test specifications defined in TS 38.561.

Discussion:

Ashwin 1

+GCF SG

-RAN4

Decision: 		The document was approved.

### 5.8	AOB

## 6	Signalling Protocol Functional Area

### 6.1	Review action points (fm A.I. 2.1)

### 6.2	Review incoming LS (fm A.I. 3) & new subject discussion papers

R5-240018	LS on Prohibition of GEA1 & GEA2 Support in all releases

Type: LS in		For: Action
					Original outgoing LS: SP-231782, to TSG RAN WG5, GCF SG, cc TSG WG SA3
					Source: TSG SA

Abstract:

3GPP SA would like to inform GCF that they have agreed to prohibit the implementation of GEA1 and GEA 2 by Mobile Equipments of all Releases.

As Release 7 and earlier Releases are closed releases, the following approach has been adopted.

•	The Rel-6 and Rel-7 CRs on TS 43.020 in SP-231344 are noted.

•	The Rel-8 to Rel-15 CRs on TS 43.020 in SP-231344 are approved with WI code TEI6.

•	The comment “GEA1/GEA2 are prohibited and shall not be implemented.” Is to be added to the latest versions for releases 4, 5, 6, 7 of TS 43.020 in the specification website (https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=2663)

•	The comment “GEA1/GEA2 are prohibited and shall not be implemented.” Is to be added to the latest versions of TS 03.20 for Releases 97, 98, 99, and 2000 in the specification website (https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=127).  Note that GPRS did not exist in Release 96 or earlier.

As a consequence of the above, TSG-SA request that RAN 5 update their test specifications and test that mobile equipment do not implement GEA 1 / GEA 2

Actions

To RAN 5: ACTION: TSG-SA requests RAN 5 to update the 3GPP test specifications to enable verification that GEA 1 / GEA 2 are not implemented by MEs.

Discussion:

moved to SIG.

Decision: 		The document was noted.

### 6.3	Open Work Items

#### 6.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083)  NR_CADC_NR_LTE_DC_R16-UEConTest

##### 6.3.1.1	TS 38.508-1

R5-241525	Addition of signalling test frequencies for DC_2A_n2A and DC_66A_n66A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3105  Cat: F (Rel-18)

					Source: WE Certification, AT&T

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241580.

R5-241580	Addition of signalling test frequencies for DC_2A_n2A and DC_66A_n66A

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3105  rev 1 Cat: F (Rel-18)

					Source: WE Certification, AT&T

(Replaces R5-241525)

Decision: 		The document was agreed.

##### 6.3.1.2	TS 38.508-2

##### 6.3.1.3	TS 38.523-1

##### 6.3.1.4	TS 38.523-2

##### 6.3.1.5	TS 38.523-3

##### 6.3.1.6	Discussion Papers / Work Plan / TC lists

#### 6.3.2	5G V2X with NR sidelink (UID-880069)  5G_V2X_NRSL_eV2XARC-UEConTest

##### 6.3.2.1	TS 38.508-1

R5-240432	Move all V2X sub-clauses from 4.7 into a new clause 4.7D

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3030  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, CATT

Discussion:

late doc

CR coversheet:  Rel-17

r1

Decision: 		The document was revised to R5-241538.

R5-241538	Move all V2X sub-clauses from 4.7 into a new clause 4.7D

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3030  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, CATT

(Replaces R5-240432)

Decision: 		The document was agreed.

R5-240764	Correction to default configuration of frequencyInfoSL

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3067  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241539.

##### 6.3.2.2	TS 38.508-2

##### 6.3.2.3	TS 38.509

##### 6.3.2.4	TS 38.523-1

R5-240433	Editorial update the derivation path for the V2X message contents table

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4226  Cat: F (Rel-17)

					Source: Huawei, HiSilicon, CATT

Discussion:

late doc

Decision: 		The document was agreed.

R5-240529	Updates to 5G V2X test case 12.1.7.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4241  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240765	Correction to V2X SIG test case 12.1.7.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4273  Cat: F (Rel-17)

					Source: Huawei,HiSilicon

Discussion:

solve the overlapping with R5-240529.

Decision: 		The document was withdrawn.

R5-240766	Addition of V2X SIG test case 12.2.9.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4274  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 6.3.2.5	TS 38.523-2

R5-240767	Correction of applicability for V2X SIG test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0444  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241540.

R5-241540	Correction of applicability for V2X SIG test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0444  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240767)

Decision: 		The document was agreed.

##### 6.3.2.6	TS 38.523-3

R5-240530	5G V2X: Test Model updates

Type: CR		For: Agreement
					38.523-3 v17.9.0	  CR-3375  Cat: F (Rel-17)

					Source: MCC TF160

Discussion:

r1

Decision: 		The document was revised to R5-241541.

R5-241541	5G V2X: Test Model updates

Type: CR		For: Agreement
					38.523-3 v17.9.0	  CR-3375  rev 1 Cat: F (Rel-17)

					Source: MCC TF160

(Replaces R5-240530)

Decision: 		The document was agreed.

##### 6.3.2.7	TS 36.509

##### 6.3.2.8	TS 37.571-4

##### 6.3.2.9	Discussion Papers / Work Plan / TC lists

#### 6.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest

##### 6.3.3.1	TS 38.508-1

##### 6.3.3.2	TS 38.508-2

##### 6.3.3.3	TS 38.523-1

##### 6.3.3.4	TS 38.523-2

##### 6.3.3.5	TS 38.523-3

##### 6.3.3.6	Discussion Papers / Work Plan / TC lists

#### 6.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest

##### 6.3.4.1	TS 38.508-1

R5-240989	SIB2 updates for NR unlicensed test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3081  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.4.2	TS 38.508-2

##### 6.3.4.3	TS 38.509

##### 6.3.4.4	TS 38.523-1

R5-240990	Addition of NR shared spectrum measurements test case 8.1.8.1.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4283  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-241035	Updates to NR shared spectrum MAC test case 7.1.1.10.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4284  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-241542.

R5-241542	Updates to NR shared spectrum MAC test case 7.1.1.10.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4284  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-241035)

Decision: 		The document was agreed.

R5-241036	Updates to NR shared spectrum idle mode test cases

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4285  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-241038	Updates to NR shared spectrum page monitoring test cases

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4287  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-241039	Updates to NR shared spectrum RRC test cases

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4288  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.4.5	TS 38.523-2

R5-240991	Applicability updates to NR shared spectrum test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0446  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.4.6	TS 38.523-3

R5-240531	NR-U: Introduction of Test Model aspects

Type: CR		For: Agreement
					38.523-3 v17.9.0	  CR-3376  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.3.4.7	Discussion Papers / Work Plan / TC lists

##### 6.3.5.	NR coverage enhancements (UID-950063) NR_cov_enh-UEConTest

##### 6.3.5.1	TS 38.508-1

##### 6.3.5.2	TS 38.508-2

##### 6.3.5.3	TS 38.523-1

R5-241044	Corrections to CovEnh TC 7.1.1.1.18

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4289  Cat: F (Rel-17)

					Source: Lenovo, MCC TF160

Decision: 		The document was agreed.

##### 6.3.5.4	TS 38.523-2

##### 6.3.5.5	TS 38.523-3

##### 6.3.5.6	Discussion Papers / Work Plan / TC lists

#### 6.3.6	Support of reduced capability NR devices (UID-950066) NR_redcap_plus_ARCH-UEConTest

##### 6.3.6.1	TS 38.508-1

##### 6.3.6.2	TS 38.508-2

##### 6.3.6.3	TS 38.523-1

R5-240151	Correction to RedCap MAC TC 7.1.1.8.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4165  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240284	Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4202  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240285	Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4203  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240286	Update URLLC TC 7.1.1.4.1.5 to test RedCap UE

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4204  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240287	Update URLLC TC 7.1.1.4.2.6 to test RedCap UE

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4205  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240441	Correction to redcap TC 6.1.2.27

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4233  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was agreed.

R5-240473	Correction to eDRX test case 11.7.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4236  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-240511	Correction to NR5GC eDRX testcase 11.7.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4239  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-240657	Corrections to NR eDRX test case 11.7.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4270  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD, Keysight Technologies

Decision: 		The document was agreed.

R5-241377	Correction to eDRX test case 11.7.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4307  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

##### 6.3.6.4	TS 38.523-2

6.3.6.5	TS 38.523-3

##### 6.3.6.6	TS 34.229-2

R5-241411	Updating applicability for RedCap

Type: CR		For: Agreement
					34.229-2 v16.9.0	  CR-0337  Cat: F (Rel-17)

					Source: MediaTek Inc.

Abstract:

triggers a Rel upgrade

Discussion:

late doc

r1

TF160 manager: video changes are not part of RedCap. Put it in a separate CR.

Part -5 will be also upgraded.

Deferred.

r3

Decision: 		The document was revised to R5-241578.

R5-241578	Updating applicability for RedCap

Type: CR		For: Agreement
					34.229-2 v16.9.0	  CR-0337  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-241411)

Decision: 		The document was agreed.

##### 6.3.6.7	Discussion Papers / Work Plan / TC lists

R5-240865	Discussion on handling R16/R17 features for RedCap UE

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Abstract:

Thanks to R5-237399 and R5-237400, RAN5 already reached a big progress on proceeding the R16/17 testcases applicable to RedCap UE in RAN5#101 meeting.

However as per the info including the RAN5 prose, TF160 status and GCF status, R16/R17 seems have a different situation with R15.

2	Discussion

When we were reviewing all R15 testcases which we all think should be the baseline of 5G devices, the RAN5 R15 WID was already closed and most testcases has been verified, we have enough experience to analyse the testcases steps to figure out whether the TC is applicable to RedCap UE and if we need to update the prose or not, so that we can make the analysis results as much accurate and stable as possible.

However for R16, some feature, such as V2X, is still ongoing, that means some new testcases are on the way. And some feature testcases are still in the status “compilable” or “not avaliable” in TF160 status. That means for those testcases, we cannot ensure the analysis result is correct enough as we don’t know those testcases detailed enough and have so much experience.

As for R17, the situation seems to get worse. More WIs are still ongoing, and less testcases are verified than R16.

Even for R18, there may be some UE supporting RedCap+R18 features. In the future, we always have to look at every new testcase coming to RAN5 and figure out if it is applicable to RedCap UE, whatever it belongs to R16, R17, even R18+.

So how to make the analysis result captured in RAN5 prose is as much accurate enough as possible, and I hope we can find an approach to minimize RAN5 delegates work to trace all of the information.

3	Proposal

The following is proposed:

Proposal 1: Create a new sub-clause 4.3.2 and add an applicability table for R16+, and keep R15 as it is.

Proposal 2: Create a new sub-clause 4.3.2 and add an applicability table for R16+, and keep R15 aligned with R16+ to add all R15 RedCap UE applicable TCs to the table.

Discussion:

TF160 manager: the work to upgrade Rel-16 test cases to RedCap is not much.

The meeting agreed to add a list of dedicated test cases, but how to implement should be further studied. Any implementation must not impact the TTCN reference implementation.

Decision: 		The document was noted.

R5-241409	Discussion paper for handling Rel-16 features and test cases for Redcap Only UEs

Type: discussion		For: Decision
					Source: MediaTek Inc.

Abstract:

At RAN5#100 [1] “Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE” was agreed that introduces the way forward on how to handle Rel-16 and Rel-17 protocol (TS 38.523-1) legacy test cases applicability to Redcap UEs.

At RAN5#102 [2] “Discussion paper for selecting optional Rel-16 features and test case for Redcap Only UE” has been agreed that all Rel-17 test cases should be upgraded and only a date needs to be decided.

At RAN2#124 [3] “Discussion on SON/MDT reports for eRedCap” has been discussed.

2.	Discussion

Below is a list of Rel-16 legacy features with protocol test cases. Features excluded from the below list:

•	features with no test cases

•	features not applicable to RedCap:DC,CA and ENDC  (for full list of restrictions see clause 4.2.21.1 in TS 38.306)

•	Conformance Test Aspects - Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData)

RAN2 discussion in [3] may impact how SON/MDT test cases need to be updated.

There is a large number of test cases that need to be updated and there is no industry interest in all the features. Also maybe not all the test cases are required per feature.

There are 2 main options for updating test cases:

1)	update all Rel-16 test cases for RedCap or

1)	only consider for update those Rel-16 test cases that belong to features with industry interest. The test case list might be further reduced.

3.	Proposal

Proposal 1:	Track list of updated Rel-16 test cases in the attached spreadsheet.

Proposal 21:	Postpone updating SON/MDT test cases until discussion in [3] concludes.

Proposal 2: Decide on the list of Rel-16 test cases to be considered for RedCap update based on the information about the industry interest, WI completeness and test case approval status.

Discussion:

r2

Proposal for any update to the spec being contribution driven based on industry interest is accepted.

Decision: 		The document was revised to R5-241572.

R5-241572	Discussion paper for handling Rel-16 features and test cases for Redcap Only UEs

Type: discussion		For: Decision
					Source: MediaTek Inc.

(Replaces R5-241409)

Decision: 		The document was noted.

R5-241412	Discussion paper for IMS test cases over Redcap Only UE

Type: discussion		For: Decision
					Source: MediaTek Inc.

Abstract:

At RAN5#100 [1] “Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE” was agreed that introduces the way forward on how to handle Rel-16 and Rel-17 protocol (TS 38.523-1) legacy test cases applicability to Redcap UEs.

At RAN5#1021 [2] “Discussion paper for selecting optional Rel-16 features and test case for Redcap Only UE” has been agreed that all Rel-17 test cases should be upgraded and only a date needs to be decided.

Rel-15 IMS test cases have been already updated to run on RedCap devices (there is no NOTE stating otherwise is added to TS 34.229-2).

2.	Discussion

Given the different form factors used by different RedCap devices sending and/or receiving short messages might not be possible.

At the moment support of these features is mandated by NG.114 as per TS 34.29-2 Table A.3A:

“3.	Proposal

Proposal 1:	Clarify that SM-over-IP sender/receiver is optional for RedCap devices although NG.114 mandates it.

Discussion:

r1

Decision: 		The document was revised to R5-241573.

R5-241573	Discussion paper for IMS test cases over Redcap Only UE

Type: discussion		For: Decision
					Source: MediaTek Inc.

(Replaces R5-241412)

Decision: 		The document was noted.

#### 6.3.7	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest

##### 6.3.7.1	TS 38.508-1

##### 6.3.7.2	TS 38.508-2

R5-240590	Modification of testcase 8.1.5.13.2 applicability clauses

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0588  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

##### 6.3.7.3	TS 38.523-1

R5-240417	Correction of SDT TC 7.1.1.13.1-2-Step RACH

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4223  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

merged into R5-240532

Decision: 		The document was withdrawn.

R5-240532	Corrections to SDT TC 7.1.1.13.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4242  Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

Discussion:

r1

Decision: 		The document was revised to R5-241543.

R5-241543	Corrections to SDT TC 7.1.1.13.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4242  rev 1 Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

(Replaces R5-240532)

Decision: 		The document was agreed.

R5-240533	Corrections to SDT TC 7.1.1.13.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4243  Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

Discussion:

offline comments from Lenovo & Qualcomm

r1

Decision: 		The document was revised to R5-241544.

R5-241544	Corrections to SDT TC 7.1.1.13.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4243  rev 1 Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

(Replaces R5-240533)

Decision: 		The document was agreed.

R5-240534	Corrections to SDT TC 7.1.1.13.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4244  Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

Discussion:

r1

Deferred.

r2

Decision: 		The document was revised to R5-241545.

R5-241545	Corrections to SDT TC 7.1.1.13.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4244  rev 1 Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

(Replaces R5-240534)

Decision: 		The document was agreed.

R5-240535	Corrections to SDT TC 7.1.1.13.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4245  Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

Decision: 		The document was agreed.

R5-240589	Modification of testcase 8.1.5.13.2 for Data on non-SDT Radio Bearers for NR SmallData

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4254  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

r1

Decision: 		The document was revised to R5-241546.

R5-241546	Modification of testcase 8.1.5.13.2 for Data on non-SDT Radio Bearers for NR SmallData

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4254  rev 1 Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240589)

Decision: 		The document was agreed.

##### 6.3.7.4	TS 38.523-2

R5-240628	Modification of testcase 8.1.5.13.2 applicability clauses

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0438  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

##### 6.3.7.5	TS 38.523-3

##### 6.3.7.6	Discussion Papers / Work Plan / TC lists

#### 6.3.8	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest

##### 6.3.8.1	TS 38.508-1

R5-240630	Update SIB1

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3061  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

maybe w/d

Clause specifiying use of NTN condition will be defined in TS 38.508-1.

Decision: 		The document was withdrawn.

R5-240648	Updates to Ephemeris Info for Multi-Cell NRN-NTN signalling test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3062  Cat: F (Rel-18)

					Source: QUALCOMM JAPAN LLC.

Discussion:

CR cover value : Rel-17.

r1

Decision: 		The document was revised to R5-241561.

R5-241561	Updates to Ephemeris Info for Multi-Cell NRN-NTN signalling test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3062  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM JAPAN LLC.

(Replaces R5-240648)

Decision: 		The document was agreed.

R5-240649	Updates to SIB19 Common Config for NR-NTN test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3063  Cat: F (Rel-18)

					Source: QUALCOMM JAPAN LLC

Discussion:

r2

Decision: 		The document was revised to R5-241565.

R5-241565	Updates to SIB19 Common Config for NR-NTN test cases

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3063  rev 1 Cat: F (Rel-18)

					Source: QUALCOMM JAPAN LLC

(Replaces R5-240649)

Decision: 		The document was agreed.

##### 6.3.8.2	TS 38.508-2

R5-240207	Correction to NR NTN PICS Mnemonic

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0568  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240672	Addition of new PICS for NTN Event D1

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0590  Cat: F (Rel-18)

					Source: Qualcomm CDMA Technologies

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240984	Addition of NR NTN TA reporting PICS

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0597  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

r2

Decision: 		The document was revised to R5-241649.

R5-241649	Addition of NR NTN TA reporting PICS

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0597  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-240984)

Decision: 		The document was agreed.

##### 6.3.8.3	TS 38.509

##### 6.3.8.4	TS 38.523-1

R5-240631	Update test case 6.7.1.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4260  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-240632	Update test case 6.7.1.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4261  Cat: F (Rel-17)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-240650	Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4265  Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

Discussion:

r3

Decision: 		The document was revised to R5-241566.

R5-241566	Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4265  rev 1 Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

(Replaces R5-240650)

Decision: 		The document was agreed.

R5-240651	Updates to cell configuration for NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4266  Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

Discussion:

r2

Decision: 		The document was revised to R5-241567.

R5-241567	Updates to cell configuration for NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4266  rev 1 Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

(Replaces R5-240651)

Decision: 		The document was agreed.

R5-240652	Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4267  Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240653	Addition of NTN / Mobility registration update / supported TACs not part of UE registration area

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4268  Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

Discussion:

CR cover value : 3913.

r1

TF160 manager: editorial issues.

ETSI MCC: heading formats.

Decision: 		The document was revised to R5-241568.

R5-241568	Addition of NTN / Mobility registration update / supported TACs not part of UE registration area

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4268  rev 1 Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

(Replaces R5-240653)

Decision: 		The document was agreed.

R5-240659	Introduction of new NR NTN RRC TC for Event D1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4272  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies,

Discussion:

cl. nr. .x needs to be resolved.

Motorola Mobilit: Even->t.

ETSI MCC: pls. renumber last 2 tables.

r1

Decision: 		The document was revised to R5-241569.

R5-241569	Introduction of new NR NTN RRC TC for Event D1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4272  rev 1 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies,

(Replaces R5-240659)

Decision: 		The document was agreed.

R5-240985	Addition of new NR NTN MAC TA reporting test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4280  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-240986	Addition of new NR NTN RLC t-reassembly timer test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4281  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-241570.

R5-241570	Addition of new NR NTN RLC t-reassembly timer test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4281  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-240986)

Decision: 		The document was agreed.

R5-240987	Addition of NR NTN PDCP discard timer test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4282  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was revised to R5-241571.

R5-241571	Addition of NR NTN PDCP discard timer test case

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4282  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-240987)

Decision: 		The document was agreed.

##### 6.3.8.5	TS 38.523-2

R5-240654	Applicability updates for NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0440  Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240655	Applicability updates for NTN / Mobility registration update / supported TACs not part of UE registration area

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0441  Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240660	Applicability of New NR NTN TC for Event D1

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0442  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies

Discussion:

CR cover value : 0422.

CR cover value : NR_NTN_solutions_plus_CT-UEConTest! 3GU: TEI15..

r2

Decision: 		The document was revised to R5-241652.

R5-241652	Applicability of New NR NTN TC for Event D1

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0442  rev 1 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies

(Replaces R5-240660)

Decision: 		The document was agreed.

R5-240988	Applicability updates to NR NTN test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0445  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-241650.

R5-241650	Applicability updates to NR NTN test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0445  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-240988)

Decision: 		The document was agreed.

R5-241414	Applicability updates for new NTN Idle mode and NAS test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0450  Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

Discussion:

Tdoc, CR#

r2

TF160 manager: NTN Idle mode test cases need to be aligned.

Decision: 		The document was revised to R5-241651.

R5-241651	Applicability updates for new NTN Idle mode and NAS test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0450  rev 1 Cat: F (Rel-17)

					Source: QUALCOMM JAPAN LLC.

(Replaces R5-241414)

Decision: 		The document was agreed.

##### 6.3.8.6	TS 38.523-3

##### 6.3.8.7	Discussion Papers / Work Plan / TC lists

#### 6.3.9	Enhancement of Private Network Support for NG-RAN including CT aspects (UID-960076) NG_RAN_PRN_enh_plus_CT-UEConTest

##### 6.3.9.1	TS 38.508-1

R5-240349	Correction of Default parameters for simulated SNPN cells.

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3027  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

TF160/Anritsu: Cells 32 & 33 are only used by RF.

Decision: 		The document was agreed.

##### 6.3.9.2	TS 38.508-2

R5-240350	Addition of capability for UEs to support steering of roaming SNPN selection information (SOR-SNPN-SI) and steering of roaming connected mode control information (SOR-CMCI) for Rel-17 eNPN

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0578  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-241581.

R5-241581	Addition of capability for UEs to support steering of roaming SNPN selection information (SOR-SNPN-SI) and steering of roaming connected mode control information (SOR-CMCI) for Rel-17 eNPN

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0578  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-240350)

Decision: 		The document was agreed.

##### 6.3.9.3	TS 38.523-1

R5-240351	Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4209  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Rohde&Schwarz: the test purpose is contradictory to the core requirement of SNPN. The test case should be reworked.

Decision: 		The document was revised to R5-241582.

R5-241582	Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4209  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240351)

Decision: 		The document was withdrawn.

R5-240352	Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4210  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-241583.

R5-241583	Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4210  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240352)

Decision: 		The document was withdrawn.

R5-240353	Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4211  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-241584.

R5-241584	Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4211  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240353)

Decision: 		The document was withdrawn.

R5-240354	Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4212  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-241585.

R5-241585	Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4212  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240354)

Decision: 		The document was withdrawn.

R5-240355	Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4213  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Rhode&Schwarz: need more info on what is a credentials holder.

Deferred.

Decision: 		The document was revised to R5-241586.

R5-241586	Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4213  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240355)

Decision: 		The document was withdrawn.

R5-240356	Addition of new test case 9.1.11.5 for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4214  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-241587.

R5-241587	Addition of new test case 9.1.11.5 for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4214  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240356)

Decision: 		The document was agreed.

R5-240357	Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4215  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Rohde&Schwarz: need more info on what is a credentials holder. Is it set by AT or MMI command or else?

ETSI MCC: pls. check 3GPP styles.

Decision: 		The document was revised to R5-241588.

R5-241588	Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4215  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240357)

Decision: 		The document was withdrawn.

R5-240358	Addition of new test case 9.1.11.7 for Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4216  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

similar to 357.

3GPP Formats!

Deferred.

Decision: 		The document was withdrawn.

R5-241184	New Emergency test case 11.4.15

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4302  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

cl. aff.

step(s) 2

Decision: 		The document was revised to R5-241590.

R5-241590	New Emergency test case 11.4.15

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4302  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-241184)

Decision: 		The document was agreed.

##### 6.3.9.4	TS 38.523-2

R5-240359	Addition of applicability for eNPN test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0433  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r2

Decision: 		The document was revised to R5-241646.

R5-241646	Addition of applicability for eNPN test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0433  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240359)

Decision: 		The document was agreed.

R5-240360	Scoping NR SA applicable TCs for SNPN-only UEs

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0434  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

need to distinguish Rel-17.

r1

Decision: 		The document was revised to R5-241589.

R5-241589	Scoping NR SA applicable TCs for SNPN-only UEs

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0434  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240360)

Decision: 		The document was agreed.

R5-241185	Applicability for new test case 11.4.15

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0449  Cat: F (Rel-17)

					Source: Ericsson

Decision: 		The document was agreed.

##### 6.3.9.5	TS 38.523-3

##### 6.3.9.6	Discussion Papers / Work Plan / TC lists

#### 6.3.10	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest

##### 6.3.10.1	TS 38.508-1

##### 6.3.10.2	TS 38.508-2

##### 6.3.10.3	TS 38.523-1

R5-240152	Correction of case title for TC 7.1.1.1.19

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4166  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

##### 6.3.10.4	TS 38.523-2

R5-241451	Correction of applicability for partial sounding test case

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0451  Cat: F (Rel-17)

					Source: Huawei, HiSilicon, TF160

Discussion:

late doc

Decision: 		The document was agreed.

##### 6.3.10.5	TS 38.523-3

##### 6.3.10.6	Discussion Papers / Work Plan / TC lists

#### 6.3.11	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest

##### 6.3.11.1	TS 38.508-1

R5-240153	Correction to RRCReestablishment message

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3003  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240478	Addition of test procedure for 5G ProSe U2N Relay Discovery

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3038  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

r1

RAN5 Chair: pls. resolve .Y

ETSI MCC: Grammar in Reaon for change.

Decision: 		The document was revised to R5-241597.

R5-241597	Addition of test procedure for 5G ProSe U2N Relay Discovery

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3038  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-240478)

Decision: 		The document was agreed.

R5-240479	Addition of test procedure for remote Initial Access procedure under NR sidelink U2N Relay / Remote UE side

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3039  Cat: F (Rel-18)

					Source: CATT, TDIA

Discussion:

r1

pls. resolve the .X

ETSI MCC: Grammar in Reaon for change.

Decision: 		The document was revised to R5-241598.

R5-241598	Addition of test procedure for remote Initial Access procedure under NR sidelink U2N Relay / Remote UE side

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3039  rev 1 Cat: F (Rel-18)

					Source: CATT, TDIA

(Replaces R5-240479)

Decision: 		The document was agreed.

R5-240480	Removal of test procedure for Out_of_Coverage_with_Relay

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3040  Cat: F (Rel-18)

					Source: CATT, TDIA

Decision: 		The document was agreed.

R5-240481	Update test procedure for registration of NR sidelink U2N Relay / Relay UE side

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3041  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

r1

Decision: 		The document was revised to R5-241599.

R5-241599	Update test procedure for registration of NR sidelink U2N Relay / Relay UE side

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3041  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-240481)

Decision: 		The document was agreed.

R5-240482	Update of 5G ProSe information elements of UE policies for 5G ProSe usage information reporting

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3042  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-240483	Update of 5G ProSe information elements of UE policies for 5G ProSe remote

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3043  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-240484	Addition of PROSE PC5 DISCOVERY

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3044  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

##### 6.3.11.2	TS 38.508-2

R5-240477	Addition of new pics for NR sidelink U2N Relay PDU session establishment

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0584  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

12->xy

Decision: 		The document was revised to R5-241600.

R5-241600	Addition of new pics for NR sidelink U2N Relay PDU session establishment

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0584  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-240477)

Decision: 		The document was agreed.

##### 6.3.11.3	TS 38.523-1

##### 6.3.11.4	TS 38.523-2

##### 6.3.11.5	TS 38.523-3

##### 6.3.11.6	Discussion Papers / Work Plan / TC lists

#### 6.3.12	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest

##### 6.3.12.1	TS 38.508-1

##### 6.3.12.2	TS 38.508-2

##### 6.3.12.3	TS 38.523-1

##### 6.3.12.4	TS 38.523-2

##### 6.3.12.5	TS 38.523-3

##### 6.3.12.6	Discussion Papers / Work Plan / TC lists

#### 6.3.13	UE power saving enhancements for NR (UID-960086) NR_UE_pow_sav_enh_plus_CT-UEConTest

##### 6.3.13.1	TS 38.508-1

R5-240154	Correction to PEIPS in REGISTRATION ACCEPT message

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3004  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

##### 6.3.13.2	TS 38.508-2

R5-240425	Add PICS for PEIPS

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0582  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was agreed.

##### 6.3.13.3	TS 38.523-1

R5-240155	Correction to TC 9.1.14.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4167  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240156	Correction to TC 11.4.1a

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4168  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241653.

R5-241653	Correction to TC 11.4.1a

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4168  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240156)

Decision: 		The document was agreed.

R5-240426	Correction the title of PEIPS TC 9.1.14.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4224  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was agreed.

R5-240918	Resubmission of Editorial update test case 8.1.1.1a.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4277  Cat: F (Rel-17)

					Source: ETSI MCC (Ericsson)

Abstract:

This is a Resubmission of the not sent to plenary by mistake R5-236366, CR# 4032

Decision: 		The document was agreed.

##### 6.3.13.4	TS 38.523-2

R5-240427	Update the applicability of PEIPS TC 9.1.14.1 and 11.4.1a

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0436  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

Pls. add the spec & CR# to resolve the XXXX.

Decision: 		The document was revised to R5-241547.

R5-241547	Update the applicability of PEIPS TC 9.1.14.1 and 11.4.1a

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0436  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240427)

Decision: 		The document was agreed.

##### 6.3.13.5	TS 38.523-3

##### 6.3.13.6	Discussion Papers / Work Plan / TC lists

#### 6.3.14	NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects (UID-960087) LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest

##### 6.3.14.1	TS 36.508

R5-240157	Correction to NGSO abbreviation

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1455  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240158	Update ephemeris information for NGSO signalling test environment

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1456  Cat: F (Rel-18)

					Source: MediaTek Inc.

Abstract:

As the NGSO satellite propagation model to be used at UE side is not defined yet, constant Doppler and constant delay shall be used for ephemeris information generation.

Ephemeris information with constant Doppler and constant delay was updated for NGSO.

Discussion:

r2

Decision: 		The document was revised to R5-241556.

R5-241556	Update ephemeris information for NGSO signalling test environment

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1456  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-240158)

Decision: 		The document was agreed.

R5-240221	Update of UE pre-configuration for NGSO signalling test environment

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1458  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

r1

all merged into R5-240527r1.

Decision: 		The document was revised to R5-241557.

R5-241557	Update of UE pre-configuration for NGSO signalling test environment

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1458  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-240221)

Decision: 		The document was withdrawn.

R5-240290	Correction to System information for NTN

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1459  Cat: F (Rel-18)

					Source: MediaTek  Inc.

Discussion:

LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest  in coversheet!

cl. aff.

r2

Decision: 		The document was revised to R5-241558.

R5-241558	Correction to System information for NTN

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1459  rev 1 Cat: F (Rel-18)

					Source: MediaTek  Inc.

(Replaces R5-240290)

Decision: 		The document was agreed.

R5-240527	Definition of a TN-to-NTN sysInfo combination mapping for legacy test cases to be run in NTN environment

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1460  Cat: F (Rel-18)

					Source: MCC TF160

Discussion:

r1

changes do not align with the title.

Decision: 		The document was revised to R5-241559.

R5-241559	Definition of a TN-to-NTN sysInfo combination mapping for legacy test cases to be run in NTN environment

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1460  rev 1 Cat: F (Rel-18)

					Source: MCC TF160

(Replaces R5-240527)

Discussion:

reissued as R5-241560 because of title change

Decision: 		The document was withdrawn.

##### 6.3.14.2	TS 36.523-1

R5-240159	Correction to NB-IoT NTN TC 22.4.13a

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5273  Cat: F (Rel-18)

					Source: MediaTek Inc., ROHDE & SCHWARZ, Qualcomm

Decision: 		The document was agreed.

R5-240160	Update of applicable legacy NB-IoT cases

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5274  Cat: F (Rel-18)

					Source: MediaTek Inc., ROHDE & SCHWARZ

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240818	Correction to NBIOT NTN testcase 22.4.13a

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5285  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was withdrawn.

R5-240979	Correction to NTN TA report test case 22.3.1.13

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5288  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-241563.

R5-241563	Correction to NTN TA report test case 22.3.1.13

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5288  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz

(Replaces R5-240979)

Decision: 		The document was agreed.

R5-240980	Correction to MAC test cases for NTN UEs

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5289  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz, MediaTek

Discussion:

r1

Decision: 		The document was revised to R5-241564.

R5-241564	Correction to MAC test cases for NTN UEs

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5289  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz, MediaTek

(Replaces R5-240980)

Decision: 		The document was agreed.

R5-240981	Correction to RLC test cases for NTN UEs

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5290  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz, MediaTek

Discussion:

r1

Decision: 		The document was revised to R5-241647.

R5-241647	Correction to RLC test cases for NTN UEs

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5290  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz, MediaTek

(Replaces R5-240981)

Decision: 		The document was agreed.

##### 6.3.14.3	TS 36.523-2

R5-240161	Update of test cases applicability for NB-IoT NTN only UE

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1436  Cat: F (Rel-18)

					Source: MediaTek Inc., ROHDE & SCHWARZ, Qualcomm Incorporated

Discussion:

r1

dependent on other CRs.

Decision: 		The document was revised to R5-241562.

R5-241562	Update of test cases applicability for NB-IoT NTN only UE

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1436  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc., ROHDE & SCHWARZ, Qualcomm Incorporated

(Replaces R5-240161)

Decision: 		The document was agreed.

R5-240587	Correction to applicability of NB-IoT TC 22.3.2.7a

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1438  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240982	Addition of legacy test cases applicable to NTN only UEs in GSO and NGSO

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1441  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz, MediaTek

Discussion:

merged into R5-240161r1.

Decision: 		The document was withdrawn.

R5-240983	PICS clarification and applicability updates for NTN test cases

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1442  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz

Discussion:

similar changes needed for the MAC TCs..

Deferred.

r1

Decision: 		The document was revised to R5-241648.

R5-241648	PICS clarification and applicability updates for NTN test cases

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1442  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz

(Replaces R5-240983)

Decision: 		The document was agreed.

##### 6.3.14.4	TS 36.523-3

##### 6.3.14.5	Discussion Papers / Work Plan / TC lists

R5-240162	Discussion on NGSO test model

Type: discussion		For: Endorsement
					Source: MediaTek Inc.

Abstract:

During the discussion of [1] in RAN5 #101 meeting, there was no consensus achieved on NGSO test model. An action point AP#101.03 was raised to investigate to decide on choosing dynamic/static/mixed signalling test environments for NGSO NTN testing.

2.	Discussion

The dynamic/static/mixed signalling test environments for NGSO (LEO with a typical orbit height 600 km) testing are:

1.	Dynamic: the NGSO satellite is moving on its orbit and the broadcasted SIB31-NB is updated according to its position and velocity within the satellite visible window (about 8 minutes for LEO 600 km).

2.	Static: the NGSO satellite is static, and the broadcasted SIB31-NB has same message content. As this is not the realistic NGSO scenario, UE uses constant Doppler/delay to keep DL/UL synchronous with eNB, and UE also needs to deactivate prediction of satellite trajectory as mentioned in [2].

3.	Mixed: During the satellite visible window, the NGSO satellite is moving on its orbit and the broadcasted SIB31-NB is updated according to its position and velocity, then after the visible window the satellite keeps static at its last location and the SIB31-NB becomes unchanged. As this is not the realistic NGSO scenario, after the satellite visible window UE shall use constant Doppler/delay and needs to deactivate prediction of satellite trajectory as does in Static model.

Currently the NGSO satellite propagator model for TE to emulate channel model with Doppler and delay shifts are not defined by 3GPP. This also impacts the UE to correctly compensate the Doppler and delay in uplink transmission. The lack of propagator model will restrict the usage of dynamic signalling test environment as well as the mixed.

Based on above analysis, we can have following proposals for NGSO test model:

Proposal #1: RAN5 to approve the usage of static as default signalling test environment for IoT NTN specific cases and applicable NB-IoT legacy cases.

When static signalling test environment is used, the ephemeris information of NGSO needs to be specified, such as dedicate satellite stateVectors to be defined for the serving and neighbour cells, that means there is no need to define a full table of ephemeris information. And the UE pre-configuration of these cases shall be updated to the trajectory prediction mode to be deactivated in UE side. RAN5 CRs [3] [4] are submitted to address this approach during this meeting.

Proposal #2: RAN5 to approve the CRs [3] [4] to update ephemeris information of NGSO and to update the UE pre-configuration for static signalling test environment.

3.	Proposal/Way Forward

In the contribution, we investigate the possible dynamic/static/mixed signalling test environments for NGSO NTN testing with following proposal:

Proposal #1: RAN5 to approve the usage of static as default signalling test environment for IoT NTN specific cases and applicable NB-IoT legacy cases.

Proposal #2: RAN5 to approve the CRs [3] [4] to update ephemeris information of NGSO and to update the UE pre-configuration for static signalling test environment.

Discussion:

Keysight UK: SIG TC can be different from RF or RRM.

It should be discussed in the meeting.

TF160 manager: the call for the AP was done without the SS vendors.

RAN5 Chair: it was agreed offline to follow the same as in RF as agreed in the offline discussion.

The Thales paper said it is not realistic.

Anritsu: it seems the propagation model was not yet ready.

Qualcomm: the model comes with its own challenges.

Anritsu: doppler effect is static?

TF160 manager: it's DL transmission.

Conclusion: Proposal 1 is accepted. Proposal 2 will be dealt in the CRs.

Decision: 		The document was noted.

#### 6.3.15	NR QoE management and optimizations for diverse services (UID-970072) NR_QoE-UEConTest

##### 6.3.15.1	TS 38.508-1

##### 6.3.15.2	TS 38.508-2

##### 6.3.15.3	TS 38.523-1

##### 6.3.15.4	TS 38.523-2

##### 6.3.15.5	TS 38.523-3

##### 6.3.15.6	Discussion Papers / Work Plan / TC lists

R5-240954	SRB4

Type: discussion		For: Endorsement
					Source: Ericsson

Discussion:

SRB4 will be configured with a generic porcedure parameter that facilitates for it to be applied initally in Application Layer measureemnt test cases.

Decision: 		The document was noted.

#### 6.3.16	User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects) (UID-970074) UPIP_SEC_LTE-RAN-UEConTest

##### 6.3.16.1	TS 38.508-1

##### 6.3.16.2	TS 38.508-2

R5-240143	New additional information for UPIP with 5GC

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0566  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was withdrawn.

##### 6.3.16.3	TS 38.523-1

R5-240139	Addition of new test case 8.1.2.1.7 for UPIP

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4163  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

r1

Decision: 		The document was revised to R5-241513.

R5-241513	Addition of new test case 8.1.2.1.7 for UPIP

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4163  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-240139)

Decision: 		The document was withdrawn.

R5-240140	Addition of new test case 8.2.2.4.4 for UPIP

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4164  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

r1

The main test purpose in TC 8.2.2.4.4  has been included in TC 7.1.3.2.6, and the rest parts in TC 8.2.2.4.4 are unnecessary for UPIP, so the TC is not needed.

Decision: 		The document was revised to R5-241551.

R5-241551	Addition of new test case 8.2.2.4.4 for UPIP

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4164  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-240140)

Decision: 		The document was withdrawn.

R5-240203	Correction of  UPIP test case 7.1.3.2.6 for specific message contents

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4189  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

remove the folder

Decision: 		The document was revised to R5-241552.

R5-241552	Correction of  UPIP test case 7.1.3.2.6 for specific message contents

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4189  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-240203)

Decision: 		The document was agreed.

R5-240205	Correction of UPIP test case 8.2.6.4.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4190  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

remove the folder

Decision: 		The document was revised to R5-241553.

R5-241553	Correction of UPIP test case 8.2.6.4.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4190  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-240205)

Decision: 		The document was agreed.

R5-241186	Correction to UPIP TC 7.1.3.2.6

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4303  Cat: F (Rel-17)

					Source: Ericsson

Decision: 		The document was agreed.

##### 6.3.16.4	TS 38.523-2

R5-240141	Addition of applicability for new UPIP TC 8.1.2.1.7 and 8.2.2.4.4

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0429  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

r1

The main test purpose in TC 8.2.2.4.4  has been included in TC 7.1.3.2.6, and the rest parts in TC 8.2.2.4.4 are unnecessary for UPIP, so the TC is not needed.

Decision: 		The document was revised to R5-241575.

R5-241575	Addition of applicability for new UPIP TC 8.1.2.1.7 and 8.2.2.4.4

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0429  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-240141)

Decision: 		The document was withdrawn.

##### 6.3.16.5	TS 38.523-3

##### 6.3.16.6	Discussion Papers / Work Plan / TC lists

#### 6.3.17	NR Positioning Enhancements (UID-970075) NR_pos_enh-UEConTest

##### 6.3.17.1	TS 38.508-1

##### 6.3.17.2	TS 37.571-2

##### 6.3.17.3	TS 37.571-3

##### 6.3.17.4	TS 37.571-4

##### 6.3.17.5	TS 37.571-5

R5-240126	Introduction of BDS B2a and B3I signal default test conditions in TS 37.571-5

Type: CR		For: Agreement
					37.571-5 v17.3.0	  CR-0227  Cat: F (Rel-17)

					Source: CATT, CAICT

Discussion:

date

r1

Decision: 		The document was revised to R5-241577.

R5-241577	Introduction of BDS B2a and B3I signal default test conditions in TS 37.571-5

Type: CR		For: Agreement
					37.571-5 v17.3.0	  CR-0227  rev 1 Cat: F (Rel-17)

					Source: CATT, CAICT

(Replaces R5-240126)

Decision: 		The document was agreed.

##### 6.3.17.6	Discussion Papers / Work Plan / TC lists

#### 6.3.18	Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT / MCVideo / MCData) (UID – 970077) MCProtoc16_enh2MCPTT_eMCData2-ConTest

##### 6.3.18.1	TS 36.579-1

R5-240897	Corrections to Table 5.5.3.3.1-3 MCData Resource-lists

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0342  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240898	Addition of Location-info for MCData

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0343  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.3.18.2	TS 36.579-2

##### 6.3.18.3	TS 36.579-3

R5-240899	Corrections to Forward, Scope, References, and Definitions

Type: CR		For: Agreement
					36.579-3 v13.3.0	  CR-0008  Cat: F (Rel-16)

					Source: NIST

Abstract:

This will trigger an upgrade of the spec to the higher Rel-16

Discussion:

spec needs to be upgraded to Rel-15 first, and CRs resubmitted at the next meeting.

The meeting agreed to upgrade the spec to Rel-14 and Rel-15.

Decision: 		The document was withdrawn.

R5-240900	Addition on new TC 6.2 MCPTT Server - MCPTT Client -  Chat Group Call

Type: CR		For: Agreement
					36.579-3 v13.3.0	  CR-0009  Cat: F (Rel-16)

					Source: NIST

Abstract:

This will trigger an upgrade of the spec to the higher Rel-16

Discussion:

spec needs to be upgraded to Rel-15 first, and CRs resubmitted at the next meeting.

Decision: 		The document was withdrawn.

##### 6.3.18.4	TS 36.579-4

R5-240901	Addition of applicability for new Rel-16 test cases

Type: CR		For: Agreement
					36.579-4 v16.3.0	  CR-0033  Cat: F (Rel-16)

					Source: NIST

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241685.

R5-241685	Addition of applicability for new Rel-16 test cases

Type: CR		For: Agreement
					36.579-4 v16.3.0	  CR-0033  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-240901)

Discussion:

Email agreed

Decision: 		The document was agreed.

##### 6.3.18.5	TS 36.579-5

R5-240902	Addition of new PIXITs for REL-16 TCs

Type: CR		For: Agreement
					36.579-5 v17.4.0	  CR-0105  Cat: F (Rel-17)

					Source: NIST

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241686.

R5-241686	Addition of new PIXITs for REL-16 TCs

Type: CR		For: Agreement
					36.579-5 v17.4.0	  CR-0105  rev 1 Cat: F (Rel-17)

					Source: NIST

(Replaces R5-240902)

Discussion:

Email agreed

Decision: 		The document was agreed.

##### 6.3.18.6	TS 36.579-6

R5-240528	Correction of testcase 6.7.3

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0099  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240633	Addition of new TC 6.5.2 One-to-server video push call CO

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0102  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

block agreed on Thu.

Decision: 		The document was revised to R5-241596.

R5-241596	Addition of new TC 6.5.2 One-to-server video push call CO

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0102  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-240633)

Decision: 		The document was agreed.

R5-240909	Addition of new TC 6.9.1 Location Information Request

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0103  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240910	Addition of new TC 6.9.2 Report triggering

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0104  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240911	Addition of new TC 6.1.1.14 Re-join

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0105  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

block agreed on Thu.

Decision: 		The document was revised to R5-241595.

R5-241595	Addition of new TC 6.1.1.14 Re-join

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0105  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-240911)

Decision: 		The document was agreed.

##### 6.3.18.7	TS 36.579-7

R5-240903	Addition of new TC 6.4.1 Emergency Alert CO

Type: CR		For: Agreement
					36.579-7 v16.2.0	  CR-0055  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

block agreed on Thu.

Decision: 		The document was revised to R5-241593.

R5-241593	Addition of new TC 6.4.1 Emergency Alert CO

Type: CR		For: Agreement
					36.579-7 v16.2.0	  CR-0055  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-240903)

Decision: 		The document was agreed.

R5-240904	Addition of new TC 6.4.2 Emergency Alert CT

Type: CR		For: Agreement
					36.579-7 v16.2.0	  CR-0056  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

block agreed on Thu.

Decision: 		The document was revised to R5-241594.

R5-241594	Addition of new TC 6.4.2 Emergency Alert CT

Type: CR		For: Agreement
					36.579-7 v16.2.0	  CR-0056  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-240904)

Decision: 		The document was agreed.

##### 6.3.18.8	TS 36.579-8 (pCRs only)

R5-240905	Addition of new TC 5.1 Configuration

Type: pCR		For: (not specified)
					36.579-8 v0.0.1
					Source: NIST

Discussion:

withdraw the pCRs until the next meeting so there can be more time to discuss how the MCX server can fit into the scope of RAN5

Decision: 		The document was withdrawn.

R5-240906	Addition of new TC 6.1 Group Call

Type: pCR		For: (not specified)
					36.579-8 v0.0.1
					Source: NIST

Discussion:

withdraw the pCRs until the next meeting so there can be more time to discuss how the MCX server can fit into the scope of RAN5

Decision: 		The document was withdrawn.

##### 6.3.18.9	TS 36.579-9 (pCRs only)

R5-240907	Addition of new TC 5.1 Configuration

Type: pCR		For: (not specified)
					36.579-9 v0.0.1
					Source: NIST

Discussion:

withdraw the pCRs until the next meeting so there can be more time to discuss how the MCX server can fit into the scope of RAN5

Decision: 		The document was withdrawn.

R5-240908	Addition of new TC 6.1 One-to-one standalone SDS using the signaling plane

Type: pCR		For: (not specified)
					36.579-9 v0.0.1
					Source: NIST

Discussion:

withdraw the pCRs until the next meeting so there can be more time to discuss how the MCX server can fit into the scope of RAN5

Decision: 		The document was withdrawn.

##### 6.3.18.10	Discussion Papers / Work Plan / TC lists

#### 6.3.19	NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) (UID-981034) LTE_NBIOT_eMTC_NTN_req-UEConTest

##### 6.3.19.1	TS 36.508

##### 6.3.19.2	TS 36.509

##### 6.3.19.3	TS 36.523-2

##### 6.3.19.4	TS 36.523-3

##### 6.3.19.5	Discussion Papers / Work Plan / TC lists

#### 6.3.20	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest

##### 6.3.20.1	TS 38.508-1

##### 6.3.20.2	TS 38.508-2

R5-240387	Addition of UE capability for inter-SN conditional PSCell change

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0579  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

xxx

r2

Decision: 		The document was revised to R5-241601.

R5-241601	Addition of UE capability for inter-SN conditional PSCell change

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0579  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240387)

Decision: 		The document was agreed.

##### 6.3.20.3	TS 38.523-1

R5-240379	Addition of new test case 8.2.3.18.4 inter-SN CPC for EN-DC

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4217  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

added the missing minimum requirements and configurations based on comments from TF 160

r3

Decision: 		The document was revised to R5-241602.

R5-241602	Addition of new test case 8.2.3.18.4 inter-SN CPC for EN-DC

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4217  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240379)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-240380	Addition of new test case 8.2.3.18.5 inter-SN CPC for NR-DC

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4218  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

added the missing minimum requirements and configurations based on comments from TF 160

r3

Decision: 		The document was revised to R5-241603.

R5-241603	Addition of new test case 8.2.3.18.5 inter-SN CPC for NR-DC

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4218  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240380)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

##### 6.3.20.4	TS 38.523-2

R5-240386	Addition of applicability for inter-SN conditional PSCell change

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0435  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-241604.

R5-241604	Addition of applicability for inter-SN conditional PSCell change

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0435  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240386)

Discussion:

for email agreement

Email agreed

Decision: 		The document was agreed.

##### 6.3.20.5	TS 38.523-3

##### 6.3.20.6	TS 36.508

##### 6.3.20.7	Discussion Papers / Work Plan / TC lists

#### 6.3.21	Support of Uncrewed Aerial Systems Connectivity / Identification / and Tracking (UID-991034) ID_UAS-UEConTest

##### 6.3.21.1	TS 38.508-1

##### 6.3.21.2	TS 38.508-2

##### 6.3.21.3	TS 38.523-1

R5-240588	Modification of testcase 9.1.6.2.3 UE or NW initiated de-registration for UAS

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4253  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

r2

Decision: 		The document was revised to R5-241605.

R5-241605	Modification of testcase 9.1.6.2.3 UE or NW initiated de-registration for UAS

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4253  rev 1 Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240588)

Decision: 		The document was agreed.

R5-240658	Corrections to UAS test case 10.1.4.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4271  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies

Decision: 		The document was revised to R5-241606.

R5-241606	Corrections to UAS test case 10.1.4.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4271  rev 1 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies

(Replaces R5-240658)

Decision: 		The document was agreed.

R5-241183	Correction to UAS TC 9.1.5.2.11

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4301  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-241607.

R5-241607	Correction to UAS TC 9.1.5.2.11

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4301  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-241183)

Decision: 		The document was agreed.

##### 6.3.21.4	TS 38.523-2

##### 6.3.21.5	TS 38.523-3

##### 6.3.21.6	TS 36.508

##### 6.3.21.7	TS 36.523-1

R5-241040	Addition of UAS EPS test case 10.10.2

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5291  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

H5->3

Decision: 		The document was revised to R5-241608.

R5-241608	Addition of UAS EPS test case 10.10.2

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5291  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-241040)

Decision: 		The document was agreed.

R5-241041	Addition of UAS EPS test case 10.10.4

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5292  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-241609.

R5-241609	Addition of UAS EPS test case 10.10.4

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5292  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-241041)

Decision: 		The document was agreed.

R5-241042	Addition of EPS UAS test case 10.10.6

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5293  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-241610.

R5-241610	Addition of EPS UAS test case 10.10.6

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5293  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-241042)

Decision: 		The document was agreed.

##### 6.3.21.8	TS 36.523-2

R5-241043	Applicability updates to EPS UAS test cases

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1443  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.21.9	TS 36.523-3

##### 6.3.21.10	Discussion Papers / Work Plan / TC lists

#### 6.3.22	IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS (UID-1000052) ING_5GS-UEConTest

##### 6.3.22.1	TS 38.508-1

##### 6.3.22.2	TS 38.508-2

##### 6.3.22.3	TS 38.523-1

R5-240028	Editorial correction for test procedure sequence in 11.1.10

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4158  Cat: F (Rel-17)

					Source: NTT DOCOMO, INC.

Discussion:

3GU had wrong ver.

cl. aff.

r2

Decision: 		The document was revised to R5-241591.

R5-241591	Editorial correction for test procedure sequence in 11.1.10

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4158  rev 1 Cat: F (Rel-17)

					Source: NTT DOCOMO, INC.

(Replaces R5-240028)

Decision: 		The document was agreed.

R5-240040	Add new ING_5GS test case 11.1.11

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4160  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

3GU had wrong ver.

r2

ETSI MCC: pls. remove the changes-over-changes.

Decision: 		The document was revised to R5-241592.

R5-241592	Add new ING_5GS test case 11.1.11

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4160  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240040)

Discussion:

needs replacement doc (9.3.1.6)

reissued as R5-241638 because of title change

Decision: 		The document was withdrawn.

R5-241638	Add new ING_5GS test case 9.3.1.6

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4309  Cat: F (Rel-17)

					Source: China Telecom

Abstract:

reissued from R5-241592 because of title change

Decision: 		The document was agreed.

##### 6.3.22.4	TS 38.523-2

R5-240041	Add applicability for ING_5GS test case 11.1.11

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0428  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was withdrawn.

R5-240235	Addition of applicability of new ING_5GS test case 11.1.11

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0431  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

needs replacement doc (9.3.1.6)!

reissued as R5-241639 because of title change

Decision: 		The document was withdrawn.

R5-241639	Addition of applicability of new ING_5GS test case 9.3.1.6

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0453  Cat: F (Rel-17)

					Source: China Telecom

Abstract:

reissued from R5-240235 because of title change

Discussion:

r1

Decision: 		The document was revised to R5-241654.

R5-241654	Addition of applicability of new ING_5GS test case 9.3.1.6

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0453  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-241639)

Decision: 		The document was agreed.

##### 6.3.22.5	TS 38.523-3

##### 6.3.22.6	Discussion Papers / Work Plan / TC lists

#### 6.3.23	IMS Data Channel (UID-1000055) IMSProtoc17_dataCH-UEContest

##### 6.3.23.1	TS 36.508

##### 6.3.23.2	TS 36.509

##### 6.3.23.3	TS 38.508-1

##### 6.3.23.4	TS 38.509

##### 6.3.23.5	TS 34.229-1

##### 6.3.23.6	TS 34.229-2

##### 6.3.23.7	TS 34.229-3

##### 6.3.23.8	TS 34.229-5

##### 6.3.23.9	Discussion Papers / Work Plan / TC lists

#### 6.3.24	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest

##### 6.3.24.1	TS 38.508-1

##### 6.3.24.2	TS 38.508-2

##### 6.3.24.3	TS 38.523-1

##### 6.3.24.4	TS 38.523-2

##### 6.3.24.5	TS 38.523-3

##### 6.3.24.6	Discussion Papers / Work Plan / TC lists

#### 6.3.25	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest

##### 6.3.25.1	TS 38.523-3

##### 6.3.25.2	Discussion Papers / Work Plan / TC lists

#### 6.3.26	Access Traffic Steering / Switch and Splitting support in the 5G system architecture; Phase 2 (UID-1010050) ATSSS_Ph2-UEConTest

##### 6.3.26.1	TS 38.508-1

##### 6.3.26.2	TS 38.508-2

R5-240027	Addition of PICS for Rel-17 ATSSS devices

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0562  Cat: F (Rel-18)

					Source: China Telecom

Decision: 		The document was agreed.

R5-240593	Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0589  Cat: F (Rel-18)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

##### 6.3.26.3	TS 38.523-1

R5-240100	Add new r17 ATSSS test case 11.9.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4162  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

3GU had wrong ver.

r1

Decision: 		The document was revised to R5-241617.

R5-241617	Add new r17 ATSSS test case 11.9.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4162  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240100)

Decision: 		The document was agreed.

R5-240240	Add new ATSSS test case 11.9.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4196  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

r2

Decision: 		The document was revised to R5-241618.

R5-241618	Add new ATSSS test case 11.9.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4196  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-240240)

Decision: 		The document was agreed.

R5-240241	Add new ATSSS test case 11.9.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4197  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

r4

Decision: 		The document was revised to R5-241619.

R5-241619	Add new ATSSS test case 11.9.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4197  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-240241)

Decision: 		The document was agreed.

R5-240282	Add new r17 ATSSS test case 11.9.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4201  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r2

Decision: 		The document was revised to R5-241620.

R5-241620	Add new r17 ATSSS test case 11.9.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4201  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-240282)

Decision: 		The document was agreed.

R5-240591	New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4255  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

R5-240592	New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4256  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

##### 6.3.26.4	TS 38.523-2

R5-240220	Add applicability for Rel-17 ATSSS test case 11.9.1

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0430  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

reissued as R5-241453 because of title change

Decision: 		The document was withdrawn.

R5-240242	Add applicabilities for new ATSSS test cases 11.9.3 and 11.9.4

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0432  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

merge it's contents into R5-240220.

Decision: 		The document was withdrawn.

R5-240629	Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0439  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

Partial overlap in testcase condition with R5-240220.

merged into R5-241453.

Decision: 		The document was withdrawn.

R5-241453	Add applicability for Rel-17 ATSSS test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0452  Cat: F (Rel-17)

					Source: China Telecom, ZTE

Abstract:

reissued from R5-240220 because of title change

Discussion:

r2

-Nokia

Decision: 		The document was revised to R5-241621.

R5-241621	Add applicability for Rel-17 ATSSS test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0452  rev 1 Cat: F (Rel-17)

					Source: China Telecom, ZTE

(Replaces R5-241453)

Decision: 		The document was agreed.

##### 6.3.26.5	TS 38.523-3

##### 6.3.26.6	Discussion Papers / Work Plan / TC lists

#### 6.3.27	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest

##### 6.3.27.1	TS 36.508

R5-240939	Additional CA_30A-48A for CA signalling test

Type: CR		For: Agreement
					36.508 v18.3.0	  CR-1461  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International, CATT

Decision: 		The document was agreed.

##### 6.3.27.2	TS 36.523-1

##### 6.3.27.3	TS 36.523-2

R5-240938	Additional supported capabilities for multiple CA combos

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1439  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International, CATT

Abstract:

Added CA_30A-48A, CA_2A-2A-29A-66A,  CA_2A-29A-66A-66A, CA_2A-2A-29A-30A-66A and CA_2A-2A-29A-66A-66A declaration

Decision: 		The document was agreed.

##### 6.3.27.4	TS 36.523-3

##### 6.3.27.5	Discussion Papers / Work Plan / TC lists

#### 6.3.28	Signal level Enhanced Network Selection (UID-1010052) SENSE-UEContest

##### 6.3.28.1	TS 36.508

##### 6.3.28.2	TS 36.523-1

R5-240418	Addition of SENSE eMTC TC 6.1.1.12-PLMN selection

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5275  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241611.

R5-241611	Addition of SENSE eMTC TC 6.1.1.12-PLMN selection

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5275  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240418)

Decision: 		The document was agreed.

R5-240419	Addition of SENSE eMTC TC 6.1.1.13-PLMN selection Exception

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5276  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241612.

R5-241612	Addition of SENSE eMTC TC 6.1.1.13-PLMN selection Exception

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5276  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240419)

Decision: 		The document was agreed.

R5-240420	Addition of SENSE eMTC TC 6.1.1.14-Periodic SENSE

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5277  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

r2

Decision: 		The document was revised to R5-241613.

R5-241613	Addition of SENSE eMTC TC 6.1.1.14-Periodic SENSE

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5277  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240420)

Decision: 		The document was agreed.

R5-240421	Addition of SENSE NB-IoT TC 22.2.14-PLMN selection

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5278  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241614.

R5-241614	Addition of SENSE NB-IoT TC 22.2.14-PLMN selection

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5278  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240421)

Decision: 		The document was agreed.

R5-240422	Addition of SENSE NB-IoT TC 22.2.15-PLMN selection Exception

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5279  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241615.

R5-241615	Addition of SENSE NB-IoT TC 22.2.15-PLMN selection Exception

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5279  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240422)

Decision: 		The document was agreed.

R5-240423	Addition of SENSE NB-IoT TC 22.2.16-Periodic SENSE

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5280  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

comment from TF160 ES

r2

Decision: 		The document was revised to R5-241616.

R5-241616	Addition of SENSE NB-IoT TC 22.2.16-Periodic SENSE

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5280  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-240423)

Decision: 		The document was agreed.

##### 6.3.28.3	TS 36.523-2

R5-240424	Addition of PICS and test applicability for SENSE TC

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1437  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was agreed.

##### 6.3.28.4	TS 36.523-3

##### 6.3.28.5	Discussion Papers / Work Plan / TC lists

### 6.4	Routine Maintenance for TS 38 Series TEIx_Test

#### 6.4.1	TS 38.508-1

##### 6.4.1.1	Generic Procedures and Test Procedures (Clauses 4.5 / 4.5A & 4.9)

R5-240163	Correction to Test procedure 4.9.27

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3005  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240164	Correction to Test procedure 4.9.38

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3006  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240236	Correction to NR RRC_Idle generic procedure

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3017  Cat: F (Rel-18)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-240237	Correction to NR RRC_Inactive generic procedure

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3018  Cat: F (Rel-18)

					Source: Keysight Technologies UK

Discussion:

cl. aff.

r1

Decision: 		The document was revised to R5-241628.

R5-241628	Correction to NR RRC_Inactive generic procedure

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3018  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK

(Replaces R5-240237)

Decision: 		The document was agreed.

R5-240536	Editorial updates to test procedure 4.9.1

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3057  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.4.1.2	Default NG-RAN RRC messages and IEs (Clause 4.6)

R5-240165	Correction to DLInformationTransfer message

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3007  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240166	Correction to UEInformationRequest message

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3008  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240167	Editorial update of CounterCheck message

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3009  Cat: F (Rel-18)

					Source: MediaTek Inc.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-240168	Correction to IE RadioLinkMonitoringConfig

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3010  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240169	Correction to IE NRDC-Parameters

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3011  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241480.

R5-241480	Correction to IE NRDC-Parameters

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3011  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-240169)

Decision: 		The document was agreed.

R5-240170	Correction to IE UE-MRDC-Capability

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3012  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241481.

R5-241481	Correction to IE UE-MRDC-Capability

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3012  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-240170)

Decision: 		The document was agreed.

R5-240171	Correction to IE UE-NR-Capability

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3013  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241482.

R5-241482	Correction to IE UE-NR-Capability

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3013  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-240171)

Decision: 		The document was agreed.

R5-240444	Add IndirectPathFailureInformation

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3031  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240445	Add MBSMulticastConfiguration

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3032  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240446	Add MeasurementReportAppLayer

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3033  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240447	Editorial update SystemInformation

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3034  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-241483.

R5-241483	Editorial update SystemInformation

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3034  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240447)

Decision: 		The document was agreed.

R5-240448	Add UEPositioningAssistanceInfo

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3035  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

TF160 mamanger :Table -CA after -B?

Decision: 		The document was agreed.

R5-240489	Add IEs SIB22, SIB23, SIB24 and SIB25

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3045  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240490	Editorial update IE PosSystemInformation-r16-IEs

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3046  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-241484.

R5-241484	Editorial update IE PosSystemInformation-r16-IEs

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3046  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240490)

Decision: 		The document was agreed.

R5-240491	Editorial update IE PosSI-SchedulingInfo

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3047  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240492	Add IE AdvancedReceiver-MU-MIMO

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3048  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240493	Add IE Altitude

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3049  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240494	Add IE ATG-Config

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3050  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240495	Add IEs CandidateTCI-State and CandidateTCI-UL-State

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3051  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240496	Add IE CellDTXDRX-Config

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3052  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240497	Add IE ClockQualityMetrics

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3053  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240498	Add IEs CSI-ReportSubConfig, CSI-ReportSubConfigId and CSI-ReportSubConfigTriggerList

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3054  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R5-241485.

R5-241485	Add IEs CSI-ReportSubConfig, CSI-ReportSubConfigId and CSI-ReportSubConfigTriggerList

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3054  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240498)

Decision: 		The document was agreed.

R5-240499	Add IE EarlyUL-SyncConfig

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3055  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240500	Add IE EUTRA-C-RNTI

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3056  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-240537	Harmonization of SCell_add condition

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3058  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240822	Update IE FeatureCombination

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3069  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-240823	Add IE HysteresisAltitude

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3070  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

date

Decision: 		The document was revised to R5-241486.

R5-241486	Add IE HysteresisAltitude

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3070  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240823)

Decision: 		The document was agreed.

R5-240824	Add IEs LTM-CandidateId, LTM-Candidate, LTM-Config and LTM-CSI-ReportConfig

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3071  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

date

Decision: 		The document was revised to R5-241487.

R5-241487	Add IEs LTM-CandidateId, LTM-Candidate, LTM-Config and LTM-CSI-ReportConfig

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3071  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240824)

Decision: 		The document was agreed.

R5-240825	Add IEs LTM-CSI-ReportConfigId, LTM-CSI-ResourceConfig and LTM-CSI-ResourceConfigId

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3072  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

date

Decision: 		The document was revised to R5-241488.

R5-241488	Add IEs LTM-CSI-ReportConfigId, LTM-CSI-ResourceConfig and LTM-CSI-ResourceConfigId

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3072  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-240825)

Decision: 		The document was agreed.

R5-241371	Add IE MeasSequence

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3096  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241372	Add IE MeasWindowConfig

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3097  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241373	Add IEs N3C-IndirectPathConfigRelay and N3C-IndirectPathAddChange

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3098  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241374	Add IEs NCR-AperiodicFwdConfig, NCR-FwdConfig, NCR-PeriodicityAndOffset and NCR-PeriodicFwdResourceSet

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3099  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241375	Add IEs NCR-PeriodicFwdResourceSetId, NCR-SemiPersistentFwdResourceSet and NCR-SemiPersistentFwdResourceSetId

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3100  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

##### 6.4.1.3	Default 5GC NAS messages and IEs (Clause 4.7)

R5-241187	Updates to default 5GMM messages

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3088  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-241188	Updates to default 5GSM messages

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3089  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-241189	Updates to default UE Policy Delivery messages

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3090  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

RR5

r1

Decision: 		The document was revised to R5-241489.

R5-241489	Updates to default UE Policy Delivery messages

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3090  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-241189)

Decision: 		The document was withdrawn.

##### 6.4.1.4	Test environment for SIG (Clause 6)

R5-241365	Test environment definition for GERAN in Conducted and OTA Environment

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3095  Cat: F (Rel-18)

					Source: Anritsu EMEA Ltd

Discussion:

TF160 manager: split UTRAN into another CR!

+ formal comments.

Deferred

r1

Decision: 		The document was revised to R5-241630.

R5-241630	Test environment definition for GERAN in Conducted and OTA Environment

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3095  rev 1 Cat: F (Rel-18)

					Source: Anritsu EMEA Ltd

(Replaces R5-241365)

Decision: 		The document was agreed.

R5-241493	Updates to OTA Signalling test environment

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3104  Cat: F (Rel-18)

					Source: Anritsu Ltd

Discussion:

late doc

Tdoc#

Decision: 		The document was revised to R5-241629.

R5-241629	Updates to OTA Signalling test environment

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3104  rev 1 Cat: F (Rel-18)

					Source: Anritsu Ltd

(Replaces R5-241493)

Decision: 		The document was agreed.

##### 6.4.1.5	Other clauses / Annexes

R5-240172	Correction to RRCReconfiguration configuration

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3014  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

#### 6.4.2	TS 38.508-2

R5-240208	Correction to Rel-16 PICS Mnemonic

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3015  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

reissued as R5-241457 because of wrong spec

Decision: 		The document was withdrawn.

R5-241457	Correction to Rel-16 PICS Mnemonic

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0606  Cat: F (Rel-18)

					Source: MediaTek Inc.

Abstract:

reissued from R5-240208 because of wrong spec

Decision: 		The document was withdrawn.

R5-240209	Correction to Rel-17 PICS Mnemonic

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0569  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240538	Updates to align PICS mnemonics

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0587  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was revised to R5-241491.

R5-241491	Updates to align PICS mnemonics

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0587  rev 1 Cat: F (Rel-18)

					Source: MCC TF160

(Replaces R5-240538)

Decision: 		The document was agreed.

R5-240661	Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI)

Type: CR		For: Agreement
					38.508-1 v18.1.0	  CR-3064  Cat: F (Rel-18)

					Source: Qualcomm CDMA Technologies

Decision: 		The document was withdrawn.

R5-240673	Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI)

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0591  Cat: F (Rel-18)

					Source: Qualcomm CDMA Technologies

Discussion:

late doc

R5

Decision: 		The document was revised to R5-241631.

R5-241631	Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI)

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0591  rev 1 Cat: F (Rel-18)

					Source: Qualcomm CDMA Technologies

(Replaces R5-240673)

Decision: 		The document was agreed.

R5-241376	Addition of PICS for support of ULCA for different type of CA Band combinations

Type: CR		For: Agreement
					38.508-2 v18.1.0	  CR-0605  Cat: F (Rel-18)

					Source: Keysight Technologies UK

Discussion:

late doc

offlline TF160 comments.

Decision: 		The document was withdrawn.

#### 6.4.3	TS 38.509

R5-240430	Correction of Set MUSIM UAI test function

Type: CR		For: Agreement
					38.509 v17.4.0	  CR-0082  Cat: F (Rel-17)

					Source: Huawei, HiSilicon,China Telecom

Discussion:

late doc

ME x!

comment from TF160 UK and CATT

r3

Decision: 		The document was revised to R5-241636.

R5-241636	Correction of Set MUSIM UAI test function

Type: CR		For: Agreement
					38.509 v17.4.0	  CR-0082  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon,China Telecom

(Replaces R5-240430)

Decision: 		The document was withdrawn.

R5-240434	Correction of Test Loop Mode C for MBS broadcast

Type: CR		For: Agreement
					38.509 v17.4.0	  CR-0083  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

ME x!

r1

Decision: 		The document was revised to R5-241492.

R5-241492	Correction of Test Loop Mode C for MBS broadcast

Type: CR		For: Agreement
					38.509 v17.4.0	  CR-0083  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240434)

Decision: 		The document was agreed.

#### 6.4.4	TS 38.523-1

##### 6.4.4.1	Clauses 1 - 5

##### 6.4.4.2	Idle Mode (Clause 6)

R5-240225	Correction to NR testcase 6.4.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4191  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Discussion:

r1

Decision: 		The document was revised to R5-241495.

R5-241495	Correction to NR testcase 6.4.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4191  rev 1 Cat: F (Rel-17)

					Source: Starpoint, TDIA

(Replaces R5-240225)

Decision: 		The document was agreed.

R5-240238	Correction to RRC Inactive Mode test case 6.4.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4194  Cat: F (Rel-18)

					Source: Keysight Technologies UK

Decision: 		The document was withdrawn.

R5-240289	Correction to NR slice TC 6.1.2.25

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4206  Cat: F (Rel-17)

					Source: MediaTek Inc.,Anritsu

Decision: 		The document was agreed.

R5-240323	Correction to eCPSOR TC 6.3.2.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4208  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240540	Correction to NR test case 6.1.2.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4247  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240645	Correction to NR RRC Idle mode test case 6.4.2.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4263  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241496.

R5-241496	Correction to NR RRC Idle mode test case 6.4.2.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4263  rev 1 Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

(Replaces R5-240645)

Decision: 		The document was agreed.

R5-240646	Correction to NR RRC Idle mode test case 6.1.2.24

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4264  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241497.

R5-241497	Correction to NR RRC Idle mode test case 6.1.2.24

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4264  rev 1 Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

(Replaces R5-240646)

Decision: 		The document was agreed.

R5-241070	Correction of test case 6.1.2.13

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4291  Cat: F (Rel-17)

					Source: NTTDOCOMO, INC.

Discussion:

CR cover value : 17.5.0.

r2

Decision: 		The document was revised to R5-241498.

R5-241498	Correction of test case 6.1.2.13

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4291  rev 1 Cat: F (Rel-17)

					Source: NTTDOCOMO, INC.

(Replaces R5-241070)

Decision: 		The document was agreed.

R5-241071	Correction of test case 6.1.2.14

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4292  Cat: F (Rel-17)

					Source: NTTDOCOMO, INC.

Discussion:

CR cover value : 17.5.0.

r1

Decision: 		The document was revised to R5-241499.

R5-241499	Correction of test case 6.1.2.14

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4292  rev 1 Cat: F (Rel-17)

					Source: NTTDOCOMO, INC.

(Replaces R5-241071)

Decision: 		The document was agreed.

R5-241073	Correction to IRAT PLMN selection test case 6.2.1.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4293  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-241500.

R5-241500	Correction to IRAT PLMN selection test case 6.2.1.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4293  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-241073)

Decision: 		The document was agreed.

R5-241166	Correction to RRC Inactive Mode test case 6.4.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4300  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-241378	Corrrection to NR5GC CAG test case 6.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4308  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Discussion:

r1

TF160 needed more time to review.

Decision: 		The document was revised to R5-241637.

R5-241637	Corrrection to NR5GC CAG test case 6.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4308  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK

(Replaces R5-241378)

Decision: 		The document was agreed.

##### 6.4.4.3	Layer 2

###### 6.4.4.3.1	NR Layer 2

6.4.4.3.1.1	Common Test Case Specific Values for Layer 2 (Clause 7.1.0)

6.4.4.3.1.2	MAC

R5-240174	Correction to NR MAC TC 7.1.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4169  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240175	Correction to 2-step RACH MAC TC 7.1.1.1.8 and 7.1.1.1.10

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4170  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240176	Correction to NR MAC TC 7.1.1.5.x

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4171  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

TF160 manager: remove some ::=

Decision: 		The document was revised to R5-241501.

R5-241501	Correction to NR MAC TC 7.1.1.5.x

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4171  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240176)

Decision: 		The document was agreed.

R5-240177	Correction to NR MAC TC 7.1.1.6.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4172  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241502.

R5-241502	Correction to NR MAC TC 7.1.1.6.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4172  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240177)

Decision: 		The document was agreed.

R5-240178	Correction to NR MAC TC 7.1.1.6.2 and 7.1.1.6.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4173  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241503.

R5-241503	Correction to NR MAC TC 7.1.1.6.2 and 7.1.1.6.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4173  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240178)

Decision: 		The document was agreed.

R5-240179	Correction to NR MAC TC 7.1.1.6.4 and 7.1.1.6.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4174  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241504.

R5-241504	Correction to NR MAC TC 7.1.1.6.4 and 7.1.1.6.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4174  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240179)

Decision: 		The document was agreed.

R5-240180	Correction to NR MAC TC 7.1.1.8.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4175  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241505.

R5-241505	Correction to NR MAC TC 7.1.1.8.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4175  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240180)

Decision: 		The document was agreed.

R5-240413	Correction of MAC TC 7.1.1.6.2-UL CG type1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4219  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was agreed.

R5-240414	Correction of MAC TC 7.1.1.3.2b-Lcp-Restriction

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4220  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

ETSI MCC: 2 coversheet typos.

Motorola Mobility: why delete the timer?

Pls. remove Note 1.

Decision: 		The document was revised to R5-241506.

R5-241506	Correction of MAC TC 7.1.1.3.2b-Lcp-Restriction

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4220  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240414)

Decision: 		The document was agreed.

R5-240415	Correction of MAC TC 7.1.1.8.1-BWP switch

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4221  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

overlap with R5-240180

r1

Deferred.

Decision: 		The document was revised to R5-241507.

R5-241507	Correction of MAC TC 7.1.1.8.1-BWP switch

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4221  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240415)

Decision: 		The document was agreed.

R5-240656	Correction and updates to NR TC 7.1.1.10.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4269  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies

Discussion:

late doc

r1

Deferred.

Decision: 		The document was revised to R5-241508.

R5-241508	Correction and updates to NR TC 7.1.1.10.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4269  rev 1 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies

(Replaces R5-240656)

Decision: 		The document was withdrawn.

R5-241037	Corrections to eMIMO TC 7.1.1.3.10

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4286  Cat: F (Rel-17)

					Source: Lenovo

Discussion:

r1

Decision: 		The document was revised to R5-241509.

R5-241509	Corrections to eMIMO TC 7.1.1.3.10

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4286  rev 1 Cat: F (Rel-17)

					Source: Lenovo

(Replaces R5-241037)

Decision: 		The document was agreed.

6.4.4.3.1.3	RLC

6.4.4.3.1.4	PDCP

6.4.4.3.1.5	SDAP

##### 6.4.4.4	RRC

###### 6.4.4.4.1	NR RRC

6.4.4.4.1.1	RRC Connection Management Procedures (clause 8.1.1)

R5-240226	Correction to NR testcase 8.1.1.4.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4192  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Discussion:

r1

Decision: 		The document was revised to R5-241510.

R5-241510	Correction to NR testcase 8.1.1.4.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4192  rev 1 Cat: F (Rel-17)

					Source: Starpoint, TDIA

(Replaces R5-240226)

Decision: 		The document was agreed.

R5-241074	Correction of test case 8.1.1.2.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4294  Cat: F (Rel-17)

					Source: NTTDOCOMO, INC., TF160

Discussion:

CR cover value : 17.5.0.

r5

TF160 needed more time to review.

Decision: 		The document was revised to R5-241511.

R5-241511	Correction of test case 8.1.1.2.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4294  rev 1 Cat: F (Rel-17)

					Source: NTTDOCOMO, INC., TF160

(Replaces R5-241074)

Decision: 		The document was agreed.

R5-241163	Correction to NR Slice test case 8.1.1.3.9

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4298  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Decision: 		The document was revised to R5-241512.

R5-241512	Correction to NR Slice test case 8.1.1.3.9

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4298  rev 1 Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

(Replaces R5-241163)

Decision: 		The document was agreed.

6.4.4.4.1.2	RRC Reconfiguration (clause 8.1.2)

R5-240429	Correction of MUSIM TC 8.1.2.1.6-MUSIM gap configuration

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4225  Cat: F (Rel-17)

					Source: Huawei, HiSilicon,China Telecom

Discussion:

late doc

comment from CATT

r2

Decision: 		The document was revised to R5-241640.

R5-241640	Correction of MUSIM TC 8.1.2.1.6-MUSIM gap configuration

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4225  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon,China Telecom

(Replaces R5-240429)

Decision: 		The document was agreed.

6.4.4.4.1.3	RRC Measurement Configuration Control and Reporting (clause 8.1.3)

R5-241309	Correction to FR2 measurement threshold for NR RRC test cases

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4305  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Decision: 		The document was agreed.

6.4.4.4.1.4	RRC Handover (clause 8.1.4)

R5-240181	Correction to NR HO TC 8.1.4.2.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4176  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240182	Correction to NR HO TC 8.1.4.2.1.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4177  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241514.

R5-241514	Correction to NR HO TC 8.1.4.2.1.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4177  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240182)

Decision: 		The document was agreed.

R5-240256	Correction to NR testcase 8.1.4.3.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4200  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Discussion:

r1

Motorola Mobility: shall not re-use an already voided table.

Decision: 		The document was revised to R5-241515.

R5-241515	Correction to NR testcase 8.1.4.3.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4200  rev 1 Cat: F (Rel-17)

					Source: Starpoint, TDIA

(Replaces R5-240256)

Decision: 		The document was agreed.

R5-240541	Corrections to 5G TC 8.1.4.2.1.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4248  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-241075	Correction of test case 8.1.4.2.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4295  Cat: F (Rel-17)

					Source: NTTDOCOMO, INC.

Discussion:

CR cover value : 17.5.0.

r1

Decision: 		The document was revised to R5-241516.

R5-241516	Correction of test case 8.1.4.2.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4295  rev 1 Cat: F (Rel-17)

					Source: NTTDOCOMO, INC.

(Replaces R5-241075)

Decision: 		The document was agreed.

6.4.4.4.1.5	RRC Others (clause 8.1.5)

R5-240034	Correction to MUSIM test case 8.1.5.10.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4159  Cat: F (Rel-17)

					Source: CATT, TDIA

Discussion:

title

3GU had wrong ver.

r2

Decision: 		The document was revised to R5-241517.

R5-241517	Correction to MUSIM test case 8.1.5.10.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4159  rev 1 Cat: F (Rel-17)

					Source: CATT, TDIA

(Replaces R5-240034)

Decision: 		The document was agreed.

R5-240183	Correction to NR RRC TC 8.1.5.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4178  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241518.

R5-241518	Correction to NR RRC TC 8.1.5.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4178  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240183)

Decision: 		The document was agreed.

R5-240184	Correction to NR RRC TC 8.1.5.8.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4179  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240239	Correction to NR testcase 8.1.5.11.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4195  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Decision: 		The document was agreed.

R5-240542	Updates for NR RRC test case 8.1.5.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4249  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240611	Updates to NR CA TCs 8.1.5.x.y.z

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4258  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-241164	Correction to title of 8.1.5.9.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4299  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Discussion:

deferred.

r1

Decision: 		The document was revised to R5-241643.

R5-241643	Correction to title of 8.1.5.9.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4299  rev 1 Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

(Replaces R5-241164)

Decision: 		The document was agreed.

6.4.4.4.1.6	RRC SON and MDT support for NR (clause 8.1.6)

R5-240095	Update of test case 8.1.6.1.2.14 for SON_MDT

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4161  Cat: F (Rel-17)

					Source: CMCC, CATT

Abstract:

Update SON/MDT TC 8.1.6.1.2.14 Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism to add one test conditions and modify the test procedure and related specific message content to resolve the problem that the UE may not be able to detect the IDC problem.

The idle UE logging suspension is “at least 10s gap” is not accurate.

Update SON/MDT TC 8.1.6.1.2.14 Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism

1. Add one test condition to facilitate the subsequent test procedure

2. Modify the test procedure and related specific message content to resolve the problem that the UE may not be able to detect the IDC problem.

3. The idle UE logging suspension “at least 10s gap” is revised into “about 10s gap”.

Discussion:

3GU had wrong ver.

comments from TF160.

r3

RAN5 Chair: we have to find out from the core experts what triggers IDC, where is it defined, before starting to test.

Deferred.

r4

Decision: 		The document was revised to R5-241635.

R5-241635	Update of test case 8.1.6.1.2.14 for SON_MDT

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4161  rev 1 Cat: F (Rel-17)

					Source: CMCC, CATT

(Replaces R5-240095)

Decision: 		The document was agreed.

R5-240867	Correction to NR TC 8.1.6.1.3.8

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4275  Cat: F (Rel-17)

					Source: Huawei, Hisilicon

Discussion:

offline comments from TF160.

Decision: 		The document was revised to R5-241641.

R5-241641	Correction to NR TC 8.1.6.1.3.8

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4275  rev 1 Cat: F (Rel-17)

					Source: Huawei, Hisilicon

(Replaces R5-240867)

Decision: 		The document was agreed.

R5-240868	Correction to NR TC 8.1.6.1.3.9

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4276  Cat: F (Rel-17)

					Source: Huawei, Hisilicon

Discussion:

same as R5-240867.

r1

Decision: 		The document was revised to R5-241642.

R5-241642	Correction to NR TC 8.1.6.1.3.9

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4276  rev 1 Cat: F (Rel-17)

					Source: Huawei, Hisilicon

(Replaces R5-240868)

Decision: 		The document was agreed.

R5-241045	Correction to IRAT MDT test case 8.1.6.2.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4290  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

6.4.4.4.1.7	RRC Non-public networks (clause 8.1.7)

###### 6.4.4.4.2	MR-DC RRC

6.4.4.4.2.1	RRC UE Capability / Others (clause 8.2.1)

R5-240185	Correction to NR RRC TC 8.2.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4180  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241519.

R5-241519	Correction to NR RRC TC 8.2.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4180  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240185)

Decision: 		The document was agreed.

R5-240186	Correction to NR RRC TC 8.2.1.1.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4181  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241520.

R5-241520	Correction to NR RRC TC 8.2.1.1.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4181  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240186)

Decision: 		The document was agreed.

R5-240543	Updates for EN-DC RRC test case 8.2.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4250  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240544	Updates for NE-DC RRC test case 8.2.1.1.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4251  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

6.4.4.4.2.2	RRC Radio Bearer (clause 8.2.2)

R5-240442	Correction to NEDC test case 8.2.2.8.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4234  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241521.

R5-241521	Correction to NEDC test case 8.2.2.8.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4234  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240442)

Decision: 		The document was agreed.

R5-240545	Update to NE-DC RRC test case 8.2.2.2.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4252  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

6.4.4.4.2.3	RRC Measurement / Handovers (clause 8.2.3)

6.4.4.4.2.4	RRC Carrier Aggregation (clause 8.2.4)

R5-240613	Updates to NR CA TCs 8.2.4.x.y.z

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4259  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

6.4.4.4.2.5	RRC Reconfiguration / Radio Link Failure (clause 8.2.5)

6.4.4.4.2.6	RRC Others (clause 8.2.6)

R5-240243	Correction to NR testcase 8.2.6.3.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4198  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Decision: 		The document was agreed.

R5-240244	Correction to NR testcase 8.2.6.3.6

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4199  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Decision: 		The document was agreed.

6.4.4.4.2.7	RRC Resume (clause 8.2.7)

##### 6.4.4.5	5GS Mobility Management

###### 6.4.4.5.1	MM Primary authentication and key agreement (clause 9.1.1)

###### 6.4.4.5.2	MM Security mode control / Identification & Generic UE configuration update (clauses 9.1.2 / 9.1.3 & 9.1.4)

R5-240609	Correction of NR TC 9.1.4.1-CUC

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4257  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

comment from TF160 and Qualcomm

r1

Decision: 		The document was revised to R5-241522.

R5-241522	Correction of NR TC 9.1.4.1-CUC

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4257  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240609)

Decision: 		The document was agreed.

###### 6.4.4.5.3	MM Registration & De-registration (clauses 9.1.5 & 9.1.6)

R5-240512	Correction to NR5GC testcase 9.1.5.1.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4240  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ, Hisilicon, MediaTek

Discussion:

deferred

r1

Decision: 		The document was revised to R5-241644.

R5-241644	Correction to NR5GC testcase 9.1.5.1.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4240  rev 1 Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ, Hisilicon, MediaTek

(Replaces R5-240512)

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241687.

R5-241687	Correction to NR5GC testcase 9.1.5.1.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4240  rev 2 Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ, Hisilicon, MediaTek

(Replaces R5-241644)

Discussion:

Email agreed

Decision: 		The document was agreed.

###### 6.4.4.5.4	MM Service Request (clause 9.1.7)

###### 6.4.4.5.5	MM SMS Over NAS (clause 9.1.8)

###### 6.4.4.5.6	RACS (clause 9.1.9)

R5-241158	Correction to RACS Test case 9.1.9.7

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4296  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Decision: 		The document was agreed.

###### 6.4.4.5.7	MM Network slice-specific authentication and authorization (clause 9.1.10)

###### 6.4.4.5.8	MM SNPN(clause 9.1.11)

###### 6.4.4.5.9	MM NSAC/NSSRG(clauses 9.1.12 & 9.1.13)

R5-240187	Correction to eNS NSAC TC 9.1.12.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4182  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

##### 6.4.4.6	5GS Non-3GPP Access Mobility Management (clause 9.2)

##### 6.4.4.7	5GS Inter-system Mobility (clause 9.3)

##### 6.4.4.8	5GS Session Management

###### 6.4.4.8.1	SM PDU session authentication and authorization (clause 10.1.1)

###### 6.4.4.8.2	SM Network-requested PDU session modification & release (clauses 10.1.2 & 10.1.3)

###### 6.4.4.8.3	SM UE-requested PDU session establishment / modification & release (clauses 10.1.4 / 10.1.5 & 10.1.6)

###### 6.4.4.8.4	SM NSAC (clauses 10.1.8)

R5-240644	Correction to SM NSAC Testcase  10.1.8.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4262  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Discussion:

3GU bug

Decision: 		The document was agreed.

R5-241161	Correction to NSAC Test case 10.1.8.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4297  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Discussion:

3GU bug

MCC160 comments

r1

Decision: 		The document was revised to R5-241523.

R5-241523	Correction to NSAC Test case 10.1.8.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4297  rev 1 Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

(Replaces R5-241161)

Decision: 		The document was agreed.

##### 6.4.4.9	EN-DC Session Management (clause 10.2)

##### 6.4.4.10	5GS Non-3GPP Access & ATSS Session Management (clauses 10.3 & 10.4)

R5-240472	Correction to 5GSM test case 10.3.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4235  Cat: F (Rel-17)

					Source: Keysight Technologies UK, Samsung

Discussion:

r1

Decision: 		The document was revised to R5-241524.

R5-241524	Correction to 5GSM test case 10.3.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4235  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK, Samsung

(Replaces R5-240472)

Decision: 		The document was agreed.

##### 6.4.4.11	5GS Multilayer and Services

###### 6.4.4.11.1	EPS Fallback (clause 11.1)

R5-240950	Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4278  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD

Discussion:

r2

Decision: 		The document was revised to R5-241526.

R5-241526	Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4278  rev 1 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD

(Replaces R5-240950)

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241688.

R5-241688	Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4278  rev 2 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD, Rohde & Schwarz

(Replaces R5-241526)

Discussion:

coversheet revision after the meeting

Decision: 		The document was revised to R5-241691.

R5-241691	Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4278  rev 3 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD, Rohde & Schwarz

(Replaces R5-241688)

Discussion:

Email agreed

Decision: 		The document was agreed.

###### 6.4.4.11.2	5G-SRVCC (clause 11.2)

###### 6.4.4.11.3	Unified Access Control (UAC) (clause 11.3)

R5-240022	Correction to NR5GC test cases 11.3.6 and 11.3.6a

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4156  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Discussion:

r1

Qualcomm commented about close test loop.

Keysight were able to verify the fix for the TTCN issue found without the need of a prose CR.->w/d

Decision: 		The document was revised to R5-241527.

R5-241527	Correction to NR5GC test cases 11.3.6 and 11.3.6a

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4156  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

(Replaces R5-240022)

Decision: 		The document was withdrawn.

R5-240510	Correction to NR5GC testcase 11.3.11

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4238  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ, Qualcomm

Decision: 		The document was agreed.

###### 6.4.4.11.4	Emergency Services (clause 11.4)

R5-240230	Correction to NR testcase 11.4.12

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4193  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Discussion:

r2

Decision: 		The document was revised to R5-241528.

R5-241528	Correction to NR testcase 11.4.12

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4193  rev 1 Cat: F (Rel-17)

					Source: Starpoint, TDIA

(Replaces R5-240230)

Decision: 		The document was agreed.

R5-240322	Correction to emergency service TC 11.4.4

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4207  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was withdrawn.

R5-240416	Editorial correction of NR TC 11.4.x-Emergency services

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4222  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

is not an editorial CR.

Decision: 		The document was agreed.

R5-240509	Correction to NR5GC testcase 11.4.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4237  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-241529.

R5-241529	Correction to NR5GC testcase 11.4.5

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4237  rev 1 Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ, Hisilicon

(Replaces R5-240509)

Decision: 		The document was agreed.

R5-240951	Correction to NR 5GC Multilayer Emergency TC 11.4.12

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4279  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD

Discussion:

r1

Decision: 		The document was revised to R5-241530.

R5-241530	Correction to NR 5GC Multilayer Emergency TC 11.4.12

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4279  rev 1 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD

(Replaces R5-240951)

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241689.

R5-241689	Correction to NR 5GC Multilayer Emergency TC 11.4.12

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4279  rev 2 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD

(Replaces R5-241530)

Discussion:

coversheet revision after the meeting

Decision: 		The document was revised to R5-241692.

R5-241692	Correction to NR 5GC Multilayer Emergency TC 11.4.12

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4279  rev 3 Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies, ANRITSU LTD

(Replaces R5-241689)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-241308	Correction to 5G NR TC 11.4.13

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4304  Cat: F (Rel-17)

					Source: MCC TF160

Discussion:

r1

Decision: 		The document was revised to R5-241531.

R5-241531	Correction to 5G NR TC 11.4.13

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4304  rev 1 Cat: F (Rel-17)

					Source: MCC TF160

(Replaces R5-241308)

Decision: 		The document was agreed.

###### 6.4.4.11.5	eCall over IMS (clause 11.5)

R5-241366	Update of GERAN Signal levels to eCall signalling tests

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4306  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Discussion:

late doc

Decision: 		The document was withdrawn.

###### 6.4.4.11.6	3GPP PS Data Off (clause 11.6)

###### 6.4.4.11.7	Inter-system mobility between untrusted Non-3GPP and 3GPP system (clause 11.8)

R5-240539	Correction to NR test case 11.8.6

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4246  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

###### 6.4.4.11.8	MBS (clause 14)

R5-240188	Correction to MBS TC 14.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4183  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240189	Correction to MBS TC 14.1.1.2

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4184  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240190	Correction to MBS TC 14.1.1.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4185  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241532.

R5-241532	Correction to MBS TC 14.1.1.3

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4185  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240190)

Decision: 		The document was agreed.

R5-240191	Correction to MBS TC 14.1.1.4.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4186  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240192	Correction to MBS TC 14.1.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4187  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-241533.

R5-241533	Correction to MBS TC 14.1.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4187  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-240192)

Decision: 		The document was agreed.

R5-240193	Correction to MBS TC 14.2.4.2.1

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4188  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-240435	Correction of  MBS broadcast TC 14.1.1.1-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4227  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241534.

R5-241534	Correction of  MBS broadcast TC 14.1.1.1-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4227  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240435)

Decision: 		The document was agreed.

R5-240436	Correction of  MBS broadcast TC 14.1.1.3-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4228  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241535.

R5-241535	Correction of  MBS broadcast TC 14.1.1.3-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4228  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240436)

Decision: 		The document was agreed.

R5-240437	Correction of  MBS broadcast TC 14.1.2.1-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4229  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241536.

R5-241536	Correction of  MBS broadcast TC 14.1.2.1-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4229  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240437)

Decision: 		The document was agreed.

R5-240438	Correction of  MBS broadcast TC 14.1.2.2-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4230  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240439	Correction of  MBS broadcast TC 14.1.2.3-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4231  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240440	Correction of  MBS broadcast TC 14.1.3.1-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4232  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

late doc

r1

Decision: 		The document was revised to R5-241537.

R5-241537	Correction of  MBS broadcast TC 14.1.3.1-Test Loop Mode C

Type: CR		For: Agreement
					38.523-1 v17.5.1	  CR-4232  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-240440)

Decision: 		The document was agreed.

#### 6.4.5	TS 38.523-2

R5-240546	Misc. updates to TS 38.523-2

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0437  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240662	Corrections to applicability of Network Slice Admission Control (NSAC) mobility management aspects test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0443  Cat: F (Rel-17)

					Source: Qualcomm CDMA Technologies

Decision: 		The document was withdrawn.

R5-241159	Correction to applicability of EN-DC CA test cases

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0447  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-241167	Correction to title of 8.1.5.9.1

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0448  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Discussion:

r1

Decision: 		The document was revised to R5-241645.

R5-241645	Correction to title of 8.1.5.9.1

Type: CR		For: Agreement
					38.523-2 v17.5.0	  CR-0448  rev 1 Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

(Replaces R5-241167)

Decision: 		The document was agreed.

#### 6.4.6	TS 38.523-3

R5-240428	Add PIXIT for MUSIM gap

Type: CR		For: Agreement
					38.523-3 v17.9.0	  CR-3373  Cat: F (Rel-17)

					Source: Huawei, HiSilicon,China Telecom

Discussion:

late doc

Decision: 		The document was withdrawn.

R5-240547	Routine maintenance for TS 38.523-3

Type: CR		For: Agreement
					38.523-3 v17.9.0	  CR-3377  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

#### 6.4.7	Discussion Papers / Work Plan / TC lists

R5-240096	Discussion on handling of MDT IDC problem

Type: discussion		For: Endorsement
					Source: CMCC

Abstract:

Observation 1: For TC 8.1.6.1.2.14, it is assumed once UEs are configured as “T1”, IDC problem will be detected by the UEs.

Actually, for UEs with good IDC mechanism and solutions, no IDC problems might be detected by the UEs. Hence, UEs may NOT suspend measurement logging or tag MDT report with inDeviceCoexDetected-r17 flag as expected. And these UEs will FAIL TC 8.1.6.1.2.14. Proposal 1: Before testing the IDC mechanism, to confirm UE will detect IDC problem with “T1”configuration.

Observation 2: For TC 8.1.6.1.2.14, it is assumed once UEs are configured as “T0/T2”, IDC problem will NOT be detected by the UEs.

Actually, UEs with “T0/T2” configuration may still detect IDC problems. Hence, UEs may NOT resume logging as expected. And these UEs will FAIL TP2 of TC 8.1.6.1.2.14.Proposal 2: Before testing the IDC mechanism, to confirm UE will NOT detect IDC problem with “T0/T2”configuration.

Observation 3: In cl. 23.4, it says “IDC problem can happen when the UE (intends to) uses WLAN on the overlapped carrier/band or adjacent carrier/band to the unlicensed carrier used for LAA operation, e.g. when related UE hardware components, such as antennas, are shared between LAA and WLAN operations”.

Observation 4: As per cl.5.7.4.2 in TS 38.331, UE detecting IDC problem will providing IDC assistance information in RRC_CONNECTED if the UE was configured to do so.

Observation 5: If UEAssistanceInformation message including IDC-Assistance-r16 is received by NW/SS, it is confirmed that UE can detect IDC problem.

Proposal 3: When checking whether UE will suspends measurement logging when IDC problem is detected, if UE detects IDC problem in RRC_CONNECTED with some configuration, to configure the IDLE UE with the same configuration as in RRC_CONNECTED to make the UE detect IDC problem in RRC_IDLE.

Proposal 4: When checking whether UE will resume logging when IDC problem is resolved, if UE does NOT detect IDC problem in RRC_CONNECTED with some configuration, to configure the IDLE UE with the same configuration as in RRC_CONNECTED to make the UE NOT detect IDC problem in RRC_IDLE.

Discussion:

r1

is UE specific.

discussion with Qualcomm, CATT, TF160.

Deferred.

Decision: 		The document was revised to R5-241634.

R5-241634	Discussion on handling of MDT IDC problem

Type: discussion		For: Endorsement
					Source: CMCC

(Replaces R5-240096)

Decision: 		The document was noted.

R5-240431	Discussion to update MUSIM gap TC

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon,China Telecom

Abstract:

Issue: How to make sure that RAN5 TC are testing the MUSIM gap pattern supported by UE?

Observation 1: 38.133 list 27 periodic MUSIM gaps and 2 aperiodic MUSIM gaps, but 38.306 only require UE to support 3 periodic MUSIM gaps and 1 aperiodic MUSIM gap.

Therefore, core spec does not require UE to support all kinds of gaps and it does not list which gap pattern is mandatory and which gap pattern is optional, either.

Observation 2: According to 38.331, for periodic gap, network always configured the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred periodic MUSIM gap configuration. For aperiodic gap, network always configured the same musim-Starting-SFN-AndSubframe as indicated by the UE's preferred aperiodic MUSIM gap configuration. The UE's preferred MUSIM gap configuration is indicated in the MUSIM UAI.

Therefore, if UE only support part of MUSIM gap listed in 38.133, and UE always indicated that supported MUSIM gap pattern to network through MUSIM UAI, network have no chance to configure MUSIM gap pattern unsupported by the UE. In addition, If TC directly (e.g 38.523-1 8.1.2.1.6) configure MUSIM gap to UE, the UE behaviour is not specified by 38.331.

Observation 3: Accroding to 38.331, UE may not trigger MUSIM UAI automatically after network configuration. In order to trigger MUSIM UAI, RAN5 design Set MUSIM UAI test function to force UE to report any type of MUSIM gap in MUSIM UAI.

“Set MUSIM UAI test function” is designed based on an assumption that UE support all the MUSIM gap pattern listed in 38.133. However based on observation 1/2, there is not enough evidence to support this assumption.

Observation 4: RAN4 has defined the RRM requirement for Rel-17 MUSIM gaps in R18 WI NR_DualTxRx_MUSIM. It is highly possible that RRM MUSIM gaps TC will face similar issue in the future.

2	Proposal

Based on the above discussion, the followings are proposed:

Proposal 1: Add 4 PIXIT in 38.523-3, UE tester can fill the PIXIT based on the MUSIM gap supported by UE.

Proposal 2: Update 38.523-1 8.1.2.1.6 to add MUSIM UAI trigger procedure (MUSIM UAI triggered by “Set MUSIM UAI test function” or other method) before MUSIM gap configuration. Only the gap pattern filled in the px_NR_SupportPeriodicMusimGapPattern_First/px_NR_SupportPeriodicMusimGapPattern_Second/px_NR_SupportAperiodicMusimGapPattern can be test in this test case.

Discussion:

comment from CATT

r4

Decision: 		The document was revised to R5-241633.

R5-241633	Discussion to update MUSIM gap TC

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon,China Telecom

(Replaces R5-240431)

Decision: 		The document was noted.

R5-240663	Discussion Paper for Rel-17 eNS_Ph2 WI tests case applicability

Type: discussion		For: Discussion
					Source: Qualcomm CDMA Technologies

Abstract:

Observation #1 – The R-17 UE has capability to indicate support of ER-NSSAI; UE’s can set this to “supported” after network IOT

Proposal#1 – Based on Observation#1, The following test cases under NR Protocol test cases defined in TS38.523-1 [2] for NSAC Mobility Management aspects test cases (Cl 9.1.12.1-5) and NSAC interworking with EPC shall be defined with an applicability in TS38.523-2 [3] and associated PICS shall be defined in 38.508-2 [4].

Below is a recap of some alternate proposals which can be considered to address this and were also discussed in RAN5#101

Alt Proposal-1 – The rapportuer of this WI to ensure that sufficient evidence exists of this feature commecialization along with IOT opportunities and ONLY then introduce this WI in certification bodies.

Alt Proposal-2 – For features, like the ones discussed in this paper, RAN5 can control the Optionality by using existing tables in TS 38.508-2 (ex- below column M) and then keep revisiting every meeting if the status of column M needs to be changed, this can be a running AP as well.

Observation#2 - None of the SM test case introduced are testing unique UE requirement specific to R-17 eNS_Ph2 WI. This may cause even earlier UE release to PASS this test case but more importantly the UE requirements are not part of R-17 eNS_Ph2 WI

Proposal#2 – The session management test cases introduced in the NSAC WI are not R-17 eNS_Ph2 unique requirements for UE hence they are misaligned in specification; these tests are currently mandatory for R-17 UE which is incorrect.

Proposal#2-1 – All NSAC SM test cases to be “VOIDED” from Spec (TS38.523-1[2] and TS 523-2 [3])

Proposal#2-2 – Study them again to be reintroduced as R-15 through 5GS maintenance WI on a need basis. Any R-17 eNS_Ph2 UE specific requirement, if found, can be covered in TEI_17 WI.

As this AP has not progressed, below w/f can be applied –

Proposal 2-2A – Add note in applicabilty that these tests have open issues to be addressed, verification of new tests shall be paused unitl the AP is complete.

Discussion:

0662!

RAN5 Vice Chair: are these deployed in Rel-15 and 16?

r1

CMCC to comment further.

AP #2 kept open, #1 kept open.

Support of NR- ENSSAI test cases will be made optional.

Decision: 		The document was revised to R5-241479.

R5-241479	Discussion Paper for Rel-17 eNS_Ph2 WI tests case applicability

Type: discussion		For: Discussion
					Source: Qualcomm CDMA Technologies

(Replaces R5-240663)

Decision: 		The document was noted.

R5-240870	TS 38.523-1 Tracker status before RAN5-102

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

### 6.5	Routine Maintenance for TS 36 Series TEIx_Test

#### 6.5.1	Routine Maintenance for TS 36.508

#### 6.5.2	Routine Maintenance for TS 36.509

#### 6.5.3	Routine Maintenance for TS 36.523-1

##### 6.5.3.1	Idle Mode

##### 6.5.3.2	Layer 2

###### 6.5.3.2.1	MAC

###### 6.5.3.2.2	RLC

###### 6.5.3.2.3	PDCP

##### 6.5.3.3	RRC

###### 6.5.3.3.1	RRC Part 1 (clauses 8.1 and 8.5)

R5-240548	Correction to LTE Inter-RAT TC 8.1.3.7

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5282  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240585	Corrections to LTE TC 8.1.3.16

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5283  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-240977	Addition of new L2L MPS priority access barring test case

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5287  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-241475.

R5-241475	Addition of new L2L MPS priority access barring test case

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5287  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-240977)

Decision: 		The document was agreed.

R5-241179	Addition of new test case 8.1.3.8a Redirection to GERAN-Redir-policy bit

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5295  Cat: F (Rel-18)

					Source: Vodafone

Discussion:

r1

TF160 needed more time to review.

R&S: maybe Step 7 not needed.

r2

Decision: 		The document was revised to R5-241476.

R5-241476	Addition of new test case 8.1.3.8a Redirection to GERAN-Redir-policy bit

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5295  rev 1 Cat: F (Rel-18)

					Source: Vodafone

(Replaces R5-241179)

Decision: 		The document was agreed.

R5-241180	Addition of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5296  Cat: F (Rel-18)

					Source: Vodafone

Discussion:

r2

Decision: 		The document was revised to R5-241477.

R5-241477	Addition of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5296  rev 1 Cat: F (Rel-18)

					Source: Vodafone

(Replaces R5-241180)

Decision: 		The document was agreed.

###### 6.5.3.3.2	RRC Part 2 (clause 8.2) /

###### 6.5.3.3.3	RRC Part 3 (clause 8.3)

###### 6.5.3.3.4	Inter-RAT (clauses 8.4 & 8.4A)

###### 6.5.3.3.5	RRC LTE MDT (clause 8.6)

###### 6.5.3.3.6	RRC ANR for UTRAN (clause 8.7)

##### 6.5.3.4	EPS Mobility Management

R5-241494	Correction to RACS Test case 9.2.5.X

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5297  Cat: F (Rel-18)

					Source: Anritsu Ltd

Discussion:

late doc

r1

Deferred.

TF160 review.

Decision: 		The document was revised to R5-241627.

R5-241627	Correction to RACS Test case 9.2.5.X

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5297  rev 1 Cat: F (Rel-18)

					Source: Anritsu Ltd

(Replaces R5-241494)

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-241690.

R5-241690	Correction to RACS Test case 9.2.5.X

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5297  rev 2 Cat: F (Rel-18)

					Source: Anritsu Ltd

(Replaces R5-241627)

Discussion:

Email agreed

Decision: 		The document was agreed.

##### 6.5.3.5	EPS Session Management

##### 6.5.3.6	General Tests

R5-241065	Addition of new test case 11.2.13

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5294  Cat: F (Rel-18)

					Source: NTT DOCOMO, INC. Rohde & Schwarz

Discussion:

r3

Decision: 		The document was revised to R5-241624.

R5-241624	Addition of new test case 11.2.13

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5294  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO, INC. Rohde & Schwarz

(Replaces R5-241065)

Decision: 		The document was agreed.

##### 6.5.3.7	Interoperability Radio Bearers

##### 6.5.3.8	Multilayer Procedures

##### 6.5.3.9	PWS - ETWS / CMAS

##### 6.5.3.10	Non-3GPP

##### 6.5.3.11	Others (TS 36.523-1 clauses not covered by other AIs under AI 6.5.3 / e.g. eMBMS / Home (e)NB / MBMS in LTE / D2D / SC-PTM / NB-IoT / CIoT...)

R5-240021	Correction to LTE RACS test case 8.5.5.1

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5298  Cat: F (Rel-18)

					Source: Keysight Technologies UK, Qualcomm

Discussion:

late doc

Tdoc, CR#

Decision: 		The document was revised to R5-241626.

R5-241626	Correction to LTE RACS test case 8.5.5.1

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5298  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK, Qualcomm

(Replaces R5-240021)

Decision: 		The document was agreed.

R5-240474	Correction to SIM profile of NB-IoT test cases

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5281  Cat: F (Rel-18)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-240586	Corrections to NB-IoT RLC TC 22.3.2.3

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5284  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was revised to R5-241478.

R5-241478	Corrections to NB-IoT RLC TC 22.3.2.3

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5284  rev 1 Cat: F (Rel-18)

					Source: MCC TF160

(Replaces R5-240586)

Decision: 		The document was agreed.

R5-240820	Correction to NBIOT testcase 22.4.13

Type: CR		For: Agreement
					36.523-1 v18.3.0	  CR-5286  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was withdrawn.

#### 6.5.4	Routine Maintenance for TS 36.523-2

R5-240978	Addition of applicability for L2L MPS priority access barring test case

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1440  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-241067	Addition of applicability for new test case 11.2.13

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1444  Cat: F (Rel-18)

					Source: NTTDOCOMO, INC.

Discussion:

CR cover value : TEI15_Test.

r1

Decision: 		The document was revised to R5-241625.

R5-241625	Addition of applicability for new test case 11.2.13

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1444  rev 1 Cat: F (Rel-18)

					Source: NTTDOCOMO, INC.

(Replaces R5-241067)

Decision: 		The document was agreed.

R5-241181	Addition of applicability of new test case 8.1.3.8a for redir-policy bit

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1445  Cat: F (Rel-18)

					Source: Vodafone

Discussion:

r2

Decision: 		The document was revised to R5-241622.

R5-241622	Addition of applicability of new test case 8.1.3.8a for redir-policy bit

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1445  rev 1 Cat: F (Rel-18)

					Source: Vodafone

(Replaces R5-241181)

Decision: 		The document was agreed.

R5-241194	Addition of applicability of new test case 8.1.3.6b for redir-policy bit

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1446  Cat: F (Rel-18)

					Source: Vodafone

Discussion:

r2

Decision: 		The document was revised to R5-241623.

R5-241623	Addition of applicability of new test case 8.1.3.6b for redir-policy bit

Type: CR		For: Agreement
					36.523-2 v18.3.0	  CR-1446  rev 1 Cat: F (Rel-18)

					Source: Vodafone

(Replaces R5-241194)

Decision: 		The document was agreed.

#### 6.5.5	Routine Maintenance for TS 36.523-3

R5-240549	Routine maintenance for TS 36.523-3

Type: CR		For: Agreement
					36.523-3 v18.2.0	  CR-4786  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

#### 6.5.6	Discussion Papers / Work Plan / TC lists

R5-240871	TS 36.523-1 Tracker status before RAN5-102

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-241063	Proposal for new test case related to ATTACH NEEDED STATE

Type: discussion		For: Endorsement
					36.523-1 v..
					Source: NTTDOCOMO, INC.

Abstract:

I have a proposal to check emergency call from substate of EMM-DEREGISTERED.ATTACH-NEEDED.

Proposal: RAN5 make a new test case to check above.

This test case is applied to UEs supporting E-UTRA and IMS emergency call.

If this proposal is agreed, following new test case has already been submitted.

11.2.13 Emergency bearer services / Normal cell / ATTACH-SERVICE / ATTACH-NEEDED STATE / PDN connect

# test case:

R5-241065

# Test applicability:

R5-241067

Discussion:

the proposal is accepted.

Decision: 		The document was noted.

R5-241178	Proposal for new test case related to network policy indication on LTE call redirection to GERAN and UTRAN

Type: discussion		For: Endorsement
					Source: Vodafone

Abstract:

In the past year false base station attacks are on the rise. Most false base station attacks work by downgrading the victim from 4G to 2G or 3G to exploit weaknesses in these earlier technologies.

Downgrade attack methods

1.	Downgrade via insecure RRC redirection

2.	Downgrade via tampering with system information to affect idle mode cell reselection

The insecure redirection is allowed by 3GPP for CSFB optimisation, a 4G RAN can redirect UEs to 2G or 3G  prior to security establishment. False base stations exploit this to perform downgrade attacks.

3GPP CT1 introduced the network policy information element including redir-policy bit sent in the ATTACH ACCEPT and TRACKING AREA UPDATE ACCEPT from Rel-14 onwards [1]. 3GPP RAN2 modified the UE behaviour at reception of RRCConnectionRelease such that UE, conditional on indication by NAS , discards redirection to GERAN, unless AS security is activated [2].

For Rel-17 the redir-policy bit was extended to prohibit insecure redirect from 4G to 3G and mandate network support [3], in [4] the UE behaviour was adapted accordingly.

2.	Discussion

The discussion is about adding the following test cases to prevent unsecure redirections to GERAN and UTRAN by setting redir-policy bit.

According to current TS 36.523-1 [5], there is no test case which checks whether the UE discards the redirection from E-UTRAN to GERAN unless AS security is activated. The same applies for the redirection from E-UTRAN to UTRAN.

3.	Proposal

RAN5 to add test cases to cover the redirection scenarios from E-UTRAN to GERAN [6], [7] and UTRAN [8], [9] under consideration of the redir-policy as described above.

Discussion:

noted, proposal accepted.

Decision: 		The document was noted.

### 6.6	Other Maintenance TEIx_Test

#### 6.6.1	Routine Maintenance for TS 34.108

#### 6.6.2	Routine Maintenance for TS 34.109

#### 6.6.3	Routine Maintenance for TS 34.123

##### 6.6.3.1	TS 34.123-1

R5-240550	Correction to UTRAN Inter-RAT TC 8.6.3.3

Type: CR		For: Agreement
					34.123-1 v15.6.0	  CR-3939  Cat: F (Rel-15)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240975	Removal of GEA2 support from inter-rat cell change order test cases

Type: CR		For: Agreement
					34.123-1 v15.6.0	  CR-3940  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.3.2	TS 34.123-2

R5-240976	Removal of GEA2 support from applicability of inter-rat cell change order test cases

Type: CR		For: Agreement
					34.123-2 v15.6.1	  CR-0801  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.3.3	TS 34.123-3

R5-240551	Removal of GEA2 from TS 34.123-3

Type: CR		For: Agreement
					34.123-3 v17.3.0	  CR-3676  Cat: F (Rel-17)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

#### 6.6.4	Discussion Papers / Work Plan / TC lists

#### 6.6.6	Routine Maintenance for TS 34.229

##### 6.6.6.1	TS 34.229-1

##### 6.6.6.2	TS 34.229-2

R5-240552	Correction of clause 4

Type: CR		For: Agreement
					34.229-2 v16.9.0	  CR-0336  Cat: F (Rel-16)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-241579	Updating applicability for video

Type: CR		For: Agreement
					34.229-2 v16.9.0	  CR-0339  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

late doc

for email agreement

Email agreed

Decision: 		The document was agreed.

R5-241417	Correction to Video capability

Type: CR		For: Agreement
					34.229-2 v16.9.0	  CR-0338  Cat: F (Rel-16)

					Source: MediaTek Inc.

Discussion:

late doc

merged into R5-241411.

Decision: 		The document was withdrawn.

##### 6.6.6.3	TS 34.229-3

##### 6.6.6.4	TS 34.229-4

##### 6.6.6.5	TS 34.229-5

R5-240866	Correction to 34229-5 A.4.2a

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0568  Cat: F (Rel-16)

					Source: Huawei, Hisilicon

Decision: 		The document was withdrawn.

R5-241190	Update to generic procedure A.4.2a

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0569  Cat: F (Rel-16)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241191	Updates to test case 7.1

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0570  Cat: F (Rel-16)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241192	Updates to test case 7.5

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0571  Cat: F (Rel-16)

					Source: Ericsson

Discussion:

TF160 manager: this change is against Proposal 3.

Deferred.

r1

Decision: 		The document was revised to R5-241655.

R5-241655	Updates to test case 7.5

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0571  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R5-241192)

Decision: 		The document was agreed.

R5-241193	Updates to test case 7.12

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0572  Cat: F (Rel-16)

					Source: Ericsson

Discussion:

TF160 manager: need to update TP3.

Deferred.

r1

Decision: 		The document was revised to R5-241656.

R5-241656	Updates to test case 7.12

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0572  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R5-241193)

Decision: 		The document was agreed.

R5-241252	New generic procedure A.15.2a

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0573  Cat: F (Rel-16)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-241554.

R5-241554	New generic procedure A.15.2a

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0573  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R5-241252)

Decision: 		The document was agreed.

R5-241253	New generic procedure A.16.2a

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0574  Cat: F (Rel-16)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-241555.

R5-241555	New generic procedure A.16.2a

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0574  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R5-241253)

Decision: 		The document was agreed.

R5-241254	Updates to test case 7.15

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0575  Cat: F (Rel-16)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241255	Updates to test case 7.17

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0576  Cat: F (Rel-16)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241256	Updates to test case 7.21

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0577  Cat: F (Rel-16)

					Source: Ericsson

Decision: 		The document was agreed.

R5-241257	Updates to test case 7.23

Type: CR		For: Agreement
					34.229-5 v16.9.0	  CR-0578  Cat: F (Rel-16)

					Source: Ericsson

Decision: 		The document was agreed.

##### 6.6.6.6	Discussion Papers / Work Plan / TC lists

R5-240643	NG.114 test coverage

Type: discussion		For: Endorsement
					Source: Ericsson

Abstract:

For NG.114 [1] we’ve agreed to provide test coverage using the default EVS Configuration A2. We’ve have also conluded that additional testing for any other non default EVS configuration is currently not desired.

For preconditions we’ve have, in addition to the default with the precondition mechanism enabled, also agreed to provide additional test coverage for the non default configuration when the use of preconditions is disabled. In test cases e.g. expressed as ” The UE is configured to not use preconditions.”.

Until now, we’ve not discussed any test coverage for any other non default configurations in detail. However, a discussion on the Reliable 18x policy has just started.

In addition we’ve also allowed an exemption for the NG.114 [1] video feature, adopted a compromise on test coverage for e.g. test case 7.1 and there’s a discussion on IMS for RedCap.

It would be helpful if we can agree on some principles for NG.114 [1] test coverage.

2	Discussion

An essential part of the RAN5 outcome is the test coverage, and expectations, for different UEs. Specifically for IMS and NG.114 [1] some agreement and clarifications may help.

It would be reasonable e.g., if the test coverage for the mandatory default configuration is fulfilled for every UE. Any additional test coverage should be consistent for every UE.

Below is a proposal for one principle as a guideline.

NG.114 principle 2-1:

The test coverage shall be consistent and aligned for all UEs independent of specific configurations.

3	Proposal

It’s proposed to apply the NG.114 principle 2-1.

Discussion:

TF160 manager: it is not clear what the proposed principle is, and how this would justify the proposed CRs.

After every meeting the test scope is reduced. R&S was adding in the past.

Proposal is accepted only to 100 Rel option tag for DUT and emulate UE.

This will not extend to other features. RAN5 will consider on a case by case basis.

Decision: 		The document was noted.

R5-241310	5G IMS voice test coverage

Type: discussion		For: Endorsement
					Source: Ericsson

Abstract:

For 5G IMS, the 5GS (NR/5GC) and NG.114 has mainly been addressed for IMS voice. But, for 5G and EN-DC the IR.92 applies for IMS voice.

The same requirements e.g. for voice and the EVS default configuration applies for IR.92 [1] and NG.114.

2	Discussion

The EVS default configuration is applied for each NG.114 IMS test case.

All 5G IMS test cases, including IR.92, could also be aligned for the EVS default configuration.

Proposal 2-1:

It’s proposed to clarify the only required and used EVS default configuration for each 5G IMS test case.

3	Proposal

It’s proposed to implement the proposal 2-1.

Discussion:

Ericsson: IR.92 was updated.

RAN5 willing to consider test coverage for EVS default configuration A.2 in IR.92 from Rel-15. Addition of test coverage has to be discussed further with the stakeholders. Qualcomm to check & confirm.

Decision: 		The document was noted.

#### 6.6.7	Routine Maintenance for TS 37.571

##### 6.6.7.1	TS 37.571-2

R5-240513	Correction to NR Physical Cell IDs for NR-PRS assistance data

Type: CR		For: Agreement
					37.571-2 v17.2.0	  CR-0182  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

##### 6.6.7.2	TS 37.571-3

##### 6.6.7.3	TS 37.571-4

R5-240553	Removal of PIXIT for deprecated signalling GNSS scenarios

Type: CR		For: Agreement
					37.571-4 v17.4.0	  CR-0198  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.6.7.4	TS 37.571-5

R5-240554	Removal of deprecated signalling GNSS scenarios

Type: CR		For: Agreement
					37.571-5 v17.3.0	  CR-0230  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.6.7.5	Discussion Papers / Work Plan / TC lists

#### 6.6.8	Routine Maintenance for TS 51.010

##### 6.6.8.1	TS 51.010-1 (Signalling)

R5-240475	Removal of GEA1 and GEA2 algorithm verification

Type: CR		For: Agreement
					51.010-1 v13.13.0	  CR-5168  Cat: F (Rel-13)

					Source: Keysight Technologies UK, Bureau Veritas

Discussion:

r1

block agreed on Thu.

Decision: 		The document was revised to R5-241657.

R5-241657	Removal of GEA1 and GEA2 algorithm verification

Type: CR		For: Agreement
					51.010-1 v13.13.0	  CR-5168  rev 1 Cat: F (Rel-13)

					Source: Keysight Technologies UK, Bureau Veritas

(Replaces R5-240475)

Decision: 		The document was agreed.

R5-240515	Correction to GSM testcase 44.2.5.2.5

Type: CR		For: Agreement
					51.010-1 v13.13.0	  CR-5169  Cat: F (Rel-13)

					Source: ROHDE & SCHWARZ

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.8.2	TS 51.010-2 (Signalling)

R5-240476	Applicability updates to reflect the removal of GEA1 and GEA2 algorithm verification from the test cases

Type: CR		For: Agreement
					51.010-2 v13.15.0	  CR-4412  Cat: F (Rel-13)

					Source: Keysight Technologies UK, Bureau Veritas

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240514	Removal of GEA1/GEA2 and verification of non-support of GEA1/GEA2

Type: CR		For: Agreement
					51.010-2 v13.15.0	  CR-4413  Cat: F (Rel-13)

					Source: ROHDE & SCHWARZ

Discussion:

r1

block agreed on Thu.

Decision: 		The document was revised to R5-241632.

R5-241632	Removal of GEA1/GEA2 and verification of non-support of GEA1/GEA2

Type: CR		For: Agreement
					51.010-2 v13.15.0	  CR-4413  rev 1 Cat: F (Rel-13)

					Source: ROHDE & SCHWARZ

(Replaces R5-240514)

Decision: 		The document was agreed.

##### 6.6.8.3	TS 51.010-5 (Signalling)

R5-240516	Correction to Table A.2: IR_G TTCN test cases

Type: CR		For: Agreement
					51.010-5 v10.20.0	  CR-0152  Cat: F (Rel-10)

					Source: ROHDE & SCHWARZ

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.8.4	TS 51.010-7 (Signalling)

##### 6.6.8.5	Discussion Papers / Work Plan / TC list & CR summary

#### 6.6.9	Routine Maintenance for TS 36.579

##### 6.6.9.1	TS 36.579-1

R5-240555	Corrections of clause 5.3.2

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0330  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240556	Corrections of clause 5.3.29

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0331  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240557	Corrections of clause 5.3.32

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0332  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240558	Corrections of clause 5.4.2

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0333  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240559	Corrections of clause 5.5.1

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0334  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240560	Corrections of clause 5.5.2.11

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0335  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240561	Corrections of clause 5.5.2.13

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0336  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240562	Corrections of clause 5.5.2.19.4

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0337  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240563	Corrections of clause 5.5.2.7.2

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0338  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240564	Corrections of clause 5.5.3.3.1A

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0339  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240565	Corrections of clause 5.5.9.1

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0340  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240566	Corrections of references to 24.282

Type: CR		For: Agreement
					36.579-1 v16.3.0	  CR-0341  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.9.2	TS 36.579-2

R5-240567	Correction of MO call release of calls using a pre-established session

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0352  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240568	Correction of RRC connection release in several test cases

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0353  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240569	Correction of testcase 6.1.1.17

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0354  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240570	Correction of testcase 6.1.1.5

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0355  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240571	Correction of testcase 6.1.2.14

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0356  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240572	Correction of testcase 6.1.2.2

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0357  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240573	Correction of testcase 6.1.3.1

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0358  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240574	Correction of testcase 6.2.10

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0359  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240575	Correction of testcase 6.2.14

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0360  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240576	Correction of testcase 6.2.15

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0361  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240577	Correction of testcase 6.2.18

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0362  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240578	Correction of testcase 6.2.19

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0363  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240579	Correction of testcase 6.2.22

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0364  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240580	Correction of testcase 6.2.26

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0365  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240581	Correction of testcase 6.2.9

Type: CR		For: Agreement
					36.579-2 v16.3.0	  CR-0366  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.9.3	TS 36.579-3

##### 6.6.9.4	TS 36.579-4

##### 6.6.9.5	TS 36.579-5

##### 6.6.9.6	TS 36.579-6

R5-240582	Correction of testcase 6.1.3.1

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0100  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240583	Correction of testcase 6.7.1

Type: CR		For: Agreement
					36.579-6 v16.3.0	  CR-0101  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.9.7	TS 36.579-7

R5-240584	Corrections of references to 24.282

Type: CR		For: Agreement
					36.579-7 v16.2.0	  CR-0053  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

R5-240647	Editorial correction to the Scope description

Type: CR		For: Agreement
					36.579-7 v16.2.0	  CR-0054  Cat: F (Rel-16)

					Source: FirstNet, NIST

Abstract:

Editorial correction of the Scope description in 36.579-7.

Discussion:

block agreed on Thu.

Decision: 		The document was agreed.

##### 6.6.9.8	Other Specs

##### 6.6.9.9	Discussion Papers / Work Plan / TC lists

### 6.7	Outgoing liaison statements for provisional approval

### 6.8	AOB

## 7	Closing Joint Session

### 7.1	Agenda for closing session

R5-240003	Agenda - closing session

Type: agenda		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

### 7.2	Pointer CRs

### 7.3	Open Issues

#### 7.3.1	RF group docs still requiring WG verdict/confirmation - original A.I. retained

#### 7.3.2	Sig group docs still requiring WG verdict/confirmation - original A.I. retained

#### 7.3.3	Other open issues from joint sessions - original A.I. retained

#### 7.3.4	Other

### 7.4	iWD/PRD Updates

#### 7.4.1	iWD-003: Record of RAN5 owned test cases not ready for RAN5 agreement or verifiable on one UE only

#### 7.4.2	PRD17: Guidance to using Work Item Codes with RAN5 test cases

R5-240925	PRD-17 on Guidance to Work Item Codes (post RAN#103 version)

Type: other		For: Approval
					Source: Bureau Veritas ADT (Rapporteur)

Abstract:

Post-meeting

Decision: 		The document was approved.

#### 7.4.3	PRD20: Status updates E-UTRA CA

R5-240832	PRD20 on E-UTRA CA configuration handling in RAN5 v1.6.0

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

14.3. 5 pm

Decision: 		The document was approved.

R5-240926	PRD20: Completed CA_2A-2A-29A-30A-66A_5DL-1UL without UL CA

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was withdrawn.

R5-240927	PRD20: Completed CA_2A-2A-29A-66A-66A_5DL-1UL without UL CA

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was withdrawn.

R5-240928	PRD20: Completed WP_CA_2A-29A-66A-66A_CA_2A-2A-29A-66A_4DL-1UL_DL only FBs

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was withdrawn.

R5-240929	PRD20: Completed CA_30A-48A_2DL-1UL without UL CA

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was noted.

R5-240930	PRD20: Completed CA_30A-48A 2DL-2UL with UL CA_30A-48A

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was withdrawn.

#### 7.4.4	PRD21: Status Updates and Completion Declaration Statements (CDS) for NR bands / NR band CBW extensions / 5G NR CADC configurations for PC3 / PC1.5 and PC2

R5-240092	PRD21 on NR bands and 5G NR CADC config handling v1.8.0

Type: other		For: Approval
					Source: CMCC

Discussion:

13.3. 4pm

Decision: 		The document was approved.

R5-240304	PRD21 CDS: PC3 SUL_N78A-N81A SUL_N78A-N84A

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240459	PRD21 CDS: NR CA PC3 FR1 CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A)

Type: WI summary		For: Information
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R5-240471	PRD21 CDS: NR band and CBW FR1 n25 and n71

Type: WI summary		For: Information
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R5-240524	PRD21 CDS: NR CA PC3 FR1 CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) fallbacks and BCSs

Type: WI summary		For: Information
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R5-240525	PRD21 CDS: NR band and CBW FR1 n100 and n101 PC1

Type: WI summary		For: Information
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R5-240640	PRD21 CDS PC3 NRCA CA_n1A-n41A, CA_n3A-n28A, CA_n3A-n77(2A), CA_n28A-n77(2A), CA_n3A-n28A-n77A

Type: other		For: Information
					Source: KDDI Corporation

Discussion:

Completion status is not 100% for the corresponding NRCA configurations.

Decision: 		The document was withdrawn.

R5-240641	PRD21 CDS PC3 NRCA CA_n41A-n77A, CA_n1A-n3A-n77A, CA_n1A-n28A-n77A, CA_n1A-n41A-n77A, CA_n3A-n41A-n77A

Type: other		For: Information
					Source: KDDI Corporation

Decision: 		The document was noted.

R5-240642	PRD21 CDS PC2 EN-DC DC_3A_n77A, DC_18A_n77A, DC_28A_n77A

Type: other		For: Information
					Source: KDDI Corporation

Discussion:

Completion status is not 100% for the corresponding EN-DC configurations.

Decision: 		The document was withdrawn.

R5-240675	PRD21 CDS PC3 EN-DC DC_18A_n257G

Type: other		For: Information
					Source: KDDI Corporation

Discussion:

Completion status is not 100% for the corresponding EN-DC configurations.

Decision: 		The document was withdrawn.

R5-240774	PRD21 CDS: NR CA PC3 FR1 CA_n1A-n28A-n78A and CA_n3A-n28A-n78A

Type: WI summary		For: Information
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R5-240834	PRD21 CDS NR CA PC3 FR1 CA_n1A-n3A-n78A

Type: other		For: Information
					Source: ZTE, China Telecom

Decision: 		The document was noted.

R5-240943	PRD21 CDS: NR CA 4 bands PC3 FR1

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was noted.

R5-240945	PRD21 CDS: PC3 NRCA 3T

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was noted.

R5-240946	PRD21 CDS: PC3 NRCA 1F+3T

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was noted.

R5-240947	PRD21 CDS: PC3 NRCA 1F+n48(2A)

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was noted.

R5-240948	PRD21 CDS: PC3 NRCA 2F+n77C

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was noted.

R5-240949	PRD21 CDS: PC2 EN-DC FR1 3 bands and 4 bands

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was withdrawn.

R5-240953	PRD21 CDS: PC2 NRCA 2, 3, 4 bands

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was noted.

R5-241077	PRD21 CDS: PC2 ENDC 2 and 3 bands combos with n78 and n79

Type: other		For: Information
					Source: NTT DOCOMO INC..

Decision: 		The document was noted.

R5-241548	PRD.21 CDS: ENDC PC3 8 EN-DC bands

Type: other		For: Approval
					Source: WE Certification

Decision: 		The document was noted.

R5-241549	PRD.21 CDS: NRCA PC2 CA_n77(2A), CA_n14A-n77A

Type: other		For: Approval
					Source: WE Certification

Decision: 		The document was noted.

R5-241550	PRD.21 CDS: ENDC PC2 13 EN-DC bands

Type: other		For: Approval
					Source: WE Certification

Decision: 		The document was noted.

#### 7.4.5	Other PRD updates

R5-241456	Update of PRD18 for NTN specs and relations between the specs in a group

Type: other		For: Approval
					Source: Bureau Veritas ADT

Discussion:

+36.521-3 with -4

+38.533.

Decision: 		The document was revised to R5-241660.

R5-241660	Update of PRD18 for NTN specs and relations between the specs in a group

Type: other		For: Approval
					Source: Bureau Veritas ADT

(Replaces R5-241456)

Decision: 		The document was approved.

### 7.5	Work Items/ Study Items

#### 7.5.1	Final version of Work Item Proposals

R5-241459	New WID on UE Conformance - IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects

Type: WID new		For: Endorsement
					Source: CMCC, MTK, CAICT

(Replaces R5-240094)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241460	New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation

Type: WID new		For: Endorsement
					Source: MediaTek Inc., China Telecom

(Replaces R5-240146)

Discussion:

added supporting companies.

is endorsed.

Decision: 		The document was agreed.

R5-241461	New WID on UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

Type: WID new		For: Endorsement
					Source: China Telecom

(Replaces R5-240278)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241462	New WID on UE Conformance - Further NR coverage enhancements

Type: WID new		For: Endorsement
					Source: China Telecom

(Replaces R5-240283)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241463	New WID on UE Conformance - Further RF requirements enhancement for NR and EN-DC in frequency range 1

Type: WID new		For: Endorsement
					Source: Huawei, Hisilicon, China Telecom

(Replaces R5-240303)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-240321	Revised WID: UE Conformance Test Aspects - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects

Type: WID revised		For: Endorsement
					Source: MediaTek Inc.

Discussion:

r1

is endorsed.

Decision: 		The document was revised to R5-241666.

R5-241666	Revised WID: UE Conformance Test Aspects - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects

Type: WID revised		For: Endorsement
					Source: MediaTek Inc.

(Replaces R5-240321)

Decision: 		The document was agreed.

R5-241464	New WID on UE Conformance - Network energy savings for NR

Type: WID new		For: Endorsement
					Source: Huawei, HiSilicon

(Replaces R5-240344)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241465	New WID on UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18

Type: WID new		For: Endorsement
					Source: Huawei, HiSilicon, CMCC

(Replaces R5-240345)

Discussion:

added more supporting complanies.

is endorsed.

Decision: 		The document was agreed.

R5-241466	New WID on UE Conformance – Multi-carrier enhancements for NR

Type: WID new		For: Endorsement
					Source: China Telecom, Huawei, HiSilicon

(Replaces R5-240367)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241467	New WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects

Type: WID new		For: Endorsement
					Source: China Unicom, Ericsson, Huawei, Hisilicon, Qualcomm

(Replaces R5-240390)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241468	New WID on UE Conformance - Introduction of 900 MHz LTE band in the US

Type: WID new		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240458)

Discussion:

Ericsson was added.

is endorsed.

Decision: 		The document was agreed.

R5-241469	New WID on UE Conformance - NR support for dedicated spectrum less than 5MHz for FR1

Type: WID new		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-240517)

Decision: 		The document was revised to R5-241659.

R5-241659	New WID on UE Conformance - NR support for dedicated spectrum less than 5MHz for FR1

Type: WID new		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

(Replaces R5-241469)

Discussion:

added Ericsson and NTT DOCOMO.

is endorsed.

Decision: 		The document was agreed.

R5-241470	New WID on UE Conformance - XR (eXtended Reality) enhancements for NR

Type: WID new		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell, CMCC, Huawei, Hisilicon, Qualcomm

(Replaces R5-240594)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241471	New WID on UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2 (FR2)

Type: WID new		For: Endorsement
					Source: Samsung

(Replaces R5-240610)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241472	New WID on UE Conformance – NR MIMO Evolution for Downlink and Uplink

Type: WID new		For: Endorsement
					Source: Samsung, Huawei, Hisilicon

(Replaces R5-240612)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241473	New WID on UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirement for NR Ues

Type: WID new		For: Approval
					Source: Apple (UK) Limited

(Replaces R5-241052)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241474	New WID on UE Conformance - NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

Type: WID new		For: Endorsement
					Source: Apple Benelux B.V., Nokia

(Replaces R5-241419)

Discussion:

is endorsed.

Decision: 		The document was agreed.

#### 7.5.2	Active Work Items/ Study Item: work plans (wp) / status reports (sr) / Work Item Descriptions (wid)

R5-240015	WI Progress and Target Completion Date Review

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-240032	WP UE Conformance - NR QoE management and optimizations for diverse services

Type: Work Plan		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-240033	SR UE Conformance - NR QoE management and optimizations for diverse services

Type: WI status report		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-240036	WP UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS

Type: Work Plan		For: Information
					Source: China Telecom

Decision: 		The document was revised to R5-241669.

R5-241669	WP UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS

Type: Work Plan		For: Information
					Source: China Telecom

(Replaces R5-240036)

Decision: 		The document was noted.

R5-240037	SR UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS

Type: WI status report		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240038	SR UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

Type: WI status report		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240039	WP UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

Type: Work Plan		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240042	Revised WID on UE Conformance - Air-to-ground network for NR

Type: WID revised		For: Endorsement
					Source: CMCC

Discussion:

r1

is endorsed.

Decision: 		The document was revised to R5-241663.

R5-241663	Revised WID on UE Conformance - Air-to-ground network for NR

Type: WID revised		For: Endorsement
					Source: CMCC

(Replaces R5-240042)

Decision: 		The document was agreed.

R5-240043	SR Rel-18 ATG for NR after RAN#102

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240044	WP Rel-18 ATG for NR after RAN#102

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240067	Revised WID on UE Conformance - NB-IoT eMTC NTN

Type: WID revised		For: Endorsement
					Source: CMCC

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-240068	SR Rel-18 NB-IoT/eMTC NTN after RAN#102

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240069	WP Rel-18 NB-IoT/eMTC NTN after RAN#102

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240077	SR Rel-18 PC1.5 n34 n39 n40 after RAN5#102

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240078	WP Rel-18 PC1.5 n34 n39 n40 after RAN5#102

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240086	Rel-15 5GS WP SIG NE-DC after RAN5#102

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240087	SR NR_Rel-16_CA_DC after RAN#102

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240088	WP NR_Rel-16_CA_DC after RAN#102

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-240134	Work plan: UE Conformance Test Aspects - NR Positioning Enhancement

Type: Work Plan		For: Information
					Source: CATT

Decision: 		The document was noted.

R5-240135	SR UE Conformance Test Aspects - NR Positioning Enhancement

Type: WI status report		For: Information
					Source: CATT

Decision: 		The document was noted.

R5-240194	WP UE Conformance - NR and MR-DC measurement gap enhancements

Type: Work Plan		For: Information
					Source: MediaTek Inc.

Decision: 		The document was noted.

R5-240195	SR UE Conformance - NR and MR-DC measurement gap enhancements

Type: WI status report		For: Information
					Source: MediaTek Inc.

Decision: 		The document was noted.

R5-240196	WP UE Conformance - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects

Type: Work Plan		For: Information
					Source: MediaTek Inc.

Decision: 		The document was noted.

R5-240197	SR UE Conformance - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects

Type: WI status report		For: Information
					Source: MediaTek Inc.

Decision: 		The document was noted.

R5-240227	Revised WID on UE Conformance-IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS

Type: WID revised		For: Agreement
					Source: China Telecom

Discussion:

update to June '24.

Decision: 		The document was revised to R5-241661.

R5-241661	Revised WID on UE Conformance-IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS

Type: WID revised		For: Agreement
					Source: China Telecom

(Replaces R5-240227)

Discussion:

is endorsed.

Decision: 		The document was noted.

R5-240245	WP UE Conformance NR Coverage Enhancement RAN5#102

Type: Work Plan		For: Discussion
					Source: China Telecom

Decision: 		The document was noted.

R5-240246	SR UE Conformance NR Coverage Enhancement  RAN5#102

Type: WI status report		For: Discussion
					Source: China Telecom

Decision: 		The document was noted.

R5-240247	SR UE Conformance Further enhancement on NR demodulation performance RAN5#102

Type: WI status report		For: Discussion
					Source: China Telecom

Decision: 		The document was noted.

R5-240248	WP UE Conformance – Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink  RAN5#102

Type: Work Plan		For: Discussion
					Source: China Telecom

Decision: 		The document was noted.

R5-240249	SR UE Conformance – Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink  RAN5#102

Type: WI status report		For: Discussion
					Source: China Telecom

Decision: 		The document was noted.

R5-240291	Revised WID on UE Conformance - New Rel-18 NR licensed bands and extension of existing NR bands

Type: WID revised		For: Endorsement
					Source: Huawei, Hisilicon

Discussion:

the RAN5 status column shall be removd.

is endorsed.

Decision: 		The document was revised to R5-241664.

R5-241664	Revised WID on UE Conformance - New Rel-18 NR licensed bands and extension of existing NR bands

Type: WID revised		For: Endorsement
					Source: Huawei, Hisilicon

(Replaces R5-240291)

Decision: 		The document was agreed.

R5-240292	WP of New Rel-18 NR licensed bands and extension of existing NR bands

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240293	SR of New Rel-18 NR licensed bands and extension of existing NR bands

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240294	Revised WID on UE Conformance - Rel-18 NR CA and DC; and NR and LTE DC Configurations

Type: WID revised		For: Endorsement
					Source: Huawei, Hisilicon

Discussion:

r1

is endorsed.

Decision: 		The document was revised to R5-241665.

R5-241665	Revised WID on UE Conformance - Rel-18 NR CA and DC; and NR and LTE DC Configurations

Type: WID revised		For: Endorsement
					Source: Huawei, Hisilicon

(Replaces R5-240294)

Decision: 		The document was agreed.

R5-240295	WP of Rel-18 NR CA and DC; and NR and LTE DC Configurations

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240296	SR of Rel-18 NR CA and DC; and NR and LTE DC Configurations

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240297	WP of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240298	SR of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240299	WP of Rel-17 NR CA and DC; and NR and LTE DC Configurations

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240300	SR of Rel-17 NR CA and DC; and NR and LTE DC Configurations

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240301	WP of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240302	SR of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-240340	WP UE Conformance - Further Multi-RAT Dual-Connectivity enhancement

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-240341	SR UE Conformance - Further Multi-RAT Dual-Connectivity enhancement

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-240342	WP UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-240343	SR UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-240361	WP UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#102

Type: Work Plan		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240362	SR UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#102

Type: WI status report		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240363	WP UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#102

Type: Work Plan		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240364	SR UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#102

Type: WI status report		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240365	WP UE Conformance – Rel-17 LTE CA Configurations RAN5#102

Type: Work Plan		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240366	SR UE Conformance – Rel-17 LTE CA Configurations RAN5#102

Type: WI status report		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-240391	WP on UE Conformance – Support of reduced capability NR devices

Type: Work Plan		For: Information
					Source: China Unicom

Decision: 		The document was noted.

R5-240392	SR on UE Conformance – Support of reduced capability NR devices

Type: WI status report		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-240393	Revised WID on UE Conformance – Support of reduced capability NR devices

Type: WID revised		For: Endorsement
					Source: China Unicom,Hisilicon, Ericsson, Huawei, Qualcomm

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-240394	WP on UE Conformance - Additional NR bands for UL-MIMO in Rel-18

Type: Work Plan		For: Information
					Source: China Unicom

Decision: 		The document was noted.

R5-240395	SR on UE Conformance - Additional NR bands for UL-MIMO in Rel-18

Type: WI status report		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-240396	 WP on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band

Type: Work Plan		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-240397	SR on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band

Type: WI status report		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-240411	SR of UE Conformance - Signal level Enhanced Network Selection

Type: WI status report		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-240412	WP of UE Conformance - Signal level Enhanced Network Selection

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-240450	WP UE Conformance – Further enhancements of NR RF requirements for FR2

Type: Work Plan		For: Information
					Source: Nokia, Nokia Shanghai Bell, Apple

Decision: 		The document was noted.

R5-240451	SR UE Conformance – Further enhancements of NR RF requirements for FR2

Type: WI status report		For: Information
					Source: Nokia, Nokia Shanghai Bell, Apple

Decision: 		The document was noted.

R5-240452	WP UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18

Type: Work Plan		For: Information
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R5-240453	SR UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18

Type: WI status report		For: Information
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

R5-240485	Work plan: UE Conformance - NR sidelink enhancement

Type: Work Plan		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-240486	SR UE Conformance - NR sidelink enhancement

Type: WI status report		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-240487	Work plan: UE Conformance - NR Sidelink Relay

Type: Work Plan		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-240488	SR UE Conformance - NR Sidelink Relay

Type: WI status report		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-240501	WP - UE Conformance - Further enhancements on MIMO for NR

Type: Work Plan		For: Approval
					Source: Samsung

Decision: 		The document was noted.

R5-240502	SR - UE Conformance - Further enhancements on MIMO for NR

Type: WI status report		For: Approval
					Source: Samsung

Decision: 		The document was noted.

R5-240503	WP - UE Conformance - NR support for high speed train scenario in frequency range 2

Type: Work Plan		For: Approval
					Source: Samsung

Decision: 		The document was noted.

R5-240504	SR - UE Conformance - NR support for high speed train scenario in frequency range 2

Type: WI status report		For: Approval
					Source: Samsung

Decision: 		The document was noted.

R5-240607	WP UE Conformance - User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects)

Type: Work Plan		For: Information
					Source: Vodafone

Decision: 		The document was noted.

R5-240608	Status Report for UE Conformance – User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects)

Type: WI status report		For: Information
					Source: Vodafone

Decision: 		The document was noted.

R5-240665	SR UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems

Type: WI status report		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-240666	SR UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state

Type: WI status report		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-240667	WP UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems

Type: Work Plan		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-240668	WP UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state

Type: Work Plan		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-240768	SR of Rel-16 V2X WI after RAN5 102

Type: WI status report		For: Information
					Source: Huawei,HiSilicon

Decision: 		The document was noted.

R5-240769	WP of Rel-16 V2X WI after RAN5 102

Type: Work Plan		For: Information
					Source: Huawei,HiSilicon

Decision: 		The document was noted.

R5-240781	Revised WID: UE Conformance - NR sidelink enhancement

Type: WID revised		For: Endorsement
					Source: CATT

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-240782	Revised WID: UE Conformance - NR Sidelink Relay

Type: WID revised		For: Endorsement
					Source: CATT

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-240784	WP UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2)

Type: Work Plan		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-240785	SR UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2)

Type: WI status report		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-240869	Revised WID on UE Conformance Test Aspects - Rel-17 IMS Data Channel

Type: WID revised		For: Endorsement
					Source: Huawei

Discussion:

r1

->Sept. '24

Decision: 		The document was revised to R5-241662.

R5-241662	Revised WID on UE Conformance Test Aspects - Rel-17 IMS Data Channel

Type: WID revised		For: Endorsement
					Source: Huawei

(Replaces R5-240869)

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-240874	SR of UE Conformance - IMS Data Channel test after RAN5 102

Type: WI status report		For: Information
					Source: Huawei

Decision: 		The document was noted.

R5-240875	WP of UE Conformance - IMS Data Channel test after RAN5 102

Type: Work Plan		For: Information
					Source: Huawei

Decision: 		The document was noted.

R5-240912	SR Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData)

Type: WI status report		For: Information
					Source: NIST

Decision: 		The document was noted.

R5-240913	WP Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData)

Type: Work Plan		For: Information
					Source: NIST

Decision: 		The document was noted.

R5-241031	WP - Rel-17 RF requirements enhancement for NR frequency range 1 (FR1)

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-241032	SR - Rel-17 RF requirements enhancement for NR frequency range 1 (FR1)

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-241076	Revised WID UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2)

Type: WID revised		For: Endorsement
					Source: Ericsson

Discussion:

is endorsed.

Decision: 		The document was agreed.

R5-241132	WP - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum

Type: Work Plan		For: Approval
					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was noted.

R5-241133	SR - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum

Type: WI status report		For: Approval
					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was noted.

R5-241134	WP UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN)

Type: Work Plan		For: Approval
					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was noted.

R5-241135	SR UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN)

Type: WI status report		For: Approval
					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was noted.

R5-241136	WP UE Conformance Test Aspects - Further enhancement on NR demodulation performance

Type: Work Plan		For: Approval
					Source: QUALCOMM Europe Inc. - Italy

Decision: 		The document was noted.

R5-241340	SR - UE Conformance Aspects - NR RRM enhancements

Type: WI status report		For: Approval
					Source: Apple

Decision: 		The document was noted.

R5-241341	WP - UE Conformance Aspects - NR RRM enhancements

Type: Work Plan		For: Approval
					Source: Apple

Decision: 		The document was noted.

R5-241449	WP WI UE Conformance – UE power saving enhancements for NR

Type: Work Plan		For: Information
					Source: MediaTek

Decision: 		The document was noted.

R5-241450	SR WI UE Conformance – UE power saving enhancements for NR

Type: WI status report		For: Information
					Source: MediaTek

Decision: 		The document was noted.

R5-241576	Revised WID: UE Conformance - User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects)

Type: WID revised		For: Approval
					Source: Vodafone

Discussion:

is endorsed.

Decision: 		The document was noted.

#### 7.5.3	Work Plan updates of recently closed work items

R5-240669	SIG WP for TEI16 - Redirection with MPS Indication

Type: Work Plan		For: Information
					Source: Qualcomm CDMA Technologies

Decision: 		The document was not treated.

R5-240670	Rel-15 5GS maintenance SIG WP (NR-SA and ENDC)

Type: Work Plan		For: Information
					Source: Qualcomm CDMA Technologies

Decision: 		The document was not treated.

R5-241033	Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-1)

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-241034	Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-2)

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-241046	WP Rel-15 NR TX and RX Test Cases – Part 3: Range 1 and Range 2 Interworking operation with other radios (TS 38.521-3)

Type: Work Plan		For: Information
					Source: Qualcomm Technologies Ireland

Decision: 		The document was revised to R5-241668.

R5-241668	WP Rel-15 NR TX and RX Test Cases – Part 3: Range 1 and Range 2 Interworking operation with other radios (TS 38.521-3)

Type: Work Plan		For: Information
					Source: Qualcomm Technologies Ireland

(Replaces R5-241046)

Decision: 		The document was noted.

R5-241251	Rel-15 5GS WI RRM work plan update

Type: Work Plan		For: Information
					Source: ROHDE & SCHWARZ

Decision: 		The document was noted.

### 7.6	Docs still needing agreement/endorsement/approval (e.g. Outgoing LS / Reports / New Specs / Info for certification bodies etc.)

R5-240020	Reply LS on Prohibition of GEA1 & GEA2 Support in all releases

Type: LS out		For: Approval
					to TSG SA, GCF SG, cc TSG WG SA3, TSG RAN, PTCRB Plenary
					Source: TSG WG RAN5

Abstract:

RAN5 would like to thank TSG-SA for the liaison statement notifying updates made to the latest version of all releases of TS 43.020 and TS 03.20 to prohibit the implementation of GEA1 and GEA2 in Mobile Stations of all releases. RAN5 also noted SA request to facilitate verification of non-support of GEA1 and GEA2 through RAN5 test specifications.

RAN5 is pleased to inform that Change Requests (CRs) proposing removal of testing of GEA1 and GEA2 from all RAN5 test specifications have been agreed by RAN5#102 Meeting. RAN5 had already defined the Test Case (TC) 44.2.5.2.5 in TS 51.010-1 to verify non-support of GEA1. A CR proposing to update TC 44.2.5.2.5 to also verify non-support of GEA2 has been agreed by RAN5#102 Meeting.

RAN5 agreed CRs shall be submitted for approval to TSG-RAN#103 Plenary meeting scheduled March 17 – 21 2024 at Maastricht. RAN5 expects TC 44.2.5.2.5 verifying non-support of GEA1 and GEA2 to be included in the next version of TS 51.010-1 scheduled to be published in March 2024 following TSG-RAN#103 meeting.

2. Actions: To GCF Steering Group: 3GPP RAN5 asks GCF Steering Group to take the above information into account.

Discussion:

+PTCRB plenary

Decision: 		The document was approved.

R5-241458	MCC TF160 Status Report

Type: report		For: Approval
					Source: MCC TF160

(Replaces R5-240526)

Decision: 		The document was approved.

R5-240872	TS 38.523-1 Tracker status after RAN5-102

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R5-240873	TS 36.523-1 Tracker status after RAN5-102

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R5-240923	TS 36.579-8 v0.1.0

Type: draft TS		For: (not specified)
					36.579-8 v0.1.0
					Source: NIST

Abstract:

post meeting doc

Decision: 		The document was withdrawn.

R5-240924	TS 36.579-9 v0.1.0

Type: draft TS		For: (not specified)
					36.579-9 v0.1.0
					Source: NIST

Abstract:

post meeting doc

Decision: 		The document was withdrawn.

R5-240931	RAN5#102 summary of changes to RAN5 test cases with potential impact on GCF and PTCRB

Type: report		For: Information
					Source: Bureau Veritas ADT

Abstract:

Post-meeting

Decision: 		The document was not treated.

R5-241455	Candidate Spec for Release upgrade after RAN5#102

Type: report		For: Information
					Source: Bureau Veritas ADT

Discussion:

r2

Decision: 		The document was revised to R5-241667.

R5-241700	Conclusion of RAN5 NR MIMO OTA Working Item

Type: LS out		For: Approval
					38.551 v..
					to CTIA Certification, CTIA MIMO OTA Sub-Working Group, cc GCF CAG, CCSA TC9 WG1
					Source: TSG WG RAN5

Abstract:

3GPP RAN5 would like to inform that the Work Item on UE Conformance – Enhancements on Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs Rel 17 was concluded in the 3GPP TSG RAN Meeting #102 (December 2023).

While there are differences among 3GPP and CTIA on FR1 MIMO OTA channel model and FoM definitions, both share the same test methodology on Multi-Probe Anechoic Chamber. Thus, Measurement Uncertainty definitions from 3GPP TS 38.551 might be relevant for CTIA to align with 3GPP RAN5 definitions.

2. Actions: To CTIA Certification and MIMO OTA Sub-Working Group: 3GPP TSG RAN5 respectfully asks CTIA Certification and CTIA MIMO OTA Sub-Working Group to take into consideration the completed Release 17 UE MIMO OTA Conformance test specifications defined in TS 38.551 [1].

Decision: 		The document was approved.

R5-241667	Candidate Spec for Release upgrade after RAN5#102

Type: report		For: Information
					Source: Bureau Veritas ADT

(Replaces R5-241455)

Decision: 		The document was noted.

### 7.7	Confirmation of Future RAN5 Matters

R5-241658	Meeting schedule for 2024-25

Type: other		For: Information
					Source: WG Chairman

(Replaces R5-240014)

Decision: 		The document was noted.

R5-240016	Review deadlines for next quarter

Type: other		For: Information
					Source: WG Chairman

Discussion:

RAN5 Chair: there is a core AI agenda item. An agenda item for AI testing and interoperability could be added for next time.

Decision: 		The document was noted.

### 7.8	AOB

## Annex A: Contribution documents and status

### A1: List of TDocs

2035 documents were submitted at RAN5#102. Plus 648 informal revisions (not shown here)

| Document | Title | Source | Decision | Replaces | Replaced by |
| --- | --- | --- | --- | --- | --- |
| R5-240001 | Agenda - opening session | WG Chairman | approved |  |  |
| R5-240002 | Agenda - midweek session | WG Chairman | approved |  |  |
| R5-240003 | Agenda - closing session | WG Chairman | noted |  |  |
| R5-240004 | RAN5#102 Session Programme | WG Chairman | noted |  |  |
| R5-240005 | RAN5 Leadership Team | WG Chairman | noted |  |  |
| R5-240006 | RAN5#101 WG Minutes | ETSI Secretariat | approved |  |  |
| R5-240007 | RAN5#101 WG Action Points | ETSI Secretariat | noted |  |  |
| R5-240008 | Latest RAN Plenary notes | WG Chairman | noted |  |  |
| R5-240009 | Latest RAN Plenary draft Report | WG Chairman | noted |  |  |
| R5-240010 | Post Plenary Active Work Item update | ETSI Secretariat | noted |  |  |
| R5-240011 | RAN5 SR to RP#102 | WG Chairman | noted |  |  |
| R5-240012 | TF160 SR to RP#102 | WG Chairman | noted |  |  |
| R5-240013 | RAN5#102 LS Template | WG Chairman | noted |  |  |
| R5-240014 | Meeting schedule for 2024-25 | WG Chairman | revised |  | R5-241658 |
| R5-240015 | WI Progress and Target Completion Date Review | WG Chairman | noted |  |  |
| R5-240016 | Review deadlines for next quarter | WG Chairman | noted |  |  |
| R5-240017 | Reply LS on frequencyInfo for NR SL RSRP measurements | TSG WG RAN2 | noted |  |  |
| R5-240018 | LS on Prohibition of GEA1 & GEA2 Support in all releases | TSG SA | noted |  |  |
| R5-240019 | draft RAN5 meeting report | ETSI Secretariat | noted |  |  |
| R5-240020 | Reply LS on Prohibition of GEA1 & GEA2 Support in all releases | TSG WG RAN5 | approved |  |  |
| R5-240021 | Correction to LTE RACS test case 8.5.5.1 | Keysight Technologies UK, Qualcomm | revised |  | R5-241626 |
| R5-240022 | Correction to NR5GC test cases 11.3.6 and 11.3.6a | Keysight Technologies UK Ltd | revised |  | R5-241527 |
| R5-240023 | Editorial corrections for NB-IoT NTN TA cases and RLM cases | MediaTek Beijing Inc. | revised |  | R5-241794 |
| R5-240024 | Update NB-IoT NTN UL timing accuracy cases | MediaTek Beijing Inc. | agreed |  |  |
| R5-240025 | Update NB-IoT NTN UL timing accuracy cases TT | MediaTek Beijing Inc. | agreed |  |  |
| R5-240026 | Editorial corrections for 6.7.3.1 and 6.7.3.2.2 | MediaTek Beijing Inc. | agreed |  |  |
| R5-240027 | Addition of PICS for Rel-17 ATSSS devices | China Telecom | agreed |  |  |
| R5-240028 | Editorial correction for test procedure sequence in 11.1.10 | NTT DOCOMO, INC. | revised |  | R5-241591 |
| R5-240029 | Update NB-IoT NTN UL timing accuracy cases TT in Annex | MediaTek Beijing Inc. | agreed |  |  |
| R5-240030 | Update GNSS margin value for NB-IoT NTN timing accuracy test cases | MediaTek Beijing Inc. | withdrawn |  |  |
| R5-240031 | Editorial correction to the wrong table number in 36.521-4 annex C | MediaTek (Hefei) Inc. | revised |  | R5-241797 |
| R5-240032 | WP UE Conformance - NR QoE management and optimizations for diverse services | Ericsson | noted |  |  |
| R5-240033 | SR UE Conformance - NR QoE management and optimizations for diverse services | Ericsson | noted |  |  |
| R5-240034 | Correction to MUSIM test case 8.1.5.10.3 | CATT, TDIA | revised |  | R5-241517 |
| R5-240035 | Add a chapter title for ReEST (CH13.3.1) | MediaTek Inc. | withdrawn |  |  |
| R5-240036 | WP UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS | China Telecom | revised |  | R5-241669 |
| R5-240037 | SR UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS | China Telecom | noted |  |  |
| R5-240038 | SR UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | China Telecom | noted |  |  |
| R5-240039 | WP UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | China Telecom | noted |  |  |
| R5-240040 | Add new ING_5GS test case 11.1.11 | China Telecom | revised |  | R5-241592 |
| R5-240041 | Add applicability for ING_5GS test case 11.1.11 | China Telecom | withdrawn |  |  |
| R5-240042 | Revised WID on UE Conformance - Air-to-ground network for NR | CMCC | revised |  | R5-241663 |
| R5-240043 | SR Rel-18 ATG for NR after RAN#102 | CMCC | noted |  |  |
| R5-240044 | WP Rel-18 ATG for NR after RAN#102 | CMCC | noted |  |  |
| R5-240045 | Introduction of Common test environments for ATG | CMCC | withdrawn |  |  |
| R5-240046 | Introduction of RF Test Environment for ATG | CMCC | revised |  | R5-241919 |
| R5-240047 | Introduction of RRM Test Environment for ATG | CMCC | revised |  | R5-241920 |
| R5-240048 | Introduction of common ICS for ATG | CMCC | revised |  | R5-241715 |
| R5-240049 | Introduction of common parts for ATG UE RF test cases | CMCC | agreed |  | - |
| R5-240050 | Introduction of General description for ATG UE Tx TCs | CMCC | revised |  | R5-242030 |
| R5-240051 | Introduction of MOP TC for ATG UE | CMCC | revised |  | R5-241751 |
| R5-240052 | Introduction of Configured transmitted power TC for ATG UE | CMCC | revised |  | R5-241752 |
| R5-240053 | Introduction of General description of Occupied bandwidth for ATG UE | CMCC | agreed |  |  |
| R5-240054 | Introduction of Occupied bandwidth TC for ATG UE | CMCC | revised |  | R5-241753 |
| R5-240055 | Introduction of General description of Out of band emission for ATG UE | CMCC | agreed |  |  |
| R5-240056 | Introduction of General description of Spurious emissions for ATG UE | CMCC | agreed |  | - |
| R5-240057 | Introduction of General Spurious emissions TC for ATG UE | CMCC | revised |  | R5-241754 |
| R5-240058 | Introduction of General description of Rx TCs for ATG UE | CMCC | agreed |  |  |
| R5-240059 | Introduction of Diversity characteristics description for ATG UE | CMCC | agreed |  |  |
| R5-240060 | Introduction of General description of Reference sensitivity for ATG UE | CMCC | agreed |  |  |
| R5-240061 | Introduction of General description of Blocking characteristics for ATG UE | CMCC | agreed |  |  |
| R5-240062 | Introduction of Spurious response TC for ATG UE | CMCC | revised |  | R5-241757 |
| R5-240063 | Introduction of General description of Intermodulation characteristics for ATG UE | CMCC | agreed |  |  |
| R5-240064 | Introduction of Wide band intermodulation TC for ATG UE | CMCC | revised |  | R5-241758 |
| R5-240065 | Introduction of Spurious emissions TC for ATG UE | CMCC | revised |  | R5-241759 |
| R5-240066 | Addition of TC applicability statements for ATG UE | CMCC, CAICT | revised |  | R5-241842 |
| R5-240067 | Revised WID on UE Conformance - NB-IoT eMTC NTN | CMCC | agreed |  |  |
| R5-240068 | SR Rel-18 NB-IoT/eMTC NTN after RAN#102 | CMCC | noted |  |  |
| R5-240069 | WP Rel-18 NB-IoT/eMTC NTN after RAN#102 | CMCC | noted |  |  |
| R5-240070 | Discussion on the closure of R18 IoT NTN UEConTest WI | CMCC | revised |  | R5-241942 |
| R5-240071 | Update of Annex F Measurement Uncertainties in TS 36.521-1 | CMCC, Keysight Technologies UK Ltd | agreed |  |  |
| R5-240072 | Update of Annex F Measurement Uncertainties in TS 36.521-4 | CMCC, Keysight Technologies UK Ltd | revised |  | R5-241798 |
| R5-240073 | Update to R18 IoT NTN test cases applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | revised |  | R5-241918 |
| R5-240074 | Clear-up CR for Editor notes of applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | revised |  | R5-241804 |
| R5-240075 | Update to common requirements of test equipment for IoT-NTN | CMCC | agreed |  |  |
| R5-240076 | Update of grouping of test cases defined in TS 36.521-4 | CMCC, MediaTek | revised |  | R5-241803 |
| R5-240077 | SR Rel-18 PC1.5 n34 n39 n40 after RAN5#102 | CMCC | noted |  |  |
| R5-240078 | WP Rel-18 PC1.5 n34 n39 n40 after RAN5#102 | CMCC | noted |  |  |
| R5-240079 | Introduction of common ICS for PC1.5 n39 | CMCC | revised |  | R5-242023 |
| R5-240080 | Addition of PC1.5 n39 MOP | CMCC | agreed |  |  |
| R5-240081 | Addition of PC2 n40 MOP | CMCC | agreed |  |  |
| R5-240082 | Addition of PC1.5 n39 MPR | CMCC | agreed |  |  |
| R5-240083 | Addition of PC1.5 n39 Configured tx power requirements | CMCC | agreed |  |  |
| R5-240084 | Addition of PC1.5 n39 Modified MPR behavior | CMCC | agreed |  |  |
| R5-240085 | Addition of PC1.5 n39 AMPR | CMCC | withdrawn |  |  |
| R5-240086 | Rel-15 5GS WP SIG NE-DC after RAN5#102 | CMCC | noted |  |  |
| R5-240087 | SR NR_Rel-16_CA_DC after RAN#102 | CMCC | noted |  |  |
| R5-240088 | WP NR_Rel-16_CA_DC after RAN#102 | CMCC | noted |  |  |
| R5-240089 | Update to R16 NR CADC configuration test cases applicability | CMCC, Sporton | revised |  | R5-241843 |
| R5-240090 | Update to R17 NR CADC configuration test cases applicability | CMCC | withdrawn |  |  |
| R5-240091 | Update to R18 NR CADC configuration test cases applicability | CMCC | withdrawn |  |  |
| R5-240092 | PRD21 on NR bands and 5G NR CADC config handling v1.8.0 | CMCC | approved |  |  |
| R5-240093 | Update NR band and CADC configs status in ICS Annex B | CMCC | agreed |  |  |
| R5-240094 | New WID on UE Conformance - IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects | CMCC, MTK, CAICT | revised |  | R5-241459 |
| R5-240095 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT | revised |  | R5-241635 |
| R5-240096 | Discussion on handling of MDT IDC problem | CMCC | revised |  | R5-241634 |
| R5-240097 | Editorial correction to the wrong citation number | MediaTek (Hefei) Inc. | revised |  | R5-241799 |
| R5-240098 | Editorial alignment for the test applicability | MediaTek (Hefei) Inc. | revised |  | R5-241800 |
| R5-240099 | Update of reference measurement channels in Annex A.3.12 | MediaTek (Hefei) Inc. | revised |  | R5-241801 |
| R5-240100 | Add new r17 ATSSS test case 11.9.1 | China Telecom | revised |  | R5-241617 |
| R5-240101 | Update TC 15.2.8 with TT analysis results | CATT | revised |  | R5-241897 |
| R5-240102 | Update TC 15.2.9 with TT analysis results | CATT | revised |  | R5-241898 |
| R5-240103 | Update TC 15.2.10 with TT analysis results | CATT | agreed |  |  |
| R5-240104 | Update TC 16.2.7 with TT analysis results | CATT | agreed |  |  |
| R5-240105 | Update TC 16.2.8 with TT analysis results | CATT | agreed |  |  |
| R5-240106 | Update TC 16.3.4 with TT analysis results | CATT | revised |  | R5-241899 |
| R5-240107 | Addition of NR PRS-based measurement requirements in annex C to include TT analysis results | CATT | agreed |  |  |
| R5-240108 | Addition of new RRC_INACTIVE NR RSTD reporting delay test case 14.4.3 | CATT | agreed |  |  |
| R5-240109 | Addition of new NR RSTD reporting delay test case 14.4.4 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-240110 | Addition of new NR RSTD accuracy test case 14.5.3 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-240111 | Addition of new NR RSTD accuracy test case 14.5.4 in RRC_INACTIVE state | CATT | revised |  | R5-241911 |
| R5-240112 | Addition of new NR PRS-RSRP reporting delay test case 16.4.3 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-240113 | Addition of new NR PRS-RSRP reporting delay test case 16.4.4 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-240114 | Addition of new NR PRS-RSRP accuracy test case 16.5.3 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-240115 | Addition of new NR PRS-RSRP accuracy test case 16.5.4 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-240116 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.1 | CATT | agreed |  |  |
| R5-240117 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.2 | CATT | agreed |  |  |
| R5-240118 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.3 | CATT | agreed |  |  |
| R5-240119 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.4 | CATT | agreed |  |  |
| R5-240120 | Addition of measurement period requirements in RRC_INACTIVE state for NR positioning methods | CATT | agreed |  |  |
| R5-240121 | Correction to PRS-RSRPP positioning test cases | CATT, Spirent | revised |  | R5-241912 |
| R5-240122 | Correction to PRS-RSRP positioning test cases | CATT, Spirent | revised |  | R5-241913 |
| R5-240123 | Correction to NR RSTD positioning test cases | CATT, Spirent | revised |  | R5-241914 |
| R5-240124 | Correction to NR UE Rx-Tx time difference positioning test cases | CATT, Spirent | revised |  | R5-241915 |
| R5-240125 | Addition of test applicabilities for Release-17 PRS-RSRP, PRS-RSRPP and RSTD test cases | CATT | agreed |  |  |
| R5-240126 | Introduction of BDS B2a and B3I signal default test conditions in TS 37.571-5 | CATT, CAICT | revised |  | R5-241577 |
| R5-240127 | Introduction of BDS B2a and B3I performance default test conditions in TS 37.571-5 | CATT, CAICT | revised |  | R5-241916 |
| R5-240128 | Corrected CellIdentity for DL-TDOA measurement period test | CATT, Spirent | revised |  | R5-241917 |
| R5-240129 | Addition of TT analysis for positioning test case 15.2.8 | CATT | agreed |  |  |
| R5-240130 | Addition of TT analysis for positioning test case 15.2.9 and 15.2.10 | CATT | revised |  | R5-241900 |
| R5-240131 | Addition of TT analysis for positioning test case 16.2.7 | CATT | agreed |  |  |
| R5-240132 | Addition of TT analysis for positioning test case 16.2.8 | CATT | agreed |  |  |
| R5-240133 | Addition of TT analysis for positioning test case 16.3.4 | CATT | agreed |  |  |
| R5-240134 | Work plan: UE Conformance Test Aspects - NR Positioning Enhancement | CATT | noted |  |  |
| R5-240135 | SR UE Conformance Test Aspects - NR Positioning Enhancement | CATT | noted |  |  |
| R5-240136 | Editorial correction to 6.4B.1 Frequency Error for UE category NB1 and NB2 TC | MediaTek Beijing Inc. | withdrawn |  |  |
| R5-240137 | Editorial correction to Additional spurious emissions TCS | MediaTek Beijing Inc. | revised |  | R5-242020 |
| R5-240138 | Update test configuration table for NS_13 | MediaTek Beijing Inc. | revised |  | R5-241727 |
| R5-240139 | Addition of new test case 8.1.2.1.7 for UPIP | ZTE Corporation | revised |  | R5-241513 |
| R5-240140 | Addition of new test case 8.2.2.4.4 for UPIP | ZTE Corporation | revised |  | R5-241551 |
| R5-240141 | Addition of applicability for new UPIP TC 8.1.2.1.7 and 8.2.2.4.4 | ZTE Corporation | revised |  | R5-241575 |
| R5-240142 | New WID on UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR | China Telecom | withdrawn |  |  |
| R5-240143 | New additional information for UPIP with 5GC | ZTE Corporation | withdrawn |  |  |
| R5-240144 | Update content of Statistical testing of Performance Requirements with probability of misdetection in Annex G.4 | MediaTek (Hefei) Inc. | agreed |  |  |
| R5-240145 | Update TT value to NTN demod cases | MediaTek (Hefei) Inc. | agreed |  |  |
| R5-240146 | New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation | MediaTek Inc., China Telecom | revised |  | R5-241460 |
| R5-240147 | Update of MG enhancements TC 6.6.18.3 | MediaTek Inc. | withdrawn |  |  |
| R5-240148 | Update of MG enhancements TC 6.6.18.4 | MediaTek Inc. | withdrawn |  |  |
| R5-240149 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | withdrawn |  |  |
| R5-240150 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | withdrawn |  |  |
| R5-240151 | Correction to RedCap MAC TC 7.1.1.8.4 | MediaTek Inc. | agreed |  |  |
| R5-240152 | Correction of case title for TC 7.1.1.1.19 | MediaTek Inc. | agreed |  |  |
| R5-240153 | Correction to RRCReestablishment message | MediaTek Inc. | agreed |  |  |
| R5-240154 | Correction to PEIPS in REGISTRATION ACCEPT message | MediaTek Inc. | agreed |  |  |
| R5-240155 | Correction to TC 9.1.14.1 | MediaTek Inc. | agreed |  |  |
| R5-240156 | Correction to TC 11.4.1a | MediaTek Inc. | revised |  | R5-241653 |
| R5-240157 | Correction to NGSO abbreviation | MediaTek Inc. | agreed |  |  |
| R5-240158 | Update ephemeris information for NGSO signalling test environment | MediaTek Inc. | revised |  | R5-241556 |
| R5-240159 | Correction to NB-IoT NTN TC 22.4.13a | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm | agreed |  |  |
| R5-240160 | Update of applicable legacy NB-IoT cases | MediaTek Inc., ROHDE & SCHWARZ | withdrawn |  |  |
| R5-240161 | Update of test cases applicability for NB-IoT NTN only UE | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm Incorporated | revised |  | R5-241562 |
| R5-240162 | Discussion on NGSO test model | MediaTek Inc. | noted |  |  |
| R5-240163 | Correction to Test procedure 4.9.27 | MediaTek Inc. | agreed |  |  |
| R5-240164 | Correction to Test procedure 4.9.38 | MediaTek Inc. | agreed |  |  |
| R5-240165 | Correction to DLInformationTransfer message | MediaTek Inc. | agreed |  |  |
| R5-240166 | Correction to UEInformationRequest message | MediaTek Inc. | agreed |  |  |
| R5-240167 | Editorial update of CounterCheck message | MediaTek Inc. | agreed |  |  |
| R5-240168 | Correction to IE RadioLinkMonitoringConfig | MediaTek Inc. | agreed |  |  |
| R5-240169 | Correction to IE NRDC-Parameters | MediaTek Inc. | revised |  | R5-241480 |
| R5-240170 | Correction to IE UE-MRDC-Capability | MediaTek Inc. | revised |  | R5-241481 |
| R5-240171 | Correction to IE UE-NR-Capability | MediaTek Inc. | revised |  | R5-241482 |
| R5-240172 | Correction to RRCReconfiguration configuration | MediaTek Inc. | agreed |  |  |
| R5-240173 | Correction to HPUE PICS Mnemonic | MediaTek Inc. | agreed |  |  |
| R5-240174 | Correction to NR MAC TC 7.1.1.1.1 | MediaTek Inc. | agreed |  |  |
| R5-240175 | Correction to 2-step RACH MAC TC 7.1.1.1.8 and 7.1.1.1.10 | MediaTek Inc. | agreed |  |  |
| R5-240176 | Correction to NR MAC TC 7.1.1.5.x | MediaTek Inc. | revised |  | R5-241501 |
| R5-240177 | Correction to NR MAC TC 7.1.1.6.1 | MediaTek Inc. | revised |  | R5-241502 |
| R5-240178 | Correction to NR MAC TC 7.1.1.6.2 and 7.1.1.6.3 | MediaTek Inc. | revised |  | R5-241503 |
| R5-240179 | Correction to NR MAC TC 7.1.1.6.4 and 7.1.1.6.5 | MediaTek Inc. | revised |  | R5-241504 |
| R5-240180 | Correction to NR MAC TC 7.1.1.8.1 | MediaTek Inc. | revised |  | R5-241505 |
| R5-240181 | Correction to NR HO TC 8.1.4.2.1.1 | MediaTek Inc. | agreed |  |  |
| R5-240182 | Correction to NR HO TC 8.1.4.2.1.2 | MediaTek Inc. | revised |  | R5-241514 |
| R5-240183 | Correction to NR RRC TC 8.1.5.1.1 | MediaTek Inc. | revised |  | R5-241518 |
| R5-240184 | Correction to NR RRC TC 8.1.5.8.1 | MediaTek Inc. | agreed |  |  |
| R5-240185 | Correction to NR RRC TC 8.2.1.1.1 | MediaTek Inc. | revised |  | R5-241519 |
| R5-240186 | Correction to NR RRC TC 8.2.1.1.2 | MediaTek Inc. | revised |  | R5-241520 |
| R5-240187 | Correction to eNS NSAC TC 9.1.12.3 | MediaTek Inc. | agreed |  |  |
| R5-240188 | Correction to MBS TC 14.1.1.1 | MediaTek Inc. | agreed |  |  |
| R5-240189 | Correction to MBS TC 14.1.1.2 | MediaTek Inc. | agreed |  |  |
| R5-240190 | Correction to MBS TC 14.1.1.3 | MediaTek Inc. | revised |  | R5-241532 |
| R5-240191 | Correction to MBS TC 14.1.1.4.1 | MediaTek Inc. | agreed |  |  |
| R5-240192 | Correction to MBS TC 14.1.2.1 | MediaTek Inc. | revised |  | R5-241533 |
| R5-240193 | Correction to MBS TC 14.2.4.2.1 | MediaTek Inc. | agreed |  |  |
| R5-240194 | WP UE Conformance - NR and MR-DC measurement gap enhancements | MediaTek Inc. | noted |  |  |
| R5-240195 | SR UE Conformance - NR and MR-DC measurement gap enhancements | MediaTek Inc. | noted |  |  |
| R5-240196 | WP UE Conformance - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | MediaTek Inc. | noted |  |  |
| R5-240197 | SR UE Conformance - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | MediaTek Inc. | noted |  |  |
| R5-240198 | Addition and correction to the NTN related abbreviations in 36.521-4 | MediaTek (Hefei) Inc. | revised |  | R5-242005 |
| R5-240199 | Correction of NB-IoT NTN cell reselection test cases | MediaTek Inc. | withdrawn |  |  |
| R5-240200 | Addition and correction to the NTN related abbreviations in 36.521-3 | MediaTek (Hefei) Inc. | revised |  | R5-242004 |
| R5-240201 | Addition and correction to the NTN related abbreviations in 38.521-5 | MediaTek (Hefei) Inc. | revised |  | R5-242031 |
| R5-240202 | Addition and correction to the NTN related abbreviations in 38.533 | MediaTek (Hefei) Inc. | revised |  | R5-242032 |
| R5-240203 | Correction of UPIP test case 7.1.3.2.6 for specific message contents | ZTE Corporation | revised |  | R5-241552 |
| R5-240204 | Correction of Redcap inter-RAT E-UTRAN CGI test cases | MediaTek Inc. | revised |  | R5-242037 |
| R5-240205 | Correction of UPIP test case 8.2.6.4.1 | ZTE Corporation | revised |  | R5-241553 |
| R5-240206 | Update PICS of NTN test cases | MediaTek (Hefei) Inc. | withdrawn |  |  |
| R5-240207 | Correction to NR NTN PICS Mnemonic | MediaTek Inc. | withdrawn |  |  |
| R5-240208 | Correction to Rel-16 PICS Mnemonic | MediaTek Inc. | withdrawn |  | - |
| R5-240209 | Correction to Rel-17 PICS Mnemonic | MediaTek Inc. | withdrawn |  |  |
| R5-240210 | Update ephemeris information of several RRM cases in 36.521-3 | MediaTek (Hefei) Inc. | withdrawn |  |  |
| R5-240211 | Update TT value to message content and test requirement of RA cases | MediaTek (Hefei) Inc. | agreed |  |  |
| R5-240212 | Editorial Correction to HST TCs on release information | MediaTek Beijing Inc. | agreed |  |  |
| R5-240213 | Editorial correction to HST TCs on test applicability description | MediaTek Beijing Inc. | withdrawn |  |  |
| R5-240214 | Editorial removal to editors notes of DCQR cases | MediaTek (Hefei) Inc., Rohde & Schwarz | revised |  | R5-241795 |
| R5-240215 | Addition of ephemeris information for GEO condition | MediaTek (Hefei) Inc. | withdrawn |  |  |
| R5-240216 | Update ephemeris to GEO condition test cases in 36.521-3 | MediaTek (Hefei) Inc. | withdrawn |  |  |
| R5-240217 | Addition of IoT NTN test cases grouping | MediaTek (Hefei) Inc. | agreed |  |  |
| R5-240218 | Corrections to test parameters for CSI test cases | MediaTek (Hefei) Inc. | agreed |  |  |
| R5-240219 | Update cell specific parameters for NB-IoT NTN RA test cases | MediaTek Beijing Inc. | agreed |  |  |
| R5-240220 | Add applicability for Rel-17 ATSSS test case 11.9.1 | China Telecom | withdrawn |  |  |
| R5-240221 | Update of UE pre-configuration for NGSO signalling test environment | MediaTek Inc. | revised |  | R5-241557 |
| R5-240222 | Correction to Table H.3.1-8A | MediaTek Beijing Inc. | revised |  | R5-241838 |
| R5-240223 | Correction to Table H.3.1-12 | MediaTek Beijing Inc. | revised |  | R5-241839 |
| R5-240224 | Correction to Table H.3.1-12A | MediaTek Beijing Inc. | revised |  | R5-241840 |
| R5-240225 | Correction to NR testcase 6.4.2.1 | Starpoint, TDIA | revised |  | R5-241495 |
| R5-240226 | Correction to NR testcase 8.1.1.4.1 | Starpoint, TDIA | revised |  | R5-241510 |
| R5-240227 | Revised WID on UE Conformance-IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS | China Telecom | revised |  | R5-241661 |
| R5-240228 | General updates of TS 38.521-1 clause 5 for R16 CA configurations | CU Digital Technology | agreed |  |  |
| R5-240229 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | CU Digital Technology, Verizon, Nokia, KDDI | revised |  | R5-241868 |
| R5-240230 | Correction to NR testcase 11.4.12 | Starpoint, TDIA | revised |  | R5-241528 |
| R5-240231 | General updates of clause 5 for R17 new CBW configurations | CU Digital Technology, Nokia | revised |  | R5-241775 |
| R5-240232 | Update 3DL CA reference sensitivity exceptions TC for CA_n1A-n8A-n78A | CU Digital Technology | agreed |  |  |
| R5-240233 | Update inter-band NR CA configuration of three bands CA_n1A-n8A-n78A | CU Digital Technology | agreed |  |  |
| R5-240234 | Update reference sensitivity test cases for CA_n1A-n8A-n78A | CU Digital Technology | agreed |  |  |
| R5-240235 | Addition of applicability of new ING_5GS test case 11.1.11 | China Telecom | withdrawn |  | - |
| R5-240236 | Correction to NR RRC_Idle generic procedure | Keysight Technologies UK | agreed |  |  |
| R5-240237 | Correction to NR RRC_Inactive generic procedure | Keysight Technologies UK | revised |  | R5-241628 |
| R5-240238 | Correction to RRC Inactive Mode test case 6.4.1.1 | Keysight Technologies UK | withdrawn |  |  |
| R5-240239 | Correction to NR testcase 8.1.5.11.2 | Starpoint, TDIA | agreed |  |  |
| R5-240240 | Add new ATSSS test case 11.9.3 | ZTE Corporation | revised |  | R5-241618 |
| R5-240241 | Add new ATSSS test case 11.9.4 | ZTE Corporation | revised |  | R5-241619 |
| R5-240242 | Add applicabilities for new ATSSS test cases 11.9.3 and 11.9.4 | ZTE Corporation | withdrawn |  |  |
| R5-240243 | Correction to NR testcase 8.2.6.3.5 | Starpoint, TDIA | agreed |  |  |
| R5-240244 | Correction to NR testcase 8.2.6.3.6 | Starpoint, TDIA | agreed |  |  |
| R5-240245 | WP UE Conformance NR Coverage Enhancement RAN5#102 | China Telecom | noted |  |  |
| R5-240246 | SR UE Conformance NR Coverage Enhancement RAN5#102 | China Telecom | noted |  |  |
| R5-240247 | SR UE Conformance Further enhancement on NR demodulation performance RAN5#102 | China Telecom | noted |  |  |
| R5-240248 | WP UE Conformance – Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink RAN5#102 | China Telecom | noted |  |  |
| R5-240249 | SR UE Conformance – Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink RAN5#102 | China Telecom | noted |  |  |
| R5-240250 | Addition of DL interruption allowed indication for CA_n1-n3-n78 | China Telecom | revised |  | R5-241941 |
| R5-240251 | Addition of MMSE-IRC CQI reporting test case with FDD 4Rx | China Telecom | revised |  | R5-241811 |
| R5-240252 | Addition of MMSE-IRC CQI reporting test case with TDD 2Rx | China Telecom | revised |  | R5-241812 |
| R5-240253 | Addition of MMSE-IRC CQI reporting test case with TDD 4Rx | China Telecom | revised |  | R5-241813 |
| R5-240254 | Completion of MMSE-IRC CQI reporting test case with FDD 2Rx | China Telecom | revised |  | R5-241814 |
| R5-240255 | Addition of MMSE-IRC CQI reporting test applicability rule | China Telecom | revised |  | R5-241845 |
| R5-240256 | Correction to NR testcase 8.1.4.3.1 | Starpoint, TDIA | revised |  | R5-241515 |
| R5-240257 | Discussion on UE channel bandwidths capabilities | CAICT | noted |  |  |
| R5-240258 | Correction of Notes in tables of test channel bandwidths | CAICT | revised |  | R5-241921 |
| R5-240259 | Discussion on removal of LTE anchor agnostic approach testing in 6.5B.3.3.2 of 38.521-3 | CAICT | noted |  |  |
| R5-240260 | Removal of LTE anchor agnostic approach testing in 6.5B.3.3.2 | CAICT | revised |  | R5-241991 |
| R5-240261 | Update of Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3 | CAICT | revised |  | R5-241993 |
| R5-240262 | Correction of applicability and test coverage rules for SA and NSA capable devices | CAICT | revised |  | R5-241905 |
| R5-240263 | Correction of applicability for test with LTE anchor agnostic approach in 6.5B.3.3.1 | CAICT | revised |  | R5-241923 |
| R5-240264 | Addition of new test case 6.3J.2 Transmit OFF power for ATG | CAICT | revised |  | R5-241755 |
| R5-240265 | Addition of new test case 6.3J.1 Minimum output power for ATG | CAICT | revised |  | R5-241756 |
| R5-240266 | Addition of new test case 7.4J Maximum input level for ATG | CAICT | revised |  | R5-241760 |
| R5-240267 | Addition of new test case 7.5J Adjacent channel selectivity for ATG | CAICT | revised |  | R5-241761 |
| R5-240268 | Addition of new test case 7.6J.2 In-band blocking for ATG | CAICT | revised |  | R5-241762 |
| R5-240269 | Addition of new test case 7.6J.3 Out-of-band blocking for ATG | CAICT | revised |  | R5-241763 |
| R5-240270 | Addition of test points analysis for ATG test cases in 38.521-1 | CAICT, CMCC | revised |  | R5-241765 |
| R5-240271 | Correction of errors in Annex C | CAICT | agreed |  |  |
| R5-240272 | Style correction for clause title of 4.3.1.1.1.100 and 4.3.1.1.1.101 | CAICT | agreed |  |  |
| R5-240273 | Addition of asymmetric UL and DL channel bandwidth combinations of band n8 in 5.3.6 | CAICT | agreed |  |  |
| R5-240274 | Addition of test frequencies for new R16 NR CA combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240275 | Addition of test frequencies for new R17 NR CA combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240276 | Addition of UE capability for new R16 NR CA combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240277 | Addition of UE capability for new R17 NR CA combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240278 | New WID on UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR | China Telecom | revised |  | R5-241461 |
| R5-240279 | Addition of PC2 for n8 into TC 6.2.1 MOP | CU Digital Technology | agreed |  |  |
| R5-240280 | Addition of PC2 for n8 into TC 6.2.3 A-MPR | CU Digital Technology | withdrawn |  |  |
| R5-240281 | Addition of PC2 for n8 into TC 7.3.2 Reference Sensitivity | CU Digital Technology | agreed |  |  |
| R5-240282 | Add new r17 ATSSS test case 11.9.2 | China Telecom | revised |  | R5-241620 |
| R5-240283 | New WID on UE Conformance - Further NR coverage enhancements | China Telecom | revised |  | R5-241462 |
| R5-240284 | Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH | MediaTek Inc. | withdrawn |  |  |
| R5-240285 | Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH | MediaTek Inc. | withdrawn |  |  |
| R5-240286 | Update URLLC TC 7.1.1.4.1.5 to test RedCap UE | MediaTek Inc. | withdrawn |  |  |
| R5-240287 | Update URLLC TC 7.1.1.4.2.6 to test RedCap UE | MediaTek Inc. | withdrawn |  |  |
| R5-240288 | Addition of r16 and r17 UL tx switching and DL interruption supporting PICS for inter band CA with 3 bands | China Telecom | withdrawn |  |  |
| R5-240289 | Correction to NR slice TC 6.1.2.25 | MediaTek Inc.,Anritsu | agreed |  |  |
| R5-240290 | Correction to System information for NTN | MediaTek Inc. | revised |  | R5-241558 |
| R5-240291 | Revised WID on UE Conformance - New Rel-18 NR licensed bands and extension of existing NR bands | Huawei, Hisilicon | revised |  | R5-241664 |
| R5-240292 | WP of New Rel-18 NR licensed bands and extension of existing NR bands | Huawei, Hisilicon | noted |  |  |
| R5-240293 | SR of New Rel-18 NR licensed bands and extension of existing NR bands | Huawei, Hisilicon | noted |  |  |
| R5-240294 | Revised WID on UE Conformance - Rel-18 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | revised |  | R5-241665 |
| R5-240295 | WP of Rel-18 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | noted |  |  |
| R5-240296 | SR of Rel-18 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | noted |  |  |
| R5-240297 | WP of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL | Huawei, Hisilicon | noted |  |  |
| R5-240298 | SR of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL | Huawei, Hisilicon | noted |  |  |
| R5-240299 | WP of Rel-17 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | noted |  |  |
| R5-240300 | SR of Rel-17 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | noted |  |  |
| R5-240301 | WP of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 | Huawei, Hisilicon | noted |  |  |
| R5-240302 | SR of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 | Huawei, Hisilicon | noted |  |  |
| R5-240303 | New WID on UE Conformance - Further RF requirements enhancement for NR and EN-DC in frequency range 1 | Huawei, Hisilicon, China Telecom | revised |  | R5-241463 |
| R5-240304 | PRD21 CDS: PC3 SUL_N78A-N81A SUL_N78A-N84A | Huawei, Hisilicon | noted |  |  |
| R5-240305 | Correction to test configuration for non-exception REFSENS testing for 3CC EN-DC | Huawei, HiSilicon | agreed |  |  |
| R5-240306 | Discussion on updating FR1 Inter-band 2UL CA AMPR test case | Huawei, Hisilicon | noted |  |  |
| R5-240307 | Updating test case AMPR for inter-band CA | Huawei, HiSilicon | agreed |  |  |
| R5-240308 | Updating test principle for AMPR for inter-band UL CA | Huawei, HiSilicon | agreed |  |  |
| R5-240309 | Updating test frequency for SUL band n83 20MHz CBW | Huawei, HiSilicon | revised |  | R5-241716 |
| R5-240310 | Updating test frequency range for SUL band n83 | Huawei, HiSilicon | agreed |  |  |
| R5-240311 | Discussion on FR1 REFSENS exceptions test requirements for CA | Huawei, Hisilicon | noted |  |  |
| R5-240312 | Updating FR1 PC2 REFSENS exceptions testing | Huawei, HiSilicon | revised |  | R5-241721 |
| R5-240313 | Introducing indicator for Power Class of CA configuration with single uplink carrier | Huawei, HiSilicon | agreed |  |  |
| R5-240314 | Correcting errors in REFSENS for CA test case for PC3 CA configurations | Huawei, HiSilicon | revised |  | R5-241770 |
| R5-240315 | Editorial correction to test requirement of MPR for CA test case | Huawei, HiSilicon | agreed |  |  |
| R5-240316 | Introducing SUL configuration SUL_n78A-n81A | Huawei, HiSilicon | agreed |  |  |
| R5-240317 | Updating REFSENS testing for SUL configuration | Huawei, HiSilicon | revised |  | R5-241771 |
| R5-240318 | Updating test points for FR1 REFSENS for SUL test case | Huawei, HiSilicon | agreed |  |  |
| R5-240319 | Updating AMPR testing for SUL band n81 | Huawei, HiSilicon | agreed |  |  |
| R5-240320 | Updating UTRA ACLR testing for SUL band n81 | Huawei, HiSilicon | agreed |  |  |
| R5-240321 | Revised WID: UE Conformance Test Aspects - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | MediaTek Inc. | revised |  | R5-241666 |
| R5-240322 | Correction to emergency service TC 11.4.4 | MediaTek Inc. | withdrawn |  |  |
| R5-240323 | Correction to eCPSOR TC 6.3.2.5 | MediaTek Inc. | withdrawn |  |  |
| R5-240324 | Addition of reference sensitivity and spurious emissions TP analysis for new R16 NR CA combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240325 | Addition of reference sensitivity and spurious emissions TP analysis for new R17 NR CA combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240326 | Addition of delta TIBc and UE maximum output power for new R16 NR CA combos within FR1 | KDDI Corporation | revised |  | R5-241728 |
| R5-240327 | Addition of spurious emissions, delta TIBc and UE maximum output power for new R17 NR CA combos within FR1 | KDDI Corporation | revised |  | R5-241734 |
| R5-240328 | Addition of delta RIBc and reference sensitivity for new R16 NR CA combos within FR1 | KDDI Corporation | revised |  | R5-241731 |
| R5-240329 | Addition of delta RIBc and reference sensitivity for new R17 NR CA combos within FR1 | KDDI Corporation | revised |  | R5-241735 |
| R5-240330 | Addition of RF baseline implementation capabilities for new PC2 EN-DC combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240331 | Addition of UE maximum output power for new PC2 EN-DC combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240332 | Addition of reference sensitivity for new PC2 EN-DC combos within FR1 | KDDI Corporation | revised |  | R5-241904 |
| R5-240333 | Updating PC2 test requirements in MPR test case for band n1 and n3 | Huawei, HiSilicon | agreed |  |  |
| R5-240334 | Addition of Measurement Uncertainties and Test Tolerances for ATG UE | CMCC | revised |  | R5-241764 |
| R5-240335 | Editorial correction to note numbering for inter-band EN-DC capabilities table | Huawei, HiSilicon | agreed |  |  |
| R5-240336 | Addition of new test case 6.3G.4.1 Absolute power tolerance for Tx Diversity | Huawei, HiSilicon | agreed |  |  |
| R5-240337 | Removal of square brackets for Tx Diversity capability | Huawei, HiSilicon | revised |  | R5-241879 |
| R5-240338 | Update of MOP test configuration for DC_20A_n28A | Huawei, HiSilicon | agreed |  |  |
| R5-240339 | Correction to H3 title for clauses including NE-DC test cases | Huawei, HiSilicon | agreed |  |  |
| R5-240340 | WP UE Conformance - Further Multi-RAT Dual-Connectivity enhancement | Huawei, HiSilicon | noted |  |  |
| R5-240341 | SR UE Conformance - Further Multi-RAT Dual-Connectivity enhancement | Huawei, HiSilicon | noted |  |  |
| R5-240342 | WP UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC | Huawei, HiSilicon | noted |  |  |
| R5-240343 | SR UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC | Huawei, HiSilicon | noted |  |  |
| R5-240344 | New WID on UE Conformance - Network energy savings for NR | Huawei, HiSilicon | revised |  | R5-241464 |
| R5-240345 | New WID on UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 | Huawei, HiSilicon, CMCC | revised |  | R5-241465 |
| R5-240346 | Update of RRC message RRCReconfigurationComplete, RRCSetupComplete and SecurityModeComplete for SL relay | China Telecom | revised |  | R5-241722 |
| R5-240347 | Update of Test procedure for establishing unicast mode ProSe Direct communication | China Telecom | revised |  | R5-241723 |
| R5-240348 | Addition of Sidelink Capabilities to support direct to indirect path switch for NR sidelink U2N Relay | China Telecom | revised |  | R5-241724 |
| R5-240349 | Correction of Default parameters for simulated SNPN cells. | China Telecom | agreed |  |  |
| R5-240350 | Addition of capability for UEs to support steering of roaming SNPN selection information (SOR-SNPN-SI) and steering of roaming connected mode control information (SOR-CMCI) for Rel-17 eNPN | China Telecom | revised |  | R5-241581 |
| R5-240351 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | revised |  | R5-241582 |
| R5-240352 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 eNPN | China Telecom | revised |  | R5-241583 |
| R5-240353 | Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 eNPN | China Telecom | revised |  | R5-241584 |
| R5-240354 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 eNPN | China Telecom | revised |  | R5-241585 |
| R5-240355 | Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN | China Telecom | revised |  | R5-241586 |
| R5-240356 | Addition of new test case 9.1.11.5 for Mobility management in Rel-17 eNPN | China Telecom | revised |  | R5-241587 |
| R5-240357 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | revised |  | R5-241588 |
| R5-240358 | Addition of new test case 9.1.11.7 for Rel-17 eNPN | China Telecom | withdrawn |  |  |
| R5-240359 | Addition of applicability for eNPN test cases | China Telecom | revised |  | R5-241646 |
| R5-240360 | Scoping NR SA applicable TCs for SNPN-only UEs | China Telecom | revised |  | R5-241589 |
| R5-240361 | WP UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#102 | China Telecom | noted |  |  |
| R5-240362 | SR UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#102 | China Telecom | noted |  |  |
| R5-240363 | WP UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#102 | China Telecom | noted |  |  |
| R5-240364 | SR UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#102 | China Telecom | noted |  |  |
| R5-240365 | WP UE Conformance – Rel-17 LTE CA Configurations RAN5#102 | China Telecom | noted |  |  |
| R5-240366 | SR UE Conformance – Rel-17 LTE CA Configurations RAN5#102 | China Telecom | noted |  |  |
| R5-240367 | New WID on UE Conformance – Multi-carrier enhancements for NR | China Telecom, Huawei, HiSilicon | revised |  | R5-241466 |
| R5-240368 | Applicability statement for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1 | China Telecom | agreed |  |  |
| R5-240369 | Addition of NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.1 | China Telecom, Huawei, HiSilicon | agreed |  |  |
| R5-240370 | Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1 | China Telecom, Huawei, HiSilicon | revised |  | R5-241896 |
| R5-240371 | Cell configuration mapping for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1 | China Telecom, Huawei, HiSilicon | revised |  | R5-241832 |
| R5-240372 | Test tolerances and measurement uncertainty for DL interruptions test cases 6.5.7A.1, 6.5.7B.1 and 6.5.7C.1 | China Telecom, Huawei, HiSilicon | agreed |  |  |
| R5-240373 | Test Tolerances for NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.1 | China Telecom, Huawei, HiSilicon | agreed |  |  |
| R5-240374 | Test Tolerances for NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1 | China Telecom, Huawei, HiSilicon | agreed |  |  |
| R5-240375 | Addition of new test case 6.2H.3.2 MPR for inter-band CA with UL MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-240376 | Addition of new test case 6.2H.2.3 MPR for inter-band EN-DC with UL MIMO | Huawei, HiSilicon | revised |  | R5-241725 |
| R5-240377 | Discussion on work plan of 3Tx inter-band UL CA or EN-DC | Huawei, HiSilicon | noted |  |  |
| R5-240378 | Update of conditional PSCell addition configuration for NR-DC | Huawei, HiSilicon | agreed |  |  |
| R5-240379 | Addition of new test case 8.2.3.18.4 inter-SN CPC for EN-DC | Huawei, HiSilicon | revised |  | R5-241602 |
| R5-240380 | Addition of new test case 8.2.3.18.5 inter-SN CPC for NR-DC | Huawei, HiSilicon | revised |  | R5-241603 |
| R5-240381 | Update of test procedure in 4.5.10.1 PSCell activation for EN-DC | Huawei, HiSilicon | agreed |  |  |
| R5-240382 | Correction to 4.5.11.1 EN-DC FR1 Conditional PSCell Addition | Huawei, HiSilicon | agreed |  |  |
| R5-240383 | Update of measurement configuration for 5.5.13.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240384 | Update of measurement configuration for 7.5.12.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240385 | Update of message contents for 4.5.3.7 and 6.5.3.11 | Huawei, HiSilicon | agreed |  |  |
| R5-240386 | Addition of applicability for inter-SN conditional PSCell change | Huawei, HiSilicon | revised |  | R5-241604 |
| R5-240387 | Addition of UE capability for inter-SN conditional PSCell change | Huawei, HiSilicon | revised |  | R5-241601 |
| R5-240388 | TP analysis for NR ATG test cases | China Mobile Com. Corporation | withdrawn |  |  |
| R5-240389 | TP analysis for ATG Configured Tx power | China Mobile Com. Corporation | withdrawn |  |  |
| R5-240390 | New WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom, Ericsson, Huawei, Hisilicon, Qualcomm | revised |  | R5-241467 |
| R5-240391 | WP on UE Conformance – Support of reduced capability NR devices | China Unicom | noted |  |  |
| R5-240392 | SR on UE Conformance – Support of reduced capability NR devices | China Unicom | noted |  |  |
| R5-240393 | Revised WID on UE Conformance – Support of reduced capability NR devices | China Unicom,Hisilicon, Ericsson, Huawei, Qualcomm | agreed |  |  |
| R5-240394 | WP on UE Conformance - Additional NR bands for UL-MIMO in Rel-18 | China Unicom | noted |  |  |
| R5-240395 | SR on UE Conformance - Additional NR bands for UL-MIMO in Rel-18 | China Unicom | noted |  |  |
| R5-240396 | WP on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band | China Unicom | noted |  |  |
| R5-240397 | SR on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band | China Unicom | noted |  |  |
| R5-240398 | Addition of test frequencies for new EN-DC comb within FR2 | KDDI Corporation | revised |  | R5-241927 |
| R5-240399 | Addition of NR NTN capabilities | CAICT | revised |  | R5-241909 |
| R5-240400 | Update of Applicability and Additional Information of RF conformance test cases for Satellite Access | CAICT | revised |  | R5-241910 |
| R5-240401 | Discussion on MU and TT analysis for FR2c | Keysight Technologies UK Ltd | noted |  |  |
| R5-240402 | FR2c MU definition in 38.903 | Keysight Technologies UK Ltd | revised |  | R5-241863 |
| R5-240403 | FR2c MU - Tx test cases update in 38.521-2 | Keysight Technologies UK Ltd | withdrawn |  |  |
| R5-240404 | FR2c MU - Rx test cases update in 38.521-2 | Keysight Technologies UK Ltd | revised |  | R5-241860 |
| R5-240405 | FR2c MU - Updates in 38.521-3 | Keysight Technologies | withdrawn |  |  |
| R5-240406 | FR2 MU - PC1 UL MIMO - Minimum output power test - 38.903 | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240407 | FR2 MU - PC1 UL MIMO - Minimum output power test - 38.521-2 | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240408 | Discussion on ACS and IBB testing | Keysight Technologies UK Ltd | noted |  |  |
| R5-240409 | Blocking measurement procedure updates in section K.1.8 | Keysight Technologies | agreed |  |  |
| R5-240410 | Addition of UE capability for new EN-DC comb within FR2 | KDDI Corporation | agreed |  |  |
| R5-240411 | SR of UE Conformance - Signal level Enhanced Network Selection | Huawei, HiSilicon | noted |  |  |
| R5-240412 | WP of UE Conformance - Signal level Enhanced Network Selection | Huawei, HiSilicon | noted |  |  |
| R5-240413 | Correction of MAC TC 7.1.1.6.2-UL CG type1 | Huawei, HiSilicon | agreed |  |  |
| R5-240414 | Correction of MAC TC 7.1.1.3.2b-Lcp-Restriction | Huawei, HiSilicon | revised |  | R5-241506 |
| R5-240415 | Correction of MAC TC 7.1.1.8.1-BWP switch | Huawei, HiSilicon | revised |  | R5-241507 |
| R5-240416 | Editorial correction of NR TC 11.4.x-Emergency services | Huawei, HiSilicon | agreed |  |  |
| R5-240417 | Correction of SDT TC 7.1.1.13.1-2-Step RACH | Huawei, HiSilicon | withdrawn |  |  |
| R5-240418 | Addition of SENSE eMTC TC 6.1.1.12-PLMN selection | Huawei, HiSilicon | revised |  | R5-241611 |
| R5-240419 | Addition of SENSE eMTC TC 6.1.1.13-PLMN selection Exception | Huawei, HiSilicon | revised |  | R5-241612 |
| R5-240420 | Addition of SENSE eMTC TC 6.1.1.14-Periodic SENSE | Huawei, HiSilicon | revised |  | R5-241613 |
| R5-240421 | Addition of SENSE NB-IoT TC 22.2.14-PLMN selection | Huawei, HiSilicon | revised |  | R5-241614 |
| R5-240422 | Addition of SENSE NB-IoT TC 22.2.15-PLMN selection Exception | Huawei, HiSilicon | revised |  | R5-241615 |
| R5-240423 | Addition of SENSE NB-IoT TC 22.2.16-Periodic SENSE | Huawei, HiSilicon | revised |  | R5-241616 |
| R5-240424 | Addition of PICS and test applicability for SENSE TC | Huawei, HiSilicon | agreed |  |  |
| R5-240425 | Add PICS for PEIPS | Huawei, HiSilicon | agreed |  |  |
| R5-240426 | Correction the title of PEIPS TC 9.1.14.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240427 | Update the applicability of PEIPS TC 9.1.14.1 and 11.4.1a | Huawei, HiSilicon | revised |  | R5-241547 |
| R5-240428 | Add PIXIT for MUSIM gap | Huawei, HiSilicon,China Telecom | withdrawn |  |  |
| R5-240429 | Correction of MUSIM TC 8.1.2.1.6-MUSIM gap configuration | Huawei, HiSilicon,China Telecom | revised |  | R5-241640 |
| R5-240430 | Correction of Set MUSIM UAI test function | Huawei, HiSilicon,China Telecom | revised |  | R5-241636 |
| R5-240431 | Discussion to update MUSIM gap TC | Huawei, HiSilicon,China Telecom | revised |  | R5-241633 |
| R5-240432 | Move all V2X sub-clauses from 4.7 into a new clause 4.7D | Huawei, HiSilicon, CATT | revised |  | R5-241538 |
| R5-240433 | Editorial update the derivation path for the V2X message contents table | Huawei, HiSilicon, CATT | agreed |  |  |
| R5-240434 | Correction of Test Loop Mode C for MBS broadcast | Huawei, HiSilicon | revised |  | R5-241492 |
| R5-240435 | Correction of MBS broadcast TC 14.1.1.1-Test Loop Mode C | Huawei, HiSilicon | revised |  | R5-241534 |
| R5-240436 | Correction of MBS broadcast TC 14.1.1.3-Test Loop Mode C | Huawei, HiSilicon | revised |  | R5-241535 |
| R5-240437 | Correction of MBS broadcast TC 14.1.2.1-Test Loop Mode C | Huawei, HiSilicon | revised |  | R5-241536 |
| R5-240438 | Correction of MBS broadcast TC 14.1.2.2-Test Loop Mode C | Huawei, HiSilicon | withdrawn |  |  |
| R5-240439 | Correction of MBS broadcast TC 14.1.2.3-Test Loop Mode C | Huawei, HiSilicon | withdrawn |  |  |
| R5-240440 | Correction of MBS broadcast TC 14.1.3.1-Test Loop Mode C | Huawei, HiSilicon | revised |  | R5-241537 |
| R5-240441 | Correction to redcap TC 6.1.2.27 | Huawei, HiSilicon | agreed |  |  |
| R5-240442 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon | revised |  | R5-241521 |
| R5-240443 | Corrections to spurious emissions for CA_n3A-n28A | KDDI Corporation | agreed |  |  |
| R5-240444 | Add IndirectPathFailureInformation | Ericsson | agreed |  |  |
| R5-240445 | Add MBSMulticastConfiguration | Ericsson | agreed |  |  |
| R5-240446 | Add MeasurementReportAppLayer | Ericsson | agreed |  |  |
| R5-240447 | Editorial update SystemInformation | Ericsson | revised |  | R5-241483 |
| R5-240448 | Add UEPositioningAssistanceInfo | Ericsson | agreed |  |  |
| R5-240449 | Resubmission of Annex B Corrections to BWP RMCs and configurations for RedCap RRM TCs | ETSI MCC ( Huawei, HiSilicon, Starpoint) | agreed |  |  |
| R5-240450 | WP UE Conformance – Further enhancements of NR RF requirements for FR2 | Nokia, Nokia Shanghai Bell, Apple | noted |  |  |
| R5-240451 | SR UE Conformance – Further enhancements of NR RF requirements for FR2 | Nokia, Nokia Shanghai Bell, Apple | noted |  |  |
| R5-240452 | WP UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R5-240453 | SR UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R5-240454 | Editorial correction to UE Power Class for n100 and n101 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240455 | Correction to Reference sensitivity power level test configuration for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | revised |  | R5-241772 |
| R5-240456 | Correction to reference sensitivity test point analysis for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | revised |  | R5-241777 |
| R5-240457 | Correction to Reference sensitivity power level for inter-band 4DL CA | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-240458 | New WID on UE Conformance - Introduction of 900 MHz LTE band in the US | Nokia, Nokia Shanghai Bell | revised |  | R5-241468 |
| R5-240459 | PRD21 CDS: NR CA PC3 FR1 CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | noted |  |  |
| R5-240460 | Introduction of test frequencies for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240461 | Introduction of CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) for physical layer baseline implementation capabilities | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240462 | Introduction of reference sensitivity test point analysis for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240463 | Adding Reference sensitivity test requirements for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | revised |  | R5-241736 |
| R5-240464 | Introduction of spurious emission TP analysis for CA_n71A-n78A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240465 | Add UE maximum power requirements for CA_n71A-n78A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240466 | Add general spurious emissions for UE co-existence requirements for CA_n71A-n77A and CA_n71A-n78A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240467 | Bands n25 and n71 asymmetric UL/DL REFSENS | Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc. | noted |  |  |
| R5-240468 | Test frequencies and channel bandwidth updates for band n71 and n25 in Rel-17 | Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc. | agreed |  |  |
| R5-240469 | Addition of reference sensitivity channel bandwidths for n25 and n71 | Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc. | agreed |  |  |
| R5-240470 | Addition of 35 MHz CBW for transmitter requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240471 | PRD21 CDS: NR band and CBW FR1 n25 and n71 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R5-240472 | Correction to 5GSM test case 10.3.2.1 | Keysight Technologies UK, Samsung | revised |  | R5-241524 |
| R5-240473 | Correction to eDRX test case 11.7.2 | Keysight Technologies UK | agreed |  |  |
| R5-240474 | Correction to SIM profile of NB-IoT test cases | Keysight Technologies UK | agreed |  |  |
| R5-240475 | Removal of GEA1 and GEA2 algorithm verification | Keysight Technologies UK, Bureau Veritas | revised |  | R5-241657 |
| R5-240476 | Applicability updates to reflect the removal of GEA1 and GEA2 algorithm verification from the test cases | Keysight Technologies UK, Bureau Veritas | agreed |  |  |
| R5-240477 | Addition of new pics for NR sidelink U2N Relay PDU session establishment | TDIA, CATT | revised |  | R5-241600 |
| R5-240478 | Addition of test procedure for 5G ProSe U2N Relay Discovery | TDIA, CATT | revised |  | R5-241597 |
| R5-240479 | Addition of test procedure for remote Initial Access procedure under NR sidelink U2N Relay / Remote UE side | CATT, TDIA | revised |  | R5-241598 |
| R5-240480 | Removal of test procedure for Out_of_Coverage_with_Relay | CATT, TDIA | agreed |  |  |
| R5-240481 | Update test procedure for registration of NR sidelink U2N Relay / Relay UE side | TDIA, CATT | revised |  | R5-241599 |
| R5-240482 | Update of 5G ProSe information elements of UE policies for 5G ProSe usage information reporting | TDIA, CATT | agreed |  |  |
| R5-240483 | Update of 5G ProSe information elements of UE policies for 5G ProSe remote | TDIA, CATT | agreed |  |  |
| R5-240484 | Addition of PROSE PC5 DISCOVERY | TDIA, CATT | agreed |  |  |
| R5-240485 | Work plan: UE Conformance - NR sidelink enhancement | CATT | noted |  |  |
| R5-240486 | SR UE Conformance - NR sidelink enhancement | CATT | noted |  |  |
| R5-240487 | Work plan: UE Conformance - NR Sidelink Relay | CATT | noted |  |  |
| R5-240488 | SR UE Conformance - NR Sidelink Relay | CATT | noted |  |  |
| R5-240489 | Add IEs SIB22, SIB23, SIB24 and SIB25 | Ericsson | agreed |  |  |
| R5-240490 | Editorial update IE PosSystemInformation-r16-IEs | Ericsson | revised |  | R5-241484 |
| R5-240491 | Editorial update IE PosSI-SchedulingInfo | Ericsson | agreed |  |  |
| R5-240492 | Add IE AdvancedReceiver-MU-MIMO | Ericsson | agreed |  |  |
| R5-240493 | Add IE Altitude | Ericsson | agreed |  |  |
| R5-240494 | Add IE ATG-Config | Ericsson | agreed |  |  |
| R5-240495 | Add IEs CandidateTCI-State and CandidateTCI-UL-State | Ericsson | agreed |  |  |
| R5-240496 | Add IE CellDTXDRX-Config | Ericsson | agreed |  |  |
| R5-240497 | Add IE ClockQualityMetrics | Ericsson | agreed |  |  |
| R5-240498 | Add IEs CSI-ReportSubConfig, CSI-ReportSubConfigId and CSI-ReportSubConfigTriggerList | Ericsson | revised |  | R5-241485 |
| R5-240499 | Add IE EarlyUL-SyncConfig | Ericsson | agreed |  |  |
| R5-240500 | Add IE EUTRA-C-RNTI | Ericsson | agreed |  |  |
| R5-240501 | WP - UE Conformance - Further enhancements on MIMO for NR | Samsung | noted |  |  |
| R5-240502 | SR - UE Conformance - Further enhancements on MIMO for NR | Samsung | noted |  |  |
| R5-240503 | WP - UE Conformance - NR support for high speed train scenario in frequency range 2 | Samsung | noted |  |  |
| R5-240504 | SR - UE Conformance - NR support for high speed train scenario in frequency range 2 | Samsung | noted |  |  |
| R5-240505 | Addition of applicability for FeMIMO test cases | Samsung | revised |  | R5-241847 |
| R5-240506 | Addition of feMIMO physical layer baseline implementation capabilities | Samsung | revised |  | R5-241958 |
| R5-240507 | Adding measurement uncertainty and test tolerance for Demod PDCCH with intra-slot repetition test cases | Samsung | revised |  | R5-241959 |
| R5-240508 | Addition of applicability for HST FR2 test cases | Samsung | agreed |  |  |
| R5-240509 | Correction to NR5GC testcase 11.4.5 | ROHDE & SCHWARZ, Hisilicon | revised |  | R5-241529 |
| R5-240510 | Correction to NR5GC testcase 11.3.11 | ROHDE & SCHWARZ, Qualcomm | agreed |  |  |
| R5-240511 | Correction to NR5GC eDRX testcase 11.7.3 | ROHDE & SCHWARZ | agreed |  |  |
| R5-240512 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, Hisilicon, MediaTek | revised |  | R5-241644 |
| R5-240513 | Correction to NR Physical Cell IDs for NR-PRS assistance data | ROHDE & SCHWARZ | agreed |  |  |
| R5-240514 | Removal of GEA1/GEA2 and verification of non-support of GEA1/GEA2 | ROHDE & SCHWARZ | revised |  | R5-241632 |
| R5-240515 | Correction to GSM testcase 44.2.5.2.5 | ROHDE & SCHWARZ | agreed |  |  |
| R5-240516 | Correction to Table A.2: IR_G TTCN test cases | ROHDE & SCHWARZ | agreed |  |  |
| R5-240517 | New WID on UE Conformance - NR support for dedicated spectrum less than 5MHz for FR1 | Nokia, Nokia Shanghai Bell | revised |  | R5-241469 |
| R5-240518 | Update to NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240519 | Update to NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240520 | Removal of NOTE 1 from applicability of FR2 feMIMO test case | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240521 | Removal of NOTE 1 from applicability of FR1 and FR2 MRDC test cases | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-240522 | Update the existing PICS of inter-band CA between FR1 and FR2 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240523 | Test frequencies for asymmetric UL/DL CBW | Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc. | noted |  |  |
| R5-240524 | PRD21 CDS: NR CA PC3 FR1 CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) fallbacks and BCSs | Nokia, Nokia Shanghai Bell | noted |  |  |
| R5-240525 | PRD21 CDS: NR band and CBW FR1 n100 and n101 PC1 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R5-240526 | MCC TF160 Status Report | MCC TF160 | revised |  | R5-241458 |
| R5-240527 | Definition of a TN-to-NTN sysInfo combination mapping for legacy test cases to be run in NTN environment | MCC TF160 | revised |  | R5-241559 |
| R5-240528 | Correction of testcase 6.7.3 | MCC TF160 | agreed |  |  |
| R5-240529 | Updates to 5G V2X test case 12.1.7.1 | MCC TF160 | agreed |  |  |
| R5-240530 | 5G V2X: Test Model updates | MCC TF160 | revised |  | R5-241541 |
| R5-240531 | NR-U: Introduction of Test Model aspects | MCC TF160 | agreed |  |  |
| R5-240532 | Corrections to SDT TC 7.1.1.13.1 | MCC TF160, Lenovo | revised |  | R5-241543 |
| R5-240533 | Corrections to SDT TC 7.1.1.13.2 | MCC TF160, Lenovo | revised |  | R5-241544 |
| R5-240534 | Corrections to SDT TC 7.1.1.13.3 | MCC TF160, Lenovo | revised |  | R5-241545 |
| R5-240535 | Corrections to SDT TC 7.1.1.13.4 | MCC TF160, Lenovo | agreed |  |  |
| R5-240536 | Editorial updates to test procedure 4.9.1 | MCC TF160 | agreed |  |  |
| R5-240537 | Harmonization of SCell_add condition | MCC TF160 | agreed |  |  |
| R5-240538 | Updates to align PICS mnemonics | MCC TF160 | revised |  | R5-241491 |
| R5-240539 | Correction to NR test case 11.8.6 | MCC TF160 | agreed |  |  |
| R5-240540 | Correction to NR test case 6.1.2.3 | MCC TF160 | agreed |  |  |
| R5-240541 | Corrections to 5G TC 8.1.4.2.1.2 | MCC TF160 | agreed |  |  |
| R5-240542 | Updates for NR RRC test case 8.1.5.1.1 | MCC TF160 | agreed |  |  |
| R5-240543 | Updates for EN-DC RRC test case 8.2.1.1.1 | MCC TF160 | agreed |  |  |
| R5-240544 | Updates for NE-DC RRC test case 8.2.1.1.2 | MCC TF160 | agreed |  |  |
| R5-240545 | Update to NE-DC RRC test case 8.2.2.2.3 | MCC TF160 | agreed |  |  |
| R5-240546 | Misc. updates to TS 38.523-2 | MCC TF160 | agreed |  |  |
| R5-240547 | Routine maintenance for TS 38.523-3 | MCC TF160 | agreed |  |  |
| R5-240548 | Correction to LTE Inter-RAT TC 8.1.3.7 | MCC TF160 | agreed |  |  |
| R5-240549 | Routine maintenance for TS 36.523-3 | MCC TF160 | agreed |  |  |
| R5-240550 | Correction to UTRAN Inter-RAT TC 8.6.3.3 | MCC TF160 | agreed |  |  |
| R5-240551 | Removal of GEA2 from TS 34.123-3 | MCC TF160 | agreed |  |  |
| R5-240552 | Correction of clause 4 | MCC TF160 | agreed |  |  |
| R5-240553 | Removal of PIXIT for deprecated signalling GNSS scenarios | MCC TF160 | agreed |  |  |
| R5-240554 | Removal of deprecated signalling GNSS scenarios | MCC TF160 | agreed |  |  |
| R5-240555 | Corrections of clause 5.3.2 | MCC TF160 | agreed |  |  |
| R5-240556 | Corrections of clause 5.3.29 | MCC TF160 | agreed |  |  |
| R5-240557 | Corrections of clause 5.3.32 | MCC TF160 | agreed |  |  |
| R5-240558 | Corrections of clause 5.4.2 | MCC TF160 | agreed |  |  |
| R5-240559 | Corrections of clause 5.5.1 | MCC TF160 | agreed |  |  |
| R5-240560 | Corrections of clause 5.5.2.11 | MCC TF160 | agreed |  |  |
| R5-240561 | Corrections of clause 5.5.2.13 | MCC TF160 | agreed |  |  |
| R5-240562 | Corrections of clause 5.5.2.19.4 | MCC TF160 | agreed |  |  |
| R5-240563 | Corrections of clause 5.5.2.7.2 | MCC TF160 | agreed |  |  |
| R5-240564 | Corrections of clause 5.5.3.3.1A | MCC TF160 | agreed |  |  |
| R5-240565 | Corrections of clause 5.5.9.1 | MCC TF160 | agreed |  |  |
| R5-240566 | Corrections of references to 24.282 | MCC TF160 | agreed |  |  |
| R5-240567 | Correction of MO call release of calls using a pre-established session | MCC TF160 | agreed |  |  |
| R5-240568 | Correction of RRC connection release in several test cases | MCC TF160 | agreed |  |  |
| R5-240569 | Correction of testcase 6.1.1.17 | MCC TF160 | agreed |  |  |
| R5-240570 | Correction of testcase 6.1.1.5 | MCC TF160 | agreed |  |  |
| R5-240571 | Correction of testcase 6.1.2.14 | MCC TF160 | agreed |  |  |
| R5-240572 | Correction of testcase 6.1.2.2 | MCC TF160 | agreed |  |  |
| R5-240573 | Correction of testcase 6.1.3.1 | MCC TF160 | agreed |  |  |
| R5-240574 | Correction of testcase 6.2.10 | MCC TF160 | agreed |  |  |
| R5-240575 | Correction of testcase 6.2.14 | MCC TF160 | agreed |  |  |
| R5-240576 | Correction of testcase 6.2.15 | MCC TF160 | agreed |  |  |
| R5-240577 | Correction of testcase 6.2.18 | MCC TF160 | agreed |  |  |
| R5-240578 | Correction of testcase 6.2.19 | MCC TF160 | agreed |  |  |
| R5-240579 | Correction of testcase 6.2.22 | MCC TF160 | agreed |  |  |
| R5-240580 | Correction of testcase 6.2.26 | MCC TF160 | agreed |  |  |
| R5-240581 | Correction of testcase 6.2.9 | MCC TF160 | agreed |  |  |
| R5-240582 | Correction of testcase 6.1.3.1 | MCC TF160 | agreed |  |  |
| R5-240583 | Correction of testcase 6.7.1 | MCC TF160 | agreed |  |  |
| R5-240584 | Corrections of references to 24.282 | MCC TF160 | agreed |  |  |
| R5-240585 | Corrections to LTE TC 8.1.3.16 | MCC TF160 | agreed |  |  |
| R5-240586 | Corrections to NB-IoT RLC TC 22.3.2.3 | MCC TF160 | revised |  | R5-241478 |
| R5-240587 | Correction to applicability of NB-IoT TC 22.3.2.7a | MCC TF160 | agreed |  |  |
| R5-240588 | Modification of testcase 9.1.6.2.3 UE or NW initiated de-registration for UAS | Nokia, Nokia Shanghai Bell | revised |  | R5-241605 |
| R5-240589 | Modification of testcase 8.1.5.13.2 for Data on non-SDT Radio Bearers for NR SmallData | Nokia, Nokia Shanghai Bell | revised |  | R5-241546 |
| R5-240590 | Modification of testcase 8.1.5.13.2 applicability clauses | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-240591 | New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-240592 | New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-240593 | Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2 | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-240594 | New WID on UE Conformance - XR (eXtended Reality) enhancements for NR | Nokia, Nokia Shanghai Bell, CMCC, Huawei, Hisilicon, Qualcomm | revised |  | R5-241470 |
| R5-240595 | On Alternate EIS Search Procedure with Interpolation | Keysight Technologies UK Ltd | noted |  |  |
| R5-240596 | CR on Alternate TRS Procedure with Linearization | Keysight Technologies UK Ltd | withdrawn |  |  |
| R5-240597 | Introduce Annex for maximum uncertainty of test system and test tolerance | Keysight Technologies UK Ltd | revised |  | R5-241934 |
| R5-240598 | Correction and alignment of Annex B title | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240599 | Further clarifications in Annex A such as MPAC description and coordinate system | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240600 | Align Test Case Structure to typical RAN5 spec | Keysight Technologies UK Ltd | revised |  | R5-241935 |
| R5-240601 | On Re-Positioning Concept for FR2 RRM TCs | Keysight Technologies UK Ltd | noted |  |  |
| R5-240602 | Introducing the Re-Positioning Concept for FR2 RRM TCs | Keysight Technologies UK Ltd, Rohde&Schwarz | revised |  | R5-241862 |
| R5-240603 | On Coarse&Fine Beam Peak Search Grids | Keysight Technologies UK Ltd, CAICT | noted |  |  |
| R5-240604 | CR on Coarse&Fine Beam Peak Search Grids | Keysight Technologies UK Ltd, CAICT | agreed |  |  |
| R5-240605 | On 30cm QoQZ MU Topics | Keysight Technologies UK Ltd | noted |  |  |
| R5-240606 | CR to define FR2d QoQZ MUs and misc QoQZ MU corrections | Keysight Technologies UK Ltd | revised |  | R5-241864 |
| R5-240607 | WP UE Conformance - User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects) | Vodafone | noted |  |  |
| R5-240608 | Status Report for UE Conformance – User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects) | Vodafone | noted |  |  |
| R5-240609 | Correction of NR TC 9.1.4.1-CUC | Huawei, HiSilicon | revised |  | R5-241522 |
| R5-240610 | New WID on UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2 (FR2) | Samsung | revised |  | R5-241471 |
| R5-240611 | Updates to NR CA TCs 8.1.5.x.y.z | MCC TF160 | agreed |  |  |
| R5-240612 | New WID on UE Conformance – NR MIMO Evolution for Downlink and Uplink | Samsung, Huawei, Hisilicon | revised |  | R5-241472 |
| R5-240613 | Updates to NR CA TCs 8.2.4.x.y.z | MCC TF160 | agreed |  |  |
| R5-240614 | E-UTRA V2X spurious emissions MU definition up to 26GHz | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240615 | NTN NB-IoT - Message exception correction in RRM test case 13.1.1.2 | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240616 | RF TRx testing - P-Max configuration extension to RX tests to enable TxD | Keysight Technologies UK Ltd | revised |  | R5-241704 |
| R5-240617 | Added 30kHz SCS for SSB in n53 to 30kHz SCS test frequencies | Keysight Technologies UK Ltd, Apple | agreed |  |  |
| R5-240618 | Further discussion on FR1 EVM testing in shorter transient period | Keysight Technologies UK Ltd | noted |  |  |
| R5-240619 | FR1 CA - Test requirement correction for n2-n48 combo in test 7.3A.1_1 | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240620 | Corrections for combo n48A-n66A in test case 7.3A.1_1 | Keysight Technologies UK Ltd, Anritsu | agreed |  |  |
| R5-240621 | TT Formula vs MU to be added for FR1 EVM test as in FR2 | Keysight Technologies | agreed |  |  |
| R5-240622 | Message exceptions clarifications for 6.3G.3.3 | Keysight Technologies | revised |  | R5-241726 |
| R5-240623 | MBW table reference corrected for inter-band case in test case 6.5A.4.1 | Keysight Technologies | agreed |  |  |
| R5-240624 | Added 30kHz SCS for SSB in n53 to be aligned with core specs | Keysight Technologies UK Ltd, Apple | revised |  | R5-241776 |
| R5-240625 | p-Max and p-NR-FR1 adjustment when higherPowerLimit-r17 applies | Keysight Technologies UK Ltd, Mediatek | revised |  | R5-241766 |
| R5-240626 | FR2 DL RMCs - Missing notes update | Keysight Technologies | agreed |  |  |
| R5-240627 | Minimum requirements update for DC_5A_n78A | Keysight Technologies | agreed |  |  |
| R5-240628 | Modification of testcase 8.1.5.13.2 applicability clauses | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240629 | Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2 | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-240630 | Update SIB1 | Ericsson | withdrawn |  |  |
| R5-240631 | Update test case 6.7.1.3 | Ericsson | withdrawn |  |  |
| R5-240632 | Update test case 6.7.1.3 | Ericsson | withdrawn |  |  |
| R5-240633 | Addition of new TC 6.5.2 One-to-server video push call CO | NIST | revised |  | R5-241596 |
| R5-240634 | Correction to FR2 SA event triggered reporting tests with Pre-MG including TT | Anritsu | revised |  | R5-241677 |
| R5-240635 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with Pre-MG | Anritsu | revised |  | R5-241680 |
| R5-240636 | Correction to FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | revised |  | R5-241678 |
| R5-240637 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with concurrent gaps | Anritsu | revised |  | R5-241681 |
| R5-240638 | Correction to FR2 SA event triggered reporting tests with NCSG including TT | Anritsu | revised |  | R5-241679 |
| R5-240639 | Introduction of Test Tolerance analysis for SA FR2 event triggered reporting tests with NCSG | Anritsu | revised |  | R5-241682 |
| R5-240640 | PRD21 CDS PC3 NRCA CA_n1A-n41A, CA_n3A-n28A, CA_n3A-n77(2A), CA_n28A-n77(2A), CA_n3A-n28A-n77A | KDDI Corporation | withdrawn |  |  |
| R5-240641 | PRD21 CDS PC3 NRCA CA_n41A-n77A, CA_n1A-n3A-n77A, CA_n1A-n28A-n77A, CA_n1A-n41A-n77A, CA_n3A-n41A-n77A | KDDI Corporation | noted |  |  |
| R5-240642 | PRD21 CDS PC2 EN-DC DC_3A_n77A, DC_18A_n77A, DC_28A_n77A | KDDI Corporation | withdrawn |  |  |
| R5-240643 | NG.114 test coverage | Ericsson | noted |  |  |
| R5-240644 | Correction to SM NSAC Testcase 10.1.8.4 | Anritsu EMEA Ltd | agreed |  |  |
| R5-240645 | Correction to NR RRC Idle mode test case 6.4.2.3 | Anritsu EMEA Ltd | revised |  | R5-241496 |
| R5-240646 | Correction to NR RRC Idle mode test case 6.1.2.24 | Anritsu EMEA Ltd | revised |  | R5-241497 |
| R5-240647 | Editorial correction to the Scope description | FirstNet, NIST | agreed |  |  |
| R5-240648 | Updates to Ephemeris Info for Multi-Cell NRN-NTN signalling test cases | QUALCOMM JAPAN LLC. | revised |  | R5-241561 |
| R5-240649 | Updates to SIB19 Common Config for NR-NTN test cases | QUALCOMM JAPAN LLC | revised |  | R5-241565 |
| R5-240650 | Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | revised |  | R5-241566 |
| R5-240651 | Updates to cell configuration for NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" test case | QUALCOMM JAPAN LLC. | revised |  | R5-241567 |
| R5-240652 | Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | withdrawn |  |  |
| R5-240653 | Addition of NTN / Mobility registration update / supported TACs not part of UE registration area | QUALCOMM JAPAN LLC. | revised |  | R5-241568 |
| R5-240654 | Applicability updates for NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | withdrawn |  |  |
| R5-240655 | Applicability updates for NTN / Mobility registration update / supported TACs not part of UE registration area | QUALCOMM JAPAN LLC. | withdrawn |  |  |
| R5-240656 | Correction and updates to NR TC 7.1.1.10.2 | Qualcomm CDMA Technologies | revised |  | R5-241508 |
| R5-240657 | Corrections to NR eDRX test case 11.7.1 | Qualcomm CDMA Technologies, ANRITSU LTD, Keysight Technologies | agreed |  |  |
| R5-240658 | Corrections to UAS test case 10.1.4.3 | Qualcomm CDMA Technologies | revised |  | R5-241606 |
| R5-240659 | Introduction of new NR NTN RRC TC for Event D1 | Qualcomm CDMA Technologies, | revised |  | R5-241569 |
| R5-240660 | Applicability of New NR NTN TC for Event D1 | Qualcomm CDMA Technologies | revised |  | R5-241652 |
| R5-240661 | Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI) | Qualcomm CDMA Technologies | withdrawn |  |  |
| R5-240662 | Corrections to applicability of Network Slice Admission Control (NSAC) mobility management aspects test cases | Qualcomm CDMA Technologies | withdrawn |  |  |
| R5-240663 | Discussion Paper for Rel-17 eNS_Ph2 WI tests case applicability | Qualcomm CDMA Technologies | revised |  | R5-241479 |
| R5-240664 | Way Forward on Machine Readable PICS | Qualcomm CDMA Technologies | revised |  | R5-241574 |
| R5-240665 | SR UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems | Qualcomm CDMA Technologies | noted |  |  |
| R5-240666 | SR UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state | Qualcomm CDMA Technologies | noted |  |  |
| R5-240667 | WP UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems | Qualcomm CDMA Technologies | noted |  |  |
| R5-240668 | WP UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state | Qualcomm CDMA Technologies | noted |  |  |
| R5-240669 | SIG WP for TEI16 - Redirection with MPS Indication | Qualcomm CDMA Technologies | reserved |  |  |
| R5-240670 | Rel-15 5GS maintenance SIG WP (NR-SA and ENDC) | Qualcomm CDMA Technologies | reserved |  |  |
| R5-240671 | Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking | Qualcomm Technologies Ireland | withdrawn |  |  |
| R5-240672 | Addition of new PICS for NTN Event D1 | Qualcomm CDMA Technologies | withdrawn |  |  |
| R5-240673 | Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI) | Qualcomm CDMA Technologies | revised |  | R5-241631 |
| R5-240674 | Addition of reference sensitivity TP analysis for new PC2 EN-DC combos within FR1 | KDDI Corporation | agreed |  |  |
| R5-240675 | PRD21 CDS PC3 EN-DC DC_18A_n257G | KDDI Corporation | withdrawn |  |  |
| R5-240676 | Correction to default configuration of SMTC for NCD-SSB | Huawei, HiSilicon, Starpoint | revised |  | R5-242038 |
| R5-240677 | Addition of RedCap demod test case 6.3.1.1.1 | Huawei, HiSilicon | revised |  | R5-241960 |
| R5-240678 | Addition of RedCap demod test case 6.3.1.2.1 | Huawei, HiSilicon | revised |  | R5-241817 |
| R5-240679 | Addition of RedCap demod test case 6.3.2.2.7 | Huawei, HiSilicon | revised |  | R5-241961 |
| R5-240680 | Addition of applicability for RedCap Demod and RRM test cases | Huawei,HiSilicon | withdrawn |  |  |
| R5-240681 | Addition of RedCap RRM test case 17.5.3.1.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240682 | Addition of RedCap RRM test case 17.5.3.2.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240683 | Addition of RedCap RRM test case 17.5.5.1.1 | Huawei,HiSilicon | agreed |  |  |
| R5-240684 | Addition of RedCap RRM test case 17.5.5.2.1 | Huawei,HiSilicon | agreed |  |  |
| R5-240685 | Addition of RedCap RRM test case 17.5.6.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240686 | Addition of RedCap RRM test case 17.7.1.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240687 | Addition of RedCap RRM test case 17.7.1.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240688 | Addition of RedCap RRM test case 17.7.2.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240689 | Addition of RedCap RRM test case 17.7.2.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240690 | Addition of RedCap RRM test case 17.7.3.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240691 | Addition of RedCap RRM test case 17.7.3.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240692 | Addition of RedCap RRM test case 17.7.4.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240693 | Correction to NCD-SSB and SMTC RMCs | Huawei, HiSilicon, Starpoint | revised |  | R5-242039 |
| R5-240694 | Correction to RedCap inter-frequency measurement test cases | Huawei, HiSilicon, Starpoint, Anritsu, Keysight | revised |  | R5-242040 |
| R5-240695 | Correction to RedCap RRM test case 16.5.2.3 | Huawei,HiSilicon,Starpoint | agreed |  |  |
| R5-240696 | Correction to RedCap RRM test case 16.5.4.1 and 16.5.4.2 | Huawei,HiSilicon,Starpoint | agreed |  |  |
| R5-240697 | Correction to RedCap RRM test case 17.4.1.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240698 | Correction to RedCap RRM test case 17.6.4.1 | Huawei,HiSilicon | agreed |  |  |
| R5-240699 | Correction to SSB time offset in NCD-SSB test cases | Huawei, HiSilicon, Starpoint | agreed |  |  |
| R5-240700 | Correction to RRM enh test case 6.5.8.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240701 | Correction to RRM enh test case 6.6.2.9 | Huawei,HiSilicon | revised |  | R5-241829 |
| R5-240702 | Correction to EN-DC and NR SA inter-frequency measurement test cases | Huawei,HiSilicon,Starpoint | withdrawn |  |  |
| R5-240703 | Correction to EN-DC and NR SA SCell activation test cases | Huawei,HiSilicon,Starpoint | revised |  | R5-241994 |
| R5-240704 | Correction to PCI updating formulas in RRM test cases | Huawei, HiSilicon, Starpoint | revised |  | R5-241995 |
| R5-240705 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | revised |  | R5-241670 |
| R5-240706 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | revised |  | R5-241671 |
| R5-240707 | Correction to RedCap RRM test cases 16.3.2.2.X with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240708 | Correction to RedCap RRM test case 16.6.5.3 with TT | Huawei, HiSilicon | revised |  | R5-241886 |
| R5-240709 | Correction to RedCap RRM test case 16.6.5.4 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240710 | Correction to RedCap RRM test case 16.7.2.3.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240711 | Correction to RedCap RRM test case 16.7.2.3.2 with TT | Huawei,HiSilicon | revised |  | R5-242041 |
| R5-240712 | Correction to RedCap RRM test case 16.7.2.4.1 with TT | Huawei, HiSilicon | revised |  | R5-242042 |
| R5-240713 | Correction to RedCap RRM test case 16.7.2.4.2 with TT | Huawei,HiSilicon | revised |  | R5-242043 |
| R5-240714 | Correction to RedCap RRM test case 16.7.3.1 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240715 | Correction to RedCap RRM test case 16.7.3.2 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240716 | Correction to RedCap RRM test case 16.7.3.3.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240717 | Correction to RedCap RRM test case 16.7.3.3.2 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240718 | Correction to RedCap RRM test case 16.7.3.4.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240719 | Correction to RedCap RRM test case 16.7.3.4.2 with TT | Huawei,HiSilicon | revised |  | R5-241963 |
| R5-240720 | Correction to RedCap RRM test case 16.7.4.1.1 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240721 | Correction to RedCap RRM test case 16.7.4.1.2 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240722 | Correction to RedCap RRM test case 16.7.4.2.1 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240723 | Correction to RedCap RRM test case 16.7.4.2.2 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240724 | Correction to RedCap RRM test case 16.7.4.3.1 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240725 | Correction to RedCap RRM test case 16.7.4.3.2 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240726 | Correction to RedCap RRM test case 16.7.4.4.1 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240727 | Correction to RedCap RRM test case 16.7.4.4.2 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240728 | Correction to RedCap RRM test case 16.7.5.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240729 | Correction to RedCap RRM test case 16.7.5.2 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240730 | Correction to RedCap RRM test case 16.7.6.1 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240731 | Correction to RedCap RRM test case 16.7.6.2 with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240732 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | revised |  | R5-241878 |
| R5-240733 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | withdrawn |  |  |
| R5-240734 | TT analysis for RedCap RRM test case 16.2.1.2 | Huawei, HiSilicon | withdrawn |  |  |
| R5-240735 | TT analysis for RedCap RRM test case 16.3.2.2.2 and 16.3.2.2.4 | Huawei, HiSilicon | agreed |  |  |
| R5-240736 | TT analysis for RedCap RRM test case 16.3.2.2.6 and 16.3.2.2.8 | Huawei, HiSilicon | agreed |  |  |
| R5-240737 | TT analysis for RedCap RRM test case 16.6.5.3 | Huawei, HiSilicon | agreed |  |  |
| R5-240738 | TT analysis for RedCap RRM test case 16.6.5.4 | Huawei, HiSilicon | agreed |  |  |
| R5-240739 | TT analysis for RedCap RRM test case 16.7.2.3.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240740 | TT analysis for RedCap RRM test case 16.7.2.3.2 | Huawei, HiSilicon | revised |  | R5-242044 |
| R5-240741 | TT analysis for RedCap RRM test case 16.7.2.4.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240742 | TT analysis for RedCap RRM test case 16.7.2.4.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240743 | TT analysis for RedCap RRM test case 16.7.3.1 and 16.7.3.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240744 | TT analysis for RedCap RRM test case 16.7.3.3.1 | Huawei, HiSilicon | revised |  | R5-241887 |
| R5-240745 | TT analysis for RedCap RRM test case 16.7.3.3.2 | Huawei, HiSilicon | revised |  | R5-241888 |
| R5-240746 | TT analysis for RedCap RRM test case 16.7.3.4.1 | Huawei, HiSilicon | revised |  | R5-241889 |
| R5-240747 | TT analysis for RedCap RRM test case 16.7.3.4.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240748 | TT analysis for RedCap RRM test case 16.7.4.1.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240749 | TT analysis for RedCap RRM test case 16.7.4.1.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240750 | TT analysis for RedCap RRM test case 16.7.4.2.1 16.7.4.3.1 and 16.7.4.4.1 | Huawei, HiSilicon | revised |  | R5-241890 |
| R5-240751 | TT analysis for RedCap RRM test case 16.7.4.2.2 16.7.4.3.2 and 16.7.4.4.2 | Huawei, HiSilicon | revised |  | R5-241891 |
| R5-240752 | TT analysis for RedCap RRM test case 16.7.5.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240753 | TT analysis for RedCap RRM test case 16.7.5.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240754 | TT analysis for RedCap RRM test case 16.7.6.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240755 | TT analysis for RedCap RRM test case 16.7.6.2 | Huawei, HiSilicon | agreed |  |  |
| R5-240756 | Correction to SDT RRM test case 6.2.1 with TT | Huawei,HiSilicon | revised |  | R5-241673 |
| R5-240757 | Correction to Annex F for SDT RRM test cases | Huawei,HiSilicon | withdrawn |  |  |
| R5-240758 | Correction to FR1 4-step RACH test cases with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240759 | Correction to FR1 2-step RACH test cases with TT | Huawei,HiSilicon | agreed |  |  |
| R5-240760 | Correction to FR1 inter frequency SS SINR relative accuracy test cases with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240761 | Correction to FR1 L1 RSRP absolute accuracy test cases with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240762 | Correction to FR1 LTE RSRP accuracy test case 6.7.5.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-240763 | Correction to Annex F for R15 RRM test cases | Huawei,HiSilicon | withdrawn |  |  |
| R5-240764 | Correction to default configuration of frequencyInfoSL | Huawei, HiSilicon | revised |  | R5-241539 |
| R5-240765 | Correction to V2X SIG test case 12.1.7.1 | Huawei,HiSilicon | withdrawn |  |  |
| R5-240766 | Addition of V2X SIG test case 12.2.9.1 | Huawei, HiSilicon | agreed |  |  |
| R5-240767 | Correction of applicability for V2X SIG test cases | Huawei, HiSilicon | revised |  | R5-241540 |
| R5-240768 | SR of Rel-16 V2X WI after RAN5 102 | Huawei,HiSilicon | noted |  |  |
| R5-240769 | WP of Rel-16 V2X WI after RAN5 102 | Huawei,HiSilicon | noted |  |  |
| R5-240770 | Correction to applicability for handover test cases | TTA | agreed |  |  |
| R5-240771 | Correction to applicability for NR-U test cases | TTA | agreed |  |  |
| R5-240772 | Addition of new test case 7.4A.4 Maximum input level for CA (5DL CA) | KTL | agreed |  |  |
| R5-240773 | Addition of new test case 7.5A.4 Adjacent channel selectivity for (5DL CA) | KTL | agreed |  |  |
| R5-240774 | PRD21 CDS: NR CA PC3 FR1 CA_n1A-n28A-n78A and CA_n3A-n28A-n78A | Nokia, Nokia Shanghai Bell | noted |  |  |
| R5-240775 | Introduction of test frequencies for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240776 | Introduction of reference sensitivity test point analysis for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240777 | Adding Reference sensitivity test requirements for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240778 | Introduction of spurious emission TP analysis for CA_n1A_n28A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240779 | Add UE maximum power requirements for CA_n1A-n28A and CA_n3A-n28A | Nokia, Nokia Shanghai Bell | revised |  | R5-241729 |
| R5-240780 | Add spurious emissions for UE co-existence requirements for CA_n1A_n28A | Nokia, Nokia Shanghai Bell | revised |  | R5-241730 |
| R5-240781 | Revised WID: UE Conformance - NR sidelink enhancement | CATT | agreed |  |  |
| R5-240782 | Revised WID: UE Conformance - NR Sidelink Relay | CATT | agreed |  |  |
| R5-240783 | Removal of NOTE 1 from applicability of FR1 and FR2 MRDC test cases | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-240784 | WP UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) | Ericsson | noted |  |  |
| R5-240785 | SR UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) | Ericsson | noted |  |  |
| R5-240786 | FR2 RRM test cases: Known Issue List | Ericsson | revised |  | R5-241867 |
| R5-240787 | Addition of Test Tolerance for NR SA Event triggered reporting RedCap test cases | Ericsson | agreed |  |  |
| R5-240788 | Test Tolerances for NR SA FR1 - E-UTRAN event-triggered reporting tests for 1 Rx RedCap UE | Ericsson | agreed |  |  |
| R5-240789 | Test Tolerances for NR SA FR1 - E-UTRAN event-triggered reporting tests for 2 Rx RedCap UE | Ericsson | agreed |  |  |
| R5-240790 | Addition of Test Tolerance into Annex F for RedCap test cases | Ericsson | agreed |  |  |
| R5-240791 | Addition of event triggered reporting test cases applicability | Ericsson | revised |  | R5-241849 |
| R5-240792 | Addition of applicability of the NR-U RRM test cases | Ericsson | revised |  | R5-241702 |
| R5-240793 | Correction of Test tolerance for FR2 Inter frequency test case | Ericsson | agreed |  |  |
| R5-240794 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE test case 16.6.3.1 including Test Tolerance | Ericsson | revised |  | R5-242033 |
| R5-240795 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE test case 16.6.3.2 including Test Tolerance | Ericsson | revised |  | R5-242034 |
| R5-240796 | Addition of NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE test case 16.6.3.3 including Test Tolerance | Ericsson | revised |  | R5-242035 |
| R5-240797 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2Rx UE test case 16.6.3.4 including Test Tolerance | Ericsson | revised |  | R5-242036 |
| R5-240798 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.1 including Test Tolerance | Ericsson | agreed |  |  |
| R5-240799 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.2 including Test Tolerance | Ericsson | agreed |  |  |
| R5-240800 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.3 including Test Tolerance | Ericsson | agreed |  |  |
| R5-240801 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.4 including Test Tolerance | Ericsson | agreed |  |  |
| R5-240802 | Addition of cell configuration mapping for event triggered measurement test cases for RedCap | Ericsson | agreed |  |  |
| R5-240803 | Addition of NR_U test case 11.5.1.1 | Ericsson | agreed |  |  |
| R5-240804 | Addition of NR_U test case 11.5.1.2 | Ericsson | agreed |  |  |
| R5-240805 | Addition of NR_U test case 11.5.1.3 | Ericsson | agreed |  |  |
| R5-240806 | Addition of NR_U test case 11.5.1.4 | Ericsson | agreed |  |  |
| R5-240807 | Addition of NR_U test case 11.5.1.9 | Ericsson | agreed |  |  |
| R5-240808 | Addition of NR_U test case 11.5.2.3 | Ericsson | revised |  | R5-241827 |
| R5-240809 | Addition of NR_U test case 11.5.2.4 | Ericsson | agreed |  |  |
| R5-240810 | Updates to NR-U Tx test cases | Ericsson | revised |  | R5-241739 |
| R5-240811 | Addition of NR_U test case 11.5.2.5 | Ericsson | agreed |  |  |
| R5-240812 | Addition of NR_U test case 11.5.2.6 | Ericsson | agreed |  |  |
| R5-240813 | Addition of NR_U test case 11.5.2.7 | Ericsson | agreed |  |  |
| R5-240814 | Addition of NR_U test case 11.5.2.8 | Ericsson | agreed |  |  |
| R5-240815 | Addition of NR_U test case 11.5.2.9 | Ericsson | agreed |  |  |
| R5-240816 | Addition of NR_U test case 11.5.2.10 | Ericsson | agreed |  |  |
| R5-240817 | Addition of test case 6.3F.4.3 relative power tolerance for shared spectrum channel access | Ericsson | agreed |  |  |
| R5-240818 | Correction to NBIOT NTN testcase 22.4.13a | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-240819 | Addition of MU and TT for NR-U test cases | Ericsson | agreed |  |  |
| R5-240820 | Correction to NBIOT testcase 22.4.13 | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-240821 | Updates of test case 6.2F.4 Configured transmitted power for shared spectrum access | Ericsson | agreed |  |  |
| R5-240822 | Update IE FeatureCombination | Ericsson | withdrawn |  |  |
| R5-240823 | Add IE HysteresisAltitude | Ericsson | revised |  | R5-241486 |
| R5-240824 | Add IEs LTM-CandidateId, LTM-Candidate, LTM-Config and LTM-CSI-ReportConfig | Ericsson | revised |  | R5-241487 |
| R5-240825 | Add IEs LTM-CSI-ReportConfigId, LTM-CSI-ResourceConfig and LTM-CSI-ResourceConfigId | Ericsson | revised |  | R5-241488 |
| R5-240826 | Editorial, correcting Test case title for 7.3I.2 in Annex A | Ericsson | agreed |  |  |
| R5-240827 | Correction of TT for RedCap demod test case 5.3.2.1.4 and 5.3.2.2.4 | Ericsson | agreed |  |  |
| R5-240828 | Addition of RMC tables for shared spectrum access in Annex A | Ericsson | agreed |  |  |
| R5-240829 | Addition of test case 6.2A.4.1.1, 4Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA) | Ericsson | withdrawn |  |  |
| R5-240830 | Core spec alignment, addition of RMC and UL-DL configuration for NR-U | Ericsson | revised |  | R5-241822 |
| R5-240831 | Addition of applicability for RedCap test case 6.2.2.1.2.4 | Ericsson | agreed |  |  |
| R5-240832 | PRD20 on E-UTRA CA configuration handling in RAN5 v1.6.0 | ZTE Corporation | approved |  |  |
| R5-240833 | Update PRD20 E-UTRA CA list v160 | ZTE Corporation | agreed |  |  |
| R5-240834 | PRD21 CDS NR CA PC3 FR1 CA_n1A-n3A-n78A | ZTE, China Telecom | noted |  |  |
| R5-240835 | Update of 6.3C.1 for minimum output power for SUL bands | ZTE Corporation | agreed |  |  |
| R5-240836 | Update of 6.3C.2 for transmit OFF power for SUL bands | ZTE Corporation | agreed |  |  |
| R5-240837 | Corrections on 6.2.3 for FR2 Redcap UE MPR test case for NS_202 and NS_203 | ZTE Corporation | revised |  | R5-241781 |
| R5-240838 | Corrections on 6.3.1 for FR2 Redcap UE minimum output power | ZTE Corporation | agreed |  |  |
| R5-240839 | Corrections on 4.6.2 and 4.6.3 for the conditions in SIB and RRC IE | ZTE Corporation | revised |  | R5-241809 |
| R5-240840 | Corrections on 5.3.3 for minimum guardband and transmission bandwidth configuration | ZTE Corporation | agreed |  |  |
| R5-240841 | Corrections on 6.2.2 for maximum output power reduction | ZTE Corporation | agreed |  |  |
| R5-240842 | Corrections on 6.2.3 for additional maximum output power reduction | ZTE Corporation | revised |  | R5-241805 |
| R5-240843 | Corrections on 6.3.2 for Transmit OFF power | ZTE Corporation | agreed |  |  |
| R5-240844 | Corrections on 6.3.3 for Transmit ON OFF time mask | ZTE Corporation | revised |  | R5-241806 |
| R5-240845 | Corrections on 6.4.1 for Frequency error | ZTE Corporation | withdrawn |  |  |
| R5-240846 | Corrections on 6.5.2 for out of band emission | ZTE Corporation | agreed |  |  |
| R5-240847 | Corrections on 6.5.3 for spurious emission | ZTE Corporation | revised |  | R5-241807 |
| R5-240848 | Corrections on 6.5.4 for transmit intermodulation | ZTE Corporation | withdrawn |  |  |
| R5-240849 | Corrections on 7.3.2 for Reference sensitivity power level | ZTE Corporation | revised |  | R5-241877 |
| R5-240850 | Corrections on 7.4 for maximum input level | ZTE Corporation | agreed |  |  |
| R5-240851 | Corrections on 7.5 for adjacent channel selectivity | ZTE Corporation | agreed |  |  |
| R5-240852 | Corrections on 7.6 for blocking characteristics | ZTE Corporation | agreed |  |  |
| R5-240853 | Corrections on A.3.2.1.1 for the reference channel for NTN PDSCH requirement | ZTE Corporation | agreed |  |  |
| R5-240854 | Update of 6.2.1 for maximum output power | ZTE Corporation | agreed |  |  |
| R5-240855 | Update of chapter 4 for RF general description | ZTE Corporation | revised |  | R5-241810 |
| R5-240856 | Corrections on C.1 for the parameters for calculation of test frequencies | ZTE Corporation | agreed |  |  |
| R5-240857 | Addition of applicability for NR-U test cases | Ericsson | agreed |  |  |
| R5-240858 | Addition of TP analysis for NR-U test case 6.3F.4.3 | Ericsson | agreed |  |  |
| R5-240859 | Addition of TP analysis for NR-U test case 6.2F.4, Configured transmitted power for shared spectrum | Ericsson | agreed |  |  |
| R5-240860 | Update SNR conditions in UDP and TCP FR2 FRC scenarios | Keysight Technologies | agreed |  |  |
| R5-240861 | Update for additional NR-CA and NR-DC band configurations | Verizon | revised |  | R5-241928 |
| R5-240862 | Clarification on NPDSCH repetitions for Demod NB-IoT NTN test cases | Keysight Technologies | agreed |  |  |
| R5-240863 | Update for additional NR-DC band configurations | Verizon Spain | revised |  | R5-241709 |
| R5-240864 | Addition of Measurement uncertainty for Redcap PDSCH demodulation test cases | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240865 | Discussion on handling R16/R17 features for RedCap UE | Huawei, Hisilicon | noted |  |  |
| R5-240866 | Correction to 34229-5 A.4.2a | Huawei, Hisilicon | withdrawn |  |  |
| R5-240867 | Correction to NR TC 8.1.6.1.3.8 | Huawei, Hisilicon | revised |  | R5-241641 |
| R5-240868 | Correction to NR TC 8.1.6.1.3.9 | Huawei, Hisilicon | revised |  | R5-241642 |
| R5-240869 | Revised WID on UE Conformance Test Aspects - Rel-17 IMS Data Channel | Huawei | revised |  | R5-241662 |
| R5-240870 | TS 38.523-1 Tracker status before RAN5-102 | Huawei, Hisilicon | noted |  |  |
| R5-240871 | TS 36.523-1 Tracker status before RAN5-102 | Huawei, Hisilicon | noted |  |  |
| R5-240872 | TS 38.523-1 Tracker status after RAN5-102 | Huawei, Hisilicon | reserved |  |  |
| R5-240873 | TS 36.523-1 Tracker status after RAN5-102 | Huawei, Hisilicon | reserved |  |  |
| R5-240874 | SR of UE Conformance - IMS Data Channel test after RAN5 102 | Huawei | noted |  |  |
| R5-240875 | WP of UE Conformance - IMS Data Channel test after RAN5 102 | Huawei | noted |  |  |
| R5-240876 | Update CA bandwidth classes table | Verizon | withdrawn |  |  |
| R5-240877 | Update on test case 7.5.1 to add FR2 testing direction | Keysight Technologies | agreed |  |  |
| R5-240878 | Update NR inter-band CA configurations FR1 four bands table | Verizon Spain | agreed |  |  |
| R5-240879 | Addition of general spurious emissions test for EN-DC combos DC_2A_n66A and DC_30A_n66A | WE Certification, AT&T | agreed |  |  |
| R5-240880 | Update reference sensitivity requirement for 4DL CA table | Verizon | withdrawn |  |  |
| R5-240881 | Test point analysis of EN-DC spurious emissions for DC_2A_n66A and DC_30A_n66A | WE Certification Oy, AT&T | revised |  | R5-241778 |
| R5-240882 | Addition of test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification Oy, AT&T | revised |  | R5-241705 |
| R5-240883 | Update reference sensitivity test cases for four bands configurations | Verizon | revised |  | R5-241701 |
| R5-240884 | Addition of many EN-DC PC2 combos to Ch 5 | WE Certification Oy, AT&T | agreed |  |  |
| R5-240885 | Addition of test requirements for EN-DC PC2 combos DC_12A_n77A, DC_14A_n77A and DC_30A_n77A max power test | WE Certification, AT&T | agreed |  |  |
| R5-240886 | Update additional inter-band NR CA configurations | Verizon | revised |  | R5-241708 |
| R5-240887 | Update additional FR1 NR CA inter-band band configurations | Verizon | agreed |  |  |
| R5-240888 | Update reference sensitivity requirement table for additional band configurations | Verizon | agreed |  |  |
| R5-240889 | Update to RRM Power saving enhancement 5.5.5.9 test case applicability | Keysight Technologies | revised |  | R5-241850 |
| R5-240890 | Removal of Editor Note affecting to TC 5.5.5.9 | Keysight Technologies | revised |  | R5-241826 |
| R5-240891 | Update reference sensitivity test cases for additional band configurations | Verizon | agreed |  |  |
| R5-240892 | Update for additional band configurations with PC2 UL | Verizon Spain | revised |  | R5-241714 |
| R5-240893 | Addition of CA_n14A-n77A PC2 and CA_n77(2A) PC2 to Ch 5 | WE Certification Oy, AT&T | revised |  | R5-241747 |
| R5-240894 | Addition of PC2 max power requirements for bands CA_n77(2A) and CA_n14A-n77A | WE Certification Oy, AT&T | agreed |  |  |
| R5-240895 | Update reference sensitivity test cases for additional band configurations with PC2 UL | Verizon Spain | revised |  | R5-241748 |
| R5-240896 | Addition of band CA_n14A-n77A PC2 reference sensitivity test | WE Certification, AT&T | revised |  | R5-241746 |
| R5-240897 | Corrections to Table 5.5.3.3.1-3 MCData Resource-lists | NIST | agreed |  |  |
| R5-240898 | Addition of Location-info for MCData | NIST | agreed |  |  |
| R5-240899 | Corrections to Forward, Scope, References, and Definitions | NIST | withdrawn |  |  |
| R5-240900 | Addition on new TC 6.2 MCPTT Server - MCPTT Client - Chat Group Call | NIST | withdrawn |  |  |
| R5-240901 | Addition of applicability for new Rel-16 test cases | NIST | revised |  | R5-241685 |
| R5-240902 | Addition of new PIXITs for REL-16 TCs | NIST | revised |  | R5-241686 |
| R5-240903 | Addition of new TC 6.4.1 Emergency Alert CO | NIST | revised |  | R5-241593 |
| R5-240904 | Addition of new TC 6.4.2 Emergency Alert CT | NIST | revised |  | R5-241594 |
| R5-240905 | Addition of new TC 5.1 Configuration | NIST | withdrawn |  |  |
| R5-240906 | Addition of new TC 6.1 Group Call | NIST | withdrawn |  |  |
| R5-240907 | Addition of new TC 5.1 Configuration | NIST | withdrawn |  |  |
| R5-240908 | Addition of new TC 6.1 One-to-one standalone SDS using the signaling plane | NIST | withdrawn |  |  |
| R5-240909 | Addition of new TC 6.9.1 Location Information Request | NIST | agreed |  |  |
| R5-240910 | Addition of new TC 6.9.2 Report triggering | NIST | agreed |  |  |
| R5-240911 | Addition of new TC 6.1.1.14 Re-join | NIST | revised |  | R5-241595 |
| R5-240912 | SR Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData) | NIST | noted |  |  |
| R5-240913 | WP Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData) | NIST | noted |  |  |
| R5-240914 | Update for additional ENDC band configurations with PC2 UL | Verizon | revised |  | R5-241720 |
| R5-240915 | Typo correction to SSB and SMTC configuration for RedCap Event Trigger TC 16.6.2.1 | Keysight Technologies UK Ltd | withdrawn |  |  |
| R5-240916 | Update reference sensitity minimum requirement for additional ENDC band configurations with PC2 UL | Verizon Spain | withdrawn |  |  |
| R5-240917 | Update reference sensitity test configuration and test requirement tables for ENDC band configurations with PC2 UL | Verizon Spain | withdrawn |  |  |
| R5-240918 | Resubmission of Editorial update test case 8.1.1.1a.3 | ETSI MCC (Ericsson) | agreed |  |  |
| R5-240919 | Editorial correction to supported EN-DC configuration table | WE Certification | revised |  | R5-241710 |
| R5-240920 | Cell Mapping correction for TC 6.5.6.1.1 in Annex E | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240921 | Cell Mapping correction for TC 6.6.1.8 in Annex E | Keysight Technologies | agreed |  |  |
| R5-240922 | Editorial update for UE timing advance for satellite access TC 13.4.2.2 | Keysight Technologies UK Ltd | agreed |  |  |
| R5-240923 | TS 36.579-8 v0.1.0 | NIST | withdrawn |  |  |
| R5-240924 | TS 36.579-9 v0.1.0 | NIST | withdrawn |  |  |
| R5-240925 | PRD-17 on Guidance to Work Item Codes (post RAN#103 version) | Bureau Veritas ADT (Rapporteur) | approved |  |  |
| R5-240926 | PRD20: Completed CA_2A-2A-29A-30A-66A_5DL-1UL without UL CA | Bureau Veritas ADT, Sporton International | withdrawn |  |  |
| R5-240927 | PRD20: Completed CA_2A-2A-29A-66A-66A_5DL-1UL without UL CA | Bureau Veritas ADT, Sporton International | withdrawn |  |  |
| R5-240928 | PRD20: Completed WP_CA_2A-29A-66A-66A_CA_2A-2A-29A-66A_4DL-1UL_DL only FBs | Bureau Veritas ADT, Sporton International | withdrawn |  |  |
| R5-240929 | PRD20: Completed CA_30A-48A_2DL-1UL without UL CA | Bureau Veritas ADT, Sporton International | noted |  |  |
| R5-240930 | PRD20: Completed CA_30A-48A 2DL-2UL with UL CA_30A-48A | Bureau Veritas ADT, Sporton International | withdrawn |  |  |
| R5-240931 | RAN5#102 summary of changes to RAN5 test cases with potential impact on GCF and PTCRB | Bureau Veritas ADT | reserved |  |  |
| R5-240932 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Rohde&Schwarz | agreed |  |  |
| R5-240933 | Update to R15 Configuration for DC | Bureau Veritas ADT | withdrawn |  |  |
| R5-240934 | Update to R16 Configuration for DC | Bureau Veritas ADT, KDDI, WE Certification, AT&T | agreed |  |  |
| R5-240935 | Update to R17 Configuration for DC | Bureau Veritas ADT | withdrawn |  |  |
| R5-240936 | Update to R18 Configuration for DC | Bureau Veritas ADT | withdrawn |  |  |
| R5-240937 | Additional supported capabilities for multiple CA combos | Bureau Veritas ADT, Sporton International | agreed |  |  |
| R5-240938 | Additional supported capabilities for multiple CA combos | Bureau Veritas ADT, Sporton International, CATT | agreed |  |  |
| R5-240939 | Additional CA_30A-48A for CA signalling test | Bureau Veritas ADT, Sporton International, CATT | agreed |  |  |
| R5-240940 | Update to CBW for multiples CA combos | Bureau Veritas ADT, Sporton International | revised |  | R5-241853 |
| R5-240941 | Update to Transmit test cases for CA_30A-48A | Bureau Veritas ADT, Sporton International | agreed |  |  |
| R5-240942 | Update to Receiver test cases for multiple CA combos | Bureau Veritas ADT, Sporton International | revised |  | R5-241874 |
| R5-240943 | PRD21 CDS: NR CA 4 bands PC3 FR1 | Verizon Spain | noted |  |  |
| R5-240944 | Time synchronization required for UE beamlock function during initial access | Apple AB Denmark | noted |  |  |
| R5-240945 | PRD21 CDS: PC3 NRCA 3T | Verizon Spain | noted |  |  |
| R5-240946 | PRD21 CDS: PC3 NRCA 1F+3T | Verizon Spain | noted |  |  |
| R5-240947 | PRD21 CDS: PC3 NRCA 1F+n48(2A) | Verizon Spain | noted |  |  |
| R5-240948 | PRD21 CDS: PC3 NRCA 2F+n77C | Verizon Spain | noted |  |  |
| R5-240949 | PRD21 CDS: PC2 EN-DC FR1 3 bands and 4 bands | Verizon Spain | withdrawn |  |  |
| R5-240950 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD | revised |  | R5-241526 |
| R5-240951 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | revised |  | R5-241530 |
| R5-240952 | Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking | Qualcomm Technologies Ireland | revised |  | R5-241925 |
| R5-240953 | PRD21 CDS: PC2 NRCA 2, 3, 4 bands | Verizon Spain | noted |  |  |
| R5-240954 | SRB4 | Ericsson | noted |  |  |
| R5-240955 | Update to NTN Frequency Error TC | Qualcomm Technologies Ireland | withdrawn |  |  |
| R5-240956 | Update to NTN Add Spurious Emission TC | Qualcomm Technologies Ireland | revised |  | R5-242008 |
| R5-240957 | Update to NTN General Spurious emission TC | Qualcomm Technologies Ireland | revised |  | R5-242009 |
| R5-240958 | Updates to NTN Spur emission UE Coex | Qualcomm Technologies Ireland | revised |  | R5-242010 |
| R5-240959 | Update to NTN Tx Intermod TC | Qualcomm Technologies Ireland, ZTE | revised |  | R5-242011 |
| R5-240960 | Update to NTN RefSens TC | Qualcomm Technologies Ireland | withdrawn |  |  |
| R5-240961 | Update to NTN Annex F MU TT | Qualcomm Technologies Ireland | revised |  | R5-242012 |
| R5-240962 | Update to FR2 ACS TC | Qualcomm Technologies Ireland | agreed |  |  |
| R5-240963 | Correct Table 7.3A.11.4.1-1 of CA Configurations | SGS Wireless | revised |  | R5-241854 |
| R5-240964 | Correct Table 7.4A.8.4.1-2 & Table 7.4A.8_H.4.1-2 & Table 7.4A.9.4.1-1 of CA Configurations | SGS Wireless | revised |  | R5-241855 |
| R5-240965 | Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations | SGS Wireless | revised |  | R5-241856 |
| R5-240966 | Correct typos in Table 7.3A.5.4.1-1 | SGS Wireless | revised |  | R5-241857 |
| R5-240967 | Correct clause 6.2B.4.1.2 initial conditions | SGS Wireless | revised |  | R5-241789 |
| R5-240968 | Correct Table 4.3.2.2.1.4.1-2 & Table 4.3.2.2.2.4.1-2 & Table 4.3.2.2.3.4.1-2 & Table 4.3.2.2.4.4.1-2 of test frequency | SGS Wireless | revised |  | R5-241833 |
| R5-240969 | Update to freq error to refer to cat 1bis refsens values table | Qualcomm Technologies Ireland | withdrawn |  |  |
| R5-240970 | Test frequencies calculation review for asymmetric channel bandwidths | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA | noted |  |  |
| R5-240971 | Annex C update to asymmetric channel bandwidths test frequencies calculation | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA | agreed |  |  |
| R5-240972 | Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8 | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA | revised |  | R5-241703 |
| R5-240973 | Addition of conditional PSCell addition and change configuration for EN-DC | Huawei, HiSilicon | revised |  | R5-241712 |
| R5-240974 | Addition of SCG activation and deactivation configuration for EN-DC | Huawei, HiSilicon | revised |  | R5-241713 |
| R5-240975 | Removal of GEA2 support from inter-rat cell change order test cases | Qualcomm Incorporated | agreed |  |  |
| R5-240976 | Removal of GEA2 support from applicability of inter-rat cell change order test cases | Qualcomm Incorporated | agreed |  |  |
| R5-240977 | Addition of new L2L MPS priority access barring test case | Qualcomm Incorporated | revised |  | R5-241475 |
| R5-240978 | Addition of applicability for L2L MPS priority access barring test case | Qualcomm Incorporated | agreed |  |  |
| R5-240979 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | revised |  | R5-241563 |
| R5-240980 | Correction to MAC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | revised |  | R5-241564 |
| R5-240981 | Correction to RLC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | revised |  | R5-241647 |
| R5-240982 | Addition of legacy test cases applicable to NTN only UEs in GSO and NGSO | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | withdrawn |  |  |
| R5-240983 | PICS clarification and applicability updates for NTN test cases | Qualcomm Incorporated, Rohde & Schwarz | revised |  | R5-241648 |
| R5-240984 | Addition of NR NTN TA reporting PICS | Qualcomm Incorporated | revised |  | R5-241649 |
| R5-240985 | Addition of new NR NTN MAC TA reporting test case | Qualcomm Incorporated | agreed |  |  |
| R5-240986 | Addition of new NR NTN RLC t-reassembly timer test case | Qualcomm Incorporated | revised |  | R5-241570 |
| R5-240987 | Addition of NR NTN PDCP discard timer test case | Qualcomm Incorporated | revised |  | R5-241571 |
| R5-240988 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | revised |  | R5-241650 |
| R5-240989 | SIB2 updates for NR unlicensed test cases | Qualcomm Incorporated | agreed |  |  |
| R5-240990 | Addition of NR shared spectrum measurements test case 8.1.8.1.3 | Qualcomm Incorporated | agreed |  |  |
| R5-240991 | Applicability updates to NR shared spectrum test cases | Qualcomm Incorporated | agreed |  |  |
| R5-240992 | Clarification of trace mode in emission testing_FR1 | Huawei, HiSilicon | revised |  | R5-241767 |
| R5-240993 | Clarification of trace mode in emission testing_FR2 | Huawei, HiSilicon | revised |  | R5-241783 |
| R5-240994 | Clarification of trace mode in emission testing_Iw | Huawei, HiSilicon | revised |  | R5-241784 |
| R5-240995 | Adding FR2 test case of SRS time mask | Huawei, HiSilicon | revised |  | R5-241990 |
| R5-240996 | Correction to SUL configuration messages | Huawei, HiSilicon | revised |  | R5-241719 |
| R5-240997 | Update to message exception of SUL time mask test cases | Huawei, HiSilicon | withdrawn |  |  |
| R5-240998 | Update to AMPR, ASEM and ASE for intra-band CA for CA_NS_04 | Huawei, HiSilicon | revised |  | R5-241768 |
| R5-240999 | Adding PICS for V2X testing | Huawei, HiSilicon | revised |  | R5-241707 |
| R5-241000 | TP analysis for AMPR, ASEM and ASE for V2X | Huawei, HiSilicon | agreed |  |  |
| R5-241001 | Adding AMPR test cases for V2X | Huawei, HiSilicon | revised |  | R5-241733 |
| R5-241002 | Update to ASEM test cases for V2X | Huawei, HiSilicon | agreed |  |  |
| R5-241003 | Update to ASE test case for V2X | Huawei, HiSilicon | agreed |  |  |
| R5-241004 | Adding test applicability for V2X test cases | Huawei, HiSilicon | agreed |  |  |
| R5-241005 | Update to MU and TT for AMPR for CA test case | Huawei, HiSilicon, China Unicom | agreed |  |  |
| R5-241006 | Test points analysis for FR1 AMPR test case with NS_47 and PC 1.5 | Huawei, HiSilicon, SoftBank Corp. | agreed |  |  |
| R5-241007 | Adding the support of NS_47 for PC 1.5 | Huawei, HiSilicon, SoftBank Corp. | agreed |  |  |
| R5-241008 | Addition of test point analysis for R17 FR1 enhancement test cases | Huawei, HiSilicon | agreed |  |  |
| R5-241009 | Addition of 6.2H.1.3 AMPR for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241970 |
| R5-241010 | Addition of 6.3H.1.4 power control for CA with UL MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-241011 | Addition of 6.4H.1.1 frequency error for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241971 |
| R5-241012 | Addition of 6.4H.1.2.1 EVM for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241972 |
| R5-241013 | Addition of 6.4H.1.2.2 Carrier leakage for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241973 |
| R5-241014 | Addition of 6.4H.1.2.3 In-band emission for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241974 |
| R5-241015 | Addition of 6.4H.1.3 time alignment error for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241975 |
| R5-241016 | Addition of 6.4H.1.4 Coherent requirement for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241976 |
| R5-241017 | Addition of 6.5H.1.1 Occupied bandwidth for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241977 |
| R5-241018 | Addition of 6.5H.1.2.2 ASEM for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241978 |
| R5-241019 | Addition of 6.5H.1.3.1 General spurious emission for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241979 |
| R5-241020 | Addition of 6.5H.1.3.2 Spurious emission UE co-existence for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241980 |
| R5-241021 | Addition of 6.5H.1.3.3 Additional spurious emission for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241981 |
| R5-241022 | Addition of 6.5H.1.4 Transmit intermodulation for CA with UL-MIMO | Huawei, HiSilicon | revised |  | R5-241982 |
| R5-241023 | Update to minimum requirement of 6.4D.4 for UL switching | Huawei, HiSilicon | agreed |  |  |
| R5-241024 | Addition of 6.3A.3.4 1Tx-2Tx UL switching between two bands | Huawei, HiSilicon | revised |  | R5-241983 |
| R5-241025 | Addition of 6.3A.3.5 2Tx-2Tx UL switching between two bands | Huawei, HiSilicon | revised |  | R5-241984 |
| R5-241026 | Addition of 6.3C.3.3 2Tx-2Tx UL switching between two uplink carriers in SUL configuration | Huawei, HiSilicon | revised |  | R5-241985 |
| R5-241027 | Addition of 6.3C.3.4 1Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | revised |  | R5-241986 |
| R5-241028 | Addition of 6.3C.3.5 2Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | revised |  | R5-241987 |
| R5-241029 | Update to PICS for R17 FR1 enhancement | Huawei, HiSilicon | agreed |  |  |
| R5-241030 | Update to test applicability for R17 FR1 enhancement | Huawei, HiSilicon | revised |  | R5-241988 |
| R5-241031 | WP - Rel-17 RF requirements enhancement for NR frequency range 1 (FR1) | Huawei, HiSilicon | noted |  |  |
| R5-241032 | SR - Rel-17 RF requirements enhancement for NR frequency range 1 (FR1) | Huawei, HiSilicon | noted |  |  |
| R5-241033 | Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-1) | Huawei, HiSilicon | noted |  |  |
| R5-241034 | Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-2) | Huawei, HiSilicon | noted |  |  |
| R5-241035 | Updates to NR shared spectrum MAC test case 7.1.1.10.3 | Qualcomm Incorporated | revised |  | R5-241542 |
| R5-241036 | Updates to NR shared spectrum idle mode test cases | Qualcomm Incorporated | agreed |  |  |
| R5-241037 | Corrections to eMIMO TC 7.1.1.3.10 | Lenovo | revised |  | R5-241509 |
| R5-241038 | Updates to NR shared spectrum page monitoring test cases | Qualcomm Incorporated | agreed |  |  |
| R5-241039 | Updates to NR shared spectrum RRC test cases | Qualcomm Incorporated | agreed |  |  |
| R5-241040 | Addition of UAS EPS test case 10.10.2 | Qualcomm Incorporated | revised |  | R5-241608 |
| R5-241041 | Addition of UAS EPS test case 10.10.4 | Qualcomm Incorporated | revised |  | R5-241609 |
| R5-241042 | Addition of EPS UAS test case 10.10.6 | Qualcomm Incorporated | revised |  | R5-241610 |
| R5-241043 | Applicability updates to EPS UAS test cases | Qualcomm Incorporated | agreed |  |  |
| R5-241044 | Corrections to CovEnh TC 7.1.1.1.18 | Lenovo, MCC TF160 | agreed |  |  |
| R5-241045 | Correction to IRAT MDT test case 8.1.6.2.2 | Qualcomm Incorporated | agreed |  |  |
| R5-241046 | WP Rel-15 NR TX and RX Test Cases – Part 3: Range 1 and Range 2 Interworking operation with other radios (TS 38.521-3) | Qualcomm Technologies Ireland | revised |  | R5-241668 |
| R5-241047 | Editorial update on Annex C | Apple (UK) Limited | revised |  | R5-241936 |
| R5-241048 | Editorial update on Annex E | Apple (UK) Limited | revised |  | R5-241937 |
| R5-241049 | Editorial update on clause 3 | Apple (UK) Limited | revised |  | R5-241938 |
| R5-241050 | Editorial update on clause 4 | Apple (UK) Limited | revised |  | R5-241939 |
| R5-241051 | On TRP test result variations adopting reduced grids | Apple (UK) Limited | noted |  |  |
| R5-241052 | New WID on UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirement for NR Ues | Apple (UK) Limited | revised |  | R5-241473 |
| R5-241053 | Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC3 | NTT DOCOMO INC. | withdrawn |  |  |
| R5-241054 | Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC2 | NTT DOCOMO, INC. | revised |  | R5-241749 |
| R5-241055 | Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC3 | NTT DOCOMO, INC. | withdrawn |  |  |
| R5-241056 | Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC2 | NTT DOCOMO, INC. | revised |  | R5-241750 |
| R5-241057 | Update to freq error cat 1bis test procedure | Qualcomm Technologies Ireland | agreed |  |  |
| R5-241058 | Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3 | NTT DOCOMO INC. | withdrawn |  |  |
| R5-241059 | Addition of REFSENS for 2CC EN-DC for 19A-n78A in PC2 | NTT DOCOMO INC. | withdrawn |  |  |
| R5-241060 | Update of REFSENS for 3CC EN-DC in PC3 | NTT DOCOMO INC. | withdrawn |  |  |
| R5-241061 | Addition of REFSENS for 3CC EN-DC in PC2 | NTT DOCOMO INC. | withdrawn |  |  |
| R5-241062 | Addition and correction of REFSENS for 2CC EN-DC in PC2 | NTT DOCOMO INC. | agreed |  |  |
| R5-241063 | Proposal for new test case related to ATTACH NEEDED STATE | NTTDOCOMO, INC. | noted |  |  |
| R5-241064 | Update of REFSENS for 3CC EN-DC in PC3 | NTT DOCOMO INC. | withdrawn |  |  |
| R5-241065 | Addition of new test case 11.2.13 | NTT DOCOMO, INC. Rohde & Schwarz | revised |  | R5-241624 |
| R5-241066 | Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3 | NTT DOCOMO INC. | revised |  | R5-241992 |
| R5-241067 | Addition of applicability for new test case 11.2.13 | NTTDOCOMO, INC. | revised |  | R5-241625 |
| R5-241068 | Addition of REFSENS for 3CC EN-DC in PC2 | NTT DOCOMO INC., Verizon | revised |  | R5-241989 |
| R5-241069 | Update to FR1 EN-DC Configurations for n78 and n79 | NTT DOCOMO INC.. | withdrawn |  |  |
| R5-241070 | Correction of test case 6.1.2.13 | NTTDOCOMO, INC. | revised |  | R5-241498 |
| R5-241071 | Correction of test case 6.1.2.14 | NTTDOCOMO, INC. | revised |  | R5-241499 |
| R5-241072 | Update to FR1 EN-DC Configurations for n78 and n79 | NTT DOCOMO INC.. | agreed |  |  |
| R5-241073 | Correction to IRAT PLMN selection test case 6.2.1.5 | Qualcomm Incorporated | revised |  | R5-241500 |
| R5-241074 | Correction of test case 8.1.1.2.4 | NTTDOCOMO, INC., TF160 | revised |  | R5-241511 |
| R5-241075 | Correction of test case 8.1.4.2.1.1 | NTTDOCOMO, INC. | revised |  | R5-241516 |
| R5-241076 | Revised WID UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) | Ericsson | agreed |  |  |
| R5-241077 | PRD21 CDS: PC2 ENDC 2 and 3 bands combos with n78 and n79 | NTT DOCOMO INC.. | noted |  |  |
| R5-241078 | On the QoQZ MU for n262 and TRP | ROHDE & SCHWARZ | noted |  |  |
| R5-241079 | Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs | NTT DOCOMO INC.. | revised |  | R5-241785 |
| R5-241080 | Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs | NTT DOCOMO INC.. | revised |  | R5-241786 |
| R5-241081 | Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs | NTT DOCOMO INC.. | revised |  | R5-241787 |
| R5-241082 | Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs | NTT DOCOMO INC.. | revised |  | R5-241788 |
| R5-241083 | MU discussion on FR2c | Anritsu | revised |  | R5-241858 |
| R5-241084 | Update for FR2c MU | Anritsu, Keysight | revised |  | R5-241859 |
| R5-241085 | Update for FR2c MU | Anritsu, Keysight | revised |  | R5-241861 |
| R5-241086 | Update for FR2c MU | Anritsu | revised |  | R5-241865 |
| R5-241087 | QoQZ up to FR2d | Anritsu | noted |  |  |
| R5-241088 | MU discussion on FR1 EVM including symbols with transient period | Anritsu | noted |  |  |
| R5-241089 | Editorial correction to CA_1A-3A-7A-28A in 7.3A.9 | Anritsu | agreed |  |  |
| R5-241090 | Correction to UL RMC descriptions for NB-IoT NTN | Anritsu | withdrawn |  |  |
| R5-241091 | Correction to test frequencies for CA_n48 - 2A | Anritsu | revised |  | R5-241706 |
| R5-241092 | Correction to CBW selection to avoid unintentional asymmetric CBW | Anritsu | agreed |  |  |
| R5-241093 | Correction to point A value for n83 | Anritsu | revised |  | R5-241717 |
| R5-241094 | Correction to parameters for Rel-17 bands | Anritsu | revised |  | R5-241718 |
| R5-241095 | Correction to message exceptions for Performance test cases | Anritsu | revised |  | R5-242022 |
| R5-241096 | Correction to Reference sensitivity for Rel-16 CA | Anritsu, Huawei, HiSilicon | revised |  | R5-241732 |
| R5-241097 | Correction to Reference sensitivity for CA_n28A-n41A-n79A | Anritsu | revised |  | R5-241737 |
| R5-241098 | Clarification of asymmetric BW in Rx test cases for CA | Anritsu | revised |  | R5-241738 |
| R5-241099 | Correction to UL configuration for intra-band contiguous CA | Anritsu | agreed |  |  |
| R5-241100 | Correction to Rel-15 A-MPR | Anritsu | revised |  | R5-241922 |
| R5-241101 | Correction to Rel-16 A-MPR | Anritsu | agreed |  |  |
| R5-241102 | Correction to test configuration in 6.2D.2 | Anritsu | revised |  | R5-241969 |
| R5-241103 | Correction to applicability of powerBoosting for PC3 UE | Anritsu | agreed |  |  |
| R5-241104 | Addition of CBW 35 MHz and 45 MHz to OBW for inter-band CA | Anritsu | agreed |  |  |
| R5-241105 | Addition of CBW 35 MHz, 45 MHz, 70 MHz to ACS for CA | Anritsu | agreed |  |  |
| R5-241106 | Addition of CBW 35 MHz, 45 MHz, 70 MHz to Narrow band blocking for inter-band CA | Anritsu, Rohde&Schwarz | revised |  | R5-241773 |
| R5-241107 | Clarification of test procedure of EIS spherical coverage for inter-band CA | Anritsu | agreed |  |  |
| R5-241108 | Correction to Reference sensitivity for DC_21A_n28A | Anritsu | agreed |  |  |
| R5-241109 | Correction to note application in Table 7.3B.2.0.3.4-2 | Anritsu | agreed |  |  |
| R5-241110 | Correction to p-Max value in Rx test cases for FR1 intra-band contiguous EN-DC | Anritsu, Eurofins KCTL, Huawei, HiSilicon | agreed |  |  |
| R5-241111 | Correction to UL power control for FR1 intra-band non-contiguous EN-DC | Anritsu | agreed |  |  |
| R5-241112 | Correction to p-Max value in out of band test cases for FR1 inter-band EN-DC | Anritsu | revised |  | R5-241924 |
| R5-241113 | Correction to 1Rx FR1 PDSCH and PDCCH performance for RedCap | Anritsu | agreed |  |  |
| R5-241114 | Correction to OCNG references in 16.1.1 and 16.1.2 | Anritsu | agreed |  |  |
| R5-241115 | Correction to title of RedCap test cases | Anritsu | agreed |  |  |
| R5-241116 | Correction to PRACH config in 16.5.2.4 | Anritsu | agreed |  |  |
| R5-241117 | Correction to 16.7.2.4.x and 16.7.3.4.x | Anritsu | withdrawn |  |  |
| R5-241118 | Addition of message contents and cell mapping for 16.7.1.2.1 and 16.7.1.2.2 | Anritsu | withdrawn |  |  |
| R5-241119 | Correction to test applicability of 4.6.1.7 | Anritsu, MediaTek Beijing Inc. | revised |  | R5-241834 |
| R5-241120 | Correction to QuantityConfigu-DEFAULT in H.3.1 | Anritsu | agreed |  |  |
| R5-241121 | Correction to note for inter-band CA including n48 | Anritsu | revised |  | R5-241945 |
| R5-241122 | Correction to Reference sensitivity for CA_n26A-n70A | Anritsu | revised |  | R5-242015 |
| R5-241123 | Correction to PDCCH level in 6.5.1.9 | Anritsu | revised |  | R5-241967 |
| R5-241124 | Correction to test configuration in 16.4.1.2 and 16.5.2.2 | Anritsu | revised |  | R5-241964 |
| R5-241125 | Correction to PDCCH level in CSI-RS based RLM test cases for RedCap | Anritsu | revised |  | R5-241965 |
| R5-241126 | Correction to gap offset and SMTC configuration in 16.6.2.9 and 16.6.2.10 | Anritsu | revised |  | R5-241824 |
| R5-241127 | Correction to SSB configuration in 16.6.2.11 and 16.6.2.12 | Anritsu | revised |  | R5-241825 |
| R5-241128 | Correction to test parameters in 5.6.1.3 and 5.6.1.4 | Anritsu | revised |  | R5-242021 |
| R5-241129 | Correction to TRS Configuration in 6.3.2.1.3 | Anritsu | revised |  | R5-241998 |
| R5-241130 | Correction to PDCCH level in 6.5.1.8 | Anritsu | revised |  | R5-241999 |
| R5-241131 | Adding applicability for newly introduced NR-U test cases | Ericsson | agreed |  |  |
| R5-241132 | WP - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum | QUALCOMM Europe Inc. - Italy | noted |  |  |
| R5-241133 | SR - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum | QUALCOMM Europe Inc. - Italy | noted |  |  |
| R5-241134 | WP UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN) | QUALCOMM Europe Inc. - Italy | noted |  |  |
| R5-241135 | SR UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN) | QUALCOMM Europe Inc. - Italy | noted |  |  |
| R5-241136 | WP UE Conformance Test Aspects - Further enhancement on NR demodulation performance | QUALCOMM Europe Inc. - Italy | noted |  |  |
| R5-241137 | Addition of SDR RMC for more FDD channel BW | QUALCOMM Europe Inc. - Italy | revised |  | R5-241819 |
| R5-241138 | Update Es for 1024QAM scenarios | QUALCOMM Europe Inc. - Italy | revised |  | R5-241684 |
| R5-241139 | Update to NR-U demod test cases | QUALCOMM Europe Inc. - Italy | revised |  | R5-241821 |
| R5-241140 | Update to applicability spec for Redcap Demod test case | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241141 | Addition of FR2 SDR test for RedCap | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241142 | Addition of 6.2.1.1.2.1 test for RedCap | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241143 | Update incorrect reference to SDR test in Table 5.1.1.11-1 for RedCap | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241144 | Correction to numCDM group for PDCCH test cases | QUALCOMM Europe Inc. - Italy | withdrawn |  |  |
| R5-241145 | Update to power imbalance test cases | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241146 | Correction to RMC name in Table A.3.2.2.2-29 | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241147 | Correction to RMC name in Table A.3.2.1.1-18 | QUALCOMM Europe Inc. - Italy | revised |  | R5-242019 |
| R5-241148 | Update to PDSCH Mapping Type A test case for Satellite Access | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241149 | General updates to RF NTN clauses | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241150 | TT analysis for FR1 PDSCH with inter-cell interference test cases | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241151 | Update to FR1 PDSCH inter-cell interference test case | QUALCOMM Europe Inc. - Italy | revised |  | R5-241815 |
| R5-241152 | Update to FR1 PDSCH intra-cell interference test case | QUALCOMM Europe Inc. - Italy | revised |  | R5-241816 |
| R5-241153 | Applicability update for PDSCH interference test cases | QUALCOMM Europe Inc. - Italy | revised |  | R5-241846 |
| R5-241154 | Editorial correction in FR1 test case 6.4G.2.1 | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241155 | Editorial correction in FR1 test case 6.5A.2.4.1.1 | Keysight Technologies UK Ltd | revised |  | R5-241769 |
| R5-241156 | Editorial correction in FR1 test case 6.2A.2.1 | Keysight Technologies | agreed |  |  |
| R5-241157 | HST-DPS channel profile clarifications | QUALCOMM Europe Inc. - Italy | agreed |  |  |
| R5-241158 | Correction to RACS Test case 9.1.9.7 | Anritsu EMEA Ltd | agreed |  |  |
| R5-241159 | Correction to applicability of EN-DC CA test cases | Qualcomm Incorporated | agreed |  |  |
| R5-241160 | Correction to EN-DC FR1 Beam Failure TCs 4.5.5.x | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241161 | Correction to NSAC Test case 10.1.8.3 | Anritsu EMEA Ltd | revised |  | R5-241523 |
| R5-241162 | Correction to EN-DC FR2 Beam Failure TCs 5.5.5.x | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241163 | Correction to NR Slice test case 8.1.1.3.9 | Anritsu EMEA Ltd | revised |  | R5-241512 |
| R5-241164 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | revised |  | R5-241643 |
| R5-241165 | Removal of duplicated RSSI measurements and channel occupancy reporting parameter | Ericsson | agreed |  |  |
| R5-241166 | Correction to RRC Inactive Mode test case 6.4.1.1 | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241167 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | revised |  | R5-241645 |
| R5-241168 | Correction to Cat1bis frequency error | Rohde & Schwarz | withdrawn |  |  |
| R5-241169 | Corrections to URLLC Test Cases | Rohde & Schwarz | revised |  | R5-241818 |
| R5-241170 | Correction to FR2 SDR requirements | Rohde & Schwarz | agreed |  |  |
| R5-241171 | Correction to FR2 256QAM CQI reporting | Rohde & Schwarz | agreed |  |  |
| R5-241172 | Editorial correction to FR1 inter-band co-existence | Rohde & Schwarz | revised |  | R5-241790 |
| R5-241173 | Correction to CSI reference measurement channels for 256QAM | Rohde & Schwarz | withdrawn |  |  |
| R5-241174 | Correction to CA A-MPR requirements | Rohde & Schwarz | agreed |  |  |
| R5-241175 | Correction RedCap CSI Test Case message exceptions | Rohde & Schwarz | agreed |  |  |
| R5-241176 | Correction to UE timing advance for satellite access TC 13.4.2.1 | Keysight Technologies UK Ltd | revised |  | R5-242003 |
| R5-241177 | Addition of test case 6.5F.3.3 Additional spurious emissions for shared spectrum channel access | Ericsson | revised |  | R5-241740 |
| R5-241178 | Proposal for new test case related to network policy indication on LTE call redirection to GERAN and UTRAN | Vodafone | noted |  |  |
| R5-241179 | Addition of new test case 8.1.3.8a Redirection to GERAN-Redir-policy bit | Vodafone | revised |  | R5-241476 |
| R5-241180 | Addition of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone | revised |  | R5-241477 |
| R5-241181 | Addition of applicability of new test case 8.1.3.8a for redir-policy bit | Vodafone | revised |  | R5-241622 |
| R5-241182 | GCF 3GPP TCL after GCF CAG#77 | Ericsson | noted |  |  |
| R5-241183 | Correction to UAS TC 9.1.5.2.11 | Ericsson | revised |  | R5-241607 |
| R5-241184 | New Emergency test case 11.4.15 | Ericsson | revised |  | R5-241590 |
| R5-241185 | Applicability for new test case 11.4.15 | Ericsson | agreed |  |  |
| R5-241186 | Correction to UPIP TC 7.1.3.2.6 | Ericsson | agreed |  |  |
| R5-241187 | Updates to default 5GMM messages | Ericsson | withdrawn |  |  |
| R5-241188 | Updates to default 5GSM messages | Ericsson | withdrawn |  |  |
| R5-241189 | Updates to default UE Policy Delivery messages | Ericsson | revised |  | R5-241489 |
| R5-241190 | Update to generic procedure A.4.2a | Ericsson | agreed |  |  |
| R5-241191 | Updates to test case 7.1 | Ericsson | agreed |  |  |
| R5-241192 | Updates to test case 7.5 | Ericsson | revised |  | R5-241655 |
| R5-241193 | Updates to test case 7.12 | Ericsson | revised |  | R5-241656 |
| R5-241194 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone | revised |  | R5-241623 |
| R5-241195 | Updates of test case 6.2F.3 UE additional maximum output power reduction for shared spectrum access | Ericsson | revised |  | R5-241741 |
| R5-241196 | Correction of 7.6A.2 for inband blocking for CA | ZTE Corporation, Rohde&Schwarz | revised |  | R5-241774 |
| R5-241197 | Addition of new test case 4.7.8.1 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Sporton | revised |  | R5-241830 |
| R5-241198 | Addition of new test case 4.7.8.2 EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | agreed |  |  |
| R5-241199 | Correct of test tolerance analysis of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 | Sporton | revised |  | R5-241902 |
| R5-241200 | Correct of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 including Test Tolerance | Sporton | agreed |  |  |
| R5-241201 | Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell | Sporton | revised |  | R5-241901 |
| R5-241202 | Adding Test Point Analysis for 4DL and 5DL RefSens | Sporton, Bureau Veritas | agreed |  |  |
| R5-241203 | Add information in Power Control test cases specific to Power Class 3 | Sporton | agreed |  |  |
| R5-241204 | Add information to non-TXD test cases with UE supports TXD | Sporton | revised |  | R5-241873 |
| R5-241205 | EIS search interpolation methods | ROHDE & SCHWARZ | noted |  |  |
| R5-241206 | CR on EIS search interpolation | ROHDE & SCHWARZ | revised |  | R5-241929 |
| R5-241207 | CR on Measurement Distance | ROHDE & SCHWARZ | revised |  | R5-241940 |
| R5-241208 | Correction to test case 6.4E.2.4.1D In-band emissions for V2X / non-concurrent operation / SL-MIMO | MTCC, KTL | agreed |  |  |
| R5-241209 | Corrections to NR SA FR1 RedCap Event Trigger TCs 16.6.2.9, 16.6.2.10 | Keysight Technologies UK Ltd | withdrawn |  |  |
| R5-241210 | Correction to test case 6.4E.2.4.2 In-band emissions for V2X / con-current operation | MTCC, KTL | agreed |  |  |
| R5-241211 | Annex E.14 correction for new RedCap test cases | Rohde & Schwarz | agreed |  |  |
| R5-241212 | Annex F correction for RedCap test cases including Test Tolerance | Rohde & Schwarz | withdrawn |  |  |
| R5-241213 | Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 1 Rx UE test case 16.7.1.1.1 including Test Tolerance | Rohde & Schwarz | agreed |  |  |
| R5-241214 | Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE test case 16.7.1.1.2 including Test Tolerance | Rohde & Schwarz | agreed |  |  |
| R5-241215 | Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.1 including Test Tolerance | Rohde & Schwarz | agreed |  |  |
| R5-241216 | Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE test case 16.7.1.2.2 including Test Tolerance | Rohde & Schwarz | agreed |  |  |
| R5-241217 | Addition of NR SA FR1 SS-RSRQ measurement accuracy for 1 Rx UE test case 16.7.2.1 including Test Tolerance | Rohde & Schwarz | agreed |  |  |
| R5-241218 | Addition of NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE test case 16.7.2.2 including Test Tolerance | Rohde & Schwarz | agreed |  |  |
| R5-241219 | Addition of RRM RedCap test cases 16.7.1.3.1, 16.7.1.3.2 | Rohde & Schwarz | agreed |  |  |
| R5-241220 | Addition of RRM RedCap test cases 16.7.1.4.1, 16.7.1.4.2 | Rohde & Schwarz | agreed |  |  |
| R5-241221 | Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 1 Rx UE test case 16.7.1.1.1 | Rohde & Schwarz | agreed |  |  |
| R5-241222 | Test Tolerance for NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE test case 16.7.1.1.2 | Rohde & Schwarz | agreed |  |  |
| R5-241223 | Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.1 | Rohde & Schwarz | agreed |  |  |
| R5-241224 | Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.2 | Rohde & Schwarz | agreed |  |  |
| R5-241225 | Test Tolerance for NR SA FR1 SS-RSRQ measurement accuracy UE test case 16.7.2.1 | Rohde & Schwarz | agreed |  |  |
| R5-241226 | Test Tolerance NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE test case 16.7.2.2 | Rohde & Schwarz | agreed |  |  |
| R5-241227 | Editorial Correction to RedCap PRACH TC 16.3.2.2.8 | Rohde & Schwarz | agreed |  |  |
| R5-241228 | Editorial corrections for RedCap tets cases 18.3.1.x | Rohde & Schwarz | agreed |  |  |
| R5-241229 | Core Spec alignment for RedCap tets cases 18.3.1.x | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-241230 | Addition of missing section for RRM CA Configuration | Rohde & Schwarz | revised |  | R5-241875 |
| R5-241231 | Corrections to test procedure for 2-step RACH TC 6.3.2.2.4 | Rohde & Schwarz | revised |  | R5-241837 |
| R5-241232 | Correction for test case 5.5.6.2.1 | Rohde & Schwarz | revised |  | R5-241835 |
| R5-241233 | Editorial correction in Table 5.6.1.3.5-2 | Rohde & Schwarz | agreed |  |  |
| R5-241234 | Corrections to BFD-LR test case 5.5.5.5 | Rohde & Schwarz | revised |  | R5-241836 |
| R5-241235 | Correction to BFD-LR test case 5.5.5.5 message content | Rohde & Schwarz | agreed |  |  |
| R5-241236 | Removal of Editors note for RRM FR2 RLM in-sync test cases | Rohde & Schwarz | revised |  | R5-241996 |
| R5-241237 | Revision of Editors note for RRM FR2 RLM test cases | Rohde & Schwarz | revised |  | R5-241997 |
| R5-241238 | Editor correction to Test Procedure 5.5.1.2 | Rohde & Schwarz | agreed |  |  |
| R5-241239 | Correction to applicability notes for FR2 RRM RLM test cases | Rohde & Schwarz | agreed |  |  |
| R5-241240 | Addition of missing applicability to new SS-RSRQ RedCap test cases | ROHDE & SCHWARZ | revised |  | R5-241962 |
| R5-241241 | Addition of SA FR1 NR-U intra-frequency SS-SINR measurement accuracy test cases | Rohde & Schwarz | agreed |  |  |
| R5-241242 | Addition of SA FR1 NR-U inter-frequency SS-SINR measurement accuracy test cases | Rohde & Schwarz | agreed |  |  |
| R5-241243 | Addition of SA FR1 NR-U intra-frequency RSSI measurement accuracy test cases | Rohde & Schwarz | agreed |  |  |
| R5-241244 | Core Spec alignment for s-IntraSearchP IE for Re-selection NB-IOT NTN RRM TCs | Rohde & Schwarz | agreed |  |  |
| R5-241245 | Re-Addition of omitted change for RRM test case 13.1.1.2 | Rohde & Schwarz | agreed |  |  |
| R5-241246 | Editorial corrections to NB-IOT NTN RRM Reselection test cases | Rohde & Schwarz | agreed |  |  |
| R5-241247 | Editorial corrections to NB-IOT NTN RRM TCs | Rohde & Schwarz | revised |  | R5-241796 |
| R5-241248 | Core Spec alignment for RRM NB-IOT NTN test cases | Rohde & Schwarz | agreed |  |  |
| R5-241249 | Core Spec alignment for test config for RRM NB-IOT NTN test cases | Rohde & Schwarz | withdrawn |  |  |
| R5-241250 | Core Spec alignment for RRM NB-IOT NTN UL Timing test cases | Rohde & Schwarz | agreed |  |  |
| R5-241251 | Rel-15 5GS WI RRM work plan update | ROHDE & SCHWARZ | noted |  |  |
| R5-241252 | New generic procedure A.15.2a | Ericsson | revised |  | R5-241554 |
| R5-241253 | New generic procedure A.16.2a | Ericsson | revised |  | R5-241555 |
| R5-241254 | Updates to test case 7.15 | Ericsson | agreed |  |  |
| R5-241255 | Updates to test case 7.17 | Ericsson | agreed |  |  |
| R5-241256 | Updates to test case 7.21 | Ericsson | agreed |  |  |
| R5-241257 | Updates to test case 7.23 | Ericsson | agreed |  |  |
| R5-241258 | Updates of test case 6.5F.2.4.2, Shared spectrum channel access ACLR with additional requirement for NS_29 | Ericsson | revised |  | R5-241742 |
| R5-241259 | Addition of RF baseline implementation capability of PC2 config n8 | China Unicom | agreed |  |  |
| R5-241260 | Update of Operating bands for UL MIMO band n5 | China Unicom | agreed |  |  |
| R5-241261 | Addition of n5 into TC 6.2D.1 MOP for UL MIMO | China Unicom | revised |  | R5-241745 |
| R5-241262 | Addition of n5 with UL MIMO capabilities | China Unicom | agreed |  |  |
| R5-241263 | Addition of n5 into TC 6.2D.2 MPR for UL MIMO | China Unicom | agreed |  |  |
| R5-241264 | Core Spec alignment for RedCap tets cases 18.3.1.x | Rohde & Schwarz | agreed |  |  |
| R5-241265 | General update to NTN tests | Qualcomm Germany | agreed |  |  |
| R5-241266 | Addition of NTN timing accuracy test 14.3.1.1 | Qualcomm Germany | agreed |  |  |
| R5-241267 | Addition of NTN time-based CHO tests 14.2.1.x | Qualcomm Germany | agreed |  |  |
| R5-241268 | Addition of NTN intra-frequency cell reselection tests 14.1.x | Qualcomm Germany | agreed |  |  |
| R5-241269 | Addition of NTN SS-SINR measurement accuracy tests 14.6.3.x | Qualcomm Germany | agreed |  |  |
| R5-241270 | Addition of PICS to support NTN RRM | Qualcomm Germany | agreed |  |  |
| R5-241271 | Core alignment for NSA NR-U tests | Qualcomm Germany | revised |  | R5-241828 |
| R5-241272 | Addition of NR-U SA BWP switch tests including TT | Qualcomm Germany | agreed |  |  |
| R5-241273 | Addition of UL active BWP switch test 10.3.5.1 including TT | Qualcomm Germany | agreed |  |  |
| R5-241274 | Update of NR-U Annex E and F including TT | Qualcomm Germany | agreed |  |  |
| R5-241275 | Update to NR-U general sections | Qualcomm Germany | agreed |  |  |
| R5-241276 | Addition of NR-U UE Timing Tests 10.2.x | Qualcomm Germany | agreed |  |  |
| R5-241277 | Addition of NR-U L1-RSRP accuracy test 10.5.4.1 | Qualcomm Germany | agreed |  |  |
| R5-241278 | Addition of NR-U RSSI measurement accuracy tests 10.5.5.x | Qualcomm Germany | agreed |  |  |
| R5-241279 | Addition of NR-U Channel occupancy measurement accuracy tests 10.5.6.x | Qualcomm Germany | agreed |  |  |
| R5-241280 | Addition of NR-U L1-RSRP tests 10.4.3.x | Qualcomm Germany | agreed |  |  |
| R5-241281 | Addition of NR-U IRAT tests 10.4.4.x | Qualcomm Germany | agreed |  |  |
| R5-241282 | Update to NSA NR-U tests including TT | Qualcomm Germany | agreed |  |  |
| R5-241283 | Update to SA NR-U tests including TT | Qualcomm Germany | agreed |  |  |
| R5-241284 | Update to OCNG noise power in UL CCA model | Qualcomm Germany | agreed |  |  |
| R5-241285 | Core alignment for NR-U common sections | Qualcomm Germany | agreed |  |  |
| R5-241286 | Addition of PICS for UL LBT Failure Detection and Recovery | Qualcomm Germany | revised |  | R5-241711 |
| R5-241287 | Update to NR-U test applicability | Qualcomm Germany, Ericsson | revised |  | R5-241851 |
| R5-241288 | Addition of TTs for NR-U Test Cases 10.3.1.2 and 11.4.1.2 | Qualcomm Germany | revised |  | R5-241881 |
| R5-241289 | Addition of TTs for NR-U Test Cases 10.3.1.3 and 11.4.1.3 | Qualcomm Germany | revised |  | R5-241882 |
| R5-241290 | Addition of TTs for NR-U Test Cases 10.3.4.1, 10.3.4.2, 11.4.4.1 and 11.4.4.2 | Qualcomm Germany | revised |  | R5-241883 |
| R5-241291 | Addition of TTs for NR-U Test Cases 10.3.5.1 and 11.4.5.1 | Qualcomm Germany | revised |  | R5-242000 |
| R5-241292 | Addition of TTs for NR-U Test Cases 10.3.5.2.1, 10.3.5.2.2, 11.4.5.2.1 and 11.4.5.2.2 | Qualcomm Germany | revised |  | R5-241884 |
| R5-241293 | Addition of TTs for NR-U Test Cases 10.3.5.3.1 and 11.4.5.3.1 | Qualcomm Germany | revised |  | R5-241885 |
| R5-241294 | Discussion on MU impact to energy detection threshold in shared spectrum | Qualcomm Germany | revised |  | R5-241823 |
| R5-241295 | Cleanup on FR1 CG-SDT test 6.2.1 | Qualcomm Germany | agreed |  |  |
| R5-241296 | Addition of RedCap SA FR2 SSB RLM OOS in DRX test case 17.5.1.3 | Qualcomm Germany | agreed |  |  |
| R5-241297 | Addition of RedCap SA FR2 SSB BFR test cases 17.5.2.1 and 17.5.2.2 | Qualcomm Germany | agreed |  |  |
| R5-241298 | Update to RRM test 16.3.1.8 including TT | Qualcomm Germany | agreed |  |  |
| R5-241299 | Correction to stationary relaxed measurement criterion tests | Qualcomm Germany | agreed |  |  |
| R5-241300 | Update to Re-establishment test cases | Qualcomm Germany | agreed |  |  |
| R5-241301 | Applcability update for several RedCap tests | Qualcomm Germany | agreed |  |  |
| R5-241302 | Applcability update for several RedCap tests | Qualcomm Germany | agreed |  |  |
| R5-241303 | Update of TT analysis for 6.3.1.4 and 16.3.1.8 | Qualcomm Germany | revised |  | R5-241892 |
| R5-241304 | Update to RRM test 6.3.1.4 including TT | Qualcomm Germany | agreed |  |  |
| R5-241305 | Update to Applicability General Section | Qualcomm Germany | agreed |  |  |
| R5-241306 | Update to test selection criteria for RRM tests | Qualcomm Germany | revised |  | R5-241841 |
| R5-241307 | Status on Test Selection Criteria for 38.522 | Qualcomm Germany | noted |  |  |
| R5-241308 | Correction to 5G NR TC 11.4.13 | MCC TF160 | revised |  | R5-241531 |
| R5-241309 | Correction to FR2 measurement threshold for NR RRC test cases | Anritsu EMEA Ltd | agreed |  |  |
| R5-241310 | 5G IMS voice test coverage | Ericsson | noted |  |  |
| R5-241311 | Addition of Clause 4.7.9.0 minimum conformance requirements for CSI-RSRQ | Apple | agreed |  |  |
| R5-241312 | Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.1 | Apple | agreed |  |  |
| R5-241313 | Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.2 | Apple | agreed |  |  |
| R5-241314 | Addition of Clause 4.7.10.0 minimum conformance requirements for CSI-SINR | Apple | agreed |  |  |
| R5-241315 | Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.10.1 | Apple | agreed |  |  |
| R5-241316 | Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell l test case 4.7.10.2 | Apple | agreed |  |  |
| R5-241317 | Addition of Clause 5.7.8.0 minimum conformance requirements for CSI-RSRQ | Apple | agreed |  |  |
| R5-241318 | Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 5.7.8.1 | Apple | agreed |  |  |
| R5-241319 | Addition of EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.8.2 | Apple | agreed |  |  |
| R5-241320 | Addition of Clause 5.7.9.0 minimum conformance requirements for CSI-SINR | Apple | agreed |  |  |
| R5-241321 | Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.1 | Apple | agreed |  |  |
| R5-241322 | Addition of EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.2 | Apple | agreed |  |  |
| R5-241323 | Addition of Clause 6.7.11.0 minimum conformance requirements for CSI-RSRQ | Apple | agreed |  |  |
| R5-241324 | Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.1 | Apple | agreed |  |  |
| R5-241325 | Addition of SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.2 | Apple | agreed |  |  |
| R5-241326 | Addition of Clause 6.7.12.0 minimum conformance requirements for CSI-SINR | Apple | agreed |  |  |
| R5-241327 | Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.1 | Apple | agreed |  |  |
| R5-241328 | Addition of SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.2 | Apple | agreed |  |  |
| R5-241329 | Addition of Clause 7.7.8.0 minimum conformance requirements for CSI-RSRQ | Apple | agreed |  |  |
| R5-241330 | Addition of SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.8.1 | Apple | agreed |  |  |
| R5-241331 | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.8.2 | Apple | agreed |  |  |
| R5-241332 | Addition of Clause 7.7.9.0 minimum conformance requirements for CSI-SINR | Apple | agreed |  |  |
| R5-241333 | Addition of SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.9.1 | Apple | agreed |  |  |
| R5-241334 | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.9.2 | Apple | revised |  | R5-241831 |
| R5-241335 | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex E | Apple | agreed |  |  |
| R5-241336 | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F | Apple | revised |  | R5-241893 |
| R5-241337 | Addition of TT analysis grouping for test cases 4.5.9.1 to Table 8-1 | Apple | agreed |  |  |
| R5-241338 | Addition of TT analysis grouping for test cases 5.5.6.3.1 to Table 8-2 | Apple | revised |  | R5-241894 |
| R5-241339 | Addition of TT analysis grouping for test cases 5.5.6.4.1 to Table 8-2 | Apple | revised |  | R5-241895 |
| R5-241340 | SR - UE Conformance Aspects - NR RRM enhancements | Apple | noted |  |  |
| R5-241341 | WP - UE Conformance Aspects - NR RRM enhancements | Apple | noted |  |  |
| R5-241342 | Correction of coding scheme in USF testing | ROHDE & SCHWARZ, Apple | agreed |  |  |
| R5-241343 | Correction of MPR CA test cases | ROHDE & SCHWARZ | agreed |  |  |
| R5-241344 | On the MU for n259 | ROHDE & SCHWARZ | noted |  |  |
| R5-241345 | Documentation of MU for n259 | ROHDE & SCHWARZ | revised |  | R5-241866 |
| R5-241346 | Update of PDSCH Config for RF test cases | ROHDE & SCHWARZ | revised |  | R5-241968 |
| R5-241347 | Editorial correction of test environment for IoT NTN RF testing | ROHDE & SCHWARZ | revised |  | R5-241951 |
| R5-241348 | Alignment of status of FR2 UL MIMO test cases | ROHDE & SCHWARZ | agreed |  |  |
| R5-241349 | Update of 4DL CA test cases | ROHDE & SCHWARZ, Verizon | revised |  | R5-241907 |
| R5-241350 | Update of applicability for FR1 4DL CA test cases | ROHDE & SCHWARZ | revised |  | R5-241946 |
| R5-241351 | Update of test point selection for 4DL CA configurations | ROHDE & SCHWARZ, Verizon | revised |  | R5-241908 |
| R5-241352 | Update of Narrow Band Blocking for CA test case | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-241353 | Editorial correction of TT for Minimum Output Power for UL MIMO | ROHDE & SCHWARZ | agreed |  |  |
| R5-241354 | Correction in A-MPR test case | ROHDE & SCHWARZ | agreed |  |  |
| R5-241355 | Correction of test environment for NR NTN RF testing | ROHDE & SCHWARZ | revised |  | R5-242007 |
| R5-241356 | On the MU for FR1 EVM including symbols with transient period | ROHDE & SCHWARZ | noted |  |  |
| R5-241357 | On the splitting of the IoT NTN Frequency Error test cases | ROHDE & SCHWARZ | revised |  | R5-241791 |
| R5-241358 | Splitting the IoT NTN frequency error test case | ROHDE & SCHWARZ, Keysight Technologies UK Ltd | revised |  | R5-241954 |
| R5-241359 | Aligning the applicability for IoT NTN frequency error | ROHDE & SCHWARZ | revised |  | R5-241953 |
| R5-241360 | On the splitting of the NR NTN Frequency Error test cases | ROHDE & SCHWARZ | noted |  |  |
| R5-241361 | Splitting the NR NTN frequency error test case | ROHDE & SCHWARZ | revised |  | R5-242013 |
| R5-241362 | Aligning the applicability for NR NTN frequency error | ROHDE & SCHWARZ | revised |  | R5-241906 |
| R5-241363 | Addition of test frequencies for CA n77(2A), BCS1, UL CA.. | Ericsson | revised |  | R5-241944 |
| R5-241364 | Discussion 2 regarding limiting number of test frequencies for CA combinations | Ericsson | noted |  |  |
| R5-241365 | Test environment definition for GERAN in Conducted and OTA Environment | Anritsu EMEA Ltd | revised |  | R5-241630 |
| R5-241366 | Update of GERAN Signal levels to eCall signalling tests | Anritsu EMEA Ltd | withdrawn |  |  |
| R5-241367 | General updates of Spurious emissions for UE co-existence for Inter-band CA | China Unicom | revised |  | R5-241880 |
| R5-241368 | Update of TP analysis for Spurious emissions test cases for FR1 UL CA | China Unicom | revised |  | R5-241903 |
| R5-241369 | Update of Refsens TC for RedCap UE | China Unicom | agreed |  |  |
| R5-241370 | Editorial correction to 5.2.2.1.1_1 Modulation format | Bureau Veritas ADT | agreed |  |  |
| R5-241371 | Add IE MeasSequence | Ericsson | agreed |  |  |
| R5-241372 | Add IE MeasWindowConfig | Ericsson | agreed |  |  |
| R5-241373 | Add IEs N3C-IndirectPathConfigRelay and N3C-IndirectPathAddChange | Ericsson | agreed |  |  |
| R5-241374 | Add IEs NCR-AperiodicFwdConfig, NCR-FwdConfig, NCR-PeriodicityAndOffset and NCR-PeriodicFwdResourceSet | Ericsson | agreed |  |  |
| R5-241375 | Add IEs NCR-PeriodicFwdResourceSetId, NCR-SemiPersistentFwdResourceSet and NCR-SemiPersistentFwdResourceSetId | Ericsson | agreed |  |  |
| R5-241376 | Addition of PICS for support of ULCA for different type of CA Band combinations | Keysight Technologies UK | withdrawn |  |  |
| R5-241377 | Correction to eDRX test case 11.7.3 | Keysight Technologies UK | agreed |  |  |
| R5-241378 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | revised |  | R5-241637 |
| R5-241379 | Add subtest selection criteria to RedCap Performance test cases | Bureau Veritas ADT | agreed |  |  |
| R5-241380 | Applicability spec update for NR NTN test cases | QUALCOMM Europe Inc. - Italy | withdrawn |  |  |
| R5-241381 | Ephemeris definition for IoT NTN RRM neighbour cells | Keysight Technologies UK Ltd, Thales | revised |  | R5-241943 |
| R5-241382 | Updates to Test environment for eMTC NTN RRM testing | Keysight Technologies UK Ltd, Thales | revised |  | R5-241952 |
| R5-241383 | Updates to Test environment for NB-IoT NTN RRM testing | Keysight Technologies UK Ltd, Thales | revised |  | R5-242002 |
| R5-241384 | UL RMCs updates for IoT NTN | Keysight Technologies UK Ltd | revised |  | R5-242006 |
| R5-241385 | Channel quality reporting updates | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241386 | Radio Link Monitoring updates | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241387 | Updates to PDSCH RMC | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241388 | Updates to IoT NTN Frequency error test | Keysight Technologies UK Ltd | revised |  | R5-241802 |
| R5-241389 | Annex A.3 update | Keysight Technologies UK Ltd | revised |  | R5-241676 |
| R5-241390 | Common Test environment updates for IoT NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | revised |  | R5-241793 |
| R5-241391 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | revised |  | R5-241955 |
| R5-241392 | Ephemeris definition for NR NTN RRM neighbor cells | Keysight Technologies UK Ltd, Thales | revised |  | R5-242001 |
| R5-241393 | Updates to Test environment for NR NTN RRM testing | Keysight Technologies UK Ltd, Thales | revised |  | R5-242016 |
| R5-241394 | UL RMCs updates for NR NTN | Keysight Technologies UK Ltd | revised |  | R5-242018 |
| R5-241395 | Updates to NR NTN Frequency error test | Keysight Technologies UK Ltd | revised |  | R5-241808 |
| R5-241396 | Updates to NR NTN Minimum output power test | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241397 | Updates to NR NTN Transmit OFF power test | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241398 | Updates to NR NTN Maximum Input Level test | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241399 | Applicability updates for NR NTN Minimum output power and maximum Input level tests | Keysight Technologies UK Ltd | withdrawn |  |  |
| R5-241400 | Common Test environment updates for NR NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | revised |  | R5-242017 |
| R5-241401 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | revised |  | R5-242014 |
| R5-241402 | Initial discussions for Rel-18 beam correspondence for initial access | Keysight Technologies UK Ltd | noted |  |  |
| R5-241403 | FR2 UL Gaps measurement uncertainties and test tolerances | Keysight Technologies UK Ltd, Apple Inc | revised |  | R5-242024 |
| R5-241404 | Measurement uncertainty definition for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | revised |  | R5-242029 |
| R5-241405 | Updates to Annex F for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | revised |  | R5-242026 |
| R5-241406 | Updates to UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | revised |  | R5-242025 |
| R5-241407 | Updates to Annex F for 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | revised |  | R5-242028 |
| R5-241408 | Updates to 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | revised |  | R5-242027 |
| R5-241409 | Discussion paper for handling Rel-16 features and test cases for Redcap Only UEs | MediaTek Inc. | revised |  | R5-241572 |
| R5-241410 | General cleanup of annex for app layer tput test cases | QUALCOMM Europe Inc. - Italy | revised |  | R5-241852 |
| R5-241411 | Updating applicability for RedCap | MediaTek Inc. | revised |  | R5-241578 |
| R5-241412 | Discussion paper for IMS test cases over Redcap Only UE | MediaTek Inc. | revised |  | R5-241573 |
| R5-241413 | Neighbor GSO satellite ephemeris file for NTN NR UE testing | THALES | revised |  | R5-241792 |
| R5-241414 | Applicability updates for new NTN Idle mode and NAS test cases | QUALCOMM JAPAN LLC. | revised |  | R5-241651 |
| R5-241415 | On Coarse Measurement Grids | Keysight Technologies UK Ltd | noted |  | - |
| R5-241416 | Add missing abbreviations | Keysight Technologies UK Ltd | agreed |  |  |
| R5-241417 | Correction to Video capability | MediaTek Inc. | withdrawn |  |  |
| R5-241418 | Correction to max throughput values for PDSCH RMC | QUALCOMM Europe Inc. - Italy | revised |  | R5-241820 |
| R5-241419 | New WID on UE Conformance - NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 | Apple Benelux B.V., Nokia | revised |  | R5-241474 |
| R5-241420 | New WID on UE Conformance - Introduction of the satellite L-/S- band for NR | Apple Benelux B.V., Globalstar | withdrawn |  |  |
| R5-241421 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | revised |  | R5-241932 |
| R5-241422 | Update of TT within TRP and TRS tests | Apple Benelux B.V. | revised |  | R5-241931 |
| R5-241423 | Views on FR1 TRP TRS TT | Apple Benelux B.V. | revised |  | R5-241930 |
| R5-241424 | Empirical data inputs on coarse grids | Apple Benelux B.V. | withdrawn |  |  |
| R5-241425 | Updates to Annex A.4.2.12 | Apple Benelux B.V. | withdrawn |  |  |
| R5-241426 | On Rel17 TRP TRS status updates | Apple Benelux B.V. | noted |  |  |
| R5-241427 | Maintainence Work Plan for Rel17 TRP TRS | Apple Benelux B.V. | revised |  | R5-241933 |
| R5-241428 | Addition of gap pattern configs | Apple Benelux B.V. | withdrawn |  |  |
| R5-241429 | Update to FR2 Tx Power test with UL-Gaps | Apple Benelux B.V. | revised |  | R5-241966 |
| R5-241430 | Update to FR2 Tx OFF Power test specific to UL-Gaps | Apple Benelux B.V. | agreed |  |  |
| R5-241431 | Updates to FR2 ACS test | Apple Benelux B.V. | revised |  | R5-241780 |
| R5-241432 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | revised |  | R5-241947 |
| R5-241433 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | revised |  | R5-241949 |
| R5-241434 | Updates and corrections to Annex in FR1 RF spec | Apple Benelux B.V. | revised |  | R5-241948 |
| R5-241435 | Updates to Annex E content and structure | Apple Benelux B.V. | revised |  | R5-241950 |
| R5-241436 | Updates to TP analysis for FR2 RF phase continuity test | Apple Benelux B.V. | revised |  | R5-241743 |
| R5-241437 | Updates to TP analysis for FR1 RF phase continuity test | Apple Benelux B.V. | revised |  | R5-241744 |
| R5-241438 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | revised |  | R5-241844 |
| R5-241439 | Work Plan for Rel17 FR2 RF Enhanced Test Methods | Apple Benelux B.V. | noted |  |  |
| R5-241440 | Updates to NTN TC 6.3.3 on Tx on-off time mask | Apple Benelux B.V. | agreed |  |  |
| R5-241441 | Updates to NTN TC 6.5.2.2 on Spectrum emission mask | Apple Benelux B.V. | agreed |  |  |
| R5-241442 | Updates to NTN TC 6.5.2.4 on ACLR | Apple Benelux B.V. | agreed |  |  |
| R5-241443 | MU TT updates for NTN RF tests | Apple Benelux B.V. | withdrawn |  |  |
| R5-241444 | Applicability updates for NTN RF tests | Apple Benelux B.V. | withdrawn |  |  |
| R5-241445 | Views on FR1 EVM test with transients | Apple Trading | noted |  |  |
| R5-241446 | Addition of applicability for RedCap Demod and RRM test cases | Huawei, HiSilicon | revised |  | R5-241848 |
| R5-241447 | Correction to Annex F for R15 RRM test cases | Huawei, HiSilicon | agreed |  |  |
| R5-241448 | Addition of CA test for EIRP test with ULGaps | Apple Trading | revised |  | R5-241782 |
| R5-241449 | WP WI UE Conformance – UE power saving enhancements for NR | MediaTek | noted | - | - |
| R5-241450 | SR WI UE Conformance – UE power saving enhancements for NR | MediaTek | noted | - | - |
| R5-241451 | Correction of applicability for partial sounding test case | Huawei, HiSilicon, TF160 | agreed | - | - |
| R5-241452 | Handling of Rel-18 ASN.1 / NAS tabular | TF160 manager | noted | - | - |
| R5-241453 | Add applicability for Rel-17 ATSSS test cases | China Telecom, ZTE | revised | - | R5-241621 |
| R5-241455 | Candidate Spec for Release upgrade after RAN5#102 | Bureau Veritas ADT | revised | - | R5-241667 |
| R5-241456 | Update of PRD18 for NTN specs and relations between the specs in a group | Bureau Veritas ADT | revised | - | R5-241660 |
| R5-241457 | Correction to Rel-16 PICS Mnemonic | MediaTek Inc. | withdrawn | - | - |
| R5-241458 | MCC TF160 Status Report | MCC TF160 | approved | R5-240526 | - |
| R5-241459 | New WID on UE Conformance - IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects | CMCC, MTK, CAICT | agreed | R5-240094 | - |
| R5-241460 | New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation | MediaTek Inc., China Telecom | agreed | R5-240146 | - |
| R5-241461 | New WID on UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR | China Telecom | agreed | R5-240278 | - |
| R5-241462 | New WID on UE Conformance - Further NR coverage enhancements | China Telecom | agreed | R5-240283 | - |
| R5-241463 | New WID on UE Conformance - Further RF requirements enhancement for NR and EN-DC in frequency range 1 | Huawei, Hisilicon, China Telecom | agreed | R5-240303 | - |
| R5-241464 | New WID on UE Conformance - Network energy savings for NR | Huawei, HiSilicon | agreed | R5-240344 | - |
| R5-241465 | New WID on UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 | Huawei, HiSilicon, CMCC | agreed | R5-240345 | - |
| R5-241466 | New WID on UE Conformance – Multi-carrier enhancements for NR | China Telecom, Huawei, HiSilicon | agreed | R5-240367 | - |
| R5-241467 | New WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom, Ericsson, Huawei, Hisilicon, Qualcomm | agreed | R5-240390 | - |
| R5-241468 | New WID on UE Conformance - Introduction of 900 MHz LTE band in the US | Nokia, Nokia Shanghai Bell | agreed | R5-240458 | - |
| R5-241469 | New WID on UE Conformance - NR support for dedicated spectrum less than 5MHz for FR1 | Nokia, Nokia Shanghai Bell | revised | R5-240517 | R5-241659 |
| R5-241470 | New WID on UE Conformance - XR (eXtended Reality) enhancements for NR | Nokia, Nokia Shanghai Bell, CMCC, Huawei, Hisilicon, Qualcomm | agreed | R5-240594 | - |
| R5-241471 | New WID on UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2 (FR2) | Samsung | agreed | R5-240610 | - |
| R5-241472 | New WID on UE Conformance – NR MIMO Evolution for Downlink and Uplink | Samsung, Huawei, Hisilicon | agreed | R5-240612 | - |
| R5-241473 | New WID on UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirement for NR Ues | Apple (UK) Limited | agreed | R5-241052 | - |
| R5-241474 | New WID on UE Conformance - NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 | Apple Benelux B.V., Nokia | agreed | R5-241419 | - |
| R5-241475 | Addition of new L2L MPS priority access barring test case | Qualcomm Incorporated | agreed | R5-240977 | - |
| R5-241476 | Addition of new test case 8.1.3.8a Redirection to GERAN-Redir-policy bit | Vodafone | agreed | R5-241179 | - |
| R5-241477 | Addition of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone | agreed | R5-241180 | - |
| R5-241478 | Corrections to NB-IoT RLC TC 22.3.2.3 | MCC TF160 | agreed | R5-240586 | - |
| R5-241479 | Discussion Paper for Rel-17 eNS_Ph2 WI tests case applicability | Qualcomm CDMA Technologies | noted | R5-240663 | - |
| R5-241480 | Correction to IE NRDC-Parameters | MediaTek Inc. | agreed | R5-240169 | - |
| R5-241481 | Correction to IE UE-MRDC-Capability | MediaTek Inc. | agreed | R5-240170 | - |
| R5-241482 | Correction to IE UE-NR-Capability | MediaTek Inc. | agreed | R5-240171 | - |
| R5-241483 | Editorial update SystemInformation | Ericsson | agreed | R5-240447 | - |
| R5-241484 | Editorial update IE PosSystemInformation-r16-IEs | Ericsson | agreed | R5-240490 | - |
| R5-241485 | Add IEs CSI-ReportSubConfig, CSI-ReportSubConfigId and CSI-ReportSubConfigTriggerList | Ericsson | agreed | R5-240498 | - |
| R5-241486 | Add IE HysteresisAltitude | Ericsson | agreed | R5-240823 | - |
| R5-241487 | Add IEs LTM-CandidateId, LTM-Candidate, LTM-Config and LTM-CSI-ReportConfig | Ericsson | agreed | R5-240824 | - |
| R5-241488 | Add IEs LTM-CSI-ReportConfigId, LTM-CSI-ResourceConfig and LTM-CSI-ResourceConfigId | Ericsson | agreed | R5-240825 | - |
| R5-241489 | Updates to default UE Policy Delivery messages | Ericsson | withdrawn | R5-241189 | - |
| R5-241491 | Updates to align PICS mnemonics | MCC TF160 | agreed | R5-240538 | - |
| R5-241492 | Correction of Test Loop Mode C for MBS broadcast | Huawei, HiSilicon | agreed | R5-240434 | - |
| R5-241493 | Updates to OTA Signalling test environment | Anritsu Ltd | revised | - | R5-241629 |
| R5-241494 | Correction to RACS Test case 9.2.5.X | Anritsu Ltd | revised | - | R5-241627 |
| R5-241495 | Correction to NR testcase 6.4.2.1 | Starpoint, TDIA | agreed | R5-240225 | - |
| R5-241496 | Correction to NR RRC Idle mode test case 6.4.2.3 | Anritsu EMEA Ltd | agreed | R5-240645 | - |
| R5-241497 | Correction to NR RRC Idle mode test case 6.1.2.24 | Anritsu EMEA Ltd | agreed | R5-240646 | - |
| R5-241498 | Correction of test case 6.1.2.13 | NTTDOCOMO, INC. | agreed | R5-241070 | - |
| R5-241499 | Correction of test case 6.1.2.14 | NTTDOCOMO, INC. | agreed | R5-241071 | - |
| R5-241500 | Correction to IRAT PLMN selection test case 6.2.1.5 | Qualcomm Incorporated | agreed | R5-241073 | - |
| R5-241501 | Correction to NR MAC TC 7.1.1.5.x | MediaTek Inc. | agreed | R5-240176 | - |
| R5-241502 | Correction to NR MAC TC 7.1.1.6.1 | MediaTek Inc. | agreed | R5-240177 | - |
| R5-241503 | Correction to NR MAC TC 7.1.1.6.2 and 7.1.1.6.3 | MediaTek Inc. | agreed | R5-240178 | - |
| R5-241504 | Correction to NR MAC TC 7.1.1.6.4 and 7.1.1.6.5 | MediaTek Inc. | agreed | R5-240179 | - |
| R5-241505 | Correction to NR MAC TC 7.1.1.8.1 | MediaTek Inc. | agreed | R5-240180 | - |
| R5-241506 | Correction of MAC TC 7.1.1.3.2b-Lcp-Restriction | Huawei, HiSilicon | agreed | R5-240414 | - |
| R5-241507 | Correction of MAC TC 7.1.1.8.1-BWP switch | Huawei, HiSilicon | agreed | R5-240415 | - |
| R5-241508 | Correction and updates to NR TC 7.1.1.10.2 | Qualcomm CDMA Technologies | withdrawn | R5-240656 | - |
| R5-241509 | Corrections to eMIMO TC 7.1.1.3.10 | Lenovo | agreed | R5-241037 | - |
| R5-241510 | Correction to NR testcase 8.1.1.4.1 | Starpoint, TDIA | agreed | R5-240226 | - |
| R5-241511 | Correction of test case 8.1.1.2.4 | NTTDOCOMO, INC., TF160 | agreed | R5-241074 | - |
| R5-241512 | Correction to NR Slice test case 8.1.1.3.9 | Anritsu EMEA Ltd | agreed | R5-241163 | - |
| R5-241513 | Addition of new test case 8.1.2.1.7 for UPIP | ZTE Corporation | withdrawn | R5-240139 | - |
| R5-241514 | Correction to NR HO TC 8.1.4.2.1.2 | MediaTek Inc. | agreed | R5-240182 | - |
| R5-241515 | Correction to NR testcase 8.1.4.3.1 | Starpoint, TDIA | agreed | R5-240256 | - |
| R5-241516 | Correction of test case 8.1.4.2.1.1 | NTTDOCOMO, INC. | agreed | R5-241075 | - |
| R5-241517 | Correction to MUSIM test case 8.1.5.10.3 | CATT, TDIA | agreed | R5-240034 | - |
| R5-241518 | Correction to NR RRC TC 8.1.5.1.1 | MediaTek Inc. | agreed | R5-240183 | - |
| R5-241519 | Correction to NR RRC TC 8.2.1.1.1 | MediaTek Inc. | agreed | R5-240185 | - |
| R5-241520 | Correction to NR RRC TC 8.2.1.1.2 | MediaTek Inc. | agreed | R5-240186 | - |
| R5-241521 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon | agreed | R5-240442 | - |
| R5-241522 | Correction of NR TC 9.1.4.1-CUC | Huawei, HiSilicon | agreed | R5-240609 | - |
| R5-241523 | Correction to NSAC Test case 10.1.8.3 | Anritsu EMEA Ltd | agreed | R5-241161 | - |
| R5-241524 | Correction to 5GSM test case 10.3.2.1 | Keysight Technologies UK, Samsung | agreed | R5-240472 | - |
| R5-241525 | Addition of signalling test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification, AT&T | revised | - | R5-241580 |
| R5-241526 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD | revised | R5-240950 | R5-241688 |
| R5-241527 | Correction to NR5GC test cases 11.3.6 and 11.3.6a | Keysight Technologies UK Ltd | withdrawn | R5-240022 | - |
| R5-241528 | Correction to NR testcase 11.4.12 | Starpoint, TDIA | agreed | R5-240230 | - |
| R5-241529 | Correction to NR5GC testcase 11.4.5 | ROHDE & SCHWARZ, Hisilicon | agreed | R5-240509 | - |
| R5-241530 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | revised | R5-240951 | R5-241689 |
| R5-241531 | Correction to 5G NR TC 11.4.13 | MCC TF160 | agreed | R5-241308 | - |
| R5-241532 | Correction to MBS TC 14.1.1.3 | MediaTek Inc. | agreed | R5-240190 | - |
| R5-241533 | Correction to MBS TC 14.1.2.1 | MediaTek Inc. | agreed | R5-240192 | - |
| R5-241534 | Correction of MBS broadcast TC 14.1.1.1-Test Loop Mode C | Huawei, HiSilicon | agreed | R5-240435 | - |
| R5-241535 | Correction of MBS broadcast TC 14.1.1.3-Test Loop Mode C | Huawei, HiSilicon | agreed | R5-240436 | - |
| R5-241536 | Correction of MBS broadcast TC 14.1.2.1-Test Loop Mode C | Huawei, HiSilicon | agreed | R5-240437 | - |
| R5-241537 | Correction of MBS broadcast TC 14.1.3.1-Test Loop Mode C | Huawei, HiSilicon | agreed | R5-240440 | - |
| R5-241538 | Move all V2X sub-clauses from 4.7 into a new clause 4.7D | Huawei, HiSilicon, CATT | agreed | R5-240432 | - |
| R5-241539 | Correction to default configuration of frequencyInfoSL | Huawei, HiSilicon | agreed | R5-240764 | - |
| R5-241540 | Correction of applicability for V2X SIG test cases | Huawei, HiSilicon | agreed | R5-240767 | - |
| R5-241541 | 5G V2X: Test Model updates | MCC TF160 | agreed | R5-240530 | - |
| R5-241542 | Updates to NR shared spectrum MAC test case 7.1.1.10.3 | Qualcomm Incorporated | agreed | R5-241035 | - |
| R5-241543 | Corrections to SDT TC 7.1.1.13.1 | MCC TF160, Lenovo | agreed | R5-240532 | - |
| R5-241544 | Corrections to SDT TC 7.1.1.13.2 | MCC TF160, Lenovo | agreed | R5-240533 | - |
| R5-241545 | Corrections to SDT TC 7.1.1.13.3 | MCC TF160, Lenovo | agreed | R5-240534 | - |
| R5-241546 | Modification of testcase 8.1.5.13.2 for Data on non-SDT Radio Bearers for NR SmallData | Nokia, Nokia Shanghai Bell | agreed | R5-240589 | - |
| R5-241547 | Update the applicability of PEIPS TC 9.1.14.1 and 11.4.1a | Huawei, HiSilicon | agreed | R5-240427 | - |
| R5-241548 | PRD.21 CDS: ENDC PC3 8 EN-DC bands | WE Certification | noted | - | - |
| R5-241549 | PRD.21 CDS: NRCA PC2 CA_n77(2A), CA_n14A-n77A | WE Certification | noted | - | - |
| R5-241550 | PRD.21 CDS: ENDC PC2 13 EN-DC bands | WE Certification | noted | - | - |
| R5-241551 | Addition of new test case 8.2.2.4.4 for UPIP | ZTE Corporation | withdrawn | R5-240140 | - |
| R5-241552 | Correction of UPIP test case 7.1.3.2.6 for specific message contents | ZTE Corporation | agreed | R5-240203 | - |
| R5-241553 | Correction of UPIP test case 8.2.6.4.1 | ZTE Corporation | agreed | R5-240205 | - |
| R5-241554 | New generic procedure A.15.2a | Ericsson | agreed | R5-241252 | - |
| R5-241555 | New generic procedure A.16.2a | Ericsson | agreed | R5-241253 | - |
| R5-241556 | Update ephemeris information for NGSO signalling test environment | MediaTek Inc. | agreed | R5-240158 | - |
| R5-241557 | Update of UE pre-configuration for NGSO signalling test environment | MediaTek Inc. | withdrawn | R5-240221 | - |
| R5-241558 | Correction to System information for NTN | MediaTek Inc. | agreed | R5-240290 | - |
| R5-241559 | Definition of a TN-to-NTN sysInfo combination mapping for legacy test cases to be run in NTN environment | MCC TF160 | withdrawn | R5-240527 | - |
| R5-241560 | Updates to the NTN test environment | MCC TF160, MediaTek | agreed | - | - |
| R5-241561 | Updates to Ephemeris Info for Multi-Cell NRN-NTN signalling test cases | QUALCOMM JAPAN LLC. | agreed | R5-240648 | - |
| R5-241562 | Update of test cases applicability for NB-IoT NTN only UE | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm Incorporated | agreed | R5-240161 | - |
| R5-241563 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | agreed | R5-240979 | - |
| R5-241564 | Correction to MAC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | agreed | R5-240980 | - |
| R5-241565 | Updates to SIB19 Common Config for NR-NTN test cases | QUALCOMM JAPAN LLC | agreed | R5-240649 | - |
| R5-241566 | Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | agreed | R5-240650 | - |
| R5-241567 | Updates to cell configuration for NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" test case | QUALCOMM JAPAN LLC. | agreed | R5-240651 | - |
| R5-241568 | Addition of NTN / Mobility registration update / supported TACs not part of UE registration area | QUALCOMM JAPAN LLC. | agreed | R5-240653 | - |
| R5-241569 | Introduction of new NR NTN RRC TC for Event D1 | Qualcomm CDMA Technologies, | agreed | R5-240659 | - |
| R5-241570 | Addition of new NR NTN RLC t-reassembly timer test case | Qualcomm Incorporated | agreed | R5-240986 | - |
| R5-241571 | Addition of NR NTN PDCP discard timer test case | Qualcomm Incorporated | agreed | R5-240987 | - |
| R5-241572 | Discussion paper for handling Rel-16 features and test cases for Redcap Only UEs | MediaTek Inc. | noted | R5-241409 | - |
| R5-241573 | Discussion paper for IMS test cases over Redcap Only UE | MediaTek Inc. | noted | R5-241412 | - |
| R5-241574 | Way Forward on Machine Readable PICS | Qualcomm CDMA Technologies | noted | R5-240664 | - |
| R5-241575 | Addition of applicability for new UPIP TC 8.1.2.1.7 and 8.2.2.4.4 | ZTE Corporation | withdrawn | R5-240141 | - |
| R5-241576 | Revised WID: UE Conformance - User Plane Integrity Protection support for EPC connected architectures (incl. CT/SA aspects) | Vodafone | noted | - | - |
| R5-241577 | Introduction of BDS B2a and B3I signal default test conditions in TS 37.571-5 | CATT, CAICT | agreed | R5-240126 | - |
| R5-241578 | Updating applicability for RedCap | MediaTek Inc. | agreed | R5-241411 | - |
| R5-241579 | Updating applicability for video | MediaTek Inc. | agreed | - | - |
| R5-241580 | Addition of signalling test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification, AT&T | agreed | R5-241525 | - |
| R5-241581 | Addition of capability for UEs to support steering of roaming SNPN selection information (SOR-SNPN-SI) and steering of roaming connected mode control information (SOR-CMCI) for Rel-17 eNPN | China Telecom | agreed | R5-240350 | - |
| R5-241582 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | withdrawn | R5-240351 | - |
| R5-241583 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 eNPN | China Telecom | withdrawn | R5-240352 | - |
| R5-241584 | Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 eNPN | China Telecom | withdrawn | R5-240353 | - |
| R5-241585 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 eNPN | China Telecom | withdrawn | R5-240354 | - |
| R5-241586 | Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN | China Telecom | withdrawn | R5-240355 | - |
| R5-241587 | Addition of new test case 9.1.11.5 for Mobility management in Rel-17 eNPN | China Telecom | agreed | R5-240356 | - |
| R5-241588 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | withdrawn | R5-240357 | - |
| R5-241589 | Scoping NR SA applicable TCs for SNPN-only UEs | China Telecom | agreed | R5-240360 | - |
| R5-241590 | New Emergency test case 11.4.15 | Ericsson | agreed | R5-241184 | - |
| R5-241591 | Editorial correction for test procedure sequence in 11.1.10 | NTT DOCOMO, INC. | agreed | R5-240028 | - |
| R5-241592 | Add new ING_5GS test case 11.1.11 | China Telecom | withdrawn | R5-240040 | - |
| R5-241593 | Addition of new TC 6.4.1 Emergency Alert CO | NIST | agreed | R5-240903 | - |
| R5-241594 | Addition of new TC 6.4.2 Emergency Alert CT | NIST | agreed | R5-240904 | - |
| R5-241595 | Addition of new TC 6.1.1.14 Re-join | NIST | agreed | R5-240911 | - |
| R5-241596 | Addition of new TC 6.5.2 One-to-server video push call CO | NIST | agreed | R5-240633 | - |
| R5-241597 | Addition of test procedure for 5G ProSe U2N Relay Discovery | TDIA, CATT | agreed | R5-240478 | - |
| R5-241598 | Addition of test procedure for remote Initial Access procedure under NR sidelink U2N Relay / Remote UE side | CATT, TDIA | agreed | R5-240479 | - |
| R5-241599 | Update test procedure for registration of NR sidelink U2N Relay / Relay UE side | TDIA, CATT | agreed | R5-240481 | - |
| R5-241600 | Addition of new pics for NR sidelink U2N Relay PDU session establishment | TDIA, CATT | agreed | R5-240477 | - |
| R5-241601 | Addition of UE capability for inter-SN conditional PSCell change | Huawei, HiSilicon | agreed | R5-240387 | - |
| R5-241602 | Addition of new test case 8.2.3.18.4 inter-SN CPC for EN-DC | Huawei, HiSilicon | agreed | R5-240379 | - |
| R5-241603 | Addition of new test case 8.2.3.18.5 inter-SN CPC for NR-DC | Huawei, HiSilicon | agreed | R5-240380 | - |
| R5-241604 | Addition of applicability for inter-SN conditional PSCell change | Huawei, HiSilicon | agreed | R5-240386 | - |
| R5-241605 | Modification of testcase 9.1.6.2.3 UE or NW initiated de-registration for UAS | Nokia, Nokia Shanghai Bell | agreed | R5-240588 | - |
| R5-241606 | Corrections to UAS test case 10.1.4.3 | Qualcomm CDMA Technologies | agreed | R5-240658 | - |
| R5-241607 | Correction to UAS TC 9.1.5.2.11 | Ericsson | agreed | R5-241183 | - |
| R5-241608 | Addition of UAS EPS test case 10.10.2 | Qualcomm Incorporated | agreed | R5-241040 | - |
| R5-241609 | Addition of UAS EPS test case 10.10.4 | Qualcomm Incorporated | agreed | R5-241041 | - |
| R5-241610 | Addition of EPS UAS test case 10.10.6 | Qualcomm Incorporated | agreed | R5-241042 | - |
| R5-241611 | Addition of SENSE eMTC TC 6.1.1.12-PLMN selection | Huawei, HiSilicon | agreed | R5-240418 | - |
| R5-241612 | Addition of SENSE eMTC TC 6.1.1.13-PLMN selection Exception | Huawei, HiSilicon | agreed | R5-240419 | - |
| R5-241613 | Addition of SENSE eMTC TC 6.1.1.14-Periodic SENSE | Huawei, HiSilicon | agreed | R5-240420 | - |
| R5-241614 | Addition of SENSE NB-IoT TC 22.2.14-PLMN selection | Huawei, HiSilicon | agreed | R5-240421 | - |
| R5-241615 | Addition of SENSE NB-IoT TC 22.2.15-PLMN selection Exception | Huawei, HiSilicon | agreed | R5-240422 | - |
| R5-241616 | Addition of SENSE NB-IoT TC 22.2.16-Periodic SENSE | Huawei, HiSilicon | agreed | R5-240423 | - |
| R5-241617 | Add new r17 ATSSS test case 11.9.1 | China Telecom | agreed | R5-240100 | - |
| R5-241618 | Add new ATSSS test case 11.9.3 | ZTE Corporation | agreed | R5-240240 | - |
| R5-241619 | Add new ATSSS test case 11.9.4 | ZTE Corporation | agreed | R5-240241 | - |
| R5-241620 | Add new r17 ATSSS test case 11.9.2 | China Telecom | agreed | R5-240282 | - |
| R5-241621 | Add applicability for Rel-17 ATSSS test cases | China Telecom, ZTE | agreed | R5-241453 | - |
| R5-241622 | Addition of applicability of new test case 8.1.3.8a for redir-policy bit | Vodafone | agreed | R5-241181 | - |
| R5-241623 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone | agreed | R5-241194 | - |
| R5-241624 | Addition of new test case 11.2.13 | NTT DOCOMO, INC. Rohde & Schwarz | agreed | R5-241065 | - |
| R5-241625 | Addition of applicability for new test case 11.2.13 | NTTDOCOMO, INC. | agreed | R5-241067 | - |
| R5-241626 | Correction to LTE RACS test case 8.5.5.1 | Keysight Technologies UK, Qualcomm | agreed | R5-240021 | - |
| R5-241627 | Correction to RACS Test case 9.2.5.X | Anritsu Ltd | revised | R5-241494 | R5-241690 |
| R5-241628 | Correction to NR RRC_Inactive generic procedure | Keysight Technologies UK | agreed | R5-240237 | - |
| R5-241629 | Updates to OTA Signalling test environment | Anritsu Ltd | agreed | R5-241493 | - |
| R5-241630 | Test environment definition for GERAN in Conducted and OTA Environment | Anritsu EMEA Ltd | agreed | R5-241365 | - |
| R5-241631 | Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI) | Qualcomm CDMA Technologies | agreed | R5-240673 | - |
| R5-241632 | Removal of GEA1/GEA2 and verification of non-support of GEA1/GEA2 | ROHDE & SCHWARZ | agreed | R5-240514 | - |
| R5-241633 | Discussion to update MUSIM gap TC | Huawei, HiSilicon,China Telecom | noted | R5-240431 | - |
| R5-241634 | Discussion on handling of MDT IDC problem | CMCC | noted | R5-240096 | - |
| R5-241635 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT | agreed | R5-240095 | - |
| R5-241636 | Correction of Set MUSIM UAI test function | Huawei, HiSilicon,China Telecom | withdrawn | R5-240430 | - |
| R5-241637 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | agreed | R5-241378 | - |
| R5-241638 | Add new ING_5GS test case 9.3.1.6 | China Telecom | agreed | - | - |
| R5-241639 | Addition of applicability of new ING_5GS test case 9.3.1.6 | China Telecom | revised | - | R5-241654 |
| R5-241640 | Correction of MUSIM TC 8.1.2.1.6-MUSIM gap configuration | Huawei, HiSilicon,China Telecom | agreed | R5-240429 | - |
| R5-241641 | Correction to NR TC 8.1.6.1.3.8 | Huawei, Hisilicon | agreed | R5-240867 | - |
| R5-241642 | Correction to NR TC 8.1.6.1.3.9 | Huawei, Hisilicon | agreed | R5-240868 | - |
| R5-241643 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | agreed | R5-241164 | - |
| R5-241644 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, Hisilicon, MediaTek | revised | R5-240512 | R5-241687 |
| R5-241645 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | agreed | R5-241167 | - |
| R5-241646 | Addition of applicability for eNPN test cases | China Telecom | agreed | R5-240359 | - |
| R5-241647 | Correction to RLC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | agreed | R5-240981 | - |
| R5-241648 | PICS clarification and applicability updates for NTN test cases | Qualcomm Incorporated, Rohde & Schwarz | agreed | R5-240983 | - |
| R5-241649 | Addition of NR NTN TA reporting PICS | Qualcomm Incorporated | agreed | R5-240984 | - |
| R5-241650 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | agreed | R5-240988 | - |
| R5-241651 | Applicability updates for new NTN Idle mode and NAS test cases | QUALCOMM JAPAN LLC. | agreed | R5-241414 | - |
| R5-241652 | Applicability of New NR NTN TC for Event D1 | Qualcomm CDMA Technologies | agreed | R5-240660 | - |
| R5-241653 | Correction to TC 11.4.1a | MediaTek Inc. | agreed | R5-240156 | - |
| R5-241654 | Addition of applicability of new ING_5GS test case 9.3.1.6 | China Telecom | agreed | R5-241639 | - |
| R5-241655 | Updates to test case 7.5 | Ericsson | agreed | R5-241192 | - |
| R5-241656 | Updates to test case 7.12 | Ericsson | agreed | R5-241193 | - |
| R5-241657 | Removal of GEA1 and GEA2 algorithm verification | Keysight Technologies UK, Bureau Veritas | agreed | R5-240475 | - |
| R5-241658 | Meeting schedule for 2024-25 | WG Chairman | noted | R5-240014 | - |
| R5-241659 | New WID on UE Conformance - NR support for dedicated spectrum less than 5MHz for FR1 | Nokia, Nokia Shanghai Bell | agreed | R5-241469 | - |
| R5-241660 | Update of PRD18 for NTN specs and relations between the specs in a group | Bureau Veritas ADT | approved | R5-241456 | - |
| R5-241661 | Revised WID on UE Conformance-IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS | China Telecom | noted | R5-240227 | - |
| R5-241662 | Revised WID on UE Conformance Test Aspects - Rel-17 IMS Data Channel | Huawei | agreed | R5-240869 | - |
| R5-241663 | Revised WID on UE Conformance - Air-to-ground network for NR | CMCC | agreed | R5-240042 | - |
| R5-241664 | Revised WID on UE Conformance - New Rel-18 NR licensed bands and extension of existing NR bands | Huawei, Hisilicon | agreed | R5-240291 | - |
| R5-241665 | Revised WID on UE Conformance - Rel-18 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | agreed | R5-240294 | - |
| R5-241666 | Revised WID: UE Conformance Test Aspects - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | MediaTek Inc. | agreed | R5-240321 | - |
| R5-241667 | Candidate Spec for Release upgrade after RAN5#102 | Bureau Veritas ADT | noted | R5-241455 | - |
| R5-241668 | WP Rel-15 NR TX and RX Test Cases – Part 3: Range 1 and Range 2 Interworking operation with other radios (TS 38.521-3) | Qualcomm Technologies Ireland | noted | R5-241046 | - |
| R5-241669 | WP UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS | China Telecom | noted | R5-240036 | - |
| R5-241670 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | agreed | R5-240705 | - |
| R5-241671 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | agreed | R5-240706 | - |
| R5-241672 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | agreed | R5-241878 | - |
| R5-241673 | Correction to SDT RRM test case 6.2.1 with TT | Huawei,HiSilicon | agreed | R5-240756 | - |
| R5-241674 | Addition of feMIMO physical layer baseline implementation capabilities | Samsung | agreed | R5-241958 | - |
| R5-241675 | Addition of applicability for FeMIMO test cases | Samsung | agreed | R5-241847 | - |
| R5-241676 | Annex A.3 update | Keysight Technologies UK Ltd | agreed | R5-241389 | - |
| R5-241677 | Correction to FR2 SA event triggered reporting tests with Pre-MG including TT | Anritsu | agreed | R5-240634 | - |
| R5-241678 | Correction to FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | agreed | R5-240636 | - |
| R5-241679 | Correction to FR2 SA event triggered reporting tests with NCSG including TT | Anritsu | agreed | R5-240638 | - |
| R5-241680 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with Pre-MG | Anritsu | agreed | R5-240635 | - |
| R5-241681 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with concurrent gaps | Anritsu | agreed | R5-240637 | - |
| R5-241682 | Introduction of Test Tolerance analysis for SA FR2 event triggered reporting tests with NCSG | Anritsu | agreed | R5-240639 | - |
| R5-241683 | Introduction of General description for ATG UE Tx TCs | CMCC | agreed | R5-242030 | - |
| R5-241684 | Update Es for 1024QAM scenarios | QUALCOMM Europe Inc. - Italy | agreed | R5-241138 | - |
| R5-241685 | Addition of applicability for new Rel-16 test cases | NIST | agreed | R5-240901 | - |
| R5-241686 | Addition of new PIXITs for REL-16 TCs | NIST | agreed | R5-240902 | - |
| R5-241687 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, Hisilicon, MediaTek | agreed | R5-241644 | - |
| R5-241688 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD, Rohde & Schwarz | revised | R5-241526 | R5-241691 |
| R5-241689 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | revised | R5-241530 | R5-241692 |
| R5-241690 | Correction to RACS Test case 9.2.5.X | Anritsu Ltd | agreed | R5-241627 | - |
| R5-241691 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD, Rohde & Schwarz | agreed | R5-241688 | - |
| R5-241692 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | agreed | R5-241689 | - |
| R5-241700 | Conclusion of RAN5 NR MIMO OTA Working Item | TSG WG RAN5 | approved | - | - |
| R5-241701 | Update reference sensitivity test cases for four bands configurations | Verizon | withdrawn | R5-240883 | - |
| R5-241702 | Addition of applicability of the NR-U RRM test cases | Ericsson | withdrawn | R5-240792 | - |
| R5-241703 | Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8 | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA | agreed | R5-240972 | - |
| R5-241704 | RF TRx testing - P-Max configuration extension to RX tests to enable TxD | Keysight Technologies UK Ltd | agreed | R5-240616 | - |
| R5-241705 | Addition of test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification Oy, AT&T | agreed | R5-240882 | - |
| R5-241706 | Correction to test frequencies for CA_n48 - 2A | Anritsu | agreed | R5-241091 | - |
| R5-241707 | Adding PICS for V2X testing | Huawei, HiSilicon | agreed | R5-240999 | - |
| R5-241708 | Update additional inter-band NR CA configurations | Verizon | agreed | R5-240886 | - |
| R5-241709 | Update for additional NR-DC band configurations | Verizon Spain | agreed | R5-240863 | - |
| R5-241710 | Editorial correction to supported EN-DC configuration table | WE Certification | agreed | R5-240919 | - |
| R5-241711 | Addition of PICS for UL LBT Failure Detection and Recovery | Qualcomm Germany | agreed | R5-241286 | - |
| R5-241712 | Addition of conditional PSCell addition and change configuration for EN-DC | Huawei, HiSilicon | agreed | R5-240973 | - |
| R5-241713 | Addition of SCG activation and deactivation configuration for EN-DC | Huawei, HiSilicon | agreed | R5-240974 | - |
| R5-241714 | Update for additional band configurations with PC2 UL | Verizon Spain | agreed | R5-240892 | - |
| R5-241715 | Introduction of common ICS for ATG | CMCC | agreed | R5-240048 | - |
| R5-241716 | Updating test frequency for SUL band n83 20MHz CBW | Huawei, HiSilicon | revised | R5-240309 | R5-241869 |
| R5-241717 | Correction to point A value for n83 | Anritsu | agreed | R5-241093 | - |
| R5-241718 | Correction to parameters for Rel-17 bands | Anritsu | agreed | R5-241094 | - |
| R5-241719 | Correction to SUL configuration messages | Huawei, HiSilicon | agreed | R5-240996 | - |
| R5-241720 | Update for additional ENDC band configurations with PC2 UL | Verizon | agreed | R5-240914 | - |
| R5-241721 | Updating FR1 PC2 REFSENS exceptions testing | Huawei, HiSilicon | revised | R5-240312 | R5-241870 |
| R5-241722 | Update of RRC message RRCReconfigurationComplete, RRCSetupComplete and SecurityModeComplete for SL relay | China Telecom | agreed | R5-240346 | - |
| R5-241723 | Update of Test procedure for establishing unicast mode ProSe Direct communication | China Telecom | agreed | R5-240347 | - |
| R5-241724 | Addition of Sidelink Capabilities to support direct to indirect path switch for NR sidelink U2N Relay | China Telecom | agreed | R5-240348 | - |
| R5-241725 | Addition of new test case 6.2H.2.3 MPR for inter-band EN-DC with UL MIMO | Huawei, HiSilicon | agreed | R5-240376 | - |
| R5-241726 | Message exceptions clarifications for 6.3G.3.3 | Keysight Technologies | withdrawn | R5-240622 | - |
| R5-241727 | Update test configuration table for NS_13 | MediaTek Beijing Inc. | agreed | R5-240138 | - |
| R5-241728 | Addition of delta TIBc and UE maximum output power for new R16 NR CA combos within FR1 | KDDI Corporation | agreed | R5-240326 | - |
| R5-241729 | Add UE maximum power requirements for CA_n1A-n28A and CA_n3A-n28A | Nokia, Nokia Shanghai Bell | agreed | R5-240779 | - |
| R5-241730 | Add spurious emissions for UE co-existence requirements for CA_n1A_n28A | Nokia, Nokia Shanghai Bell | agreed | R5-240780 | - |
| R5-241731 | Addition of delta RIBc and reference sensitivity for new R16 NR CA combos within FR1 | KDDI Corporation | agreed | R5-240328 | - |
| R5-241732 | Correction to Reference sensitivity for Rel-16 CA | Anritsu, Huawei, HiSilicon | agreed | R5-241096 | - |
| R5-241733 | Adding AMPR test cases for V2X | Huawei, HiSilicon | agreed | R5-241001 | - |
| R5-241734 | Addition of spurious emissions, delta TIBc and UE maximum output power for new R17 NR CA combos within FR1 | KDDI Corporation | agreed | R5-240327 | - |
| R5-241735 | Addition of delta RIBc and reference sensitivity for new R17 NR CA combos within FR1 | KDDI Corporation | agreed | R5-240329 | - |
| R5-241736 | Adding Reference sensitivity test requirements for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | agreed | R5-240463 | - |
| R5-241737 | Correction to Reference sensitivity for CA_n28A-n41A-n79A | Anritsu | agreed | R5-241097 | - |
| R5-241738 | Clarification of asymmetric BW in Rx test cases for CA | Anritsu | agreed | R5-241098 | - |
| R5-241739 | Updates to NR-U Tx test cases | Ericsson | agreed | R5-240810 | - |
| R5-241740 | Addition of test case 6.5F.3.3 Additional spurious emissions for shared spectrum channel access | Ericsson | agreed | R5-241177 | - |
| R5-241741 | Updates of test case 6.2F.3 UE additional maximum output power reduction for shared spectrum access | Ericsson | agreed | R5-241195 | - |
| R5-241742 | Updates of test case 6.5F.2.4.2, Shared spectrum channel access ACLR with additional requirement for NS_29 | Ericsson | agreed | R5-241258 | - |
| R5-241743 | Updates to TP analysis for FR2 RF phase continuity test | Apple Benelux B.V. | agreed | R5-241436 | - |
| R5-241744 | Updates to TP analysis for FR1 RF phase continuity test | Apple Benelux B.V. | agreed | R5-241437 | - |
| R5-241745 | Addition of n5 into TC 6.2D.1 MOP for UL MIMO | China Unicom | agreed | R5-241261 | - |
| R5-241746 | Addition of band CA_n14A-n77A PC2 reference sensitivity test | WE Certification, AT&T | agreed | R5-240896 | - |
| R5-241747 | Addition of CA_n14A-n77A PC2 and CA_n77(2A) PC2 to Ch 5 | WE Certification Oy, AT&T | agreed | R5-240893 | - |
| R5-241748 | Update reference sensitivity test cases for additional band configurations with PC2 UL | Verizon Spain | agreed | R5-240895 | - |
| R5-241749 | Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC2 | NTT DOCOMO, INC. | agreed | R5-241054 | - |
| R5-241750 | Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC2 | NTT DOCOMO, INC. | agreed | R5-241056 | - |
| R5-241751 | Introduction of MOP TC for ATG UE | CMCC | agreed | R5-240051 | - |
| R5-241752 | Introduction of Configured transmitted power TC for ATG UE | CMCC | agreed | R5-240052 | - |
| R5-241753 | Introduction of Occupied bandwidth TC for ATG UE | CMCC | agreed | R5-240054 | - |
| R5-241754 | Introduction of General Spurious emissions TC for ATG UE | CMCC | agreed | R5-240057 | - |
| R5-241755 | Addition of new test case 6.3J.2 Transmit OFF power for ATG | CAICT | agreed | R5-240264 | - |
| R5-241756 | Addition of new test case 6.3J.1 Minimum output power for ATG | CAICT | agreed | R5-240265 | - |
| R5-241757 | Introduction of Spurious response TC for ATG UE | CMCC | agreed | R5-240062 | - |
| R5-241758 | Introduction of Wide band intermodulation TC for ATG UE | CMCC | agreed | R5-240064 | - |
| R5-241759 | Introduction of Spurious emissions TC for ATG UE | CMCC | agreed | R5-240065 | - |
| R5-241760 | Addition of new test case 7.4J Maximum input level for ATG | CAICT | agreed | R5-240266 | - |
| R5-241761 | Addition of new test case 7.5J Adjacent channel selectivity for ATG | CAICT | agreed | R5-240267 | - |
| R5-241762 | Addition of new test case 7.6J.2 In-band blocking for ATG | CAICT | agreed | R5-240268 | - |
| R5-241763 | Addition of new test case 7.6J.3 Out-of-band blocking for ATG | CAICT | agreed | R5-240269 | - |
| R5-241764 | Addition of Measurement Uncertainties and Test Tolerances for ATG UE | CMCC | agreed | R5-240334 | - |
| R5-241765 | Addition of test points analysis for ATG test cases in 38.521-1 | CAICT, CMCC | agreed | R5-240270 | - |
| R5-241766 | p-Max and p-NR-FR1 adjustment when higherPowerLimit-r17 applies | Keysight Technologies UK Ltd, Mediatek | agreed | R5-240625 | - |
| R5-241767 | Clarification of trace mode in emission testing_FR1 | Huawei, HiSilicon | agreed | R5-240992 | - |
| R5-241768 | Update to AMPR, ASEM and ASE for intra-band CA for CA_NS_04 | Huawei, HiSilicon | agreed | R5-240998 | - |
| R5-241769 | Editorial correction in FR1 test case 6.5A.2.4.1.1 | Keysight Technologies UK Ltd | agreed | R5-241155 | - |
| R5-241770 | Correcting errors in REFSENS for CA test case for PC3 CA configurations | Huawei, HiSilicon | revised | R5-240314 | R5-241871 |
| R5-241771 | Updating REFSENS testing for SUL configuration | Huawei, HiSilicon | revised | R5-240317 | R5-241872 |
| R5-241772 | Correction to Reference sensitivity power level test configuration for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | agreed | R5-240455 | - |
| R5-241773 | Addition of CBW 35 MHz, 45 MHz, 70 MHz to Narrow band blocking for inter-band CA | Anritsu, Rohde&Schwarz | agreed | R5-241106 | - |
| R5-241774 | Correction of 7.6A.2 for inband blocking for CA | ZTE Corporation, Rohde&Schwarz | agreed | R5-241196 | - |
| R5-241775 | General updates of clause 5 for R17 new CBW configurations | CU Digital Technology, Nokia | agreed | R5-240231 | - |
| R5-241776 | Added 30kHz SCS for SSB in n53 to be aligned with core specs | Keysight Technologies UK Ltd, Apple | agreed | R5-240624 | - |
| R5-241777 | Correction to reference sensitivity test point analysis for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | agreed | R5-240456 | - |
| R5-241778 | Test point analysis of EN-DC spurious emissions for DC_2A_n66A and DC_30A_n66A | WE Certification Oy, AT&T | agreed | R5-240881 | - |
| R5-241779 | Corrections for 6.3G.3.3 | Keysight Technologies | agreed | - | - |
| R5-241780 | Updates to FR2 ACS test | Apple Benelux B.V. | agreed | R5-241431 | - |
| R5-241781 | Corrections on 6.2.3 for FR2 Redcap UE MPR test case for NS_202 and NS_203 | ZTE Corporation | agreed | R5-240837 | - |
| R5-241782 | Addition of CA test for EIRP test with ULGaps | Apple Trading | agreed | R5-241448 | - |
| R5-241783 | Clarification of trace mode in emission testing_FR2 | Huawei, HiSilicon | agreed | R5-240993 | - |
| R5-241784 | Clarification of trace mode in emission testing_Iw | Huawei, HiSilicon | agreed | R5-240994 | - |
| R5-241785 | Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs | NTT DOCOMO INC.. | agreed | R5-241079 | - |
| R5-241786 | Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs | NTT DOCOMO INC.. | agreed | R5-241080 | - |
| R5-241787 | Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs | NTT DOCOMO INC.. | agreed | R5-241081 | - |
| R5-241788 | Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs | NTT DOCOMO INC.. | agreed | R5-241082 | - |
| R5-241789 | Correct clause 6.2B.4.1.2 initial conditions | SGS Wireless | agreed | R5-240967 | - |
| R5-241790 | Editorial correction to FR1 inter-band co-existence | Rohde & Schwarz | agreed | R5-241172 | - |
| R5-241791 | On the splitting of the IoT NTN Frequency Error test cases | ROHDE & SCHWARZ | noted | R5-241357 | - |
| R5-241792 | Neighbor GSO satellite ephemeris file for NTN NR UE testing | THALES | noted | R5-241413 | - |
| R5-241793 | Common Test environment updates for IoT NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | agreed | R5-241390 | - |
| R5-241794 | Editorial corrections for NB-IoT NTN TA cases and RLM cases | MediaTek Beijing Inc. | agreed | R5-240023 | - |
| R5-241795 | Editorial removal to editors notes of DCQR cases | MediaTek (Hefei) Inc., Rohde & Schwarz | agreed | R5-240214 | - |
| R5-241796 | Editorial corrections to NB-IOT NTN RRM TCs | Rohde & Schwarz | agreed | R5-241247 | - |
| R5-241797 | Editorial correction to the wrong table number in 36.521-4 annex C | MediaTek (Hefei) Inc. | agreed | R5-240031 | - |
| R5-241798 | Update of Annex F Measurement Uncertainties in TS 36.521-4 | CMCC, Keysight Technologies UK Ltd | agreed | R5-240072 | - |
| R5-241799 | Editorial correction to the wrong citation number | MediaTek (Hefei) Inc. | agreed | R5-240097 | - |
| R5-241800 | Editorial alignment for the test applicability | MediaTek (Hefei) Inc. | agreed | R5-240098 | - |
| R5-241801 | Update of reference measurement channels in Annex A.3.12 | MediaTek (Hefei) Inc. | agreed | R5-240099 | - |
| R5-241802 | Updates to IoT NTN Frequency error test | Keysight Technologies UK Ltd | withdrawn | R5-241388 | - |
| R5-241803 | Update of grouping of test cases defined in TS 36.521-4 | CMCC, MediaTek | agreed | R5-240076 | - |
| R5-241804 | Clear-up CR for Editor notes of applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | agreed | R5-240074 | - |
| R5-241805 | Corrections on 6.2.3 for additional maximum output power reduction | ZTE Corporation | agreed | R5-240842 | - |
| R5-241806 | Corrections on 6.3.3 for Transmit ON OFF time mask | ZTE Corporation | agreed | R5-240844 | - |
| R5-241807 | Corrections on 6.5.3 for spurious emission | ZTE Corporation | agreed | R5-240847 | - |
| R5-241808 | Updates to NR NTN Frequency error test | Keysight Technologies UK Ltd | withdrawn | R5-241395 | - |
| R5-241809 | Corrections on 4.6.2 and 4.6.3 for the conditions in SIB and RRC IE | ZTE Corporation | withdrawn | R5-240839 | - |
| R5-241810 | Update of chapter 4 for RF general description | ZTE Corporation | agreed | R5-240855 | - |
| R5-241811 | Addition of MMSE-IRC CQI reporting test case with FDD 4Rx | China Telecom | agreed | R5-240251 | - |
| R5-241812 | Addition of MMSE-IRC CQI reporting test case with TDD 2Rx | China Telecom | agreed | R5-240252 | - |
| R5-241813 | Addition of MMSE-IRC CQI reporting test case with TDD 4Rx | China Telecom | agreed | R5-240253 | - |
| R5-241814 | Completion of MMSE-IRC CQI reporting test case with FDD 2Rx | China Telecom | agreed | R5-240254 | - |
| R5-241815 | Update to FR1 PDSCH inter-cell interference test case | QUALCOMM Europe Inc. - Italy | agreed | R5-241151 | - |
| R5-241816 | Update to FR1 PDSCH intra-cell interference test case | QUALCOMM Europe Inc. - Italy | agreed | R5-241152 | - |
| R5-241817 | Addition of RedCap demod test case 6.3.1.2.1 | Huawei, HiSilicon | agreed | R5-240678 | - |
| R5-241818 | Corrections to URLLC Test Cases | Rohde & Schwarz | agreed | R5-241169 | - |
| R5-241819 | Addition of SDR RMC for more FDD channel BW | QUALCOMM Europe Inc. - Italy | agreed | R5-241137 | - |
| R5-241820 | Correction to max throughput values for PDSCH RMC | QUALCOMM Europe Inc. - Italy | agreed | R5-241418 | - |
| R5-241821 | Update to NR-U demod test cases | QUALCOMM Europe Inc. - Italy | agreed | R5-241139 | - |
| R5-241822 | Core spec alignment, addition of RMC and UL-DL configuration for NR-U | Ericsson | agreed | R5-240830 | - |
| R5-241823 | Discussion on MU impact to energy detection threshold in shared spectrum | Qualcomm Germany | noted | R5-241294 | - |
| R5-241824 | Correction to gap offset and SMTC configuration in 16.6.2.9 and 16.6.2.10 | Anritsu | withdrawn | R5-241126 | - |
| R5-241825 | Correction to SSB configuration in 16.6.2.11 and 16.6.2.12 | Anritsu | withdrawn | R5-241127 | - |
| R5-241826 | Removal of Editor Note affecting to TC 5.5.5.9 | Keysight Technologies | agreed | R5-240890 | - |
| R5-241827 | Addition of NR_U test case 11.5.2.3 | Ericsson | agreed | R5-240808 | - |
| R5-241828 | Core alignment for NSA NR-U tests | Qualcomm Germany | agreed | R5-241271 | - |
| R5-241829 | Correction to RRM enh test case 6.6.2.9 | Huawei,HiSilicon | agreed | R5-240701 | - |
| R5-241830 | Addition of new test case 4.7.8.1 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Sporton | agreed | R5-241197 | - |
| R5-241831 | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.9.2 | Apple | agreed | R5-241334 | - |
| R5-241832 | Cell configuration mapping for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1 | China Telecom, Huawei, HiSilicon | agreed | R5-240371 | - |
| R5-241833 | Correct Table 4.3.2.2.1.4.1-2 & Table 4.3.2.2.2.4.1-2 & Table 4.3.2.2.3.4.1-2 & Table 4.3.2.2.4.4.1-2 of test frequency | SGS Wireless | agreed | R5-240968 | - |
| R5-241834 | Correction to test applicability of 4.6.1.7 | Anritsu, MediaTek Beijing Inc. | agreed | R5-241119 | - |
| R5-241835 | Correction for test case 5.5.6.2.1 | Rohde & Schwarz | agreed | R5-241232 | - |
| R5-241836 | Corrections to BFD-LR test case 5.5.5.5 | Rohde & Schwarz | agreed | R5-241234 | - |
| R5-241837 | Corrections to test procedure for 2-step RACH TC 6.3.2.2.4 | Rohde & Schwarz | agreed | R5-241231 | - |
| R5-241838 | Correction to Table H.3.1-8A | MediaTek Beijing Inc. | agreed | R5-240222 | - |
| R5-241839 | Correction to Table H.3.1-12 | MediaTek Beijing Inc. | agreed | R5-240223 | - |
| R5-241840 | Correction to Table H.3.1-12A | MediaTek Beijing Inc. | agreed | R5-240224 | - |
| R5-241841 | Update to test selection criteria for RRM tests | Qualcomm Germany | agreed | R5-241306 | - |
| R5-241842 | Addition of TC applicability statements for ATG UE | CMCC, CAICT | agreed | R5-240066 | - |
| R5-241843 | Update to R16 NR CADC configuration test cases applicability | CMCC, Sporton | agreed | R5-240089 | - |
| R5-241844 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | agreed | R5-241438 | - |
| R5-241845 | Addition of MMSE-IRC CQI reporting test applicability rule | China Telecom | agreed | R5-240255 | - |
| R5-241846 | Applicability update for PDSCH interference test cases | QUALCOMM Europe Inc. - Italy | agreed | R5-241153 | - |
| R5-241847 | Addition of applicability for FeMIMO test cases | Samsung | revised | R5-240505 | R5-241675 |
| R5-241848 | Addition of applicability for RedCap Demod and RRM test cases | Huawei, HiSilicon | agreed | R5-241446 | - |
| R5-241849 | Addition of event triggered reporting test cases applicability | Ericsson | agreed | R5-240791 | - |
| R5-241850 | Update to RRM Power saving enhancement 5.5.5.9 test case applicability | Keysight Technologies | agreed | R5-240889 | - |
| R5-241851 | Update to NR-U test applicability | Qualcomm Germany, Ericsson | agreed | R5-241287 | - |
| R5-241852 | General cleanup of annex for app layer tput test cases | QUALCOMM Europe Inc. - Italy | agreed | R5-241410 | - |
| R5-241853 | Update to CBW for multiples CA combos | Bureau Veritas ADT, Sporton International | agreed | R5-240940 | - |
| R5-241854 | Correct Table 7.3A.11.4.1-1 of CA Configurations | SGS Wireless | agreed | R5-240963 | - |
| R5-241855 | Correct Table 7.4A.8.4.1-2 & Table 7.4A.8_H.4.1-2 & Table 7.4A.9.4.1-1 of CA Configurations | SGS Wireless | agreed | R5-240964 | - |
| R5-241856 | Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations | SGS Wireless | agreed | R5-240965 | - |
| R5-241857 | Correct typos in Table 7.3A.5.4.1-1 | SGS Wireless | agreed | R5-240966 | - |
| R5-241858 | MU discussion on FR2c | Anritsu | noted | R5-241083 | - |
| R5-241859 | Update for FR2c MU | Anritsu, Keysight | agreed | R5-241084 | - |
| R5-241860 | FR2c MU - Rx test cases update in 38.521-2 | Keysight Technologies UK Ltd | withdrawn | R5-240404 | - |
| R5-241861 | Update for FR2c MU | Anritsu, Keysight | agreed | R5-241085 | - |
| R5-241862 | Introducing the Re-Positioning Concept for FR2 RRM TCs | Keysight Technologies UK Ltd, Rohde&Schwarz | agreed | R5-240602 | - |
| R5-241863 | FR2c MU definition in 38.903 | Keysight Technologies UK Ltd | agreed | R5-240402 | - |
| R5-241864 | CR to define FR2d QoQZ MUs and misc QoQZ MU corrections | Keysight Technologies UK Ltd | agreed | R5-240606 | - |
| R5-241865 | Update for FR2c MU | Anritsu | agreed | R5-241086 | - |
| R5-241866 | Documentation of MU for n259 | ROHDE & SCHWARZ | agreed | R5-241345 | - |
| R5-241867 | FR2 RRM test cases: Known Issue List | Ericsson | noted | R5-240786 | - |
| R5-241868 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | CU Digital Technology, Verizon, Nokia, KDDI | agreed | R5-240229 | - |
| R5-241869 | Updating test frequency for SUL band n83 20MHz CBW | Huawei, HiSilicon | agreed | R5-241716 | - |
| R5-241870 | Updating FR1 PC2 REFSENS exceptions testing | Huawei, HiSilicon | agreed | R5-241721 | - |
| R5-241871 | Correcting errors in REFSENS for CA test case for PC3 CA configurations | Huawei, HiSilicon | agreed | R5-241770 | - |
| R5-241872 | Updating REFSENS testing for SUL configuration | Huawei, HiSilicon | agreed | R5-241771 | - |
| R5-241873 | Add information to non-TXD test cases with UE supports TXD | Sporton | agreed | R5-241204 | - |
| R5-241874 | Update to Receiver test cases for multiple CA combos | Bureau Veritas ADT, Sporton International | agreed | R5-240942 | - |
| R5-241875 | Addition of missing section for RRM CA Configuration | Rohde & Schwarz | agreed | R5-241230 | - |
| R5-241876 | LS reply on FR2 ACS/IBB testing | TSG WG RAN5 | approved | - | - |
| R5-241877 | Corrections on 7.3.2 for Reference sensitivity power level | ZTE Corporation | agreed | R5-240849 | - |
| R5-241878 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | revised | R5-240732 | R5-241672 |
| R5-241879 | Removal of square brackets for Tx Diversity capability | Huawei, HiSilicon | agreed | R5-240337 | - |
| R5-241880 | General updates of Spurious emissions for UE co-existence for Inter-band CA | China Unicom | agreed | R5-241367 | - |
| R5-241881 | Addition of TTs for NR-U Test Cases 10.3.1.2 and 11.4.1.2 | Qualcomm Germany | agreed | R5-241288 | - |
| R5-241882 | Addition of TTs for NR-U Test Cases 10.3.1.3 and 11.4.1.3 | Qualcomm Germany | agreed | R5-241289 | - |
| R5-241883 | Addition of TTs for NR-U Test Cases 10.3.4.1, 10.3.4.2, 11.4.4.1 and 11.4.4.2 | Qualcomm Germany | agreed | R5-241290 | - |
| R5-241884 | Addition of TTs for NR-U Test Cases 10.3.5.2.1, 10.3.5.2.2, 11.4.5.2.1 and 11.4.5.2.2 | Qualcomm Germany | agreed | R5-241292 | - |
| R5-241885 | Addition of TTs for NR-U Test Cases 10.3.5.3.1 and 11.4.5.3.1 | Qualcomm Germany | agreed | R5-241293 | - |
| R5-241886 | Correction to RedCap RRM test case 16.6.5.3 with TT | Huawei, HiSilicon | agreed | R5-240708 | - |
| R5-241887 | TT analysis for RedCap RRM test case 16.7.3.3.1 | Huawei, HiSilicon | agreed | R5-240744 | - |
| R5-241888 | TT analysis for RedCap RRM test case 16.7.3.3.2 | Huawei, HiSilicon | agreed | R5-240745 | - |
| R5-241889 | TT analysis for RedCap RRM test case 16.7.3.4.1 | Huawei, HiSilicon | agreed | R5-240746 | - |
| R5-241890 | TT analysis for RedCap RRM test case 16.7.4.2.1 16.7.4.3.1 and 16.7.4.4.1 | Huawei, HiSilicon | agreed | R5-240750 | - |
| R5-241891 | TT analysis for RedCap RRM test case 16.7.4.2.2 16.7.4.3.2 and 16.7.4.4.2 | Huawei, HiSilicon | agreed | R5-240751 | - |
| R5-241892 | Update of TT analysis for 6.3.1.4 and 16.3.1.8 | Qualcomm Germany | agreed | R5-241303 | - |
| R5-241893 | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F | Apple | agreed | R5-241336 | - |
| R5-241894 | Addition of TT analysis grouping for test cases 5.5.6.3.1 to Table 8-2 | Apple | agreed | R5-241338 | - |
| R5-241895 | Addition of TT analysis grouping for test cases 5.5.6.4.1 to Table 8-2 | Apple | agreed | R5-241339 | - |
| R5-241896 | Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1 | China Telecom, Huawei, HiSilicon | agreed | R5-240370 | - |
| R5-241897 | Update TC 15.2.8 with TT analysis results | CATT | agreed | R5-240101 | - |
| R5-241898 | Update TC 15.2.9 with TT analysis results | CATT | agreed | R5-240102 | - |
| R5-241899 | Update TC 16.3.4 with TT analysis results | CATT | agreed | R5-240106 | - |
| R5-241900 | Addition of TT analysis for positioning test case 15.2.9 and 15.2.10 | CATT | agreed | R5-240130 | - |
| R5-241901 | Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell | Sporton | agreed | R5-241201 | - |
| R5-241902 | Correct of test tolerance analysis of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 | Sporton | agreed | R5-241199 | - |
| R5-241903 | Update of TP analysis for Spurious emissions test cases for FR1 UL CA | China Unicom | agreed | R5-241368 | - |
| R5-241904 | Addition of reference sensitivity for new PC2 EN-DC combos within FR1 | KDDI Corporation | agreed | R5-240332 | - |
| R5-241905 | Correction of applicability and test coverage rules for SA and NSA capable devices | CAICT | agreed | R5-240262 | - |
| R5-241906 | Aligning the applicability for NR NTN frequency error | ROHDE & SCHWARZ | withdrawn | R5-241362 | - |
| R5-241907 | Update of 4DL CA test cases | ROHDE & SCHWARZ, Verizon | agreed | R5-241349 | - |
| R5-241908 | Update of test point selection for 4DL CA configurations | ROHDE & SCHWARZ, Verizon | agreed | R5-241351 | - |
| R5-241909 | Addition of NR NTN capabilities | CAICT | agreed | R5-240399 | - |
| R5-241910 | Update of Applicability and Additional Information of RF conformance test cases for Satellite Access | CAICT, ROHDE & SCHWARZ, Keysight Technologies UK Ltd, Apple Benelux B.V., QUALCOMM Europe Inc. - Italy | agreed | R5-240400 | - |
| R5-241911 | Addition of new NR RSTD accuracy test case 14.5.4 in RRC_INACTIVE state | CATT | agreed | R5-240111 | - |
| R5-241912 | Correction to PRS-RSRPP positioning test cases | CATT, Spirent | agreed | R5-240121 | - |
| R5-241913 | Correction to PRS-RSRP positioning test cases | CATT, Spirent | agreed | R5-240122 | - |
| R5-241914 | Correction to NR RSTD positioning test cases | CATT, Spirent | agreed | R5-240123 | - |
| R5-241915 | Correction to NR UE Rx-Tx time difference positioning test cases | CATT, Spirent | agreed | R5-240124 | - |
| R5-241916 | Introduction of BDS B2a and B3I performance default test conditions in TS 37.571-5 | CATT, CAICT | agreed | R5-240127 | - |
| R5-241917 | Corrected CellIdentity for DL-TDOA measurement period test | CATT, Spirent | agreed | R5-240128 | - |
| R5-241918 | Update to R18 IoT NTN test cases applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | agreed | R5-240073 | - |
| R5-241919 | Introduction of RF Test Environment for ATG | CMCC | agreed | R5-240046 | - |
| R5-241920 | Introduction of RRM Test Environment for ATG | CMCC | agreed | R5-240047 | - |
| R5-241921 | Correction of Notes in tables of test channel bandwidths | CAICT | agreed | R5-240258 | - |
| R5-241922 | Correction to Rel-15 A-MPR | Anritsu | agreed | R5-241100 | - |
| R5-241923 | Correction of applicability for test with LTE anchor agnostic approach in 6.5B.3.3.1 | CAICT | agreed | R5-240263 | - |
| R5-241924 | Correction to p-Max value in out of band test cases for FR1 inter-band EN-DC | Anritsu | agreed | R5-241112 | - |
| R5-241925 | Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking | Qualcomm Technologies Ireland | agreed | R5-240952 | - |
| R5-241926 | Update of abbreviations for ATG | CMCC | agreed | - | - |
| R5-241927 | Addition of test frequencies for new EN-DC comb within FR2 | KDDI Corporation | agreed | R5-240398 | - |
| R5-241928 | Update for additional NR-CA and NR-DC band configurations | Verizon | agreed | R5-240861 | - |
| R5-241929 | CR on EIS search interpolation | ROHDE & SCHWARZ | agreed | R5-241206 | - |
| R5-241930 | Views on FR1 TRP TRS TT | Apple Benelux B.V. | noted | R5-241423 | - |
| R5-241931 | Update of TT within TRP and TRS tests | Apple Benelux B.V. | agreed | R5-241422 | - |
| R5-241932 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | agreed | R5-241421 | - |
| R5-241933 | Maintainence Work Plan for Rel17 TRP TRS | Apple Benelux B.V. | noted | R5-241427 | - |
| R5-241934 | Introduce Annex for maximum uncertainty of test system and test tolerance | Keysight Technologies UK Ltd | agreed | R5-240597 | - |
| R5-241935 | Align Test Case Structure to typical RAN5 spec | Keysight Technologies UK Ltd | agreed | R5-240600 | - |
| R5-241936 | Editorial update on Annex C | Apple (UK) Limited | agreed | R5-241047 | - |
| R5-241937 | Editorial update on Annex E | Apple (UK) Limited | agreed | R5-241048 | - |
| R5-241938 | Editorial update on clause 3 | Apple (UK) Limited | agreed | R5-241049 | - |
| R5-241939 | Editorial update on clause 4 | Apple (UK) Limited | agreed | R5-241050 | - |
| R5-241940 | CR on Measurement Distance | ROHDE & SCHWARZ | agreed | R5-241207 | - |
| R5-241941 | Addition of DL interruption allowed indication for CA_n1-n3-n78 | China Telecom | agreed | R5-240250 | - |
| R5-241942 | Discussion on the closure of R18 IoT NTN UEConTest WI | CMCC | noted | R5-240070 | - |
| R5-241943 | Ephemeris definition for IoT NTN RRM neighbour cells | Keysight Technologies UK Ltd, Thales | noted | R5-241381 | - |
| R5-241944 | Addition of test frequencies for CA n77(2A), BCS1, UL CA.. | Ericsson | agreed | R5-241363 | - |
| R5-241945 | Correction to note for inter-band CA including n48 | Anritsu | agreed | R5-241121 | - |
| R5-241946 | Update of applicability for FR1 4DL CA test cases | ROHDE & SCHWARZ | agreed | R5-241350 | - |
| R5-241947 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | agreed | R5-241432 | - |
| R5-241948 | Updates and corrections to Annex in FR1 RF spec | Apple Benelux B.V. | agreed | R5-241434 | - |
| R5-241949 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | agreed | R5-241433 | - |
| R5-241950 | Updates to Annex E content and structure | Apple Benelux B.V. | agreed | R5-241435 | - |
| R5-241951 | Editorial correction of test environment for IoT NTN RF testing | ROHDE & SCHWARZ | agreed | R5-241347 | - |
| R5-241952 | Updates to Test environment for eMTC NTN RRM testing | Keysight Technologies UK Ltd, Thales | agreed | R5-241382 | - |
| R5-241953 | Aligning the applicability for IoT NTN frequency error | ROHDE & SCHWARZ | agreed | R5-241359 | - |
| R5-241954 | Splitting the IoT NTN frequency error test case | ROHDE & SCHWARZ, Keysight Technologies UK Ltd | agreed | R5-241358 | - |
| R5-241955 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | agreed | R5-241391 | - |
| R5-241956 | LS to RAN4 on TT work for Rel-18 NR FR1 TRP TRS | TSG WG RAN5 | approved | - | - |
| R5-241957 | LS on 3GPP 5G NR FR1 OTA Conformance Test Specification | TSG WG RAN5 | approved | - | - |
| R5-241958 | Addition of feMIMO physical layer baseline implementation capabilities | Samsung | revised | R5-240506 | R5-241674 |
| R5-241959 | Adding measurement uncertainty and test tolerance for Demod PDCCH with intra-slot repetition test cases | Samsung | agreed | R5-240507 | - |
| R5-241960 | Addition of RedCap demod test case 6.3.1.1.1 | Huawei, HiSilicon | agreed | R5-240677 | - |
| R5-241961 | Addition of RedCap demod test case 6.3.2.2.7 | Huawei, HiSilicon | agreed | R5-240679 | - |
| R5-241962 | Addition of missing applicability to new SS-RSRQ RedCap test cases | ROHDE & SCHWARZ | agreed | R5-241240 | - |
| R5-241963 | Correction to RedCap RRM test case 16.7.3.4.2 with TT | Huawei,HiSilicon | agreed | R5-240719 | - |
| R5-241964 | Correction to test configuration in 16.4.1.2 and 16.5.2.2 | Anritsu | agreed | R5-241124 | - |
| R5-241965 | Correction to PDCCH level in CSI-RS based RLM test cases for RedCap | Anritsu | agreed | R5-241125 | - |
| R5-241966 | Update to FR2 Tx Power test with UL-Gaps | Apple Benelux B.V. | agreed | R5-241429 | - |
| R5-241967 | Correction to PDCCH level in 6.5.1.9 | Anritsu | agreed | R5-241123 | - |
| R5-241968 | Update of PDSCH Config for RF test cases | ROHDE & SCHWARZ | agreed | R5-241346 | - |
| R5-241969 | Correction to test configuration in 6.2D.2 | Anritsu | agreed | R5-241102 | - |
| R5-241970 | Addition of 6.2H.1.3 AMPR for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241009 | - |
| R5-241971 | Addition of 6.4H.1.1 frequency error for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241011 | - |
| R5-241972 | Addition of 6.4H.1.2.1 EVM for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241012 | - |
| R5-241973 | Addition of 6.4H.1.2.2 Carrier leakage for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241013 | - |
| R5-241974 | Addition of 6.4H.1.2.3 In-band emission for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241014 | - |
| R5-241975 | Addition of 6.4H.1.3 time alignment error for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241015 | - |
| R5-241976 | Addition of 6.4H.1.4 Coherent requirement for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241016 | - |
| R5-241977 | Addition of 6.5H.1.1 Occupied bandwidth for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241017 | - |
| R5-241978 | Addition of 6.5H.1.2.2 ASEM for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241018 | - |
| R5-241979 | Addition of 6.5H.1.3.1 General spurious emission for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241019 | - |
| R5-241980 | Addition of 6.5H.1.3.2 Spurious emission UE co-existence for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241020 | - |
| R5-241981 | Addition of 6.5H.1.3.3 Additional spurious emission for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241021 | - |
| R5-241982 | Addition of 6.5H.1.4 Transmit intermodulation for CA with UL-MIMO | Huawei, HiSilicon | agreed | R5-241022 | - |
| R5-241983 | Addition of 6.3A.3.4 1Tx-2Tx UL switching between two bands | Huawei, HiSilicon | agreed | R5-241024 | - |
| R5-241984 | Addition of 6.3A.3.5 2Tx-2Tx UL switching between two bands | Huawei, HiSilicon | agreed | R5-241025 | - |
| R5-241985 | Addition of 6.3C.3.3 2Tx-2Tx UL switching between two uplink carriers in SUL configuration | Huawei, HiSilicon | agreed | R5-241026 | - |
| R5-241986 | Addition of 6.3C.3.4 1Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | agreed | R5-241027 | - |
| R5-241987 | Addition of 6.3C.3.5 2Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | agreed | R5-241028 | - |
| R5-241988 | Update to test applicability for R17 FR1 enhancement | Huawei, HiSilicon | agreed | R5-241030 | - |
| R5-241989 | Addition of REFSENS for 3CC EN-DC in PC2 | NTT DOCOMO INC., Verizon | agreed | R5-241068 | - |
| R5-241990 | Adding FR2 test case of SRS time mask | Huawei, HiSilicon | agreed | R5-240995 | - |
| R5-241991 | Removal of LTE anchor agnostic approach testing in 6.5B.3.3.2 | CAICT | agreed | R5-240260 | - |
| R5-241992 | Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3 | NTT DOCOMO INC. | agreed | R5-241066 | - |
| R5-241993 | Update of Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3 | CAICT | agreed | R5-240261 | - |
| R5-241994 | Correction to EN-DC and NR SA SCell activation test cases | Huawei,HiSilicon,Starpoint | agreed | R5-240703 | - |
| R5-241995 | Correction to PCI updating formulas in RRM test cases | Huawei, HiSilicon, Starpoint | agreed | R5-240704 | - |
| R5-241996 | Removal of Editors note for RRM FR2 RLM in-sync test cases | Rohde & Schwarz | agreed | R5-241236 | - |
| R5-241997 | Revision of Editors note for RRM FR2 RLM test cases | Rohde & Schwarz | agreed | R5-241237 | - |
| R5-241998 | Correction to TRS Configuration in 6.3.2.1.3 | Anritsu | agreed | R5-241129 | - |
| R5-241999 | Correction to PDCCH level in 6.5.1.8 | Anritsu | agreed | R5-241130 | - |
| R5-242000 | Addition of TTs for NR-U Test Cases 10.3.5.1 and 11.4.5.1 | Qualcomm Germany | agreed | R5-241291 | - |
| R5-242001 | Ephemeris definition for NR NTN RRM neighbor cells | Keysight Technologies UK Ltd, Thales | noted | R5-241392 | - |
| R5-242002 | Updates to Test environment for NB-IoT NTN RRM testing | Keysight Technologies UK Ltd, Thales | agreed | R5-241383 | - |
| R5-242003 | Correction to UE timing advance for satellite access TC 13.4.2.1 | Keysight Technologies UK Ltd | agreed | R5-241176 | - |
| R5-242004 | Addition and correction to the NTN related abbreviations in 36.521-3 | MediaTek (Hefei) Inc. | agreed | R5-240200 | - |
| R5-242005 | Addition and correction to the NTN related abbreviations in 36.521-4 | MediaTek (Hefei) Inc. | agreed | R5-240198 | - |
| R5-242006 | UL RMCs updates for IoT NTN | Keysight Technologies UK Ltd | agreed | R5-241384 | - |
| R5-242007 | Correction of test environment for NR NTN RF testing | ROHDE & SCHWARZ | agreed | R5-241355 | - |
| R5-242008 | Update to NTN Add Spurious Emission TC | Qualcomm Technologies Ireland | agreed | R5-240956 | - |
| R5-242009 | Update to NTN General Spurious emission TC | Qualcomm Technologies Ireland | agreed | R5-240957 | - |
| R5-242010 | Updates to NTN Spur emission UE Coex | Qualcomm Technologies Ireland | agreed | R5-240958 | - |
| R5-242011 | Update to NTN Tx Intermod TC | Qualcomm Technologies Ireland, ZTE | agreed | R5-240959 | - |
| R5-242012 | Update to NTN Annex F MU TT | Qualcomm Technologies Ireland | agreed | R5-240961 | - |
| R5-242013 | Splitting the NR NTN frequency error test case | ROHDE & SCHWARZ | agreed | R5-241361 | - |
| R5-242014 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | agreed | R5-241401 | - |
| R5-242015 | Correction to Reference sensitivity for CA_n26A-n70A | Anritsu | agreed | R5-241122 | - |
| R5-242016 | Updates to Test environment for NR NTN RRM testing | Keysight Technologies UK Ltd, Thales | agreed | R5-241393 | - |
| R5-242017 | Common Test environment updates for NR NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | agreed | R5-241400 | - |
| R5-242018 | UL RMCs updates for NR NTN | Keysight Technologies UK Ltd | agreed | R5-241394 | - |
| R5-242019 | Correction to RMC name in Table A.3.2.1.1-18 | QUALCOMM Europe Inc. - Italy | agreed | R5-241147 | - |
| R5-242020 | Editorial correction to Additional spurious emissions TCS | MediaTek Beijing Inc. | agreed | R5-240137 | - |
| R5-242021 | Correction to test parameters in 5.6.1.3 and 5.6.1.4 | Anritsu | agreed | R5-241128 | - |
| R5-242022 | Correction to message exceptions for Performance test cases | Anritsu | agreed | R5-241095 | - |
| R5-242023 | Introduction of common ICS for PC1.5 n39 | CMCC | agreed | R5-240079 | - |
| R5-242024 | FR2 UL Gaps measurement uncertainties and test tolerances | Keysight Technologies UK Ltd, Apple Inc | noted | R5-241403 | - |
| R5-242025 | Updates to UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | agreed | R5-241406 | - |
| R5-242026 | Updates to Annex F for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | agreed | R5-241405 | - |
| R5-242027 | Updates to 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | agreed | R5-241408 | - |
| R5-242028 | Updates to Annex F for 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | agreed | R5-241407 | - |
| R5-242029 | Measurement uncertainty definition for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | agreed | R5-241404 | - |
| R5-242030 | Introduction of General description for ATG UE Tx TCs | CMCC | revised | R5-240050 | R5-241683 |
| R5-242031 | Addition and correction to the NTN related abbreviations in 38.521-5 | MediaTek (Hefei) Inc. | withdrawn | R5-240201 | - |
| R5-242032 | Addition and correction to the NTN related abbreviations in 38.533 | MediaTek (Hefei) Inc. | withdrawn | R5-240202 | - |
| R5-242033 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE test case 16.6.3.1 including Test Tolerance | Ericsson | agreed | R5-240794 | - |
| R5-242034 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE test case 16.6.3.2 including Test Tolerance | Ericsson | agreed | R5-240795 | - |
| R5-242035 | Addition of NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE test case 16.6.3.3 including Test Tolerance | Ericsson | agreed | R5-240796 | - |
| R5-242036 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2Rx UE test case 16.6.3.4 including Test Tolerance | Ericsson | agreed | R5-240797 | - |
| R5-242037 | Correction of Redcap inter-RAT E-UTRAN CGI test cases | MediaTek Inc. | agreed | R5-240204 | - |
| R5-242038 | Correction to default configuration of SMTC for NCD-SSB | Huawei, HiSilicon, Starpoint | agreed | R5-240676 | - |
| R5-242039 | Correction to NCD-SSB and SMTC RMCs | Huawei, HiSilicon, Starpoint | agreed | R5-240693 | - |
| R5-242040 | Correction to RedCap inter-frequency measurement test cases | Huawei, HiSilicon, Starpoint, Anritsu, Keysight | agreed | R5-240694 | - |
| R5-242041 | Correction to RedCap RRM test case 16.7.2.3.2 with TT | Huawei,HiSilicon | agreed | R5-240711 | - |
| R5-242042 | Correction to RedCap RRM test case 16.7.2.4.1 with TT | Huawei, HiSilicon | agreed | R5-240712 | - |
| R5-242043 | Correction to RedCap RRM test case 16.7.2.4.2 with TT | Huawei,HiSilicon | agreed | R5-240713 | - |
| R5-242044 | TT analysis for RedCap RRM test case 16.7.2.3.2 | Huawei, HiSilicon | agreed | R5-240740 | - |

## Annex B: List of change requests

1736 CRs and their final revisions at RAN5#102 (+648 intermediate revisions are not shown)

| Document | Title | Source | Spec | CR | Rev | Rel | Cat | WI | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R5-240550 | Correction to UTRAN Inter-RAT TC 8.6.3.3 | MCC TF160 | 34.123-1 | 3939 | - | Rel-15 | F | TEI10_Test | agreed |
| R5-240975 | Removal of GEA2 support from inter-rat cell change order test cases | Qualcomm Incorporated | 34.123-1 | 3940 | - | Rel-15 | F | TEI_Test | agreed |
| R5-240976 | Removal of GEA2 support from applicability of inter-rat cell change order test cases | Qualcomm Incorporated | 34.123-2 | 0801 | - | Rel-15 | F | TEI_Test | agreed |
| R5-240551 | Removal of GEA2 from TS 34.123-3 | MCC TF160 | 34.123-3 | 3676 | - | Rel-17 | F | TEI_Test | agreed |
| R5-240552 | Correction of clause 4 | MCC TF160 | 34.229-2 | 0336 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241411 | Updating applicability for RedCap | MediaTek Inc. | 34.229-2 | 0337 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241578 | Updating applicability for RedCap | MediaTek Inc. | 34.229-2 | 0337 | 1 | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241417 | Correction to Video capability | MediaTek Inc. | 34.229-2 | 0338 | - | Rel-16 | F | TEI16_Test | withdrawn |
| R5-241579 | Updating applicability for video | MediaTek Inc. | 34.229-2 | 0339 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240866 | Correction to 34229-5 A.4.2a | Huawei, Hisilicon | 34.229-5 | 0568 | - | Rel-16 | F | TEI16_Test, NR_EIEI-UEConTest | withdrawn |
| R5-241190 | Update to generic procedure A.4.2a | Ericsson | 34.229-5 | 0569 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241191 | Updates to test case 7.1 | Ericsson | 34.229-5 | 0570 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241192 | Updates to test case 7.5 | Ericsson | 34.229-5 | 0571 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241655 | Updates to test case 7.5 | Ericsson | 34.229-5 | 0571 | 1 | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241193 | Updates to test case 7.12 | Ericsson | 34.229-5 | 0572 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241656 | Updates to test case 7.12 | Ericsson | 34.229-5 | 0572 | 1 | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241252 | New generic procedure A.15.2a | Ericsson | 34.229-5 | 0573 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241554 | New generic procedure A.15.2a | Ericsson | 34.229-5 | 0573 | 1 | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241253 | New generic procedure A.16.2a | Ericsson | 34.229-5 | 0574 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241555 | New generic procedure A.16.2a | Ericsson | 34.229-5 | 0574 | 1 | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241254 | Updates to test case 7.15 | Ericsson | 34.229-5 | 0575 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241255 | Updates to test case 7.17 | Ericsson | 34.229-5 | 0576 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241256 | Updates to test case 7.21 | Ericsson | 34.229-5 | 0577 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241257 | Updates to test case 7.23 | Ericsson | 34.229-5 | 0578 | - | Rel-16 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240075 | Update to common requirements of test equipment for IoT-NTN | CMCC | 36.508 | 1454 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240157 | Correction to NGSO abbreviation | MediaTek Inc. | 36.508 | 1455 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240158 | Update ephemeris information for NGSO signalling test environment | MediaTek Inc. | 36.508 | 1456 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241556 | Update ephemeris information for NGSO signalling test environment | MediaTek Inc. | 36.508 | 1456 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240215 | Addition of ephemeris information for GEO condition | MediaTek (Hefei) Inc. | 36.508 | 1457 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240221 | Update of UE pre-configuration for NGSO signalling test environment | MediaTek Inc. | 36.508 | 1458 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241557 | Update of UE pre-configuration for NGSO signalling test environment | MediaTek Inc. | 36.508 | 1458 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-240290 | Correction to System information for NTN | MediaTek Inc. | 36.508 | 1459 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241558 | Correction to System information for NTN | MediaTek Inc. | 36.508 | 1459 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240527 | Definition of a TN-to-NTN sysInfo combination mapping for legacy test cases to be run in NTN environment | MCC TF160 | 36.508 | 1460 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241559 | Definition of a TN-to-NTN sysInfo combination mapping for legacy test cases to be run in NTN environment | MCC TF160 | 36.508 | 1460 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-240939 | Additional CA_30A-48A for CA signalling test | Bureau Veritas ADT, Sporton International, CATT | 36.508 | 1461 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-240973 | Addition of conditional PSCell addition and change configuration for EN-DC | Huawei, HiSilicon | 36.508 | 1462 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-241712 | Addition of conditional PSCell addition and change configuration for EN-DC | Huawei, HiSilicon | 36.508 | 1462 | 1 | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240974 | Addition of SCG activation and deactivation configuration for EN-DC | Huawei, HiSilicon | 36.508 | 1463 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-241713 | Addition of SCG activation and deactivation configuration for EN-DC | Huawei, HiSilicon | 36.508 | 1463 | 1 | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-241347 | Editorial correction of test environment for IoT NTN RF testing | ROHDE & SCHWARZ | 36.508 | 1464 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241951 | Editorial correction of test environment for IoT NTN RF testing | ROHDE & SCHWARZ | 36.508 | 1464 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241382 | Updates to Test environment for eMTC NTN RRM testing | Keysight Technologies UK Ltd, Thales | 36.508 | 1465 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241952 | Updates to Test environment for eMTC NTN RRM testing | Keysight Technologies UK Ltd, Thales | 36.508 | 1465 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241383 | Updates to Test environment for NB-IoT NTN RRM testing | Keysight Technologies UK Ltd, Thales | 36.508 | 1466 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-242002 | Updates to Test environment for NB-IoT NTN RRM testing | Keysight Technologies UK Ltd, Thales | 36.508 | 1466 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241390 | Common Test environment updates for IoT NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | 36.508 | 1467 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241793 | Common Test environment updates for IoT NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | 36.508 | 1467 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241560 | Updates to the NTN test environment | MCC TF160, MediaTek | 36.508 | 1468 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240071 | Update of Annex F Measurement Uncertainties in TS 36.521-1 | CMCC, Keysight Technologies UK Ltd | 36.521-1 | 5487 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-240614 | E-UTRA V2X spurious emissions MU definition up to 26GHz | Keysight Technologies UK Ltd | 36.521-1 | 5488 | - | Rel-18 | F | TEI14_Test, LTE_SL_V2V-UEConTest | agreed |
| R5-240671 | Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking | Qualcomm Technologies Ireland | 36.521-1 | 5489 | - | Rel-16 | F | LTE_MTCe2_L1-UEConTest, LTE_feMTC-UEConTest, TEI16_Test | withdrawn |
| R5-240940 | Update to CBW for multiples CA combos | Bureau Veritas ADT, Sporton International | 36.521-1 | 5490 | - | Rel-18 | F | LTE_CA_R17-UEConTest | revised |
| R5-241853 | Update to CBW for multiples CA combos | Bureau Veritas ADT, Sporton International | 36.521-1 | 5490 | 1 | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-240941 | Update to Transmit test cases for CA_30A-48A | Bureau Veritas ADT, Sporton International | 36.521-1 | 5491 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-240942 | Update to Receiver test cases for multiple CA combos | Bureau Veritas ADT, Sporton International | 36.521-1 | 5492 | - | Rel-18 | F | LTE_CA_R17-UEConTest | revised |
| R5-241874 | Update to Receiver test cases for multiple CA combos | Bureau Veritas ADT, Sporton International | 36.521-1 | 5492 | 1 | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-240952 | Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking | Qualcomm Technologies Ireland | 36.521-1 | 5493 | - | Rel-18 | F | TEI13_Test, LTE_MTCe2_L1-UEConTest | revised |
| R5-241925 | Update Table notes in TS36.521-1 TC-7.6.3EA Table-7.6.3EA.3-1: Narrow-band blocking | Qualcomm Technologies Ireland | 36.521-1 | 5493 | 1 | Rel-18 | F | TEI13_Test, LTE_MTCe2_L1-UEConTest | agreed |
| R5-240963 | Correct Table 7.3A.11.4.1-1 of CA Configurations | SGS Wireless | 36.521-1 | 5494 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241854 | Correct Table 7.3A.11.4.1-1 of CA Configurations | SGS Wireless | 36.521-1 | 5494 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240964 | Correct Table 7.4A.8.4.1-2 & Table 7.4A.8_H.4.1-2 & Table 7.4A.9.4.1-1 of CA Configurations | SGS Wireless | 36.521-1 | 5495 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241855 | Correct Table 7.4A.8.4.1-2 & Table 7.4A.8_H.4.1-2 & Table 7.4A.9.4.1-1 of CA Configurations | SGS Wireless | 36.521-1 | 5495 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240965 | Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations | SGS Wireless | 36.521-1 | 5496 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241856 | Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations | SGS Wireless | 36.521-1 | 5496 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240966 | Correct typos in Table 7.3A.5.4.1-1 | SGS Wireless | 36.521-1 | 5497 | - | Rel-18 | F | TEI12_Test, LTE_CA_Rel12_3DL-UEConTest | revised |
| R5-241857 | Correct typos in Table 7.3A.5.4.1-1 | SGS Wireless | 36.521-1 | 5497 | 1 | Rel-18 | F | TEI12_Test, LTE_CA_Rel12_3DL-UEConTest | agreed |
| R5-240969 | Update to freq error to refer to cat 1bis refsens values table | Qualcomm Technologies Ireland | 36.521-1 | 5498 | - | Rel-18 | F | TEI14_Test, LTE_UE_cat_1RX-UEConTest | withdrawn |
| R5-241057 | Update to freq error cat 1bis test procedure | Qualcomm Technologies Ireland | 36.521-1 | 5499 | - | Rel-18 | F | TEI14_Test, LTE_UE_cat_1RX-UEConTest | agreed |
| R5-241089 | Editorial correction to CA_1A-3A-7A-28A in 7.3A.9 | Anritsu | 36.521-1 | 5500 | - | Rel-18 | F | TEI13_Test, LTE_CA_Rel13-UEConTest | agreed |
| R5-241168 | Correction to Cat1bis frequency error | Rohde & Schwarz | 36.521-1 | 5501 | - | Rel-18 | F | TEI13_Test, LTE_UE_cat_1RX-UEConTest | withdrawn |
| R5-240073 | Update to R18 IoT NTN test cases applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | 36.521-2 | 1021 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241918 | Update to R18 IoT NTN test cases applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | 36.521-2 | 1021 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240206 | Update PICS of NTN test cases | MediaTek (Hefei) Inc. | 36.521-2 | 1022 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240770 | Correction to applicability for handover test cases | TTA | 36.521-2 | 1023 | - | Rel-18 | F | TEI16_Test, LTE_feMob-UEConTest | agreed |
| R5-240833 | Update PRD20 E-UTRA CA list v160 | ZTE Corporation | 36.521-2 | 1024 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-240937 | Additional supported capabilities for multiple CA combos | Bureau Veritas ADT, Sporton International | 36.521-2 | 1025 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-241203 | Add information in Power Control test cases specific to Power Class 3 | Sporton | 36.521-2 | 1026 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-241359 | Aligning the applicability for IoT NTN frequency error | ROHDE & SCHWARZ | 36.521-2 | 1027 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241953 | Aligning the applicability for IoT NTN frequency error | ROHDE & SCHWARZ | 36.521-2 | 1027 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240023 | Editorial corrections for NB-IoT NTN TA cases and RLM cases | MediaTek Beijing Inc. | 36.521-3 | 2997 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241794 | Editorial corrections for NB-IoT NTN TA cases and RLM cases | MediaTek Beijing Inc. | 36.521-3 | 2997 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240024 | Update NB-IoT NTN UL timing accuracy cases | MediaTek Beijing Inc. | 36.521-3 | 2998 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240029 | Update NB-IoT NTN UL timing accuracy cases TT in Annex | MediaTek Beijing Inc. | 36.521-3 | 2999 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240030 | Update GNSS margin value for NB-IoT NTN timing accuracy test cases | MediaTek Beijing Inc. | 36.521-3 | 3000 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240035 | Add a chapter title for ReEST (CH13.3.1) | MediaTek Inc. | 36.521-3 | 3001 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240199 | Correction of NB-IoT NTN cell reselection test cases | MediaTek Inc. | 36.521-3 | 3002 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240200 | Addition and correction to the NTN related abbreviations in 36.521-3 | MediaTek (Hefei) Inc. | 36.521-3 | 3003 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-242004 | Addition and correction to the NTN related abbreviations in 36.521-3 | MediaTek (Hefei) Inc. | 36.521-3 | 3003 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240210 | Update ephemeris information of several RRM cases in 36.521-3 | MediaTek (Hefei) Inc. | 36.521-3 | 3004 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240211 | Update TT value to message content and test requirement of RA cases | MediaTek (Hefei) Inc. | 36.521-3 | 3005 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240214 | Editorial removal to editors notes of DCQR cases | MediaTek (Hefei) Inc., Rohde & Schwarz | 36.521-3 | 3006 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241795 | Editorial removal to editors notes of DCQR cases | MediaTek (Hefei) Inc., Rohde & Schwarz | 36.521-3 | 3006 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240216 | Update ephemeris to GEO condition test cases in 36.521-3 | MediaTek (Hefei) Inc. | 36.521-3 | 3007 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240219 | Update cell specific parameters for NB-IoT NTN RA test cases | MediaTek Beijing Inc. | 36.521-3 | 3008 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240615 | NTN NB-IoT - Message exception correction in RRM test case 13.1.1.2 | Keysight Technologies UK Ltd | 36.521-3 | 3009 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240922 | Editorial update for UE timing advance for satellite access TC 13.4.2.2 | Keysight Technologies UK Ltd | 36.521-3 | 3010 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241176 | Correction to UE timing advance for satellite access TC 13.4.2.1 | Keysight Technologies UK Ltd | 36.521-3 | 3011 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-242003 | Correction to UE timing advance for satellite access TC 13.4.2.1 | Keysight Technologies UK Ltd | 36.521-3 | 3011 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241244 | Core Spec alignment for s-IntraSearchP IE for Re-selection NB-IOT NTN RRM TCs | Rohde & Schwarz | 36.521-3 | 3012 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241245 | Re-Addition of omitted change for RRM test case 13.1.1.2 | Rohde & Schwarz | 36.521-3 | 3013 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241246 | Editorial corrections to NB-IOT NTN RRM Reselection test cases | Rohde & Schwarz | 36.521-3 | 3014 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241247 | Editorial corrections to NB-IOT NTN RRM TCs | Rohde & Schwarz | 36.521-3 | 3015 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241796 | Editorial corrections to NB-IOT NTN RRM TCs | Rohde & Schwarz | 36.521-3 | 3015 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241248 | Core Spec alignment for RRM NB-IOT NTN test cases | Rohde & Schwarz | 36.521-3 | 3016 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241249 | Core Spec alignment for test config for RRM NB-IOT NTN test cases | Rohde & Schwarz | 36.521-3 | 3017 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-241250 | Core Spec alignment for RRM NB-IOT NTN UL Timing test cases | Rohde & Schwarz | 36.521-3 | 3018 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241385 | Channel quality reporting updates | Keysight Technologies UK Ltd | 36.521-3 | 3019 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241386 | Radio Link Monitoring updates | Keysight Technologies UK Ltd | 36.521-3 | 3020 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241389 | Annex A.3 update | Keysight Technologies UK Ltd | 36.521-3 | 3021 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241676 | Annex A.3 update | Keysight Technologies UK Ltd | 36.521-3 | 3021 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240031 | Editorial correction to the wrong table number in 36.521-4 annex C | MediaTek (Hefei) Inc. | 36.521-4 | 0001 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241797 | Editorial correction to the wrong table number in 36.521-4 annex C | MediaTek (Hefei) Inc. | 36.521-4 | 0001 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240072 | Update of Annex F Measurement Uncertainties in TS 36.521-4 | CMCC, Keysight Technologies UK Ltd | 36.521-4 | 0002 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241798 | Update of Annex F Measurement Uncertainties in TS 36.521-4 | CMCC, Keysight Technologies UK Ltd | 36.521-4 | 0002 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240074 | Clear-up CR for Editor notes of applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | 36.521-4 | 0003 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241804 | Clear-up CR for Editor notes of applicability | CMCC, MediaTek, CAICT, Sporton, Keysight | 36.521-4 | 0003 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240097 | Editorial correction to the wrong citation number | MediaTek (Hefei) Inc. | 36.521-4 | 0004 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241799 | Editorial correction to the wrong citation number | MediaTek (Hefei) Inc. | 36.521-4 | 0004 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240098 | Editorial alignment for the test applicability | MediaTek (Hefei) Inc. | 36.521-4 | 0005 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241800 | Editorial alignment for the test applicability | MediaTek (Hefei) Inc. | 36.521-4 | 0005 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240099 | Update of reference measurement channels in Annex A.3.12 | MediaTek (Hefei) Inc. | 36.521-4 | 0006 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241801 | Update of reference measurement channels in Annex A.3.12 | MediaTek (Hefei) Inc. | 36.521-4 | 0006 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240136 | Editorial correction to 6.4B.1 Frequency Error for UE category NB1 and NB2 TC | MediaTek Beijing Inc. | 36.521-4 | 0007 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240137 | Editorial correction to Additional spurious emissions TCS | MediaTek Beijing Inc. | 36.521-4 | 0008 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-242020 | Editorial correction to Additional spurious emissions TCS | MediaTek Beijing Inc. | 36.521-4 | 0008 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240144 | Update content of Statistical testing of Performance Requirements with probability of misdetection in Annex G.4 | MediaTek (Hefei) Inc. | 36.521-4 | 0009 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240145 | Update TT value to NTN demod cases | MediaTek (Hefei) Inc. | 36.521-4 | 0010 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240198 | Addition and correction to the NTN related abbreviations in 36.521-4 | MediaTek (Hefei) Inc. | 36.521-4 | 0011 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-242005 | Addition and correction to the NTN related abbreviations in 36.521-4 | MediaTek (Hefei) Inc. | 36.521-4 | 0011 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240862 | Clarification on NPDSCH repetitions for Demod NB-IoT NTN test cases | Keysight Technologies | 36.521-4 | 0012 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241090 | Correction to UL RMC descriptions for NB-IoT NTN | Anritsu | 36.521-4 | 0013 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-241358 | Splitting the IoT NTN frequency error test case | ROHDE & SCHWARZ, Keysight Technologies UK Ltd | 36.521-4 | 0014 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241954 | Splitting the IoT NTN frequency error test case | ROHDE & SCHWARZ, Keysight Technologies UK Ltd | 36.521-4 | 0014 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241384 | UL RMCs updates for IoT NTN | Keysight Technologies UK Ltd | 36.521-4 | 0015 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-242006 | UL RMCs updates for IoT NTN | Keysight Technologies UK Ltd | 36.521-4 | 0015 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241387 | Updates to PDSCH RMC | Keysight Technologies UK Ltd | 36.521-4 | 0016 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241388 | Updates to IoT NTN Frequency error test | Keysight Technologies UK Ltd | 36.521-4 | 0017 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241802 | Updates to IoT NTN Frequency error test | Keysight Technologies UK Ltd | 36.521-4 | 0017 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-240159 | Correction to NB-IoT NTN TC 22.4.13a | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm | 36.523-1 | 5273 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240160 | Update of applicable legacy NB-IoT cases | MediaTek Inc., ROHDE & SCHWARZ | 36.523-1 | 5274 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-240418 | Addition of SENSE eMTC TC 6.1.1.12-PLMN selection | Huawei, HiSilicon | 36.523-1 | 5275 | - | Rel-18 | F | SENSE-UEContest | revised |
| R5-241611 | Addition of SENSE eMTC TC 6.1.1.12-PLMN selection | Huawei, HiSilicon | 36.523-1 | 5275 | 1 | Rel-18 | F | SENSE-UEContest | agreed |
| R5-240419 | Addition of SENSE eMTC TC 6.1.1.13-PLMN selection Exception | Huawei, HiSilicon | 36.523-1 | 5276 | - | Rel-18 | F | SENSE-UEContest | revised |
| R5-241612 | Addition of SENSE eMTC TC 6.1.1.13-PLMN selection Exception | Huawei, HiSilicon | 36.523-1 | 5276 | 1 | Rel-18 | F | SENSE-UEContest | agreed |
| R5-240420 | Addition of SENSE eMTC TC 6.1.1.14-Periodic SENSE | Huawei, HiSilicon | 36.523-1 | 5277 | - | Rel-18 | F | SENSE-UEContest | revised |
| R5-241613 | Addition of SENSE eMTC TC 6.1.1.14-Periodic SENSE | Huawei, HiSilicon | 36.523-1 | 5277 | 1 | Rel-18 | F | SENSE-UEContest | agreed |
| R5-240421 | Addition of SENSE NB-IoT TC 22.2.14-PLMN selection | Huawei, HiSilicon | 36.523-1 | 5278 | - | Rel-18 | F | SENSE-UEContest | revised |
| R5-241614 | Addition of SENSE NB-IoT TC 22.2.14-PLMN selection | Huawei, HiSilicon | 36.523-1 | 5278 | 1 | Rel-18 | F | SENSE-UEContest | agreed |
| R5-240422 | Addition of SENSE NB-IoT TC 22.2.15-PLMN selection Exception | Huawei, HiSilicon | 36.523-1 | 5279 | - | Rel-18 | F | SENSE-UEContest | revised |
| R5-241615 | Addition of SENSE NB-IoT TC 22.2.15-PLMN selection Exception | Huawei, HiSilicon | 36.523-1 | 5279 | 1 | Rel-18 | F | SENSE-UEContest | agreed |
| R5-240423 | Addition of SENSE NB-IoT TC 22.2.16-Periodic SENSE | Huawei, HiSilicon | 36.523-1 | 5280 | - | Rel-18 | F | SENSE-UEContest | revised |
| R5-241616 | Addition of SENSE NB-IoT TC 22.2.16-Periodic SENSE | Huawei, HiSilicon | 36.523-1 | 5280 | 1 | Rel-18 | F | SENSE-UEContest | agreed |
| R5-240474 | Correction to SIM profile of NB-IoT test cases | Keysight Technologies UK | 36.523-1 | 5281 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-240548 | Correction to LTE Inter-RAT TC 8.1.3.7 | MCC TF160 | 36.523-1 | 5282 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-240585 | Corrections to LTE TC 8.1.3.16 | MCC TF160 | 36.523-1 | 5283 | - | Rel-18 | F | TEI16_Test | agreed |
| R5-240586 | Corrections to NB-IoT RLC TC 22.3.2.3 | MCC TF160 | 36.523-1 | 5284 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | revised |
| R5-241478 | Corrections to NB-IoT RLC TC 22.3.2.3 | MCC TF160 | 36.523-1 | 5284 | 1 | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-240818 | Correction to NBIOT NTN testcase 22.4.13a | ROHDE & SCHWARZ | 36.523-1 | 5285 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-240820 | Correction to NBIOT testcase 22.4.13 | ROHDE & SCHWARZ | 36.523-1 | 5286 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | withdrawn |
| R5-240977 | Addition of new L2L MPS priority access barring test case | Qualcomm Incorporated | 36.523-1 | 5287 | - | Rel-18 | F | TEI16_Test | revised |
| R5-241475 | Addition of new L2L MPS priority access barring test case | Qualcomm Incorporated | 36.523-1 | 5287 | 1 | Rel-18 | F | TEI16_Test | agreed |
| R5-240979 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | 36.523-1 | 5288 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241563 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | 36.523-1 | 5288 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240980 | Correction to MAC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | 36.523-1 | 5289 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241564 | Correction to MAC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | 36.523-1 | 5289 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240981 | Correction to RLC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | 36.523-1 | 5290 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241647 | Correction to RLC test cases for NTN UEs | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | 36.523-1 | 5290 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-241040 | Addition of UAS EPS test case 10.10.2 | Qualcomm Incorporated | 36.523-1 | 5291 | - | Rel-18 | F | ID_UAS-UEConTest | revised |
| R5-241608 | Addition of UAS EPS test case 10.10.2 | Qualcomm Incorporated | 36.523-1 | 5291 | 1 | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-241041 | Addition of UAS EPS test case 10.10.4 | Qualcomm Incorporated | 36.523-1 | 5292 | - | Rel-18 | F | ID_UAS-UEConTest | revised |
| R5-241609 | Addition of UAS EPS test case 10.10.4 | Qualcomm Incorporated | 36.523-1 | 5292 | 1 | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-241042 | Addition of EPS UAS test case 10.10.6 | Qualcomm Incorporated | 36.523-1 | 5293 | - | Rel-18 | F | ID_UAS-UEConTest | revised |
| R5-241610 | Addition of EPS UAS test case 10.10.6 | Qualcomm Incorporated | 36.523-1 | 5293 | 1 | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-241065 | Addition of new test case 11.2.13 | NTT DOCOMO, INC. Rohde & Schwarz | 36.523-1 | 5294 | - | Rel-18 | F | TEI11_Test | revised |
| R5-241624 | Addition of new test case 11.2.13 | NTT DOCOMO, INC. Rohde & Schwarz | 36.523-1 | 5294 | 1 | Rel-18 | F | TEI11_Test | agreed |
| R5-241179 | Addition of new test case 8.1.3.8a Redirection to GERAN-Redir-policy bit | Vodafone | 36.523-1 | 5295 | - | Rel-18 | F | TEI14_Test | revised |
| R5-241476 | Addition of new test case 8.1.3.8a Redirection to GERAN-Redir-policy bit | Vodafone | 36.523-1 | 5295 | 1 | Rel-18 | F | TEI14_Test | agreed |
| R5-241180 | Addition of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone | 36.523-1 | 5296 | - | Rel-18 | F | TEI17_Test | revised |
| R5-241477 | Addition of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone | 36.523-1 | 5296 | 1 | Rel-18 | F | TEI17_Test | agreed |
| R5-241494 | Correction to RACS Test case 9.2.5.X | Anritsu Ltd | 36.523-1 | 5297 | - | Rel-18 | F | TEI16_Test, RACS-UEConTest | revised |
| R5-241627 | Correction to RACS Test case 9.2.5.X | Anritsu Ltd | 36.523-1 | 5297 | 1 | Rel-18 | F | TEI16_Test, RACS-UEConTest | revised |
| R5-241690 | Correction to RACS Test case 9.2.5.X | Anritsu Ltd | 36.523-1 | 5297 | 2 | Rel-18 | F | TEI16_Test, RACS-UEConTest | agreed |
| R5-240021 | Correction to LTE RACS test case 8.5.5.1 | Keysight Technologies UK, Qualcomm | 36.523-1 | 5298 | - | Rel-18 | F | TEI16_Test, RACS-UEConTest | revised |
| R5-241626 | Correction to LTE RACS test case 8.5.5.1 | Keysight Technologies UK, Qualcomm | 36.523-1 | 5298 | 1 | Rel-18 | F | TEI16_Test, RACS-UEConTest | agreed |
| R5-240161 | Update of test cases applicability for NB-IoT NTN only UE | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm Incorporated | 36.523-2 | 1436 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241562 | Update of test cases applicability for NB-IoT NTN only UE | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm Incorporated | 36.523-2 | 1436 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240424 | Addition of PICS and test applicability for SENSE TC | Huawei, HiSilicon | 36.523-2 | 1437 | - | Rel-18 | F | SENSE-UEContest | agreed |
| R5-240587 | Correction to applicability of NB-IoT TC 22.3.2.7a | MCC TF160 | 36.523-2 | 1438 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-240938 | Additional supported capabilities for multiple CA combos | Bureau Veritas ADT, Sporton International, CATT | 36.523-2 | 1439 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-240978 | Addition of applicability for L2L MPS priority access barring test case | Qualcomm Incorporated | 36.523-2 | 1440 | - | Rel-18 | F | TEI16_Test | agreed |
| R5-240982 | Addition of legacy test cases applicable to NTN only UEs in GSO and NGSO | Qualcomm Incorporated, Rohde & Schwarz, MediaTek | 36.523-2 | 1441 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-240983 | PICS clarification and applicability updates for NTN test cases | Qualcomm Incorporated, Rohde & Schwarz | 36.523-2 | 1442 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-241648 | PICS clarification and applicability updates for NTN test cases | Qualcomm Incorporated, Rohde & Schwarz | 36.523-2 | 1442 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-241043 | Applicability updates to EPS UAS test cases | Qualcomm Incorporated | 36.523-2 | 1443 | - | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-241067 | Addition of applicability for new test case 11.2.13 | NTTDOCOMO, INC. | 36.523-2 | 1444 | - | Rel-18 | F | TEI11_Test | revised |
| R5-241625 | Addition of applicability for new test case 11.2.13 | NTTDOCOMO, INC. | 36.523-2 | 1444 | 1 | Rel-18 | F | TEI11_Test | agreed |
| R5-241181 | Addition of applicability of new test case 8.1.3.8a for redir-policy bit | Vodafone | 36.523-2 | 1445 | - | Rel-18 | F | TEI14_Test | revised |
| R5-241622 | Addition of applicability of new test case 8.1.3.8a for redir-policy bit | Vodafone | 36.523-2 | 1445 | 1 | Rel-18 | F | TEI14_Test | agreed |
| R5-241194 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone | 36.523-2 | 1446 | - | Rel-18 | F | TEI17_Test | revised |
| R5-241623 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone | 36.523-2 | 1446 | 1 | Rel-18 | F | TEI17_Test | agreed |
| R5-240549 | Routine maintenance for TS 36.523-3 | MCC TF160 | 36.523-3 | 4786 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-240555 | Corrections of clause 5.3.2 | MCC TF160 | 36.579-1 | 0330 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240556 | Corrections of clause 5.3.29 | MCC TF160 | 36.579-1 | 0331 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240557 | Corrections of clause 5.3.32 | MCC TF160 | 36.579-1 | 0332 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240558 | Corrections of clause 5.4.2 | MCC TF160 | 36.579-1 | 0333 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240559 | Corrections of clause 5.5.1 | MCC TF160 | 36.579-1 | 0334 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240560 | Corrections of clause 5.5.2.11 | MCC TF160 | 36.579-1 | 0335 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240561 | Corrections of clause 5.5.2.13 | MCC TF160 | 36.579-1 | 0336 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240562 | Corrections of clause 5.5.2.19.4 | MCC TF160 | 36.579-1 | 0337 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240563 | Corrections of clause 5.5.2.7.2 | MCC TF160 | 36.579-1 | 0338 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240564 | Corrections of clause 5.5.3.3.1A | MCC TF160 | 36.579-1 | 0339 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240565 | Corrections of clause 5.5.9.1 | MCC TF160 | 36.579-1 | 0340 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240566 | Corrections of references to 24.282 | MCC TF160 | 36.579-1 | 0341 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240897 | Corrections to Table 5.5.3.3.1-3 MCData Resource-lists | NIST | 36.579-1 | 0342 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240898 | Addition of Location-info for MCData | NIST | 36.579-1 | 0343 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240567 | Correction of MO call release of calls using a pre-established session | MCC TF160 | 36.579-2 | 0352 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240568 | Correction of RRC connection release in several test cases | MCC TF160 | 36.579-2 | 0353 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240569 | Correction of testcase 6.1.1.17 | MCC TF160 | 36.579-2 | 0354 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240570 | Correction of testcase 6.1.1.5 | MCC TF160 | 36.579-2 | 0355 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-240571 | Correction of testcase 6.1.2.14 | MCC TF160 | 36.579-2 | 0356 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240572 | Correction of testcase 6.1.2.2 | MCC TF160 | 36.579-2 | 0357 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-240573 | Correction of testcase 6.1.3.1 | MCC TF160 | 36.579-2 | 0358 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240574 | Correction of testcase 6.2.10 | MCC TF160 | 36.579-2 | 0359 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-240575 | Correction of testcase 6.2.14 | MCC TF160 | 36.579-2 | 0360 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240576 | Correction of testcase 6.2.15 | MCC TF160 | 36.579-2 | 0361 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240577 | Correction of testcase 6.2.18 | MCC TF160 | 36.579-2 | 0362 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240578 | Correction of testcase 6.2.19 | MCC TF160 | 36.579-2 | 0363 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240579 | Correction of testcase 6.2.22 | MCC TF160 | 36.579-2 | 0364 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240580 | Correction of testcase 6.2.26 | MCC TF160 | 36.579-2 | 0365 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240581 | Correction of testcase 6.2.9 | MCC TF160 | 36.579-2 | 0366 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-240899 | Corrections to Forward, Scope, References, and Definitions | NIST | 36.579-3 | 0008 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | withdrawn |
| R5-240900 | Addition on new TC 6.2 MCPTT Server - MCPTT Client - Chat Group Call | NIST | 36.579-3 | 0009 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | withdrawn |
| R5-240901 | Addition of applicability for new Rel-16 test cases | NIST | 36.579-4 | 0033 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-241685 | Addition of applicability for new Rel-16 test cases | NIST | 36.579-4 | 0033 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240902 | Addition of new PIXITs for REL-16 TCs | NIST | 36.579-5 | 0105 | - | Rel-17 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-241686 | Addition of new PIXITs for REL-16 TCs | NIST | 36.579-5 | 0105 | 1 | Rel-17 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240528 | Correction of testcase 6.7.3 | MCC TF160 | 36.579-6 | 0099 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240582 | Correction of testcase 6.1.3.1 | MCC TF160 | 36.579-6 | 0100 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240583 | Correction of testcase 6.7.1 | MCC TF160 | 36.579-6 | 0101 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-240633 | Addition of new TC 6.5.2 One-to-server video push call CO | NIST | 36.579-6 | 0102 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-241596 | Addition of new TC 6.5.2 One-to-server video push call CO | NIST | 36.579-6 | 0102 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240909 | Addition of new TC 6.9.1 Location Information Request | NIST | 36.579-6 | 0103 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240910 | Addition of new TC 6.9.2 Report triggering | NIST | 36.579-6 | 0104 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240911 | Addition of new TC 6.1.1.14 Re-join | NIST | 36.579-6 | 0105 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-241595 | Addition of new TC 6.1.1.14 Re-join | NIST | 36.579-6 | 0105 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240584 | Corrections of references to 24.282 | MCC TF160 | 36.579-7 | 0053 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240647 | Editorial correction to the Scope description | FirstNet, NIST | 36.579-7 | 0054 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-240903 | Addition of new TC 6.4.1 Emergency Alert CO | NIST | 36.579-7 | 0055 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-241593 | Addition of new TC 6.4.1 Emergency Alert CO | NIST | 36.579-7 | 0055 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240904 | Addition of new TC 6.4.2 Emergency Alert CT | NIST | 36.579-7 | 0056 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-241594 | Addition of new TC 6.4.2 Emergency Alert CT | NIST | 36.579-7 | 0056 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-240025 | Update NB-IoT NTN UL timing accuracy cases TT | MediaTek Beijing Inc. | 36.903 | 0460 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240217 | Addition of IoT NTN test cases grouping | MediaTek (Hefei) Inc. | 36.903 | 0461 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-240076 | Update of grouping of test cases defined in TS 36.521-4 | CMCC, MediaTek | 36.904 | 0066 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241803 | Update of grouping of test cases defined in TS 36.521-4 | CMCC, MediaTek | 36.904 | 0066 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241202 | Adding Test Point Analysis for 4DL and 5DL RefSens | Sporton, Bureau Veritas | 36.905 | 0271 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-241391 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | 36.905 | 0272 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-241955 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | 36.905 | 0272 | 1 | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-241207 | CR on Measurement Distance | ROHDE & SCHWARZ | 37.544 | 0032 | - | Rel-16 | F | TEI14_Test, LTE_MIMO_OTA-UEConTest | revised |
| R5-241940 | CR on Measurement Distance | ROHDE & SCHWARZ | 37.544 | 0032 | 1 | Rel-16 | F | TEI14_Test, LTE_MIMO_OTA-UEConTest | agreed |
| R5-240101 | Update TC 15.2.8 with TT analysis results | CATT | 37.571-1 | 0448 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241897 | Update TC 15.2.8 with TT analysis results | CATT | 37.571-1 | 0448 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240102 | Update TC 15.2.9 with TT analysis results | CATT | 37.571-1 | 0449 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241898 | Update TC 15.2.9 with TT analysis results | CATT | 37.571-1 | 0449 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240103 | Update TC 15.2.10 with TT analysis results | CATT | 37.571-1 | 0450 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240104 | Update TC 16.2.7 with TT analysis results | CATT | 37.571-1 | 0451 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240105 | Update TC 16.2.8 with TT analysis results | CATT | 37.571-1 | 0452 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240106 | Update TC 16.3.4 with TT analysis results | CATT | 37.571-1 | 0453 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241899 | Update TC 16.3.4 with TT analysis results | CATT | 37.571-1 | 0453 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240107 | Addition of NR PRS-based measurement requirements in annex C to include TT analysis results | CATT | 37.571-1 | 0454 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240108 | Addition of new RRC_INACTIVE NR RSTD reporting delay test case 14.4.3 | CATT | 37.571-1 | 0455 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240109 | Addition of new NR RSTD reporting delay test case 14.4.4 in RRC_INACTIVE state | CATT | 37.571-1 | 0456 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240110 | Addition of new NR RSTD accuracy test case 14.5.3 in RRC_INACTIVE state | CATT | 37.571-1 | 0457 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240111 | Addition of new NR RSTD accuracy test case 14.5.4 in RRC_INACTIVE state | CATT | 37.571-1 | 0458 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241911 | Addition of new NR RSTD accuracy test case 14.5.4 in RRC_INACTIVE state | CATT | 37.571-1 | 0458 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240112 | Addition of new NR PRS-RSRP reporting delay test case 16.4.3 in RRC_INACTIVE state | CATT | 37.571-1 | 0459 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240113 | Addition of new NR PRS-RSRP reporting delay test case 16.4.4 in RRC_INACTIVE state | CATT | 37.571-1 | 0460 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240114 | Addition of new NR PRS-RSRP accuracy test case 16.5.3 in RRC_INACTIVE state | CATT | 37.571-1 | 0461 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240115 | Addition of new NR PRS-RSRP accuracy test case 16.5.4 in RRC_INACTIVE state | CATT | 37.571-1 | 0462 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240116 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.1 | CATT | 37.571-1 | 0463 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240117 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.2 | CATT | 37.571-1 | 0464 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240118 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.3 | CATT | 37.571-1 | 0465 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240119 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.3.4 | CATT | 37.571-1 | 0466 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240120 | Addition of measurement period requirements in RRC_INACTIVE state for NR positioning methods | CATT | 37.571-1 | 0467 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240121 | Correction to PRS-RSRPP positioning test cases | CATT, Spirent | 37.571-1 | 0468 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241912 | Correction to PRS-RSRPP positioning test cases | CATT, Spirent | 37.571-1 | 0468 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240122 | Correction to PRS-RSRP positioning test cases | CATT, Spirent | 37.571-1 | 0469 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241913 | Correction to PRS-RSRP positioning test cases | CATT, Spirent | 37.571-1 | 0469 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240123 | Correction to NR RSTD positioning test cases | CATT, Spirent | 37.571-1 | 0470 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241914 | Correction to NR RSTD positioning test cases | CATT, Spirent | 37.571-1 | 0470 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240124 | Correction to NR UE Rx-Tx time difference positioning test cases | CATT, Spirent | 37.571-1 | 0471 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241915 | Correction to NR UE Rx-Tx time difference positioning test cases | CATT, Spirent | 37.571-1 | 0471 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240513 | Correction to NR Physical Cell IDs for NR-PRS assistance data | ROHDE & SCHWARZ | 37.571-2 | 0182 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240125 | Addition of test applicabilities for Release-17 PRS-RSRP, PRS-RSRPP and RSTD test cases | CATT | 37.571-3 | 0167 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240553 | Removal of PIXIT for deprecated signalling GNSS scenarios | MCC TF160 | 37.571-4 | 0198 | - | Rel-17 | F | TEI17_Test | agreed |
| R5-240126 | Introduction of BDS B2a and B3I signal default test conditions in TS 37.571-5 | CATT, CAICT | 37.571-5 | 0227 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241577 | Introduction of BDS B2a and B3I signal default test conditions in TS 37.571-5 | CATT, CAICT | 37.571-5 | 0227 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240127 | Introduction of BDS B2a and B3I performance default test conditions in TS 37.571-5 | CATT, CAICT | 37.571-5 | 0228 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241916 | Introduction of BDS B2a and B3I performance default test conditions in TS 37.571-5 | CATT, CAICT | 37.571-5 | 0228 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240128 | Corrected CellIdentity for DL-TDOA measurement period test | CATT, Spirent | 37.571-5 | 0229 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-241917 | Corrected CellIdentity for DL-TDOA measurement period test | CATT, Spirent | 37.571-5 | 0229 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-240554 | Removal of deprecated signalling GNSS scenarios | MCC TF160 | 37.571-5 | 0230 | - | Rel-17 | F | TEI17_Test | agreed |
| R5-240860 | Update SNR conditions in UDP and TCP FR2 FRC scenarios | Keysight Technologies | 37.901-5 | 0038 | - | Rel-16 | F | TEI16_Test, FS_UE_5GNR_App_Data_Perf | agreed |
| R5-241410 | General cleanup of annex for app layer tput test cases | QUALCOMM Europe Inc. - Italy | 37.901-5 | 0039 | - | Rel-16 | F | TEI16_Test, FS_UE_5GNR_App_Data_Perf | revised |
| R5-241852 | General cleanup of annex for app layer tput test cases | QUALCOMM Europe Inc. - Italy | 37.901-5 | 0039 | 1 | Rel-16 | F | TEI16_Test, FS_UE_5GNR_App_Data_Perf | agreed |
| R5-240045 | Introduction of Common test environments for ATG | CMCC | 38.508-1 | 3000 | - | Rel-18 | F | NR_ATG-UEConTest | withdrawn |
| R5-240046 | Introduction of RF Test Environment for ATG | CMCC | 38.508-1 | 3001 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241919 | Introduction of RF Test Environment for ATG | CMCC | 38.508-1 | 3001 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240047 | Introduction of RRM Test Environment for ATG | CMCC | 38.508-1 | 3002 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241920 | Introduction of RRM Test Environment for ATG | CMCC | 38.508-1 | 3002 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240153 | Correction to RRCReestablishment message | MediaTek Inc. | 38.508-1 | 3003 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240154 | Correction to PEIPS in REGISTRATION ACCEPT message | MediaTek Inc. | 38.508-1 | 3004 | - | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240163 | Correction to Test procedure 4.9.27 | MediaTek Inc. | 38.508-1 | 3005 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240164 | Correction to Test procedure 4.9.38 | MediaTek Inc. | 38.508-1 | 3006 | - | Rel-18 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240165 | Correction to DLInformationTransfer message | MediaTek Inc. | 38.508-1 | 3007 | - | Rel-18 | F | TEI16_Test, NR_IioT-UEConTest | agreed |
| R5-240166 | Correction to UEInformationRequest message | MediaTek Inc. | 38.508-1 | 3008 | - | Rel-18 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-240167 | Editorial update of CounterCheck message | MediaTek Inc. | 38.508-1 | 3009 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240168 | Correction to IE RadioLinkMonitoringConfig | MediaTek Inc. | 38.508-1 | 3010 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240169 | Correction to IE NRDC-Parameters | MediaTek Inc. | 38.508-1 | 3011 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241480 | Correction to IE NRDC-Parameters | MediaTek Inc. | 38.508-1 | 3011 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240170 | Correction to IE UE-MRDC-Capability | MediaTek Inc. | 38.508-1 | 3012 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241481 | Correction to IE UE-MRDC-Capability | MediaTek Inc. | 38.508-1 | 3012 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240171 | Correction to IE UE-NR-Capability | MediaTek Inc. | 38.508-1 | 3013 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241482 | Correction to IE UE-NR-Capability | MediaTek Inc. | 38.508-1 | 3013 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240172 | Correction to RRCReconfiguration configuration | MediaTek Inc. | 38.508-1 | 3014 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240208 | Correction to Rel-16 PICS Mnemonic | MediaTek Inc. | 38.508-1 | 3015 | - | Rel-18 | F | TEI16_Test | withdrawn |
| R5-240233 | Update inter-band NR CA configuration of three bands CA_n1A-n8A-n78A | CU Digital Technology | 38.508-1 | 3016 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240236 | Correction to NR RRC_Idle generic procedure | Keysight Technologies UK | 38.508-1 | 3017 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240237 | Correction to NR RRC_Inactive generic procedure | Keysight Technologies UK | 38.508-1 | 3018 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241628 | Correction to NR RRC_Inactive generic procedure | Keysight Technologies UK | 38.508-1 | 3018 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240258 | Correction of Notes in tables of test channel bandwidths | CAICT | 38.508-1 | 3019 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241921 | Correction of Notes in tables of test channel bandwidths | CAICT | 38.508-1 | 3019 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240271 | Correction of errors in Annex C | CAICT | 38.508-1 | 3020 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240272 | Style correction for clause title of 4.3.1.1.1.100 and 4.3.1.1.1.101 | CAICT | 38.508-1 | 3021 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240274 | Addition of test frequencies for new R16 NR CA combos within FR1 | KDDI Corporation | 38.508-1 | 3022 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240275 | Addition of test frequencies for new R17 NR CA combos within FR1 | KDDI Corporation | 38.508-1 | 3023 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240309 | Updating test frequency for SUL band n83 20MHz CBW | Huawei, HiSilicon | 38.508-1 | 3024 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241716 | Updating test frequency for SUL band n83 20MHz CBW | Huawei, HiSilicon | 38.508-1 | 3024 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241869 | Updating test frequency for SUL band n83 20MHz CBW | Huawei, HiSilicon | 38.508-1 | 3024 | 2 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240346 | Update of RRC message RRCReconfigurationComplete, RRCSetupComplete and SecurityModeComplete for SL relay | China Telecom | 38.508-1 | 3025 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-241722 | Update of RRC message RRCReconfigurationComplete, RRCSetupComplete and SecurityModeComplete for SL relay | China Telecom | 38.508-1 | 3025 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240347 | Update of Test procedure for establishing unicast mode ProSe Direct communication | China Telecom | 38.508-1 | 3026 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-241723 | Update of Test procedure for establishing unicast mode ProSe Direct communication | China Telecom | 38.508-1 | 3026 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240349 | Correction of Default parameters for simulated SNPN cells. | China Telecom | 38.508-1 | 3027 | - | Rel-18 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-240378 | Update of conditional PSCell addition configuration for NR-DC | Huawei, HiSilicon | 38.508-1 | 3028 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240398 | Addition of test frequencies for new EN-DC comb within FR2 | KDDI Corporation | 38.508-1 | 3029 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241927 | Addition of test frequencies for new EN-DC comb within FR2 | KDDI Corporation | 38.508-1 | 3029 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240432 | Move all V2X sub-clauses from 4.7 into a new clause 4.7D | Huawei, HiSilicon, CATT | 38.508-1 | 3030 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-241538 | Move all V2X sub-clauses from 4.7 into a new clause 4.7D | Huawei, HiSilicon, CATT | 38.508-1 | 3030 | 1 | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-240444 | Add IndirectPathFailureInformation | Ericsson | 38.508-1 | 3031 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240445 | Add MBSMulticastConfiguration | Ericsson | 38.508-1 | 3032 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240446 | Add MeasurementReportAppLayer | Ericsson | 38.508-1 | 3033 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-240447 | Editorial update SystemInformation | Ericsson | 38.508-1 | 3034 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241483 | Editorial update SystemInformation | Ericsson | 38.508-1 | 3034 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240448 | Add UEPositioningAssistanceInfo | Ericsson | 38.508-1 | 3035 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-240460 | Introduction of test frequencies for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | 38.508-1 | 3036 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240468 | Test frequencies and channel bandwidth updates for band n71 and n25 in Rel-17 | Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc. | 38.508-1 | 3037 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240478 | Addition of test procedure for 5G ProSe U2N Relay Discovery | TDIA, CATT | 38.508-1 | 3038 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-241597 | Addition of test procedure for 5G ProSe U2N Relay Discovery | TDIA, CATT | 38.508-1 | 3038 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240479 | Addition of test procedure for remote Initial Access procedure under NR sidelink U2N Relay / Remote UE side | CATT, TDIA | 38.508-1 | 3039 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-241598 | Addition of test procedure for remote Initial Access procedure under NR sidelink U2N Relay / Remote UE side | CATT, TDIA | 38.508-1 | 3039 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240480 | Removal of test procedure for Out_of_Coverage_with_Relay | CATT, TDIA | 38.508-1 | 3040 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240481 | Update test procedure for registration of NR sidelink U2N Relay / Relay UE side | TDIA, CATT | 38.508-1 | 3041 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-241599 | Update test procedure for registration of NR sidelink U2N Relay / Relay UE side | TDIA, CATT | 38.508-1 | 3041 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240482 | Update of 5G ProSe information elements of UE policies for 5G ProSe usage information reporting | TDIA, CATT | 38.508-1 | 3042 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240483 | Update of 5G ProSe information elements of UE policies for 5G ProSe remote | TDIA, CATT | 38.508-1 | 3043 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240484 | Addition of PROSE PC5 DISCOVERY | TDIA, CATT | 38.508-1 | 3044 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240489 | Add IEs SIB22, SIB23, SIB24 and SIB25 | Ericsson | 38.508-1 | 3045 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240490 | Editorial update IE PosSystemInformation-r16-IEs | Ericsson | 38.508-1 | 3046 | - | Rel-18 | F | TEI16_Test | revised |
| R5-241484 | Editorial update IE PosSystemInformation-r16-IEs | Ericsson | 38.508-1 | 3046 | 1 | Rel-18 | F | TEI16_Test | agreed |
| R5-240491 | Editorial update IE PosSI-SchedulingInfo | Ericsson | 38.508-1 | 3047 | - | Rel-18 | F | TEI16_Test | agreed |
| R5-240492 | Add IE AdvancedReceiver-MU-MIMO | Ericsson | 38.508-1 | 3048 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240493 | Add IE Altitude | Ericsson | 38.508-1 | 3049 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240494 | Add IE ATG-Config | Ericsson | 38.508-1 | 3050 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240495 | Add IEs CandidateTCI-State and CandidateTCI-UL-State | Ericsson | 38.508-1 | 3051 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240496 | Add IE CellDTXDRX-Config | Ericsson | 38.508-1 | 3052 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240497 | Add IE ClockQualityMetrics | Ericsson | 38.508-1 | 3053 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240498 | Add IEs CSI-ReportSubConfig, CSI-ReportSubConfigId and CSI-ReportSubConfigTriggerList | Ericsson | 38.508-1 | 3054 | - | Rel-18 | F | TEI18_Test | revised |
| R5-241485 | Add IEs CSI-ReportSubConfig, CSI-ReportSubConfigId and CSI-ReportSubConfigTriggerList | Ericsson | 38.508-1 | 3054 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-240499 | Add IE EarlyUL-SyncConfig | Ericsson | 38.508-1 | 3055 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240500 | Add IE EUTRA-C-RNTI | Ericsson | 38.508-1 | 3056 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-240536 | Editorial updates to test procedure 4.9.1 | MCC TF160 | 38.508-1 | 3057 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240537 | Harmonization of SCell_add condition | MCC TF160 | 38.508-1 | 3058 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240616 | RF TRx testing - P-Max configuration extension to RX tests to enable TxD | Keysight Technologies UK Ltd | 38.508-1 | 3059 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | revised |
| R5-241704 | RF TRx testing - P-Max configuration extension to RX tests to enable TxD | Keysight Technologies UK Ltd | 38.508-1 | 3059 | 1 | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-240617 | Added 30kHz SCS for SSB in n53 to 30kHz SCS test frequencies | Keysight Technologies UK Ltd, Apple | 38.508-1 | 3060 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-240630 | Update SIB1 | Ericsson | 38.508-1 | 3061 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240648 | Updates to Ephemeris Info for Multi-Cell NRN-NTN signalling test cases | QUALCOMM JAPAN LLC. | 38.508-1 | 3062 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241561 | Updates to Ephemeris Info for Multi-Cell NRN-NTN signalling test cases | QUALCOMM JAPAN LLC. | 38.508-1 | 3062 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240649 | Updates to SIB19 Common Config for NR-NTN test cases | QUALCOMM JAPAN LLC | 38.508-1 | 3063 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241565 | Updates to SIB19 Common Config for NR-NTN test cases | QUALCOMM JAPAN LLC | 38.508-1 | 3063 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240661 | Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI) | Qualcomm CDMA Technologies | 38.508-1 | 3064 | - | Rel-18 | F | TEI17_Test, eNS_Ph2-UEConTest | withdrawn |
| R5-240676 | Correction to default configuration of SMTC for NCD-SSB | Huawei, HiSilicon, Starpoint | 38.508-1 | 3065 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242038 | Correction to default configuration of SMTC for NCD-SSB | Huawei, HiSilicon, Starpoint | 38.508-1 | 3065 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240680 | Addition of applicability for RedCap Demod and RRM test cases | Huawei,HiSilicon | 38.508-1 | 3066 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240764 | Correction to default configuration of frequencyInfoSL | Huawei, HiSilicon | 38.508-1 | 3067 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-241539 | Correction to default configuration of frequencyInfoSL | Huawei, HiSilicon | 38.508-1 | 3067 | 1 | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-240775 | Introduction of test frequencies for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A | Nokia, Nokia Shanghai Bell | 38.508-1 | 3068 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240822 | Update IE FeatureCombination | Ericsson | 38.508-1 | 3069 | - | Rel-18 | F | TEI18_Test | withdrawn |
| R5-240823 | Add IE HysteresisAltitude | Ericsson | 38.508-1 | 3070 | - | Rel-18 | F | TEI18_Test | revised |
| R5-241486 | Add IE HysteresisAltitude | Ericsson | 38.508-1 | 3070 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-240824 | Add IEs LTM-CandidateId, LTM-Candidate, LTM-Config and LTM-CSI-ReportConfig | Ericsson | 38.508-1 | 3071 | - | Rel-18 | F | TEI18_Test | revised |
| R5-241487 | Add IEs LTM-CandidateId, LTM-Candidate, LTM-Config and LTM-CSI-ReportConfig | Ericsson | 38.508-1 | 3071 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-240825 | Add IEs LTM-CSI-ReportConfigId, LTM-CSI-ResourceConfig and LTM-CSI-ResourceConfigId | Ericsson | 38.508-1 | 3072 | - | Rel-18 | F | TEI18_Test | revised |
| R5-241488 | Add IEs LTM-CSI-ReportConfigId, LTM-CSI-ResourceConfig and LTM-CSI-ResourceConfigId | Ericsson | 38.508-1 | 3072 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-240839 | Corrections on 4.6.2 and 4.6.3 for the conditions in SIB and RRC IE | ZTE Corporation | 38.508-1 | 3073 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241809 | Corrections on 4.6.2 and 4.6.3 for the conditions in SIB and RRC IE | ZTE Corporation | 38.508-1 | 3073 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240856 | Corrections on C.1 for the parameters for calculation of test frequencies | ZTE Corporation | 38.508-1 | 3074 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240861 | Update for additional NR-CA and NR-DC band configurations | Verizon | 38.508-1 | 3075 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241928 | Update for additional NR-CA and NR-DC band configurations | Verizon | 38.508-1 | 3075 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240878 | Update NR inter-band CA configurations FR1 four bands table | Verizon Spain | 38.508-1 | 3076 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240882 | Addition of test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification Oy, AT&T | 38.508-1 | 3077 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241705 | Addition of test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification Oy, AT&T | 38.508-1 | 3077 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240886 | Update additional inter-band NR CA configurations | Verizon | 38.508-1 | 3078 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241708 | Update additional inter-band NR CA configurations | Verizon | 38.508-1 | 3078 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240971 | Annex C update to asymmetric channel bandwidths test frequencies calculation | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA | 38.508-1 | 3079 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240972 | Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8 | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA | 38.508-1 | 3080 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-241703 | Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8 | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA | 38.508-1 | 3080 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240989 | SIB2 updates for NR unlicensed test cases | Qualcomm Incorporated | 38.508-1 | 3081 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240996 | Correction to SUL configuration messages | Huawei, HiSilicon | 38.508-1 | 3082 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241719 | Correction to SUL configuration messages | Huawei, HiSilicon | 38.508-1 | 3082 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241091 | Correction to test frequencies for CA_n48 - 2A | Anritsu | 38.508-1 | 3083 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241706 | Correction to test frequencies for CA_n48 - 2A | Anritsu | 38.508-1 | 3083 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241092 | Correction to CBW selection to avoid unintentional asymmetric CBW | Anritsu | 38.508-1 | 3084 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241093 | Correction to point A value for n83 | Anritsu | 38.508-1 | 3085 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241717 | Correction to point A value for n83 | Anritsu | 38.508-1 | 3085 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241094 | Correction to parameters for Rel-17 bands | Anritsu | 38.508-1 | 3086 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-241718 | Correction to parameters for Rel-17 bands | Anritsu | 38.508-1 | 3086 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-241095 | Correction to message exceptions for Performance test cases | Anritsu | 38.508-1 | 3087 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-242022 | Correction to message exceptions for Performance test cases | Anritsu | 38.508-1 | 3087 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241187 | Updates to default 5GMM messages | Ericsson | 38.508-1 | 3088 | - | Rel-18 | F | TEI18_Test | withdrawn |
| R5-241188 | Updates to default 5GSM messages | Ericsson | 38.508-1 | 3089 | - | Rel-18 | F | TEI18_Test | withdrawn |
| R5-241189 | Updates to default UE Policy Delivery messages | Ericsson | 38.508-1 | 3090 | - | Rel-18 | F | TEI18_Test | revised |
| R5-241489 | Updates to default UE Policy Delivery messages | Ericsson | 38.508-1 | 3090 | 1 | Rel-18 | F | TEI18_Test | withdrawn |
| R5-241230 | Addition of missing section for RRM CA Configuration | Rohde & Schwarz | 38.508-1 | 3091 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241875 | Addition of missing section for RRM CA Configuration | Rohde & Schwarz | 38.508-1 | 3091 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241346 | Update of PDSCH Config for RF test cases | ROHDE & SCHWARZ | 38.508-1 | 3092 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241968 | Update of PDSCH Config for RF test cases | ROHDE & SCHWARZ | 38.508-1 | 3092 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241355 | Correction of test environment for NR NTN RF testing | ROHDE & SCHWARZ | 38.508-1 | 3093 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242007 | Correction of test environment for NR NTN RF testing | ROHDE & SCHWARZ | 38.508-1 | 3093 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241363 | Addition of test frequencies for CA n77(2A), BCS1, UL CA.. | Ericsson | 38.508-1 | 3094 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241944 | Addition of test frequencies for CA n77(2A), BCS1, UL CA.. | Ericsson | 38.508-1 | 3094 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241365 | Test environment definition for GERAN in Conducted and OTA Environment | Anritsu EMEA Ltd | 38.508-1 | 3095 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241630 | Test environment definition for GERAN in Conducted and OTA Environment | Anritsu EMEA Ltd | 38.508-1 | 3095 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241371 | Add IE MeasSequence | Ericsson | 38.508-1 | 3096 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-241372 | Add IE MeasWindowConfig | Ericsson | 38.508-1 | 3097 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-241373 | Add IEs N3C-IndirectPathConfigRelay and N3C-IndirectPathAddChange | Ericsson | 38.508-1 | 3098 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-241374 | Add IEs NCR-AperiodicFwdConfig, NCR-FwdConfig, NCR-PeriodicityAndOffset and NCR-PeriodicFwdResourceSet | Ericsson | 38.508-1 | 3099 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-241375 | Add IEs NCR-PeriodicFwdResourceSetId, NCR-SemiPersistentFwdResourceSet and NCR-SemiPersistentFwdResourceSetId | Ericsson | 38.508-1 | 3100 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-241393 | Updates to Test environment for NR NTN RRM testing | Keysight Technologies UK Ltd, Thales | 38.508-1 | 3101 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242016 | Updates to Test environment for NR NTN RRM testing | Keysight Technologies UK Ltd, Thales | 38.508-1 | 3101 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241400 | Common Test environment updates for NR NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | 38.508-1 | 3102 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242017 | Common Test environment updates for NR NTN RF, demod and RRM testing | Keysight Technologies UK Ltd | 38.508-1 | 3102 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241493 | Updates to OTA Signalling test environment | Anritsu Ltd | 38.508-1 | 3104 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241629 | Updates to OTA Signalling test environment | Anritsu Ltd | 38.508-1 | 3104 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241525 | Addition of signalling test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification, AT&T | 38.508-1 | 3105 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241580 | Addition of signalling test frequencies for DC_2A_n2A and DC_66A_n66A | WE Certification, AT&T | 38.508-1 | 3105 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241926 | Update of abbreviations for ATG | CMCC | 38.508-1 | 3106 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240027 | Addition of PICS for Rel-17 ATSSS devices | China Telecom | 38.508-2 | 0562 | - | Rel-18 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-240048 | Introduction of common ICS for ATG | CMCC | 38.508-2 | 0563 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241715 | Introduction of common ICS for ATG | CMCC | 38.508-2 | 0563 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240079 | Introduction of common ICS for PC1.5 n39 | CMCC | 38.508-2 | 0564 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | revised |
| R5-242023 | Introduction of common ICS for PC1.5 n39 | CMCC | 38.508-2 | 0564 | 1 | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-240093 | Update NR band and CADC configs status in ICS Annex B | CMCC | 38.508-2 | 0565 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-240143 | New additional information for UPIP with 5GC | ZTE Corporation | 38.508-2 | 0566 | - | Rel-18 | F | UPIP_SEC_LTE-RAN-UEConTest | withdrawn |
| R5-240173 | Correction to HPUE PICS Mnemonic | MediaTek Inc. | 38.508-2 | 0567 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240207 | Correction to NR NTN PICS Mnemonic | MediaTek Inc. | 38.508-2 | 0568 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240209 | Correction to Rel-17 PICS Mnemonic | MediaTek Inc. | 38.508-2 | 0569 | - | Rel-18 | F | TEI17_Test | withdrawn |
| R5-240276 | Addition of UE capability for new R16 NR CA combos within FR1 | KDDI Corporation | 38.508-2 | 0570 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240277 | Addition of UE capability for new R17 NR CA combos within FR1 | KDDI Corporation | 38.508-2 | 0571 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240288 | Addition of r16 and r17 UL tx switching and DL interruption supporting PICS for inter band CA with 3 bands | China Telecom | 38.508-2 | 0572 | - | Rel-18 | F | DL_intrpt_combos_TxSW_R17-UEConTest | withdrawn |
| R5-240313 | Introducing indicator for Power Class of CA configuration with single uplink carrier | Huawei, HiSilicon | 38.508-2 | 0573 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240316 | Introducing SUL configuration SUL_n78A-n81A | Huawei, HiSilicon | 38.508-2 | 0574 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240330 | Addition of RF baseline implementation capabilities for new PC2 EN-DC combos within FR1 | KDDI Corporation | 38.508-2 | 0575 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240335 | Editorial correction to note numbering for inter-band EN-DC capabilities table | Huawei, HiSilicon | 38.508-2 | 0576 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240348 | Addition of Sidelink Capabilities to support direct to indirect path switch for NR sidelink U2N Relay | China Telecom | 38.508-2 | 0577 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-241724 | Addition of Sidelink Capabilities to support direct to indirect path switch for NR sidelink U2N Relay | China Telecom | 38.508-2 | 0577 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240350 | Addition of capability for UEs to support steering of roaming SNPN selection information (SOR-SNPN-SI) and steering of roaming connected mode control information (SOR-CMCI) for Rel-17 eNPN | China Telecom | 38.508-2 | 0578 | - | Rel-18 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241581 | Addition of capability for UEs to support steering of roaming SNPN selection information (SOR-SNPN-SI) and steering of roaming connected mode control information (SOR-CMCI) for Rel-17 eNPN | China Telecom | 38.508-2 | 0578 | 1 | Rel-18 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-240387 | Addition of UE capability for inter-SN conditional PSCell change | Huawei, HiSilicon | 38.508-2 | 0579 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-241601 | Addition of UE capability for inter-SN conditional PSCell change | Huawei, HiSilicon | 38.508-2 | 0579 | 1 | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240399 | Addition of NR NTN capabilities | CAICT | 38.508-2 | 0580 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241909 | Addition of NR NTN capabilities | CAICT | 38.508-2 | 0580 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240410 | Addition of UE capability for new EN-DC comb within FR2 | KDDI Corporation | 38.508-2 | 0581 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240425 | Add PICS for PEIPS | Huawei, HiSilicon | 38.508-2 | 0582 | - | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240461 | Introduction of CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) for physical layer baseline implementation capabilities | Nokia, Nokia Shanghai Bell | 38.508-2 | 0583 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240477 | Addition of new pics for NR sidelink U2N Relay PDU session establishment | TDIA, CATT | 38.508-2 | 0584 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-241600 | Addition of new pics for NR sidelink U2N Relay PDU session establishment | TDIA, CATT | 38.508-2 | 0584 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-240506 | Addition of feMIMO physical layer baseline implementation capabilities | Samsung | 38.508-2 | 0585 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-241958 | Addition of feMIMO physical layer baseline implementation capabilities | Samsung | 38.508-2 | 0585 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-241674 | Addition of feMIMO physical layer baseline implementation capabilities | Samsung | 38.508-2 | 0585 | 2 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-240522 | Update the existing PICS of inter-band CA between FR1 and FR2 | Nokia, Nokia Shanghai Bell | 38.508-2 | 0586 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-240538 | Updates to align PICS mnemonics | MCC TF160 | 38.508-2 | 0587 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241491 | Updates to align PICS mnemonics | MCC TF160 | 38.508-2 | 0587 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240590 | Modification of testcase 8.1.5.13.2 applicability clauses | Nokia, Nokia Shanghai Bell | 38.508-2 | 0588 | - | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | withdrawn |
| R5-240593 | Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2 | Nokia, Nokia Shanghai Bell | 38.508-2 | 0589 | - | Rel-18 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-240672 | Addition of new PICS for NTN Event D1 | Qualcomm CDMA Technologies | 38.508-2 | 0590 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240673 | Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI) | Qualcomm CDMA Technologies | 38.508-2 | 0591 | - | Rel-18 | F | TEI17_Test, eNS_Ph2-UEConTest | revised |
| R5-241631 | Addition of new PICS for UE supporting extended rejected NSSAI (ER-NSSAI) | Qualcomm CDMA Technologies | 38.508-2 | 0591 | 1 | Rel-18 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-240863 | Update for additional NR-DC band configurations | Verizon Spain | 38.508-2 | 0592 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241709 | Update for additional NR-DC band configurations | Verizon Spain | 38.508-2 | 0592 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240887 | Update additional FR1 NR CA inter-band band configurations | Verizon | 38.508-2 | 0593 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240892 | Update for additional band configurations with PC2 UL | Verizon Spain | 38.508-2 | 0594 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241714 | Update for additional band configurations with PC2 UL | Verizon Spain | 38.508-2 | 0594 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240914 | Update for additional ENDC band configurations with PC2 UL | Verizon | 38.508-2 | 0595 | - | Rel-18 | F | ENDC_PC2_R17_xLTE_yNR-UEConTest | revised |
| R5-241720 | Update for additional ENDC band configurations with PC2 UL | Verizon | 38.508-2 | 0595 | 1 | Rel-18 | F | ENDC_PC2_R17_xLTE_yNR-UEConTest | agreed |
| R5-240919 | Editorial correction to supported EN-DC configuration table | WE Certification | 38.508-2 | 0596 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241710 | Editorial correction to supported EN-DC configuration table | WE Certification | 38.508-2 | 0596 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240984 | Addition of NR NTN TA reporting PICS | Qualcomm Incorporated | 38.508-2 | 0597 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241649 | Addition of NR NTN TA reporting PICS | Qualcomm Incorporated | 38.508-2 | 0597 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240999 | Adding PICS for V2X testing | Huawei, HiSilicon | 38.508-2 | 0598 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-241707 | Adding PICS for V2X testing | Huawei, HiSilicon | 38.508-2 | 0598 | 1 | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241029 | Update to PICS for R17 FR1 enhancement | Huawei, HiSilicon | 38.508-2 | 0599 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241165 | Removal of duplicated RSSI measurements and channel occupancy reporting parameter | Ericsson | 38.508-2 | 0600 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241259 | Addition of RF baseline implementation capability of PC2 config n8 | China Unicom | 38.508-2 | 0601 | - | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | agreed |
| R5-241262 | Addition of n5 with UL MIMO capabilities | China Unicom | 38.508-2 | 0602 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-241270 | Addition of PICS to support NTN RRM | Qualcomm Germany | 38.508-2 | 0603 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241286 | Addition of PICS for UL LBT Failure Detection and Recovery | Qualcomm Germany | 38.508-2 | 0604 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241711 | Addition of PICS for UL LBT Failure Detection and Recovery | Qualcomm Germany | 38.508-2 | 0604 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241376 | Addition of PICS for support of ULCA for different type of CA Band combinations | Keysight Technologies UK | 38.508-2 | 0605 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-241457 | Correction to Rel-16 PICS Mnemonic | MediaTek Inc. | 38.508-2 | 0606 | - | Rel-18 | F | TEI16_Test | withdrawn |
| R5-240430 | Correction of Set MUSIM UAI test function | Huawei, HiSilicon,China Telecom | 38.509 | 0082 | - | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | revised |
| R5-241636 | Correction of Set MUSIM UAI test function | Huawei, HiSilicon,China Telecom | 38.509 | 0082 | 1 | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | withdrawn |
| R5-240434 | Correction of Test Loop Mode C for MBS broadcast | Huawei, HiSilicon | 38.509 | 0083 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-241492 | Correction of Test Loop Mode C for MBS broadcast | Huawei, HiSilicon | 38.509 | 0083 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240049 | Introduction of common parts for ATG UE RF test cases | CMCC | 38.521-1 | 2585 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240050 | Introduction of General description for ATG UE Tx TCs | CMCC | 38.521-1 | 2586 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-242030 | Introduction of General description for ATG UE Tx TCs | CMCC | 38.521-1 | 2586 | 1 | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241683 | Introduction of General description for ATG UE Tx TCs | CMCC | 38.521-1 | 2586 | 2 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240051 | Introduction of MOP TC for ATG UE | CMCC | 38.521-1 | 2587 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241751 | Introduction of MOP TC for ATG UE | CMCC | 38.521-1 | 2587 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240052 | Introduction of Configured transmitted power TC for ATG UE | CMCC | 38.521-1 | 2588 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241752 | Introduction of Configured transmitted power TC for ATG UE | CMCC | 38.521-1 | 2588 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240053 | Introduction of General description of Occupied bandwidth for ATG UE | CMCC | 38.521-1 | 2589 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240054 | Introduction of Occupied bandwidth TC for ATG UE | CMCC | 38.521-1 | 2590 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241753 | Introduction of Occupied bandwidth TC for ATG UE | CMCC | 38.521-1 | 2590 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240055 | Introduction of General description of Out of band emission for ATG UE | CMCC | 38.521-1 | 2591 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240056 | Introduction of General description of Spurious emissions for ATG UE | CMCC | 38.521-1 | 2592 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240057 | Introduction of General Spurious emissions TC for ATG UE | CMCC | 38.521-1 | 2593 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241754 | Introduction of General Spurious emissions TC for ATG UE | CMCC | 38.521-1 | 2593 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240058 | Introduction of General description of Rx TCs for ATG UE | CMCC | 38.521-1 | 2594 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240059 | Introduction of Diversity characteristics description for ATG UE | CMCC | 38.521-1 | 2595 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240060 | Introduction of General description of Reference sensitivity for ATG UE | CMCC | 38.521-1 | 2596 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240061 | Introduction of General description of Blocking characteristics for ATG UE | CMCC | 38.521-1 | 2597 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240062 | Introduction of Spurious response TC for ATG UE | CMCC | 38.521-1 | 2598 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241757 | Introduction of Spurious response TC for ATG UE | CMCC | 38.521-1 | 2598 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240063 | Introduction of General description of Intermodulation characteristics for ATG UE | CMCC | 38.521-1 | 2599 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240064 | Introduction of Wide band intermodulation TC for ATG UE | CMCC | 38.521-1 | 2600 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241758 | Introduction of Wide band intermodulation TC for ATG UE | CMCC | 38.521-1 | 2600 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240065 | Introduction of Spurious emissions TC for ATG UE | CMCC | 38.521-1 | 2601 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241759 | Introduction of Spurious emissions TC for ATG UE | CMCC | 38.521-1 | 2601 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240080 | Addition of PC1.5 n39 MOP | CMCC | 38.521-1 | 2602 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-240081 | Addition of PC2 n40 MOP | CMCC | 38.521-1 | 2603 | - | Rel-18 | F | TEI16_Test | agreed |
| R5-240082 | Addition of PC1.5 n39 MPR | CMCC | 38.521-1 | 2604 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-240083 | Addition of PC1.5 n39 Configured tx power requirements | CMCC | 38.521-1 | 2605 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-240084 | Addition of PC1.5 n39 Modified MPR behavior | CMCC | 38.521-1 | 2606 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-240085 | Addition of PC1.5 n39 AMPR | CMCC | 38.521-1 | 2607 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | withdrawn |
| R5-240138 | Update test configuration table for NS_13 | MediaTek Beijing Inc. | 38.521-1 | 2608 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241727 | Update test configuration table for NS_13 | MediaTek Beijing Inc. | 38.521-1 | 2608 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240228 | General updates of TS 38.521-1 clause 5 for R16 CA configurations | CU Digital Technology | 38.521-1 | 2609 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240229 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | CU Digital Technology, Verizon, Nokia, KDDI | 38.521-1 | 2610 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241868 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | CU Digital Technology, Verizon, Nokia, KDDI | 38.521-1 | 2610 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240231 | General updates of clause 5 for R17 new CBW configurations | CU Digital Technology, Nokia | 38.521-1 | 2611 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-241775 | General updates of clause 5 for R17 new CBW configurations | CU Digital Technology, Nokia | 38.521-1 | 2611 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240232 | Update 3DL CA reference sensitivity exceptions TC for CA_n1A-n8A-n78A | CU Digital Technology | 38.521-1 | 2612 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240250 | Addition of DL interruption allowed indication for CA_n1-n3-n78 | China Telecom | 38.521-1 | 2613 | - | Rel-18 | F | DL_intrpt_combos_TxSW_R17-UEConTest | revised |
| R5-241941 | Addition of DL interruption allowed indication for CA_n1-n3-n78 | China Telecom | 38.521-1 | 2613 | 1 | Rel-18 | F | DL_intrpt_combos_TxSW_R17-UEConTest | agreed |
| R5-240264 | Addition of new test case 6.3J.2 Transmit OFF power for ATG | CAICT | 38.521-1 | 2614 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241755 | Addition of new test case 6.3J.2 Transmit OFF power for ATG | CAICT | 38.521-1 | 2614 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240265 | Addition of new test case 6.3J.1 Minimum output power for ATG | CAICT | 38.521-1 | 2615 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241756 | Addition of new test case 6.3J.1 Minimum output power for ATG | CAICT | 38.521-1 | 2615 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240266 | Addition of new test case 7.4J Maximum input level for ATG | CAICT | 38.521-1 | 2616 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241760 | Addition of new test case 7.4J Maximum input level for ATG | CAICT | 38.521-1 | 2616 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240267 | Addition of new test case 7.5J Adjacent channel selectivity for ATG | CAICT | 38.521-1 | 2617 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241761 | Addition of new test case 7.5J Adjacent channel selectivity for ATG | CAICT | 38.521-1 | 2617 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240268 | Addition of new test case 7.6J.2 In-band blocking for ATG | CAICT | 38.521-1 | 2618 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241762 | Addition of new test case 7.6J.2 In-band blocking for ATG | CAICT | 38.521-1 | 2618 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240269 | Addition of new test case 7.6J.3 Out-of-band blocking for ATG | CAICT | 38.521-1 | 2619 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241763 | Addition of new test case 7.6J.3 Out-of-band blocking for ATG | CAICT | 38.521-1 | 2619 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240273 | Addition of asymmetric UL and DL channel bandwidth combinations of band n8 in 5.3.6 | CAICT | 38.521-1 | 2620 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240279 | Addition of PC2 for n8 into TC 6.2.1 MOP | CU Digital Technology | 38.521-1 | 2621 | - | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | agreed |
| R5-240280 | Addition of PC2 for n8 into TC 6.2.3 A-MPR | CU Digital Technology | 38.521-1 | 2622 | - | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | withdrawn |
| R5-240281 | Addition of PC2 for n8 into TC 7.3.2 Reference Sensitivity | CU Digital Technology | 38.521-1 | 2623 | - | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | agreed |
| R5-240307 | Updating test case AMPR for inter-band CA | Huawei, HiSilicon | 38.521-1 | 2624 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240310 | Updating test frequency range for SUL band n83 | Huawei, HiSilicon | 38.521-1 | 2625 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240312 | Updating FR1 PC2 REFSENS exceptions testing | Huawei, HiSilicon | 38.521-1 | 2626 | - | Rel-18 | F | TEI17_Test, NR_PC2_CA_R17_2BDL_2BUL-UEConTest | revised |
| R5-241721 | Updating FR1 PC2 REFSENS exceptions testing | Huawei, HiSilicon | 38.521-1 | 2626 | 1 | Rel-18 | F | TEI17_Test, NR_PC2_CA_R17_2BDL_2BUL-UEConTest | revised |
| R5-241870 | Updating FR1 PC2 REFSENS exceptions testing | Huawei, HiSilicon | 38.521-1 | 2626 | 2 | Rel-18 | F | TEI17_Test, NR_PC2_CA_R17_2BDL_2BUL-UEConTest | agreed |
| R5-240314 | Correcting errors in REFSENS for CA test case for PC3 CA configurations | Huawei, HiSilicon | 38.521-1 | 2627 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241770 | Correcting errors in REFSENS for CA test case for PC3 CA configurations | Huawei, HiSilicon | 38.521-1 | 2627 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241871 | Correcting errors in REFSENS for CA test case for PC3 CA configurations | Huawei, HiSilicon | 38.521-1 | 2627 | 2 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240315 | Editorial correction to test requirement of MPR for CA test case | Huawei, HiSilicon | 38.521-1 | 2628 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240317 | Updating REFSENS testing for SUL configuration | Huawei, HiSilicon | 38.521-1 | 2629 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241771 | Updating REFSENS testing for SUL configuration | Huawei, HiSilicon | 38.521-1 | 2629 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241872 | Updating REFSENS testing for SUL configuration | Huawei, HiSilicon | 38.521-1 | 2629 | 2 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240319 | Updating AMPR testing for SUL band n81 | Huawei, HiSilicon | 38.521-1 | 2630 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240320 | Updating UTRA ACLR testing for SUL band n81 | Huawei, HiSilicon | 38.521-1 | 2631 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240326 | Addition of delta TIBc and UE maximum output power for new R16 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2632 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241728 | Addition of delta TIBc and UE maximum output power for new R16 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2632 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240327 | Addition of spurious emissions, delta TIBc and UE maximum output power for new R17 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2633 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241734 | Addition of spurious emissions, delta TIBc and UE maximum output power for new R17 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2633 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240328 | Addition of delta RIBc and reference sensitivity for new R16 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2634 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241731 | Addition of delta RIBc and reference sensitivity for new R16 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2634 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240329 | Addition of delta RIBc and reference sensitivity for new R17 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2635 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241735 | Addition of delta RIBc and reference sensitivity for new R17 NR CA combos within FR1 | KDDI Corporation | 38.521-1 | 2635 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240333 | Updating PC2 test requirements in MPR test case for band n1 and n3 | Huawei, HiSilicon | 38.521-1 | 2636 | - | Rel-18 | F | TEI17_Test, NR_PC2_UE_FDD-UEConTest | agreed |
| R5-240334 | Addition of Measurement Uncertainties and Test Tolerances for ATG UE | CMCC | 38.521-1 | 2637 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241764 | Addition of Measurement Uncertainties and Test Tolerances for ATG UE | CMCC | 38.521-1 | 2637 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240336 | Addition of new test case 6.3G.4.1 Absolute power tolerance for Tx Diversity | Huawei, HiSilicon | 38.521-1 | 2638 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-240337 | Removal of square brackets for Tx Diversity capability | Huawei, HiSilicon | 38.521-1 | 2639 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | revised |
| R5-241879 | Removal of square brackets for Tx Diversity capability | Huawei, HiSilicon | 38.521-1 | 2639 | 1 | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-240375 | Addition of new test case 6.2H.3.2 MPR for inter-band CA with UL MIMO | Huawei, HiSilicon | 38.521-1 | 2640 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-240443 | Corrections to spurious emissions for CA_n3A-n28A | KDDI Corporation | 38.521-1 | 2641 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240454 | Editorial correction to UE Power Class for n100 and n101 | Nokia, Nokia Shanghai Bell | 38.521-1 | 2642 | - | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | agreed |
| R5-240455 | Correction to Reference sensitivity power level test configuration for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2643 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241772 | Correction to Reference sensitivity power level test configuration for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2643 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240457 | Correction to Reference sensitivity power level for inter-band 4DL CA | Nokia, Nokia Shanghai Bell | 38.521-1 | 2644 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-240463 | Adding Reference sensitivity test requirements for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | 38.521-1 | 2645 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241736 | Adding Reference sensitivity test requirements for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | 38.521-1 | 2645 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240465 | Add UE maximum power requirements for CA_n71A-n78A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2646 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240466 | Add general spurious emissions for UE co-existence requirements for CA_n71A-n77A and CA_n71A-n78A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2647 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240469 | Addition of reference sensitivity channel bandwidths for n25 and n71 | Nokia, T-Mobile USA, Keysight Technologies, Skyworks Solutions, Inc. | 38.521-1 | 2648 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240470 | Addition of 35 MHz CBW for transmitter requirements | Nokia, Nokia Shanghai Bell | 38.521-1 | 2649 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-240619 | FR1 CA - Test requirement correction for n2-n48 combo in test 7.3A.1_1 | Keysight Technologies UK Ltd | 38.521-1 | 2650 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240620 | Corrections for combo n48A-n66A in test case 7.3A.1_1 | Keysight Technologies UK Ltd, Anritsu | 38.521-1 | 2651 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240621 | TT Formula vs MU to be added for FR1 EVM test as in FR2 | Keysight Technologies | 38.521-1 | 2652 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240622 | Message exceptions clarifications for 6.3G.3.3 | Keysight Technologies | 38.521-1 | 2653 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | revised |
| R5-241726 | Message exceptions clarifications for 6.3G.3.3 | Keysight Technologies | 38.521-1 | 2653 | 1 | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | withdrawn |
| R5-240623 | MBW table reference corrected for inter-band case in test case 6.5A.4.1 | Keysight Technologies | 38.521-1 | 2654 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240624 | Added 30kHz SCS for SSB in n53 to be aligned with core specs | Keysight Technologies UK Ltd, Apple | 38.521-1 | 2655 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | revised |
| R5-241776 | Added 30kHz SCS for SSB in n53 to be aligned with core specs | Keysight Technologies UK Ltd, Apple | 38.521-1 | 2655 | 1 | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-240625 | p-Max and p-NR-FR1 adjustment when higherPowerLimit-r17 applies | Keysight Technologies UK Ltd, Mediatek | 38.521-1 | 2656 | - | Rel-18 | F | TEI17_Test, Power_Limit_CA_DC-UEConTest | revised |
| R5-241766 | p-Max and p-NR-FR1 adjustment when higherPowerLimit-r17 applies | Keysight Technologies UK Ltd, Mediatek | 38.521-1 | 2656 | 1 | Rel-18 | F | TEI17_Test, Power_Limit_CA_DC-UEConTest | agreed |
| R5-240772 | Addition of new test case 7.4A.4 Maximum input level for CA (5DL CA) | KTL | 38.521-1 | 2657 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240773 | Addition of new test case 7.5A.4 Adjacent channel selectivity for (5DL CA) | KTL | 38.521-1 | 2658 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240777 | Adding Reference sensitivity test requirements for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2659 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240779 | Add UE maximum power requirements for CA_n1A-n28A and CA_n3A-n28A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2660 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241729 | Add UE maximum power requirements for CA_n1A-n28A and CA_n3A-n28A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2660 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240780 | Add spurious emissions for UE co-existence requirements for CA_n1A_n28A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2661 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241730 | Add spurious emissions for UE co-existence requirements for CA_n1A_n28A | Nokia, Nokia Shanghai Bell | 38.521-1 | 2661 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240810 | Updates to NR-U Tx test cases | Ericsson | 38.521-1 | 2662 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241739 | Updates to NR-U Tx test cases | Ericsson | 38.521-1 | 2662 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240817 | Addition of test case 6.3F.4.3 relative power tolerance for shared spectrum channel access | Ericsson | 38.521-1 | 2663 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240819 | Addition of MU and TT for NR-U test cases | Ericsson | 38.521-1 | 2664 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240821 | Updates of test case 6.2F.4 Configured transmitted power for shared spectrum access | Ericsson | 38.521-1 | 2665 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240826 | Editorial, correcting Test case title for 7.3I.2 in Annex A | Ericsson | 38.521-1 | 2666 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240828 | Addition of RMC tables for shared spectrum access in Annex A | Ericsson | 38.521-1 | 2667 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240835 | Update of 6.3C.1 for minimum output power for SUL bands | ZTE Corporation | 38.521-1 | 2668 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240836 | Update of 6.3C.2 for transmit OFF power for SUL bands | ZTE Corporation | 38.521-1 | 2669 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240880 | Update reference sensitivity requirement for 4DL CA table | Verizon | 38.521-1 | 2670 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-240888 | Update reference sensitivity requirement table for additional band configurations | Verizon | 38.521-1 | 2671 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240893 | Addition of CA_n14A-n77A PC2 and CA_n77(2A) PC2 to Ch 5 | WE Certification Oy, AT&T | 38.521-1 | 2672 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241747 | Addition of CA_n14A-n77A PC2 and CA_n77(2A) PC2 to Ch 5 | WE Certification Oy, AT&T | 38.521-1 | 2672 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240894 | Addition of PC2 max power requirements for bands CA_n77(2A) and CA_n14A-n77A | WE Certification Oy, AT&T | 38.521-1 | 2673 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240896 | Addition of band CA_n14A-n77A PC2 reference sensitivity test | WE Certification, AT&T | 38.521-1 | 2674 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241746 | Addition of band CA_n14A-n77A PC2 reference sensitivity test | WE Certification, AT&T | 38.521-1 | 2674 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240992 | Clarification of trace mode in emission testing_FR1 | Huawei, HiSilicon | 38.521-1 | 2675 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241767 | Clarification of trace mode in emission testing_FR1 | Huawei, HiSilicon | 38.521-1 | 2675 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240997 | Update to message exception of SUL time mask test cases | Huawei, HiSilicon | 38.521-1 | 2676 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240998 | Update to AMPR, ASEM and ASE for intra-band CA for CA_NS_04 | Huawei, HiSilicon | 38.521-1 | 2677 | - | Rel-18 | F | TEI16_Test, NR_RF_FR1 | revised |
| R5-241768 | Update to AMPR, ASEM and ASE for intra-band CA for CA_NS_04 | Huawei, HiSilicon | 38.521-1 | 2677 | 1 | Rel-18 | F | TEI16_Test, NR_RF_FR1 | agreed |
| R5-241001 | Adding AMPR test cases for V2X | Huawei, HiSilicon | 38.521-1 | 2678 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-241733 | Adding AMPR test cases for V2X | Huawei, HiSilicon | 38.521-1 | 2678 | 1 | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241002 | Update to ASEM test cases for V2X | Huawei, HiSilicon | 38.521-1 | 2679 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241003 | Update to ASE test case for V2X | Huawei, HiSilicon | 38.521-1 | 2680 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241007 | Adding the support of NS_47 for PC 1.5 | Huawei, HiSilicon, SoftBank Corp. | 38.521-1 | 2681 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-241009 | Addition of 6.2H.1.3 AMPR for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2682 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241970 | Addition of 6.2H.1.3 AMPR for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2682 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241010 | Addition of 6.3H.1.4 power control for CA with UL MIMO | Huawei, HiSilicon | 38.521-1 | 2683 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241011 | Addition of 6.4H.1.1 frequency error for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2684 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241971 | Addition of 6.4H.1.1 frequency error for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2684 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241012 | Addition of 6.4H.1.2.1 EVM for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2685 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241972 | Addition of 6.4H.1.2.1 EVM for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2685 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241013 | Addition of 6.4H.1.2.2 Carrier leakage for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2686 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241973 | Addition of 6.4H.1.2.2 Carrier leakage for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2686 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241014 | Addition of 6.4H.1.2.3 In-band emission for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2687 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241974 | Addition of 6.4H.1.2.3 In-band emission for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2687 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241015 | Addition of 6.4H.1.3 time alignment error for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2688 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241975 | Addition of 6.4H.1.3 time alignment error for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2688 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241016 | Addition of 6.4H.1.4 Coherent requirement for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2689 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241976 | Addition of 6.4H.1.4 Coherent requirement for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2689 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241017 | Addition of 6.5H.1.1 Occupied bandwidth for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2690 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241977 | Addition of 6.5H.1.1 Occupied bandwidth for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2690 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241018 | Addition of 6.5H.1.2.2 ASEM for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2691 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241978 | Addition of 6.5H.1.2.2 ASEM for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2691 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241019 | Addition of 6.5H.1.3.1 General spurious emission for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2692 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241979 | Addition of 6.5H.1.3.1 General spurious emission for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2692 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241020 | Addition of 6.5H.1.3.2 Spurious emission UE co-existence for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2693 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241980 | Addition of 6.5H.1.3.2 Spurious emission UE co-existence for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2693 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241021 | Addition of 6.5H.1.3.3 Additional spurious emission for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2694 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241981 | Addition of 6.5H.1.3.3 Additional spurious emission for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2694 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241022 | Addition of 6.5H.1.4 Transmit intermodulation for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2695 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241982 | Addition of 6.5H.1.4 Transmit intermodulation for CA with UL-MIMO | Huawei, HiSilicon | 38.521-1 | 2695 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241023 | Update to minimum requirement of 6.4D.4 for UL switching | Huawei, HiSilicon | 38.521-1 | 2696 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241024 | Addition of 6.3A.3.4 1Tx-2Tx UL switching between two bands | Huawei, HiSilicon | 38.521-1 | 2697 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241983 | Addition of 6.3A.3.4 1Tx-2Tx UL switching between two bands | Huawei, HiSilicon | 38.521-1 | 2697 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241025 | Addition of 6.3A.3.5 2Tx-2Tx UL switching between two bands | Huawei, HiSilicon | 38.521-1 | 2698 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241984 | Addition of 6.3A.3.5 2Tx-2Tx UL switching between two bands | Huawei, HiSilicon | 38.521-1 | 2698 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241026 | Addition of 6.3C.3.3 2Tx-2Tx UL switching between two uplink carriers in SUL configuration | Huawei, HiSilicon | 38.521-1 | 2699 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241985 | Addition of 6.3C.3.3 2Tx-2Tx UL switching between two uplink carriers in SUL configuration | Huawei, HiSilicon | 38.521-1 | 2699 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241027 | Addition of 6.3C.3.4 1Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | 38.521-1 | 2700 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241986 | Addition of 6.3C.3.4 1Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | 38.521-1 | 2700 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241028 | Addition of 6.3C.3.5 2Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | 38.521-1 | 2701 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241987 | Addition of 6.3C.3.5 2Tx-2Tx UL switching between two bands in SUL configuration | Huawei, HiSilicon | 38.521-1 | 2701 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241069 | Update to FR1 EN-DC Configurations for n78 and n79 | NTT DOCOMO INC.. | 38.521-1 | 2702 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-241096 | Correction to Reference sensitivity for Rel-16 CA | Anritsu, Huawei, HiSilicon | 38.521-1 | 2703 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241732 | Correction to Reference sensitivity for Rel-16 CA | Anritsu, Huawei, HiSilicon | 38.521-1 | 2703 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241097 | Correction to Reference sensitivity for CA_n28A-n41A-n79A | Anritsu | 38.521-1 | 2704 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241737 | Correction to Reference sensitivity for CA_n28A-n41A-n79A | Anritsu | 38.521-1 | 2704 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241098 | Clarification of asymmetric BW in Rx test cases for CA | Anritsu | 38.521-1 | 2705 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241738 | Clarification of asymmetric BW in Rx test cases for CA | Anritsu | 38.521-1 | 2705 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241099 | Correction to UL configuration for intra-band contiguous CA | Anritsu | 38.521-1 | 2706 | - | Rel-18 | F | TEI16_Test, NR_RF_FR1-UEConTest | agreed |
| R5-241100 | Correction to Rel-15 A-MPR | Anritsu | 38.521-1 | 2707 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241922 | Correction to Rel-15 A-MPR | Anritsu | 38.521-1 | 2707 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241101 | Correction to Rel-16 A-MPR | Anritsu | 38.521-1 | 2708 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-241102 | Correction to test configuration in 6.2D.2 | Anritsu | 38.521-1 | 2709 | - | Rel-18 | F | TEI16_Test, NR_eMIMO-UEConTest | revised |
| R5-241969 | Correction to test configuration in 6.2D.2 | Anritsu | 38.521-1 | 2709 | 1 | Rel-18 | F | TEI16_Test, NR_eMIMO-UEConTest | agreed |
| R5-241103 | Correction to applicability of powerBoosting for PC3 UE | Anritsu | 38.521-1 | 2710 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241104 | Addition of CBW 35 MHz and 45 MHz to OBW for inter-band CA | Anritsu | 38.521-1 | 2711 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-241105 | Addition of CBW 35 MHz, 45 MHz, 70 MHz to ACS for CA | Anritsu | 38.521-1 | 2712 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-241106 | Addition of CBW 35 MHz, 45 MHz, 70 MHz to Narrow band blocking for inter-band CA | Anritsu, Rohde&Schwarz | 38.521-1 | 2713 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-241773 | Addition of CBW 35 MHz, 45 MHz, 70 MHz to Narrow band blocking for inter-band CA | Anritsu, Rohde&Schwarz | 38.521-1 | 2713 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-241121 | Correction to note for inter-band CA including n48 | Anritsu | 38.521-1 | 2714 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241945 | Correction to note for inter-band CA including n48 | Anritsu | 38.521-1 | 2714 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241122 | Correction to Reference sensitivity for CA_n26A-n70A | Anritsu | 38.521-1 | 2715 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-242015 | Correction to Reference sensitivity for CA_n26A-n70A | Anritsu | 38.521-1 | 2715 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241154 | Editorial correction in FR1 test case 6.4G.2.1 | Keysight Technologies UK Ltd | 38.521-1 | 2716 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-241155 | Editorial correction in FR1 test case 6.5A.2.4.1.1 | Keysight Technologies UK Ltd | 38.521-1 | 2717 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241769 | Editorial correction in FR1 test case 6.5A.2.4.1.1 | Keysight Technologies UK Ltd | 38.521-1 | 2717 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241156 | Editorial correction in FR1 test case 6.2A.2.1 | Keysight Technologies | 38.521-1 | 2718 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241177 | Addition of test case 6.5F.3.3 Additional spurious emissions for shared spectrum channel access | Ericsson | 38.521-1 | 2719 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241740 | Addition of test case 6.5F.3.3 Additional spurious emissions for shared spectrum channel access | Ericsson | 38.521-1 | 2719 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241195 | Updates of test case 6.2F.3 UE additional maximum output power reduction for shared spectrum access | Ericsson | 38.521-1 | 2720 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241741 | Updates of test case 6.2F.3 UE additional maximum output power reduction for shared spectrum access | Ericsson | 38.521-1 | 2720 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241196 | Correction of 7.6A.2 for inband blocking for CA | ZTE Corporation, Rohde&Schwarz | 38.521-1 | 2721 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241774 | Correction of 7.6A.2 for inband blocking for CA | ZTE Corporation, Rohde&Schwarz | 38.521-1 | 2721 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241208 | Correction to test case 6.4E.2.4.1D In-band emissions for V2X / non-concurrent operation / SL-MIMO | MTCC, KTL | 38.521-1 | 2722 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241210 | Correction to test case 6.4E.2.4.2 In-band emissions for V2X / con-current operation | MTCC, KTL | 38.521-1 | 2723 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241258 | Updates of test case 6.5F.2.4.2, Shared spectrum channel access ACLR with additional requirement for NS_29 | Ericsson | 38.521-1 | 2724 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241742 | Updates of test case 6.5F.2.4.2, Shared spectrum channel access ACLR with additional requirement for NS_29 | Ericsson | 38.521-1 | 2724 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241260 | Update of Operating bands for UL MIMO band n5 | China Unicom | 38.521-1 | 2725 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-241261 | Addition of n5 into TC 6.2D.1 MOP for UL MIMO | China Unicom | 38.521-1 | 2726 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | revised |
| R5-241745 | Addition of n5 into TC 6.2D.1 MOP for UL MIMO | China Unicom | 38.521-1 | 2726 | 1 | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-241263 | Addition of n5 into TC 6.2D.2 MPR for UL MIMO | China Unicom | 38.521-1 | 2727 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-241349 | Update of 4DL CA test cases | ROHDE & SCHWARZ, Verizon | 38.521-1 | 2728 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241907 | Update of 4DL CA test cases | ROHDE & SCHWARZ, Verizon | 38.521-1 | 2728 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241352 | Update of Narrow Band Blocking for CA test case | ROHDE & SCHWARZ | 38.521-1 | 2729 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-241354 | Correction in A-MPR test case | ROHDE & SCHWARZ | 38.521-1 | 2730 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241367 | General updates of Spurious emissions for UE co-existence for Inter-band CA | China Unicom | 38.521-1 | 2731 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241880 | General updates of Spurious emissions for UE co-existence for Inter-band CA | China Unicom | 38.521-1 | 2731 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241369 | Update of Refsens TC for RedCap UE | China Unicom | 38.521-1 | 2732 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241432 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | 38.521-1 | 2733 | - | Rel-18 | F | NR_cov_enh-UEConTest | revised |
| R5-241947 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | 38.521-1 | 2733 | 1 | Rel-18 | F | NR_cov_enh-UEConTest | agreed |
| R5-241434 | Updates and corrections to Annex in FR1 RF spec | Apple Benelux B.V. | 38.521-1 | 2734 | - | Rel-18 | F | NR_cov_enh-UEConTest | revised |
| R5-241948 | Updates and corrections to Annex in FR1 RF spec | Apple Benelux B.V. | 38.521-1 | 2734 | 1 | Rel-18 | F | NR_cov_enh-UEConTest | agreed |
| R5-241779 | Corrections for 6.3G.3.3 | Keysight Technologies | 38.521-1 | 2735 | 1 | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-240403 | FR2c MU - Tx test cases update in 38.521-2 | Keysight Technologies UK Ltd | 38.521-2 | 1015 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240404 | FR2c MU - Rx test cases update in 38.521-2 | Keysight Technologies UK Ltd | 38.521-2 | 1016 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241860 | FR2c MU - Rx test cases update in 38.521-2 | Keysight Technologies UK Ltd | 38.521-2 | 1016 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240407 | FR2 MU - PC1 UL MIMO - Minimum output power test - 38.521-2 | Keysight Technologies UK Ltd | 38.521-2 | 1017 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240409 | Blocking measurement procedure updates in section K.1.8 | Keysight Technologies | 38.521-2 | 1018 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240604 | CR on Coarse&Fine Beam Peak Search Grids | Keysight Technologies UK Ltd, CAICT | 38.521-2 | 1019 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240626 | FR2 DL RMCs - Missing notes update | Keysight Technologies | 38.521-2 | 1020 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240837 | Corrections on 6.2.3 for FR2 Redcap UE MPR test case for NS_202 and NS_203 | ZTE Corporation | 38.521-2 | 1021 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241781 | Corrections on 6.2.3 for FR2 Redcap UE MPR test case for NS_202 and NS_203 | ZTE Corporation | 38.521-2 | 1021 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240838 | Corrections on 6.3.1 for FR2 Redcap UE minimum output power | ZTE Corporation | 38.521-2 | 1022 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240876 | Update CA bandwidth classes table | Verizon | 38.521-2 | 1023 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-240962 | Update to FR2 ACS TC | Qualcomm Technologies Ireland | 38.521-2 | 1024 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240993 | Clarification of trace mode in emission testing_FR2 | Huawei, HiSilicon | 38.521-2 | 1025 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241783 | Clarification of trace mode in emission testing_FR2 | Huawei, HiSilicon | 38.521-2 | 1025 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240995 | Adding FR2 test case of SRS time mask | Huawei, HiSilicon | 38.521-2 | 1026 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241990 | Adding FR2 test case of SRS time mask | Huawei, HiSilicon | 38.521-2 | 1026 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241005 | Update to MU and TT for AMPR for CA test case | Huawei, HiSilicon, China Unicom | 38.521-2 | 1027 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241084 | Update for FR2c MU | Anritsu, Keysight | 38.521-2 | 1028 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | revised |
| R5-241859 | Update for FR2c MU | Anritsu, Keysight | 38.521-2 | 1028 | 1 | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-241107 | Clarification of test procedure of EIS spherical coverage for inter-band CA | Anritsu | 38.521-2 | 1029 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241174 | Correction to CA A-MPR requirements | Rohde & Schwarz | 38.521-2 | 1030 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241343 | Correction of MPR CA test cases | ROHDE & SCHWARZ | 38.521-2 | 1031 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241353 | Editorial correction of TT for Minimum Output Power for UL MIMO | ROHDE & SCHWARZ | 38.521-2 | 1032 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241405 | Updates to Annex F for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-2 | 1033 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-242026 | Updates to Annex F for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-2 | 1033 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-241406 | Updates to UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-2 | 1034 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-242025 | Updates to UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-2 | 1034 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-241429 | Update to FR2 Tx Power test with UL-Gaps | Apple Benelux B.V. | 38.521-2 | 1035 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-241966 | Update to FR2 Tx Power test with UL-Gaps | Apple Benelux B.V. | 38.521-2 | 1035 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-241430 | Update to FR2 Tx OFF Power test specific to UL-Gaps | Apple Benelux B.V. | 38.521-2 | 1036 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-241431 | Updates to FR2 ACS test | Apple Benelux B.V. | 38.521-2 | 1037 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-241780 | Updates to FR2 ACS test | Apple Benelux B.V. | 38.521-2 | 1037 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-241433 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | 38.521-2 | 1038 | - | Rel-18 | F | NR_cov_enh-UEConTest | revised |
| R5-241949 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | 38.521-2 | 1038 | 1 | Rel-18 | F | NR_cov_enh-UEConTest | agreed |
| R5-241435 | Updates to Annex E content and structure | Apple Benelux B.V. | 38.521-2 | 1039 | - | Rel-18 | F | NR_cov_enh-UEConTest | revised |
| R5-241950 | Updates to Annex E content and structure | Apple Benelux B.V. | 38.521-2 | 1039 | 1 | Rel-18 | F | NR_cov_enh-UEConTest | agreed |
| R5-241448 | Addition of CA test for EIRP test with ULGaps | Apple Trading | 38.521-2 | 1040 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-241782 | Addition of CA test for EIRP test with ULGaps | Apple Trading | 38.521-2 | 1040 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-240260 | Removal of LTE anchor agnostic approach testing in 6.5B.3.3.2 | CAICT | 38.521-3 | 1712 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241991 | Removal of LTE anchor agnostic approach testing in 6.5B.3.3.2 | CAICT | 38.521-3 | 1712 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240262 | Correction of applicability and test coverage rules for SA and NSA capable devices | CAICT | 38.521-3 | 1713 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241905 | Correction of applicability and test coverage rules for SA and NSA capable devices | CAICT | 38.521-3 | 1713 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240263 | Correction of applicability for test with LTE anchor agnostic approach in 6.5B.3.3.1 | CAICT | 38.521-3 | 1714 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241923 | Correction of applicability for test with LTE anchor agnostic approach in 6.5B.3.3.1 | CAICT | 38.521-3 | 1714 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240305 | Correction to test configuration for non-exception REFSENS testing for 3CC EN-DC | Huawei, HiSilicon | 38.521-3 | 1715 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240331 | Addition of UE maximum output power for new PC2 EN-DC combos within FR1 | KDDI Corporation | 38.521-3 | 1716 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240332 | Addition of reference sensitivity for new PC2 EN-DC combos within FR1 | KDDI Corporation | 38.521-3 | 1717 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241904 | Addition of reference sensitivity for new PC2 EN-DC combos within FR1 | KDDI Corporation | 38.521-3 | 1717 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240338 | Update of MOP test configuration for DC_20A_n28A | Huawei, HiSilicon | 38.521-3 | 1718 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240339 | Correction to H3 title for clauses including NE-DC test cases | Huawei, HiSilicon | 38.521-3 | 1719 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240376 | Addition of new test case 6.2H.2.3 MPR for inter-band EN-DC with UL MIMO | Huawei, HiSilicon | 38.521-3 | 1720 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | revised |
| R5-241725 | Addition of new test case 6.2H.2.3 MPR for inter-band EN-DC with UL MIMO | Huawei, HiSilicon | 38.521-3 | 1720 | 1 | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-240405 | FR2c MU - Updates in 38.521-3 | Keysight Technologies | 38.521-3 | 1721 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240627 | Minimum requirements update for DC_5A_n78A | Keysight Technologies | 38.521-3 | 1722 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240879 | Addition of general spurious emissions test for EN-DC combos DC_2A_n66A and DC_30A_n66A | WE Certification, AT&T | 38.521-3 | 1723 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240884 | Addition of many EN-DC PC2 combos to Ch 5 | WE Certification Oy, AT&T | 38.521-3 | 1724 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240885 | Addition of test requirements for EN-DC PC2 combos DC_12A_n77A, DC_14A_n77A and DC_30A_n77A max power test | WE Certification, AT&T | 38.521-3 | 1725 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240916 | Update reference sensitity minimum requirement for additional ENDC band configurations with PC2 UL | Verizon Spain | 38.521-3 | 1726 | - | Rel-18 | F | ENDC_PC2_R17_xLTE_yNR-UEConTest | withdrawn |
| R5-240917 | Update reference sensitity test configuration and test requirement tables for ENDC band configurations with PC2 UL | Verizon Spain | 38.521-3 | 1727 | - | Rel-18 | F | ENDC_PC2_R17_xLTE_yNR-UEConTest | withdrawn |
| R5-240933 | Update to R15 Configuration for DC | Bureau Veritas ADT | 38.521-3 | 1728 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240934 | Update to R16 Configuration for DC | Bureau Veritas ADT, KDDI, WE Certification, AT&T | 38.521-3 | 1729 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240935 | Update to R17 Configuration for DC | Bureau Veritas ADT | 38.521-3 | 1730 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-240936 | Update to R18 Configuration for DC | Bureau Veritas ADT | 38.521-3 | 1731 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-240967 | Correct clause 6.2B.4.1.2 initial conditions | SGS Wireless | 38.521-3 | 1732 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241789 | Correct clause 6.2B.4.1.2 initial conditions | SGS Wireless | 38.521-3 | 1732 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240994 | Clarification of trace mode in emission testing_Iw | Huawei, HiSilicon | 38.521-3 | 1733 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241784 | Clarification of trace mode in emission testing_Iw | Huawei, HiSilicon | 38.521-3 | 1733 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241058 | Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3 | NTT DOCOMO INC. | 38.521-3 | 1734 | - | Rel-18 | F | TEI15_Test | withdrawn |
| R5-241059 | Addition of REFSENS for 2CC EN-DC for 19A-n78A in PC2 | NTT DOCOMO INC. | 38.521-3 | 1735 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-241060 | Update of REFSENS for 3CC EN-DC in PC3 | NTT DOCOMO INC. | 38.521-3 | 1736 | - | Rel-18 | F | TEI15_Test | withdrawn |
| R5-241061 | Addition of REFSENS for 3CC EN-DC in PC2 | NTT DOCOMO INC. | 38.521-3 | 1737 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-241062 | Addition and correction of REFSENS for 2CC EN-DC in PC2 | NTT DOCOMO INC. | 38.521-3 | 1738 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-241064 | Update of REFSENS for 3CC EN-DC in PC3 | NTT DOCOMO INC. | 38.521-3 | 1739 | - | Rel-18 | F | TEI15_Test | withdrawn |
| R5-241066 | Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3 | NTT DOCOMO INC. | 38.521-3 | 1740 | - | Rel-18 | F | TEI15_Test | revised |
| R5-241992 | Update of REFSENS for DC_19A_n77A and DC_19A_n78A in PC3 | NTT DOCOMO INC. | 38.521-3 | 1740 | 1 | Rel-18 | F | TEI15_Test | agreed |
| R5-241068 | Addition of REFSENS for 3CC EN-DC in PC2 | NTT DOCOMO INC., Verizon | 38.521-3 | 1741 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241989 | Addition of REFSENS for 3CC EN-DC in PC2 | NTT DOCOMO INC., Verizon | 38.521-3 | 1741 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-241072 | Update to FR1 EN-DC Configurations for n78 and n79 | NTT DOCOMO INC.. | 38.521-3 | 1742 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-241079 | Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1743 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241785 | Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1743 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241080 | Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1744 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241786 | Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1744 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241081 | Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1745 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241787 | Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1745 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241082 | Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1746 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241788 | Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs | NTT DOCOMO INC.. | 38.521-3 | 1746 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241085 | Update for FR2c MU | Anritsu, Keysight | 38.521-3 | 1747 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | revised |
| R5-241861 | Update for FR2c MU | Anritsu, Keysight | 38.521-3 | 1747 | 1 | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-241108 | Correction to Reference sensitivity for DC_21A_n28A | Anritsu | 38.521-3 | 1748 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241109 | Correction to note application in Table 7.3B.2.0.3.4-2 | Anritsu | 38.521-3 | 1749 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241110 | Correction to p-Max value in Rx test cases for FR1 intra-band contiguous EN-DC | Anritsu, Eurofins KCTL, Huawei, HiSilicon | 38.521-3 | 1750 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241111 | Correction to UL power control for FR1 intra-band non-contiguous EN-DC | Anritsu | 38.521-3 | 1751 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241112 | Correction to p-Max value in out of band test cases for FR1 inter-band EN-DC | Anritsu | 38.521-3 | 1752 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241924 | Correction to p-Max value in out of band test cases for FR1 inter-band EN-DC | Anritsu | 38.521-3 | 1752 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241172 | Editorial correction to FR1 inter-band co-existence | Rohde & Schwarz | 38.521-3 | 1753 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241790 | Editorial correction to FR1 inter-band co-existence | Rohde & Schwarz | 38.521-3 | 1753 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241407 | Updates to Annex F for 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-3 | 1754 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-242028 | Updates to Annex F for 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-3 | 1754 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-241408 | Updates to 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-3 | 1755 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-242027 | Updates to 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.521-3 | 1755 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-240218 | Corrections to test parameters for CSI test cases | MediaTek (Hefei) Inc. | 38.521-4 | 0791 | - | Rel-18 | F | TEI15_Test | agreed |
| R5-240251 | Addition of MMSE-IRC CQI reporting test case with FDD 4Rx | China Telecom | 38.521-4 | 0792 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241811 | Addition of MMSE-IRC CQI reporting test case with FDD 4Rx | China Telecom | 38.521-4 | 0792 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-240252 | Addition of MMSE-IRC CQI reporting test case with TDD 2Rx | China Telecom | 38.521-4 | 0793 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241812 | Addition of MMSE-IRC CQI reporting test case with TDD 2Rx | China Telecom | 38.521-4 | 0793 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-240253 | Addition of MMSE-IRC CQI reporting test case with TDD 4Rx | China Telecom | 38.521-4 | 0794 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241813 | Addition of MMSE-IRC CQI reporting test case with TDD 4Rx | China Telecom | 38.521-4 | 0794 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-240254 | Completion of MMSE-IRC CQI reporting test case with FDD 2Rx | China Telecom | 38.521-4 | 0795 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241814 | Completion of MMSE-IRC CQI reporting test case with FDD 2Rx | China Telecom | 38.521-4 | 0795 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-240507 | Adding measurement uncertainty and test tolerance for Demod PDCCH with intra-slot repetition test cases | Samsung | 38.521-4 | 0796 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-241959 | Adding measurement uncertainty and test tolerance for Demod PDCCH with intra-slot repetition test cases | Samsung | 38.521-4 | 0796 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-240677 | Addition of RedCap demod test case 6.3.1.1.1 | Huawei, HiSilicon | 38.521-4 | 0797 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241960 | Addition of RedCap demod test case 6.3.1.1.1 | Huawei, HiSilicon | 38.521-4 | 0797 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240678 | Addition of RedCap demod test case 6.3.1.2.1 | Huawei, HiSilicon | 38.521-4 | 0798 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241817 | Addition of RedCap demod test case 6.3.1.2.1 | Huawei, HiSilicon | 38.521-4 | 0798 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240679 | Addition of RedCap demod test case 6.3.2.2.7 | Huawei, HiSilicon | 38.521-4 | 0799 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241961 | Addition of RedCap demod test case 6.3.2.2.7 | Huawei, HiSilicon | 38.521-4 | 0799 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240827 | Correction of TT for RedCap demod test case 5.3.2.1.4 and 5.3.2.2.4 | Ericsson | 38.521-4 | 0800 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240829 | Addition of test case 6.2A.4.1.1, 4Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA) | Ericsson | 38.521-4 | 0801 | - | Rel-18 | F | NR_unlic-UEConTest | withdrawn |
| R5-240830 | Core spec alignment, addition of RMC and UL-DL configuration for NR-U | Ericsson | 38.521-4 | 0802 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241822 | Core spec alignment, addition of RMC and UL-DL configuration for NR-U | Ericsson | 38.521-4 | 0802 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240864 | Addition of Measurement uncertainty for Redcap PDSCH demodulation test cases | Keysight Technologies UK Ltd | 38.521-4 | 0803 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240877 | Update on test case 7.5.1 to add FR2 testing direction | Keysight Technologies | 38.521-4 | 0804 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241113 | Correction to 1Rx FR1 PDSCH and PDCCH performance for RedCap | Anritsu | 38.521-4 | 0805 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241137 | Addition of SDR RMC for more FDD channel BW | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0806 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241819 | Addition of SDR RMC for more FDD channel BW | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0806 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241138 | Update Es for 1024QAM scenarios | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0807 | - | Rel-18 | F | TEI17_Test, NR_DL1024QAM_FR1-UEConTest | revised |
| R5-241684 | Update Es for 1024QAM scenarios | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0807 | 1 | Rel-18 | F | TEI17_Test, NR_DL1024QAM_FR1-UEConTest | agreed |
| R5-241139 | Update to NR-U demod test cases | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0808 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241821 | Update to NR-U demod test cases | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0808 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241141 | Addition of FR2 SDR test for RedCap | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0809 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241142 | Addition of 6.2.1.1.2.1 test for RedCap | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0810 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241143 | Update incorrect reference to SDR test in Table 5.1.1.11-1 for RedCap | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0811 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241144 | Correction to numCDM group for PDCCH test cases | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0812 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-241145 | Update to power imbalance test cases | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0813 | - | Rel-18 | F | TEI16_Test, NR_perf_enh-UEConTest | agreed |
| R5-241146 | Correction to RMC name in Table A.3.2.2.2-29 | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0814 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-241147 | Correction to RMC name in Table A.3.2.1.1-18 | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0815 | - | Rel-18 | F | TEI17_Test, NR_demod_enh2-UEConTest | revised |
| R5-242019 | Correction to RMC name in Table A.3.2.1.1-18 | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0815 | 1 | Rel-18 | F | TEI17_Test, NR_demod_enh2-UEConTest | agreed |
| R5-241151 | Update to FR1 PDSCH inter-cell interference test case | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0816 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241815 | Update to FR1 PDSCH inter-cell interference test case | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0816 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-241152 | Update to FR1 PDSCH intra-cell interference test case | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0817 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241816 | Update to FR1 PDSCH intra-cell interference test case | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0817 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-241157 | HST-DPS channel profile clarifications | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0818 | - | Rel-18 | F | TEI16_Test, NR_perf_enh-UEConTest | agreed |
| R5-241169 | Corrections to URLLC Test Cases | Rohde & Schwarz | 38.521-4 | 0819 | - | Rel-18 | F | TEI16_Test, NR_L1enh_URLLC-UEConTest | revised |
| R5-241818 | Corrections to URLLC Test Cases | Rohde & Schwarz | 38.521-4 | 0819 | 1 | Rel-18 | F | TEI16_Test, NR_L1enh_URLLC-UEConTest | agreed |
| R5-241170 | Correction to FR2 SDR requirements | Rohde & Schwarz | 38.521-4 | 0820 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241171 | Correction to FR2 256QAM CQI reporting | Rohde & Schwarz | 38.521-4 | 0821 | - | Rel-18 | F | TEI16_Test, NR_DL256QAM_FR2-UEConTest | agreed |
| R5-241173 | Correction to CSI reference measurement channels for 256QAM | Rohde & Schwarz | 38.521-4 | 0822 | - | Rel-18 | F | TEI16_Test, NR_DL256QAM_FR2-UEConTest | withdrawn |
| R5-241175 | Correction RedCap CSI Test Case message exceptions | Rohde & Schwarz | 38.521-4 | 0823 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241370 | Editorial correction to 5.2.2.1.1_1 Modulation format | Bureau Veritas ADT | 38.521-4 | 0824 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241418 | Correction to max throughput values for PDSCH RMC | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0825 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241820 | Correction to max throughput values for PDSCH RMC | QUALCOMM Europe Inc. - Italy | 38.521-4 | 0825 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240201 | Addition and correction to the NTN related abbreviations in 38.521-5 | MediaTek (Hefei) Inc. | 38.521-5 | 0002 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242031 | Addition and correction to the NTN related abbreviations in 38.521-5 | MediaTek (Hefei) Inc. | 38.521-5 | 0002 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240840 | Corrections on 5.3.3 for minimum guardband and transmission bandwidth configuration | ZTE Corporation | 38.521-5 | 0003 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240841 | Corrections on 6.2.2 for maximum output power reduction | ZTE Corporation | 38.521-5 | 0004 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240842 | Corrections on 6.2.3 for additional maximum output power reduction | ZTE Corporation | 38.521-5 | 0005 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241805 | Corrections on 6.2.3 for additional maximum output power reduction | ZTE Corporation | 38.521-5 | 0005 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240843 | Corrections on 6.3.2 for Transmit OFF power | ZTE Corporation | 38.521-5 | 0006 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240844 | Corrections on 6.3.3 for Transmit ON OFF time mask | ZTE Corporation | 38.521-5 | 0007 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241806 | Corrections on 6.3.3 for Transmit ON OFF time mask | ZTE Corporation | 38.521-5 | 0007 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240845 | Corrections on 6.4.1 for Frequency error | ZTE Corporation | 38.521-5 | 0008 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240846 | Corrections on 6.5.2 for out of band emission | ZTE Corporation | 38.521-5 | 0009 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240847 | Corrections on 6.5.3 for spurious emission | ZTE Corporation | 38.521-5 | 0010 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241807 | Corrections on 6.5.3 for spurious emission | ZTE Corporation | 38.521-5 | 0010 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240848 | Corrections on 6.5.4 for transmit intermodulation | ZTE Corporation | 38.521-5 | 0011 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240849 | Corrections on 7.3.2 for Reference sensitivity power level | ZTE Corporation | 38.521-5 | 0012 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241877 | Corrections on 7.3.2 for Reference sensitivity power level | ZTE Corporation | 38.521-5 | 0012 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240850 | Corrections on 7.4 for maximum input level | ZTE Corporation | 38.521-5 | 0013 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240851 | Corrections on 7.5 for adjacent channel selectivity | ZTE Corporation | 38.521-5 | 0014 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240852 | Corrections on 7.6 for blocking characteristics | ZTE Corporation | 38.521-5 | 0015 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240853 | Corrections on A.3.2.1.1 for the reference channel for NTN PDSCH requirement | ZTE Corporation | 38.521-5 | 0016 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240854 | Update of 6.2.1 for maximum output power | ZTE Corporation | 38.521-5 | 0017 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240855 | Update of chapter 4 for RF general description | ZTE Corporation | 38.521-5 | 0018 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241810 | Update of chapter 4 for RF general description | ZTE Corporation | 38.521-5 | 0018 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240955 | Update to NTN Frequency Error TC | Qualcomm Technologies Ireland | 38.521-5 | 0019 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240956 | Update to NTN Add Spurious Emission TC | Qualcomm Technologies Ireland | 38.521-5 | 0020 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242008 | Update to NTN Add Spurious Emission TC | Qualcomm Technologies Ireland | 38.521-5 | 0020 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240957 | Update to NTN General Spurious emission TC | Qualcomm Technologies Ireland | 38.521-5 | 0021 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242009 | Update to NTN General Spurious emission TC | Qualcomm Technologies Ireland | 38.521-5 | 0021 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240958 | Updates to NTN Spur emission UE Coex | Qualcomm Technologies Ireland | 38.521-5 | 0022 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242010 | Updates to NTN Spur emission UE Coex | Qualcomm Technologies Ireland | 38.521-5 | 0022 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240959 | Update to NTN Tx Intermod TC | Qualcomm Technologies Ireland, ZTE | 38.521-5 | 0023 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242011 | Update to NTN Tx Intermod TC | Qualcomm Technologies Ireland, ZTE | 38.521-5 | 0023 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240960 | Update to NTN RefSens TC | Qualcomm Technologies Ireland | 38.521-5 | 0024 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240961 | Update to NTN Annex F MU TT | Qualcomm Technologies Ireland | 38.521-5 | 0025 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242012 | Update to NTN Annex F MU TT | Qualcomm Technologies Ireland | 38.521-5 | 0025 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241148 | Update to PDSCH Mapping Type A test case for Satellite Access | QUALCOMM Europe Inc. - Italy | 38.521-5 | 0026 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241149 | General updates to RF NTN clauses | QUALCOMM Europe Inc. - Italy | 38.521-5 | 0027 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241361 | Splitting the NR NTN frequency error test case | ROHDE & SCHWARZ | 38.521-5 | 0028 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242013 | Splitting the NR NTN frequency error test case | ROHDE & SCHWARZ | 38.521-5 | 0028 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241394 | UL RMCs updates for NR NTN | Keysight Technologies UK Ltd | 38.521-5 | 0029 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242018 | UL RMCs updates for NR NTN | Keysight Technologies UK Ltd | 38.521-5 | 0029 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241395 | Updates to NR NTN Frequency error test | Keysight Technologies UK Ltd | 38.521-5 | 0030 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241808 | Updates to NR NTN Frequency error test | Keysight Technologies UK Ltd | 38.521-5 | 0030 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-241396 | Updates to NR NTN Minimum output power test | Keysight Technologies UK Ltd | 38.521-5 | 0031 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241397 | Updates to NR NTN Transmit OFF power test | Keysight Technologies UK Ltd | 38.521-5 | 0032 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241398 | Updates to NR NTN Maximum Input Level test | Keysight Technologies UK Ltd | 38.521-5 | 0033 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241440 | Updates to NTN TC 6.3.3 on Tx on-off time mask | Apple Benelux B.V. | 38.521-5 | 0034 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241441 | Updates to NTN TC 6.5.2.2 on Spectrum emission mask | Apple Benelux B.V. | 38.521-5 | 0035 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241442 | Updates to NTN TC 6.5.2.4 on ACLR | Apple Benelux B.V. | 38.521-5 | 0036 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240066 | Addition of TC applicability statements for ATG UE | CMCC, CAICT | 38.522 | 0366 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241842 | Addition of TC applicability statements for ATG UE | CMCC, CAICT | 38.522 | 0366 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240089 | Update to R16 NR CADC configuration test cases applicability | CMCC, Sporton | 38.522 | 0367 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241843 | Update to R16 NR CADC configuration test cases applicability | CMCC, Sporton | 38.522 | 0367 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240090 | Update to R17 NR CADC configuration test cases applicability | CMCC | 38.522 | 0368 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-240091 | Update to R18 NR CADC configuration test cases applicability | CMCC | 38.522 | 0369 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-240212 | Editorial Correction to HST TCs on release information | MediaTek Beijing Inc. | 38.522 | 0370 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-240255 | Addition of MMSE-IRC CQI reporting test applicability rule | China Telecom | 38.522 | 0371 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241845 | Addition of MMSE-IRC CQI reporting test applicability rule | China Telecom | 38.522 | 0371 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-240261 | Update of Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3 | CAICT | 38.522 | 0372 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241993 | Update of Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3 | CAICT | 38.522 | 0372 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240368 | Applicability statement for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1 | China Telecom | 38.522 | 0373 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-240400 | Update of Applicability and Additional Information of RF conformance test cases for Satellite Access | CAICT | 38.522 | 0374 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241910 | Update of Applicability and Additional Information of RF conformance test cases for Satellite Access | CAICT, ROHDE & SCHWARZ, Keysight Technologies UK Ltd, Apple Benelux B.V., QUALCOMM Europe Inc. - Italy | 38.522 | 0374 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240505 | Addition of applicability for FeMIMO test cases | Samsung | 38.522 | 0375 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-241847 | Addition of applicability for FeMIMO test cases | Samsung | 38.522 | 0375 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-241675 | Addition of applicability for FeMIMO test cases | Samsung | 38.522 | 0375 | 2 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-240508 | Addition of applicability for HST FR2 test cases | Samsung | 38.522 | 0376 | - | Rel-18 | F | NR_HST_FR2-UEConTest | agreed |
| R5-240520 | Removal of NOTE 1 from applicability of FR2 feMIMO test case | Nokia, Nokia Shanghai Bell | 38.522 | 0377 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-240771 | Correction to applicability for NR-U test cases | TTA | 38.522 | 0378 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240783 | Removal of NOTE 1 from applicability of FR1 and FR2 MRDC test cases | Nokia, Nokia Shanghai Bell | 38.522 | 0379 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240791 | Addition of event triggered reporting test cases applicability | Ericsson | 38.522 | 0380 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241849 | Addition of event triggered reporting test cases applicability | Ericsson | 38.522 | 0380 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240792 | Addition of applicability of the NR-U RRM test cases | Ericsson | 38.522 | 0381 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241702 | Addition of applicability of the NR-U RRM test cases | Ericsson | 38.522 | 0381 | 1 | Rel-18 | F | NR_unlic-UEConTest | withdrawn |
| R5-240831 | Addition of applicability for RedCap test case 6.2.2.1.2.4 | Ericsson | 38.522 | 0382 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240857 | Addition of applicability for NR-U test cases | Ericsson | 38.522 | 0383 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240889 | Update to RRM Power saving enhancement 5.5.5.9 test case applicability | Keysight Technologies | 38.522 | 0384 | - | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | revised |
| R5-241850 | Update to RRM Power saving enhancement 5.5.5.9 test case applicability | Keysight Technologies | 38.522 | 0384 | 1 | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240932 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Rohde&Schwarz | 38.522 | 0385 | - | Rel-18 | F | TEI15_Test | agreed |
| R5-241004 | Adding test applicability for V2X test cases | Huawei, HiSilicon | 38.522 | 0386 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241030 | Update to test applicability for R17 FR1 enhancement | Huawei, HiSilicon | 38.522 | 0387 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241988 | Update to test applicability for R17 FR1 enhancement | Huawei, HiSilicon | 38.522 | 0387 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241131 | Adding applicability for newly introduced NR-U test cases | Ericsson | 38.522 | 0388 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241140 | Update to applicability spec for Redcap Demod test case | QUALCOMM Europe Inc. - Italy | 38.522 | 0389 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241153 | Applicability update for PDSCH interference test cases | QUALCOMM Europe Inc. - Italy | 38.522 | 0390 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-241846 | Applicability update for PDSCH interference test cases | QUALCOMM Europe Inc. - Italy | 38.522 | 0390 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-241204 | Add information to non-TXD test cases with UE supports TXD | Sporton | 38.522 | 0391 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241873 | Add information to non-TXD test cases with UE supports TXD | Sporton | 38.522 | 0391 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241239 | Correction to applicability notes for FR2 RRM RLM test cases | Rohde & Schwarz | 38.522 | 0392 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241240 | Addition of missing applicability to new SS-RSRQ RedCap test cases | ROHDE & SCHWARZ | 38.522 | 0393 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241962 | Addition of missing applicability to new SS-RSRQ RedCap test cases | ROHDE & SCHWARZ | 38.522 | 0393 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241287 | Update to NR-U test applicability | Qualcomm Germany, Ericsson | 38.522 | 0394 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241851 | Update to NR-U test applicability | Qualcomm Germany, Ericsson | 38.522 | 0394 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241302 | Applcability update for several RedCap tests | Qualcomm Germany | 38.522 | 0395 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241305 | Update to Applicability General Section | Qualcomm Germany | 38.522 | 0396 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241306 | Update to test selection criteria for RRM tests | Qualcomm Germany | 38.522 | 0397 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241841 | Update to test selection criteria for RRM tests | Qualcomm Germany | 38.522 | 0397 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241348 | Alignment of status of FR2 UL MIMO test cases | ROHDE & SCHWARZ | 38.522 | 0398 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241350 | Update of applicability for FR1 4DL CA test cases | ROHDE & SCHWARZ | 38.522 | 0399 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241946 | Update of applicability for FR1 4DL CA test cases | ROHDE & SCHWARZ | 38.522 | 0399 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241362 | Aligning the applicability for NR NTN frequency error | ROHDE & SCHWARZ | 38.522 | 0400 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241906 | Aligning the applicability for NR NTN frequency error | ROHDE & SCHWARZ | 38.522 | 0400 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-241379 | Add subtest selection criteria to RedCap Performance test cases | Bureau Veritas ADT | 38.522 | 0401 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241380 | Applicability spec update for NR NTN test cases | QUALCOMM Europe Inc. - Italy | 38.522 | 0402 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-241399 | Applicability updates for NR NTN Minimum output power and maximum Input level tests | Keysight Technologies UK Ltd | 38.522 | 0403 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-241438 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | 38.522 | 0404 | - | Rel-18 | F | NR_cov_enh-UEConTest | revised |
| R5-241844 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | 38.522 | 0404 | 1 | Rel-18 | F | NR_cov_enh-UEConTest | agreed |
| R5-241444 | Applicability updates for NTN RF tests | Apple Benelux B.V. | 38.522 | 0405 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-241446 | Addition of applicability for RedCap Demod and RRM test cases | Huawei, HiSilicon | 38.522 | 0406 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241848 | Addition of applicability for RedCap Demod and RRM test cases | Huawei, HiSilicon | 38.522 | 0406 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240022 | Correction to NR5GC test cases 11.3.6 and 11.3.6a | Keysight Technologies UK Ltd | 38.523-1 | 4156 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241527 | Correction to NR5GC test cases 11.3.6 and 11.3.6a | Keysight Technologies UK Ltd | 38.523-1 | 4156 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240028 | Editorial correction for test procedure sequence in 11.1.10 | NTT DOCOMO, INC. | 38.523-1 | 4158 | - | Rel-17 | F | ING_5GS-UEConTest | revised |
| R5-241591 | Editorial correction for test procedure sequence in 11.1.10 | NTT DOCOMO, INC. | 38.523-1 | 4158 | 1 | Rel-17 | F | ING_5GS-UEConTest | agreed |
| R5-240034 | Correction to MUSIM test case 8.1.5.10.3 | CATT, TDIA | 38.523-1 | 4159 | - | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | revised |
| R5-241517 | Correction to MUSIM test case 8.1.5.10.3 | CATT, TDIA | 38.523-1 | 4159 | 1 | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | agreed |
| R5-240040 | Add new ING_5GS test case 11.1.11 | China Telecom | 38.523-1 | 4160 | - | Rel-17 | F | ING_5GS-UEConTest | revised |
| R5-241592 | Add new ING_5GS test case 11.1.11 | China Telecom | 38.523-1 | 4160 | 1 | Rel-17 | F | ING_5GS-UEConTest | withdrawn |
| R5-240095 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT | 38.523-1 | 4161 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-241635 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT | 38.523-1 | 4161 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-240100 | Add new r17 ATSSS test case 11.9.1 | China Telecom | 38.523-1 | 4162 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-241617 | Add new r17 ATSSS test case 11.9.1 | China Telecom | 38.523-1 | 4162 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-240139 | Addition of new test case 8.1.2.1.7 for UPIP | ZTE Corporation | 38.523-1 | 4163 | - | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | revised |
| R5-241513 | Addition of new test case 8.1.2.1.7 for UPIP | ZTE Corporation | 38.523-1 | 4163 | 1 | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | withdrawn |
| R5-240140 | Addition of new test case 8.2.2.4.4 for UPIP | ZTE Corporation | 38.523-1 | 4164 | - | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | revised |
| R5-241551 | Addition of new test case 8.2.2.4.4 for UPIP | ZTE Corporation | 38.523-1 | 4164 | 1 | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | withdrawn |
| R5-240151 | Correction to RedCap MAC TC 7.1.1.8.4 | MediaTek Inc. | 38.523-1 | 4165 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240152 | Correction of case title for TC 7.1.1.1.19 | MediaTek Inc. | 38.523-1 | 4166 | - | Rel-17 | F | NR_feMIMO-UEConTest | agreed |
| R5-240155 | Correction to TC 9.1.14.1 | MediaTek Inc. | 38.523-1 | 4167 | - | Rel-17 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240156 | Correction to TC 11.4.1a | MediaTek Inc. | 38.523-1 | 4168 | - | Rel-17 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | revised |
| R5-241653 | Correction to TC 11.4.1a | MediaTek Inc. | 38.523-1 | 4168 | 1 | Rel-17 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240174 | Correction to NR MAC TC 7.1.1.1.1 | MediaTek Inc. | 38.523-1 | 4169 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240175 | Correction to 2-step RACH MAC TC 7.1.1.1.8 and 7.1.1.1.10 | MediaTek Inc. | 38.523-1 | 4170 | - | Rel-17 | F | TEI16_Test, NR_2step_RACH-UEConTest | agreed |
| R5-240176 | Correction to NR MAC TC 7.1.1.5.x | MediaTek Inc. | 38.523-1 | 4171 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241501 | Correction to NR MAC TC 7.1.1.5.x | MediaTek Inc. | 38.523-1 | 4171 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240177 | Correction to NR MAC TC 7.1.1.6.1 | MediaTek Inc. | 38.523-1 | 4172 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241502 | Correction to NR MAC TC 7.1.1.6.1 | MediaTek Inc. | 38.523-1 | 4172 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240178 | Correction to NR MAC TC 7.1.1.6.2 and 7.1.1.6.3 | MediaTek Inc. | 38.523-1 | 4173 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241503 | Correction to NR MAC TC 7.1.1.6.2 and 7.1.1.6.3 | MediaTek Inc. | 38.523-1 | 4173 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240179 | Correction to NR MAC TC 7.1.1.6.4 and 7.1.1.6.5 | MediaTek Inc. | 38.523-1 | 4174 | - | Rel-17 | F | TEI16_Test, NR_IioT-UEConTest | revised |
| R5-241504 | Correction to NR MAC TC 7.1.1.6.4 and 7.1.1.6.5 | MediaTek Inc. | 38.523-1 | 4174 | 1 | Rel-17 | F | TEI16_Test, NR_IioT-UEConTest | agreed |
| R5-240180 | Correction to NR MAC TC 7.1.1.8.1 | MediaTek Inc. | 38.523-1 | 4175 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241505 | Correction to NR MAC TC 7.1.1.8.1 | MediaTek Inc. | 38.523-1 | 4175 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240181 | Correction to NR HO TC 8.1.4.2.1.1 | MediaTek Inc. | 38.523-1 | 4176 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240182 | Correction to NR HO TC 8.1.4.2.1.2 | MediaTek Inc. | 38.523-1 | 4177 | - | Rel-17 | F | TEI16_Test | revised |
| R5-241514 | Correction to NR HO TC 8.1.4.2.1.2 | MediaTek Inc. | 38.523-1 | 4177 | 1 | Rel-17 | F | TEI16_Test | agreed |
| R5-240183 | Correction to NR RRC TC 8.1.5.1.1 | MediaTek Inc. | 38.523-1 | 4178 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241518 | Correction to NR RRC TC 8.1.5.1.1 | MediaTek Inc. | 38.523-1 | 4178 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240184 | Correction to NR RRC TC 8.1.5.8.1 | MediaTek Inc. | 38.523-1 | 4179 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240185 | Correction to NR RRC TC 8.2.1.1.1 | MediaTek Inc. | 38.523-1 | 4180 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241519 | Correction to NR RRC TC 8.2.1.1.1 | MediaTek Inc. | 38.523-1 | 4180 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240186 | Correction to NR RRC TC 8.2.1.1.2 | MediaTek Inc. | 38.523-1 | 4181 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241520 | Correction to NR RRC TC 8.2.1.1.2 | MediaTek Inc. | 38.523-1 | 4181 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240187 | Correction to eNS NSAC TC 9.1.12.3 | MediaTek Inc. | 38.523-1 | 4182 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-240188 | Correction to MBS TC 14.1.1.1 | MediaTek Inc. | 38.523-1 | 4183 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240189 | Correction to MBS TC 14.1.1.2 | MediaTek Inc. | 38.523-1 | 4184 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240190 | Correction to MBS TC 14.1.1.3 | MediaTek Inc. | 38.523-1 | 4185 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-241532 | Correction to MBS TC 14.1.1.3 | MediaTek Inc. | 38.523-1 | 4185 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240191 | Correction to MBS TC 14.1.1.4.1 | MediaTek Inc. | 38.523-1 | 4186 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240192 | Correction to MBS TC 14.1.2.1 | MediaTek Inc. | 38.523-1 | 4187 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-241533 | Correction to MBS TC 14.1.2.1 | MediaTek Inc. | 38.523-1 | 4187 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240193 | Correction to MBS TC 14.2.4.2.1 | MediaTek Inc. | 38.523-1 | 4188 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240203 | Correction of UPIP test case 7.1.3.2.6 for specific message contents | ZTE Corporation | 38.523-1 | 4189 | - | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | revised |
| R5-241552 | Correction of UPIP test case 7.1.3.2.6 for specific message contents | ZTE Corporation | 38.523-1 | 4189 | 1 | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | agreed |
| R5-240205 | Correction of UPIP test case 8.2.6.4.1 | ZTE Corporation | 38.523-1 | 4190 | - | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | revised |
| R5-241553 | Correction of UPIP test case 8.2.6.4.1 | ZTE Corporation | 38.523-1 | 4190 | 1 | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | agreed |
| R5-240225 | Correction to NR testcase 6.4.2.1 | Starpoint, TDIA | 38.523-1 | 4191 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241495 | Correction to NR testcase 6.4.2.1 | Starpoint, TDIA | 38.523-1 | 4191 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240226 | Correction to NR testcase 8.1.1.4.1 | Starpoint, TDIA | 38.523-1 | 4192 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241510 | Correction to NR testcase 8.1.1.4.1 | Starpoint, TDIA | 38.523-1 | 4192 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240230 | Correction to NR testcase 11.4.12 | Starpoint, TDIA | 38.523-1 | 4193 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241528 | Correction to NR testcase 11.4.12 | Starpoint, TDIA | 38.523-1 | 4193 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240238 | Correction to RRC Inactive Mode test case 6.4.1.1 | Keysight Technologies UK | 38.523-1 | 4194 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240239 | Correction to NR testcase 8.1.5.11.2 | Starpoint, TDIA | 38.523-1 | 4195 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-240240 | Add new ATSSS test case 11.9.3 | ZTE Corporation | 38.523-1 | 4196 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-241618 | Add new ATSSS test case 11.9.3 | ZTE Corporation | 38.523-1 | 4196 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-240241 | Add new ATSSS test case 11.9.4 | ZTE Corporation | 38.523-1 | 4197 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-241619 | Add new ATSSS test case 11.9.4 | ZTE Corporation | 38.523-1 | 4197 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-240243 | Correction to NR testcase 8.2.6.3.5 | Starpoint, TDIA | 38.523-1 | 4198 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-240244 | Correction to NR testcase 8.2.6.3.6 | Starpoint, TDIA | 38.523-1 | 4199 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-240256 | Correction to NR testcase 8.1.4.3.1 | Starpoint, TDIA | 38.523-1 | 4200 | - | Rel-17 | F | TEI16_Test, NR_Mob_enh-UEConTest | revised |
| R5-241515 | Correction to NR testcase 8.1.4.3.1 | Starpoint, TDIA | 38.523-1 | 4200 | 1 | Rel-17 | F | TEI16_Test, NR_Mob_enh-UEConTest | agreed |
| R5-240282 | Add new r17 ATSSS test case 11.9.2 | China Telecom | 38.523-1 | 4201 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-241620 | Add new r17 ATSSS test case 11.9.2 | China Telecom | 38.523-1 | 4201 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-240284 | Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH | MediaTek Inc. | 38.523-1 | 4202 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240285 | Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH | MediaTek Inc. | 38.523-1 | 4203 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240286 | Update URLLC TC 7.1.1.4.1.5 to test RedCap UE | MediaTek Inc. | 38.523-1 | 4204 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240287 | Update URLLC TC 7.1.1.4.2.6 to test RedCap UE | MediaTek Inc. | 38.523-1 | 4205 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240289 | Correction to NR slice TC 6.1.2.25 | MediaTek Inc.,Anritsu | 38.523-1 | 4206 | - | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | agreed |
| R5-240322 | Correction to emergency service TC 11.4.4 | MediaTek Inc. | 38.523-1 | 4207 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240323 | Correction to eCPSOR TC 6.3.2.5 | MediaTek Inc. | 38.523-1 | 4208 | - | Rel-17 | F | TEI17_Test, eCPSOR_CON-UEConTest | withdrawn |
| R5-240351 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4209 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241582 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4209 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-240352 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4210 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241583 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4210 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-240353 | Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4211 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241584 | Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4211 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-240354 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4212 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241585 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4212 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-240355 | Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4213 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241586 | Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4213 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-240356 | Addition of new test case 9.1.11.5 for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4214 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241587 | Addition of new test case 9.1.11.5 for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4214 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-240357 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4215 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241588 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4215 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-240358 | Addition of new test case 9.1.11.7 for Rel-17 eNPN | China Telecom | 38.523-1 | 4216 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-240379 | Addition of new test case 8.2.3.18.4 inter-SN CPC for EN-DC | Huawei, HiSilicon | 38.523-1 | 4217 | - | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-241602 | Addition of new test case 8.2.3.18.4 inter-SN CPC for EN-DC | Huawei, HiSilicon | 38.523-1 | 4217 | 1 | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240380 | Addition of new test case 8.2.3.18.5 inter-SN CPC for NR-DC | Huawei, HiSilicon | 38.523-1 | 4218 | - | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-241603 | Addition of new test case 8.2.3.18.5 inter-SN CPC for NR-DC | Huawei, HiSilicon | 38.523-1 | 4218 | 1 | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240413 | Correction of MAC TC 7.1.1.6.2-UL CG type1 | Huawei, HiSilicon | 38.523-1 | 4219 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240414 | Correction of MAC TC 7.1.1.3.2b-Lcp-Restriction | Huawei, HiSilicon | 38.523-1 | 4220 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241506 | Correction of MAC TC 7.1.1.3.2b-Lcp-Restriction | Huawei, HiSilicon | 38.523-1 | 4220 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240415 | Correction of MAC TC 7.1.1.8.1-BWP switch | Huawei, HiSilicon | 38.523-1 | 4221 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241507 | Correction of MAC TC 7.1.1.8.1-BWP switch | Huawei, HiSilicon | 38.523-1 | 4221 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240416 | Editorial correction of NR TC 11.4.x-Emergency services | Huawei, HiSilicon | 38.523-1 | 4222 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240417 | Correction of SDT TC 7.1.1.13.1-2-Step RACH | Huawei, HiSilicon | 38.523-1 | 4223 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | withdrawn |
| R5-240426 | Correction the title of PEIPS TC 9.1.14.1 | Huawei, HiSilicon | 38.523-1 | 4224 | - | Rel-17 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240429 | Correction of MUSIM TC 8.1.2.1.6-MUSIM gap configuration | Huawei, HiSilicon,China Telecom | 38.523-1 | 4225 | - | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | revised |
| R5-241640 | Correction of MUSIM TC 8.1.2.1.6-MUSIM gap configuration | Huawei, HiSilicon,China Telecom | 38.523-1 | 4225 | 1 | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | agreed |
| R5-240433 | Editorial update the derivation path for the V2X message contents table | Huawei, HiSilicon, CATT | 38.523-1 | 4226 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-240435 | Correction of MBS broadcast TC 14.1.1.1-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4227 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-241534 | Correction of MBS broadcast TC 14.1.1.1-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4227 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240436 | Correction of MBS broadcast TC 14.1.1.3-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4228 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-241535 | Correction of MBS broadcast TC 14.1.1.3-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4228 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240437 | Correction of MBS broadcast TC 14.1.2.1-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4229 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-241536 | Correction of MBS broadcast TC 14.1.2.1-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4229 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240438 | Correction of MBS broadcast TC 14.1.2.2-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4230 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | withdrawn |
| R5-240439 | Correction of MBS broadcast TC 14.1.2.3-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4231 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | withdrawn |
| R5-240440 | Correction of MBS broadcast TC 14.1.3.1-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4232 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-241537 | Correction of MBS broadcast TC 14.1.3.1-Test Loop Mode C | Huawei, HiSilicon | 38.523-1 | 4232 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-240441 | Correction to redcap TC 6.1.2.27 | Huawei, HiSilicon | 38.523-1 | 4233 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240442 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon | 38.523-1 | 4234 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241521 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon | 38.523-1 | 4234 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240472 | Correction to 5GSM test case 10.3.2.1 | Keysight Technologies UK, Samsung | 38.523-1 | 4235 | - | Rel-17 | F | TEI15_Test, 5GS_Ph1-CT_n3GPPA-UEConTest | revised |
| R5-241524 | Correction to 5GSM test case 10.3.2.1 | Keysight Technologies UK, Samsung | 38.523-1 | 4235 | 1 | Rel-17 | F | TEI15_Test, 5GS_Ph1-CT_n3GPPA-UEConTest | agreed |
| R5-240473 | Correction to eDRX test case 11.7.2 | Keysight Technologies UK | 38.523-1 | 4236 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240509 | Correction to NR5GC testcase 11.4.5 | ROHDE & SCHWARZ, Hisilicon | 38.523-1 | 4237 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241529 | Correction to NR5GC testcase 11.4.5 | ROHDE & SCHWARZ, Hisilicon | 38.523-1 | 4237 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240510 | Correction to NR5GC testcase 11.3.11 | ROHDE & SCHWARZ, Qualcomm | 38.523-1 | 4238 | - | Rel-17 | F | TEI16_Test | agreed |
| R5-240511 | Correction to NR5GC eDRX testcase 11.7.3 | ROHDE & SCHWARZ | 38.523-1 | 4239 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240512 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, Hisilicon, MediaTek | 38.523-1 | 4240 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241644 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, Hisilicon, MediaTek | 38.523-1 | 4240 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241687 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, Hisilicon, MediaTek | 38.523-1 | 4240 | 2 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240529 | Updates to 5G V2X test case 12.1.7.1 | MCC TF160 | 38.523-1 | 4241 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-240532 | Corrections to SDT TC 7.1.1.13.1 | MCC TF160, Lenovo | 38.523-1 | 4242 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-241543 | Corrections to SDT TC 7.1.1.13.1 | MCC TF160, Lenovo | 38.523-1 | 4242 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-240533 | Corrections to SDT TC 7.1.1.13.2 | MCC TF160, Lenovo | 38.523-1 | 4243 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-241544 | Corrections to SDT TC 7.1.1.13.2 | MCC TF160, Lenovo | 38.523-1 | 4243 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-240534 | Corrections to SDT TC 7.1.1.13.3 | MCC TF160, Lenovo | 38.523-1 | 4244 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-241545 | Corrections to SDT TC 7.1.1.13.3 | MCC TF160, Lenovo | 38.523-1 | 4244 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-240535 | Corrections to SDT TC 7.1.1.13.4 | MCC TF160, Lenovo | 38.523-1 | 4245 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-240539 | Correction to NR test case 11.8.6 | MCC TF160 | 38.523-1 | 4246 | - | Rel-17 | F | TEI15_Test | agreed |
| R5-240540 | Correction to NR test case 6.1.2.3 | MCC TF160 | 38.523-1 | 4247 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240541 | Corrections to 5G TC 8.1.4.2.1.2 | MCC TF160 | 38.523-1 | 4248 | - | Rel-17 | F | TEI16_Test | agreed |
| R5-240542 | Updates for NR RRC test case 8.1.5.1.1 | MCC TF160 | 38.523-1 | 4249 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240543 | Updates for EN-DC RRC test case 8.2.1.1.1 | MCC TF160 | 38.523-1 | 4250 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240544 | Updates for NE-DC RRC test case 8.2.1.1.2 | MCC TF160 | 38.523-1 | 4251 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240545 | Update to NE-DC RRC test case 8.2.2.2.3 | MCC TF160 | 38.523-1 | 4252 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240588 | Modification of testcase 9.1.6.2.3 UE or NW initiated de-registration for UAS | Nokia, Nokia Shanghai Bell | 38.523-1 | 4253 | - | Rel-17 | F | ID_UAS-UEConTest | revised |
| R5-241605 | Modification of testcase 9.1.6.2.3 UE or NW initiated de-registration for UAS | Nokia, Nokia Shanghai Bell | 38.523-1 | 4253 | 1 | Rel-17 | F | ID_UAS-UEConTest | agreed |
| R5-240589 | Modification of testcase 8.1.5.13.2 for Data on non-SDT Radio Bearers for NR SmallData | Nokia, Nokia Shanghai Bell | 38.523-1 | 4254 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-241546 | Modification of testcase 8.1.5.13.2 for Data on non-SDT Radio Bearers for NR SmallData | Nokia, Nokia Shanghai Bell | 38.523-1 | 4254 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-240591 | New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource | Nokia, Nokia Shanghai Bell | 38.523-1 | 4255 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-240592 | New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session | Nokia, Nokia Shanghai Bell | 38.523-1 | 4256 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-240609 | Correction of NR TC 9.1.4.1-CUC | Huawei, HiSilicon | 38.523-1 | 4257 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241522 | Correction of NR TC 9.1.4.1-CUC | Huawei, HiSilicon | 38.523-1 | 4257 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240611 | Updates to NR CA TCs 8.1.5.x.y.z | MCC TF160 | 38.523-1 | 4258 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240613 | Updates to NR CA TCs 8.2.4.x.y.z | MCC TF160 | 38.523-1 | 4259 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240631 | Update test case 6.7.1.3 | Ericsson | 38.523-1 | 4260 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240632 | Update test case 6.7.1.3 | Ericsson | 38.523-1 | 4261 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240644 | Correction to SM NSAC Testcase 10.1.8.4 | Anritsu EMEA Ltd | 38.523-1 | 4262 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-240645 | Correction to NR RRC Idle mode test case 6.4.2.3 | Anritsu EMEA Ltd | 38.523-1 | 4263 | - | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | revised |
| R5-241496 | Correction to NR RRC Idle mode test case 6.4.2.3 | Anritsu EMEA Ltd | 38.523-1 | 4263 | 1 | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | agreed |
| R5-240646 | Correction to NR RRC Idle mode test case 6.1.2.24 | Anritsu EMEA Ltd | 38.523-1 | 4264 | - | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | revised |
| R5-241497 | Correction to NR RRC Idle mode test case 6.1.2.24 | Anritsu EMEA Ltd | 38.523-1 | 4264 | 1 | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | agreed |
| R5-240650 | Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | 38.523-1 | 4265 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241566 | Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | 38.523-1 | 4265 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240651 | Updates to cell configuration for NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" test case | QUALCOMM JAPAN LLC. | 38.523-1 | 4266 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241567 | Updates to cell configuration for NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" test case | QUALCOMM JAPAN LLC. | 38.523-1 | 4266 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240652 | Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | 38.523-1 | 4267 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240653 | Addition of NTN / Mobility registration update / supported TACs not part of UE registration area | QUALCOMM JAPAN LLC. | 38.523-1 | 4268 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241568 | Addition of NTN / Mobility registration update / supported TACs not part of UE registration area | QUALCOMM JAPAN LLC. | 38.523-1 | 4268 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240656 | Correction and updates to NR TC 7.1.1.10.2 | Qualcomm CDMA Technologies | 38.523-1 | 4269 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241508 | Correction and updates to NR TC 7.1.1.10.2 | Qualcomm CDMA Technologies | 38.523-1 | 4269 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240657 | Corrections to NR eDRX test case 11.7.1 | Qualcomm CDMA Technologies, ANRITSU LTD, Keysight Technologies | 38.523-1 | 4270 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240658 | Corrections to UAS test case 10.1.4.3 | Qualcomm CDMA Technologies | 38.523-1 | 4271 | - | Rel-17 | F | ID_UAS-UEConTest | revised |
| R5-241606 | Corrections to UAS test case 10.1.4.3 | Qualcomm CDMA Technologies | 38.523-1 | 4271 | 1 | Rel-17 | F | ID_UAS-UEConTest | agreed |
| R5-240659 | Introduction of new NR NTN RRC TC for Event D1 | Qualcomm CDMA Technologies, | 38.523-1 | 4272 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241569 | Introduction of new NR NTN RRC TC for Event D1 | Qualcomm CDMA Technologies, | 38.523-1 | 4272 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240765 | Correction to V2X SIG test case 12.1.7.1 | Huawei,HiSilicon | 38.523-1 | 4273 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | withdrawn |
| R5-240766 | Addition of V2X SIG test case 12.2.9.1 | Huawei, HiSilicon | 38.523-1 | 4274 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-240867 | Correction to NR TC 8.1.6.1.3.8 | Huawei, Hisilicon | 38.523-1 | 4275 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-241641 | Correction to NR TC 8.1.6.1.3.8 | Huawei, Hisilicon | 38.523-1 | 4275 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-240868 | Correction to NR TC 8.1.6.1.3.9 | Huawei, Hisilicon | 38.523-1 | 4276 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-241642 | Correction to NR TC 8.1.6.1.3.9 | Huawei, Hisilicon | 38.523-1 | 4276 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-240918 | Resubmission of Editorial update test case 8.1.1.1a.3 | ETSI MCC (Ericsson) | 38.523-1 | 4277 | - | Rel-17 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240950 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD | 38.523-1 | 4278 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241526 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD | 38.523-1 | 4278 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241688 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD, Rohde & Schwarz | 38.523-1 | 4278 | 2 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241691 | Correction to NR 5GC Multilayer Emergency EPS FB TC 11.1.7 | Qualcomm CDMA Technologies, ANRITSU LTD, Rohde & Schwarz | 38.523-1 | 4278 | 3 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240951 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | 38.523-1 | 4279 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241530 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | 38.523-1 | 4279 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241689 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | 38.523-1 | 4279 | 2 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241692 | Correction to NR 5GC Multilayer Emergency TC 11.4.12 | Qualcomm CDMA Technologies, ANRITSU LTD | 38.523-1 | 4279 | 3 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240985 | Addition of new NR NTN MAC TA reporting test case | Qualcomm Incorporated | 38.523-1 | 4280 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240986 | Addition of new NR NTN RLC t-reassembly timer test case | Qualcomm Incorporated | 38.523-1 | 4281 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241570 | Addition of new NR NTN RLC t-reassembly timer test case | Qualcomm Incorporated | 38.523-1 | 4281 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240987 | Addition of NR NTN PDCP discard timer test case | Qualcomm Incorporated | 38.523-1 | 4282 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241571 | Addition of NR NTN PDCP discard timer test case | Qualcomm Incorporated | 38.523-1 | 4282 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240990 | Addition of NR shared spectrum measurements test case 8.1.8.1.3 | Qualcomm Incorporated | 38.523-1 | 4283 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-241035 | Updates to NR shared spectrum MAC test case 7.1.1.10.3 | Qualcomm Incorporated | 38.523-1 | 4284 | - | Rel-17 | F | NR_unlic-UEConTest | revised |
| R5-241542 | Updates to NR shared spectrum MAC test case 7.1.1.10.3 | Qualcomm Incorporated | 38.523-1 | 4284 | 1 | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-241036 | Updates to NR shared spectrum idle mode test cases | Qualcomm Incorporated | 38.523-1 | 4285 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-241037 | Corrections to eMIMO TC 7.1.1.3.10 | Lenovo | 38.523-1 | 4286 | - | Rel-17 | F | TEI16_Test, NR_eMIMO-UEConTest | revised |
| R5-241509 | Corrections to eMIMO TC 7.1.1.3.10 | Lenovo | 38.523-1 | 4286 | 1 | Rel-17 | F | TEI16_Test, NR_eMIMO-UEConTest | agreed |
| R5-241038 | Updates to NR shared spectrum page monitoring test cases | Qualcomm Incorporated | 38.523-1 | 4287 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-241039 | Updates to NR shared spectrum RRC test cases | Qualcomm Incorporated | 38.523-1 | 4288 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-241044 | Corrections to CovEnh TC 7.1.1.1.18 | Lenovo, MCC TF160 | 38.523-1 | 4289 | - | Rel-17 | F | NR_cov_enh-UEConTest | agreed |
| R5-241045 | Correction to IRAT MDT test case 8.1.6.2.2 | Qualcomm Incorporated | 38.523-1 | 4290 | - | Rel-17 | F | TEI16_Test, NR_SON_MDT-UEConTest | agreed |
| R5-241070 | Correction of test case 6.1.2.13 | NTTDOCOMO, INC. | 38.523-1 | 4291 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241498 | Correction of test case 6.1.2.13 | NTTDOCOMO, INC. | 38.523-1 | 4291 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241071 | Correction of test case 6.1.2.14 | NTTDOCOMO, INC. | 38.523-1 | 4292 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241499 | Correction of test case 6.1.2.14 | NTTDOCOMO, INC. | 38.523-1 | 4292 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241073 | Correction to IRAT PLMN selection test case 6.2.1.5 | Qualcomm Incorporated | 38.523-1 | 4293 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241500 | Correction to IRAT PLMN selection test case 6.2.1.5 | Qualcomm Incorporated | 38.523-1 | 4293 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241074 | Correction of test case 8.1.1.2.4 | NTTDOCOMO, INC., TF160 | 38.523-1 | 4294 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241511 | Correction of test case 8.1.1.2.4 | NTTDOCOMO, INC., TF160 | 38.523-1 | 4294 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241075 | Correction of test case 8.1.4.2.1.1 | NTTDOCOMO, INC. | 38.523-1 | 4295 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241516 | Correction of test case 8.1.4.2.1.1 | NTTDOCOMO, INC. | 38.523-1 | 4295 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241158 | Correction to RACS Test case 9.1.9.7 | Anritsu EMEA Ltd | 38.523-1 | 4296 | - | Rel-17 | F | TEI16_Test, RACS-UEConTest | agreed |
| R5-241161 | Correction to NSAC Test case 10.1.8.3 | Anritsu EMEA Ltd | 38.523-1 | 4297 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | revised |
| R5-241523 | Correction to NSAC Test case 10.1.8.3 | Anritsu EMEA Ltd | 38.523-1 | 4297 | 1 | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-241163 | Correction to NR Slice test case 8.1.1.3.9 | Anritsu EMEA Ltd | 38.523-1 | 4298 | - | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | revised |
| R5-241512 | Correction to NR Slice test case 8.1.1.3.9 | Anritsu EMEA Ltd | 38.523-1 | 4298 | 1 | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | agreed |
| R5-241164 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | 38.523-1 | 4299 | - | Rel-17 | F | TEI16_Test, RACS-UEConTest | revised |
| R5-241643 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | 38.523-1 | 4299 | 1 | Rel-17 | F | TEI16_Test, RACS-UEConTest | agreed |
| R5-241166 | Correction to RRC Inactive Mode test case 6.4.1.1 | Keysight Technologies UK Ltd | 38.523-1 | 4300 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241183 | Correction to UAS TC 9.1.5.2.11 | Ericsson | 38.523-1 | 4301 | - | Rel-17 | F | ID_UAS-UEConTest | revised |
| R5-241607 | Correction to UAS TC 9.1.5.2.11 | Ericsson | 38.523-1 | 4301 | 1 | Rel-17 | F | ID_UAS-UEConTest | agreed |
| R5-241184 | New Emergency test case 11.4.15 | Ericsson | 38.523-1 | 4302 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241590 | New Emergency test case 11.4.15 | Ericsson | 38.523-1 | 4302 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-241186 | Correction to UPIP TC 7.1.3.2.6 | Ericsson | 38.523-1 | 4303 | - | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | agreed |
| R5-241308 | Correction to 5G NR TC 11.4.13 | MCC TF160 | 38.523-1 | 4304 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241531 | Correction to 5G NR TC 11.4.13 | MCC TF160 | 38.523-1 | 4304 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241309 | Correction to FR2 measurement threshold for NR RRC test cases | Anritsu EMEA Ltd | 38.523-1 | 4305 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241366 | Update of GERAN Signal levels to eCall signalling tests | Anritsu EMEA Ltd | 38.523-1 | 4306 | - | Rel-17 | F | TEI16_Test, NR_EIEI-UEConTest | withdrawn |
| R5-241377 | Correction to eDRX test case 11.7.3 | Keysight Technologies UK | 38.523-1 | 4307 | - | Rel-17 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241378 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | 38.523-1 | 4308 | - | Rel-17 | F | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest | revised |
| R5-241637 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | 38.523-1 | 4308 | 1 | Rel-17 | F | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest | agreed |
| R5-241638 | Add new ING_5GS test case 9.3.1.6 | China Telecom | 38.523-1 | 4309 | - | Rel-17 | F | ING_5GS-UEConTest | agreed |
| R5-240041 | Add applicability for ING_5GS test case 11.1.11 | China Telecom | 38.523-2 | 0428 | - | Rel-17 | F | ING_5GS-UEConTest | withdrawn |
| R5-240141 | Addition of applicability for new UPIP TC 8.1.2.1.7 and 8.2.2.4.4 | ZTE Corporation | 38.523-2 | 0429 | - | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | revised |
| R5-241575 | Addition of applicability for new UPIP TC 8.1.2.1.7 and 8.2.2.4.4 | ZTE Corporation | 38.523-2 | 0429 | 1 | Rel-17 | F | UPIP_SEC_LTE-RAN-UEConTest | withdrawn |
| R5-240220 | Add applicability for Rel-17 ATSSS test case 11.9.1 | China Telecom | 38.523-2 | 0430 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-240235 | Addition of applicability of new ING_5GS test case 11.1.11 | China Telecom | 38.523-2 | 0431 | - | Rel-17 | F | ING_5GS-UEConTest | withdrawn |
| R5-240242 | Add applicabilities for new ATSSS test cases 11.9.3 and 11.9.4 | ZTE Corporation | 38.523-2 | 0432 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-240359 | Addition of applicability for eNPN test cases | China Telecom | 38.523-2 | 0433 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241646 | Addition of applicability for eNPN test cases | China Telecom | 38.523-2 | 0433 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-240360 | Scoping NR SA applicable TCs for SNPN-only UEs | China Telecom | 38.523-2 | 0434 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-241589 | Scoping NR SA applicable TCs for SNPN-only UEs | China Telecom | 38.523-2 | 0434 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-240386 | Addition of applicability for inter-SN conditional PSCell change | Huawei, HiSilicon | 38.523-2 | 0435 | - | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-241604 | Addition of applicability for inter-SN conditional PSCell change | Huawei, HiSilicon | 38.523-2 | 0435 | 1 | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240427 | Update the applicability of PEIPS TC 9.1.14.1 and 11.4.1a | Huawei, HiSilicon | 38.523-2 | 0436 | - | Rel-17 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | revised |
| R5-241547 | Update the applicability of PEIPS TC 9.1.14.1 and 11.4.1a | Huawei, HiSilicon | 38.523-2 | 0436 | 1 | Rel-17 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240546 | Misc. updates to TS 38.523-2 | MCC TF160 | 38.523-2 | 0437 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240628 | Modification of testcase 8.1.5.13.2 applicability clauses | Nokia, Nokia Shanghai Bell | 38.523-2 | 0438 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-240629 | Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2 | Nokia, Nokia Shanghai Bell | 38.523-2 | 0439 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-240654 | Applicability updates for NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer test case | QUALCOMM JAPAN LLC. | 38.523-2 | 0440 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240655 | Applicability updates for NTN / Mobility registration update / supported TACs not part of UE registration area | QUALCOMM JAPAN LLC. | 38.523-2 | 0441 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240660 | Applicability of New NR NTN TC for Event D1 | Qualcomm CDMA Technologies | 38.523-2 | 0442 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241652 | Applicability of New NR NTN TC for Event D1 | Qualcomm CDMA Technologies | 38.523-2 | 0442 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240662 | Corrections to applicability of Network Slice Admission Control (NSAC) mobility management aspects test cases | Qualcomm CDMA Technologies | 38.523-2 | 0443 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | withdrawn |
| R5-240767 | Correction of applicability for V2X SIG test cases | Huawei, HiSilicon | 38.523-2 | 0444 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-241540 | Correction of applicability for V2X SIG test cases | Huawei, HiSilicon | 38.523-2 | 0444 | 1 | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-240988 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | 38.523-2 | 0445 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241650 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | 38.523-2 | 0445 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-240991 | Applicability updates to NR shared spectrum test cases | Qualcomm Incorporated | 38.523-2 | 0446 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-241159 | Correction to applicability of EN-DC CA test cases | Qualcomm Incorporated | 38.523-2 | 0447 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241167 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | 38.523-2 | 0448 | - | Rel-17 | F | TEI16_Test, RACS-UEConTest | revised |
| R5-241645 | Correction to title of 8.1.5.9.1 | Anritsu EMEA Ltd | 38.523-2 | 0448 | 1 | Rel-17 | F | TEI16_Test, RACS-UEConTest | agreed |
| R5-241185 | Applicability for new test case 11.4.15 | Ericsson | 38.523-2 | 0449 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-241414 | Applicability updates for new NTN Idle mode and NAS test cases | QUALCOMM JAPAN LLC. | 38.523-2 | 0450 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-241651 | Applicability updates for new NTN Idle mode and NAS test cases | QUALCOMM JAPAN LLC. | 38.523-2 | 0450 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241451 | Correction of applicability for partial sounding test case | Huawei, HiSilicon, TF160 | 38.523-2 | 0451 | - | Rel-17 | F | NR_feMIMO-UEConTest | agreed |
| R5-241453 | Add applicability for Rel-17 ATSSS test cases | China Telecom, ZTE | 38.523-2 | 0452 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-241621 | Add applicability for Rel-17 ATSSS test cases | China Telecom, ZTE | 38.523-2 | 0452 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-241639 | Addition of applicability of new ING_5GS test case 9.3.1.6 | China Telecom | 38.523-2 | 0453 | - | Rel-17 | F | ING_5GS-UEConTest | revised |
| R5-241654 | Addition of applicability of new ING_5GS test case 9.3.1.6 | China Telecom | 38.523-2 | 0453 | 1 | Rel-17 | F | ING_5GS-UEConTest | agreed |
| R5-240428 | Add PIXIT for MUSIM gap | Huawei, HiSilicon,China Telecom | 38.523-3 | 3373 | - | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | withdrawn |
| R5-240530 | 5G V2X: Test Model updates | MCC TF160 | 38.523-3 | 3375 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-241541 | 5G V2X: Test Model updates | MCC TF160 | 38.523-3 | 3375 | 1 | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-240531 | NR-U: Introduction of Test Model aspects | MCC TF160 | 38.523-3 | 3376 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-240547 | Routine maintenance for TS 38.523-3 | MCC TF160 | 38.523-3 | 3377 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240026 | Editorial corrections for 6.7.3.1 and 6.7.3.2.2 | MediaTek Beijing Inc. | 38.533 | 2875 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240147 | Update of MG enhancements TC 6.6.18.3 | MediaTek Inc. | 38.533 | 2876 | - | Rel-18 | F | NR_MG_enh-UEConTest | withdrawn |
| R5-240148 | Update of MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.533 | 2877 | - | Rel-18 | F | NR_MG_enh-UEConTest | withdrawn |
| R5-240202 | Addition and correction to the NTN related abbreviations in 38.533 | MediaTek (Hefei) Inc. | 38.533 | 2878 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242032 | Addition and correction to the NTN related abbreviations in 38.533 | MediaTek (Hefei) Inc. | 38.533 | 2878 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240204 | Correction of Redcap inter-RAT E-UTRAN CGI test cases | MediaTek Inc. | 38.533 | 2879 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242037 | Correction of Redcap inter-RAT E-UTRAN CGI test cases | MediaTek Inc. | 38.533 | 2879 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240213 | Editorial correction to HST TCs on test applicability description | MediaTek Beijing Inc. | 38.533 | 2880 | - | Rel-18 | F | TEI17_Test | withdrawn |
| R5-240222 | Correction to Table H.3.1-8A | MediaTek Beijing Inc. | 38.533 | 2881 | - | Rel-18 | F | TEI17_Test, NR_feMIMO-UEConTest | revised |
| R5-241838 | Correction to Table H.3.1-8A | MediaTek Beijing Inc. | 38.533 | 2881 | 1 | Rel-18 | F | TEI17_Test, NR_feMIMO-UEConTest | agreed |
| R5-240223 | Correction to Table H.3.1-12 | MediaTek Beijing Inc. | 38.533 | 2882 | - | Rel-18 | F | TEI16_Test, NR_feMIMO-UEConTest | revised |
| R5-241839 | Correction to Table H.3.1-12 | MediaTek Beijing Inc. | 38.533 | 2882 | 1 | Rel-18 | F | TEI16_Test, NR_feMIMO-UEConTest | agreed |
| R5-240224 | Correction to Table H.3.1-12A | MediaTek Beijing Inc. | 38.533 | 2883 | - | Rel-18 | F | TEI17_Test, NR_feMIMO-UEConTest | revised |
| R5-241840 | Correction to Table H.3.1-12A | MediaTek Beijing Inc. | 38.533 | 2883 | 1 | Rel-18 | F | TEI17_Test, NR_feMIMO-UEConTest | agreed |
| R5-240369 | Addition of NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.1 | China Telecom, Huawei, HiSilicon | 38.533 | 2884 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-240370 | Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1 | China Telecom, Huawei, HiSilicon | 38.533 | 2885 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241896 | Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1 | China Telecom, Huawei, HiSilicon | 38.533 | 2885 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-240371 | Cell configuration mapping for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1 | China Telecom, Huawei, HiSilicon | 38.533 | 2886 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | revised |
| R5-241832 | Cell configuration mapping for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1 | China Telecom, Huawei, HiSilicon | 38.533 | 2886 | 1 | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-240372 | Test tolerances and measurement uncertainty for DL interruptions test cases 6.5.7A.1, 6.5.7B.1 and 6.5.7C.1 | China Telecom, Huawei, HiSilicon | 38.533 | 2887 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-240381 | Update of test procedure in 4.5.10.1 PSCell activation for EN-DC | Huawei, HiSilicon | 38.533 | 2888 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240382 | Correction to 4.5.11.1 EN-DC FR1 Conditional PSCell Addition | Huawei, HiSilicon | 38.533 | 2889 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240383 | Update of measurement configuration for 5.5.13.1 | Huawei, HiSilicon | 38.533 | 2890 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240384 | Update of measurement configuration for 7.5.12.1 | Huawei, HiSilicon | 38.533 | 2891 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240385 | Update of message contents for 4.5.3.7 and 6.5.3.11 | Huawei, HiSilicon | 38.533 | 2892 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-240449 | Resubmission of Annex B Corrections to BWP RMCs and configurations for RedCap RRM TCs | ETSI MCC ( Huawei, HiSilicon, Starpoint) | 38.533 | 2893 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240518 | Update to NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case | Nokia, Nokia Shanghai Bell | 38.533 | 2894 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240519 | Update to NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case | Nokia, Nokia Shanghai Bell | 38.533 | 2895 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240521 | Removal of NOTE 1 from applicability of FR1 and FR2 MRDC test cases | Nokia, Nokia Shanghai Bell | 38.533 | 2896 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | withdrawn |
| R5-240602 | Introducing the Re-Positioning Concept for FR2 RRM TCs | Keysight Technologies UK Ltd, Rohde&Schwarz | 38.533 | 2897 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241862 | Introducing the Re-Positioning Concept for FR2 RRM TCs | Keysight Technologies UK Ltd, Rohde&Schwarz | 38.533 | 2897 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240634 | Correction to FR2 SA event triggered reporting tests with Pre-MG including TT | Anritsu | 38.533 | 2898 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-241677 | Correction to FR2 SA event triggered reporting tests with Pre-MG including TT | Anritsu | 38.533 | 2898 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-240636 | Correction to FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | 38.533 | 2899 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-241678 | Correction to FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | 38.533 | 2899 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-240638 | Correction to FR2 SA event triggered reporting tests with NCSG including TT | Anritsu | 38.533 | 2900 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-241679 | Correction to FR2 SA event triggered reporting tests with NCSG including TT | Anritsu | 38.533 | 2900 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-240681 | Addition of RedCap RRM test case 17.5.3.1.1 | Huawei, HiSilicon | 38.533 | 2901 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240682 | Addition of RedCap RRM test case 17.5.3.2.1 | Huawei, HiSilicon | 38.533 | 2902 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240683 | Addition of RedCap RRM test case 17.5.5.1.1 | Huawei,HiSilicon | 38.533 | 2903 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240684 | Addition of RedCap RRM test case 17.5.5.2.1 | Huawei,HiSilicon | 38.533 | 2904 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240685 | Addition of RedCap RRM test case 17.5.6.1 | Huawei, HiSilicon | 38.533 | 2905 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240686 | Addition of RedCap RRM test case 17.7.1.1 | Huawei, HiSilicon | 38.533 | 2906 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240687 | Addition of RedCap RRM test case 17.7.1.2 | Huawei, HiSilicon | 38.533 | 2907 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240688 | Addition of RedCap RRM test case 17.7.2.1 | Huawei, HiSilicon | 38.533 | 2908 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240689 | Addition of RedCap RRM test case 17.7.2.2 | Huawei, HiSilicon | 38.533 | 2909 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240690 | Addition of RedCap RRM test case 17.7.3.1 | Huawei, HiSilicon | 38.533 | 2910 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240691 | Addition of RedCap RRM test case 17.7.3.2 | Huawei, HiSilicon | 38.533 | 2911 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240692 | Addition of RedCap RRM test case 17.7.4.1 | Huawei, HiSilicon | 38.533 | 2912 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240693 | Correction to NCD-SSB and SMTC RMCs | Huawei, HiSilicon, Starpoint | 38.533 | 2913 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242039 | Correction to NCD-SSB and SMTC RMCs | Huawei, HiSilicon, Starpoint | 38.533 | 2913 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240694 | Correction to RedCap inter-frequency measurement test cases | Huawei, HiSilicon, Starpoint, Anritsu, Keysight | 38.533 | 2914 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242040 | Correction to RedCap inter-frequency measurement test cases | Huawei, HiSilicon, Starpoint, Anritsu, Keysight | 38.533 | 2914 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240695 | Correction to RedCap RRM test case 16.5.2.3 | Huawei,HiSilicon,Starpoint | 38.533 | 2915 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240696 | Correction to RedCap RRM test case 16.5.4.1 and 16.5.4.2 | Huawei,HiSilicon,Starpoint | 38.533 | 2916 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240697 | Correction to RedCap RRM test case 17.4.1.1 | Huawei, HiSilicon | 38.533 | 2917 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240698 | Correction to RedCap RRM test case 17.6.4.1 | Huawei,HiSilicon | 38.533 | 2918 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240699 | Correction to SSB time offset in NCD-SSB test cases | Huawei, HiSilicon, Starpoint | 38.533 | 2919 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240700 | Correction to RRM enh test case 6.5.8.1 | Huawei, HiSilicon | 38.533 | 2920 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-240701 | Correction to RRM enh test case 6.6.2.9 | Huawei,HiSilicon | 38.533 | 2921 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-241829 | Correction to RRM enh test case 6.6.2.9 | Huawei,HiSilicon | 38.533 | 2921 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-240702 | Correction to EN-DC and NR SA inter-frequency measurement test cases | Huawei,HiSilicon,Starpoint | 38.533 | 2922 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240703 | Correction to EN-DC and NR SA SCell activation test cases | Huawei,HiSilicon,Starpoint | 38.533 | 2923 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241994 | Correction to EN-DC and NR SA SCell activation test cases | Huawei,HiSilicon,Starpoint | 38.533 | 2923 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240704 | Correction to PCI updating formulas in RRM test cases | Huawei, HiSilicon, Starpoint | 38.533 | 2924 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241995 | Correction to PCI updating formulas in RRM test cases | Huawei, HiSilicon, Starpoint | 38.533 | 2924 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240705 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | 38.533 | 2925 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241670 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | 38.533 | 2925 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240706 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | 38.533 | 2926 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241671 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | 38.533 | 2926 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240707 | Correction to RedCap RRM test cases 16.3.2.2.X with TT | Huawei, HiSilicon | 38.533 | 2927 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240708 | Correction to RedCap RRM test case 16.6.5.3 with TT | Huawei, HiSilicon | 38.533 | 2928 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241886 | Correction to RedCap RRM test case 16.6.5.3 with TT | Huawei, HiSilicon | 38.533 | 2928 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240709 | Correction to RedCap RRM test case 16.6.5.4 with TT | Huawei,HiSilicon | 38.533 | 2929 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240710 | Correction to RedCap RRM test case 16.7.2.3.1 with TT | Huawei, HiSilicon | 38.533 | 2930 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240711 | Correction to RedCap RRM test case 16.7.2.3.2 with TT | Huawei,HiSilicon | 38.533 | 2931 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242041 | Correction to RedCap RRM test case 16.7.2.3.2 with TT | Huawei,HiSilicon | 38.533 | 2931 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240712 | Correction to RedCap RRM test case 16.7.2.4.1 with TT | Huawei, HiSilicon | 38.533 | 2932 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242042 | Correction to RedCap RRM test case 16.7.2.4.1 with TT | Huawei, HiSilicon | 38.533 | 2932 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240713 | Correction to RedCap RRM test case 16.7.2.4.2 with TT | Huawei,HiSilicon | 38.533 | 2933 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242043 | Correction to RedCap RRM test case 16.7.2.4.2 with TT | Huawei,HiSilicon | 38.533 | 2933 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240714 | Correction to RedCap RRM test case 16.7.3.1 with TT | Huawei,HiSilicon | 38.533 | 2934 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240715 | Correction to RedCap RRM test case 16.7.3.2 with TT | Huawei, HiSilicon | 38.533 | 2935 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240716 | Correction to RedCap RRM test case 16.7.3.3.1 with TT | Huawei, HiSilicon | 38.533 | 2936 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240717 | Correction to RedCap RRM test case 16.7.3.3.2 with TT | Huawei,HiSilicon | 38.533 | 2937 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240718 | Correction to RedCap RRM test case 16.7.3.4.1 with TT | Huawei, HiSilicon | 38.533 | 2938 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240719 | Correction to RedCap RRM test case 16.7.3.4.2 with TT | Huawei,HiSilicon | 38.533 | 2939 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241963 | Correction to RedCap RRM test case 16.7.3.4.2 with TT | Huawei,HiSilicon | 38.533 | 2939 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240720 | Correction to RedCap RRM test case 16.7.4.1.1 with TT | Huawei,HiSilicon | 38.533 | 2940 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240721 | Correction to RedCap RRM test case 16.7.4.1.2 with TT | Huawei,HiSilicon | 38.533 | 2941 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240722 | Correction to RedCap RRM test case 16.7.4.2.1 with TT | Huawei,HiSilicon | 38.533 | 2942 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240723 | Correction to RedCap RRM test case 16.7.4.2.2 with TT | Huawei,HiSilicon | 38.533 | 2943 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240724 | Correction to RedCap RRM test case 16.7.4.3.1 with TT | Huawei,HiSilicon | 38.533 | 2944 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240725 | Correction to RedCap RRM test case 16.7.4.3.2 with TT | Huawei,HiSilicon | 38.533 | 2945 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240726 | Correction to RedCap RRM test case 16.7.4.4.1 with TT | Huawei,HiSilicon | 38.533 | 2946 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240727 | Correction to RedCap RRM test case 16.7.4.4.2 with TT | Huawei,HiSilicon | 38.533 | 2947 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240728 | Correction to RedCap RRM test case 16.7.5.1 with TT | Huawei, HiSilicon | 38.533 | 2948 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240729 | Correction to RedCap RRM test case 16.7.5.2 with TT | Huawei,HiSilicon | 38.533 | 2949 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240730 | Correction to RedCap RRM test case 16.7.6.1 with TT | Huawei,HiSilicon | 38.533 | 2950 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240731 | Correction to RedCap RRM test case 16.7.6.2 with TT | Huawei,HiSilicon | 38.533 | 2951 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240732 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | 38.533 | 2952 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241878 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | 38.533 | 2952 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241672 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | 38.533 | 2952 | 2 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240756 | Correction to SDT RRM test case 6.2.1 with TT | Huawei,HiSilicon | 38.533 | 2953 | - | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-241673 | Correction to SDT RRM test case 6.2.1 with TT | Huawei,HiSilicon | 38.533 | 2953 | 1 | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-240757 | Correction to Annex F for SDT RRM test cases | Huawei,HiSilicon | 38.533 | 2954 | - | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | withdrawn |
| R5-240758 | Correction to FR1 4-step RACH test cases with TT | Huawei, HiSilicon | 38.533 | 2955 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240759 | Correction to FR1 2-step RACH test cases with TT | Huawei,HiSilicon | 38.533 | 2956 | - | Rel-18 | F | TEI16_Test, NR_2step_RACH-UEConTest | agreed |
| R5-240760 | Correction to FR1 inter frequency SS SINR relative accuracy test cases with TT | Huawei, HiSilicon | 38.533 | 2957 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240761 | Correction to FR1 L1 RSRP absolute accuracy test cases with TT | Huawei, HiSilicon | 38.533 | 2958 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240762 | Correction to FR1 LTE RSRP accuracy test case 6.7.5.1 with TT | Huawei, HiSilicon | 38.533 | 2959 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240763 | Correction to Annex F for R15 RRM test cases | Huawei,HiSilicon | 38.533 | 2960 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-240790 | Addition of Test Tolerance into Annex F for RedCap test cases | Ericsson | 38.533 | 2961 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240793 | Correction of Test tolerance for FR2 Inter frequency test case | Ericsson | 38.533 | 2962 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240794 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE test case 16.6.3.1 including Test Tolerance | Ericsson | 38.533 | 2963 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242033 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE test case 16.6.3.1 including Test Tolerance | Ericsson | 38.533 | 2963 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240795 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE test case 16.6.3.2 including Test Tolerance | Ericsson | 38.533 | 2964 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242034 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE test case 16.6.3.2 including Test Tolerance | Ericsson | 38.533 | 2964 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240796 | Addition of NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE test case 16.6.3.3 including Test Tolerance | Ericsson | 38.533 | 2965 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242035 | Addition of NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE test case 16.6.3.3 including Test Tolerance | Ericsson | 38.533 | 2965 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240797 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2Rx UE test case 16.6.3.4 including Test Tolerance | Ericsson | 38.533 | 2966 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242036 | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2Rx UE test case 16.6.3.4 including Test Tolerance | Ericsson | 38.533 | 2966 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240798 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.1 including Test Tolerance | Ericsson | 38.533 | 2967 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240799 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.2 including Test Tolerance | Ericsson | 38.533 | 2968 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240800 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.3 including Test Tolerance | Ericsson | 38.533 | 2969 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240801 | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.4 including Test Tolerance | Ericsson | 38.533 | 2970 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240802 | Addition of cell configuration mapping for event triggered measurement test cases for RedCap | Ericsson | 38.533 | 2971 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240803 | Addition of NR_U test case 11.5.1.1 | Ericsson | 38.533 | 2972 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240804 | Addition of NR_U test case 11.5.1.2 | Ericsson | 38.533 | 2973 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240805 | Addition of NR_U test case 11.5.1.3 | Ericsson | 38.533 | 2974 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240806 | Addition of NR_U test case 11.5.1.4 | Ericsson | 38.533 | 2975 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240807 | Addition of NR_U test case 11.5.1.9 | Ericsson | 38.533 | 2976 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240808 | Addition of NR_U test case 11.5.2.3 | Ericsson | 38.533 | 2977 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241827 | Addition of NR_U test case 11.5.2.3 | Ericsson | 38.533 | 2977 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240809 | Addition of NR_U test case 11.5.2.4 | Ericsson | 38.533 | 2978 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240811 | Addition of NR_U test case 11.5.2.5 | Ericsson | 38.533 | 2979 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240812 | Addition of NR_U test case 11.5.2.6 | Ericsson | 38.533 | 2980 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240813 | Addition of NR_U test case 11.5.2.7 | Ericsson | 38.533 | 2981 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240814 | Addition of NR_U test case 11.5.2.8 | Ericsson | 38.533 | 2982 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240815 | Addition of NR_U test case 11.5.2.9 | Ericsson | 38.533 | 2983 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240816 | Addition of NR_U test case 11.5.2.10 | Ericsson | 38.533 | 2984 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240890 | Removal of Editor Note affecting to TC 5.5.5.9 | Keysight Technologies | 38.533 | 2985 | - | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | revised |
| R5-241826 | Removal of Editor Note affecting to TC 5.5.5.9 | Keysight Technologies | 38.533 | 2985 | 1 | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-240915 | Typo correction to SSB and SMTC configuration for RedCap Event Trigger TC 16.6.2.1 | Keysight Technologies UK Ltd | 38.533 | 2986 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240920 | Cell Mapping correction for TC 6.5.6.1.1 in Annex E | Keysight Technologies UK Ltd | 38.533 | 2987 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240921 | Cell Mapping correction for TC 6.6.1.8 in Annex E | Keysight Technologies | 38.533 | 2988 | - | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | agreed |
| R5-240968 | Correct Table 4.3.2.2.1.4.1-2 & Table 4.3.2.2.2.4.1-2 & Table 4.3.2.2.3.4.1-2 & Table 4.3.2.2.4.4.1-2 of test frequency | SGS Wireless | 38.533 | 2989 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241833 | Correct Table 4.3.2.2.1.4.1-2 & Table 4.3.2.2.2.4.1-2 & Table 4.3.2.2.3.4.1-2 & Table 4.3.2.2.4.4.1-2 of test frequency | SGS Wireless | 38.533 | 2989 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241114 | Correction to OCNG references in 16.1.1 and 16.1.2 | Anritsu | 38.533 | 2990 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241115 | Correction to title of RedCap test cases | Anritsu | 38.533 | 2991 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241116 | Correction to PRACH config in 16.5.2.4 | Anritsu | 38.533 | 2992 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241117 | Correction to 16.7.2.4.x and 16.7.3.4.x | Anritsu | 38.533 | 2993 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-241118 | Addition of message contents and cell mapping for 16.7.1.2.1 and 16.7.1.2.2 | Anritsu | 38.533 | 2994 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-241119 | Correction to test applicability of 4.6.1.7 | Anritsu, MediaTek Beijing Inc. | 38.533 | 2995 | - | Rel-18 | F | TEI16_Test, NR_HST-UEConTest | revised |
| R5-241834 | Correction to test applicability of 4.6.1.7 | Anritsu, MediaTek Beijing Inc. | 38.533 | 2995 | 1 | Rel-18 | F | TEI16_Test, NR_HST-UEConTest | agreed |
| R5-241120 | Correction to QuantityConfigu-DEFAULT in H.3.1 | Anritsu | 38.533 | 2996 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241123 | Correction to PDCCH level in 6.5.1.9 | Anritsu | 38.533 | 2997 | - | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | revised |
| R5-241967 | Correction to PDCCH level in 6.5.1.9 | Anritsu | 38.533 | 2997 | 1 | Rel-18 | F | NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-241124 | Correction to test configuration in 16.4.1.2 and 16.5.2.2 | Anritsu | 38.533 | 2998 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241964 | Correction to test configuration in 16.4.1.2 and 16.5.2.2 | Anritsu | 38.533 | 2998 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241125 | Correction to PDCCH level in CSI-RS based RLM test cases for RedCap | Anritsu | 38.533 | 2999 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241965 | Correction to PDCCH level in CSI-RS based RLM test cases for RedCap | Anritsu | 38.533 | 2999 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241126 | Correction to gap offset and SMTC configuration in 16.6.2.9 and 16.6.2.10 | Anritsu | 38.533 | 3000 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241824 | Correction to gap offset and SMTC configuration in 16.6.2.9 and 16.6.2.10 | Anritsu | 38.533 | 3000 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-241127 | Correction to SSB configuration in 16.6.2.11 and 16.6.2.12 | Anritsu | 38.533 | 3001 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241825 | Correction to SSB configuration in 16.6.2.11 and 16.6.2.12 | Anritsu | 38.533 | 3001 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-241128 | Correction to test parameters in 5.6.1.3 and 5.6.1.4 | Anritsu | 38.533 | 3002 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-242021 | Correction to test parameters in 5.6.1.3 and 5.6.1.4 | Anritsu | 38.533 | 3002 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241129 | Correction to TRS Configuration in 6.3.2.1.3 | Anritsu | 38.533 | 3003 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241998 | Correction to TRS Configuration in 6.3.2.1.3 | Anritsu | 38.533 | 3003 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241130 | Correction to PDCCH level in 6.5.1.8 | Anritsu | 38.533 | 3004 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241999 | Correction to PDCCH level in 6.5.1.8 | Anritsu | 38.533 | 3004 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241160 | Correction to EN-DC FR1 Beam Failure TCs 4.5.5.x | Keysight Technologies UK Ltd | 38.533 | 3005 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241162 | Correction to EN-DC FR2 Beam Failure TCs 5.5.5.x | Keysight Technologies UK Ltd | 38.533 | 3006 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241197 | Addition of new test case 4.7.8.1 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Sporton | 38.533 | 3007 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-241830 | Addition of new test case 4.7.8.1 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Sporton | 38.533 | 3007 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241198 | Addition of new test case 4.7.8.2 EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.533 | 3008 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241200 | Correct of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 including Test Tolerance | Sporton | 38.533 | 3009 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241201 | Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell | Sporton | 38.533 | 3010 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241901 | Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell | Sporton | 38.533 | 3010 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241209 | Corrections to NR SA FR1 RedCap Event Trigger TCs 16.6.2.9, 16.6.2.10 | Keysight Technologies UK Ltd | 38.533 | 3011 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-241211 | Annex E.14 correction for new RedCap test cases | Rohde & Schwarz | 38.533 | 3012 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241212 | Annex F correction for RedCap test cases including Test Tolerance | Rohde & Schwarz | 38.533 | 3013 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-241213 | Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 1 Rx UE test case 16.7.1.1.1 including Test Tolerance | Rohde & Schwarz | 38.533 | 3014 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241214 | Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE test case 16.7.1.1.2 including Test Tolerance | Rohde & Schwarz | 38.533 | 3015 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241215 | Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.1 including Test Tolerance | Rohde & Schwarz | 38.533 | 3016 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241216 | Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE test case 16.7.1.2.2 including Test Tolerance | Rohde & Schwarz | 38.533 | 3017 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241217 | Addition of NR SA FR1 SS-RSRQ measurement accuracy for 1 Rx UE test case 16.7.2.1 including Test Tolerance | Rohde & Schwarz | 38.533 | 3018 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241218 | Addition of NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE test case 16.7.2.2 including Test Tolerance | Rohde & Schwarz | 38.533 | 3019 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241219 | Addition of RRM RedCap test cases 16.7.1.3.1, 16.7.1.3.2 | Rohde & Schwarz | 38.533 | 3020 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241220 | Addition of RRM RedCap test cases 16.7.1.4.1, 16.7.1.4.2 | Rohde & Schwarz | 38.533 | 3021 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241227 | Editorial Correction to RedCap PRACH TC 16.3.2.2.8 | Rohde & Schwarz | 38.533 | 3022 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241228 | Editorial corrections for RedCap tets cases 18.3.1.x | Rohde & Schwarz | 38.533 | 3023 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241231 | Corrections to test procedure for 2-step RACH TC 6.3.2.2.4 | Rohde & Schwarz | 38.533 | 3024 | - | Rel-18 | F | TEI16_Test, NR_2step_RACH-UEConTest | revised |
| R5-241837 | Corrections to test procedure for 2-step RACH TC 6.3.2.2.4 | Rohde & Schwarz | 38.533 | 3024 | 1 | Rel-18 | F | TEI16_Test, NR_2step_RACH-UEConTest | agreed |
| R5-241232 | Correction for test case 5.5.6.2.1 | Rohde & Schwarz | 38.533 | 3025 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241835 | Correction for test case 5.5.6.2.1 | Rohde & Schwarz | 38.533 | 3025 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241233 | Editorial correction in Table 5.6.1.3.5-2 | Rohde & Schwarz | 38.533 | 3026 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241234 | Corrections to BFD-LR test case 5.5.5.5 | Rohde & Schwarz | 38.533 | 3027 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241836 | Corrections to BFD-LR test case 5.5.5.5 | Rohde & Schwarz | 38.533 | 3027 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241235 | Correction to BFD-LR test case 5.5.5.5 message content | Rohde & Schwarz | 38.533 | 3028 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241236 | Removal of Editors note for RRM FR2 RLM in-sync test cases | Rohde & Schwarz | 38.533 | 3029 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241996 | Removal of Editors note for RRM FR2 RLM in-sync test cases | Rohde & Schwarz | 38.533 | 3029 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241237 | Revision of Editors note for RRM FR2 RLM test cases | Rohde & Schwarz | 38.533 | 3030 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241997 | Revision of Editors note for RRM FR2 RLM test cases | Rohde & Schwarz | 38.533 | 3030 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241238 | Editor correction to Test Procedure 5.5.1.2 | Rohde & Schwarz | 38.533 | 3031 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241241 | Addition of SA FR1 NR-U intra-frequency SS-SINR measurement accuracy test cases | Rohde & Schwarz | 38.533 | 3032 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241242 | Addition of SA FR1 NR-U inter-frequency SS-SINR measurement accuracy test cases | Rohde & Schwarz | 38.533 | 3033 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241243 | Addition of SA FR1 NR-U intra-frequency RSSI measurement accuracy test cases | Rohde & Schwarz | 38.533 | 3034 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241264 | Core Spec alignment for RedCap tets cases 18.3.1.x | Rohde & Schwarz | 38.533 | 3035 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241265 | General update to NTN tests | Qualcomm Germany | 38.533 | 3036 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241266 | Addition of NTN timing accuracy test 14.3.1.1 | Qualcomm Germany | 38.533 | 3037 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241267 | Addition of NTN time-based CHO tests 14.2.1.x | Qualcomm Germany | 38.533 | 3038 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241268 | Addition of NTN intra-frequency cell reselection tests 14.1.x | Qualcomm Germany | 38.533 | 3039 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241269 | Addition of NTN SS-SINR measurement accuracy tests 14.6.3.x | Qualcomm Germany | 38.533 | 3040 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241271 | Core alignment for NSA NR-U tests | Qualcomm Germany | 38.533 | 3041 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241828 | Core alignment for NSA NR-U tests | Qualcomm Germany | 38.533 | 3041 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241272 | Addition of NR-U SA BWP switch tests including TT | Qualcomm Germany | 38.533 | 3042 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241273 | Addition of UL active BWP switch test 10.3.5.1 including TT | Qualcomm Germany | 38.533 | 3043 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241274 | Update of NR-U Annex E and F including TT | Qualcomm Germany | 38.533 | 3044 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241275 | Update to NR-U general sections | Qualcomm Germany | 38.533 | 3045 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241276 | Addition of NR-U UE Timing Tests 10.2.x | Qualcomm Germany | 38.533 | 3046 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241277 | Addition of NR-U L1-RSRP accuracy test 10.5.4.1 | Qualcomm Germany | 38.533 | 3047 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241278 | Addition of NR-U RSSI measurement accuracy tests 10.5.5.x | Qualcomm Germany | 38.533 | 3048 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241279 | Addition of NR-U Channel occupancy measurement accuracy tests 10.5.6.x | Qualcomm Germany | 38.533 | 3049 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241280 | Addition of NR-U L1-RSRP tests 10.4.3.x | Qualcomm Germany | 38.533 | 3050 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241281 | Addition of NR-U IRAT tests 10.4.4.x | Qualcomm Germany | 38.533 | 3051 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241282 | Update to NSA NR-U tests including TT | Qualcomm Germany | 38.533 | 3052 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241283 | Update to SA NR-U tests including TT | Qualcomm Germany | 38.533 | 3053 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241284 | Update to OCNG noise power in UL CCA model | Qualcomm Germany | 38.533 | 3054 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241285 | Core alignment for NR-U common sections | Qualcomm Germany | 38.533 | 3055 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241295 | Cleanup on FR1 CG-SDT test 6.2.1 | Qualcomm Germany | 38.533 | 3056 | - | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-241296 | Addition of RedCap SA FR2 SSB RLM OOS in DRX test case 17.5.1.3 | Qualcomm Germany | 38.533 | 3057 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241297 | Addition of RedCap SA FR2 SSB BFR test cases 17.5.2.1 and 17.5.2.2 | Qualcomm Germany | 38.533 | 3058 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241298 | Update to RRM test 16.3.1.8 including TT | Qualcomm Germany | 38.533 | 3059 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241299 | Correction to stationary relaxed measurement criterion tests | Qualcomm Germany | 38.533 | 3060 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241300 | Update to Re-establishment test cases | Qualcomm Germany | 38.533 | 3061 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241301 | Applcability update for several RedCap tests | Qualcomm Germany | 38.533 | 3062 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241304 | Update to RRM test 6.3.1.4 including TT | Qualcomm Germany | 38.533 | 3063 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241311 | Addition of Clause 4.7.9.0 minimum conformance requirements for CSI-RSRQ | Apple | 38.533 | 3064 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241312 | Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.1 | Apple | 38.533 | 3065 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241313 | Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.2 | Apple | 38.533 | 3066 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241314 | Addition of Clause 4.7.10.0 minimum conformance requirements for CSI-SINR | Apple | 38.533 | 3067 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241315 | Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.10.1 | Apple | 38.533 | 3068 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241316 | Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell l test case 4.7.10.2 | Apple | 38.533 | 3069 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241317 | Addition of Clause 5.7.8.0 minimum conformance requirements for CSI-RSRQ | Apple | 38.533 | 3070 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241318 | Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 5.7.8.1 | Apple | 38.533 | 3071 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241319 | Addition of EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.8.2 | Apple | 38.533 | 3072 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241320 | Addition of Clause 5.7.9.0 minimum conformance requirements for CSI-SINR | Apple | 38.533 | 3073 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241321 | Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.1 | Apple | 38.533 | 3074 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241322 | Addition of EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.2 | Apple | 38.533 | 3075 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241323 | Addition of Clause 6.7.11.0 minimum conformance requirements for CSI-RSRQ | Apple | 38.533 | 3076 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241324 | Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.1 | Apple | 38.533 | 3077 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241325 | Addition of SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.2 | Apple | 38.533 | 3078 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241326 | Addition of Clause 6.7.12.0 minimum conformance requirements for CSI-SINR | Apple | 38.533 | 3079 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241327 | Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.1 | Apple | 38.533 | 3080 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241328 | Addition of SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.2 | Apple | 38.533 | 3081 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241329 | Addition of Clause 7.7.8.0 minimum conformance requirements for CSI-RSRQ | Apple | 38.533 | 3082 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241330 | Addition of SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.8.1 | Apple | 38.533 | 3083 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241331 | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.8.2 | Apple | 38.533 | 3084 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241332 | Addition of Clause 7.7.9.0 minimum conformance requirements for CSI-SINR | Apple | 38.533 | 3085 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241333 | Addition of SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.9.1 | Apple | 38.533 | 3086 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241334 | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.9.2 | Apple | 38.533 | 3087 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-241831 | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.9.2 | Apple | 38.533 | 3087 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241335 | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex E | Apple | 38.533 | 3088 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241336 | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F | Apple | 38.533 | 3089 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-241893 | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F | Apple | 38.533 | 3089 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241428 | Addition of gap pattern configs | Apple Benelux B.V. | 38.533 | 3090 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | withdrawn |
| R5-241447 | Correction to Annex F for R15 RRM test cases | Huawei, HiSilicon | 38.533 | 3091 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240597 | Introduce Annex for maximum uncertainty of test system and test tolerance | Keysight Technologies UK Ltd | 38.551 | 0001 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-241934 | Introduce Annex for maximum uncertainty of test system and test tolerance | Keysight Technologies UK Ltd | 38.551 | 0001 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-240598 | Correction and alignment of Annex B title | Keysight Technologies UK Ltd | 38.551 | 0002 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-240599 | Further clarifications in Annex A such as MPAC description and coordinate system | Keysight Technologies UK Ltd | 38.551 | 0003 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-240600 | Align Test Case Structure to typical RAN5 spec | Keysight Technologies UK Ltd | 38.551 | 0004 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-241935 | Align Test Case Structure to typical RAN5 spec | Keysight Technologies UK Ltd | 38.551 | 0004 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-241047 | Editorial update on Annex C | Apple (UK) Limited | 38.551 | 0005 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-241936 | Editorial update on Annex C | Apple (UK) Limited | 38.551 | 0005 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-241048 | Editorial update on Annex E | Apple (UK) Limited | 38.551 | 0006 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-241937 | Editorial update on Annex E | Apple (UK) Limited | 38.551 | 0006 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-241049 | Editorial update on clause 3 | Apple (UK) Limited | 38.551 | 0007 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-241938 | Editorial update on clause 3 | Apple (UK) Limited | 38.551 | 0007 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-241050 | Editorial update on clause 4 | Apple (UK) Limited | 38.551 | 0008 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-241939 | Editorial update on clause 4 | Apple (UK) Limited | 38.551 | 0008 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-241416 | Add missing abbreviations | Keysight Technologies UK Ltd | 38.551 | 0009 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-240596 | CR on Alternate TRS Procedure with Linearization | Keysight Technologies UK Ltd | 38.561 | 0001 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | withdrawn |
| R5-241206 | CR on EIS search interpolation | ROHDE & SCHWARZ | 38.561 | 0002 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | revised |
| R5-241929 | CR on EIS search interpolation | ROHDE & SCHWARZ | 38.561 | 0002 | 1 | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-241421 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | 38.561 | 0003 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | revised |
| R5-241932 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | 38.561 | 0003 | 1 | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-241422 | Update of TT within TRP and TRS tests | Apple Benelux B.V. | 38.561 | 0004 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | revised |
| R5-241931 | Update of TT within TRP and TRS tests | Apple Benelux B.V. | 38.561 | 0004 | 1 | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-241425 | Updates to Annex A.4.2.12 | Apple Benelux B.V. | 38.561 | 0005 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | withdrawn |
| R5-240129 | Addition of TT analysis for positioning test case 15.2.8 | CATT | 38.903 | 0641 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-240130 | Addition of TT analysis for positioning test case 15.2.9 and 15.2.10 | CATT | 38.903 | 0642 | - | Rel-18 | F | NR_pos_enh-UEConTest | revised |
| R5-241900 | Addition of TT analysis for positioning test case 15.2.9 and 15.2.10 | CATT | 38.903 | 0642 | 1 | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-240131 | Addition of TT analysis for positioning test case 16.2.7 | CATT | 38.903 | 0643 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-240132 | Addition of TT analysis for positioning test case 16.2.8 | CATT | 38.903 | 0644 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-240133 | Addition of TT analysis for positioning test case 16.3.4 | CATT | 38.903 | 0645 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-240149 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | 38.903 | 0646 | - | Rel-18 | F | NR_MG_enh-UEConTest | withdrawn |
| R5-240150 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.903 | 0647 | - | Rel-18 | F | NR_MG_enh-UEConTest | withdrawn |
| R5-240373 | Test Tolerances for NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.1 | China Telecom, Huawei, HiSilicon | 38.903 | 0648 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-240374 | Test Tolerances for NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1 | China Telecom, Huawei, HiSilicon | 38.903 | 0649 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-240402 | FR2c MU definition in 38.903 | Keysight Technologies UK Ltd | 38.903 | 0650 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241863 | FR2c MU definition in 38.903 | Keysight Technologies UK Ltd | 38.903 | 0650 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240406 | FR2 MU - PC1 UL MIMO - Minimum output power test - 38.903 | Keysight Technologies UK Ltd | 38.903 | 0651 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240606 | CR to define FR2d QoQZ MUs and misc QoQZ MU corrections | Keysight Technologies UK Ltd | 38.903 | 0652 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241864 | CR to define FR2d QoQZ MUs and misc QoQZ MU corrections | Keysight Technologies UK Ltd | 38.903 | 0652 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240635 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with Pre-MG | Anritsu | 38.903 | 0653 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-241680 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with Pre-MG | Anritsu | 38.903 | 0653 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-240637 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with concurrent gaps | Anritsu | 38.903 | 0654 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-241681 | Introduction of Test Tolerance analysis for FR2 SA event triggered reporting tests with concurrent gaps | Anritsu | 38.903 | 0654 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-240639 | Introduction of Test Tolerance analysis for SA FR2 event triggered reporting tests with NCSG | Anritsu | 38.903 | 0655 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-241682 | Introduction of Test Tolerance analysis for SA FR2 event triggered reporting tests with NCSG | Anritsu | 38.903 | 0655 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-240733 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | 38.903 | 0656 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240734 | TT analysis for RedCap RRM test case 16.2.1.2 | Huawei, HiSilicon | 38.903 | 0657 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-240735 | TT analysis for RedCap RRM test case 16.3.2.2.2 and 16.3.2.2.4 | Huawei, HiSilicon | 38.903 | 0658 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240736 | TT analysis for RedCap RRM test case 16.3.2.2.6 and 16.3.2.2.8 | Huawei, HiSilicon | 38.903 | 0659 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240737 | TT analysis for RedCap RRM test case 16.6.5.3 | Huawei, HiSilicon | 38.903 | 0660 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240738 | TT analysis for RedCap RRM test case 16.6.5.4 | Huawei, HiSilicon | 38.903 | 0661 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240739 | TT analysis for RedCap RRM test case 16.7.2.3.1 | Huawei, HiSilicon | 38.903 | 0662 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240740 | TT analysis for RedCap RRM test case 16.7.2.3.2 | Huawei, HiSilicon | 38.903 | 0663 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-242044 | TT analysis for RedCap RRM test case 16.7.2.3.2 | Huawei, HiSilicon | 38.903 | 0663 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240741 | TT analysis for RedCap RRM test case 16.7.2.4.1 | Huawei, HiSilicon | 38.903 | 0664 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240742 | TT analysis for RedCap RRM test case 16.7.2.4.2 | Huawei, HiSilicon | 38.903 | 0665 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240743 | TT analysis for RedCap RRM test case 16.7.3.1 and 16.7.3.2 | Huawei, HiSilicon | 38.903 | 0666 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240744 | TT analysis for RedCap RRM test case 16.7.3.3.1 | Huawei, HiSilicon | 38.903 | 0667 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241887 | TT analysis for RedCap RRM test case 16.7.3.3.1 | Huawei, HiSilicon | 38.903 | 0667 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240745 | TT analysis for RedCap RRM test case 16.7.3.3.2 | Huawei, HiSilicon | 38.903 | 0668 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241888 | TT analysis for RedCap RRM test case 16.7.3.3.2 | Huawei, HiSilicon | 38.903 | 0668 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240746 | TT analysis for RedCap RRM test case 16.7.3.4.1 | Huawei, HiSilicon | 38.903 | 0669 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241889 | TT analysis for RedCap RRM test case 16.7.3.4.1 | Huawei, HiSilicon | 38.903 | 0669 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240747 | TT analysis for RedCap RRM test case 16.7.3.4.2 | Huawei, HiSilicon | 38.903 | 0670 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240748 | TT analysis for RedCap RRM test case 16.7.4.1.1 | Huawei, HiSilicon | 38.903 | 0671 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240749 | TT analysis for RedCap RRM test case 16.7.4.1.2 | Huawei, HiSilicon | 38.903 | 0672 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240750 | TT analysis for RedCap RRM test case 16.7.4.2.1 16.7.4.3.1 and 16.7.4.4.1 | Huawei, HiSilicon | 38.903 | 0673 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241890 | TT analysis for RedCap RRM test case 16.7.4.2.1 16.7.4.3.1 and 16.7.4.4.1 | Huawei, HiSilicon | 38.903 | 0673 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240751 | TT analysis for RedCap RRM test case 16.7.4.2.2 16.7.4.3.2 and 16.7.4.4.2 | Huawei, HiSilicon | 38.903 | 0674 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241891 | TT analysis for RedCap RRM test case 16.7.4.2.2 16.7.4.3.2 and 16.7.4.4.2 | Huawei, HiSilicon | 38.903 | 0674 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240752 | TT analysis for RedCap RRM test case 16.7.5.1 | Huawei, HiSilicon | 38.903 | 0675 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240753 | TT analysis for RedCap RRM test case 16.7.5.2 | Huawei, HiSilicon | 38.903 | 0676 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240754 | TT analysis for RedCap RRM test case 16.7.6.1 | Huawei, HiSilicon | 38.903 | 0677 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240755 | TT analysis for RedCap RRM test case 16.7.6.2 | Huawei, HiSilicon | 38.903 | 0678 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240787 | Addition of Test Tolerance for NR SA Event triggered reporting RedCap test cases | Ericsson | 38.903 | 0679 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240788 | Test Tolerances for NR SA FR1 - E-UTRAN event-triggered reporting tests for 1 Rx RedCap UE | Ericsson | 38.903 | 0680 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-240789 | Test Tolerances for NR SA FR1 - E-UTRAN event-triggered reporting tests for 2 Rx RedCap UE | Ericsson | 38.903 | 0681 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241086 | Update for FR2c MU | Anritsu | 38.903 | 0682 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | revised |
| R5-241865 | Update for FR2c MU | Anritsu | 38.903 | 0682 | 1 | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-241150 | TT analysis for FR1 PDSCH with inter-cell interference test cases | QUALCOMM Europe Inc. - Italy | 38.903 | 0683 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-241199 | Correct of test tolerance analysis of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 | Sporton | 38.903 | 0684 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241902 | Correct of test tolerance analysis of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 | Sporton | 38.903 | 0684 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241221 | Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 1 Rx UE test case 16.7.1.1.1 | Rohde & Schwarz | 38.903 | 0685 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241222 | Test Tolerance for NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE test case 16.7.1.1.2 | Rohde & Schwarz | 38.903 | 0686 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241223 | Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.1 | Rohde & Schwarz | 38.903 | 0687 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241224 | Test Tolerance for NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.2 | Rohde & Schwarz | 38.903 | 0688 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241225 | Test Tolerance for NR SA FR1 SS-RSRQ measurement accuracy UE test case 16.7.2.1 | Rohde & Schwarz | 38.903 | 0689 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241226 | Test Tolerance NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE test case 16.7.2.2 | Rohde & Schwarz | 38.903 | 0690 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241229 | Core Spec alignment for RedCap tets cases 18.3.1.x | ROHDE & SCHWARZ | 38.903 | 0691 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-241288 | Addition of TTs for NR-U Test Cases 10.3.1.2 and 11.4.1.2 | Qualcomm Germany | 38.903 | 0692 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241881 | Addition of TTs for NR-U Test Cases 10.3.1.2 and 11.4.1.2 | Qualcomm Germany | 38.903 | 0692 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241289 | Addition of TTs for NR-U Test Cases 10.3.1.3 and 11.4.1.3 | Qualcomm Germany | 38.903 | 0693 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241882 | Addition of TTs for NR-U Test Cases 10.3.1.3 and 11.4.1.3 | Qualcomm Germany | 38.903 | 0693 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241290 | Addition of TTs for NR-U Test Cases 10.3.4.1, 10.3.4.2, 11.4.4.1 and 11.4.4.2 | Qualcomm Germany | 38.903 | 0694 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241883 | Addition of TTs for NR-U Test Cases 10.3.4.1, 10.3.4.2, 11.4.4.1 and 11.4.4.2 | Qualcomm Germany | 38.903 | 0694 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241291 | Addition of TTs for NR-U Test Cases 10.3.5.1 and 11.4.5.1 | Qualcomm Germany | 38.903 | 0695 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-242000 | Addition of TTs for NR-U Test Cases 10.3.5.1 and 11.4.5.1 | Qualcomm Germany | 38.903 | 0695 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241292 | Addition of TTs for NR-U Test Cases 10.3.5.2.1, 10.3.5.2.2, 11.4.5.2.1 and 11.4.5.2.2 | Qualcomm Germany | 38.903 | 0696 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241884 | Addition of TTs for NR-U Test Cases 10.3.5.2.1, 10.3.5.2.2, 11.4.5.2.1 and 11.4.5.2.2 | Qualcomm Germany | 38.903 | 0696 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241293 | Addition of TTs for NR-U Test Cases 10.3.5.3.1 and 11.4.5.3.1 | Qualcomm Germany | 38.903 | 0697 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-241885 | Addition of TTs for NR-U Test Cases 10.3.5.3.1 and 11.4.5.3.1 | Qualcomm Germany | 38.903 | 0697 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-241303 | Update of TT analysis for 6.3.1.4 and 16.3.1.8 | Qualcomm Germany | 38.903 | 0698 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | revised |
| R5-241892 | Update of TT analysis for 6.3.1.4 and 16.3.1.8 | Qualcomm Germany | 38.903 | 0698 | 1 | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-241337 | Addition of TT analysis grouping for test cases 4.5.9.1 to Table 8-1 | Apple | 38.903 | 0699 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241338 | Addition of TT analysis grouping for test cases 5.5.6.3.1 to Table 8-2 | Apple | 38.903 | 0700 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-241894 | Addition of TT analysis grouping for test cases 5.5.6.3.1 to Table 8-2 | Apple | 38.903 | 0700 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241339 | Addition of TT analysis grouping for test cases 5.5.6.4.1 to Table 8-2 | Apple | 38.903 | 0701 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-241895 | Addition of TT analysis grouping for test cases 5.5.6.4.1 to Table 8-2 | Apple | 38.903 | 0701 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-241345 | Documentation of MU for n259 | ROHDE & SCHWARZ | 38.903 | 0702 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241866 | Documentation of MU for n259 | ROHDE & SCHWARZ | 38.903 | 0702 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-241404 | Measurement uncertainty definition for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.903 | 0703 | - | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | revised |
| R5-242029 | Measurement uncertainty definition for UE Maximum Output Power - EIRP with UL Gaps test case | Keysight Technologies UK Ltd, Apple Inc | 38.903 | 0703 | 1 | Rel-18 | F | NR_RF_FR2_req_enh2-UEConTest | agreed |
| R5-241443 | MU TT updates for NTN RF tests | Apple Benelux B.V. | 38.903 | 0704 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-240234 | Update reference sensitivity test cases for CA_n1A-n8A-n78A | CU Digital Technology | 38.905 | 0850 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240270 | Addition of test points analysis for ATG test cases in 38.521-1 | CAICT, CMCC | 38.905 | 0851 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-241765 | Addition of test points analysis for ATG test cases in 38.521-1 | CAICT, CMCC | 38.905 | 0851 | 1 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-240308 | Updating test principle for AMPR for inter-band UL CA | Huawei, HiSilicon | 38.905 | 0852 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240318 | Updating test points for FR1 REFSENS for SUL test case | Huawei, HiSilicon | 38.905 | 0853 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240324 | Addition of reference sensitivity and spurious emissions TP analysis for new R16 NR CA combos within FR1 | KDDI Corporation | 38.905 | 0854 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240325 | Addition of reference sensitivity and spurious emissions TP analysis for new R17 NR CA combos within FR1 | KDDI Corporation | 38.905 | 0855 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240388 | TP analysis for NR ATG test cases | China Mobile Com. Corporation | 38.905 | 0856 | - | Rel-18 | F | NR_ATG-UEConTest | withdrawn |
| R5-240389 | TP analysis for ATG Configured Tx power | China Mobile Com. Corporation | 38.905 | 0857 | - | Rel-18 | F | NR_ATG-UEConTest | withdrawn |
| R5-240456 | Correction to reference sensitivity test point analysis for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | 38.905 | 0858 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241777 | Correction to reference sensitivity test point analysis for CA_n28A-n78A | Nokia, Nokia Shanghai Bell | 38.905 | 0858 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240462 | Introduction of reference sensitivity test point analysis for CA_n66A-n71A-n77(2A) and CA_n66A-n71A-n78(2A) | Nokia, Nokia Shanghai Bell | 38.905 | 0859 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240464 | Introduction of spurious emission TP analysis for CA_n71A-n78A | Nokia, Nokia Shanghai Bell | 38.905 | 0860 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240674 | Addition of reference sensitivity TP analysis for new PC2 EN-DC combos within FR1 | KDDI Corporation | 38.905 | 0861 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-240776 | Introduction of reference sensitivity test point analysis for CA_n1A-n28A-n78A and CA_n3A-n28A-n78A | Nokia, Nokia Shanghai Bell | 38.905 | 0862 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240778 | Introduction of spurious emission TP analysis for CA_n1A_n28A | Nokia, Nokia Shanghai Bell | 38.905 | 0863 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-240858 | Addition of TP analysis for NR-U test case 6.3F.4.3 | Ericsson | 38.905 | 0864 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240859 | Addition of TP analysis for NR-U test case 6.2F.4, Configured transmitted power for shared spectrum | Ericsson | 38.905 | 0865 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-240881 | Test point analysis of EN-DC spurious emissions for DC_2A_n66A and DC_30A_n66A | WE Certification Oy, AT&T | 38.905 | 0866 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-241778 | Test point analysis of EN-DC spurious emissions for DC_2A_n66A and DC_30A_n66A | WE Certification Oy, AT&T | 38.905 | 0866 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-240883 | Update reference sensitivity test cases for four bands configurations | Verizon | 38.905 | 0867 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241701 | Update reference sensitivity test cases for four bands configurations | Verizon | 38.905 | 0867 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-240891 | Update reference sensitivity test cases for additional band configurations | Verizon | 38.905 | 0868 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-240895 | Update reference sensitivity test cases for additional band configurations with PC2 UL | Verizon Spain | 38.905 | 0869 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241748 | Update reference sensitivity test cases for additional band configurations with PC2 UL | Verizon Spain | 38.905 | 0869 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-241000 | TP analysis for AMPR, ASEM and ASE for V2X | Huawei, HiSilicon | 38.905 | 0870 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-241006 | Test points analysis for FR1 AMPR test case with NS_47 and PC 1.5 | Huawei, HiSilicon, SoftBank Corp. | 38.905 | 0871 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-241008 | Addition of test point analysis for R17 FR1 enhancement test cases | Huawei, HiSilicon | 38.905 | 0872 | - | Rel-18 | F | NR_RF_FR1_enh-UEConTest | agreed |
| R5-241053 | Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC3 | NTT DOCOMO INC. | 38.905 | 0873 | - | Rel-18 | F | TEI15_Test | withdrawn |
| R5-241054 | Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC2 | NTT DOCOMO, INC. | 38.905 | 0874 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241749 | Addition of REFSENS TP analysis for DC_19A_n78A and DC_19A_n77A in PC2 | NTT DOCOMO, INC. | 38.905 | 0874 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-241055 | Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC3 | NTT DOCOMO, INC. | 38.905 | 0875 | - | Rel-18 | F | TEI15_Test | withdrawn |
| R5-241056 | Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC2 | NTT DOCOMO, INC. | 38.905 | 0876 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-241750 | Addition of REFSENS TP analysis for 3CC EN-DC for n78 and n79 in PC2 | NTT DOCOMO, INC. | 38.905 | 0876 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-241351 | Update of test point selection for 4DL CA configurations | ROHDE & SCHWARZ, Verizon | 38.905 | 0877 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-241908 | Update of test point selection for 4DL CA configurations | ROHDE & SCHWARZ, Verizon | 38.905 | 0877 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-241368 | Update of TP analysis for Spurious emissions test cases for FR1 UL CA | China Unicom | 38.905 | 0878 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-241903 | Update of TP analysis for Spurious emissions test cases for FR1 UL CA | China Unicom | 38.905 | 0878 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-241401 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | 38.905 | 0879 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-242014 | Description of ephemeris calculation process | Keysight Technologies UK Ltd, Thales | 38.905 | 0879 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-241436 | Updates to TP analysis for FR2 RF phase continuity test | Apple Benelux B.V. | 38.905 | 0880 | - | Rel-18 | F | NR_cov_enh-UEConTest | revised |
| R5-241743 | Updates to TP analysis for FR2 RF phase continuity test | Apple Benelux B.V. | 38.905 | 0880 | 1 | Rel-18 | F | NR_cov_enh-UEConTest | agreed |
| R5-241437 | Updates to TP analysis for FR1 RF phase continuity test | Apple Benelux B.V. | 38.905 | 0881 | - | Rel-18 | F | NR_cov_enh-UEConTest | revised |
| R5-241744 | Updates to TP analysis for FR1 RF phase continuity test | Apple Benelux B.V. | 38.905 | 0881 | 1 | Rel-18 | F | NR_cov_enh-UEConTest | agreed |
| R5-240475 | Removal of GEA1 and GEA2 algorithm verification | Keysight Technologies UK, Bureau Veritas | 51.010-1 | 5168 | - | Rel-13 | F | TEI_Test | revised |
| R5-241657 | Removal of GEA1 and GEA2 algorithm verification | Keysight Technologies UK, Bureau Veritas | 51.010-1 | 5168 | 1 | Rel-13 | F | TEI_Test | agreed |
| R5-240515 | Correction to GSM testcase 44.2.5.2.5 | ROHDE & SCHWARZ | 51.010-1 | 5169 | - | Rel-13 | F | TEI_Test | agreed |
| R5-241342 | Correction of coding scheme in USF testing | ROHDE & SCHWARZ, Apple | 51.010-1 | 5170 | - | Rel-13 | F | TEI8_Test | agreed |
| R5-240476 | Applicability updates to reflect the removal of GEA1 and GEA2 algorithm verification from the test cases | Keysight Technologies UK, Bureau Veritas | 51.010-2 | 4412 | - | Rel-13 | F | TEI_Test | agreed |
| R5-240514 | Removal of GEA1/GEA2 and verification of non-support of GEA1/GEA2 | ROHDE & SCHWARZ | 51.010-2 | 4413 | - | Rel-13 | F | TEI_Test | revised |
| R5-241632 | Removal of GEA1/GEA2 and verification of non-support of GEA1/GEA2 | ROHDE & SCHWARZ | 51.010-2 | 4413 | 1 | Rel-13 | F | TEI_Test | agreed |
| R5-240516 | Correction to Table A.2: IR_G TTCN test cases | ROHDE & SCHWARZ | 51.010-5 | 0152 | - | Rel-10 | F | TEI_Test | agreed |

## Annex C: Lists of liaisons

### C1: Incoming liaison statements

2 incoming LSs at RAN5#102

| Document | Original | Title | From | Decision | Reply TDoc |
| --- | --- | --- | --- | --- | --- |
| R5-240017 | R2-2313618 | Reply LS on frequencyInfo for NR SL RSRP measurements | TSG WG RAN2 | noted | (none) |
| R5-240018 | SP-231782 | LS on Prohibition of GEA1 & GEA2 Support in all releases | TSG SA | noted | (none) |

### C2: Outgoing liaison statements

5 outgoing LSs at RAN5#102

| Document | Title | To | Cc | reply to i/c LS |
| --- | --- | --- | --- | --- |
| R5-240020 | Reply LS on Prohibition of GEA1 & GEA2 Support in all releases | TSG SA, GCF SG | TSG WG SA3, TSG RAN, PTCRB Plenary | R5-240018 / SP-231782 |
| R5-241700 | Conclusion of RAN5 NR MIMO OTA Working Item | CTIA Certification, CTIA MIMO OTA Sub-Working Group | GCF CAG, CCSA TC9 WG1 |  |
| R5-241876 | LS reply on FR2 ACS/IBB testing | TSG WG RAN4 | - | R5-235987 / R4-2313211 |
| R5-241956 | LS to RAN4 on TT work for Rel-18 NR FR1 TRP TRS | TSG WG RAN4 | - |  |
| R5-241957 | LS on 3GPP 5G NR FR1 OTA Conformance Test Specification | ETSI MSG TFES, GCF CAG, GCF PAG, CTIA OTA WG, CTIA Certification, GSMA TSG-AP, NGMN Alliance, PTCRB plenary, CCSA TC9 WG1 | GCF SG |  |

## Annex D: List of agreed/approved new and revised Work Items

16 new WIDs were endorsed at RAN5#102, 10 WIDs revised

| Document | Title | Source | new/revised |
| --- | --- | --- | --- |
| R5-241459 | New WID on UE Conformance - IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects | CMCC, MTK, CAICT | WID new |
| R5-241460 | New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation | MediaTek Inc., China Telecom | WID new |
| R5-241461 | New WID on UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR | China Telecom | WID new |
| R5-241462 | New WID on UE Conformance - Further NR coverage enhancements | China Telecom | WID new |
| R5-241463 | New WID on UE Conformance - Further RF requirements enhancement for NR and EN-DC in frequency range 1 | Huawei, Hisilicon, China Telecom | WID new |
| R5-241464 | New WID on UE Conformance - Network energy savings for NR | Huawei, HiSilicon | WID new |
| R5-241465 | New WID on UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 | Huawei, HiSilicon, CMCC | WID new |
| R5-241466 | New WID on UE Conformance – Multi-carrier enhancements for NR | China Telecom, Huawei, HiSilicon | WID new |
| R5-241467 | New WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom, Ericsson, Huawei, Hisilicon, Qualcomm | WID new |
| R5-241468 | New WID on UE Conformance - Introduction of 900 MHz LTE band in the US | Nokia, Nokia Shanghai Bell | WID new |
| R5-241470 | New WID on UE Conformance - XR (eXtended Reality) enhancements for NR | Nokia, Nokia Shanghai Bell, CMCC, Huawei, Hisilicon, Qualcomm | WID new |
| R5-241471 | New WID on UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2 (FR2) | Samsung | WID new |
| R5-241472 | New WID on UE Conformance – NR MIMO Evolution for Downlink and Uplink | Samsung, Huawei, Hisilicon | WID new |
| R5-241473 | New WID on UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirement for NR Ues | Apple (UK) Limited | WID new |
| R5-241474 | New WID on UE Conformance - NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 | Apple Benelux B.V., Nokia | WID new |
| R5-241659 | New WID on UE Conformance - NR support for dedicated spectrum less than 5MHz for FR1 | Nokia, Nokia Shanghai Bell | WID new |
| R5-240067 | Revised WID on UE Conformance - NB-IoT eMTC NTN | CMCC | WID revised |
| R5-240393 | Revised WID on UE Conformance – Support of reduced capability NR devices | China Unicom,Hisilicon, Ericsson, Huawei, Qualcomm | WID revised |
| R5-240781 | Revised WID: UE Conformance - NR sidelink enhancement | CATT | WID revised |
| R5-240782 | Revised WID: UE Conformance - NR Sidelink Relay | CATT | WID revised |
| R5-241076 | Revised WID UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) | Ericsson | WID revised |
| R5-241662 | Revised WID on UE Conformance Test Aspects - Rel-17 IMS Data Channel | Huawei | WID revised |
| R5-241663 | Revised WID on UE Conformance - Air-to-ground network for NR | CMCC | WID revised |
| R5-241664 | Revised WID on UE Conformance - New Rel-18 NR licensed bands and extension of existing NR bands | Huawei, Hisilicon | WID revised |
| R5-241665 | Revised WID on UE Conformance - Rel-18 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | WID revised |
| R5-241666 | Revised WID: UE Conformance Test Aspects - NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | MediaTek Inc. | WID revised |

## Annex E: List of draft Technical Specifications and Reports

None

## Annex F: List of action items

## SIG:

## Action Points at RAN5#102

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#102.01 | SIG | Review and update TP4 of TS 38.523-1 TC 9.1.5.1.4 Initial registration / 5GS services / MICO mode / TAI list handling to make it a testable requirement | R&S, TF160, Keysight, MediaTek | R5-241687 | RAN5#103 | Pending |
| AP#102.02 | SIG | Resolve open issues captured in Editor’s note for TS 38.523-1 Emergency Call TCs 11.1.7 and 11.4.12 | Qualcomm, Huawei, TF160, R&S, Anritsu | R5-241691 R5-241692 | RAN5#103 | Pending |

## Action Points at RAN5#101

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#101.01 | SIG | Investigate on the solution to how to treat the capability of ER-NSSAI based on the network vendors' support. | CMCC, Qualcomm, CATT, Huawei, Hisilicon, MCC TF160, Ericsson, MediaTek | R5-236570 | RAN5#102 | Closed R5-240663 |
| AP#101.02 | SIG | Investigate on the solution of which release/WIs current test cases in 10.1.8 (NSAC) should be allocated. | CATT, CMCC, Qualcomm, MCC TF160, ZTE | R5-236570 | RAN5#103 | Pending R5-240663 |
| AP#101.03 | SIG | Investigate to decide on choosing dynamic/static/mixed signalling test environments for NGSO NTN testing. | Mediatek, MCC TF160，Qualcomm, Keysight, R&S, Anritsu | R5-237381 | RAN5#102 | Closed R5-240162 |
| AP#101.04 | SIG | Investigate on a proper title of TS38.508-1 cl4.7 to show the content of this sub-clause including NAS and V2X messages. | MCC TF160, CATT, Huawei, Hisilicon | R5-237433 | RAN5#102 | Closed R5-240432 |

## RF:

## Action Points at RAN5#102

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#102.21 | RF | Update Annex C how to calculate test frequencies for UL CA (Intraband NC) | E///, KEYS, R&S, Qualcomm, Huawei, CMCC, China Unicom | R5-241364 | RAN5#103 | Open |

## Action Points at RAN5#101

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#101.21 | RF | TE/TS vendors to study the minimum conditions and applicability of the reference measurements of the alternate EIS search linearization, which determine the coefficients of the linearization curves | KEYS, R&S, ETS-L | R5-237839 R5-241205/6 R5-240595 | RAN5#102 | Closed |
| AP#101.22 | RF | Check the simplication of UE co-existence for UL CA in TS38.521-1/-3, TS36.521-1 | China Unicom, CAICT, CMCC, China Telecom, Huawei, Nokia | R5-236443 R5-241367/8 | RAN5#106 | Open |

## Action Points at RAN5#100

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#100.21 | RF | System vendors to provide QoQZ MU measurements for EIRP/EIS and TRP metric for NTC and ETC (including FR2d) | KEYS, R&S, Anritsu | R5-235754 R5-240606/5 R5-241087 R5-241078 | RAN5#102 | Closed |

## Action Points at RAN5#99

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#99.21 | RF | Define MTSU for regulatory test cases (MOP, EIRP, TRP, Tx OFF power, Freq error, OBW, SEM, ACLR, Ref Sense, ACS, Inband Blocking, Tx/Rx Spurious emissions) for FR2 Band n259 | Anritsu, R&S, KEYS, DCM | R5-232983 R5-233174 R5-233971-5 R5-235140 R5-234891-3 R5-237725/7 R5-237731 R5-236067 R5-237733 R5-237734 R5-237723 R5-241083/4/5/6 R5-240401/2/3/4/5 R5-241344/5 | RAN5#103 | Open |

## Action Points at RAN5#98

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#98.22 | RF | Investigate whether UE shall meet the spurious emission requirement of new protected bands introduced in a later release | CAICT, Qualcomm, Apple | R5-230247 R5-232374 R5-232380 R5-234114 | RAN5#102 | Closed |

## Action Points at RAN5#97

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#97.25 | RF | Investigate and come up with testability proposal to ensure 1x2 channel in FR2 RRM tests is achieved with UE receiving signal power on both its Rx branches | Keysight, R&S, Anritsu, Qualcomm | R5-226658 R5-230934 R5-232673 R5-234680 R5-237848 R5-241237 | RAN5#103 | Open |
| AP#97.27 | RF | Update on-on transient period 6.4.2.1a in TS38.521-1 to address issues identified in R5-226794 | Anritsu, Qualcomm, Ericsson, R&S, Apple | R5-226794 R5-230058 R5-230217 R5-230218 R5-232987 R5-234898/9 R5-234178 R5-237653 R5-237859 R5-241088 R5-241445 R5-240618 R5-241356 | RAN5#103 | Open |

## Annex G: Void

## Annex H: List of participants

178 delegates and officials attended the RAN5#102 meeting.

| TITLE | Family Name | Given Name | Email | Organization Represented |
| --- | --- | --- | --- | --- |
| Mr. | Agarwal | Deepak | deepak@cewit.org.in | CEWiT |
| Mr. | Akao | Takashi | takashi.akao.wa@nttdocomo.com | NTT DOCOMO INC.. |
| Mr. | Ariza | Damian | damian.ariza@fcc.gov | FCC |
| Mr. | Balasubramanian | Vijay | vijayb@qti.qualcomm.com | QUALCOMM Europe Inc. - Italy |
| Ms. | Bardaux | Virginie | virginie.bardaux_tf160@vbtelecom.fr | Anritsu EMEA Ltd |
| Mr. | Bhise | Parikshit | parikshit.bhise@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | Biswas | Sudipto | sudipto.biswas@motorolasolutions.com | Motorola Solutions Germany |
| Mr. | Borsato | Ronald | rb354e@att.com | AT&T |
| Dr. | Boukley Hasan | Wael | wael.boukleyhasan@vodafone.com | Vodafone Telekomünikasyon A.S. |
| Dr. | Cao | Jeffrey (Jianfei) | caojianfei@oppo.com | OTECH |
| Dr. | Chaudhari | Amar | ird13724@iitd.ac.in | IIT Delhi |
| Mr. | Chen | Ben | chenben@bjtu.edu.cn | BJTU |
| Miss | chen | jingjing | chenjingjing@chinamobile.com | China Mobile Group Device Co. |
| Mr. | Chen | Ningyu | chenningyu@chinamobile.com | China Mobile M2M Company Ltd. |
| Dr. | Chen | Xiang (Steven) | steven.x.chen@apple.com | Apple Switzerland AG |
| Mr. | Chen | Xiaozhong | chenxiaozhong@catt.cn | CICT Mobile |
| Mr. | Cheng | Ivan | IvanCheng@sporton.com.tw | Sporton International Inc |
| Mr. | Cheruvu | Bharadwaj Kumar | bcheruvu@qti.qualcomm.com | QUALCOMM Europe Inc. - Spain |
| Mr. | Choi | Jin | HyungjinChoi@eurofins.com | Eurofins KCTL |
| Mr. | Coroescu | Liviu-Lucian | liviu.coroescu@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | Cui | Shengjiang | cuishengjiang@oppo.com | OPPO Beijing |
| Mr. | Das | Siddharth | siddharth.1.das@nokia.com | Nokia Japan |
| Mr. | Desai | Vipul | vdesai@futurewei.com | Futurewei Technologies |
| Mr. | Du | Hao | duhao.txyjy@vivo.com | VIVO TECH GmbH |
| Ms. | Eitoku | Haruka | haruka.eitoku@ntt.com | NTT |
| Mr. | Feng | Sanjun | fengsanjun@vivo.com | vivo Mobile Communication (H) |
| Miss | Fernandez Martos | Flores | flores_fernandez@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Fortes Lopez | Jose M. | Jose.Fortes@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | fu | danbo | Danbo.Fu@mediatek.com | MediaTek Beijing Inc. |
| Mr. | GAO | Lei | gaol8@chinatelecom.cn | Esurfing IoT |
| Mr. | Genoud | Olivier | olivier.genoud@etsi.org | ETSI |
| Mr. | Gowda | Pradeep | pgowda@qti.qualcomm.com | Qualcomm Incorporated |
| Mr. | Grunditz | Håkan | hakan.grunditz@ericsson.com | Ericsson GmbH, Eurolab |
| Ms. | Gu | Chunying | guchunying@huawei.com | Huawei Technologies Japan K.K. |
| Mr. | Gudimitla | Koteswara Rao | ee16resch01005@iith.ac.in | Indian Institute of Tech (H) |
| Mr. | Hagenfeldt | Calle | calle.hagenfeldt@ericsson.com | Ericsson Limited |
| Miss | Hao | Yuxin | haoyuxin@huawei.com | HUAWEI TECHNOLOGIES Co. Ltd. |
| Ms. | Hapsari | Wuri | wuri.hapsari@dish.com | Dish Network |
| Dr. | Harris | Paul | paul.harris@viavisolutions.com | VIAVI Solutions |
| Mr. | Heino | Aleksi | aleksi.heino@wecertification.com | WE Certification Oy |
| Dr. | Hertel | Thorsten | thorsten.hertel@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Hitomi | Shinya | hitomishinya@murata.com | Murata Manufacturing Co Ltd. |
| Mr. | hu | Youjun | hu.youjun1@zte.com.cn | ZTE |
| Miss | Huang | Xueyan | huangxueyan@chinamobile.com | China Mobile (Hangzhou) Inf. |
| Mr. | Inoue | Yoshihiro | yoshihiro.inoue@ntt-at.co.jp | NTT-AT Corp. |
| Dr. | Jafarian | Javad | javad.jafarian@bell.ca | Bell Mobility |
| Ms. | Jaffar | Munira | munirajaffar@hughes.com | HUGHES Network Systems Ltd |
| Dr. | Jiang | Zheng | jiangzheng@hotmail.com | E-surfing Digital |
| Mr. | Jin | Yiran | yiran.jin@samsung.com | Samsung R&D Institute UK |
| Mr. | Jing | Xiangwei | jing.xiangwei@zte.com.cn | Sanechips |
| Mr. | John | Jacob | Jacob.John@motorola.com | Motorola Mobility España SA |
| Mr. | Jönsson | Bo | bo.jonsson@ericsson.com | Ericsson LM |
| Mr. | Kahn | Jason | jason.kahn@nist.gov | NIST |
| Mr. | Kalahasti | Narendra | narendra.kalahasti@anritsu.com | Anritsu EMEA Ltd |
| Mr. | Kanchan | Mohit | mohit.kanchan@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Kangas | Matti | matti.kangas@nokia.com | Nokia Corporation |
| Mr. | Karlsson | Petter | petter.karlsson@ericsson.com | Ericsson France S.A.S |
| Mr. | Kato | Shun | syun.katou.fv@nttdocomo.com | NTT DOCOMO INC. |
| Mr. | Kitagawa | Ryu | ryuu.kitagawa.pn@nttdocomo.com | NTT Advanced Technology Corpor |
| Mr. | Kolodziej | Kuba | kuba.kolodziej@ericsson.com | Ericsson Inc. |
| Mr. | Kulakov | Alexey | Alexey.Kulakov1@vodafone.com | Vodafone GmbH |
| Mr. | Kwak | Phil | pkkwak@ktl.re.kr | Korea Testing Laboratory |
| Ms. | Kwon | Soohee | soohee.kwon@element.com | Element Materials Technology |
| Ms. | Lam | Maria | maria.lam@verizonwireless.com | Verizon Switzerland AG |
| Mr. | LI | Nanxi | linanxi@chinatelecom.cn | China Telecomunication Corp. |
| Mr. | li | qiang | liq03@pcl.ac.cn | Pengcheng laboratory |
| Mr. | Li | Qiming | li_qiming@apple.com | Apple (Guizhou) |
| Mr. | LI | Xiaoqiang | ben@cybercore.cn | Cybercore |
| Miss | Li | Yan | liyanwx@chinamobile.com | China Mobile (Suzhou) Software |
| Mr. | Lim | Suhwan | suhlim@meta.com | Meta Ireland |
| Ms. | Lin | Pei | linp@chinatelecom.cn | CTSI |
| Mr. | Liu | Bo | liubo1@chinatelecom.cn | Chinatelecom Cloud |
| Mr. | Liu | Bo | liubo1@bupt.edu.cn | BUPT |
| Mr. | Liu | Jiaxiang | liujiaxiang6@chinatelecom.cn | China Telecommunications |
| Mr. | Lo | Anthony | alo2@futurewei.com | Futurewei |
| Dr. | LU | Feng | ho-ro@kddi.com | KDDI Corporation |
| Miss | Lu | Li | lu.li@zte.com.cn | ZTE Korea Limited |
| Dr. | Ma | Jinwen | jinwen.ma@verizonwireless.com | Verizon Spain |
| Mr. | Ma | Teng | mateng1@oppo.com | OPPO |
| Mr. | Ma | Wei | ma.wei4@zte.com.cn | ZTE Corporation |
| Dr. | Ma | Zhifeng | ma.zhifeng@zte.com.cn | ZONSON |
| Mr. | Martins | Diogo | diogo.martins@vodafone.com | Vodafone Italia SpA |
| Mr. | Masal | Abhijeet | abhijeetmasal@cewit.org.in | CEWiT |
| Mr. | Matsuyoshi | Taiki | ta-matsuyoshi@kddi.com | KDDI Corporation |
| Dr. | Menzel | Edwin | edwin.menzel@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | Miyashita | Takuya | miyashita-takuya@sptvjsat.com | SKY Perfect JSAT Corporation |
| Dr. | Moayeri | Nader | moayeri@nist.gov | NIST |
| Mr. | Mohammed | Abdul Rasheed | rasheed@motorola.com | Motorola Mobility España SA |
| Mr. | Mohan | Ashwin | ashwin_mohan@apple.com | Apple Trading |
| Mr. | NAKAMURA | Kazuo | k-nakamura@nict.go.jp | NICT |
| Mr. | Ning | Weichen | ningwc@chinatelecom.cn | Esurfing IoT |
| Mr. | Oguma | Yuta | yuuta.oguma.yt@nttdocomo.com | NTT |
| Mr. | Olbrich | Emil | eolbrich@anterix.com | Anterix |
| Dr. | Parikh | Hemish | hparikh@qti.qualcomm.com | Qualcomm Technologies Ireland |
| Mr. | Petrovic | Niels | niels.petrovic@rohde-schwarz.com | ROHDE & SCHWARZ |
| Miss | Pi | Qiping | piqp@docomolabs-beijing.com.cn | DOCOMO Beijing Labs |
| Dr. | Pittampalli | Eshwar | eshwar.pittampalli@firstnet.gov | FirstNet |
| Mr. | Polaki | Kiran | k.polaki@samsung.com | Samsung R&D Institute UK |
| Mr. | Pudney | Chris | chris.pudney@vodafone.com | VODAFONE Group Plc |
| Ms. | Raghavan | Manasa | manasa_raghavan@apple.com | Apple Inc |
| Dr. | Ramamurthi | Vishwanath | vishwanath.ramamurthi@verizonwireless.com | Verizon UK Ltd |
| Mrs. | Rauer | Petra | petra.rauer@vodafone.com | Vodafone Ireland Plc |
| Mr. | Ruiz | Emilio | emilio_ruiz@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Säynäjäkangas | Tuomo | tuomo.saynajakangas@nokia.com | Nokia Germany |
| Mr. | Schumacher | Joseph | joseph.schumacher@att.com | AT&T Services, Inc. |
| Mr. | Sekino | Keita | ke-sekino@kddi.com | KDDI Corporation |
| Mr. | Setsu | Masafumi | Masafumi.Setsu@anritsu.com | Anritsu Corporation |
| Mr. | Shen | Jia | sj@oppo.com | OnePlus |
| Mr. | Shen | Xiaodong | shenxiaodong@chinamobile.com | China Mobile M2M Company Ltd. |
| Mr. | Sheoran | Deepak | Deepak_k_sheoran@apple.com | Apple Solutions |
| Miss | Shi | Yu | shiyu19@chinaunicom.cn | China Unicom |
| Dr. | SHI | Zhihua | szh@oppo.com | Hangzhou Douku |
| Mr. | Sigovich | Ingbert | ingbert.sigovich@etsi.org | ETSI |
| Mr. | Sinsathitchai | Wisuit | wsinsathitchai@apple.com | Apple AB Denmark |
| Mr. | Skousen | Derek | derek.skousen@bluetest.se | Bluetest AB |
| Mr. | Sommer | Markus | markus.sommer@vodafone.com | Vodafone Romania S.A. |
| Dr. | Song | Dan | songdan@chinamobile.com | China Mobile Com. Corporation |
| Ms. | Song | Huayue | hy.song@tta.or.kr | TTA |
| Dr. | Song | Lei | lei.song@verizon.com | Verizon Sweden |
| Miss | Sun | Huifang | sunhf50@chinaunicom.cn | CU Digital Technology |
| Dr. | Sun | Roy | r.sun@cablelabs.com | CableLabs |
| Mr. | Suzuki | Yasuki | ui-suzuki@kddi.com | KDDI Corporation |
| Dr. | Szini | Istvan | istvan@apple.com | Apple (UK) Limited |
| Mr. | Szpila | Rafal | Rafal.Szpila@orange.com | Orange Romania |
| Mr. | Takano | Masahiro | masahiro.takano.ru@nttdocomo.com | DOCOMO Beijing Labs |
| Mr. | Takeda | Hiroki | ho-takeda@kddi.com | KDDI Corporation |
| Dr. | Tang | Yang | yang.tang@apple.com | Apple GmbH |
| Miss | Tao | Amy | amy.tao@bureauveritas.com | Bureau Veritas ADT |
| Mr. | Tesserault | Guillaume | guillaume.tesserault@orange.com | Orange Spain |
| Dr. | Thompson | Steve | steve.thompson@phywireless.com | PHY Wireless |
| Dr. | Tian | Wenqiang | tianwenqiang@oppo.com | OnePlus |
| Miss | Tian | Xiaoyang | tianxiaoyang@catt.cn | CATT |
| Mr. | Toril | Adan | adan_toril@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Tugnawat | Yogesh | yogesht@qti.qualcomm.com | Qualcomm France |
| Mr. | Ubicini | Massimiliano | massimiliano.ubicini@telecomitalia.it | TELECOM ITALIA S.p.A. |
| Mr. | Vogedes | Jerome | jv0145@att.com | AT&T Labs, Inc |
| Mr. | Volnay | Christophe | christophe.volnay@etsi.org | ETSI |
| Mr. | WANG | FEI | wangfei@chinamobile.com | China Mobile International Ltd |
| Dr. | Wang | Ruixin | ruixin.wang@vivo.com | iQoo |
| Miss | Wang | Shiyuan | wangshiyuan@chinamobile.com | China Mobile International Ltd |
| Mr. | Wang | Xinrong | xinrong_wang@apple.com | Apple R&D |
| Dr. | Wang | Xuesong | cedar.wang@hisilicon.com | HUAWEI Technologies Japan K.K. |
| Dr. | wei | xusheng | xusheng.wei@vivo.com | vivo Mobile Communication Co., |
| Mr. | Weiler | Niclas | niclas.weiler@ericsson.com | Ericsson-LG Co., LTD |
| Mr. | Wu | Jingzhou | wujingzhou@chinatelecom.cn | China Telecom Corporation Ltd. |
| Ms. | Xie | Fang | xiefang@chinamobile.com | China Mobile (Suzhou) Software |
| Mr. | Xing | Jinqiang | xingjinqiang@oppo.com | OPPO (chongqing) Intelligence |
| Miss | Xu | Jiali | xu.jiali@zte.com.cn | ZTE Corporation. |
| Mr. | Xue | Fei | xue.fei25@zte.com.cn | ZTE Corporation |
| Mr. | Yamashita | Osamu | osamu.yamashita@anritsu.com | Anritsu Corporation |
| Mr. | Yang | Tuo | yangtuo@chinamobile.com | China Mobile E-Commerce Co. |
| Mr. | Yao | Kun | yao.kun1@zte.com.cn | Nubia Technology Co.,Ltd |
| Mr. | Yashima | Takuya | takuya.yashima.zu@nttdocomo.com | NTT DOCOMO INC.. |
| Dr. | Yerrapragada | Anil Kumar | anilkumar@5gtbiitm.in | Indian Institute of Tech (M) |
| Dr. | Zhang | Dawei | dawei_zhang@apple.com | Apple France |
| Dr. | Zhang | Meng | meng.zhang@intel.com | Intel China Ltd. |
| Mr. | Zhang | Miaoqi | zhangmiaoqi@chinamobile.com | CMDI |
| Mr. | ZHANG | Shichang | shichangzhang@oppo.com | Hangzhou Mengyuxiang |
| Mr. | Zhang | Shuang(Allen) | allen.zhang@mediatek.com | MediaTek (Hefei) Inc. |
| Mr. | Zhang | Wenfeng | zhang.wenfeng@zohomail.com | Guangdong OPPO Mobile Telecom. |
| Miss | Zhang | Yaping | zhangyaping13@huawei.com | HuaWei Technologies Co., Ltd |
| Ms. | Zhang | Yufeng | zhangyufeng@caict.ac.cn | CAICT. |
| Mr. | Zhao | Huawei | derrick.chao@zte.com.cn | ZTE Photonics |
| Ms. | ZHAO | JING | zhaoj16@chinatelecom.cn | China Telecom Corporation Ltd. |
| Mr. | Zhao | Luyang | zhaoluyang@chinamobile.com | China Mobile (Hangzhou) Inf. |
| Miss | Zhao | Ya | zhaoya@hisilicon.com | HiSilicon Technologies Co. Ltd |
| Mr. | Zhao | Zheng | zheng.zhao@verizonwireless.com | Verizon Denmark |
| Mr. | Zhao | Zhenshan | zhaozhenshan@oppo.com | Guangdong OPPO Mobile Telecom. |
| Miss | Zheng | Minhua | minhua.zheng@vivo.com | Nanjing Weibo |
| Mr. | Zheng | Yi | zhengyi@chinamobile.com | China Mobile (Hangzhou) Inf. |
| Mr. | Zhou | Daiwei | daiwei.zhou@mediatek.com | MediaTek Korea Inc. |
| Dr. | Zhou | Lei | zhou.leiH@h3c.com | New H3C Technologies Co., Ltd. |
| Mr. | Zhou | Shuai | shuai.zhou@vivo.com | GUANGDONG GENIUS TECHNOLOGY CO |
| Mr. | Zhou | Wubin | zhou.wubin@zte.com.cn | ShenZhen Zhongxing Shitong |
| Mr. | Zhou | Xutao | xutao.zhou@vivo.com | vivo Mobile Communication Co., |
| Miss | Zhu | Siting | zhusiting@caict.ac.cn | CAICT. |
| Mr. | Zhu | Zengbao | zengbaozhu@bupt.edu.cn | BUPT |
| Mr. | Zuo | Zhisong | zuozhisong@oppo.com | Shenzhen Heytap |

## Annex I: List of future meetings