3GPP TSG-RAN5 Meeting #103	R5-242394

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3122 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 3 MHz CBW test frequencies for NR Bands n26, n28, n85 and n100 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3 MHz CBW test frequencies need to be specified for NR Bands n26, n28, n85 and n100 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses updated to add test frequencies for bands n26, n28, n85 and n100 - Modify Low test channel bandwidths in clause 4.3.1.0B - Add 3MHz CBW in clause 4.3.1.0D Bandwidth part - Add 3MHz CBW test frequencies bands n26, n28, n85 and n100 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3 MHz CBW test frequencies for bands n26, n28, n85 and n100 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0B, 4.3.1.0D, 4.3.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Band n85 Low test channel bandwidths and test frequencies for wider than 3 MHz are specified in R5-242362 (CR 3118) including notes for Table 4.3.1.1.1.85-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 4.3	Reference test conditions

### 4.3.1	Test frequencies

Editor's Note: n7 (High: 50MHz), n8 (High: 35MHz), n40 (High: 100MHz), n65 (High: 50MHz), n66 (High: 45MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

#### 4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8] and TS 38.101-3 [9].

-------- Skipped unchanged clauses --------

#### 4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1, Table 4.3.1.0B-2 and Table 4.3.1.0B-3 for FR1, FR2 and NTN respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band | UE Low Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Low Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 5 | 5 |
| n2 | 5 | 5 |
| n3 | 5 | 5 |
| n5 | 5 | 5 |
| n7 | 5 | 5 |
| n8 | 5 | 5 |
| n12 | 5 | 5 |
| n13 | 5 | 5 |
| n14 | 5 | 5 |
| n20 | 5 | 5 |
| n24 | 5 | 5 |
| n25 | 5 | 5 |
| n26 | 36,5 | 5 |
| n28 | 36,5 | 5 |
| n29 | 52 | N/A |
| n30 | 5 | 5 |
| n34 | 5 | 5 |
| n38 | 5 | 5 |
| n39 | 5 | 5 |
| n40 | 54,105 | 10 |
| n41 | 10 | 10 |
| n46 | 104,205 | N/A |
| n48 | 54, 105 | 10 |
| n50 | 54,105 | 10 |
| n51 | 5 | 5 |
| n53 | 5 | 5 |
| n65 | 5 | 5 |
| n66 | 5 | 5 |
| n70 | 5 | 5 |
| n71 | 5 | 5 |
| n74 | 5 | 5 |
| n75 | 52 | N/A |
| n76 | 52 | N/A |
| n77 | 10 | 10 |
| n78 | 10 | 10 |
| n79 | 10 | 10 |
| n80 | 53 | [5] |
| n81 | 53 | [5] |
| n82 | 53 | [5] |
| n83 | 53 | [5] |
| n84 | 53 | [5] |
| n85 | 36 |  |
| n86 | 53 | [5] |
| n91 | 5 | [5] |
| n92 | 5 | [5] |
| n93 | 5 | [5] |
| n94 | 5 | [5] |
| n95 | 53 | [5] |
| n96 | 20 | N/A |
| n97 | 53 | [5] |
| n99 | 53 | [5] |
| n100 | 36,5 | 5 |
| n101 | 5 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.1 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as SCell in CA or SCell in DC configuration. Note 5: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 6: This UE channel bandwidth is optional in R18. |  |  |

-------- Skipped unchanged clauses --------

#### 4.3.1.0D	Bandwidth part

The value of locationAndBandwidth in BWP for FR1 is given in Table 4.3.1.0D-1. The value of locationAndBandwidth in BWP for FR2 is given in Table 4.3.1.0D-2.

Table 4.3.1.0D-1: locationAndBandwidth in BWP for FR1

| BW (MHz) | SCS (kHz) | LRBs (Note 2) | locationAndBandwidth (Note 1) |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 3850 |
| 5 | 15 | 25 | 6600 |
| 5 | 30 | 11 | 2750 |
| 5 | 60 | N/A | N/A |
| 10 | 15 | 52 | 14025 |
| 10 | 30 | 24 | 6325 |
| 10 | 60 | 11 | 2750 |
| 15 | 15 | 79 | 21450 |
| 15 | 30 | 38 | 10175 |
| 15 | 60 | 18 | 4675 |
| 20 | 15 | 106 | 28875 |
| 20 | 30 | 51 | 13750 |
| 20 | 60 | 24 | 6325 |
| 25 | 15 | 133 | 36300 |
| 25 | 30 | 65 | 17600 |
| 25 | 60 | 31 | 8250 |
| 30 | 15 | 160 | 32174 |
| 30 | 30 | 78 | 21175 |
| 30 | 60 | 38 | 10175 |
| 35 | 15 | 188 | 24474 |
| 35 | 30 | 92 | 25025 |
| 35 | 60 | 44 | 11825 |
| 40 | 15 | 216 | 16774 |
| 40 | 30 | 106 | 28875 |
| 40 | 60 | 51 | 13750 |
| 45 | 15 | 242 | 9624 |
| 45 | 30 | 119 | 32450 |
| 45 | 60 | 58 | 15675 |
| 50 | 15 | 270 | 1924 |
| 50 | 30 | 133 | 36300 |
| 50 | 60 | 65 | 17600 |
| 60 | 15 | N/A | N/A |
| 60 | 30 | 162 | 31624 |
| 60 | 60 | 79 | 21450 |
| 70 | 15 | N/A | N/A |
| 70 | 30 | 189 | 24199 |
| 70 | 60 | 93 | 25300 |
| 80 | 15 | N/A | N/A |
| 80 | 30 | 217 | 16499 |
| 80 | 60 | 107 | 29150 |
| 90 | 15 | N/A | N/A |
| 90 | 30 | 245 | 8799 |
| 90 | 60 | 121 | 33000 |
| 100 | 15 | N/A | N/A |
| 100 | 30 | 273 | 1099 |
| 100 | 60 | 135 | 36850 |
| Note 1: The value for locationAndBandwidth parameter is calculated as the RIV value in accordance to TS 38.214 [21] with = 275, =0 and for each bandwidth and subcarrier spacing. Note 2: = Max NRB is the default configuration, unless explicitly specified. |  |  |  |

-------- Skipped unchanged clauses --------

###### 4.3.1.1.1.26	Reference test frequencies for NR operating band n26

Table 4.3.1.1.1.26-1: Test frequencies for NR operating band n26 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 860.5 | 172100 | 859.15 | 171830 | 0 | 15 | 30938 | 172070 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 876.5 | 175300 | 856.79 | 171358 | 102 |  | 31017 | 175270 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 892.5 | 178500 | 800.43 | 160086 | 504 |  | 31099 | 178530 | 4 | 0 | 3 (2) | 506 |
|  |  | Uplink | Low | 815.5 | 163100 | 814.15 | 162830 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 739.43 | 147886 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 847.5 | 169500 | 845.07 | 169014 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 861.5 | 172300 | 859.25 | 171850 | 0 | 15 | 2153 | 172330 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 855.89 | 171178 | 102 |  | 2189 | 175210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 816.5 | 163300 | 814.25 | 162850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 738.53 | 147706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 864 | 172800 | 859.32 | 171864 | 0 | 15 | 2154 | 172350 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 853.46 | 170692 | 102 |  | 2183 | 174730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 819 | 163800 | 814.32 | 162864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 736.1 | 147220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 866.5 | 173300 | 859.39 | 171878 | 0 | 15 | 2155 | 172370 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 851.03 | 170206 | 102 |  | 2177 | 174250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 821.5 | 164300 | 814.39 | 162878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 733.67 | 146734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 869 | 173800 | 859.46 | 171892 | 0 | 15 | 2153 | 172330 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 876.5 | 175300 | 848.6 | 169720 | 102 |  | 2171 | 173770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 824 | 164800 | 814.46 | 162892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 731.24 | 146248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

###### 4.3.1.1.1.28	Reference test frequencies for NR operating band n28

Table 4.3.1.1.1.28-1: Test frequencies for NR operating band n28 and SCS 15 kHz when not used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 759.5 | 151900 | 758.15 | 151630 | 0 | 15 | 30432 | 151870 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 780.5 | 156100 | 760.79 | 152158 | 102 |  | 30537 | 156070 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 801.5 | 160300 | 709.43 | 141886 | 504 |  | 30642 | 160270 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 704.5 | 140900 | 703.15 | 140630 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 633.43 | 126686 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 746.5 | 149300 | 744.07 | 148814 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 759.89 | 151978 | 102 |  | 1949 | 156010 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 800.5 | 160100 | 707.53 | 141506 | 504 |  | 2002 | 160130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 757.46 | 151492 | 102 |  | 1943 | 155530 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 798 | 159600 | 702.6 | 140520 | 504 |  | 1989 | 159150 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 765.5 | 153100 | 758.39 | 151678 | 0 | 15 | 1901 | 152170 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 780.5 | 156100 | 755.03 | 151006 | 102 |  | 1937 | 155050 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 795.5 | 159100 | 697.67 | 139534 | 504 |  | 1976 | 158170 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 768 | 153600 | 758.46 | 151692 | 0 | 15 | 1902 | 152190 | 10 | 1 | 1 (2) | 3 |
| (Note 2) |  |  | Mid | 783 | 156600 | 755.1 | 151020 | 102 |  | 1938 | 155070 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 793 | 158600 | 692.74 | 138548 | 504 |  | 1966 | 157250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 627.74 | 125548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low Mid | 773 | 154600 | 758.6 | 151720 | 0 | 15 | 1901 | 152170 | 6 | 0 | 1 (2) | 2 |
| (Note 4) |  |  | High | 788 | 157600 | 682.88 | 136576 | 504 |  | 1940 | 155290 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low Mid | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

###### 4.3.1.1.1.85	 Reference test frequencies for NR operating band n85

Table 4.3.1.1.1.85-1: Test frequencies for NR operating band n85 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 729.5 | 145900 | 728.15 | 145630 | 0 | 15 | 30282 | 145870 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 737 | 147400 | 717.29 | 143458 | 102 |  | 30321 | 147430 | 4 | 0 | 3 (2) | 104 |
|  |  |  | High | 744.5 | 148900 | 652.43 | 130486 | 504 |  | 30357 | 148870 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 699.5 | 139900 | 698.15 | 139630 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 614.93 | 122986 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 714.5 | 142900 | 712.07 | 142414 | 6 |  | - | - | - | - | - | - |

-------- Skipped unchanged clauses --------

###### 4.3.1.1.1.100	Reference test frequencies for NR operating band n100

Table 4.3.1.1.1.100-1: Test frequencies for NR operating band n100 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 920.9 | 184180 | 919.55 | 183910 | 0 | 15 | 31239 | 184150 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 922.2 | 184440 | 902.49 | 180498 | 102 |  | 31246 | 184410 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 923.5 | 184700 | 831.43 | 166286 | 504 |  | 31253 | 184670 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 875.9 | 175180 | 874.55 | 174910 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 877.2 | 175440 | 785.13 | 157026 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 878.5 | 175700 | 876.07 | 175214 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 921.9 | 184380 | 919.65 | 183930 | 0 | 15 | 2304 | 184350 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 922.2 | 184440 | 901.59 | 180318 | 102 |  | 2304 | 184350 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 922.5 | 184500 | 829.53 | 165906 | 504 |  | 2307 | 184590 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 876.9 | 175380 | 874.65 | 174930 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 877.2 | 175440 | 784.23 | 156846 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 877.5 | 175500 | 874.17 | 174834 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- End of changes --------