3GPP TSG-RAN WG5 Meeting #106	R5-251392

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3415 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RRC IEs for R18 XR PDCP discard TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-1-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RRC IEs for R18 XR PDCP discard TCs are missing and need to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RRC IEs for R18 XR PDCP discard TCs have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 4.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.523-1 CR 4805, 4806, 4807 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.6.3	Radio resource control information elements

–	MAC-CellGroupConfig

Table 4.6.3-68: MAC-CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config | Not present |  |  |
| drx-Config CHOICE { |  |  | DRX |
| setup | DRX-Config |  |  |
| } |  |  |  |
| schedulingRequestConfig | SchedulingRequestConfig |  |  |
| bsr-Config | BSR-Config |  |  |
| tag-Config | TAG-Config |  |  |
| phr-Config CHOICE { |  |  |  |
| setup | PHR-Config |  |  |
| } |  |  |  |
| skipUplinkTxDynamic | false |  |  |
| intraCG-Prioritization-r17 | Not present |  |  |
| drx-ConfigSL-r17 | Not present |  |  |
| drx-ConfigExt-v1700 | Not present |  |  |
| schedulingRequestID-BFR-r17 | Not present |  |  |
| schedulingRequestID-BFR2-r17 | Not present |  |  |
| schedulingRequestConfig-v1700 | Not present |  |  |
| tar-Config-r17 | Not present |  |  |
| g-RNTI-ConfigToAddModList-r17 | Not present |  |  |
| g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 { | 1 entry |  | MBS_Multicast |
| MBS-RNTI-SpecificConfig-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-RNTI-SpecificConfigId-r17 | 0 |  |  |
| groupCommon-RNTI-r17 CHOICE { |  |  |  |
| g-RNTI | RNTI-Value |  |  |
| } |  |  |  |
| drx-ConfigPTM-r17 | Not present |  |  |
| drx-ConfigPTM-r17 CHOICE { |  |  | DRX_MBS_Multicast |
| setup | DRX-ConfigPTM |  |  |
| } |  |  |  |
| harq-FeedbackEnablerMulticast-r17 | Not present |  |  |
|  | dci-enabler |  | DCI |
|  | enabled |  | RRC_Enable_HARQFeedback |
| harq-FeedbackOptionMulticast-r17 | Not present |  |  |
|  | ack-nack |  | ACK_NACK |
|  | nack-only |  | NACK_ONLY |
| pdsch-AggregationFactor-r17 | Not present |  |  |
| } |  |  |  |
| g-RNTI-ConfigToReleaseList-r17 | Not present |  |  |
| g-CS-RNTI-ConfigToAddModList-r17 | Not present |  |  |
| g-CS-RNTI-ConfigToReleaseList-r17 | Not present |  |  |
| allowCSI-SRS-Tx-MulticastDRX-Active-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX | This condition applies when DRX is configured |
| MBS_Multicast | Used for MBS Multicast reception |
| DRX_MBS_Multicast | DRX is used for MBS Multicast reception test |
| DCI | HARQ feedback for MBS multicast is indicated by DCI |
| RRC_Enable_HARQFeedback | HARQ feedback for MBS multicast is enabled by RRC |
| ACK_NACK | ACK/NACK based HARQ-ACK feedback |
| NACK_ONLY | NACK-only based HARQ-ACK feedback |

<<< Unchanged Parts Skipped >>>

–	PDCP-Config

Table 4.6.3-99: PDCP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
|  | len12bits |  | pc_supportOfRedCap_r17, pc_supportOfERedCap_r18 |
|  | Not present |  | MRBm |
| pdcp-SN-Size-DL | len18bits |  |  |
|  | len12bits |  | pc_supportOfRedCap_r17, pc_supportOfERedCap_r18 |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true | AM is default |  |
|  | Not present |  | UM, UM_MRB |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| drb | Not present |  | SRB, Split_SRB |
| moreThanOneRLC | Not present |  |  |
| moreThanOneRLC SEQUENCE { |  |  | Split, Split_SRB, NR_split |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | CellGroupId |  |  |
|  | 0 | MCG path | Split_SRB, NR_split |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
|  | Not present |  | Split_SRB |
| pdcp-Duplication | false |  |  |
|  | Not present | one UL path | Split_SRB |
| } |  |  |  |
| t-Reordering | Not present |  |  |
|  | 80ms |  | UM, UM_MRB |
| survivalTimeStateSupport-r17 | Not present |  |  |
| uplinkDataCompression-r17 | Not present |  |  |
| uplinkDataCompression-r17 CHOICE { |  |  | UDC |
| setup CHOICE { |  |  |  |
| newSetup SEQUENCE { |  |  |  |
| bufferSize-r17 | kbyte2 |  |  |
| dictionary-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| discardTimerExt2-r17 | Not present |  |  |
| initialRX-DELIV-r17 | Not present |  |  |
|  | 0 |  | MRB_Initialization |
| pdu-SetDiscard-r18 | Not present |  |  |
|  | true |  | PDUSet_BasedDiscard |
| discardTimerForLowImportance-r18 | Not present |  |  |
| discardTimerForLowImportance-r18 CHOICE { |  |  | PSI_BasedDiscard |
| setup | ms100 |  |  |
| } |  |  |  |
| primaryPathOnIndirectPath-r18 | Not present |  |  |
| sn-GapReport-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Split | More than one RLC |
| SRB | SRB |
| UM | RLC UM DRB |
| Split_SRB | SRB with more than one RLC |
| NR_split | MCG and split for NR-DC. |
| UDC | RLC AM DRB with uplinkDataCompression |
| MRB_Initialization | Multicast MRB setup or PDCP re-establishment for multicast MRB |
| UM_MRB | UM multicast MRB |
| MRBm | Establishment of MRBm |
| PSI_BasedDiscard | For XR enh PSI based discard TCs |
| PDUSet_BasedDiscard | For XR enh PDU Set based discard TCs |

<< END OF CHANGES >>