3GPP TSG-RAN WG5 Meeting #107	R5-252274

St Julian’s, Malta, 19th-23nd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0192 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SLPP position capability transfer procedure test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of sidelink positioning in NR systems is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new test case 9.5.1.1 Position Capability Transfer between LMF and UE - Added new test case 9.5.1.2 Position Capability Transfer between UEs (Target UE is in coverage) - Added new test case 9.5.1.3 Position Capability Transfer between UEs (Target UE is out of coverage) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0175 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.5	SLPP Procedures

9.5.1	SLPP Common Procedures

9.5.1.1	Position Capability Transfer between LMF and UE

9.5.1.1.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {
  when { UE receives an SLPP message of type REQUEST CAPABILITIES }

then { UE sends an SLPP message of type PROVIDE CAPABILITIES with the correct supported capabilities }

}

9.5.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.355 clauses 5.1.2 and clause 5.1.5. Unless otherwise stated these are Rel-18 requirements.

[TS 38.355, clause 5.1.2]

The Capability Transfer procedure is shown in Figure 5.1.2-1.

Figure 5.1.2-1: SLPP Capability Transfer procedure

1.	Endpoint B sends a RequestCapabilities message to Endpoint A. Endpoint B may indicate the types of capability requested.

2.	Endpoint A responds with a ProvideCapabilities message to Endpoint B. The capabilities shall correspond to the capability types specified in step 1. This message shall include the field endTransaction set to TRUE.

[TS 38.355, clause 5.1.5]

Upon receiving a RequestCapabilities message, Endpoint A shall generate a ProvideCapabilities message as a response.

Endpoint A shall:

1>	for each positioning method for which a request for capabilities is included in the message:

2>	if Endpoint A supports this positioning method:

3>	include the capabilities of Endpoint A for that supported positioning method in the response message;

1>	set the field sessionID in the response message to the same value as the field sessionID in the received message if received;

1>	set the field transactionID in the response message to the same value as the field transactionID in the received message;

1>	deliver the response message to lower layers for transmission.

9.5.1.1.3	Test description

9.5.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

Related PICS/PIXIT Statements:

-

9.5.1.1.3.2	Test procedure sequence

Table 9.5.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an SLPP message of type Request Capabilities. | <-- | DLInformationTransfer (SLPP REQUEST CAPABILITIES) | - | - |
| 2 | The UE sends an SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (SLPP PROVIDE CAPABILITIES) | 1 | P |
| 3 | IF the UE SLPP message at step 2 includes an acknowledgment request THEN SS sends an SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |

9.5.1.1.3.3	Specific message contents

Table 9.5.1.1.3.3-1: DL InformationTransfer (steps 1 and 3, Table 9.5.1.1.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.1.1.3.3-2 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.1.1.3.3-2: DL NAS TRANSPORT (steps 1 and 3, Table 9.5.1.1.3.2-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1: Set according to Table 8.5-1 | SLPP Request Capabilities |  |
|  |  | Step 3: Set according to Table 9.5.1.1.3.3-3 | SLPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.5.1.1.3.3-3: SLPP Acknowledgement (step 3, Table 9.5.1.1.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in step 2, Table 9.5.1.1.3.2-1. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 9.5.1.1.3.3-4: ULInformationTransfer (step 2, Table 9.5.1.1.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.1.1.3.3-5 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.1.1.3.3-5: UL NAS TRANSPORT (step 2, Table 9.5.1.1.3.2-1)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Set according to Table 9.5.1.1.3.3-6 | SLPP Provide Capabilities |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 9.5.1.1.3.2-1) |  |

Table 9.5.1.1.3.3-6: SLPP Provide Capabilities (step 2, Table 9.5.1.1.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Application Layer ID in target UE side |  |  |
| transactionID | Present | Contains the same value of the transactionID field in step 1, Table 9.5.1.1.3.2-1. |  |
| endTransaction | True |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { | Present, or not present |  |  |
| ackRequested | True |  |  |
| ackIndicator | (0..255) |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |
| commonIEsProvideCapabilities | Not checked |  |  |
| commonSL-PRS-MethodsIEsProvideCapabilities | Present or not present dependent on UE capabilities. Set according to Table 9.5.1.1.3.3-7. | OCTET STRING containing CommonIEsRequestCapabilities |  |
| sl-AoA-ProvideCapabilities | Present or not present dependent on (pc_Target_UEB_SL_AoA OR pc_Target_UEA_SL_AoA) Set according to Table 9.5.1.1.3.3-8. | OCTET STRING containing CommonIEsRequestCapabilities |  |
| sl-RTT-ProvideCapabilities | Present or not present dependent on (pc_Target_UEB_SL_RTT OR pc_Target_UEA_SL_RTT) Set according to Table 9.5.1.1.3.3-9. | OCTET STRING containing CommonIEsRequestCapabilities |  |
| sl-TDOA-ProvideCapabilities | Present or not present dependent on (pc_Target_UEB_SL_TDOA OR pc_Target_UEA_SL_TDOA). Set according to Table 9.5.1.1.3.3-10. | OCTET STRING containing CommonIEsRequestCapabilities |  |
| sl-TOA-ProvideCapabilities | Present or not present dependent on (pc_Target_UEB_SL_TOA OR pc_Target_UEA_SL_TOA) Set according to Table 9.5.1.1.3.3-11. | OCTET STRING containing CommonIEsRequestCapabilities |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtensio | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.1.1.3.3-7: CommonSL-PRS-MethodsIEsProvideCapabilities (Table 9.5.1.1.3.3-6)

| Derivation Path: 38.355 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CommonSL-PRS-MethodsIEsProvideCapabilities ::= SEQUENCE { |  |  |  |
| sl-PRS-CommonProcCapabilityPerUE | Not checked |  |  |
| sl-PRS-CapabilityBandList SEQUENCE (SIZE (1..nrMaxBands)) OF SEQUENCE { |  | Size n of SEQUENCE is dependent on UE capabilities |  |
| freqBandIndicatorNR | Present. Any value acceptable. |  |  |
| sl-PRS-CommonProcCapabilityPerBand SEQUENCE { |  |  |  |
| maxSL-PRS-Bandwidth | Dependent on UE capabilities |  |  |
| maxNumOfActiveSL-PRS-ResourcesInOneSlot | Dependent on UE capabilities |  |  |
| maxNumOfSlotsWithActiveSL-PRS-Resources | Dependent on UE capabilities |  |  |
| minTimeAfterEndofSlotCarryActiveSL-PRS-Resources | Dependent on UE capabilities |  |  |
| } |  |  |  |
| sl-PositioningARP-LocationProvision | Not checked |  |  |
| dummy1 | Not checked |  |  |
| dummy2 | Not checked |  |  |
| sl-PRS-RxInSharedResourcePool | Not checked |  |  |
| sl-PRS-RxInDedicatedResourcePool | Not checked |  |  |
| sl-PRS-TxInSharedResourcePool | Not checked |  |  |
| sl-PRS-TxScheme1InDedicatedResourcePool | Not checked |  |  |
| sl-PRS-TxScheme2InDedicatedResourcePool | Not checked |  |  |
| sl-PRS-RSRP-Meas | Not checked |  |  |
| sl-PRS-RSRPP-Meas | Not checked |  |  |
| sl-PRS-TDM-Multiplexing | Not checked |  |  |
| sl-PRS-RxCombMultiplexing | Not checked |  |  |
| sl-PRS-AdditionalPathsReport | Not checked |  |  |
| sl-PRS-LOS-NLOS-Indication | Not checked |  |  |
| sl-PRS-RxForBandWithSL-CA | Not checked |  |  |
| sl-PRS-TxForBandWithSL-CA | Not checked |  |  |
| sl-PRS-ReportTxARP-ID | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.1.1.3.3-8: SL-AoA-ProvideCapabilities (Table 9.5.1.1.3.3-6)

| Derivation Path: 38.355 clause 6.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-AoA-ProvideCapabilities ::= SEQUENCE { |  |  |  |
| positioningModes | Dependent on UE capabilities |  |  |
| tenMsUnitResponseTime | Not checked |  |  |
| periodicalReporting | Not checked |  |  |
| scheduledLocationRequestSupported | Not checked |  |  |
| locationCoordinateTypes | Not checked |  |  |
| velocityTypes | Not checked |  |  |
| sl-AoA-CapabilityBandList SEQUENCE (SIZE (1..nrMaxBands)) SEQUENCE { |  | Size n of SEQUENCE is dependent on UE capabilities |  |
| sl-AoA-Meas | Dependent on UE capabilities |  |  |
| dummy | Not checked |  |  |
| measurementsForMultipleARP-IDs-Rx | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.1.1.3.3-9: SL-RTT-ProvideCapabilities (Table 9.5.1.1.3.3-6)

| Derivation Path: 38.355 clause 6.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-RTT-ProvideCapabilities ::= SEQUENCE { |  |  |  |
| positioningModes | Dependent on UE capabilities |  |  |
| tenMsUnitResponseTime | Not checked |  |  |
| periodicalReporting | Not checked |  |  |
| scheduledLocationRequestSupported | Not checked |  |  |
| locationCoordinateTypes | Not checked |  |  |
| velocityTypes | Not checked |  |  |
| sl-RTT-CapabilityBandList SEQUENCE (SIZE (1..nrMaxBands)) SEQUENCE { |  | Size n of SEQUENCE is dependent on UE capabilities |  |
| sl-PRS-RxTxTimeDiffWithoutTxTimeStamp | Dependent on UE capabilities |  |  |
| sl-PRS-RxTxTimeDiffWithTxTimeStamp SEQUENCE { | Present or not Present dependent on UE capabilities |  |  |
| numOfMeasForSameSL-PRS | Not checked |  |  |
| maxMeasReportingForDiffSL-PRS | Not checked |  |  |
| } |  |  |  |
| dummy | Not checked |  |  |
| measurementsForMultipleARP-IDs-Rx | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.1.1.3.3-10: SL-TDOA-ProvideCapabilities (Table 9.5.1.1.3.3-6)

| Derivation Path: 38.355 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TDOA-ProvideCapabilities ::= SEQUENCE { |  |  |  |
| positioningModes | Dependent on UE capabilities |  |  |
| tenMsUnitResponseTime | Not checked |  |  |
| periodicalReporting | Not checked |  |  |
| scheduledLocationRequestSupported | Not checked |  |  |
| locationCoordinateTypes | Not checked |  |  |
| velocityTypes | Not checked |  |  |
| sl-TDOA-CapabilityBandList SEQUENCE (SIZE (1..nrMaxBands)) SEQUENCE { |  | Size n of SEQUENCE is dependent on UE capabilities |  |
| sl-PRS-RSTD-Meas | Dependent on UE capabilities |  |  |
| dummy | Not checked |  |  |
| measurementsForMultipleARP-IDs-Rx | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.1.1.3.3-11: SL-TOA-ProvideCapabilities (Table 9.5.1.1.3.3-6)

| Derivation Path: 38.355 clause 6.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TOA-ProvideCapabilities ::= SEQUENCE { |  |  |  |
| positioningModes | Dependent on UE capabilities |  |  |
| tenMsUnitResponseTime | Not checked |  |  |
| periodicalReporting | Not checked |  |  |
| scheduledLocationRequestSupported | Not checked |  |  |
| locationCoordinateTypes | Not checked |  |  |
| velocityTypes | Not checked |  |  |
| sl-TOA-CapabilityBandList SEQUENCE (SIZE (1..nrMaxBands)) SEQUENCE { |  | Size n of SEQUENCE is dependent on UE capabilities |  |
| sl-RTOA-Meas | Dependent on UE capabilities |  |  |
| dummy | Not checked |  |  |
| measurementsForMultipleARP-IDs-Rx | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

9.5.1.2	Position Capability Transfer between UEs (Target UE is in coverage)

9.5.1.2.1	Test Purpose (TP)

(1)

with { Target UE is in coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when { UE receives an SLPP message of type REQUEST CAPABILITIES }

then { UE sends an SLPP message of type PROVIDE CAPABILITIES with the correct supported capabilities }

}

9.5.1.2.2	Conformance requirements

As defined in clause 9.5.1.1.2.

9.5.1.2.3	Test description

9.5.1.2.3.1	Pre-test conditions

System Simulator:

NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 3N-B RRC_CONNECTED_with_SL and Test Mode (On) with UE test loop mode E as defined in TS 38.508-1 [30], subclause 4.4A using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [30], subclause 4.5.1 using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

Related PICS/PIXIT Statements:

-

9.5.1.2.3.2	Test procedure sequence

Table 9.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The NR-SS-UE 1 sends an SLPP message of type Request Capabilities. | <-- | SLPP REQUEST CAPABILITIES | - | - |
| 2 | The UE sends an SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | SLPP PROVIDE CAPABILITIES | 1 | P |
| 3 | IF the UE SLPP message at step 2 includes an acknowledgment request THEN NR-SS-UE 1 sends an SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |

9.5.1.2.3.3	Specific message contents

Table 9.5.1.2.3.3-1: SLPP REQUEST CAPABILITIES (step 1, Table 9.5.1.2.3.2-1)

Derivation Path: Table 8.5-1 with UE-only mode condition.

Table 9.5.1.2.3.3-2: SLPP PROVIDE CAPABILITIES (step 2, Table 9.5.1.2.3.2-1)

| Derivation Path: Table 9.5.1.1.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 9.5.1.1.3.3-6 with the following exception: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 1, Table 9.5.1.2.3.2-1. |  |
| } |  |  |  |

Table 9.5.1.2.3.3-3: SLPP Acknowledgement (step 3, Table 9.5.1.2.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | '00 00 01 00'H | Application Layer ID in initiating UE side |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 1, Table 9.5.1.2.3.2-1. |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in step 2, Table 9.5.1.2.3.2-1. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

9.5.1.3	Position Capability Transfer between UEs (Target UE is out of coverage)

9.5.1.3.1	Test Purpose (TP)

(1)

with { Target UE is out of coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when { UE receives an SLPP message of type REQUEST CAPABILITIES }

then { UE sends an SLPP message of type PROVIDE CAPABILITIES with the correct supported capabilities }

}

9.5.1.3.2	Conformance requirements

As defined in clause 9.5.1.1.2.

9.5.1.3.3	Test description

9.5.1.3.3.1	Pre-test conditions

System Simulator:

NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 4-A and Test Mode (On), Test Loop Function (Off) as defined in TS 38.508-1 [30], Table 4.5.7.2-1 using generic procedure parameter Sidelink (On),Cast Type (Unicast), UE initiating unicast mode NR sidelink communication, GNSS Sync (On).

Related PICS/PIXIT Statements:

-

9.5.1.3.3.2	Test procedure sequence

As defined in clause 9.5.1.2.3.2.

9.5.1.3.3.3	Specific message contents

As defined in clause 9.5.1.2.3.3.