3GPP TSG-RAN5 Meeting #107	R5-252468

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0368 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test applicability for NR IMS data channel test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR IMS data channel test case 7.37/7.38/7.39/7.40/7.41/7.42 does not have test applicability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test applicability for NR IMS data channel test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.2	Applicabilities for test cases specified in TS 34.229-5

Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| 6 | Registration |  |  |  |
| 6.1 | Initial Registration / 5GS | Rel-15 | C01 | UE supports NR (see Note 1 below) |
| 6.2 | Initial Registration Failures / 5GS | Rel-15 | C01 | UE supports NR |
| 6.3 | Re-Registration Scenarios / 5GS | Rel-15 | C01 | UE supports NR |
| 6.4 | De-Registration Scenarios / 5GS | Rel-15 | C13 | UE supports NR and IMS de-registration |
| 6.5 | Void |  |  |  |
| 6.6 | Re-Registration after capability update / 5GS | Rel-15 | C15 | UE supports NR and Enabling/Disabling SMS over IMS |
| 6.7 | Authentication / MAC Parameter Invalid / Only two consecutive challenges / 5GS | Rel-15 | C01 | UE supports NR |
| 6.8 | Authentication / Security Server missing / SQN out of range / 5GS | Rel-15 | C01 | UE supports NR |
| 6.9 | Subscription / 503 Service Unavailable / 5GS | Rel-15 | C01 | UE supports NR |
| 6.10 | Initial Registration / IMS data channel / 5GS | Rel-18 | C62 | UE supports NR and IMS data channel |
| 7 | Call Control |  |  |  |
| 7.1 | MTSI MO Voice Call / 503 Service Unavailable / 5GS | Rel-15 | C28 | UE supports NR and IMS voice over NR and MTSI speech and preconditions and disabling preconditions and NG114 v1.0 |
| 7.2 | MTSI MO Voice Call / 504 Server Time-out / 5GS | Rel-15 | C28 | UE supports NR and IMS voice over NR and MTSI speech and preconditions and disabling preconditions and NG114 v1.0 |
| 7.3 | Void |  |  |  |
| 7.4 | Void |  |  |  |
| 7.4a | MTSI MO Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.5 | MTSI MO Voice call without preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C28 | UE supports NR and IMS voice over NR and MTSI speech and preconditions and disabling preconditions and NG114 v1.0 |
| 7.6 | Void |  |  |  |
| 7.6a | MTSI MT Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.7 | MTSI MT Voice Call without preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C28 | UE supports NR and IMS voice over NR and MTSI speech and preconditions and disabling preconditions and NG114 v1.0 |
| 7.8 | MTSI MT Voice Call without preconditions at originating UE and with preconditions at terminating UE / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.9 | MTSI MT Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 | C28 | UE supports NR and IMS voice over NR and MTSI speech and preconditions and disabling preconditions and NG114 v1.0 |
| 7.10 | MTSI MT Voice call without preconditions and without SDP offer in MT INVITE / 5GS | Rel-15 | C28 | UE supports NR and IMS voice over NR and MTSI speech and preconditions and disabling preconditions and NG114 v1.0 |
| 7.11 | MTSI MT Voice call without preconditions at terminating UE and originating UE requiring them / 5GS | Rel-15 | C05 | UE supports NR and IMS voice over NR and MTSI and MTSI speech and (not preconditions or (preconditions and disabling preconditions)) |
| 7.12 | MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.13 | MTSI MT Voice Call with RTCP disabled / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.14 | MTSI MO Video Call with preconditions at both originating and terminating UE / 5GS | Rel-15 | C61 | NR and IMS voice over NR and MTSI Speech and MTSI video and preconditions and NG.114 v1.0 NOTE 3 |
| 7.15 | MTSI MO Video call without preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C31 | UE supports NR and MTSI video and disabling preconditions and NG.114 |
| 7.16 | MTSI MT Video call with preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C61 | NR and IMS voice over NR and MTSI Speech and MTSI video and preconditions and NG.114 v1.0 NOTE 3 |
| 7.17 | MTSI MT Video call without preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C31 | UE supports NR and MTSI video and disabling preconditions and NG.114 |
| 7.18 | MTSI MO Voice Call / EVS / AMR-WB / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.19 | MTSI MT Voice Call / EVS / AMR-WB IO mode / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.20 | MTSI MO Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C61 | NR and IMS voice over NR and MTSI Speech and MTSI video and preconditions and NG.114 v1.0 NOTE 3 |
| 7.21 | MTSI MO Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C31 | UE supports NR and MTSI video and disabling preconditions and NG.114 |
| 7.22 | MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C61 | NR and IMS voice over NR and MTSI Speech and MTSI video and preconditions and NG.114 v1.0 NOTE 3 |
| 7.23 | MTSI MT Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS | Rel-15 | C31 | UE supports NR and MTSI video and disabling preconditions and NG.114 |
| 7.24 | MTSI MT Voice Call / Forking / UE receives CANCEL request for a forked MT voice call / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.24a | MTSI MO Voice Call / Forking / UE receives two preliminary responses and one early dialog termination / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.24b | MTSI MO Voice Call / UE receives two preliminary responses and one final response / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.25 | MTSI MT Voice call without SDP offer in INVITE / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.26 | Mobile Originating CAT / Forking Model / MO Voice Call / 5GS | Rel-15 | C17 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.27 | Session Timer / MO Voice Call / UE is able to refresh the session / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.28 | Session Timer / MO Voice Call / Remote end is refresher / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.29 | Session Timer / MO Voice Call / Remote end does not support Session Timer / 5GS | Rel-15 | C29 | NR and IMS voice over NR and MTSI Speech and preconditions and Session Timer for own benefit and NG.114 v1.0 |
| 7.30 | Session Timer / MO Voice Call / Remote end supports but does not use Session Timer / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.31 | Session Timer / MT Voice Call / Remote end supports but does not send Session-Expires / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.32 | Session Timer / MT Voice Call / Remote end sends Session-Expires but does not choose refresher / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.33 | Session Timer / MT Voice Call / Remote end chooses UE as refresher / 5GS | Rel-15 | C30 | UE supports NR and IMS voice over NR and MTSI speech and preconditions and Session Timer and re-INVITE for session refresh and NG114 v1.0 |
| 7.34 | Session Timer / MT Voice Call / Remote end does not support Session Timer / 5GS | Rel-15 | C44 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.37 | IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel failure / without preconditions at both originating UE and terminating UE | Rel-18 | Cxx | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and disabling preconditions and NG.114 v1.0 |
| 7.38 | IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel failure / with preconditions at both originating UE and terminating UE | Rel-18 | Cyy | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.39 | IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE | Rel-18 | Cxx | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and disabling preconditions and NG.114 v1.0 |
| 7.40 | IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE | Rel-18 | Cyy | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 7.41 | IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE | Rel-18 | Cxx | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and disabling preconditions and NG.114 v1.0 |
| 7.42 | IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE | Rel-18 | Cyy | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |

<Skip unchanged part>

Table 4.2a: Applicability Conditions for TS 34.229-5 tests

| Condition | Test Case Selection Expression | Comment |
| --- | --- | --- |
| C01 | IF A.18/5 THEN R ELSE N/A | NR |
| C02 | Void |  |
| C03 | Void |  |
| C04 | Void |  |
| C05 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND ( NOT A.4/16 OR (A.4/16 AND A.12/59)) THEN R ELSE N/A | NR and IMS voice over NR and MTSI and MTSI speech and (not preconditions or (preconditions and disabling preconditions)) |
| C06 | IF A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/1 THEN R ELSE N/A | NR and (GBA or HTTP Digest) and MTSI and MTSI Originating Identification Presentation |
| C07 | IF A.18/5 AND A.3A/50 AND A.16/7 AND A.6a/3 THEN R ELSE N/A | NR and MTSI and MTSI Communication Barring and GBA for XCAP authentication |
| C08 | IF A.18/5 AND A.3A/61 AND [73] A.4.4-2/32 THEN R ELSE N/A | NR and SM-over-IP sender and UE configured to use SMS over IP |
| C09 | IF A.18/5 AND A.3A/62 AND [73] A.4.4-2/32 THEN R ELSE N/A | NR and SM-over-IP receiver and UE configured to use SMS over IP |
| C10 | IF A.18/5 AND A.3A/64 AND [73] A.4.4-2/32 THEN R ELSE N/A | NR and concatenated SM-over-IP sender and UE configured to use SMS over IP |
| C11 | IF A.18/5 AND A.3A/65 AND [73] A.4.4-2/32 THEN R ELSE N/A | NR and concatenated SM-over-IP receiver and UE configured to use SMS over IP |
| C12 | IF A.18/5 AND [93] A.4.3.7-1/14 AND A.12/27 THEN R ELSE N/A | NR and emergency services in NR connected to 5GCN and is capable of obtaining location information |
| C13 | IF A.18/5 AND A.12/39 THEN R ELSE N/A | NR and IMS de-registration |
| C14 | Void |  |
| C15 | IF A.18/5 AND A.12/58 THEN R ELSE N/A | NR and Enabling/Disabling SMS over IMS |
| C16 | Void |  |
| C17 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.12/45 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI Speech and preconditions and early media and NG.114 v1.0 |
| C18 | Void |  |
| C19 | Void |  |
| C20 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.16/6 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and MTSI Communication Hold and preconditions and NG114 v1.0 |
| C21 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.16/6 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR MTSI speech and MTSI Communication Hold and preconditions and NG114 v1.0 |
| C22 | IF A.18/5 AND A.16/8 THEN R ELSE N/A | NR and MTSI Message Waiting Indication |
| C23 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.16/9 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and MTSI Conference and preconditions and NG114 v1.0 |
| C24 | IF A.18/5 AND A.21/3 THEN R ELSE N/A | NR and NG.114 |
| C25 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.16/11 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI and MTSI speech and MTSI Explicit Communication Transfer - consultative transfer and preconditions and NG114 v1.0 |
| C26 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.16/13 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI and MTSI speech and MTSI Communication Waiting and preconditions and NG114 v1.0 |
| C27 | Void |  |
| C28 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.12/59 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and preconditions and disabling preconditions and NG114 v1.0 |
| C29 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.12/60 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI Speech and preconditions and Session Timer for own benefit and NG.114 v1.0 |
| C30 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.12/57 AND A.12/61 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and preconditions and Session Timer and re-INVITE for session refresh and NG114 v1.0 |
| C31 | IF A.18/5 AND A.15/3 AND A.12/59 AND A.21/3 THEN R ELSE N/A | NR and MTSI video and disabling preconditions and NG.114 |
| C32 | Void |  |
| C33 | Void |  |
| C34 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.15/3 AND A.16/6 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and MTSI video and MTSI Communication Hold and preconditions and NG114 v1.0 |
| C35 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.15/3 AND A.16/9 AND A.16/14 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and MTSI video and MTSI Conference and MTSI three way session and preconditions and NG114 v1.0 |
| C36 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.15/3 AND A.16/9 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and MTSI video and MTSI Conference and preconditions and NG114 v1.0 |
| C37 | IF A.18/5 AND A.3A/50 AND A.15/1 AND A.16/19 AND A.4/2B THEN R ELSE N/A | NR and MTSI and MTSI speech and USSI and initiating a session |
| C38 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.16/5 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and MTSI Communication Diversion and preconditions and NG114 v1.0 |
| C39 | IF A.18/5 AND A.3A/50 AND A.15/1 AND [93] A.4.3.7-1/14 THEN R ELSE N/A | NR and MTSI and MTSI speech and emergency services in NR connected to 5GCN |
| C40 | IF A.18/5 AND A.3A/50 AND A.15/1 AND [93] A.4.3.7-1/14 AND A.4/16 AND A.12/33 THEN R ELSE N/A | NR and MTSI and MTSI speech and emergency services in NR connected to 5GCN and preconditions and Communication Hold during emergency call |
| C41 | IF A.18/5 AND A.3A/50 AND A.15/1 AND A.4/16 AND [93] A.4.3.7-1/14 THEN R ELSE N/A | NR and MTSI and MTSI speech and preconditions and emergency services in NR connected to 5GCN |
| C42 | IF A.3A/50 AND A.15/1 AND A.12/63 AND [3] A.10/16 THEN R ELSE N/A | MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and Manual type of eCall |
| C43 | IF A.3A/50 AND A.15/1 AND A.12/63 AND [3] A.10/17 THEN R ELSE N/A | MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and Automatic type of eCall |
| C44 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and preconditions and NG.114 v1.0 |
| C45 | IF A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/2A THEN R ELSE N/A | NR and (GBA or HTTP Digest) and MTSI and Originating Identification Restriction - Configuration |
| C46 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.16/2 AND A.15/1 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI Originating Identification Restriction and MTSI speech and preconditions and NG114 v1.0 |
| C47 | IF A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/3 THEN R ELSE N/A | NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Presentation |
| C48 | IF A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/4A THEN R ELSE N/A | NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Restriction - Configuration |
| C49 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.16/4 AND A.15/1 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and Terminating Identification Restriction and MTSI speech and preconditions and NG114 v1.0 |
| C50 | IF A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/5 THEN R ELSE N/A | NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |
| C51 | IF A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/7 THEN R ELSE N/A | NR and (GBA or HTTP Digest) and MTSI and Incoming Communication Barring |
| C52 | IF A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/15 THEN R ELSE N/A | NR and (GBA or HTTP Digest) and MTSI and Outgoing Communication Barring |
| C53 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.16/5 AND A.15/1 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NRand Communication Diversion and MTSI speech and preconditions and NG114 v1.0 |
| C54 | Void |  |
| C55 | IF A.18/5 AND A.6a/3 AND A.16/1 THEN R ELSE N/A | NR and GBA and MTSI Originating Identification Presentation |
| C56 | IF A.18/5 AND A.6a/4 AND A.16/1 THEN R ELSE N/A | NR and Digest and MTSI Originating Identification Presentation |
| C57 | Void |  |
| C58 | Void |  |
| C59 | Void |  |
| C60 | Void |  |
| C61 | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.15/3 AND A.4/16 AND A.21/1 THEN R ELSE N/A | NR and IMS voice over NR and MTSI speech and MTSI video and preconditions and NG.114 v1.0 |
| C62 | IF A.18/5 AND A.15/14 THEN R ELSE N/A | NR and IMS data channel |
| C63 | IF A.18/5 AND A.3A/50 AND A.15/1 AND A.15/15 AND [93] A.4.3.7-1/15 THEN R ELSE N/A | NR and MTSI and MTSI speech and emergency services in NR connected to 5GCN and real-time text |
| C64 | IF A.3A/50 AND A.15/1 AND A.12/63 AND A.12/68 AND [3] A.10/16 THEN R ELSE N/A | MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and UE subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over 5GS and Manual type of eCall (NOTE 2) |
| C65 | IF A.3A/50 AND A.15/1 AND A.12/63 AND A.12/68 AND [3] A.10/17 THEN R ELSE N/A | MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and UE subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over 5GS and Automatic type of eCall (NOTE 2) |
| C66 | IF A.3A/50 AND A.15/1 AND A.12/63 AND A.12/68 AND [3] A.10/19 THEN R ELSE N/A | UE supports and MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and UE subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over 5GS IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall (NOTE 2) |
| Cxx | IF A.18/5 AND A.15/14 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.12/59 AND A.21/1 THEN R ELSE N/A | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and disabling preconditions and NG.114 v1.0 |
| Cyy | IF A.18/5 AND A.15/14 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.21/1 THEN R ELSE N/A | UE supports NR and IMS data channel and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |

<End of modified section 1>