3GPP TSG-RAN5 Meeting #107	R5-252542

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4998 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.2.17 Sidelink Relay / L2 U2N / Relay UE / SRAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 12.3.1.2.17 Sidelink Relay / L2 U2N / Relay UE / SRAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 12.3.1.2.17 Sidelink Relay / L2 U2N / Relay UE / SRAP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.2.17 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0589 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.2.17	Sidelink Relay / L2 U2N / Relay UE / SRAP

12.3.1.2.17.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED and acting as a L2 U2N Relay UE }

ensure that {

when { UE receives RRCReconfiguration message containing sl-L2RelayUE-Config set to setup with the sl-RemoteUE-ToAddModList including sl-SRAP-ConfigRelay with SRB and sl-EgressRLC-ChannelUu-r17 and sl-EgressRLC-ChannelPC5 in sl-MappingToAddModList }

then { UE applies the configuration in accordance with the sl-SRAP-ConfigRelay for Uu and PC5 Relay RLC channel of the SRB }

}

(2)

with { UE in RRC_CONNECTED and acting as a L2 U2N Relay UE }

ensure that {

when { UE receives RRCReconfiguration message containing sl-L2RelayUE-Config set to setup with the sl-RemoteUE-ToAddModList including sl-SRAP-ConfigRelay with DRB and sl-EgressRLC-ChannelUu-r17 and sl-EgressRLC-ChannelPC5 in sl-MappingToAddModList }

then { UE applies the configuration in accordance with the sl-SRAP-ConfigRelay for Uu and PC5 Relay RLC channel of the DRB }

}

12.3.1.2.17.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.5.15.3, TS 38.351 clause 4.2.2 and 6.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.15.3]

The L2 U2N Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

3>	release SL-RLC1, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

3>	if SL-RLC1 is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-ConfigRelay;

[TS 38.351, clause 4.2.2]

On the U2N Relay UE, the SRAP sublayer contains one SRAP entity at Uu interface and a separate collocated SRAP entity at the PC5 interface. …

Each SRAP entity has a transmitting part and a receiving part. Across the PC5 interface in the U2N case, the transmitting part of the SRAP entity at the U2N Remote UE has a corresponding receiving part of an SRAP entity at the U2N Relay UE, and vice versa. Across the Uu interface, the transmitting part of the SRAP entity at the U2N Relay UE has a corresponding receiving part of an SRAP entity at the gNB, and vice versa.

…

Figure 4.2.2-2 and Figure 4.2.2-3 represents the functional view of the SRAP entity in U2N Relay case for the SRAP sublayer at PC5 interface and at Uu interface respectively. Figure 4.2.2-4 represents the functional view of the SRAP entity in U2U Relay case for the SRAP sublayer at PC5 interface.

Figure 4.2.2-2: Example of functional view of SRAP sublayer at PC5 interface in U2N Relay case

Figure 4.2.2-3: Example of functional view of SRAP sublayer at Uu interface

In the example of Figure 4.2.2-2 and Figure 4.2.2-3, at relay UE:

-	For data packet not corresponding to SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the receiving part on the SRAP entity of PC5 interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of Uu interface. As an alternative, the receiving part may deliver SRAP SDUs to the transmitting part on the collocated SRAP entity. When passing SRAP SDUs, the receiving part removes the SRAP header and the transmitting part of the relay UE adds the SRAP header with the same SRAP header content as carried on the SRAP Data PDU header prior to removal. Passing SRAP SDUs in this manner is therefore functionally equivalent to passing SRAP Data PDUs, in implementation. The following specification therefore refers to the passing of SRAP data packets.

-	For UL data packet corresponding to SRB0, the receiving part on the SRAP entity of PC5 interface delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of Uu interface, and the transmitting part on the SRAP entity of Uu interface adds the SRAP header in accordance with clause 5.3.3.

-	For DL data packet corresponding to SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the transmitting part on the SRAP entity of PC5 interface removes the SRAP header in accordance with clause 5.2.2. As an alternative for handling DL data packet corresponding to SRB0 not shown in Figure 4.2.2-2 or Figure 4.2.2-3, the receiving part on the SRAP entity of Uu interface removes the SRAP header and delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of PC5 interface.

[TS 38.331, clause 6.2.2]

Figure 6.2.2-1 shows the format of the U2N SRAP Data PDU with SRAP header being configured. This SRAP Data PDU format is applicable to U2N SRAP SDU except those for SRB0 delivered over PC5 interface.

Figure 6.2.2-1: U2N SRAP Data PDU format with SRAP header

Figure 6.2.2-2 shows the format of the U2N SRAP Data PDU consisting only of a data field without any SRAP header. This SRAP Data PDU format is applicable to U2N SRAP SDU for SRB0 delivered over PC5 interface.

Figure 6.2.2-2: U2N SRAP Data PDU format without SRAP header

12.3.1.2.17.3	Test Description

12.3.1.2.17.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1.

-	System information combination NR-31 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used.

-	NR-SS-UE

-	NR-SS-UE1 operating as L2 U2N Remote UE.

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using condition Announcing_Restricted_Model_A as per TS 38.508-1 [4] clause 4.7C.1.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

12.3.1.2.17.3.2	Test procedure sequence

Table 12.3.1.2.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-25 | Steps 1 to 25 of the procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side in Table 4.9.41.2.2-1 in TS 38.508-1[4] is performed. | - | - | - | - |
| 26 | The SS-NW transmits an RRCReconfiguration message to configure the Uu Relay RLC channels and PC5 Relay RLC channels and bearer mapping for relaying of U2N Remote UE’s SRB2/DRB1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 27 | The UE transmits an RRCReconfigurationSidelink message to NR-SS-UE1 to indicate PC5 Relay RLC channel addition? | --> | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 28 | NR-SS-UE1 transmits an RRCReconfigurationCompleteSidelink message. | <-- | PC5 RRC: RRCReconfigurationCompleteSidelink | - | - |
| 29 | Check: Does the UE relay an RRCReconfigurationComplete message to SS-NW? | --> | NR RRC: RRCReconfigurationComplete | 1,2 | P |
| 30 | The SS-NW transmits an ACTIVATE TEST MODE message on SRB2 to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: DLInformationTransfer TC: ACTIVATE TEST MODE | - | - |
| 31 | The UE relays the ACTIVATE TEST MODE message received at step 30 to NR-SS-UE1. | --> | PC5 Relay RLC Channel: DLInformationTransfer TC: ACTIVATE TEST MODE | - | - |
| 32 | The NR-SS-UE1 transmits an ACTIVATE TEST MODE COMPLETE message on SRB2 via Relay UE. | <-- | PC5 Relay RLC Channel: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE | - | - |
| 33 | Check: Does the UE relay the ACTIVATE TEST MODE COMPLETE message received at step 32 to SS-NW? | --> | Uu Relay RLC Channel: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE | 1 | P |
| 34 | The SS-NW transmits one IP Packet on DRB1 to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: IP Packet | - | - |
| 35 | The UE relays the IP Packet received at step 34 to NR-SS-UE1. | --> | PC5 Relay RLC Channel: IP Packet | - | - |
| 36 | NR-SS-UE1 transmits one IP Packet on DRB1. | <-- | PC5 Relay RLC Channel: IP Packet | - | - |
| 37 | Check: Does the UE relay the IP packet received in step 36 to SS-NW? | --> | PC5 Relay RLC Channel: IP Packet | 2 | P |

12.3.1.2.17.3.3	Specific message contents

Table 12.3.1.2.17.3.3-1: RRCReconfiguration (Step 26, Table 12.3.1.2.17.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition L2RelayUE and RELAYuuRelaySRB2_DRB1 and PC5RelaySRB2_DRB1

Table 12.3.1.2.17.3.3-2: RRCReconfigurationSidelink (Step 27, Table 12.3.1.2.17.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition TX and L2RelayUE

Table 12.3.1.2.17.3.3-3: RRCReconfigurationCompleteSidelink (Step 28, Table 12.3.1.2.17.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-4 with condition RX

<End of Changed Section>