3GPP TSG-RAN5 Meeting #107	R5-253053

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5449 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of ACB per PLMN test case 8.1.2.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Implementing FPLMN USIM will impose additional SIM requirements and same functionality as adding 002-11 to FPLMN list can be achieved by modifying cellReservedForOperatorUse IE in SIB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Removed SIM configuration requirement and added cellReservedForOperatorUse IE in specific message contents. 2. ASN.1 corrections in Table 8.1.2.5a.3.3-1. 3. Incorrect clause numbering corrected. 4. Paging with system information modification cannot be sent on Cell 11 at step 1 as the cell is off. Main behavior table at steps 1, 2 and Note is updated. Accordingly corrected specific message content tabes. 5. To transmit IP packet at step 9, radio bearers shall to be established. Added steps 8A-8C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-253053 is the final version of R5-252580. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.1.2.5a	RRC connection establishment / 0% access probability for MO data, no restriction for MO signalling / AC-Barring per PLMN

8.1.2.5a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE reselects a new cell which belongs to different TA and broadcasts a SystemInformationBlockType2 message including  an ac-BarringForMO-Data-r12 set to 0% accessibility and indicating no restriction for MO signalling }

then { UE transmits an RRCConnectionRequest message }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received a SystemInformationBlockType2 message including  an ac-BarringForMO-Data-r12 set to 0% accessibility }

ensure that {
  when { UE has user data pending }

then { UE does not transmit any RRCConnectionRequest message }

}

(3)

with { UE in E-UTRA RRC_IDLE state having received a SysteminformationBlockType2 message including an ac-BarringForMO-Data-r12 set to 0% accessibility and indicating no restriction for MO signalling and having failed to initiate a MO data }

ensure that {
  when { ac-BarringForMO-Data-r12 is removed and UE is requested to send one IP packet after ac-BarringTime expired}

then { UE transmits an RRCConnectionRequest message }

}

8.1.2.5a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.2. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

...

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

3>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls is applicable, upon which the procedure ends;

3>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4>	if timer T306 is not running, start T306 with the timer value of T303;

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

...

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

1>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

8.1.2.5a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11 are in different TA.

-	Cell 1 and Cell 11 as specified in TS 36.508[18] clause 4.4.1.2 are configured as shown in Table 8.1.2.5a.3.1–1.

Table 8.1.2.5a.3.1-1: PLMN identifiers

| Cell | PLMN names | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN1 | 001 | 01 |
|  | PLMN2 | 002 | 11 |
| 11 | PLMN1 | 001 | 01 |
|  | PLMN2 | 002 | 11 |

UE:

-	None.

Preamble:

-	The UE is in Idle mode (state 2) according to TS 36.508[18] after the UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 (serving cell) according to [18].

8.1.2.5a.3.2	Test procedure sequence

Table 8.1.2.5a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 8.1.2.5a.3.2-2.

Table 8.1.2.5a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |

Table 8.1.2.5a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures SystemInformationBlockType2 parameters to 0% access probability for MO Data . | - | - | - | - |
| 2 | Void |  |  |  |  |
| 3 | The SS changes Cell 1 and Cell 11 level according to the row "T1" in table 8.1.2.5a.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 1 | P |
| 5-8A | Steps 2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 11. Note: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 8B | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 8C | Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 11. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 9 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. | - | - | - | - |
| 10 | Wait for 1 s after the IP packet has been transmitted in step 9. (Note 3) | - | - | - | - |
| 11 | The SS transmits an RRCConnectionRelease message on Cell 11. | <-- | RRCConnectionRelease | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 30s? | --> | RRCConnectionRequest | 2 | F |
| 13 | SS changes SIB2 according to TS 36.508, table 4.4.3.3-1 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased. (Note 1) | <-- | Paging | - | - |
| 14 | Wait for 15 s (Note 2) to allow the new system information to take effect. | - | - | - | - |
| 15 | Wait for 167 s (Note 4) to make the Tbarring expire. | - | - | - | - |
| 16 | Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 11. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 17 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. | - | - | - | - |
| 18 | Wait for 1 s after the IP packet has been transmitted in step 17. (Note 3) | - | - | - | - |
| 19 | The SS transmits an RRCConnectionRelease message on Cell 11. | <-- | RRCConnectionRelease | - | - |
| 20 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 30s? | --> | RRCConnectionRequest | 3 | P |
| 21-28 | Steps 3 to 10 generic test procedure in TS 36.508 subclause 6.4.2.2 are performed. | - | - | - | - |
| 29 | The UE loop backs the IP packet received in step 18 on the DRB associated with the default EPS bearer context on Cell 11. | - | - | - | - |
| Note 1: To guarantee that the UE will receive at least one Paging in the Modification Period preceding the SysInfo change, SS should send the Paging message in every eligible PO in this Modification Period. Note 2: The wait time of 15 s in step 14 is to allow for the network to page the system information change during the next modification period and update the system information at the subsequent modification period. UE should acquire the updated system information within 100ms of the start of modification period. Note 3: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 9 and 17 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 10 and 18. Note 4: The UE starts ac-BarringTime in step14. Maximum time is 166.4 sec ((0.7 + 0.6 * 1) * s128). Therefore about 167 sec is enough to wait timer expiry. |  |  |  |  |  |

8.1.2.5a.3.3	Specific message contents

Table 8.1.2.5a.3.3-1: SystemInformationBlockType1 for Cell 1 and 11 (preamble and all steps, Table 8.1.2.5a.3.2-2)

| Derivation path: TS 36.508 [18], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity | 00101 | PLMN 1 |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity | 00211 | PLMN 2 |  |
| cellReservedForOperatorUse | reserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.5a.3.3-2: CLOSE UE TEST LOOP (preamble, Table 8.1.2.5a.3.2-2)

| Derivation Path: TS 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0101'B | 5 seconds |  |

Table 8.1.2.5a.3.3-3: Paging (step 13, Table 8.1.2.5a.3.2-2)

| Derivation Path: TS 36.508[18], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

Table 8.1.2.5a.3.3-4: Void

Table 8.1.2.5a.3.3-5: SystemInformationBlockType2 for Cell 11 (step1, Table 8.1.2.5a.3.2-2)

| Derivation Path: TS 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { | 1 entry |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| ac-BarringInfo-r12 SEQUENCE { |  |  |  |
| ac-BarringForEmergency-r12 | FALSE |  |  |
| ac-BarringForMO-Signalling-r12 | Not present |  |  |
| ac-BarringForMO-Data-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s128 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.5a.3.3-6: RRCConnectionRequest (step 4, Table 8.1.2.5a.3.2-2)

| Derivation Path: TS 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.5a.3.3-7: SystemInformationBlockType1 for Cell 11 (step 13, Table 8.1.2.5a.3.2-2)

| Derivation Path: TS 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 8.1.2.5a.3.3-8: RRCConnectionRequest (step 20, Table 8.1.2.5a.3.2-2)

| Derivation Path: TS 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>