3GPP TSG-RAN5 Meeting #107	R5-253096

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4996 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.1.10 Sidelink Relay / L2 U2N / Remote UE / SRAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 12.3.1.1.10 Sidelink Relay / L2 U2N / Remote UE / SRAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 12.3.1.1.10 Sidelink Relay / L2 U2N / Remote UE / SRAP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.1.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0589 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.1.10	Sidelink Relay / L2 U2N / Remote UE / SRAP

12.3.1.1.10.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE and acting as a L2 U2N Remote UE and has sent an RRCSetupRequest message to the connected L2 U2N Relay UE }

ensure that {

when { UE receives RRCSetup message containing sl-L2RemoteUE-Config with the sl-SRAP-ConfigRemote including SRB1 and sl-EgressRLC-ChannelPC5 in sl-MappingToAddModList }

then { UE applies the sl-EgressRLC-ChannelPC5 for PC5 Relay RLC channel of SRB1 }

}

(2)

with { UE in RRC_CONNECTED and acting as a L2 U2N Remote UE }

ensure that {

when { UE receives RRCReconfiguration message containing sl-L2RemoteUE-Config set to setup with the sl-SRAP-ConfigRemote including SRB2 and sl-EgressRLC-ChannelPC5 in sl-MappingToAddModList }

then { UE applies the sl-EgressRLC-ChannelPC5 for PC5 Relay RLC channel of SRB2 }

}

(3)

with { UE in RRC_CONNECTED and acting as a L2 U2N Remote UE }

ensure that {

when { UE receives RRCReconfiguration message containing sl-L2RemoteUE-Config set to setup with the sl-SRAP-ConfigRemote including DRB and sl-EgressRLC-ChannelPC5 in sl-MappingToAddModList }

then { UE applies the sl-EgressRLC-ChannelPC5 for PC5 Relay RLC channel of DRB }

}

12.3.1.1.10.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.5.16, TS 38.351 clause 4.2.2, 6.2.2, 5.2.3, 5.3.1, 5.3.1.1 and 5.3.1.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.16]

The network configures the L2 U2N or U2U Remote UE with relay operation related configurations, e.g. SRAP configuration.

The L2 U2N Remote UE shall:

1>	if sl-L2RemoteUE-Config is set to setup or received from RRCSetup message:

2>	if the sl-L2RemoteUE-Config contains the sl-SRAP-ConfigRemote:

3>	if no SRAP entity has been established:

4>	establish a SRAP entity as specified in TS 38.351 [66];

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRemote;

3>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

4>	release SL-RLC1, if established;

4> associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

3>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

4>	if SL-RLC1 is not established:

5>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

2>	if the sl-L2RemoteUE-Config contains the sl-UEIdentityRemote:

3>	use the value of the sl-UEIdentityRemote as the C-RNTI in the PCell.

1>	else if sl-L2RemoteUE-Config is set to release:

2>	release the L2 U2N relay operation related configurations.

[TS 38.351, clause 4.2.2]

On the U2N Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface. …

Each SRAP entity has a transmitting part and a receiving part. Across the PC5 interface in the U2N case, the transmitting part of the SRAP entity at the U2N Remote UE has a corresponding receiving part of an SRAP entity at the U2N Relay UE, and vice versa. Across the Uu interface, the transmitting part of the SRAP entity at the U2N Relay UE has a corresponding receiving part of an SRAP entity at the gNB, and vice versa.

…

Figure 4.2.2-2 and Figure 4.2.2-3 represents the functional view of the SRAP entity in U2N Relay case for the SRAP sublayer at PC5 interface and at Uu interface respectively. Figure 4.2.2-4 represents the functional view of the SRAP entity in U2U Relay case for the SRAP sublayer at PC5 interface.

Figure 4.2.2-2: Example of functional view of SRAP sublayer at PC5 interface in U2N Relay case

[TS 38.331, clause 6.2.2]

Figure 6.2.2-1 shows the format of the U2N SRAP Data PDU with SRAP header being configured. This SRAP Data PDU format is applicable to U2N SRAP SDU except those for SRB0 delivered over PC5 interface.

Figure 6.2.2-1: U2N SRAP Data PDU format with SRAP header

Figure 6.2.2-2 shows the format of the U2N SRAP Data PDU consisting only of a data field without any SRAP header. This SRAP Data PDU format is applicable to U2N SRAP SDU for SRB0 delivered over PC5 interface.

Figure 6.2.2-2: U2N SRAP Data PDU format without SRAP header

[TS 38.351, clause 5.2.3]

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	if the SRAP Data PDU is not for SRB0 (not received from SL-RLC0 as specified in TS 38.331 [3]):

-	if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to PDCP entity of SRB1 by ignoring the UE ID field and BEARER ID field of this SRAP Data PDU;

-	else:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer entity corresponding to the BEARER ID field of this SRAP Data PDU (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1);

-	else:

-	deliver the SRAP SDU (i.e., same as SRAP PDU for SRB0) to upper layer, i.e., RRC layer entity (TS 38.331 [3]).

[TS 38.351, clause 5.3.1]

The transmitting part of the SRAP entity on the PC5 interface of U2N Remote UE can receive SRAP SDU from upper layer and constructs SRAP Data PDU.

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	if the SRAP SDU is not for SRB0:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.1.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	else:

-	Construct an SRAP Data PDU without SRAP header in accordance with clause 6.2.2.

-	Determine the egress RLC channel in accordance with clause 5.3.1.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

[TS 38.351, clause 5.3.1.1]

For an SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID field corresponding to sl-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID field corresponding to SRB identity for SRB (i.e., set the BEARER ID field to srb-Identity), or corresponding to DRB identity minus 1 for DRB (i.e., set the BEARER ID field to drb-Identity minus 1), from which the SRAP SDU is received, configured as specified in TS 38.331 [3].

[TS 38.351, clause 5.3.1.2]

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the SRAP Data PDU is for SRB0:

-	Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3].

-	else if the SRAP Data PDU is for SRB1 and if there is not an entry in sl-MappingToAddModList, whose sl-RemoteUE-RB-Identity matches the SRB identity of the SRAP Data PDU, or if there is an entry in sl-MappingToAddModList without the corresponding sl-EgressRLC-ChannelPC5:

-	Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3].

-	else if there is an entry in sl-MappingToAddModList, whose sl-RemoteUE-RB-Identity matches the SRB identity or DRB identity of the SRAP Data PDU:

-	Determine the egress PC5 Relay RLC channel of the link with U2N Relay UE corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

12.3.1.1.10.3	Test Description

12.3.1.1.10.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 operating as L2 U2N Relay UE.

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using parameter 5G ProSe direct discovery model A as per TS 38.508-1 [4] clause 4.7C.1.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 0-A as defined in TS 38.508-1 [4].

12.3.1.1.10.3.2	Test procedure sequence

Table 12.3.1.1.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-7 | Steps 1 to 6 of the procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side in TS 38.508-1 Table 4.9.42.2.2-1 using parameter Cast Type (Unicast) and NR-SS-UE1 initiated unicast mode ProSe Direct communication are executed. | - | - | - | - |
| 8 | The UE transmits an RRCSetupRequest message. | --> | PC5 Relay RLC Channel: RRCSetupRequest | - | - |
| 9 | The NR-SS-UE1 is configured with sl-EgressRLC-ChannelPC5 for SRB1 and transmits an RRCSetup message containing sl-L2RemoteUE-Config with the sl-SRAP-ConfigRemote including SRB1 and sl-EgressRLC-ChannelPC5 in sl-MappingToAddModList. | <-- | PC5 Relay RLC Channel: RRCSetup | - | - |
| - | EXCEPTION: Steps 10 and 11 can be performed in any order. | - | - | - | - |
| 10 | Check: Does the UE transmit an RRCSetupComplete message and a REGISTRATION REQUEST message on SRB1. | --> | PC5 Relay RLC Channel: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 11 | The UE transmit an RemoteUEInformationSidelink message with sl-RequestedSIB-List-r17 to the value release? | --> | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 12-21 | Steps 11 to 20 of the procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side in TS 38.508-1 Table 4.9.42.2.2-1 using parameter Cast Type (Unicast) and NR-SS-UE1 initiated unicast mode ProSe Direct communication are executed. | - | - | - | - |
| 22 | The UE transmits an ULInformationTransfer message and a PDU SESSION ESTABLISHMENT REQUEST. | --> | PC5 Relay RLC Channel: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 23 | The NR-SS-UE1 is configured with sl-EgressRLC-ChannelPC5 for SRB2 and DRB1 and transmits an RRCReconfiguration message containing sl-L2RemoteUE-Config with the sl-SRAP-ConfigRemote including SRB2 and DRB1 and sl-EgressRLC-ChannelPC5 in sl-MappingToAddModList and an PDU SESSION ESTABLISHMENT ACCEPT. | <-- | PC5 Relay RLC Channel: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 24 | The UE transmits an RRCReconfigurationComplete message. | --> | PC5 Relay RLC Channel: RRCReconfigurationComplete | - | - |
| 25 | NR-SS-UE1 transmits an ACTIVATE TEST MODE message to activate UE radio bearer test mode procedure on SRB2. | <-- | PC5 Relay RLC Channel: DLInformationTransfer TC: ACTIVATE TEST MODE | - | - |
| 26 | Check: Does the UE transmit an ACTIVATE TEST MODE COMPLETE message on SRB2. | --> | PC5 Relay RLC Channel: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE | 2 | P |
| 27 | NR-SS-UE1 transmits a CLOSE UE TEST LOOP message on SRB2. | <-- | PC5 Relay RLC Channel: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 28 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message on SRB2. | --> | PC5 Relay RLC Channel: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 29 | NR-SS-UE1 transmits one IP Packet to verify data path on DRB1. | <-- | PC5 Relay RLC Channel: IP Packet | - | - |
| 30 | Check: Does the UE loop back the IP packet received in step 29 on the DRB1? | --> | PC5 Relay RLC Channel: IP Packet | 3 | P |

12.3.1.1.10.3.3	Specific message contents

Table 12.3.1.1.10.3.3-1: RRCSetup (step 9, Table 12.3.1.1.10.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-21 with condition L2RemoteUE

Table 12.3.1.1.10.3.3-2: SL-SRAP-Config (Table 12.3.1.1.10.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-30A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-SRAP-Config-r17 ::= SEQUENCE { |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { | 1 entry |  |  |
| sl-EgressRLC-ChannelPC5-r17 | SL-RLC-ChannelID using condition PC5RelaySRB1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.3.1.1.10.3.3-3: RRCReconfiguration (step 23, Table 12.3.1.1.10.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition L2RemoteUE and PC5RelaySRB2_DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} | DedicatedNAS-Message |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>