3GPP TSG-RAN5 Meeting #107 	R5-253423

# St Julian's, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3341 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of common UL config of intra-band contiguous 2CA non-contiguous allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, Keysight, Apple Trading, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RB allocations for intra-band contiguous 2UL CA (non-contiguous RB allocation) in Table 6.1A-1b are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RB allocations in Table 6.1A-1b were updated. Edotor’s note was added to 6.1A for clarifying when the RB allocation needs to be updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-band contiguous 2UL CA (non-contiguous RB allocation) will be tested with incorrect RB allocations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | AP#105.22, Depending on the outcome of discussion R5-252492 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-252493. rev1: Editor’s note was updated. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.1A	General

Editor’s note:

Uplink RB allocations for intra-band UL CA with mixed numerology is FFS.

The RB allocations for intra-band contiguous UL CA with non-contiguous allocation need to be reconsidered if a band which does not support SCS = 60 kHz is introduced to intra-band contiguous 2UL CA configuration with BWchannel_CA ≥ 50 MHz.

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

Uplink RB allocations for intra-band CA given in Table 6.1A-1a are used throughout this section, unless otherwise stated by the test case.

<<Unchanged table skipped>>

Table 6.1A-1b: Common uplink configuration for intra-band contiguous 2UL CA (non-contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 24 | 1@24 | 1@22 | 63 | 1@21 | 1@58 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 33 | 1@36 | 1@16 | 62 | 1@21 | 1@30 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 23 | 1@57 | 1@0 | 63 | 1@21 | 1@4 | 104 | 1@0 | 1@24 |
| 25 | 15 | 5+20 | 24 | 1@24 | 1@22 | 79 | 1@24 | 1@77 | 131 | 1@0 | 1@105 |
|  |  | 10+15 | 42 | 1@45 | 1@34 | 79 | 1@26 | 1@52 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 41 | 1@45 | 1@6 | 79 | 1@26 | 1@25 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 23 | 1@84 | 1@0 | 79 | 1@28 | 1@0 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 38 | 1@22 | 1@16 | 74 | 1@13 | 1@25 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 38 | 1@22 | 1@2 | 74 | 1@13 | 1@11 | 124 | 1@0 | 1@23 |
| 30 | 15 | 10+20 | 51 | 1@51 | 1@49 | 96 | 1@31 | 1@74 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 50 | 1@54 | 1@24 | 95 | 1@32 | 1@47 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 49 | 1@58 | 1@0 | 96 | 1@31 | 1@20 | 158 | 1@0 | 1@51 |
|  | 30 | 10+20 | 42 | 1@23 | 1@19 | 90 | 1@15 | 1@35 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 48 | 1@26 | 1@11 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 42 | 1@31 | 1@0 | 90 | 1@15 | 1@8 | 150 | 1@0 | 1@23 |
| 35 | 15 | 15+20 | 60 | 1@63 | 1@43 | 112 | 1@37 | 1@69 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 61 | 1@62 | 1@16 | 112 | 1@37 | 1@42 | 185 | 1@0 | 1@78 |
|  | 30 | 15+20 | 56 | 1@31 | 1@20 | 108 | 1@18 | 1@33 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 56 | 1@31 | 1@7 | 108 | 1@18 | 1@20 | 178 | 1@0 | 1@37 |
| 40 | 15 | 10+30 | 51 | 1@51 | 1@49 | 129 | 1@42 | 1@118 | 212 | 1@0 | 1@159 |
|  |  | 20+20 | 68 | 1@72 | 1@33 | 128 | 1@42 | 1@63 | 212 | 1@0 | 1@105 |
|  |  | 30+10 | 50 | 1@111 | 1@0 | 129 | 1@42 | 1@10 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 44 | 1@23 | 1@20 | 124 | 1@20 | 1@57 | 204 | 1@0 | 1@77 |
|  |  | 20+20 | 64 | 1@35 | 1@15 | 124 | 1@20 | 1@30 | 204 | 1@0 | 1@50 |
|  |  | 30+10 | 42 | 1@58 | 1@0 | 124 | 1@20 | 1@3 | 204 | 1@0 | 1@23 |
| 45 | 15 | 15+30 | 78 | 1@78 | 1@76 | 145 | 1@47 | 1@112 | 239 | 1@0 | 1@159 |
|  | 30 | 15+30 | 72 | 1@37 | 1@34 | 142 | 1@23 | 1@55 | 232 | 1@0 | 1@77 |
| 50 | 15 | 10+40 | 55 | 1@51 | 1@53 | 164 | 1@51 | 1@162 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 86 | 1@90 | 1@69 | 160 | 1@53 | 1@106 | 266 | 1@0 | 1@159 |
|  |  | 30+20 | 86 | 1@90 | 1@15 | 160 | 1@53 | 1@52 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 54 | 1@163 | 1@0 | 164 | 1@53 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 48 | 1@23 | 1@22 | 158 | 1@23 | 1@77 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@44 | 1@33 | 156 | 1@26 | 1@52 | 258 | 1@0 | 1@77 |
|  |  | 30+20 | 82 | 1@44 | 1@6 | 156 | 1@26 | 1@25 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 46 | 1@84 | 1@0 | 158 | 1@28 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 15+40 | 82 | 1@78 | 1@80 | 180 | 1@58 | 1@158 | 295 | 1@0 | 1@215 |
|  | 30 | 15+40 | 76 | 1@37 | 1@36 | 176 | 1@28 | 1@77 | 288 | 1@0 | 1@105 |
| 60 | 15 | 10+50 | 54 | 1@51 | 1@52 | 191 | 1@51 | 1@189 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 106 | 1@105 | 1@104 | 195 | 1@64 | 1@152 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@108 | 1@51 | 192 | 1@64 | 1@95 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 104 | 1@113 | 1@0 | 195 | 1@64 | 1@42 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 53 | 1@218 | 1@0 | 191 | 1@80 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 46 | 1@23 | 1@21 | 186 | 1@23 | 1@91 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 98 | 1@50 | 1@47 | 190 | 1@31 | 1@74 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 100 | 1@53 | 1@24 | 188 | 1@31 | 1@46 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@31 | 1@19 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 46 | 1@111 | 1@0 | 186 | 1@41 | 1@0 | 314 | 1@0 | 1@23 |
| 70 | 15 | 20+50 | 104 | 1@105 | 1@102 | 228 | 1@74 | 1@195 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 124 | 1@126 | 1@89 | 228 | 1@74 | 1@141 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 125 | 1@126 | 1@34 | 228 | 1@74 | 1@85 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 103 | 1@168 | 1@0 | 228 | 1@74 | 1@31 | 376 | 1@0 | 1@105 |
|  | 30 | 20+50 | 96 | 1@50 | 1@46 | 224 | 1@36 | 1@96 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 120 | 1@62 | 1@43 | 224 | 1@36 | 1@69 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 120 | 1@62 | 1@15 | 224 | 1@36 | 1@41 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 96 | 1@86 | 1@0 | 224 | 1@36 | 1@14 | 368 | 1@0 | 1@50 |
| 80 | 15 | 30+50 | 141 | 1@145 | 1@125 | 260 | 1@85 | 1@184 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 141 | 1@146 | 1@70 | 260 | 1@86 | 1@129 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 142 | 1@144 | 1@15 | 260 | 1@85 | 1@74 | 430 | 1@0 | 1@159 |
|  | 30 | 30+50 | 136 | 1@72 | 1@61 | 256 | 1@42 | 1@91 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 136 | 1@72 | 1@33 | 256 | 1@42 | 1@63 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 256 | 1@42 | 1@36 | 422 | 1@0 | 1@77 |
| 90 | 15 | 40+50 | 160 | 1@163 | 1@106 | 293 | 1@97 | 1@173 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 159 | 1@164 | 1@52 | 292 | 1@97 | 1@118 | 486 | 1@0 | 1@215 |
|  | 30 | 40+50 | 154 | 1@81 | 1@51 | 288 | 1@48 | 1@85 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 154 | 1@81 | 1@24 | 288 | 1@48 | 1@58 | 478 | 1@0 | 1@105 |
| 100 | 15 | 50+50 | 176 | 1@182 | 1@87 | 325 | 1@108 | 1@162 | 540 | 1@0 | 1@269 |
|  | 30 | 20+80 | 96 | 1@50 | 1@46 | 320 | 1@50 | 1@158 | 536 | 1@0 | 1@216 |
|  |  | 50+50 | 172 | 1@90 | 1@42 | 320 | 1@53 | 1@79 | 532 | 1@0 | 1@132 |
|  |  | 80+20 | 94 | 1@171 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 74 | 1@37 | 1@35 | 324 | 1@37 | 1@160 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 124 | 1@64 | 1@60 | 340 | 1@56 | 1@160 | 564 | 1@0 | 1@216 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 46 | 1@23 | 1@21 | 324 | 1@23 | 1@160 | 594 | 1@0 | 1@272 |
|  |  | 20+90 | 96 | 1@50 | 1@46 | 348 | 1@50 | 1@172 | 592 | 1@0 | 1@244 |
|  |  | 30+80 | 150 | 1@77 | 1@73 | 356 | 1@59 | 1@158 | 590 | 1@0 | 1@216 |
|  |  | 40+70 | 194 | 1@99 | 1@89 | 356 | 1@59 | 1@130 | 590 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@100 | 1@62 | 354 | 1@59 | 1@102 | 590 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@100 | 1@33 | 354 | 1@59 | 1@73 | 590 | 1@0 | 1@132 |
|  |  | 70+40 | 192 | 1@100 | 1@6 | 356 | 1@59 | 1@47 | 590 | 1@0 | 1@105 |
|  |  | 80+30 | 150 | 1@143 | 1@0 | 356 | 1@59 | 1@19 | 590 | 1@0 | 1@77 |
|  |  | 90+20 | 94 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  |  | 100+10 | 44 | 1@252 | 1@0 | 324 | 1@112 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 74 | 1@37 | 1@35 | 352 | 1@37 | 1@174 | 622 | 1@0 | 1@272 |
|  |  | 25+90 | 124 | 1@64 | 1@60 | 372 | 1@62 | 1@182 | 620 | 1@0 | 1@244 |
|  |  | 90+25 | 124 | 1@184 | 1@0 | 372 | 1@62 | 1@2 | 620 | 1@0 | 1@64 |
|  |  | 100+15 | 72 | 1@238 | 1@0 | 352 | 1@98 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 96 | 1@50 | 1@46 | 376 | 1@50 | 1@186 | 648 | 1@0 | 1@272 |
|  |  | 30+90 | 150 | 1@77 | 1@73 | 388 | 1@65 | 1@180 | 646 | 1@0 | 1@244 |
|  |  | 40+80 | 206 | 1@105 | 1@101 | 390 | 1@64 | 1@152 | 646 | 1@0 | 1@216 |
|  |  | 50+70 | 210 | 1@109 | 1@80 | 388 | 1@64 | 1@124 | 644 | 1@0 | 1@188 |
|  |  | 60+60 | 212 | 1@109 | 1@52 | 390 | 1@65 | 1@97 | 648 | 1@0 | 1@161 |
|  |  | 70+50 | 210 | 1@109 | 1@24 | 388 | 1@64 | 1@68 | 644 | 1@0 | 1@132 |
|  |  | 80+40 | 204 | 1@116 | 1@0 | 390 | 1@64 | 1@41 | 646 | 1@0 | 1@105 |
|  |  | 90+30 | 150 | 1@171 | 1@0 | 388 | 1@65 | 1@13 | 646 | 1@0 | 1@77 |
|  |  | 100+20 | 94 | 1@227 | 1@0 | 376 | 1@86 | 1@0 | 648 | 1@0 | 1@50 |
| 125 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 25+100 | 124 | 1@64 | 1@60 | 404 | 1@64 | 1@200 | 676 | 1@0 | 1@272 |
|  |  | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 150 | 1@77 | 1@73 | 422 | 1@70 | 1@202 | 702 | 1@0 | 1@272 |
|  |  | 40+90 | 206 | 1@105 | 1@101 | 422 | 1@70 | 1@174 | 702 | 1@0 | 1@244 |
|  |  | 50+80 | 228 | 1@118 | 1@98 | 422 | 1@70 | 1@147 | 700 | 1@0 | 1@216 |
|  |  | 60+70 | 230 | 1@118 | 1@70 | 422 | 1@70 | 1@118 | 702 | 1@0 | 1@188 |
|  |  | 70+60 | 230 | 1@118 | 1@43 | 422 | 1@70 | 1@91 | 702 | 1@0 | 1@161 |
|  |  | 80+50 | 228 | 1@118 | 1@14 | 422 | 1@70 | 1@63 | 700 | 1@0 | 1@132 |
|  |  | 90+40 | 204 | 1@144 | 1@0 | 422 | 1@70 | 1@35 | 702 | 1@0 | 1@105 |
|  |  | 100+30 | 148 | 1@200 | 1@0 | 422 | 1@70 | 1@7 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 206 | 1@105 | 1@101 | 456 | 1@76 | 1@197 | 758 | 1@0 | 1@272 |
|  |  | 50+90 | 248 | 1@127 | 1@117 | 456 | 1@75 | 1@169 | 756 | 1@0 | 1@244 |
|  |  | 60+80 | 250 | 1@127 | 1@89 | 456 | 1@76 | 1@141 | 758 | 1@0 | 1@216 |
|  |  | 70+70 | 248 | 1@127 | 1@61 | 454 | 1@76 | 1@113 | 756 | 1@0 | 1@188 |
|  |  | 80+60 | 248 | 1@128 | 1@34 | 456 | 1@76 | 1@86 | 758 | 1@0 | 1@161 |
|  |  | 90+50 | 248 | 1@127 | 1@5 | 456 | 1@75 | 1@57 | 756 | 1@0 | 1@132 |
|  |  | 100+40 | 206 | 1@171 | 1@0 | 456 | 1@76 | 1@30 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 260 | 1@132 | 1@128 | 488 | 1@81 | 1@191 | 812 | 1@0 | 1@272 |
|  |  | 60+90 | 268 | 1@137 | 1@108 | 490 | 1@81 | 1@163 | 814 | 1@0 | 1@244 |
|  |  | 70+80 | 266 | 1@137 | 1@80 | 488 | 1@81 | 1@135 | 812 | 1@0 | 1@216 |
|  |  | 80+70 | 266 | 1@137 | 1@52 | 488 | 1@81 | 1@107 | 812 | 1@0 | 1@188 |
|  |  | 90+60 | 266 | 1@137 | 1@24 | 490 | 1@81 | 1@80 | 814 | 1@0 | 1@161 |
|  |  | 100+50 | 260 | 1@144 | 1@0 | 488 | 1@81 | 1@51 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 286 | 1@146 | 1@126 | 524 | 1@87 | 1@186 | 870 | 1@0 | 1@272 |
|  |  | 70+90 | 284 | 1@146 | 1@98 | 522 | 1@87 | 1@158 | 868 | 1@0 | 1@244 |
|  |  | 80+80 | 284 | 1@146 | 1@70 | 522 | 1@87 | 1@130 | 868 | 1@0 | 1@216 |
|  |  | 90+70 | 284 | 1@146 | 1@42 | 522 | 1@87 | 1@102 | 868 | 1@0 | 1@188 |
|  |  | 100+60 | 286 | 1@146 | 1@15 | 524 | 1@87 | 1@75 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 304 | 1@155 | 1@117 | 556 | 1@92 | 1@180 | 924 | 1@0 | 1@272 |
|  |  | 80+90 | 304 | 1@155 | 1@89 | 556 | 1@92 | 1@152 | 924 | 1@0 | 1@244 |
|  |  | 90+80 | 304 | 1@155 | 1@61 | 556 | 1@92 | 1@124 | 924 | 1@0 | 1@216 |
|  |  | 100+70 | 304 | 1@155 | 1@33 | 556 | 1@92 | 1@96 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 322 | 1@165 | 1@108 | 588 | 1@98 | 1@174 | 980 | 1@0 | 1@272 |
|  |  | 90+90 | 322 | 1@165 | 1@80 | 588 | 1@98 | 1@146 | 980 | 1@0 | 1@244 |
|  |  | 100+80 | 322 | 1@165 | 1@52 | 588 | 1@98 | 1@118 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 340 | 1@174 | 1@98 | 622 | 1@104 | 1@169 | 1036 | 1@0 | 1@272 |
|  |  | 100+90 | 340 | 1@174 | 1@70 | 622 | 1@104 | 1@141 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@183 | 1@89 | 656 | 1@109 | 1@163 | 1092 | 1@0 | 1@272 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3. NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0. |  |  |  |  |  |  |  |  |  |  |  |

<<End of change>>