3GPP TSG-RAN5 Meeting #107	R5-252320r1

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3319 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MPR, SEM and ACLR for 4Tx UL-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 introduced the requirements for 4Tx UL-MIMO. RAN5 needs to introduce the corresponding test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum requirements is introduced for 4Tx MPR, SEM and ACLR - Adding the test details for MPR, SEM and ACLR of 4Tx UL-MIMO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR, SEM and ACLR of 4Tx UL-MIMO can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.2, 6.2D.2_2 (new), 6.5D.2.2, 6.5D.2.2_2(new), 6.5D.2.4.1, 6.5D.2.4.1_2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 1020 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Undo changes across titles of Table 6.2D.2.3-1~3 to avoid conflit with R5-252846 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2D.2	UE maximum output power reduction for UL MIMO

6.2D.2.1	Test purpose

To verify that the power reduction of UE due to higher order modulations and transmit bandwidth configuration does not exceed the specified maximum power reduction.

6.2D.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed.

6.2D.2.3	Minimum conformance requirements

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 for 2Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band and Table 6.2.2.3-2 for 2Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 with 2Tx, Table 6.2D.2.3-4, 6.2D.2.3-5 for PC1.5 with 4Tx respectively. For UE power class 1.5 with 2Tx, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behaviour specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16, the allowed MPR for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 when TxD is indicated and Table 6.2.2.3-2 when TxD is not indicated for PC2 , Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 with 2Tx, Table 6.2D.2-4, 6.2D.2-5 for PC1.5 with 4Tx respectively, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode. A UE with 2Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the maximum output power requirement with MPR according to clause 6.2.2.3. When a UE that indicates PC1.5 for a given band is limited to PC2 by the rules in clause 6.2.1.3, the MPR requirements in Table 6.2.2.3-2 apply. For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors.

The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of .

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the corresponding requirements in clause 6.2D.1.3 apply for the power class as indicated by the ue-PowerClass field in capability signalling. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 with MPR according to clause 6.2.2.3 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port with precoding matrix W=1 [6.3.1.5 TS 38.211].

Unless otherwise specified, ‘pi/2 BPSK’ refers to both variants of pi/2 BPSK referenced in Table 6.2.2.3-1.

Table 6.2D.2.3-1: Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6.2D.2.3-2: Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Table 6.2D.2.3-3: Maximum power reduction (MPR) for power class 1.5 FWA with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.2D.2.3-4: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 8.0 | ≤ 3.0 | ≤ 2.0 |
|  | QPSK | ≤ 8.5 | ≤ 3.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.5 | ≤ 4.0 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.5 | ≤ 4.7 | ≤ 4.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 5.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 5.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |
| NOTE 1: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.2D.2.3-5: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 7.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 8.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 4.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 8.0 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 5.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ 8.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Inner, outer and edge allocations are as defined in section 6.2.2.3 except for 	PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.2.

6.2D.2.4	Test description

6.2D.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.2.4.1-1, Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2, Table 6.2D.2.4.1-2a, Table 6.2D.2.4.1-3 and Table 6.2D.2.4.1-3a. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2.4.1-1: Test Configuration Table for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-1a: Test Configuration Table for power class 3 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 24 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 34 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 44 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2D.2.4.1-2: Test Configuration Table for Power Class 2 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-2a: Test Configuration Table for power class 2 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

Table 6.2D.2.4.1-3: Test Configuration Table for Power Class 1.5 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-3a: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2.4.1-1, Table 6.2D.2.4.1-2 or Table 6.2D.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2.4.3.

6.2D.2.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1, Table 6.2D.2.5-2, Table 6.2D.2.5-2a, Table 6.2D.2.5-3, or Table 6.2D.2.5-4. The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1a, Table 6.2D.2.5-2b, Table 6.2D2.5-2c, Table 6.2D.2.5-3a, or Table 6.2D.2.5-4a.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n13, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79, n85, n97, n104, n105) for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. NOTE 4: Applicable for band n24. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-1a: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n13, n24, n25, n26, n28, n30, n34, n38, n41, n48, n66, n70, n71, n77, n78, n79, n85, n104, n105) for Power Class 3 supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （14.02 - TT） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. NOTE 4: Only applicable for band n24. NOTE 5: For band n24 when transmission bandwidths are not confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2: UE Power Class test requirements (for Bands n1, n3, n8, n40, n41, n66, n70, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO without indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For n1, n3, and Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2a: UE Power Class test requirements (for Bands n8, n40, n41, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO and indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 5 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 9 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 13 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 14 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2b: UE Power Class test requirements (for Bands n1, n3, n8, n40, n41, n66, n70, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and ul-FullPwrMode1-TPMIGroup-r1 and indicating TxD

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52- TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 4 | 26 | 0 | 1.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.02 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2c: UE Power Class test requirements (for Bands n1, n3, n8, n40, n41, n66, n70, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and, (supporting ul-FullPwrMode1-TPMIGroup-r1 without indicating TxD) or (supporting ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52 - TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.02 - TT） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 2 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.02 - TT） |
| 5 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.02 - TT） |
| 9 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | （17.52 - TT） |
| 12 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 13 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.02 - TT） |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.52 - TT） |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.52 - TT） |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.52 - TT） |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.02 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.02 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.02 - TT） |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | （26.02） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.02 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.02 - TT） |
| 13 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 15 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.52 - TT） |
| 16 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.03 - TT） |
| 2 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 3 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 4 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 – TT | （20.53 - TT） |
| 5 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.53 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 8 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.53 - TT） |
| 9 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 11 | 29 | 0 | 4.5 | 0 | （1.53） | 24.5 | （23.03） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.03 - TT） |
| 12 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| 13 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| 14 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.53 - TT） |
| 2 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.53 - TT） |
| 3 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.53 - TT） |
| 4 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 – TT | （23.03 - TT） |
| 5 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.53 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 8 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.53 - TT） |
| 9 | 29 | 0 | 1 | 0 | （1.53） | 28.0 | （26.53） | 3.0 |  | 3 | 31.0 + TT | 25.0 - TT | （23.53 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 11 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 12 | 29 | 0 | 3 | 0 | （1.53） | 26.0 | （24.53） | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | （21.53 - TT） |
| 13 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 14 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 15 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | （24.03） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.03 - TT） |
| 16 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 17 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 18 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | （22.03） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.5 - TT | （17.03 - TT） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.2_1	UE maximum output power reduction for SUL with UL MIMO

6.2D.2_1.1	Test purpose

Same test purpose as in clause 6.2D.2.1

6.2D.2_1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1_1 is executed.

6.2D.2_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The minimum requirements are same as in 6.2D.1.3 with following exceptions:

Instead of Table 6.2D.1.3-1  use Table 6.2D.1_1.3-1

6.2D.2_1.4	Test description

6.2D.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.2D.2_1.4.1-1 ~ 6.2D.2_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2_1.4-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2_1.4.1-2: Test Configuration Table for Power Class 2

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2_1.4.1-1 ~ Table 6.2D.2_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2_1.4.3.

6.2D.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2_1.4.1-1 and Table 6.2D.2_1.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

6.2D.2_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2_1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2_1.5-1~6.2D.2_1.5-2.

Table 6.2D.2_1.5-1: UE Power Class test requirements (for Band n80, n81, n84, n95, n97) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 25.0 + TT | 16.5 - TT | （14.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2D.2_1.5-1a: UE Power Class test requirements for Band n99 for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 4 | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 4 | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-2: UE Power Class test requirements (for Band n80, n84, n95, n97) for Power Class 2

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （162 - TT） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （222） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （172 - TT） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （162 - TT） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （152 - TT） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （152 - TT） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （152 - TT） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80 and n84, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-3: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.2_2	UE maximum output power reduction for UL MIMO for UE supporting 4Tx

6.2D.2_2.1	Test purpose

Same test purpose as in clause 6.2D.2.1.

6.2D.2_2.2	Test applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4-layer codebook based UL MIMO.

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx and UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed.

6.2D.2_2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2D.2.3.

6.2D.2_2.4	Test description

6.2D.2_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.2_2.4.1-1, Table 6.2D.2_2.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2_2.4.1-1: Test Configuration Table for Power Class 1.5 UEs supporting 4-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  | CP-OFDM QPSK | Inner_Full |
| 2 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 3 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 4 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 5 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 6 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 7 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. NOTE 3: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.2D.2_2.4.1-2: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 6 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 7 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 8 | Default |  | DFT-s-OFDM 64 QAM | Inner Full |
| 9 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 12 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.2D.2_2.4.1-3: Test Configuration Table for Power Class 1.5 FWA UEs supporting 4-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  | CP-OFDM QPSK | Inner_Full |
| 2 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 3 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 4 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 5 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 6 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 7 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 8 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 9 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. NOTE 3: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.2D.2_2.4.1-4: Test Configuration Table for power class 1.5 FWA UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 6 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 7 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 8 | Default |  | DFT-s-OFDM 64 QAM | Inner Full |
| 9 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2_2.4.1-1, Table 6.2D.2_2.4.1-2 or Table 6.2D.2_2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2_2.4.3.

6.2D.2_2.4.2	Test procedure

Sub-test for 4-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2_2.4.1-1 and Table 6.2D.2_2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from all transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.2_2.4.1-2 and Table 6.2D.2_2.4.1-4. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1_4Tx in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration in clause 6.2D.2_2.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2D.2_2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 4TX_UL_MIMO.

6.2D.2_2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2_2.5-1, Table 6.2D.2_2.5-3.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2_2.5-2, Table 6.2D.2_2.5-4.

Table 6.2D.2_2.5-1: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs supporting 4-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.02 - TT） |
| 2 | 29 | 0 | 4.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.0 - TT | （20.02 - TT） |
| 3 | 29 | 0 | 5.0 | 0 | （1.52） | 24.0 | （22.52） | 5.0 | 5.0 | 3 | 31.0 + TT | 19.0 - TT | （17.52 - TT） |
| 4 | 29 | 0 | 7.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | 5.0 | 3 | 31.0 + TT | 17.0 - TT | （15.52 - TT） |
| 5 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | 5.0 | 3 | 31.0 + TT | 14.5 - TT | （13.02 - TT） |
| 6 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | 5.0 | 3 | 31.0 + TT | 14.5 - TT | （13.02 - TT） |
| 7 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | 5.0 | 3 | 31.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_2.5-2: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 2 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 3 | 29 | 0 | 3.0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | （21.52 - TT） |
| 4 | 29 | 0 | 2.0 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 5 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.5 - TT | （19.02 - TT） |
| 6 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 7 | 29 | 0 | 4.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.0 - TT | （18.52 - TT） |
| 8 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 21.5 - TT | （18.02 - TT） |
| 9 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 10 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 11 | 29 | 0 | 4.7 | 0 | （1.52） | 24.3 | （22.82） | 5.0 |  | 3 | 31.0 + TT | 19.3 - TT | （17.82 - TT） |
| 12 | 29 | 0 | 7.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 31.0 + TT | 17.0 - TT | （15.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_2.5-3: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs supporting 4-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 2.0 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 2 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 3 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 21.5 - TT | （18.02 - TT） |
| 4 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 5 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 29 | 0 | 5.0 | 0 | （1.52） | 24.0 | （22.52） | 5.0 |  | 3 | 31.0 + TT | 19.0 - TT | （17.52 - TT） |
| 7 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 8 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 9 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_2.5-4: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.02 - TT） |
| 3 | 29 | 0 | 2.0 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 4 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | （24.02 - TT） |
| 5 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 6 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | （26.02） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.02 - TT） |
| 7 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.5 - TT | （19.02 - TT） |
| 8 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.5 - TT | （19.02 - TT） |
| 9 | 29 | 0 | 4.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 | 5.0 | 3 | 31.0 + TT | 22.0 - TT | （18.52 - TT） |
| 10 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 11 | 29 | 0 | 8.0 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 31.0 + TT | 16.0 - TT | （14.52 - TT） |
| 12 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

< Unchanged Text Skipped >

6.5D.2.2	Spectrum emission mask for UL MIMO

6.5D.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth.

6.5D.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that supports 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.5D.2.2.3	Minimum conformance requirements

For UE supporting UL MIMO or uplink full power transmission (ULFPTx) for UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from all UE transmit antenna connectors.

For UEs with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.3.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2.2.3 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.5.2.2 apply when TxD is not indicated, and the requirements in clause 6.5G.2 apply when TxD is indicated.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.2 and 6.5.2.2

6.5D.2.2.4	Test description

6.5D.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in tables 6.5D.2.2.4.1-1 to 6.5D.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.2.4.1-1: Test Configuration Table for power class 3, power class 2 and power class 1.5

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 10 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.5D.2.2.4.1-2: Test Configuration Table for power class 3 with supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 24 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 34 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 44 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.5D.2.2.4.1-3: Test Configuration Table for power class 2 with supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

Table 6.5D.2.2.4.1-4: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to tables 6.5D.2.2.4.1-1 to 6.5D.2.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.2.4.3

6.5D.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.2.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2D.2.5-1 or 6.2D.2.5-2 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of power of the transmitted signal from both antenna connectors with a measurement filter of bandwidths according to Table 6.5D.2.2.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.5D.2.2.4.1-2 through Table 6.5D.2.2.4.1-4 as appropriate. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.5D.2.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5D.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.2.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2D.2.5 as appropriate, and the sum of power of any UE emission measured from both antennas in step 4 and step 5 shall fulfill requirements in Table 6.5D.2.2.5-1.

Table 6.5D.2.2.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.2.5-2. |  |  |  |  |

Table 6.5D.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5D.2.2_1	Spectrum emission mask for SUL with UL MIMO

6.5D.2.2_1.1	Test purpose

Same as test purpose in clause 6.5D.2.2.1.

6.5D.2.2_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

This test applies to all types of NR UE release 17 and forward that support SUL and UL full power transmission (ULFPTx) on the SUL bands.

6.5D.2.2_1.3	Minimum conformance requirements

Same as minimum conformance requirements in clause 6.5D.2.2.3.

6.5D.2.2_1.4	Test description

6.5D.2.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing and are shown in tables 6.5D.2.2_1.4.1-1 to 6.5D.2.2_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.2_1.4.1-1: Test Configuration Table for power class 3 and power class 2

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range for SUL carrier Mid-range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  | N/A | N/A | Modulation |  | RB allocation |
| 1 | Low |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | High |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 3 | Default |  |  | CP-OFDM QPSK |  | Outer_Full |
| 4 | Low |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 5 | High |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 6 | Default |  |  | CP-OFDM 16 QAM |  | Outer_Full |
| 7 | Low |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 8 | High |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  | CP-OFDM 64 QAM |  | Outer_Full |
| 10 | Low |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 11 | High |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  | CP-OFDM 256 QAM |  | Outer_Full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.5D.2.2_1.4.1-2: Test Configuration Table for UE supporting ULFPTx

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range for SUL carrier Mid-range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  | N/A | N/A | Modulation |  | RB allocation |
| 15 | Low |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 25 | High |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 35 | Default |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer_Full |
| 45 | Low |  |  | DFT-s-OFDM QPSK |  | Edger_1RB_Left |
| 5 | High |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  | DFT-s-OFDM QPSK |  | Outer_Full |
| 7 | Low |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  | DFT-s-OFDM 16 QAM |  | Outer_Full |
| 10 | Low |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  | DFT-s-OFDM 64 QAM |  | Outer_Full |
| 13 | Low |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  | DFT-s-OFDM 256 QAM |  | Outer_Full |
| 16 | Low |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  | CP-OFDM QPSK |  | Outer_Full |
| 19 | Low |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  | CP-OFDM 16 QAM |  | Outer_Full |
| 22 | Low |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  | CP-OFDM 64 QAM |  | Outer_Full |
| 25 | Low |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  | CP-OFDM 256 QAM |  | Outer_Full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RF allocation is defined in Table 6.1-1. Note 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. Note 4: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. Note 5: For UE indicating PC2, or UE indicating PC3 and operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5D.2.2_1.4.1-1 and 6.5D.2.2_1.4.1-2

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.4.3.4.3.

6.5D.2.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.2.2_1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2D.1_1.5-1. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of power of the transmitted signal from both antenna connectors with a measurement filter of bandwidths according to Table 6.5D.2.2_1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.5D.2.2_1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.5D.2.2_1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5D.2.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.5D.2.2_1.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables [6.2D.2.5-1, 6.2D.2.5-2, 6.2D.2.5-2a and 6.2D.2.5-2b] as appropriate, and the sum of power of any UE emission measured from both antennas in step 4 and step 5 shall fulfil requirements in Table 6.5D.2.2.5-1.

Table 6.5D.2.2_1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.2.5-2. |  |  |  |  |

Table 6.5D.2.2_1.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5D.2.2_2	Spectrum emission mask for UL MIMO for UE supporting 4Tx

6.5D.2.2_2.1	Test purpose

Same as the test purpose in 6.5D.2.2.1

6.5D.2.2_2.2	Test applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4-layer codebook based UL MIMO.

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx and UL full power transmission (ULFPTx).

6.5D.2.2_2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5D.2.2.3.

6.5D.2.2_2.4	Test description

6.5D.2.2_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in tables 6.5D.2.2_2.4.1-1 to 6.5D.2.2_2.4.1-4. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.2.2_2.4.1-1: Test Configuration Table for Power Class 1.5 UEs supporting 4-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 2 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 3 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 4 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 5 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. NOTE 3: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.5D.2_2.4.1-2: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 6 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 9 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.5D.2.2_2.4.1-3: Test Configuration Table for Power Class 1.5 FWA UEs supporting 4-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 2 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 3 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 5 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 6 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 7 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. NOTE 3: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.5D.2.2_2.4.1-4: Test Configuration Table for power class 1.5 FWA UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 6 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 7 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to tables 6.5D.2.2_2.4.1-1 to 6.5D.2.2_2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.2_2.4.3

6.5D.2.2_2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.2.2_2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2D.2.5-1 or 6.2D.2.5-2 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of power of the transmitted signal from all antenna connectors with a measurement filter of bandwidths according to Table 6.5D.2.2_2.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.5D.2.2_2.4.1-2 and Table 6.5D.2.2_2.4.1-4 as appropriate. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1_4Tx in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in clause 6.5D.2.2_2.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5D.2.2_2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO.

6.5D.2.2_2.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2D.2_2.5 as appropriate, and the sum of power of any UE emission measured from both antennas in step 4 and step 5 shall fulfill requirements in Table 6.5D.2.2_2.5-1.

Table 6.5D.2.2_2.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.2_2.5-2. |  |  |  |  |

Table 6.5D.2.2_2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5D.2.3	Additional spectrum emission mask for UL MIMO

Editor’s note:

-Test coverage for the NS_XXs other than NS_03, NS_03U, NS_04 and NS_35 is FFS

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U in A-MPR

6.5D.2.3.1	Test purpose

To verify that the sum of power of any UE emission from both transmit antennas shall not exceed specified lever for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5D.2.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3UE release 15 and forward that supports 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3UE release 16 and forward that supports UL full power transmission (ULFPTx).

< Unchanged Text Skipped >

6.5D.2.4.1	NR ACLR for UL MIMO

6.5D.2.4.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5D.2.4.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that supports 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.5D.2.4.1.3	Minimum conformance requirements

For UE supporting UL MIMO or uplink full power transmission (ULFPTx) for UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UEtransmit antenna connectors.

For UEs with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2.4.1.3 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.3.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2.4.1.3 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.5.2.4.1.3 apply when TxD is not indicated, and the requirements in clause 6.5G.2.3 apply when TxD is indicated.

The normative reference for this requirement is TS 38.101-1 [2] clauses and 6.5D.2 and 6.5.2.4.1.

6.5D.2.4.1.4	Test description

6.5D.2.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in the test configuration tables in clause 6.2D.2. 4.1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.4.1.4.1-1: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to the test configuration tables in clause 6.2D.2.4.1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.4.1.4.3 (If UE supports ULFPTx. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.)

6.5D.2.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2D.2. 4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at from both antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2D.2.4.2,which shall meet the requirements described in clause 6.2D.2.5 as appropriate.

4.	Measure the sum of the rectangular filtered mean power for the assigned NR channel at each antenna connector of UE using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of rectangular filtered mean power of the first NR adjacent channel at each antenna connector of UE on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

7.	If UE supports ULFPTx, repeat test steps 1~6 with UL RMC according to Table 6.2D.2.4.1-1a through Table 6.2D.2.4.1-3a. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.5D.2.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.4.1.5	Test requirement

The measured UE mean total power derived in step 3, shall fulfil requirements in Clause 6.2D.2.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6 for each antenna connector, shall be higher than the limits in Table 6.5D.2.4.1.5-2.

Table 6.5D.2.4.1.5-1: Void

Table 6.5D.2.4.1.5-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR |  | 31 - TT dB | 31 - TT dB | 30 - TT dB |
| NOTE 1: TT = 0.8 dB |  |  |  |  |

6.5D.2.4.1_1	NR ACLR for SUL with UL MIMO

6.5D.2.4.1_1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage Power Ratio (ACLR).

6.5D.2.4.1_1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.2.4.1_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UEtransmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2.4.2.3 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1_1.3.

The normative reference for this requirement is TS 38.101-1 [2] clauses and 6.5D.2 and 6.5.2.4.1.

6.5D.2.4.1_1.4	Test description

6.5D.2.4.1_1.4.1	Initial conditions

Same initial conditions as in 6.2D.2_1.4.1.

6.5D.2.4.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2D.2_1. 4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at from both antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2D.2_1.4.2, which shall meet the requirements described in clauses 6.2D.2_1.5 as appropriate.

4.	Measure the sum of the rectangular filtered mean power for the assigned NR channel at each antenna connector of UE using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of rectangular filtered mean power of the first NR adjacent channel at each antenna connector of UE on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

6.5D.2.4.1_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.4.1_1.5	Test requirement

The measured UE mean total power derived in step 3, shall fulfil requirements in clause 6.2D.2_1.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6 for each antenna connector, shall be higher than the limits in Table 6.5D.2.4.1_1.5-1.

Table 6.5D.2.4.1_1.5-1: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR |  | 31 - TT dB | 30 - TT dB |
| NOTE 1: TT = 0.8 dB |  |  |  |

6.5D.2.4.1_2	NR ACLR for UL MIMO for UE supporting 4Tx

6.5D.2.4.1_2.1	Test purpose

Same as the test purpose in 6.5D.2.4.1.1.

6.5D.2.4.1_2.2	Test applicability

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4-layer codebook based UL MIMO.

This test applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx and UL full power transmission (ULFPTx).

6.5D.2.4.1_2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5D.2.4.1.3.

6.5D.2.4.1_2.4	Test description

Same test description as specified in clause 6.5D.2.4.1.4 with following exceptions:

For initial condition:

1.	The requirements shall be tested according to test configuration tables in clause 6.2D.2_2.4.1.

2.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

For test procedure:

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to configuration tables in clause 6.2D.2_2.4.1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 4TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

3.	Measure the sum of mean power of the UE from all antenna connectors in the channel bandwidth of the radio access mode according to the test configurations, as measured in step 3 of 6.2D.2_2.4.2, which shall meet the requirements described in clause 6.2D.2_2.5 as appropriate.

4	Measure the sum of the rectangular filtered mean power for the assigned NR channel from all antenna connectors of UE using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of rectangular filtered mean power of the first NR adjacent channel from all antenna connectors of UE on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

7.	If UE supports ULFPTx, repeat test steps 1~6 with UL RMC according to Table 6.2D.2_2.4.1-2 and Table 6.2D.2_2.4.1-4. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1_4Tx in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

For message contents:

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 4TX_UL_MIMO.

6.5D.2.4.1_2.5	Test requirement

The measured UE mean total power derived in step 3, shall fulfil requirements in Clause 6.2D.2_2.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6 for each antenna connector, shall be higher than the limits in Table 6.5D.2.4.1_2.5-1.

Table 6.5D.2.4.1_2.5-1: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR |  | 31 - TT dB | 31 - TT dB | 30 - TT dB |
| NOTE 1: TT = 0.8 dB |  |  |  |  |

< End of Changes >